Alfred Arnold, Stefan Hilse, Stephan Kanthak, Oliver Sellke, Vittorio

De Tomasi

Macro Assembler AS V1.42

--------------------------------------------

User's Manual

Edition June 2024

IBM,  PPC403Gx,  OS/2,  and  PowerPC  are  registered trademarks of IBM

Corporation.

Intel,  MCS-48,  MCS-51,  MCS-251,  MCS-96,  MCS-196  und  MCS-296  are

registered trademarks of Intel Corp. .

Motorola and ColdFire are registered trademarks of Motorola Inc. .

MagniV is a registered trademark of Freescale Semiconductor.

PicoBlaze is a registered trademark of Xilinx Inc.

eZ80 and Z80 are registered trademarks of Zilog Inc.

UNIX is a registered trademark of the The Open Group.

Linux is a registered trademark of Linus Thorvalds.

Microsoft,  Windows, and MS-DOS are  registered trademarks of Microsoft

Corporation.

All  other  trademarks  not  explicitly  mentioned  in this section and

used in this manual are properties of their respective owners.

This  document has  been processed  with the  LaTeX typesetting system,

using the Linux operating system.

Contents

1. Introduction

1.1. License Agreement

1.2. General Capabilities of the Assembler

1.3. Supported Platforms

2. Assembler Usage

2.1. Hardware Requirements

2.2. Delivery

2.3. Installation

2.4. Start-Up Command, Parameters

2.5. Format of the Input Files

2.6. Format of the Listing

2.7. Symbol Conventions

2.8. Temporary Symbols

2.9. Named Temporary Symbols

2.9.1. Nameless Temporary Symbols

2.9.2. Composed Temporary Symbols

2.10. Formula Expressions

2.10.1. Integer Constants

2.10.2. Floating Point Constants

2.10.3. String Constants

2.10.4. String to Integer Conversion and Character Constants

2.10.5. Evaluation

2.10.6. Operators

2.10.7. Functions

2.11. Forward References and Other Disasters

2.12. Register Symbols

2.13. Share File

2.14. Processor Aliases

3. Pseudo Instructions

3.1. Definitions

3.1.1. SET, EQU, and CONSTANT

3.1.2. SFR and SFRB

3.1.3. XSFR and YSFR

3.1.4. LABEL

3.1.5. BIT

3.1.6. DBIT

3.1.7. DEFBIT and DEFBITB

3.1.8. DEFBITFIELD

3.1.9. PORT

3.1.10. REG and NAMEREG

3.1.11. LIV and RIV

3.1.12. CHARSET

3.1.13. CODEPAGE

3.1.14. ENUM, NEXTENUM, and ENUMCONF

3.1.15. PUSHV and POPV

3.2. Code Modification

3.2.1. ORG

3.2.2. RORG

3.2.3. CPU

3.2.4. SUPMODE, FPU, PMMU, CUSTOM

3.2.5. ACCMODE

3.2.6. CIS, EIS, FIS and FP11

3.2.7. FULLPMMU

3.2.8. PADDING

3.2.9. PACKING

3.2.10. WARNRELATIVE

3.2.11. MAXMODE

3.2.12. EXTMODE and LWORDMODE

3.2.13. SRCMODE

3.2.14. PLAINBASE

3.2.15. BIGENDIAN

3.2.16. WRAPMODE

3.2.17. PANEL

3.2.18. SEGMENT

3.2.19. PHASE and DEPHASE

3.2.20. SAVE and RESTORE

3.2.21. ASSUME

3.2.22. CKPT

3.2.23. EMULATED

3.2.24. BRANCHEXT

3.2.25. Z80SYNTAX

3.2.26. EXPECT and ENDEXPECT

3.3. Data Definitions

3.3.1. DC[.Size]

3.3.2. DS[.Size]

3.3.3. BLKB, BLKW, BLKL, BLKD

3.3.4. DN,DB,DW,DD,DQ,DT, and DO

3.3.5. FLT2, FLT3, FLT4

3.3.6. x_FLOATING

3.3.7. DS, DS8

3.3.8. BLKx

3.3.9. BYT or FCB

3.3.10. BYTE

3.3.11. DC8

3.3.12. ADR or FDB

3.3.13. DDB

3.3.14. DCM

3.3.15. WORD

3.3.16. DW16

3.3.17. ACON

3.3.18. LONG

3.3.19. SINGLE, DOUBLE, and EXTENDED

3.3.20. FLOAT and DOUBLE

3.3.21. SINGLE and DOUBLE

3.3.22. EFLOAT, BFLOAT, and TFLOAT

3.3.23. Qxx and LQxx

3.3.24. DATA

3.3.25. ZERO, CP-1600

3.3.26. FB and FW

3.3.27. ASCII, ASCIC, and ASCIZ

3.3.28. STRING and RSTRING

3.3.29. PACKED

3.3.30. RADIX50

3.3.31. FCC

3.3.32. TEXT

3.3.33. DFS or RMB

3.3.34. BLOCK

3.3.35. SPACE

3.3.36. RES

3.3.37. BSS

3.3.38. DSB and DSW

3.3.39. DS16

3.3.40. ALIGN

3.3.41. LTORG

3.4. Macro Instructions

3.4.1. MACRO

3.4.2. IRP

3.4.3. IRPC

3.4.4. REPT

3.4.5. WHILE

3.4.6. EXITM

3.4.7. SHIFT

3.4.8. MAXNEST

3.4.9. FUNCTION

3.5. Structures

3.5.1. Definition

3.5.2. Usage

3.5.3. Nested Structures

3.5.4. Unions

3.5.5. Nameless Structures

3.5.6. Structures and Sections

3.5.7. Structures and Macros

3.6. Conditional Assembly

3.6.1. IF / ELSEIF / ENDIF

3.6.2. SWITCH / CASE / ELSECASE / ENDCASE

3.7. Listing Control

3.7.1. PAGE, PAGESIZE

3.7.2. NEWPAGE

3.7.3. MACEXP_DFT and MACEXP_OVR

3.7.4. LISTING

3.7.5. PRTINIT and PRTEXIT

3.7.6. TITLE

3.7.7. RADIX

3.7.8. OUTRADIX

3.8. Local Symbols

3.8.1. Basic Definition (SECTION/ENDSECTION)

3.8.2. Nesting and Scope Rules

3.8.3. PUBLIC and GLOBAL

3.8.4. FORWARD

3.8.5. Performance Aspects

3.9. Miscellaneous

3.9.1. SHARED

3.9.2. INCLUDE

3.9.3. BINCLUDE

3.9.4. MESSAGE, WARNING, ERROR, and FATAL

3.9.5. READ

3.9.6. INTSYNTAX

3.9.7. RELAXED

3.9.8. COMPMODE

3.9.9. END

4. Processor-specific Hints

4.1. 6811

4.2. PowerPC

4.3. IBM PALM

4.4. DSP56xxx

4.5. H8/300

4.6. H8/500

4.7. SH7000/7600/7700

4.8. HMCS400

4.9. H16

4.10. OLMS-40

4.11. OLMS-50

4.12. MELPS-4500

4.13. 6502UNDOC

4.14. MELPS-740

4.15. MELPS-7700/65816

4.16. M16

4.17. CP-3F

4.18. 4004/4040

4.19. MCS-48

4.20. MCS-51

4.21. MCS-251

4.22. 8080/8085

4.23. 8085UNDOC

4.24. 8086..V35

4.25. 8X30x

4.26. XA

4.27. AVR

4.28. Z80, Z380

4.29. Z80UNDOC

4.30. GB_Z80 resp. LR35902

4.31. Z380

4.32. Z8, Super8, and eZ8

4.33. Z8000

4.33.1. Conditions

4.33.2. Flags

4.33.3. Indirect Addressing

4.33.4. Direct versus Immediate Addressing

4.34. TLCS-900(L)

4.35. TLCS-90

4.36. TLCS-870

4.37. TLCS-47

4.38. TLCS-9000

4.39. TC9331

4.40. 29xxx

4.41. 80C16x

4.42. PIC16C5x/16C8x

4.43. PIC 17C4x

4.44. SX20/28

4.45. ST6

4.46. ST7

4.47. ST9

4.48. 6804

4.49. TMS3201x

4.50. TMS320C2x

4.51. TMS320C3x/C4x

4.52. TMS9900

4.53. TMS70Cxx

4.54. TMS370xxx

4.55. MSP430(X)

4.56. TMS1000

4.57. COP8

4.58. SC/MP

4.59. SC144xxx

4.60. NS32xxx

4.61. uPD78(C)1x

4.62. 75K0

4.63. 78K0

4.64. 78K2/78K3/78K4

4.65. uPD772x

4.66. uCOM-43

4.67. F2MC16L

4.68. MN161x

4.69. CDP180x

4.70. KENBAK

4.71. HP Nanoprocessor

4.72. IM61x0

5. File Formats

5.1. Code Files

5.2. Debug Files

6. Utility Programs

6.1. PLIST

6.2. BIND

6.3. P2HEX

6.4. P2BIN

6.5. AS2MSG

A. Error Messages of AS

B. I/O Error Messages

C. Programming Examples

C.1. 16 Bit Instructions via Macros

D. Frequently Asked Questions

E. Pseudo-Instructions and Integer Syntax

F. Predefined Symbols

G. Shipped Include Files

G.1. BITFUNCS.INC

G.2. CTYPE.INC

H. Acknowledgments

I. Changes since Version 1.3

J. Hints for the AS Source Code

J.1. Language Preliminaries

J.2. Capsuling System dependencies

J.3. System-Independent Files

J.3.1. Modules Used by AS

J.3.2. Additional Modules for the Tools

J.4. Modules Needed During the Build of AS

J.5. Generation of Message Files

J.5.1. Format of the Source Files

J.6. Creation of Documentation

J.7. Test Suite

J.8. Adding a New Target Processor

J.9. Localization to a New Language

1. Introduction

===============

This  instruction  is  meant  for  programmers  who  are  already  very

familiar  with Assembler and who  like to know how  to work with AS. It

is  rather a reference than a user's  manual and so it neither tries to

explain  the ''language assembler''  nor the processors.  I have listed

further  literature in  the bibliography  which was  substantial in the

implementation  of the  different code  generators. There  is no book I

know  where  you  can  learn  Assembler  from the start, so I generally

learned this by ''trial and error''.

1.1. License Agreement

----------------------

Before  we  can  go  ''in  medias  res'',  first  of all the inevitable

prologue:

As  in the  present version  is licensed  according to  the Gnu General

Public  License (GPL); the details  of the GPL may  be read in the file

COPYING  bundled with  this distribution.  If you  did not  get it with

AS, complain to the one you got AS from!

Shortly said, the GPL covers the following points:

- Programs  based upon AS must also be licensed according to the GPL;

- distribution is explicitly allowed;

- explicit  disclaiming of all warranties  for damages resulting from

usage of this program.

...however,  I  really  urge  you  to  read  the  file  COPYING for the

details!

To  accelerate  the  error  diagnose  and  correction,  please  add the

following details to the bug report:

- Operating system (DOS, Windows, Linux) and its version

- Version of AS used, resp. dates of the EXE-files

- If  you compiled the assembler yourself,  the compiler used and its

version

- If possible, the source file that triggered the bug

You can contact me as follows:

- by Surface Mail:

Alfred Arnold

Hirschgraben 29

D-52062 Aachen

Germany

- by E-Mail: alfred@ccac.rwth-aachen.de

If  someone  likes  to  meet  me  personally to ask questions and lives

near  Aachen (=  Aix-la-Chapelle), you  will be  able to meet me there.

You  can do  this most  probably on  thursdays from  8pm to  9pm at the

RWTH  Aachen Computer Club (Elisabethstrasse  16, first floor, corridor

on the right).

Please  don't call me by phone.  First, complex relations are extremely

hard  to  discuss  at  phone.  Secondly,  the  telephone  companies are

already rich enough...

The  latest  version  of  AS  (DPMI,  Win32,  C)  is available from the

following Server:

http://john.ccac.rwth-aachen.de:8000/as

or shortly

http://www.alfsembler.de

Whoever  has  no  access  to  an  FTP-Server  can  ask  me  to send the

assembler  by mail. Only  requests containing a  blank CD-R and a self-

addressed,  (correctly) stamped  envelope will  be answered. Don't send

any money!

Now,  after  this  inevitable  introduction  we  can turn to the actual

documentation:

1.2. General Capabilities of the Assembler

------------------------------------------

In  contrast  to  ordinary  assemblers,  AS  offers  the possibility to

generate  code  for  totally  different  processors. At the moment, the

following processor families have been implemented:

- Motorola  68000..68040,, 683xx, and  Coldfire incl. coprocessor and

MMU

- Motorola ColdFire

- Motorola DSP5600x,DSP56300

- Motorola M-Core

- Motorola/IBM MPC601/MPC505/PPC403/MPC821

- IBM PALM

- Motorola 6800, 6801, 68(HC)11(K4) and Hitachi 6301

- Motorola/Freescale 6805, 68HC(S)08

- Motorola 6809 / Hitachi 6309

- Motorola/Freescale 68HC12(X) including XGATE

- Freescale/NXP S12Z (''MagniV'')

- Motorola 68HC16

- Freescale 68RS08

- Konami 052001

- Hitachi H8/300(H)

- Hitachi H8/500

- Hitachi SH7000/7600/7700

- Hitachi HMCS400

- Hitachi H16

- Rockwell  6502, 65(S)C02,  Commodore 65CE02,  WDC W65C02S, Rockwell

65C19, and Hudson HuC6280

- Rockwell PPS-4

- CMD 65816

- Mitsubishi MELPS-740

- Mitsubishi MELPS-7700

- Mitsubishi MELPS-4500

- Mitsubishi M16

- Mitsubishi M16C

- DEC PDP-11

- DEC VAX

- Western Digital WD16

- Intel 4004/4040

- Intel   MCS-48/41,  including   Siemens  SAB80C382,   and  the  OKI

variants

- Intel MCS-51/251, Dallas DS80C390

- Intel MCS-96/196(Nx)/296

- Intel 8080/8085

- Intel i960

- Signetics 8X30x

- Signetics 2650

- Philips XA

- Atmel (Mega-)AVR

- AMD 29K

- Siemens 80C166/167

- Zilog Z80 (including undocumented instructions), Z180, Z380, eZ80

- Sharp LR35902 (,,Gameboy Z80'')

- Sharp SC61860

- Sharp SC62015

- Zilog Z8, Super8, Z8 Encore

- Zilog Z8000

- Xilinx KCPSM/KCPSM3 ('PicoBlaze')

- LatticeMico8

- Toshiba TLCS-900(L)

- Toshiba TLCS-90

- Toshiba TLCS-870(/C)

- Toshiba TLCS-47

- Toshiba TLCS-9000

- Toshiba TC9331

- Microchip PIC16C54..16C57

- Microchip PIC16C84/PIC16C64

- Microchip PIC17C42

- Parallax SX20/28

- SGS M380/GI LP8000

- SGS-Thomson ST6

- SGS-Thomson ST7/STM8

- SGS-Thomson ST9

- SGS-Thomson 6804

- Texas Instruments TMS32010/32015

- Texas Instruments TMS3202x

- Texas Instruments TMS320C3x/TMS320C4x

- Texas Instruments TMS320C20x/TMS320C5x

- Texas Instruments TMS320C54x

- Texas Instruments TMS320C6x

- Texas Instruments TMS99xx/99xxx

- Texas Instruments TMS7000

- Texas Instruments TMS1000

- Texas Instruments TMS370xxx

- Texas Instruments MSP430(X)

- National Semiconductor IMP-16

- National Semiconductor IPC-16 ('PACE'), INS8900

- National Semiconductor SC/MP

- National Semiconductor INS807x

- National Semiconductor COP4

- National Semiconductor COP8

- National Semiconductor SC144xx

- National Semiconductor NS32xxx

- Olympia CP-3F (resp. SGS M380, GI LP8000)

- Fairchild ACE

- Fairchild F8

- NEC �PD78(C)0x/�PD 78(C)1x

- NEC �COM-43/44/45

- NEC �PD75xx

- NEC �PD 75xxx (alias 75K0)

- NEC 78K0

- NEC 78K2

- NEC 78K3

- NEC 78K4

- NEC �PD7720/7725

- NEC �PD77230

- NEC V60

- Fujitsu F�MC8L

- Fujitsu F�MC16L

- OKI OLMS-40

- OKI OLMS-50

- Panafacom MN1610/MN1613

- Renesas RX

- Padauk PMS/PMC/PFSxxx

- Symbios Logic SYM53C8xx (yes, they are programmable!)

- Intersil CDP1802/1804/1805(A)

- Intersil IM6100/6120

- XMOS XS1

- MIL STD 1750

- KENBAK-1

- GI CP-1600

- HP Nano Processor

under work / planned / in consideration :

- ARM

- Analog Devices ADSP21xx

- DEC VAX

- SGS-Thomson ST20

- Texas Instruments TMS320C8x

- Zilog eZ80

Unloved, but now, however, present :

- Intel 80x86, 80186, Nec V20..V55 incl. coprocessor 8087

The  switch to  a different  code generator  is allowed even within one

file, and as often as one wants!

The  reason for this  flexibility is that  AS has a  history, which may

also  be recognized  by looking  at the  version number. AS was created

as  an extension of a macro assembler  for the 68000 family. On special

request,  I  extended  the  original  assembler  so that it was able to

translate  8051 mnemonics. On  this way (decline  ?!) from the 68000 to

8051,  some other  processors were  created as  by-products. All others

were  added  over  time  due  to  user  requests.  So  At least for the

processor-independent  core  of  AS,  one  may  assume that it is well-

tested  and free  of obvious  bugs. However,  I often  do not  have the

chance  to  test  a  new  code  generator  in  practice (due to lack of

appropriate  hardware), so  surprises are  not impossible  when working

with  new features. You  see, the things  stated in section  1.1 have a

reason...

This  flexibility implies  a somewhat  exotic code  format, therefore I

added  some tools to  work with it.  Their description can  be found in

chapter 6.

AS  is  a  macro  assembler,  which  means  that the programmer has the

possibility   to   define   new   ''commands''   by  means  of  macros.

Additionally  it masters  conditional assembling.  Labels inside macros

are automatically processed as being local.

For   the  assembler,  symbols  may  have  either  integer,  string  or

floating  point values. These  will be stored  - like interim values in

formulas  - with a width  of 32 bits for  integer values, 80 or 64 bits

for  floating  point  values,  and  255  characters  for strings. For a

couple  of  micro  controllers,  there  is  the possibility to classify

symbols  by segmentation. So the  assembler has a (limited) possibility

to recognize accesses to wrong address spaces.

The  assembler does  not know  explicit limits  in the nesting depth of

include  files or macros;  a limit is  only given by  the program stack

restricting  the recursion depth.  Nor is there  a limit for the symbol

length, which is only restricted by the maximum line length.

From  version 1.38 on,  AS is a  multipass-assembler. This pompous term

means  no more  than the  fact that  the number  of passes  through the

source  code  need  not  be  exactly  two.  If the source code does not

contain  any forward  references, AS  needs only  one pass.  In case AS

recognizes  in the  second pass  that it  must use  a shorter or longer

instruction  coding,  it  needs  a  third  (fourth,  fifth...)  pass to

process  all symbol references correctly.  There is nothing more behind

the  term ''multipass'', so  it will not  be used further  more in this

documentation.

After  so much praise a bitter  pill: AS cannot generate linkable code.

An  extension  with  a  linker  needs  considerable  effort  and is not

planned at the moment.

Those  who want to take a look at  the sources of AS can simply get the

Unix  version  of  AS,  which  comes  as source for self-compiling. The

sources  are  definitely  not  in  a  format  that  is targeted at easy

understanding  - the original  Pascal version still  raises its head at

a  couple of  places, and  I do  not share  a couple of common opinions

about 'good' C coding.

1.3. Supported Platforms

------------------------

Though  AS  started  as  a  pure  DOS  program,  there  are a couple of   DOS

versions  available that are able  to exploit a bit  more than the Real

Mode  of an  Intel CPU.  Their usage  is kept  as compatible to the DOS

version  as possible,  but there  are of  course differences concerning

installation  and  embedding  into  the  operating  system in question.

Sections  in this manual that are only  valid for a specific version of

AS  are marked  with a  corresponding sidemark  (at this  paragraph for

the  DOS version)  aheaded to  the paragraph.  In detail, the following

further versions exist (distributed as separate packages):

In  case you runinto memory problems  when assembling large and complex   DPMI

programs  under  DOS,  there  is  a  DOS version that runs in protected

mode  via  a  DOS  extender  and  can  therefore  make use of the whole

extended  memory of  an AT.  The assembly  becomes significantly slower

by the extender, but at least it works...

There  is  a  native  OS/2  version  of  AS  for  friends of IBM's OS/2   OS/2

operating  system. Since  version 1.41r8,  this is  a full  32-bit OS/2

application,  which  of  course  means  that  OS/2  2.x and at least an

80386 CPU are mandatory.

You  can leave the area  of PCs-only with the  C version of AS that was   UNIX

designed  to  be  compilable  on  a  large number of UNIX systems (this

includes  OS/2 with the emx compiler)  without too much of tweaking. In

contrast  to  the  previously  mentioned  versions,  the  C  version is

delivered  in  source  code,  i.e.  one  has  to create the binaries by

oneself  using a C  compiler. This is  by far the  simpler way (for me)

than   providing  a  dozen  of  precompiled  binaries  for  machines  I

sometimes only have limited access to...

2. Assembler Usage

==================

Scotty: Captain, we din' can reference it!

Kirk: Analysis, Mr. Spock?

Spock: Captain, it doesn't appear in the symbol table.

Kirk: Then it's of external origin?

Spock: Affirmative.

Kirk: Mr. Sulu, go to pass two.

Sulu: Aye aye, sir, going to pass two.

2.1. Hardware Requirements

--------------------------

The  hardware  requirements  of  AS  vary substantially from version to

version:

The  DOS  version  will  principally  run  on  any  IBM-compatible  PC,   DOS

ranging  from  a  PC/XT  with  4-dot-little  megahertz up to a Pentium.

However,  similar to  other programs,  the fun  using AS  increases the

better  your  hardware  is.  An  XT  user  without  a  hard  drive will

probably  have  significant  trouble  placing  the  overlay  file  on a

floppy  because it is larger than  500 Kbytes...the PC should therefore

have  at least a  hard drive, allowing  acceptable loading times. AS is

not  very  advanced  in  its  main  memory  needs:  the  program itself

allocates  less than 300  Kbytes main memory,  AS should therefore work

on machines with at least 512 Kbytes of memory.

The  version  of  AS  compiled  for  the  DOS  Protected Mode Interface   DPMI

(DPMI)  requires  at  least  1  Mbyte  of free extended memory. A total

memory  capacity  of  at  least  2  Mbytes  is  therefore  the absolute

minimum  given one  does not  have other  tools in  the XMS  (like disk

caches,  RAM  disks,  or  a  hi-loaded  DOS);  the needs will rise then

appropriately.  If one uses the DPMI version  in a DOS box of OS/2, one

has  to assure that  DPMI has been  enabled via the  box's DOS settings

(set  to on  or auto)  and that  a sufficient  amount of XMS memory has

been  assigned to the  box. The virtual  memory management of OS/2 will

free you from thinking about the amount of free real memory.

The  C  version  of  AS  is  delivered  as  source  code  and therefore   UNIX

requires  a  UNIX  or  OS/2  system  equipped  with  a  C compiler. The

compiler  has to fulfill the ANSI  standard (GNU-C for example is ANSI-

compliant).  You  can  look  up  in  the  README file whether your UNIX

system  has already been tested so  that the necessary definitions have

been  made. You should reserve about 15  Mbytes of free hard disk space

for  compilation; this value  (and the amount  needed after compilation

to  store  the  compiled  programs)  strongly  differs  from  system to

system, so you should take this value only as a rough approximation.

2.2. Delivery

-------------

Principally,  you can obtain AS  in one of two  forms: as a binary or a

source  distribution. In  case of  a binary  distribution, one gets AS,

the  accomanying  tools  and  auxiliary  files readily compiled, so you

can  immediately start  to use  it after  unpacking the  archive to the

desired  destination on your  hard drive. Binary  distibutions are made

for  widespread platforms, where either the  majority of users does not

have   a  compiler  or  the  compilation  is  tricky  (currently,  this

includes  DOS  and  OS/2).  A  source distribution in contrast contains

the  complete  set  of  C  sources  to  generate AS; it is ultimately a

snapshot  of  the  source  tree  I  use  for  development  on  AS.  The

generation  of AS from the sources  and their structure is described in

detail  in appendix J,  which is why  at this place,  only the contents

and installation of a binary distribution will be described:

The  contents of the archive  is separated into several subdirectories,

therefore  you  get  a  directory  subtree  immediately after unpacking

without   having   to   sort   out   things  manually.  The  individual

directories contain the following groups of files:

-  BIN: executable programs, text resources;

-  INCLUDE:  include  files  for  assembler  programs,  e.g. register

definitions or standard macros;

-  MAN:  quick references for  the individual programs  in Unix 'man'

format.

A  list of  the files  found in  every binary  distribution is given in

table  2.0. In case  a file listed  in one of  these (or the following)

tables is missing, someone took a nap during copying (probably me)...

Table 2.1: Standard Contents of a Binary Distribution

+-------------------+---------------------------------------------+

| File              | Function                                    |

+-------------------+---------------------------------------------+

+-------------------+---------------------------------------------+

| Directory BIN     |                                             |

+-------------------+---------------------------------------------+

| AS.EXE            | executable of assembler                     |

| PLIST.EXE         | lists contents of code files                |

| PBIND.EXE         | merges code files                           |

| P2HEX.EXE         | converts code files to hex files            |

| P2BIN.EXE         | converts code files to binary files         |

| AS.MSG            | text resources for AS (DOS only)            |

| PLIST.MSG         | text resources for PLIST *)                 |

| PBIND.MSG         | text resources for PBIND *)                 |

| P2HEX.MSG         | text resources for P2HEX *)                 |

| P2BIN.MSG         | text resources for P2BIN *)                 |

| TOOLS.MSG         | common text resources for all tools *)      |

| CMDARG.MSG        | common text resources for all programs *)   |

| IOERRS.MSG        |                                             |

+-------------------+---------------------------------------------+

| *) DOS only                                                     |

+-------------------+---------------------------------------------+

| Directory DOC     |                                             |

+-------------------+---------------------------------------------+

| AS_DE.DOC         | german documentation, ASCII format          |

| AS_DE.HTML        | german documentation, HTML format           |

| AS_DE.TEX         | german documentation, LaTeX format          |

| AS_EN.DOC         | english documentation, ASCII format         |

| AS_EN.HTML        | english documentation, HTML format          |

| AS_EN.TEX         | english documentation, LaTeX format         |

+-------------------+---------------------------------------------+

| Directory INCLUDE |                                             |

+-------------------+---------------------------------------------+

| BCDIC.INC         | definition of BCDIC/code page 359           |

| BITFUNCS.INC      | functions for bit manipulation              |

| CTYPE.INC         | functions for classification of             |

|                   | characters                                  |

| EBCDIC.INC        | includes all EBCDIC variants                |

| CP037.INC         | definition EBCDIC (code page 037)           |

| CP5100.INC        | definition character set IBM 5100           |

| CP5110.INC        | definition EBCDIC (IBM 5110)                |

| 80C50X.INC        | register addresses SAB C50x                 |

| 80C552.INC        | register addresses 80C552                   |

| H8_3048.INC       | register addresses H8/3048                  |

| KENBAK.INC        | register addressed Kenbak-1                 |

| RADIX50.INC       | definition of RADIX 50 character set        |

| REG166.INC        | addresses and instruction macros 80C166/167 |

| REG251.INC        | addresses and bits 80C251                   |

| REG29K.INC        | peripheral addresses AMD 2924x              |

| REG53X.INC        | register addresses H8/53x                   |

| REG6303.INC       | register addresses 6303                     |

| REG683XX.INC      | register addresses 68332/68340/68360        |

| REG7000.INC       | register addresses TMS70Cxx                 |

| REG78310.INC      | register addresses & vectors 78K3           |

| REG78K0.INC       | register addresses 78K0                     |

| REG96.INC         | register addresses MCS-96                   |

| REGACE.INC        | register addresses ACE                      |

| REGEZ80.INC       | register addresses eZ80                     |

| REGF8.INC         | register and memory addresses F8            |

| REGAVROLD.INC     | register and bit addresses AVR family (old) |

| REGAVR.INC        | register and bit addresses AVR family       |

| REGCOLD.INC       | register and bit addresses Coldfire family  |

| REGCOP8.INC       | register addresses COP8                     |

| REGGP32.INC       | register addresses 68HC908GP32              |

| REGH16.INC        | register addresses H16                      |

| REGHC12.INC       | register addresses 68HC12                   |

| REGM16C.INC       | register addresses Mitsubishi M16C          |

| REGMSP.INC        | register addresses TI MSP430                |

| REGPDK.INC        | register and bit addresses PMC/PMS/PFSxxx   |

| REGS12Z.INC       | register and bit addresses S12Z family      |

| REGST6.INC        | register and macro definitions ST6          |

| REGST7.INC        | register and macro definitions ST7          |

| REGSTM8.INC       | register and macro definitions STM8         |

| REGST9.INC        | register and macro definitions ST9          |

| REGV60.INC        | register addresses NEC V60                  |

| REGZ380.INC       | register addresses Z380                     |

| STDDEF04.INC      | register addresses 6804                     |

| STDDEF16.INC      | instruction macros and register addresses   |

|                   | PIC16C5x                                    |

| STDDEF17.INC      | register addresses PIC17C4x                 |

| STDDEF18.INC      | register addresses PIC16C8x                 |

| STDDEF2X.INC      | register addresses TMS3202x                 |

| STDDEF37.INC      | register and bit addresses TMS370xxx        |

| STDDEF3X.INC      | peripheral addresses TMS320C3x              |

| STDDEF4X.INC      | peripheral addresses TMS320C4x              |

| STDDEF47.INC      | instruction macros TLCS-47                  |

| STDDEF51.INC      | definition of SFRs and bits for             |

|                   | 8051/8052/80515                             |

| STDDEF56K.INC     | register addresses DSP56000                 |

| STDDEF5X.INC      | peripheral addresses TMS320C5x              |

| STDDEF60.INC      | instruction macros and register addresses   |

|                   | PowerPC                                     |

| REGSX20.INC       | register and bit addresses Parallax SX20/28 |

| AVR/*.INC         | register and bit addresses AVR family       |

|                   | (do not include directly, use REGAVR.INC)   |

| COLDFIRE/*.INC    | register and bit addresses ColdFire family  |

|                   | (do not include directly, use REGCOLD.INC)  |

| EZ80/*.INC        | register and bit addresses eZ80 family      |

|                   | (do not include directly, use REGCOLD.INC)  |

| PDK/*.INC         | register and bit addresses PMC/PMS/PFSxxx   |

|                   | (do not include directly, use REGPDK.INC)   |

| S12Z/*.INC        | register and bit addresses S12Z family      |

|                   | (do not include directly, use REGS12Z.INC)  |

| ST6/*.INC         | register and bit addresses ST6 family       |

|                   | (do not include directly, use REGST6.INC)   |

| ST7/*.INC         | register and bit addresses ST7 family       |

|                   | (do not include directly, use REGST7.INC)   |

| STM8/*.INC        | register and bit addresses STM8 family      |

|                   | (do not include directly, use REGSTM8.INC)  |

| STDDEF62.INC      | register addresses and macros ST6 (old)     |

| STDDEF75.INC      | register addresses 75K0                     |

| STDDEF87.INC      | register and memory addresses TLCS-870      |

| STDDEF90.INC      | register and memory addresses TLCS-90       |

| STDDEF96.INC      | register and memory addresses TLCS-900      |

| STDDEFXA.INC      | SFR and bit addresses Philips XA            |

| STDDEFZ8.INC      | register addresses Z8 family (old)          |

| REGZ8.INC         | register addresses Z8 family (new)          |

| Z8/*.INC          | register and bit addresses Z8 family        |

|                   | (do not include directly, use REGZ8.INC)    |

+-------------------+---------------------------------------------+

| Directory LIB     |                                             |

+-------------------+---------------------------------------------+

| Directory MAN     |                                             |

+-------------------+---------------------------------------------+

| ASL.1             | Short Reference for AS                      |

| PLIST.1           | Short Reference for PLIST                   |

| PBIND.1           | Short Reference for PBIND                   |

| P2HEX.1           | Short Reference for P2HEX                   |

| P2BIN.1           | Short Reference for P2BIN                   |

+-------------------+---------------------------------------------+

Depending  on the platform,  a binary distribution  however may contain

more   files  to  allow   operation,  like  files   necessary  for  DOS

extenders.  In case of  the DOS DPMI  version, the extensions listed in   DPMI

table  2.1 result. Just to mention it:  it is perfectly O.K. to replace

the  tools with their  counterparts from a  DOS binary distribution; on

the  on hand, they execute  significantly faster without the extender's

overhead,  and on the other hand, they  do not need the extended memory

provided by the extender.

+---------------+--------------------------------+

| File          | Function                       |

+---------------+--------------------------------+

+---------------+--------------------------------+

| Directory MAN |                                |

+---------------+--------------------------------+

| ASL.1         | quick reference for AS         |

| PLIST.1       | quick reference for PLIST      |

| PBIND.1       | quick reference for PBIND      |

| P2HEX.1       | quick reference for P2HEX      |

| P2BIN.1       | quick reference for P2BIN      |

+---------------+--------------------------------+

+---------------+--------------------------------+

| Directory BIN |                                |

+---------------+--------------------------------+

| DPMI16BI.OVL  | DPMI server for the assembler  |

| RTM.EXE       | runtime module of the extender |

+---------------+--------------------------------+

Table 2.1: Additional Files in a DPMI Binary Distribution

An  OS/2 binary distribution  contains in addition  to the base files a   OS/2

set  of DLLs belonging  to the runtime  environment of the emx compiler

used  to build AS (table 2.2). In  case you already have these DLLs (or

newer  versions  of  them),  you  may  delete  these  and use your ones

insted.

+---------------+------------------------------+

| File          | function                     |

+---------------+------------------------------+

+---------------+------------------------------+

| Directory BIN |                              |

+---------------+------------------------------+

| EMX.DLL       | runtime libraries for AS and |

| EMXIO.DLL     | its tools                    |

| EMXLIBC.DLL   |                              |

| EMXWRAP.DLL   |                              |

+---------------+------------------------------+

Table 2.2: Additional Files in an OS/2 binary distribution

2.3. Installation

-----------------

There  is no need for  a special installation prior  to usage of AS. It   DOS

is  sufficient to unpack  the archive in  a fitting place  and to add a

few  minor settings. For  example, this is  an installation a user used

to UNIX-like operating systems might choose:

Create  a directory c:\as an  (I will assume in  the following that you

are  going to  install AS  on drive  C), change  to this  directory and

unpack  the archiv, keeping the path  names stored in the archive (when

using  PKUNZIP, the command line option  -d is necessary for that). You

now should have the following directory tree:

c:\as

c:\as\bin

c:\as\include

c:\as\lib

c:\as\man

c:\as\doc

c:\as\demos

Now,  append  the  directory  c:\as\bin  to  the PATH statement in your

AUTOEXEC.BAT,  which allows the  system to find  AS and its tools. With

your  favourite  text  editor,  create  a  file  named AS.RC in the lib

directory with the following contents:

-i c:\as\include

This  so-called  key  file  tells  AS  where  to search for its include

files.  The following statement  must be added  to your AUTOEXEC.BAT to

tell AS to read this file:

set ASCMD=@c:\as\lib\as.rc

There  are many more things  you can preset via  the key file; they are

listed in the following section.

The  installation of the DPMI version  should principally take the same   DPMI

course  as for the pure  DOS version; as soon  as the PATH contains the

bin  directory, the  DOS extender's  files will  be found automatically

and  you should not  notice anything of  this mechanism (except for the

longer  startup time...). When  working on an  80286-based computer, it

is  theoretically possible  tha you  get confronted  with the following

message upon the first start:

machine not in database (run DPMIINST)

Since  the DPMIINST  tool ins  not any  more included in newer versions

of  Borland's DOS  extender, I  suppose that  this is  not an  item any

more...in case you run into this, contact me!

The  installation of the  OS/2 version can  generally be done just like   OS/2

for  the DOS version, with  the addition that the  DLLs have to be made

visible  for the operating  system. In case  you do not  want to extend

the  LIBPATH entry in  your CONFIG.SYS, it  is of course  also valid to

move the DLLs into a directory already listed in LIBPATH.

As  already mentioned,  the installation  instructions in  this section

limit  themselves to binary distributions.  Since an installation under

Unix  is currently alway  a source-based installation,  the only hint I   UNIX

can give here is a reference to appendix J.

2.4. Start-Up Command, Parameters

---------------------------------

AS  is  a  command  line  driven  program, i.e. all parameters and file

options are to be given in the command line.

A  couple of message files belongs  to AS (recognizable by their suffix

MSG)  AS accesses to dynamically load  the messages appropriate for the

national  language.  AS  searches  the  following directories for these

files:

- the current directory;

- the EXE-file's directory;

- the  directory  named  in  the  AS_MSGPATH environment variable, or

alternitavely  the  directories  listed  in  the  PATH  environment

variable;

- the directory compiled into AS via the LIBDIR macro.

These  files are  indispensable for  a proper  operation of AS, i.e. AS

will terminate immediately if these files are not found.

The  language selection  (currently only  German and  English) is based

on  the COUNTRY  setting under  DOS and  OS/2 respectively  on the LANG

environment variable under Unix.

In  order to  fulfill AS's  memory requirements  under DOS, the various   DOS

code  generator modules of  the DOS version  were moved into an overlay

which  is part  of the  EXE file.  A separate  OVR file like in earlier

versions  of  AS  therefore  dose  not  exist any more, AS will however

still  attempt  to  reduce  the  overlaying  delays by using eventually

available  EMS or XMS memory. In case  this results in trouble, you may

suppress  usage  of  EMS  or  XMS  by  setting the environment variable

USEXMS  or USEEMS to n.  E.g., it is possible  to suppress the using of

XMS by the command:

SET USEXMS=n

Since  AS performs  all in-  and output  via the  operating system (and

therefore  it  should  run  also  on  not 100% compatible DOS-PC's) and

needs  some  basic  display  control,  it  emits ANSI control sequences

during  the assembly. In case you  should see strange characters in the   DOS/

messages  displayed by AS, your CONFIG.SYS  is obviously lacking a line

like this:

device=ansi.sys

but  the  further  functions  of  AS  will  not  be  influenced hereby.   DPMI

Alternatively  you are  able to  suppress the  output of ANSI sequences

completely by setting the environment variable USEANSI to n.

The  DOS extender of the  DPMI version can be  influenced in its memory   DPMI

allocation  strategies  by  a  couple  of environment variables; if you

need  to  know  their  settings,  you  may  look  up  them  in the file

DPMIUSER.DOC.  It is additionally  able to extend  the available memory

by  a swap file. To do this,  set up an environment variable ASXSWAP in

the following way:

SET ASXSWAP=<size>[,file name]

The  size  specification  has  to  be  done  in megabytes and has to be

done.  The file  name in  contrast is  optional; if  it is missing, the

file  is  named  ASX.TMP  and  placed  in the current directory. In any

case, the swap file is deleted after program end.

The   command  line  parameters  can  roughly  be  divided  into  three

categories:   switches,  key  file  references  (see  below)  and  file

specifications.  Parameters of these two  categories may be arbitrarily

mixed  in  the  command  line.  The  assembler  evaluates  at first all

parameters  and then  assembles the  specified files.  From this follow

two things:

- the  specified  switches  affect  all  specified  source  files. If

several  source files  shall be  assembled with different switches,

this has to be done in separate runs.

- it  is possible to assemble  more than one file  in one shot and to

bring  it to  the top,  it is  allowed that  the file specs contain

wildcards.

Parameter  switches are recognized  by AS by  starting with a slash (/)

or  hyphen (-).  There are  switches that  are only  one character long

and  additionally  switches  composed  of  a  whole  word.  Whenever AS

cannot  interpret  a  switch  as  a  whole word, it tries to interprete

every letter as an individual switch. For example, if you write

-queit

instead of

-quiet

AS  will take  the letters  q, u,  e, i,  and t as individual switches.

Multiple-letter  switches additionally  have the  difference to single-

letter  switches that AS will accept  an arbitrary mixture of upper and

lower  casing,  whereas  single-letter  switches  may  have a different

meaning depending on whether upper or lower case is used.

At the moment, the following switches are defined:

- l:  sends assembler  listing to  console terminal  (mostly screen).

In  case several passes have to be  done, the listing of all passes

will be send to the console (in opposite to the next option).

- L:  writes assembler  listing into  a file.  The list file will get

the  same name as  the source file,  only the extension is replaced

by LST. Except one uses...

- OLIST:  with  a  fiel  name  as  argument  allows  to  redirect the

listing  to a different  file or a  different path. This option may

be  used multiple times  in case multiple  files are assembled with

one execution.

- listline-prefix:  This  option  allows  to  modify the data that is

printed  in  front  of  every  source  line  in  the  listing.  The

structure of the format string is described in 2.6.

- LISTRADIX:   By  default,   all  numeric   output  in  the  listing

(addresses,   generated   code,   symbol   values)  is  written  in

hexadecimal  notation. This  switch requests  usage of  a different

number  system in the  range of 2  to 36. For instance, '-listradix

8'  requests octal  output. If  the radix  value is  written with a

leading  zero  (e.g.  08  instead  of  8),  the  program  counter's

current value is prited with leading zeros in the listing.

- SPLITBYTE  [character]:  Display  numbers  in  the  listing in byte

groups,  separated  by  the  given  character.  A period is used as

separator  if  no  explicit  character  is  given.  This  option is

usually   used  in  conjunction  with  the  LISTRADIX  option.  For

instance,  list radix 8  with a period  as character results in the

so-called 'split octal' notation.

- o:  Sets the  new name  of the  code file  generated by AS. If this

option  is used  multiple times,  the names  will be  assigned, one

after  the other, to  the source files  which have to be assembled.

A  negation (see  below) of  this option  in connection with a name

erases  this name from  the list. A  negation without a name erases

the whole list.

- SHAREOUT:ditto for a SHARE file eventually to be created.

- c:  SHARED-variables will be  written in a  format which permits an

easy  integration into a  C-source file. The  extension of the file

is H.

- p:  SHARED-variables  will  be  written  in  a format which permits

easy  integration  into  the  CONST-block  of a Pascal program. The

extension of the file is INC.

- a:  SHARED-variables  will  be  written  in  a format which permits

easy  integration into an  assembler source file.  The extension of

the file is INC.

Concerning  effect  and  function  of  the  SHARED-symbols  please  see

chapters 2.13 resp. 3.9.1.

- g  [format]: This switch instructs AS  to create an additional file

that  contains debug  information for  the program. Allowed formats

are  the AS-specific  MAP format  ( format=MAP), a NoICE-compatible

command  file (  format=NOICE), and  the Atmel  format used  by the

AVR  tools  (  format=ATMEL).  The  information  stored  in the MAP

format  is comprised of  a symbol table  and a table describing the

assignment  of source lines  to machine addresses.  A more detailed

description  of  the  MAP  format  can  be found in section 5.2 The

file's  extension is MAP,  NOI, resp. OBJ,  depending on the chosen

format.  If  no  explicit  format  specification  is  done, the MAP

format is chosen.

- noicemask  [value]:  By  default,  AS  lists  only symbols from the

CODE  segment in  NoICE debug  info files.  With this option and an

integer  value  interpreted  as  a  bit  mask,  symbols  fom  other

segments   may  be  added.  The   assignment  of  segments  to  bit

positions may be taken from table 5.1.

- w: suppress issue of warnings;

- E  [file]:  error  messages  and  warnings  produced  by AS will be

redirected  to a  file. Instead  of a  file, the 5 standard handles

(STDIN..STDPRN)  can also  be specified  as !0  to !4  . Default is

!2,  meaning STDERR. If  the file option  is left out,  the name of

the  error file  is the  same as  of the  source file, but with the

extension LOG.

- q:  This switch suppresses  all messages of  AS, the exceptions are

error  messages and  outputs which  are are  forced from the source

file.  The time needed for assembly  is slightly reduced hereby and

if  you call AS from a shell  there is no redirection required. The

disadvantage  is  that  you  may  ''stay  in the dark'' for several

minutes ... It is valid to write quiet instead of q.

- v:  This  is  verbose,  i.e.  the  opposite of quiet operation. The

only  additional  information  that  is  currently  printed  is the

version info.

- version: Prints version information and exits.

- h:  write  hexadecimal  numbers  in  lowercase  instead  of capital

letters. This option is primarily a question of personal taste.

- i  <path list>:  issues a  list of  directories where the assembler

shall  automatically search  for include  files, in  case it didn't

find  a file  in the  current directory.  The different directories

have to be separated by semicolons.

- u:  calculate a list  of areas which  are occupied in the segments.

This  option is effective only in  case a listing is produced. This

option   requires  considerable  additional  memory  and  computing

performance. In normal operation it should be switched off.

- C:  generates a list  of cross references.  It lists which (global)

symbols  are  used  in  files  and  lines.  This  list will also be

generated   only  in  case  a  listing  is  produced.  This  option

occupies, too, additional memory capacity during assembly.

- s:  issues a  list of  all sections  (see chapter 3.8). The nesting

is indicated by indentations (Pascal like).

- t:  by  means  of  this  switch  it  is possible to separate single

components   of   the   standard   issued   assembler-listing.  The

assignment  of  bits  to  parts  can  be found in the next section,

where the exact format of the assembly listing is explained.

- D:  defines symbols.  The symbols  which are  specified behind this

option  and separated  by commas  are written  to the global symbol

table  before starting the  assembly. As default  these symbols are

written  as integer  numbers with  the value  TRUE, by  means of an

appended  equal  sign,  however,  you  can select other values. The

expression  following  the  equals  sign  may  include operators or

internal  functions,  but  not  any  further symbols, even if these

should  have been  defined before  in the  list! Together  with the

commands  for  conditional  assembly  (see  there)  you may produce

different  program versions out of one  source file by command line

inputs.  CAUTION! If the  case-sensitive mode is  used, this has to

be  specified in  the command  line before  any symbol definitions,

otherwise  symbol names  will be  converted to  upper case  at this

place!

- A:  stores  the  list  of  global  symbols in another, more compact

form.  Use  this  option  if  the  assembler  crashes  with a stack

overflow  because of too long  symbol tables. Sometimes this option

can  increase  the  processing  speed  of  the  assembler, but this

depends on the sources.

- x:  Sets  the  level  of  detail  for  error messages. The level is

increased  resp. decreased by  one each time  this option is given.

While  on  level  0  (default)  only  the  error  message itself is

printed,  an extended  message is  added beginning  at level 1 that

should  simplify the identification of  the error's cause. Appendix

A  lists  which  error  messages  carry which extended messages. At

level  2  (maximum),  the  source  line  containing  the  error  is

additionally printed.

- n:  If  this  option  is  set,  the  error  messages will be issued

additionally  with  their  error  number  (see appendix A). This is

primarily  intended  for  use  with  shells  or  IDE's  to make the

identification of errors easier by those numbers.

- U:  This option switches AS to  the case-sensitive mode, i.e. upper

and   lower  case  in  the  names  of  symbols,  sections,  macros,

character  sets, and user-defined  functions will be distinguished.

This is not the case by default.

- P:  Instructs  AS  to  write  the  source  text  processed by macro

processor  and conditional  assembly into  a file. Additional blank

and  pure comment lines are missing  in this file. The extension of

this file is I.

- M:  If this  switch is  given, AS  generates a  file, that contains

definitions  of macros defined in the  source file that did not use

the  NOEXPORT  option.  This  new  file  has  the  same name as the

source file, only the extension is modified into MAC.

- G:  this switch defines  whether AS should  produce code or not. If

switched  off,  the  processing  will  be  stopped  after the macro

processor.   This  switch  is   activated  by  default  (logically,

otherwise  you would not get a code  file). This switch can be used

in  conjunction with the  P switch, if  only the macro processor of

AS shall be used.

- r  [n]: issue  warnings if  situations occur  that force  a further

pass.  This  information  can  be  used  to  reduce  the  number of

passes.  You may  optionally specify  the number  of the first pass

where   issuing  of   such  messages   shall  start.  Without  this

argument,  warnings  will  come  starting  with  the first pass. Be

prepared for a bunch of messages!!

- bigendian:  This switch sets  big endian mode  for values placed in

memory  right  from  the  program's  beginning,  given  the  target

architecture  supports  the  pseudo  instruction  of same name (see

3.2.15).

- plainbase:   This  switch  enables  omission   of  an  empty  index

argument right from the program's beginning (see 3.2.14).

- underscrore-macroargs:  This  switch  enables  usage  of underscore

characters in macro argument names (see 3.4.1).

- relaxed:  this  switch  enables  the  RELAXED  mode  right from the

beginning  of the  program, which  otherwise has  to be  enabled by

the pseudo instruction of sane name(see section 3.9.7).

- supmode:  this  switch  enables  right  from  the  beginning of the

program  usage of  machine instructions  that may  only be  used in

the processor's supervisor mode (see section 3.2.4).

- Y:  This switch instructs AS to to suppress all messages about out-

of-branch  conditions,  once  the  necessity  for  another  pass is

given.  See section 2.11 for the  (rare) situations that might make

use of this switch necessary.

- cpu  <name>:  this  switch  allows  to  set the target processor AS

shall  generate code for, in case  the source file does not contain

a  CPU instruction. If  the selected target  supports CPU arguments

(see  section  3.2.3),  they  may  be  used  on the command line as

well.  Using  this  switch  with  ?  or  list as argument lists all

implemented targets.

- alias  <new>=<old>:  defines  the  processor  type  <new>  to be an

alias  for  the  type  <old>.  See  section  2.14  for the sense of

processor aliases.

-  gnuerrors:  display messages  about errors  resp. warnings  not in

the  AS standard  format, but  instead in  a format  similar to the

GNU  C  compiler.  This  simplifies  the  integration  of  AS  into

environments  tuned for  this format,  however also  suppresses the

display of precise error positions in macro bodies!

-  maxerrors  [n]:  instructs  the  assembler  to  terminate assembly

after the given number of errors.

-  maxerrors  [n]: instructs  the assembler  to terminate assembly if

the  include  nesting  level  exceeds  the  given limit (default is

200).

-  Werror: instructs the assembler to treat warnings as errors.

-  wrelative  resp.  wno-relative:  instructs  the assembler to issue

warnings  if a  relative jump  instead of  an absolute  is possible

(only for Z80 target).

-  wsign-extension    resp.    wno-sign-extension:    instructs   the

assembler  to issue  warnings about  implicit sign extensions (onlx

68K, MOVEQ).

- compmode:  This  switch  instructs  the  assembler  to  operate  by

default   in  compatibility  mode.  See   section  3.9.8  for  more

information about this mode.

- packing:  This switch  overrides the  architecture specific default

of the PACKING option (see section 3.2.9).

As  long as switches require no  arguments and their concatenation does

not  result  in  a  multi-letter  switch,  it  is  possible  to specify

several switches at one time, as in the following example :

as test*.asm firstprog -cl /i c:\as\8051\include

All  files  TEST*.ASM  as  well  as  the  file  FIRSTPROG.ASM  will  be

assembled,  whereby listings of all files  are displayed on the console

terminal.  Additional sharefiles  will be  generated in  the C- format.

The  assembler  should  search  for  additional  include  files  in the

directory C:\AS\8051\INCLUDE.

This  example  shows  that  the  assembler  assumes  ASM as the default

extension for source files.

A  bit  of  caution  should  be  applied  when using switches that have

optional  arguments: if  a file  specification immediately follows such

a  switch without the optional argument,  AS will try to interprete the

file specification as argument - what of course fails:

as -g test.asm

The  solution in this  case would either  be to move  the -g option the

end or to specify an explicit MAP argument.

Beside  from  specifying  options  in  the  command  line,  permanently

needed  options may  be placed  in the  environment variable ASCMD. For

example,  if someone always  wants to have  assembly listings and has a

fixed  directory for include  files, he can  save a lot  of typing with

the following command:

set ascmd=-L -i c:\as\8051\include

The  environment  options  are  processed  before  the command line, so

options  in the  command line  can override  contradicting ones  in the

environment variable.

In  the case of very  long path names, space  in the ASCMD variable may

become  a problem. For  such cases a  key file may  be the alternative,

in  which the options can be written in  the same way as in the command

line  or the  ASCMD-variable. But  this file  may contain several lines

each  with a  maximum length  of 255  characters. In  a key  file it is

important,  that for  options which  require an  argument, switches and

argument  have to be written in the  same line. AS gets informed of the

name of the key file by a @ aheaded in the ASCMD variable, e.g.

set ASCMD=@c:\as\as.key

In  order to neutralize  options in the  ASCMD variable (or  in the key

file),  prefix the option with a plus  sign. For example, if you do not

want  to  generate  an  assembly  listing  in  an  individual case, the

option can be retracted in this way:

as +L <file>

Naturally  it is not consequently  logical to deny an  option by a plus

sign.... UNIX soit qui mal y pense.

References  to key  files may  not only  come from  the ASCMD variable,

but  also  directly  from  the  command  line.  Similarly  to the ASCMD

variable, prepend the file's name with a @ character:

as @<file> ....

The  options read from  a key file  in this situation  are processed as

if  they  had  been  written  out  in  the command line in place of the

reference,  not  like  the  key  file  referenced by the ASCMD variable

that is processed prior to the command line options.

Referencing  a key file from a key  file itself is not allowed and will

be answered wit an error message by AS.

In  case that you like to start AS  from another program or a shell and

this  shell  hands  over  only  lower-case  or  capital  letters in the

command  line, the following  workaround exists: if  a tilde (~) is put

in   front  of  an  option  letter,  the  following  letter  is  always

interpreted   as  a  lower-case  letter.  Similarly  a  #  demands  the

interpretation   as  a  capital  letter.  For  example,  the  following

transformations result for:

/~I ---> /i

-#u ---> -U

In  dependence of the  assembly's outcome, the  assembler ends with the

following return codes:

0 error free run, at maximum warnings occurred

1 The  assembler  displayed  only  its  command-line  parameters  and

terminated immediately afterwards.

2 Errors occurred during assembly, no code file has been produced.

3 A  fatal error  occurred what  led to  immediate termination of the

run.

4 An  error occurred already  while starting the  assembler. This may

be a parameter error or a faulty overlay file.

255 An  internal error  occurred during  initialization that should not

occur  in  any  case...reboot,  try  again,  and  contact me if the

problem is reproducible!

Similar  to UNIX, OS/2 extends an  application's data segment on demand   OS/2

when  the  application  really  needs  the memory. Therefore, an output

like

511 KByte available memory

does  not indicate a shortly  to come system crash  due to memory lack,

it  simply shows the distance  to the limit when  OS/2 will push up the

data segment's size again...

As  there is no  compatible way in  C under different operating systens   UNIX

to  find out  the amount  of available  memory resp.  stack, both lines

are missing completely from the statistics the C version prints.

2.5. Format of the Input Files

------------------------------

Like  most  assemblers,  AS  expects  exactly  one instruction per line

(blank  lines are  naturally allowed  as well).  The lines  must not be

longer than 255 characters, additional characters are discarded.

A single line has following format:

[label[:]] <mnemonic>[.attr] [param[,param..]] [;comment]

A  line  may  also  be  split  over  several  lines in the source file,

continuation  characters chain these  parts together to  a single line.

One  must however consider that, due  to the internal buffer structure,

the   total  line  must  not  be   longer  than  256  characters.  Line

references  in error messages always relate to  the last line of such a

composed source line.

The  colon for the label  is optional, in case  the label starts in the

first  column (the consequence is that  a machine or pseudo instruction

must  not start in column 1). It is  necessary to set the colon in case

the  label does  not start  in the  first column  so that AS is able to

distinguish  it from a mnemonic.  In the latter case,  there must be at

least  one space  between colon  and mnemonic  if the processor belongs

to  a family  that supports  an attribute  that denotes  an instruction

format   and  is  separated   from  the  mnemonic   by  a  colon.  This

restriction  is necessary  to avoid  ambiguities: a distinction between

a  mnemonic with  format and  a label  with mnemonic would otherwise be

impossible.

Some  signal processor  families from  Texas Instruments optionally use

a  double  line  (||)  in  place  of  the label to signify the parallel

execution  with  the  previous  instruction(s).  If these two assembler

instructions  become a single  instruction word at  machine level (C3x/

C4x),  an  additional  label  in  front  of  the  second instruction of

course  does  not  make  sense  and  is  not  allowed. The situation is

different  for  the  C6x  with  its  instruction  packets  of  variable

length:  If someone  wants to  jump into  the middle  of an instruction

packet  (bad style, if  you ask me...),  he has to  place the necessary

label  before into a  separate line. The  same is valid for conditions,

which  however may be combined with the  double line in a single source

line.

The  attribute is used by a  couple of processors to specify variations

or  different  codings  of  a  certain  instruction. The most prominent

usage  of the  attibute is  is the  specification of  the operand size,

for example in the case of the 680x0 family (table 2.3).

+-----------+-------------------------------------+---------------------+

| attribute | arithmetic-logic instruction        | jump instruction    |

+-----------+-------------------------------------+---------------------+

+-----------+-------------------------------------+---------------------+

| B         | byte (8 bits)                       | 8-bit-displacement  |

| W         | word (16 bits)                      | 16-bit-displacement |

| L         | long word (32 bits)                 | 16-bit-displacement |

| Q         | quad word (64 bits)                 | ---------           |

| C         | half precision (16 bits)            | ---------           |

| S         | single precision (32 bits)          | 8-bit-displacement  |

| D         | double precision (64 bits)          | ---------           |

| X         | extended precision (80/96 bits)     | 32-bit-displacement |

| P         | decimal floating point (80/96 bits) | ---------           |

+-----------+-------------------------------------+---------------------+

Table 2.3: Allowed Attributes (Example 680x0)

Since  this  manual  is  not  also  meant  as  a  user's manual for the

processor  families  supported  by  AS,  this  is unfortunately not the

place  to  enumerate  all  possible  attributes  for  all  families. It

should  however be mentioned  that in general,  not all instructions of

a  given instruction set allow all  attributes and that the omission of

an  attribute generally leads  to the usage  of the ''natural'' operand

size  of  a  processor  family.  For  more  thorough studies, consult a

reasonable programmer's manual, e.g. [1] for the 68K's.

In  the case  of TLCS-9000,  H8/500, and  M16(C), the  attribute serves

both  as  an  operand  size  specifier  (if  it is not obvious from the

operands)  and as a  description of the  instruction format to be used.

A  colon has to be  used to separate the  format from the operand size,

e.g. like this:

add.w:g   rw10,rw8

This  example does  not show  that there  may be a format specification

without  an  operand  size.  In  contrast,  if  an operand size is used

without   a  format  specification,  AS   will  automatically  use  the

shortest  possible format. The allowed  formats and operand sizes again

depend  on the machine instruction and may  be looked up e.g. in [166],

[35], [66], resp. [67].

The  number of  instruction parameters  depends on  the mnemonic and is

principally   located  between  0   and  20.  The   separation  of  the

parameters   from  each  other  is  to  be  performed  only  by  commas

(exception:  DSP56xxx, its  parallel data  transfers are separated with

blanks).  Commas that  are included  in brackets  or quotes, of course,

are not taken into consideration.

Instead  of  a  comment  at  the  end,  the  whole  line can consist of

comment if it starts in the first column with a semicolon.

To  separate  the  individual  components  you  may also use tabulators

instead of spaces.

2.6. Format of the Listing

--------------------------

The  listing produced by  AS using the  command line options  i or I is

roughly divisible into the following parts :

1 augmented reproduction of the source code;

2 symbol list;

3 usage list;

4 cross reference list.

The  two last  ones are  only generated  if they  have been demanded by

additional command line options.

In  the first part, AS lists the  complete contents of all source files

including  the produced code. A line  of this listing has the following

form:

[<n>] <line>/<address> <code> <source>

In  the field n, AS  displays the include nesting  level. The main file

(the  file where  assembly was  started) has  the depth  0, an included

file  from  there  has  depth  1  etc..  Depth  0 is not displayed: for

source   lines  in  the  main  file,  this  field  is  replaced  by  an

appropriate  amount of  spaces, or  is omitted  entirely if  no include

statements  have  been  used  so  far.  The 'memory' whether there have

been  include statements  and up  to which  level, spans  more than one

pass.  This way,  the assembler  'learns' the  maximum include depth in

the  first pass and is  able to print this  field with consistent width

throughout the whole listing.

In  the field line,  the source line  number of the  referenced file is

issued.  The first  line of  a file  has the  number 1.  The address to

which  the code generated  from this line  is written follows after the

slash  in the field address. The number  system used for the address is

set  via  the  listradix  command  line  option (2.4), also whether the

address  is  printed  with  leading  zeros  or  not. The currently used

target  defines the  width of  this field  by the  size of  the address

space:  for a processor with  a 64K address space,  four hex digits are

sufficient,  while eight digits are needed  if the address space's size

is 4 GBytes.

The  code produced  is written  behind address  in the field code, also

in  the  number  system  defined  by  the  list radix. Depending on the

processor  type and actual  segment the values  are formatted either as

bytes  or 16/32-bit-words.  If more  code is  generated than  the field

can  take, additional lines will be  generated, in which case only this

field is used.

Finally,  in the field  source, the line  of the source  file is issued

in its original form.

Internally,  the structure  of the  data in  front of every source line

is  controlled  by  a  format  string,  which  may  be modified via the

command   line  switch  -listline-prefix.  It  supports  the  following

placeholders:

-  %[c]i:  The current include nesting  depth, with an optional field

width.  Without an  explicit field  width, this  may be expanded to

nothing or an equivalent number of spaces if the depth is zero.

-  %[c]n:  The  current  source  line  number, with an optional field

width.  The number is printed  right-aligned within this field. The

default for the field width is five characters.

-  %[c]a:  Similarly,  the  current  memory  address,  with a target-

specific default field width, as described above.

In  all cases, a field width with  a leading zero results in filling up

the  unused space  with zeros  instead of  blanks. The  default of this

format  string  is  %i%n/%a.  To  achieve  an output similar to the one

generated  by AS  versions previous  to 1.42  Build 249, %1i%5n/%8a may

be used as format string.

The  symbol table was designed in a way  that it can be displayed on an

80-column   display   whenever   possible.   For  symbols  of  ''normal

length'',  a  double  column  output  is  used. If symbols exceed (with

their  name and value) the limit  of 40 columns (characters), they will

be  issued  in  a  separate  line.  The  output is done in alphabetical

order.  Symbols that have  been defined but  were never used are marked

with a star (*) as prefix.

The  parts  mentioned  so  far  as  well  as  the  list  of all macros/

functions  defined  can  be  selectively  masked  out from the listing.

This  can  be  done  by  the  already mentioned command line switch -t.

There  is an internal  byte inside AS  whose bits represent which parts

are  to be written. The  assignment of bits to  parts of the listing is

listed in table 2.4.

+-----+--------------------------------+

| bit | part                           |

+-----+--------------------------------+

+-----+--------------------------------+

| 0   | source file(s) + produced code |

| 1   | symbol table                   |

| 2   | macro list                     |

| 3   | function list                  |

| 4   | line numbering                 |

| 5   | register symbol list           |

| 7   | character set table            |

+-----+--------------------------------+

Table 2.4: Assignment of Bits to Listing Components

All bits are set to 1 by default, when using the switch

-t <mask>

Bits  set in <mask>  are cleared, so  that the respective listing parts

are  suppressed. Accordingly it  is possible to  switch on single parts

again  with a  plus sign,  in case  you had  switched off too much with

the  ASCMD variable... If someone wants  to have, for example, only the

symbol table, it is enough to write:

-t 2

The  usage  list  issues  the  occupied  areas  hexadecimally for every

single  segment.  If  the  area  has  only  one  address,  only this is

written, otherwise the first and last address.

The  cross  reference  list  issues  any defined symbol in alphabetical

order and has the following form:

symbol <symbol name> (=<value>,<file>/<line>):

file <file 1>:

<n1>[(m1)]  ..... <nk>[(mk)]

.

.

file <file l>:

<n1>[(m1)]  ..... <nk>[(mk)]

The  cross reference  list lists  for every  symbol in  which files and

lines  it has  been used.  If a  symbol was  used several  times in the

same  line, this would be indicated by  a number in brackets behind the

line  number. If a  symbol was never  used, it would  not appear in the

list;  The  same  is  true  for  a  file  that  does  not  contain  any

references for the symbol in question.

CAUTION!  AS can only print the  listing correctly if it was previously

informed  about the output  media's page length  and width! This has to

be  done with the  PAGE instruction (see  3.7.1). The preset default is

a length of 60 lines and an unlimited line width.

2.7. Symbol Conventions

-----------------------

Symbols  are  allowed  to  be  up  to  255  characters  long (as hinted

already  in the introduction) and are  being distinguished on the whole

length, but the symbol names have to meet some conventions:

Symbol  names  are  allowed  to  consist  of  a  random  combination of

letters,  digits,  underlines  and  dots,  whereby  the first character

must  not  be  a  digit.  The  dot  is  only allowed to meet the MCS-51

notation  of register  bits and  should -  as far  as possible - not be

used  in own  symbol names.  To separate  symbol names  in any case the

underline (_) and not the dot (.) should be used .

AS  is by default  not case-sensitive, i.e.  it does not matter whether

one  uses upper  or lower  case characters.  The command  line switch U

however  allows to  switch AS  into a  mode where  upper and lower case

makes  a  difference.  The  predefined  symbol  CASESENSITIVE signifies

whether   AS  has  been  switched  to   this  mode:  TRUE  means  case-

sensitiveness, and FALSE its absence.

Table  2.5 shows  the most  important symbols  which are  predefined by

AS.

+-------------------+---------------------------------------------+

| name              | meaning                                     |

+-------------------+---------------------------------------------+

+-------------------+---------------------------------------------+

| TRUE              | logically ''true''                          |

| FALSE             | logically ''false''                         |

| CONSTPI           | Pi (3.1415.....)                            |

| FLOATMAX          | largest representable floating point number |

| VERSION           | version of AS in BCD-coding,                |

|                   | e.g. 1331 hex for version 1.33p1            |

| ARCHITECTURE      | target platform AS was compiled for, in     |

|                   | the style processor-manufacturer-operating  |

|                   | system                                      |

| DATE              | date and                                    |

| TIME              | time of the assembly (start)                |

| MOMCPU            | current target CPU                          |

|                   | (see the CPU instruction)                   |

| MOMFILE           | current source file                         |

| MOMLINE           | line number in source file                  |

| MOMPASS           | number of the currently running pass        |

| MOMSECTION        | name of the current section                 |

|                   | or an empty string                          |

|  *, ,. $ resp. PC | current value of program counter            |

+-------------------+---------------------------------------------+

Table 2.5: Predefined Symbols

CAUTION!  While  it  does  not  matter  in  case-sensitive  mode  which

combination  of upper  and lower  case to  use to  reference predefined

symbols,  one has  to use  exactly the  version given above (only upper

case) when AS is in case-sensitive mode!

Additionally  some pseudo instructions define  symbols that reflect the

value  that has  been set  with these  instructions. Their descriptions

are explained at the individual commands belonging to them.

On  most platforms,  the name  INF is  reserved as infinity in floating

point format. It therefore may not be used for user-defined symbols.

A  hidden feature (that has to be  used with care) is that symbol names

may  be  assembled  from  the  contents  of string symbols. This can be

achieved  by framing  the string  symbol's name  with curly  braces and

inserting  it into  the new  symbol's name.  This allows for example to

define a symbol's name based on the value of another symbol:

cnt             set     cnt+1

temp            equ     "\{CNT}"

jnz     skip{temp}

.

.

skip{temp}:     nop

CAUTION:  The programmer  has to  assure that  only valid  symbol names

are generated!

A  complete  list  of  all  symbols  predefined  by  AS can be found in

appendix F.

Apart  from its value, every symbol  also owns a marker which signifies

to  which segment it  belongs. Such a  distinction is mainly needed for

processors  that  have  more  than  one  address  space. The additional

information  allows AS to  issue a warning  when a wrong instruction is

used  to  access  a  symbol  from  a  certain  address space. A segment

attribute  is automatically added to a  symbol when is gets defined via

a  label or a  special instruction like  BIT; a symbol  defined via the

''allround  instructions'' SET resp. EQU  is however ''typeless'', i.e.

its  usage will  never trigger  warnings. A  symbol's segment attribute

may be queried via the buit-in function SYMTYPE, e.g.:

Label:

.

.

Attr    equ     symtype(Label)  ; results in 1

The  individual  segment  types  have  the  assigned  numbers listed in

table  2.6. Register symbols which do not  really fit into the order of

normal  symbols  are  explained  in  section 2.12. The SYMTYPE function

delivers  -1  as  result  when  called  with  an  undefined  symbol  as

argument.  However, if  all you  want to  know is  whether a  symbol is

defined  or not, or  whether an expression  contains no no symbols that

are undefined so far, you may as well use the DEFINED function.

+-------------------+--------------+

| segment           | return value |

+-------------------+--------------+

| <none>            |       0      |

| CODE              |       1      |

| DATA              |       2      |

| IDATA             |       3      |

| XDATA             |       4      |

| YDATA             |       5      |

| BITDATA           |       6      |

| IO                |       7      |

| REG               |       8      |

| ROMDATA           |       9      |

| EEDATA            |      10      |

| <Register Symbol> |      128     |

+-------------------+--------------+

Table 2.6: return values of the SYMTYPE function

2.8. Temporary Symbols

----------------------

Especially  when dealing with programs  that contain sequences of loops

of  if-like statements, one  is continuously faced  with the problem of

inventing  new  names  for  labels  -  labels of which you know exactly

that  you will  never need  to reference  them again afterwards and you

really  would like to get  'rid' of them somehow.  A simple solution if

you  don't  want  to  swing  the  large hammer of sections (see chapter

3.8)  are temporary symbols which  remain valid as long  as a new, non-

temporary  symbol  gets  defined.  Other  assemblers  offer  a  similar

mechanism  which is commonly referred  as 'local symbols'; however, for

the  sake  of  a  better  distinction,  I  want  to  stay with the term

'temporary  symbols'.  AS  knows  three  different  types  of temporary

symbols,  in the  hope to  offer everyone  'switching' to AS a solution

that  makes conversion as easy  as possible. However, practically every

assembler  has its  own interpretation  of this  feature, so there will

be only few cases where a 1:1 solution for existing code:

2.9. Named Temporary Symbols

----------------------------

A  symbol whose  name starts  with two  dollar signs (something that is

neither  allowed  for  non-temporary  symbols  nor  for constants) is a

named  temporary symbol.  AS keeps  an internal  counter which is reset

to  0  before  assembly  begins  and  which gets incremented upon every

definition  of  a  non-temporary  symbol.  When  a  temporary symbol is

defined  or  referenced,  both  leading  dollar signs are discarded and

the  counter's current  value is  appended. This  way, one  regains the

used  symbol names  with every  definition of  a non-temporary symbol -

but  you also cannot  reach the previously  symbols any more! Temporary

symbols   are   therefore   especially   suited   for  usage  in  small

instruction   blocks,  typically  a   dozen  of  machine  instructions,

definitely  not  more  than  one  screen.  Otherwise,  one  easily gets

confused...

Here is a small example:

$$loop: nop

dbra    d0,$$loop

split:

$$loop: nop

dbra    d0,$$loop

Without  the non-temporary label between the  loops, of course an error

message about a double-defined symbol would be the result.

2.9.1. Nameless Temporary Symbols

- - - - - - - - - - - - - - - - -

For  all  those  who  regard  named  temporary  symbols  still  as  too

complicated,  there is an even simpler  variant: If one places a single

puls  or minus sign  as a label,  this is converted  to symbol names of

__forwnn   respectively  __backmm,   with  nn   respectively  mm  being

counters  that  start  counting  at  zero. Those symbols are referenced

via  the special names -  -- --- respectively +  ++ +++, which refer to

the  three  last  'minus  symbols'  and  the next three 'plus symbols'.

Therefore,   the  selection  between  these  two  variants  depends  on

whether one wants to forward- or backward-reference a symbol.

Apart  from plus  and minus,  defining nameless  temporary symbols also

exists  in  a  third  variant,  namely  a slash (/). A temporary symbol

defined  in this way may be  referenced both backward and forward, i.e.

it  is treated either as a plus or  a minus, depending on the way it is

being referenced.

Nameless  temporary symbols are usually used  in constructs that fit on

one  screen page,  like skipping  a few  machine instructions  or tight

loops  - things  would becone  to puzzling  otherwise (this only a good

advice,  however...). An  example for  this is  the following  piece of

code, this time as 65xx code:

cpu     6502

-       ldx     #00

-       dex

bne     -           ; branch to 'dex'

lda     RealSymbol

beq     +           ; branch to 'bne --'

jsr     SomeRtn

iny

+       bne     --          ; branch to 'ldx #00'

SomeRtn:

rts

RealSymbol:

dfs     1

inc	ptr

bne 	+      	    ; branch to 'tax'

inc 	ptr+1

+ 	tax

bpl 	++     	    ; branch to 'dex'

beq 	+      	    ; branch forward to 'rts'

lda 	#0

/  	rts            	    ; slash used as wildcard.

+ 	dex

beq 	-           ; branch backward to 'rts'

ptr:	dfs	2

2.9.2. Composed Temporary Symbols

- - - - - - - - - - - - - - - - -

This  is maybe  the type  of temporary  symbols that  is nearest to the

concept  of  local  symbols  and  sections.  Whenever  a  symbol's name

begins  with a  dot (.),  the symbol  is not  directly stored with this

name  in  the  symbol  table.  Instead,  the name of the most recently-

defined  symbol not  beginning with  a dot  is prepended to the symbols

name.   This  way,  'non-dotted'  symbols  take  the  role  of  section

separators  and  'dotted'  symbol  names  may  be  reused after a 'non-

dotted'  symbol has been  defined. Take a  look at the following little

example:

proc1:				; non-temporary symbol 'proc1'

.loop	moveq	#20,d0		; actually defines 'proc1.loop'

dbra	d0,.loop

rts

proc2:				; non-temporary symbol 'proc2'

.loop	moveq	#10,d1		; actually defines 'proc2.loop'

jsr	proc1

dbra	d1,.loop

rts

Note  that it is  still possible to  access all temporary symbols, even

without  being in  the same  'area', by  simply using the composed name

(like 'proc2.loop' in the previous example).

It  is principally possible to  combine composed temporary symbols with

sections,  which makes  them also  to local  symbols. Take however into

account  that the most  recent non-temporary symbol  is not stored per-

section,  but  simply  globally.  This  may  change however in a future

version, so one shouldn't rely on the current behaviour.

2.10. Formula Expressions

-------------------------

In  most  places  where  the  assembler  expects  numeric inputs, it is

possible  to specify  not only  simple symbols  or constants,  but also

complete   formula  expressions.   The  components   of  these  formula

expressions  can be either single  symbols and constants. Constants may

be either integer, floating point, or string constants.

2.10.1. Integer Constants

- - - - - - - - - - - - -

Integer  constants describe non-fractional numbers.  They are witten as

a  sequence of digits. This may  be done in different numbering systems

(see table 2.7).

+---------+---------------+---------------+-----------+----------------+

|         |   Intel Mode  | Motorola Mode |   C Mode  |    IBM Mode    |

+---------+---------------+---------------+-----------+----------------+

+---------+---------------+---------------+-----------+----------------+

| Decimal |     Direct    |     Direct    |   Direct  |     Direct     |

| Hex     |    Suffix H   |    Prefix $   | Prefix 0x | X'..' or H'..' |

| Ident   |      hexh     |      $hex     |   0xhex   |     x'hex'     |

|         |               |               |           |     h'hex'     |

| Binary  |    Suffix B   |    Prefix %   | Prefix 0b |      O'..'     |

| Ident   |      binb     |      %bin     |   0bbin   |     b'bin'     |

| Octal   | Suffix O or Q |    Prefix @   |  Prefix 0 |      B'..'     |

| Ident   |      octo     |      @oct     |    0oct   |     o'oct'     |

|         |      octq     |               |           |                |

| ASCII   |               |               |           |      A'..'     |

| Ident   |               |               |           |     a'asc'     |

+---------+---------------+---------------+-----------+----------------+

Table 2.7: Defined Numbering Systems and Notations

In  case the numbering system has  not been explicitly stated by adding

the  special control  characters listed  in the  table, AS  assumes the

base  given with the RADIX statement  (which has itself 10 as default).

This  statement  allows  to  set  up  'unusual' numbering systems, i.e.

others than 2, 8, 10, or 16.

Valid  digits are numbers  from 0 to  9 and letters  from A to Z (value

10  to 35) up  to the numbering  system's base minus  one. An exception

from  this is the ASCII represenation:  For this variant, a character's

ASCII  value  (or  its  code  in  the  currently  active code page, see

section  3.1.12) describes  a whole  byte. Therefore, integer constants

written  this  way  are  identical  to multi character constants. These

two expressions:

'ABCD'

A'ABCD'

are  identical,  the  'A'  prefix  is  redundant.  One  may enable this

syntax  for existing code, because there  are a few original assemblers

(e.g. for the Signetics 2650) that support this syntax.

Independent  of the target, AS  implements multi character constants in

big  endian order, which means that  'ABCD' results in an integer value

of  0x41424344.  Why  this?  Well,  AS's  first target was the Motorola

60008,  and no one ever objected...the  only exception from this is the

PDP-11  (and the WD16  which uses an  LSI-11): For better compatibility

to  DEC's MACRO-11,  multi character  are little  endian if this target

is used. For instance, 'AB' results in ain integer value of 0x4241.

The  usage of  letters in  integer constants  however brings along some

ambiguities  since  symbol  names  are  also  sequences  of numbers and

letters:  a symbol name however must not  start with a character from 0

to  9. This means that an integer  constant which is not clearly marked

a  such with a special  prefix character must not  begin with a letter.

One  has to add  an additional, otherwise  superfluous zero in front in

such  cases.  The  most  prominent  case  is the writing of hexadecimal

constants  in Intel  mode: If  the leftmost  digit is  between A and F,

the  trailing H  doesn't help  to clarify,  an additional  0 has  to be

prefixed  (e.g.  0F0H  instead  of  F0H).  The  Motorola and C syntaxes

which  both mark  the numbering  system at  the front  of a constant do

not have this issue.

Quite  tricky  is  furthermore  that  the  higher the default numbering

system  set  via  RADIX  becomes,  the  more  letters  used  to  denote

numbering  systems in Intel  and C syntax  become 'eaten'. For example,

you  cannot  write  binary  constants  anymore  after  a  RADIX 16, and

starting  at RADIX  18, the  Intel syntax  even doesn't  allow to write

hexadecimal constants any more. Therefore CAUTION!

Appendix  E  lists  which  syntax  is  used by which target by default.

Independent  of  this  default,  there  is  always the option to add or

delete  individual syntax  variants via  the INTSYNTAX instruction (see

section  3.9.6). The  names listed  as Ident,  prefixed with  a plus or

minus sign, serve as arguments to this instruction.

The  RELAXED instruction (see  section 3.9.7) serves  as a sort 'global

enable   switch':  in  relaxed   mode,  all  notations   may  be  used,

independent  of the  selected target  processor. The  result is that an

arbitrary  syntax  may  be  used  (possibly  loosing  compatibility  to

standard assemblers).

Both  INTSYNTAX  and  RELAXED  specifically  enable  usage  of the 'IBM

syntax'   for  all   targets,  which   is  sometimes   found  on  other

assemblers:

This  notation puts the actual value  into apostrophes and prepends the

numbering  system ('x'  or 'h'  for hexadecimal,  'o' for octal and 'b'

for  binary). So, the integer constant  305419896 can be written in the

following ways:

x'12345678'

h'12345678'

o'2215053170'

b'00010010001101000101011001111000'

Another  variant of  this notation  for some  targets is  to leave away

the  closing apostrophe, to allow simpler  porting of existing code. It

is not recommended for new programs.

2.10.2.  Floating Point Constants

- - - - - - - - - - - - - - - - -

Floating  point constants  are to  be written  in the  usual scientific

notation, which is known in the most general form:

[-]<integer digits>[.post decimal positions][E[-]exponent]

CAUTION!  The  assembler  first  tries  to  interprete a constant as an

integer  constant and  makes a  floating-point format  try only in case

the  first one failed. If someone wants  to enforce the evaluation as a

floating  point  number,  this  can  be  done  by  dummy  post  decimal

positions, e.g. 2.0 instead of 2.

2.10.3.  String Constants

- - - - - - - - - - - - -

String  constants have  to be  enclosed in  single or  double quotation

marks.  In  order  to  make  it  possible to include quotation marks or

special  characters in  string constants,  an ''escape  mechanism'' has

been implemented, which should sound familiar for C programmers:

The  assembler  understands  a  backslash  (\) with a following decimal

number  of three digits maximum  in the string as  a character with the

according  decimal ASCII  value. The  numerical value may alternitavely

be  written in hexadecimal or octal notation  if it is prefixed with an

x  resp. a 0.  In case of  hexadecimal notation, the  maximum number of

digits  is limited to 2. For example,  it is possible to include an ETC

character  by  writing\3.  But  be  careful  with the definition of NUL

characters!  The C  version currently  uses C  strings to store strings   UNIX

internally.  As  C  strings  use  a  NUL character for termination, the

usage of NUL characters in strings is currently not portable!

Some  frequently used control  characters can also  be reached with the

following abbreviations:

\b : Backspace           \a : Bell         \e : Escape

\t : Tabulator           \n : Linefeed     \r : Carriage Return

\\ : Backslash           \' or \H : Apostrophe

\" or \I : Quotation marks

Both   upper  and   lower  case   characters  may   be  used   for  the

identification letters.

By   means  of  this  escape  character,  you  can  even  work  formula

expressions into a string, if they are enclosed by curly braces: e.g.

message "root of 81 : \{sqrt(81)}"

results in

root of 81 : 9

AS  chooses  with  the  help  of  the  formula  result type the correct

output  format, further  string constants,  however, are  to be avoided

in  the expression.  Otherwise the  assembler will  get mixed up at the

transformation  of capitals  into lower  case letters.  Integer results

will  by  default  be  written  in  hexadecimal  notation, which may be

changed via the OUTRADIX instruction.

Except  for  the  insertion  of  formula  expressions, you can use this

''escape-mechanism''  as well in ASCII  defined integer constants, like

this:

move.b   #'\n',d0

However,  everything  has  its  limits,  because the parser with higher

priority,  which disassembles a line  into op-code and parameters, does

not know what it is actually working with, e.g. here:

move.l   #'\'abc',d0

After  the  third  apostrophe,  it  will  not  find the comma any more,

because  it  presumes  that  it  is  the  start  of a further character

constant.  An  error  message  about  a  wrong  parameter number is the

result. A workaround would be to write e.g., \i instead of \'.

2.10.4.  String to Integer Conversion and Character Constants

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Earlier   versions  of  AS  strictly  distinguished  between  character

strings  and  so-called  ''character  constants'':  At  first glance, a

character  constant  looks  like  a  string, the characters are however

enclosed  in single instead  of double quotation  marks. Such an object

had  the data  type 'Integer',  i.e. it  represented a  number with the

value  given  by  the  (ASCII)  code  of  the  character,  and  it  was

something completely different:

move.b   #65,d0

move.b   #'A',d0      ; equal to first instruction

move.b   #"A",d0      ; not allowed in older versions!

This  strict  differentiation  no  longer  exists,  so it is irrelevant

whether  single  or  double  quotes  are  used.  If an integer value is

expected  as argument,  and a  string is  used, the  conversion via the

character's  (ASCII) value is  done ''on the  fly'' at this place. This

means  that in the  example given, all  three lines result  in the same

machine code.

Such  an  implicit  conversion  to  integer  values also take place for

strings  consisting of  multiple constancs,  which are sometimes called

''multi character constants'':

'A'    ==$41

'AB'   ==$4142

'ABCD' ==$41424344

Multi  character  constants  are  the  only  case where using single or

double  quotes  still  makes  a  difference. Many targets define pseudo

instructions   to  dispose  constants  in   memory,  and  which  accept

different  data types.  In such  a case,  it is  still necessary to use

double quotes if a character string shall be placed in memory:

dc.w    "ab"  ; disposes two words (0x0041,0x0042)

dc.w    'ab'  ; disposes one word (0x4142)

Important:  using  the  correct  quotation  is  not  necessary  if  the

character  string is  longer than  the used  operand size, which is two

characters or 16 bits in this example.

2.10.5.  Evaluation

- - - - - - - - - -

The  calculation of intermediary results  within formula expressions is

always  done with the highest resolution  available on the host system.

For  integer  numbers,  this  is  32  or  64  bits.  For floating point

numbers,   the  range  is   approximately  +/-1.8*10^308  (IEEE  Double

Precision)  or  +/-1.1*10^4932  (IEEE  Extended  Precision). A possible

test for value range overflows is done only on the final result.

2.10.6. Operators

- - - - - - - - -

The   assembler  provides  the   operands  listed  in   table  2.8  for

combination.

+---------+------------------+-------+-------+-------+--------+-----+------+

| Operand | Function         | #Args |  Int  | Float | String | Reg | Rank |

+---------+------------------+-------+-------+-------+--------+-----+------+

+---------+------------------+-------+-------+-------+--------+-----+------+

|    <>   | inequality       |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    !=   | alias for <>     |       |       |       |        |     |      |

|    >=   | greater or equal |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    <=   | less or equal    |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    <    | truly smaller    |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    >    | truly greater    |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    =    | equality         |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    ==   | alias for =      |       |       |       |        |     |      |

|         |                  |       |       |       |        |     |      |

|    !!   | log. XOR         |   2   |  yes  |   no  |   no   |  no |  13  |

|    ||   | log. OR          |   2   |  yes  |   no  |   no   |  no |  12  |

|    &&   | log. AND         |   2   |  yes  |   no  |   no   |  no |  11  |

|    ~~   | log. NOT         |   1   |  yes  |   no  |   no   |  no |   2  |

|         |                  |       |       |       |        |     |      |

|    -    | difference       |   2   |  yes  |  yes  |   no   |  no |  10  |

|    +    | sum              |   2   |  yes  |  yes  |   yes  |  no |  10  |

|    #    | modulo division  |   2   |  yes  |   no  |   no   |  no |   9  |

|    /    | quotient         |   2   | yes*) |  yes  |   no   |  no |   9  |

|     *   | product          |   2   |  yes  |  yes  |   no   |  no |   9  |

|     ^   | power            |   2   |  yes  |  yes  |   no   |  no |   8  |

|         |                  |       |       |       |        |     |      |

|    !    | binary XOR       |   2   |  yes  |   no  |   no   |  no |   7  |

|    |    | binary OR        |   2   |  yes  |   no  |   no   |  no |   6  |

|    &    | binary AND       |   2   |  yes  |   no  |   no   |  no |   5  |

|    ><   | mirror of bits   |   2   |  yes  |   no  |   no   |  no |   4  |

|    >>   | log. shift right |   2   |  yes  |   no  |   no   |  no |   3  |

|    <<   | log. shift left  |   2   |  yes  |   no  |   no   |  no |   3  |

|     ~   | binary NOT       |   1   |  yes  |   no  |   no   |  no |   1  |

+---------+------------------+-------+-------+-------+--------+-----+------+

|                      *) remainder will be discarded                      |

+---------+------------------+-------+-------+-------+--------+-----+------+

Table 2.8: Operators Predefined by AS

''Rank''   is  the  priority  of  an  operator  at  the  separation  of

expressions  into subexpressions.  The operator  with the  highest rank

will  be evaluated  at the  very end.  The order  of evaluation  can be

defined by new bracketing.

The  comparison operators deliver TRUE in  case the condition fits, and

FALSE  in case it  doesn't. For the  logical operators an expression is

TRUE in case it is not 0, otherwise it is FALSE.

For  operators with two  arguments, the order  of operand evaluation is

undefined.  The  only  exception  from  this  is  the  logical  AND and

logical  OR:  If  the  result  is  unambiguously  defined  by  the left

operand, the right operand is not evaluated at all.

Two  details have to  be kept im  mind when comparing register symbols.

First,  two  register  symbols  are  equal  if  they  refer to the same

register.  Some processors  have alias  names for  registers, and these

aliases  are  regarded  as  equal.  Fr  instance,  the A7 register of a

68000  may also be  referred to as  SP, and those  two register symbols

are  equal. On the other  hand, some processors have  more than one set

of  registers.  The  68040,  fo  rinstance,  has 'normal' (integer) and

floating  point  registers.  There  is  no  greater or smaller relation

between  registers from  different groups,  the corresponding operators

always  return  FALSE.  Only  a  test  for equality or inequality makes

sense.

The  mirroring of bits probably needs  a little bit of explanation: the

operator  mirrors the lowest  bits in the  first operand and leaves the

higher  priority  bits  unchanged.  The  number  of bits which is to be

mirrored is given by the right operand and may be between 1 and 32 .

A   small  pitfall  is   hidden  in  the   binary  complement:  As  the

computation  is always done  with 32 resp.  64 bits, its application on

e.g.  8-bit masks usually results in values  taht do not fit into 8-bit

numbers  any more due to the leading  ones. A binary AND with a fitting

mask is therefore unavoidable!

2.10.7. Functions

- - - - - - - - -

In  addition to  the operators,  the assembler  defines another line of

primarily   transcendental  functions  with  floating  point  arguments

which are listed in tables 2.9 and 2.10.

+----------+----------------------+------------------------+----------------+

| name     | meaning              | argument               | result         |

+----------+----------------------+------------------------+----------------+

+----------+----------------------+------------------------+----------------+

| SQRT     | square root          | arg >= 0               | floating point |

|          |                      |                        |                |

| SIN      | sine                 | arg in R               | floating point |

| COS      | cosine               | arg in R               | floating point |

| TAN      | tangent              | arg != (2n+1)*(Pi)/(2) | floating point |

| COT      | cotangent            | arg != n*Pi            | floating point |

|          |                      |                        |                |

| ASIN     | inverse sine         | | arg | <= 1           | floating point |

| ACOS     | inverse cosine       | | arg | <= 1           | floating point |

| ATAN     | inverse tangent      | arg in R               | floating point |

| ACOT     | inverse cotangent    | arg in R               | floating point |

|          |                      |                        |                |

| EXP      | exponential function | arg in R               | floating point |

| ALOG     | 10 power of argument | arg in R               | floating point |

| ALD      | 2 power of argument  | arg in R               | floating point |

| SINH     | hyp. sine            | arg in R               | floating point |

| COSH     | hyp. cosine          | arg in R               | floating point |

| TANH     | hyp. tangent         | arg in R               | floating point |

| COTH     | hyp. cotangent       | arg != 0               | floating point |

|          |                      |                        |                |

| LN       | nat. logarithm       | arg > 0                | floating point |

| LOG      | dec. logarithm       | arg > 0                | floating point |

| LD       | bin. logarithm       | arg > 0                | floating point |

| ASINH    | inv. hyp. Sine       | arg in R               | floating point |

| ACOSH    | inv. hyp. Cosine     | arg >= 1               | floating point |

| ATANH    | inv. hyp. Tangent    | arg < 1                | floating point |

| ACOTH    | inv. hyp. Cotangent  | arg > 1                | floating point |

|          |                      |                        |                |

| INT      | integer part         | arg in R               | floating point |

+----------+----------------------+------------------------+----------------+

| BITCNT   | number of one's      | integer                | integer        |

| FIRSTBIT | lowest 1-bit         | integer                | integer        |

+----------+----------------------+------------------------+----------------+

Table  2.9: Functions Predefined  by AS -  Part 1 (Integer and Floating

Point Functions

+-------------+----------------------+----------------+----------------+

| name        | meaning              | argument       | result         |

+-------------+----------------------+----------------+----------------+

+-------------+----------------------+----------------+----------------+

| LASTBIT     | highest 1-bit        | integer        | integer        |

| BITPOS      | unique 1-bit         | integer        | integer        |

|             |                      |                |                |

| SGN         | sign (0/1/-1)        | floating point | integer        |

|             |                      | or integer     |                |

| ABS         | absolute value       | integer or     | integer or     |

|             |                      | floating point | floating point |

| TOUPPER     | matching capital     | integer        | integer        |

| TOLOWER     | matching lower case  | integer        | integer        |

|             |                      |                |                |

| UPSTRING    | changes all          | string         | string         |

|             | characters           |                |                |

|             | into capitals        |                |                |

|             |                      |                |                |

| LOWSTRING   | changes all          | string         | string         |

|             | characters           |                |                |

|             | into to lower case   |                |                |

|             |                      |                |                |

| STRLEN      | returns the length   | string         | integer        |

|             | of a string          |                |                |

|             |                      |                |                |

| SUBSTR      | extracts parts of a  | string,        | string         |

|             | string               | integer,       |                |

|             |                      | integer        |                |

| CHARFROMSTR | extracts a character | string,        | integer        |

|             | from a string        | integer        |                |

| STRSTR      | searches a substring | string,        | integer        |

|             | in a string          | string         |                |

| VAL         | evaluates contents   | string         | depends on     |

|             | as expression        |                | argument       |

| EXPRTYPE    | delivers type of     | integer,       | 0              |

|             | argument             | float,         | 1              |

|             |                      | string         | 2              |

+-------------+----------------------+----------------+----------------+

Table  2.10: Functions  Predefined by  AS -  Part 2 (Integer and String

Functions

The  functions FIRSTBIT, LASTBIT, and BITPOS  return -1 as result if no

resp.  not exactly one bit is  set. BITPOS additionally issues an error

message in such a case.

The   string  function  SUBSTR  expects  the  source  string  as  first

parameter,  the start position  as second and  the number of characters

to  be  extracted  as  third  parameter  (a  0  means  to  extract  all

characters  up to the  end). Similarly, CHARFROMSTR  expects the source

string   as  first  argument  and  the  character  position  as  second

argument.  In  case  the  position  argument  is larger or equal to the

source   string's  length,   SUBSTR  returns   an  empty  string  while

CHARFROMSTR  returns  -1.  A  position  argument  smaller  than zero is

treated  as zero  by SUBSTR,  while CHARFROMSTR  will return -1 also in

this case.

Here  is an  example how  to use  these both  functions. The task is to

put  a string  into memory,  with the  string end  being signified by a

set MSB in the last character:

dbstr   macro   arg

if      strlen(arg) > 1

db     substr(arg, 0, strlen(arg) - 1)

endif

if      strlen(arg) > 0

db     charfromstr(arg, strlen(arg) - 1) | 80h

endif

endm

STRSTR  returns the  first occurence  of the  second string  within the

first  one resp. -1 if  the search pattern was  not found. Similarly to

SUBSTR and CHARFROMSTR, the first character has the position 0.

If  a function expects floating point  arguments, this does not mean it

is impossible to write e.g.

sqr2 equ sqrt(2)

In  such cases an automatic type  conversion is engaged. In the reverse

case  the INT-function  has to  be applied  to convert a floating point

number  to  an  integer.  When  using  this  function,  you have to pay

attention  that  the  result  produced  always  is a signed integer and

therefore has a value range of approximately +/-2.0E9.

When  AS is switched  to case-sensitive mode,  predefined functions may

be  accessed with an arbitrary combination  of upper and lower case (in

contrast  to predefined symbols). However,  in the case of user-defined

functions  (see section 3.4.9),  a distinction between  upper and lower

case  is made. This has e.g. the  result that if one defines a function

Sin,  one can  afterwards access  this function  via Sin, but all other

combinations  of  upper  and  lower  case  will  lead to the predefined

function.

For  a correct conversion of lower  case letters into capital letters a   DOS/DPMI

DOS version >= 3.30 is required.

2.11. Forward References and Other Disasters

--------------------------------------------

This  section is  the result  of a  significant amount  of hate  on the

(legal)  way  some  people  program.  This  way  can lead to trouble in

conjunction  with  AS  in  some  cases.  The section will deal with so-

called  'forward references'. What makes  a forward reference different

from  a usual reference?  To understand the  difference, take a look at

the  following  programming  example  (please  excuse  my  bias for the

68000 family that is also present in the rest of this manual):

move.l  #10,d0

loop:   move.l  (a1),d1

beq     skip

neg.l   d1

skip:   move.l  d1,(a1+)

dbra    d0,loop

If  one overlooks  the loop  body with  its branch statement, a program

remains  that is  extremely simple  to assemble:  the only reference is

the   branch  back  to  the  body's  beginning,  and  as  an  assembler

processes  a program from the beginning  to the end, the symbol's value

is  already known  before it  is needed  the first  time. If  one has a

program  that only contains such backward  references, one has the nice

situation  that  only  one  pass  through  the source code is needed to

generate   a  correct  and  optimal   machine  code.  Some  high  level

languages  like Pascal  with their  strict rule  that everything has to

be  defined before  it is  used exploit  exactly this property to speed

up the compilation.

Unfortunately,  things are  not that  simple in  the case of assembler,

because  one sometimes  has to  jump forward  in the  code or there are

reasons  why one has to move  variable definitions behind the code. For

our  example, this is the case for  the conditional branch that is used

to  skip over another  instruction. When the  assembler hits the branch

instruction  in the first pass, it  is confronted with the situation of

either  leaving  blank  all  instruction  fields  related to the target

address  or  offering  a  value  that  ''hurts  noone'' via the formula

parser  (which  has  to  evaluate  the  address argument). In case of a

''simple''  assembler that  supports only  one target architecture with

a  relatively small  number of  instructions to  treat, one will surely

prefer  the first solution,  but the effort  for AS with  its dozens of

target  architectures  would  have  become  extremely  high.  Only  the

second  way  was  possible:  If  an  unknown  symbol is detected in the

first  pass, the formula parser  delivers the program counter's current

value  as result! This is  the only value suitable  to offer an address

to  a branch  instruction with  unknown distance  length that  will not

lead  to errors.  This answers  also a  frequently asked question why a

first-pass  listing (it will not be erased  e.g. when AS does not start

a   second  pass  due  to  additional  errors)  partially  shows  wrong

addresses  in  the  generated  binary  code  -  they  are the result of

unresolved forward references.

The  example listed above however  uncovers an additional difficulty of

forward  references: Depending  on the  distance of  branch instruction

and  target  in  the  source  code,  the  branch  may be either long or

short.  The  decision  however  about  the  code length - and therefore

about  the addresses of following labels -  cannot be made in the first

pass  due to  missing knowledge  about the  target address. In case the

programmer  did  not  explicitly  mark  whether  a long or short branch

shall  be used, genuine  2-pass assemblers like  older versions of MASM

from  Microsoft  ''solve''  the  problem  by  reserving  space  for the

longest  version  in  the  first  pass  (all label addresses have to be

fixed  after the first pass) and  filling the remaining space with NOPs

in  the  second  pass.  AS  versions  up  to 1.37 did the same before I

switched   to  the   multipass  principle   that  removes   the  strict

separation  into two passes  and allows an  arbitrary number of passes.

Said  in detail, the  optimal code for  the assumed values is generated

in  the first pass. In  case AS detects that  values of symbols changed

in  the second  pass due  to changes  in code  lengths, simply  a third

pass  is done, and as the second  pass'es new symbol values might again

shorten  or  lengthen  the  code,  a  further pass is not impossible. I

have  seen  8086  programs  that  needed  12  passes  to get everything

correct  and optimal. Unfortunately,  this mechanism does  not allow to

specify  a maximum number passes; I can  only advise that the number of

passes   goes  down  when  one  makes   more  use  of  explicit  length

specifications.

Especially  for  large  programs,  another  situation  might arise: the

position  of a forward directed branch has  moved so much in the second

pass  relative to the first  pass that the old  label value still valid

is  out of  the allowed  branch distance.  AS knows  of such situations

and  suppresses all error  messages about too  long branches when it is

clear  that another pass  is needed. This  works for 99%  of all cases,

but  there  are  also  constructs  where the first critical instruction

appears  so early  that AS  had no  chance up  to now to recognize that

another  pass  is  needed.  The  following  example  constructs  such a

situation  with the  help of  a forward  reference (and  was the reason

for this section's heading...):

cpu   6811

org     $8000

beq     skip

rept    60

ldd    Var

endm

skip:   nop

Var     equ     $10

Due  to the  address position,  AS assumes  long addresses in the first

pass  for the LDD  instructions, what results  in a code  length of 180

bytes  and an out  of branch error  message in the  second pass (at the

point  of the BEQ  instruction, the old  value of skip  is still valid,

i.e.  AS does not  know at this  point that the  code is only 120 bytes

long  in reality)  is the  result. The  error can  be avoided  in three

different ways:

1 Explicitly   tell  AS   to  use   short  addressing   for  the  LDD

instructions (ldd <Var)

2 Remove  this  damned,  rotten  forward  reference and place the EQU

statement  at  the  beginning  where  it  has to be (all right, I'm

already calming down...)

3 For  real die-hards:  use the  -Y command  line option. This option

tells  AS to forget  the error message  when the address change has

been detected. Not pretty, but...

Another  tip regarding  the EQU  instruction: AS  cannot know  in which

context  a symbol defined with  EQU will be used,  so an EQU containing

forward  references will not  be done at  all in the  first pass. Thus,

if  the symbol defined  with EQU gets  forward-referenced in the second

pass:

move.l  #sym2,d0

sym2    equ     sym1+5

sym1    equ     0

one  gets an  error message  due to  an undefined  symbol in the second

pass...but why on earth do people do such things?

Admittedly,  this was quite  a lengthy excursion,  but I thought it was

necessary. Which is the essence you should learn from this section?

1 AS  always tries to  generate the shortest  code possible. A finite

number  of  passes  is  needed  for  this.  If  you do not tweak AS

extremely, AS will know no mercy...

2 Whenever  sensible  and  possible,  explicitly  specify  branch and

address  lengths. There is  a chance of  significantly reducing the

number of passes by this.

3 Limit  forward references  to what  is absolutely  needed. You make

your and AS's live much easier this way!

2.12. Register Symbols

----------------------

valid  for:  PowerPC,  M-Core,  XGate,  4004/4040,  MCS-48/(2)51, 8086,

80C16x,  AVR, XS1,  Z8, KCPSM,  Mico8, MSP430(X),  ST9, M16,  M16C, H8/

300, H8/500, SH7x00, H16, i960, XA, 29K, TLCS-9000, KENBAK, SC/MP

Sometimes  it is desirable not only  to assign symbolic names to memory

addresses  or  constants,  but  also  to  a  register, to emphasize its

function  in  a  certain  program  section.  This  is  no  problem  for

processors  that treat  registers simply  as another  address space, as

this  allows to use numeric expressions and  one can use simple EQUs to

define  such symbols. (e.g.  for the MCS-96  or TMS70000). However, for

most  processors,  register  identifiers  are  fixed literals which are

seperately  treated  by  AS  for  speed  reasons.  Therefore, registers

symbols  (sometime also called 'register  aliases') are also a separate

type  of symbols  in the  symbol table.  Just like  other symbols, they

may  be  defined  or  re-defined  with  EQU  or  SET,  and  there  is a

specialized   REG   instruction   which   accepts   only   symbols  and

expressions of this type.

On  the  other  hand,  register  symbols  are  subject  of  a couple of

restrictions:  the number  of literals  is limited  and depends  on the

selected  target processor,  and arithmetic  operations are not possibl

eon registers A construct like tihs:

myreg   reg     r17         ; definition of register symbol

addi    myreg+1,3   ; does not work!

is not valid. Simple assignments are however possible:

myreg   reg     r17         ; definition of register symbol

myreg2  reg     myreg       ; myreg2 -> r17

Furthermore,  forward references are even  more critical than for other

types  of symbols. If a  symbol is not (yet)  defined, AS does not know

which  type it is  going to have,a  nd will decide  for a plain integer

number.  For  most  target  processors,  a  number is the equivalent of

absolute  memory addressing,  and on  most processors,  usage of memory

operands  is more  limited than  of registers.  Depending on situation,

one  will get  an error  message about  a non-allowed  addressing mode,

and no second pass will be started...

Analogous  to ordinary symbols, register  symbols are local to sections

and  it  is  possible  to  access  a  register  symbol  from a specific

section by appending the section's name enclosed in brackets.

2.13. Share File

----------------

This  function is a by-product from  the old pure-68000 predecessors of

AS,  I  have  kept  them  in  case  someone  really needs it. The basic

problem   is  to  access  certain  symbols  produced  during  assembly,

because  possibly  someone  would  like  to  access  the  memory of the

target  system via  this address  information. The  assembler allows to

export  symbol values by  means of SHARED  pseudo commands (see there).

For  this  purpose,  the  assembler  produces  a  text  file  with  the

required  symbols and its values  in the second pass.  This file may be

included  into a  higher-level language  or another  assembler program.

The  format of  the text  file (C,  Pascal or  Assembler) can be set by

the command line switches p, c or, a.

CAUTION!  If none of the  switches is given, no  file will be generated

and  it makes no  difference if SHARED-commands  are in the source text

or not!

When  creating a Sharefile, AS  does not check if  a file with the same

name  already exists,  such a  file will  be simply  overwritten. In my

opinion  a request does  not make sense,  because AS would  ask at each

run  if it should overwrite the old  version of the Sharefile, and that

would be really annoying...

2.14. Processor Aliases

-----------------------

Common  microcontroller families are like  rabbits: They become more at

a  higher speed than  you can provide  support for them. Especially the

development  of processor  cores as  building blocks  for ASICs  and of

microcontroller  families with user-definable peripherals  has led to a

steeply  rising number  of controllers  that only  deviate from a well-

known  type by a slightly modified  peripheral set. But the distinction

among  them is  still important,  e.g. for  the design of include files

that  only  define  the  appropriate  subset  of  peripherals.  I  have

struggled  up to now  to integrate the  most important reperesentatives

of  a processor family  into AS (and  I will continue  to do this), but

sometimes  I just cannot keep pace  with the development...there was an

urgent  need for a  mechanism to extend  the list of  processors by the

user.

The  result  are  processor  aliases:  the  alias  command  line option

allows  to define a new processor  type, whose instruction set is equal

to  another processor built into AS.  After switching to this processor

via  the  CPU  instruction,  AS  behaves  exactly  as  if  the original

processor  had  been  used,  with  a  single  difference: the variables

MOMCPU  resp. MOMCPUNAME  are set  to the  alias name,  which allows to

use the new name for differentiation, e.g. in include files.

There  were two  reasons to  realize the  definition of  aliases by the

command  line and  not by  pseudo instructions:  first, it would anyway

be  difficult  to  put  the  alias  definitions  together with register

definitions  into a single  include file, because  a program that wants

to  use such a file  would have to include  it before and after the CPU

instruction  -  an  imagination  that  lies somewhere between inelegant

and  impossible. Second, the  definition in the  command line allows to

put  the definitions  in a  key file  that is executed automatically at

startup  via  the  ASCMD  variable,  without  a need for the program to

take any further care about this.

3. Pseudo Instructions

======================

Not  all pseudo  instructions are  defined for  all processors.  A note

that  shows  the  range  of  validity  is  therefore prepended to every

individual description.

3.1. Definitions

----------------

3.1.1. SET, EQU, and CONSTANT

- - - - - - - - - - - - - - -

valid for: all processors, CONSTANT only for KCPSM(3)

SET  and  EQU  allow  the  definition  of typeless constants, i.e. they

will  not be assigned  to a segment  and their usage  will not generate

warnings  because of  segment mixing.  EQU defines  constants which can

not  be  modified  (by  EQU)  again,  but SET permits the definition of

variables,  which can be  modified during the  assembly. This is useful

e.g.  for the allocation of resources  like interrupt vectors, as shown

in the following example:

VecCnt  set     0       ; somewhere at the beginning

.

.

.

DefVec  macro   Name    ; allocate a new vector

Name    equ     VecCnt

VecCnt  set     VecCnt+4

endm

.

.

.

DefVec  Vec1    ; results in Vec1=0

DefVec  Vec2    ; results in Vec2=4

constants  and variables  are internally  stored in  the same  way, the

only  difference is  that they  are marked  as unchangeable  if defined

via  EQU. Trying to change a constant  with SET will result in an error

message.

EQU/SET allow to define constants of all possible types, e.g.

IntTwo   equ    2

FloatTwo equ    2.0

Some  processors  unfortunately  have  already  a  SET instruction. For

these  targets, EVAL must be used  instead of SET if no differentiation

via  the argument  count is  possible. As  an alternative, it is always

possible  to explicitly invoke  the pseudo instruction  by prepending a

period (.SET instead of SET).

A  single equation sign or .EQU may  be used instead of EQU. Similarly,

one  may simply write := instead  of SET resp. EVAL. Furthermore, there

is  an 'alternate'  syntax that  does not  take the  symbol's name from

the   label  field,  but  instead  from  the  first  argument.  So  for

instance, it is valid to write:

EQU   IntTwo,2

EQU   FloatTwo,2.0

For  compatibility reasons to the  original assembler, the KCPSM target

also  knows  the  CONSTANT  statement,  which  -  in  contrast to EQU -

always expects name and value as arguments. For example:

CONSTANT  const1, 2

CONSTANT is however limited to integer constants.

Symbols   defined  with  SET  or  EQU  are  typeless  by  default,  but

optionally  a segment name  (CODE, DATA, IDATA,  XDATA, YDATA, BITDATA,

IO,  or  REG)  or  MOMSEGMENT  for  the currently active segment may be

given  as a  second or  third parameter,  allowing to assign the symbol

to  a specific address  space. AS does  not check at  this point if the

used address space exists on the currently active target processor!

A  little hidden  extra feature  allows to  set the program counter via

SET  or EQU, something  one would ordinarily  do via ORG. To accomplish

this,  use the special  value as symbol  name that may  also be used to

query  the current program  counter's value. Depending  on the selected

target  architecture,  this  is  either  an  asterisk, a dollar sign, a

period, or PC.

In  case  the  target  architecture  supports instruction attributes to

define  the operand  size (e.g.  on 680x0),  those are also allowed for

SET  and EQU. The operand  size will be stored  along with the symbol's

value in the symbol table. Its use is architecture-dependant.

3.1.2. SFR and SFRB

- - - - - - - - - -

valid for: various, SFRB only MCS-51

These  instructions act  like EQU,  but symbols  defined with  them are

assigned  to  the  directly  addressable  data  resp. I/O segment, i.e.

they  are  preferrably  used  for  the  definition of (as the name lets

guess)  hardware  registers  mapped  into  the  data res. I/O area. The

allowed  range of values is  equal to the range  allowed for ORG in the

data  segment (see section 3.2.1). The  difference between SFR and SFRB

is  that SFRB marks  the register as  bit addressable, which  is why AS

generates  8  additional  symbols  which  will  be  assigned to the bit

segment and carry the names xx.0 to xx.7, e.g.

PSW     sfr     0d0h    ; results in PSW = D0H (data segment)

PSW     sfrb    0d0h    ; results in extra PSW.0 = D0H (bit)

;               to PSW.7 = D7H (bit)

The  SFRB instruction  is not  any more  defined for  the 80C251  as it

allows  direct  bit  access  to  all  SFRs without special bit symbols;

bits like PSW.0 to PSW.7 are automatically present.

Whenever  a bit-addressable register is defined  via SFRB, AS checks if

the  memory address is bit addressable  (range 20h..3fh resp. 80h, 88h,

90h,  98h...0f8h). If  it is  not bit-addressable,  a warning is issued

and the generated bit symbols are undefined.

3.1.3.  XSFR and YSFR

- - - - - - - - - - -

valid for: DSP56xxx

Also  the DSP56000 has a few  peripheral registers memory-mapped to the

RAM,  but the  affair becomes  complicated because  there are  two data

areas,  the X- and Y-area.  This architecture allows on  the one hand a

higher  parallelism, but forces on the  other hand to divide the normal

SFR  instruction into  the two  above mentioned  variations. They works

identically  to  SFR,  just  that  XSFR  defines  a  symbol  in  the X-

addressing  space  and  YSFR  a  corresponding  one in the Y-addressing

space. The allowed value range is 0..$ffff.

3.1.4.  LABEL

- - - - - - -

valid for: all processors

The  function of  the LABEL  instruction is  identical to  EQU, but the

symbol  does  not  become  typeless,  it  gets  the attribute ''code''.

LABEL  is needed exactly for one  purpose: Labels are normally local in

macros,  that means  they are  not accessible  outside of a macro. With

an EQU instruction you could get out of it nicely, but the phrasing

<name>  label   $

generates a symbol with correct attributes.

3.1.5.  BIT

- - - - - -

valid  for: MCS/(2)51, XA, 80C166, 75K0,  ST9, AVR, S12Z, SX20/28, H16,

H8/300, H8/500, KENBAK, Padauk

BIT  serves to  equate a  single bit  of a  memory cell with a symbolic

name.  This instruction varies from  target platform to target platform

due  to the different ways in  which processors handle bit manipulation

and addressing:

The  MCS/51  family  has  an  own  address  space for bit operands. The

function  of  BIT  is  therefore  quite  similar  to SFR, i.e. a simple

integer  symbol with the  specified value is  generated and assigned to

the  BDATA segment.  For all  other processors,  bit addressing is done

in  a two-dimensional fashion  with address and  bit position. In these

cases,  AS  packs  both  parts  into  an  integer  symbol in a way that

depends  on the  currently active  target processor  and separates both

parts  again when the symbol  is used. The latter  is is also valid for

the 80C251: While an instruction like

My_Carry bit    PSW.7

would  assign  the  value  0d7h  to  My_Carry  on  an  8051, a value of

070000d0h  would  be  generated  on  an  80C251,  i.e.  the  address is

located  in  bits  0..7  and  the  bit  position  in  bits 24..26. This

procedure  is equal to  the way the  DBIT instruction handles things on

a  TMS370 and  is also  used on  the 80C166,  with the  only difference

that bit positions may range from 0..15:

MSB     BIT     r5.15

On  a Philips XA, the  bit's address is located  in bits 0..9 just with

the  same coding as used  in machine instructions, and  the 64K bank of

bits in RAM memory is placed in bits 16..23.

The  BIT  instruction  of  the  75K0  family  even goes further: As bit

expressions   may   not   only   use   absolute  base  addresses,  even

expressions like

bit1    BIT     @h+5.2

are allowed.

The  ST9 in  turn allows  to invert  bits, what  is also allowed in the

BIT instruction:

invbit  BIT     r6.!3

More  about the  ST9's BIT  instruction can  be found  in the processor

specific hints.

In  case of H16, note  that the address and  bit position arguments are

swapped.  This was done to  make the syntax of  BIT consistent with the

machine instructions that maipulate individual bits.

3.1.6. DBIT

- - - - - -

valid for: TMS 370xxx

Though  the  TMS370  series  does  not  have  an  explicit bit segment,

single  bit  symbols  may  be  simulated  with  this  instruction. DBIT

requires  two operands,  the address  of the  memory cell that contains

the bit and the exact position of the bit in the byte. For example,

INT3        EQU  P019

INT3_ENABLE DBIT 0,INT3

defines  the  bit  that  enables  interrupts  via  the  INT3  pin. Bits

defined  this  way  may  be  used  in  the  instructions  SBIT0, SBIT1,

CMPBIT, JBIT0, and JBIT.

3.1.7. DEFBIT and DEFBITB

- - - - - - - - - - - - -

S12Z

....

The  S12Z family's  processor core  provides instructions to manipulate

individual  bits in registers or  memory cells. To conveniently address

bits  in the CPU's  I/O area (first  4 Kbytes of  the address space), a

bit  may be  given a  symbolic name.  The bit  is defined by its memory

address and the bit position:

<name>         defbit[.size]   <address>,<position>

The  address  must  be  located  within  the  first  4  Kbytes, and the

operand  size may be 8, 16,  or 32 bits (size=b/w/l). Consequently, the

position  may at  most be  7, 15  or 31.  If no  operand size is given,

byte  size  (.b)  is  assumed.  A  bit  defined this way may be used as

argument for the instructions BCLR, BSET, BTGL, BRSET, and BRCLR:

mybit   defbit.b  $200,4

bclr.b    $200,#4

bclr      mybit

Both  uses of  bclr in  this example  generate identical  code. Since a

bit  defined this  way ''knows''  its size,  the size  attribute may be

omitted when using it.

It  is  also  possible  to  define  bits  that  are  located  within  a

structure's element:

mystruct struct    dots

reg      ds.w      1

flag     defbit    reg,4

ends

org       $100

data     mystruct

bset      data.flag  ; same as bset.w $100,#4

Super8

......

Opposed  to the 'classic' Z8, the  Super8 core supports instructions to

operate  on bits  in working  or general  registers. ONe however has to

to  regard that some of them can only  operate on bits in one of the 16

working  registers.  The  DEFBIT  instruction  allows to define bits of

either type:

workbit defbit  r3,#4

slow    defbit  emt,#6

Bits  that have been defined this way  may be used just like a argument

duple of register and bit position:

ldb     r3,emt,#6

ldb     r3,slo          ; same result

bitc    r3,#4

bitc    workbit         ; same result

Z8000

.....

The  Z8000 features  instructions to  set and  clear bits, however they

cannot  access addresses  in I/O  space. For  this reason,  both DEFBIT

and  DEFBITB  only  allow  to  define  bit objects in memory space. The

differentiation  in operand  size is  important because  the Z8000 is a

big  endian processor: bit n of a  16 bit word at address m corresponds

to bit n of an 8-bit byte at address m+1.

�PD7807...�PD7809

.................

The  lowest 16 bytes of the working  area and special registers with an

address less than 16 are bit addressable.

3.1.8.  DEFBITFIELD

- - - - - - - - - -

valid for: S12Z

The  S12Z family's CPU core not only  deals with individual bits, it is

also  able to  extract a  field of  consecutive bits from an 8/16/24/32

value  or to insert a  bit field into such  a value. Similar to DEFBIT,

a bit field may be defined symbolically:

<Name>     defbitfield[.size] <address>,<width>:<position>

Opposed  to individual bits,  an operand size  of 24 bits  (.p) is also

alloweed.  The  range  of  position  and  width  is accordingly 0 to 23

resp.  1 to  24. It  is also  allowed to  define bit fields as parts of

structures:

mystruct struct      dots

reg      ds.w        1

clksel   defbitfield reg,4:8

ends

org       $100

data     mystruct

bfext     d2,data.clksel ; fetch $100.w bits 4..11

; to D2 bits 0..7

bfins     data.clksel,d2 ; insert D2 bits 0..7 into

; $100.w bits 4..11

The  internal representation of  bits defined via  DEFBIT is equivalent

to  bit fields with  a width of  one. Therefore, a symbolically defined

bit may also be used as argument for BFINS and BFEXT.

3.1.9. PORT

- - - - - -

valid  for: PALM, 8008/8080/8085/8086, XA, Z80, Z8000, 320C2x/5x, TLCS-

47, AVR, F8, IMP-16

PORT  works similar to EQU, just the  symbol becomes assigned to the I/

O-address  range.  Allowed  values  are  0..7  for  the 3201x and 8008,

0..15  for  the  320C2x  and  PALM,  0..65535  for the 8086, Z8000, and

320C5x, 0..63 for the AVR, and 0..255 for the rest.

Example : an 8255 PIO is located at address 20H:

PIO_port_A port 20h

PIO_port_B port PIO_port_A+1

PIO_port_C port PIO_port_A+2

PIO_ctrl   port PIO_port_A+3

3.1.10. REG and NAMEREG

- - - - - - - - - - - -

valid for: 680x0, AVR, M*Core, ST9, 80C16x, Z8000, KCPSM,

PDP-11, WD16

( NAMEREG valid only for KCPSM(3)), LatticeMico8, MSP430(X)

Though  it always has the same  syntax, this instruction has a slightly

different  meaning from processor  to processor: If  the processor uses

a  separate addressing space for registers,  REG has the same effect as

a  simple EQU for  this address space  (e.g. for the  ST9). REG defines

register  symbols for all other  processors whose function is described

in section 2.12.

NAMEREG   exists  for  compatibility  reasons  to  the  original  KCPSM

assembler.  It  has  an  identical  function, however both register and

symbolic name are given as arguments, for example:

NAMEREG  s08, treg

On  the PDP-11,  REG may  additionally be  used without  a name  in the

label  field.  It  then  expects  a  single  ON  or OFF as argument and

enables  or disables the built-in  register aliases (Rn =  %n, SP = R6,

PC  = R7). They are  available by default, and  should only be disabled

if  they  conflict  with  own  synmbol  names in a program. The current

setting may be read from the symbol DEFAULT_REGSYMS.

3.1.11. LIV and RIV

- - - - - - - - - -

valid for: 8X30x

LIV  and RIV  allow to  define so-called  ''IV bus objects''. These are

groups  of bits located in a peripheral  memory cell with a length of 1

up  to 8  bits, which  can afterwards  be referenced  symbolically. The

result   is  that  one  does  not  anymore  have  to  specify  address,

position,  and  length  separately  for  instructions that can refer to

peripheral  bit groups. As the  8X30x processors feature two peripheral

address  spaces  (a  ''left''  and  a  ''right''  one),  there  are two

separate  pseudo  instructions.  The  parameters  of these instructions

are  however  equal:  three  parameters  have  to be given that specify

address,  start position  and length.  Further hints  for the  usage of

bus objects can be found in section 4.25 .

3.1.12. CHARSET

- - - - - - - -

valid for: all processors

Single  board  systems,  especially  when  driving LCDs, frequently use

character   sets  different  to   ASCII.  So  it   is  probably  purely

coincidental  that the umlaut  coding corresponds with  the one used by

the  PC. And  there are  of course  also (historical)  systems that use

some  variant of EBCDIC...to  avoid error-prone manual  encoding in the

source   code,  the   assembler  contains   a  translation   table  for

characters  which assigns a target  character to each (ASCII) character

in  the source code. Use the  CHARSET instruction to modify this table,

which  initial  translates  one-to-one.  CHARSET  may  be  used  with a

variety of arguments:

A simple

CHARSET

without any argument resets the table to the one-to-one default.

If  only a single argument  is given, it has  to be a string expression

which is interpreted as a file name by AS:

CHARSET  "mapping.bin"

AS  reads the first 256 bytes from  this table and copies them into the

translation   table.  This  allows   to  activate  complex,  externally

generated tables with a single statement.

All  other variants modify a  single entry or a  sequence of entries in

the table. Use two (integer) arguments to change a single entry:

CHARSET '�',128

means  that the target system codes the  '�' into the number 128. It is

als  possible to define that a  certain character is unavailable on the

target system. Leave the second argument empty to define this:

CHARSET '[',

If  the 'deleted' character shall be  disposed in memory, this reported

as an error.

Use  three arguments  to remap  a whole  range of characters. The first

and  second  argument  define  the  character  range, and the third one

defines  the  mapping  of  the  first  character.  For instance, if the

target system does not support lower case characters,

CHARSET 'a','z','A'

translates  all lower-case  characters automatically  into the matching

capital  letters. Similar  to a  single character,  it is also possible

to 'unmap' a range of characters:

CHARSET 'a','z',

forbids usage of lower case letters.

The  last variant  (again only  with two  arguments), a  string defines

the  mapping of  a sequence  of characters.  Mapping of  lower to upper

case may therefore also be written like this: be written as

CHARSET 'a',"ABCDEFGHIJKLMNOPQRSTUVWXYZ"

CAUTION!  CHARSET not only  affects string constants  stored in memory,

but  also multi character constants,  i.e. integer constants written as

''ASCII''.  This means that  an already modified  translation table can

lead to different results in the examples mentioned above!

The  built-in function CODEPAGE_VAL allows  to query the translation of

a  single character  in the  current code  page. It  will return -1 for

unmapped characters.

3.1.13.  CODEPAGE

- - - - - - - - -

valid for: all processors

Though  the CHARSET statement gives  unlimited freedom in the character

assignment   between   host   and   target  platform,  switching  among

different   character  sets   can  become   quite  tedious  if  several

character  sets  have  to  be  supported  on  the  target platform. The

CODEPAGE  instruction  however  allows  to  define  and  keep different

character  sets  and  to  switch  with  a  single statement among them.

CODEPAGE  expects one or two arguments: the  name of the set to be used

hereafter  and optionally  the name  of another  table that defines its

initial  contents (the  second parameter  therefore only  has a meaning

for  the first switch  to the table  when AS automatically creates it).

If  the second  parameter is  missing, the  initial contents of the new

table  are  copied  from  the  previously  active  set.  All subsequent

CHARSET statements only modify the new set.

At  the  beginning  of  a  pass,  AS  automatically  creates  a  single

character  set with  the name  STANDARD with  a one-to-one translation.

If  no CODEPAGE  instructions are  used, all  settings made via CHARSET

refer to this table.

3.1.14.  ENUM, NEXTENUM, and ENUMCONF

- - - - - - - - - - - - - - - - - - -

valid for: all processors

Similar  to the  same-named instruction  known from  C, ENUM is used to

define  enumeration types,  i.e. a  sequence of  integer constants that

are  assigned sequential values  starting at 0.  The parameters are the

names of the symbols, like in the following example:

ENUM    SymA,SymB,SymC

This  instruction will  assign the  values 0,  1, and  2 to the symbols

SymA, SymB, and SymC.

If  you  want  to  split  an  enumeration  over more than one line, use

NEXTENUM  instead of ENUM  for the second  and all following lines. The

internal  counter that assigns  sequential values to  alls symbols will

then not be reset to zero, like in the following case:

ENUM     January=1,February,March,April,May,June

NEXTENUM July,August,September,October

NEXTENUM November,December

This  example also demonstrates that it  is possible to assign explicit

values  to  individual  symbols.  The  internal counter will be updated

accordingly if this feature is used.

A  definition of a symbol with ENUM  is equal to a definition with EQU,

i.e.  it  is  not  possible  to  assign  a  new  value to a symbol that

already exists.

The  ENUMCONF  statement  allows  to  influence  the behaviour of ENUM.

ENUMCONF  accepts one  or two  arguments. The  first argument is always

the  value the internal  counter is incremented  for every symbol in an

enumeration. For instance, the statement

ENUMCONF 2

has  the  effect  that  symbols  get  the  values 0,2,4,6... instead of

0,1,2,3...

The  second (optional) argument  of ENUMCONF rules  which address space

the  defined symbols  are assigned  to. By  default, symbols defined by

ENUM are typeless. For instance, the statement

ENUMCONF 1,CODE

defines  that  they  should  be  assigned  to  the  instruction address

space.  The  names  of  the  address  spaces  are  the  same as for the

SEGMENT   instruction  (3.2.18),  with  the   addition  of  NOTHING  to

generate typeless symbols again.

3.1.15.  PUSHV and POPV

- - - - - - - - - - - -

valid for: all processors

PUSHV  and POPV allow to  temporarily save the value  of a symbol (that

is  not macro-local) and  to restore it  at a later  point of time. The

storage  is done  on stacks,  i.e. Last-In-First-Out memory structures.

A  stack has a  name that has  to fulfill the  general rules for symbol

names  and it  exists as  long as  it contains  at least one element: a

stack  that did not  exist before is  automatically created upon PUSHV,

and  a stack becoming  empty upon a  POPV is deleted automatically. The

name  of the  stack that  shall be  used to  save or restore symbols is

the  first parameter of PUSH resp. POPV,  followed by a list of symbols

as  further  parameters.  All  symbols  referenced  in the list already

have  to  exist,  it  is  therefore  not  possible to implicitly define

symbols with a POPV instruction.

Stacks  are  a  global  resource,  i.e.  their  names  are not local to

sections.

It  is important  to note  that symbol  lists are always processed from

left  to  right.  Someone  who  wants  to  pop several variables from a

stack  with a POPV  therefore has to  use the exact  reverse order used

in the corresponding PUSHV!

The name of the stack may be left blank, like this:

pushv   ,var1,var2,var3

.

.

popv    ,var3,var2,var1

AS will then use a predefined internal default stack.

AS  checks at the end of a pass  if there are stacks that are not empty

and  issues  their  names  together  with their ''filling level''. This

allows  to  find  out  if  there  are  any  unpaired  PUSHVs  or POPVs.

However,  it is in  no case possible  to save values  in a stack beyond

the end of a pass: all stacks are cleared at the beginning of a pass!

3.2. Code Modification

----------------------

3.2.1.  ORG

- - - - - -

valid for: all processors

ORG  allows to  load the  internal address  counter (of  the assembler)

with  a new  value. The  value range  depends on the currently selected

segment  and  on  the  processor  type  (table 3.1). The lower bound is

always zero, and the upper bound is the given value minus 1.

CAUTION:  If the  PHASE instruction  is also  used, one  has to keep in

mind  that the argument of ORG always  is the load address of the code.

Expressions  using the $  or * symbol  to refer to  the current program

counter  however deliver the  execution address of  the code and do not

yield  the  desired  result  when  used  as  argument for ORG. The RORG

statement (3.2.2) should be used in such cases.

Table 3.1: Address Ranges for ORG

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Target     |  CODE  |  DATA |   I-  |   X-  |  Y-  | BIT- |   IO  | REG | ROM- |  EE-  |

|            |        |       |  DATA |  DATA | DATA | DATA |       |     | DATA |  DATA |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68xxx/     |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| MCF        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| DSP56000   |  64K/  |  ---  |  ---  |  64K/ | 64K/ |  --- |  ---  | --- |  --- |  ---  |

| DSP56300   |   16M  |       |       |  16M  |  16M |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PowerPC    |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PALM       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| M*Core     |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6800,6301, |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 6811       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6805/      |   8K/  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| HC08       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6809,      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 6309,      |        |       |       |       |      |      |       |     |      |       |

| 052001     |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68HC12,    |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 68HC12X,   |        |       |       |       |      |      |       |     |      |       |

| XGATE      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| S12Z       |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68HC16     |   1M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68RS08     |   16K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| H8/300     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| H8/300H    |   16M  |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| H8/500     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| (Min)      |        |       |       |       |      |      |       |     |      |       |

| H8/500     |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| (Max)      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SH7000/    |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 7600/7700  |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| HD614023   |   2K   |  160  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

| HD614043   |   4K   |  256  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

| HD614081   |   8K   |  512  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| HD641016   |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6502,      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| MELPS-     |        |       |       |       |      |      |       |     |      |       |

| 740        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| HUC6280    |   2M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 65816,     |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| MELPS-     |        |       |       |       |      |      |       |     |      |       |

| 7700       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PPS-4      |   4K   |   4K  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MELPS-     |   8K   |  416  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 4500       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| M16        |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| M16C       |   1M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PDP-11     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |  256K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |  4M^10 |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| VAX        |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| WD16       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 4004       |   4K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 8008       |   16K  |   8   |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MCS-48,    | 1/2/4/ |  ---  |  256  | 256^8 |  --- |  --- |  ---  | --- |  --- |  ---  |

| MCS-41     | 6/8K^6 |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MCS-51     |   64K  |  256  | 256^1 |  64K  |  --- |  256 |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 80C390     |   16M  |  256  | 256^1 |  16M  |  --- |  256 |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MCS-251    |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  512  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MCS-96     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 196(N)/    |   16M  |       |       |       |      |      |       |     |      |       |

| 296        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 8080,      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

| 8085       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 80x86,     |   64K  |  64K  |  ---  |  64K  |  --- |  --- |  64K  | --- |  --- |  ---  |

| V20..55    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68xx0      |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 8X30x      |   8K   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 2650       |   32K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| XA         |   16M  |  16M  |  ---  |  ---  |  --- |  --- |  2K^3 | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| AVR        | 128K^6 | 32K^6 |  ---  |  ---  |  --- |  --- |   64  | --- |  --- |  8K^7 |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 29XXX      |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 80C166,    |  256K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 80C167     |   16M  |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| GBZ80      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---- | --- |  --- |  ---  |

| Z80,       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

| Z180,      | 512K^2 |       |       |       |      |      |  256  |     |      |  ---  |

| eZ80,      |   16M  |       |       |       |      |      |  64K  |     |      |  ---  |

| Z380       |   4G   |       |       |       |      |      |   4G  |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Z8         |   64K  |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| eZ8        |   64K  |  256  |  ---  |  64K  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Z8001,     |   8M   |  ---  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

| Z8003      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Z8002,     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

| Z8004      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| KCPSM      |   256  |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| KCPSM3     |   256  |   64  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Mico8      |  4096  |  256  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-      |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 900(L)     |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-90    |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 870(/C)    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-47    |   64K  |   1K  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-      |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 9000       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TC9331     |   320  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PIC        |   2K   |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 16C5x      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PIC        |   2K   |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 16C5x      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PIC        |        |       |       |       |      |      |       |     |      |       |

| 16C64,     |   8K   |  512  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- | 256^6 |

| 16C86      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PIC        |   64K  |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 17C42      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SX20       |   2K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ST6        |   4K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ST7        |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| STM8       |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ST9        |   64K  |  64K  |  ---  |  ---  |  --- |  --- |  ---  | 256 |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6804       |   4K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 32010      |   4K   |  144  |  ---  |  ---  |  --- |  --- |   8   | --- |  --- |  ---  |

| 32015      |   4K   |  256  |       |       |      |      |   8   |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C2x     |   64K  |  64K  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C3x     |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C40     |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C44     |   32M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C5x/    |   64K  |  64K  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

| 320C20x/   |        |       |       |       |      |      |       |     |      |       |

| 320C54x    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TMS        |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 9900       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TMS        |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 70Cxx      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 370xxx     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSP430     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TMS1000    |   1K   |   64  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| TMS1200    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TMS1100    |   2K   |  128  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| TMS1300    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| IMP-16     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  128  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| IPC-16     |   16K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SC/MP      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 807x       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| COP4       |   512  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| COP8       |   8K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SC144xx    |   256  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| NS16008/   |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| NS32008/   |        |       |       |       |      |      |       |     |      |       |

| NS08032/   |        |       |       |       |      |      |       |     |      |       |

| NS16032/   |        |       |       |       |      |      |       |     |      |       |

| NS32016/   |        |       |       |       |      |      |       |     |      |       |

| NS32032/   |        |       |       |       |      |      |       |     |      |       |

| NS32CG16   |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| NS32332/   |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| NS32532    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ACE        |  4K^4  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| CP-3F/     |   16K  |   48  |  ---  |  ---  |  --- |  --- |   8   |     |      |       |

| M380/      |        |       |       |       |      |      |       |     |      |       |

| LP8000     |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| F3850      |   64K  |   64  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

| F8         |   4K   |   64  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 78(C)xx    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |   640  |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 550        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |  1000  |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 554, 652   |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |  1000  |   64  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 547, 552   |        |       |       |       |      |      |       |     |      |       |

| 651        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |  2000  |   96  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 546, 553   |        |       |       |       |      |      |       |     |      |       |

| 556, 557   |        |       |       |       |      |      |       |     |      |       |

| 650        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 7566       |   1K   |   64  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 7508       |   4K   |  256  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 75K0       |   16K  |   4K  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 78K0       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 78K2       |   1M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 78K3       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 78K4       |  16M^5 |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 7720       |   512  |  128  |  ---  |  ---  |  --- |  --- |  ---  | --- |  512 |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 7725       |   2K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- | 1024 |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 77230      |   8K   |  ---  |  ---  |  512  |  512 |  --- |  ---  | --- |  1K  |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 70616      |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  16M  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 53C8XX     |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| F^2MC8L    |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| F^2MC16L   |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5840    |   2K   |  128  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5842    |   768  |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM58421   |  1.5K  |   40  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| MSM58422   |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5847    |  1.5K  |   96  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5054    |   1K   |   62  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5055    |  1.75K |   96  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5056    |  1.75K |   90  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM6051    |  2.5K  |  119  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MN1610     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MN1613     |  256K  |  ---  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PMCxxx/    |   1..  |  64.. |  ---  |  ---  |  --- |  --- |  32.. | --- |  --- |  ---  |

| PMSxxx/    |  4K^9  | 256^9 |       |       |      |      | 128^9 |     |      |       |

| PFSxxx     |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 180x       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |   8   | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| XS1        |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 1750       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| KENBAK     |   256  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| CP1600     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| NANO       |   2K   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| IM6100     |   4K   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| IM6120     |   32K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| RX...      |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SC61860    |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SC62015    |   1M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^1 Initial value 80h.                                                                  |

| As the 8051 does not have any RAM beyond 80h, this value has to be                     |

| adapted with ORG for the 8051 as target processor!                                     |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^2 As the Z180 still can address only 64K logically, the whole                         |

| address space can only be reached via PHASE instructions!                              |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^3 initial value 400h.                                                                 |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^4 initial value 800h resp. 0C00h                                                      |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^5 area for program code is limited to 1 MByte                                         |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^6 size depends on target processor                                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^7 size and availibility depend on target processor                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^8 only on variants supporting the MOVX instruction                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^9 device dependent                                                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^10 model dependent                                                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

In  case that different  variations in a  processor family have address

spaces of different size, the maximum range is listed for each.

ORG  is mostly  needed to  give the  code a  new starting address or to

put  different,  non-continuous  code  parts  into  one source file. In

case  there is  no explicit  other value  listet in  a table entry, the

initial  address for this segment (i.e.  the start address used without

ORG) is 0.

3.2.2. RORG

- - - - - -

valid for: all processors

RORG  modifies the program  counter just like  ORG, however it does not

expect   an  absolute  address  as  argument.  Instead,  it  expects  a

relative  value (positive  or negative)  that is  added to  the current

program  counter.  A  possible  application  of  this  statement is the

reservation  of a certain amount  of address space, or  the use in code

parts  that are included  multiple times (e.g.  via macros or includes)

and  that  shall  be  position-independent.  Another application is the

use  in  code  that  has  an  execution address different from the load

address  (i.e.  the  PHASE  statement  is  used). There is no symbol to

refer  to  the  current  load  address,  but  it  can  be  referred  to

indirectly via the RORG statement.

3.2.3.  CPU

- - - - - -

valid for: all processors

This  command  rules  for  which  processor  the  further code shall be

generated.   Instructions   of   other   processor   families  are  not

accessible afterwards and will produce error messages!

The  processors can  roughly be  distinguished in  families, inside the

families   different   types   additionally   serve   for   a  detailed

distinction:

a) 68008 -> 68000 -> 68010 -> 68012 ->

MCF5202 -> MCF5204 -> MCF5206 -> MCF5208->

MCF52274 -> MCF52277 -> MCF5307 -> MCF5329 ->

MCF5373 -> MCF5407 -> MCF5470 -> MCF5471 ->

MCF5472 -> MCF5473 -> MCF5474 -> MCF5475 ->

MCF51QM ->

68332 -> 68340 -> 68360 ->

68020 -> 68030 -> 68040

The   differences  in  this  family  are  additional  instructions  and

addressing  modes (starting from  the 68020). A  small exception is the

step  to the  68030 that  misses two  instructions: CALLM  and RTM. The

three  representatives  of  the  683xx  family  have the same processor

core  (a  slightly  reduced  68020  CPU),  however completely different

peripherals.   MCF5xxx  represents   various  ColdFire   variants  from

Motorola/Freescale/NXP,  RISC  processors  downwardly binary compatible

to  the 680x0. For  the 68040, additional  control registers (reachable

via  MOVEC) and instructions for control  of the on-chip MMU and caches

were added.

b) 56000 --> 56002 --> 56300

While   the  56002   only  adds   instructions  for   incrementing  and

decrementing   the  accumulators,  the  56300  core  is  almost  a  new

processor:  all address spaces  are enlarged from  64K words to 16M and

the number of instructions almost has been doubled.

c) PPC403 -> MPPC403 -> MPC505 -> MPC601

-> MPC821 -> RS6000

The  PPC403  is  a  reduced  version  of  the  PowerPC  line  without a

floating  point unit, which is why  all floating point instructions are

disabled  for him; in  turn, some microcontroller-specific instructions

have  been added  which are  unique in  this family.  The GC variant of

the  PPC403  incorporates  an  additional  MMU  and  has therefore some

additional    instructions   for    its   control.    The   MPC505   (a

microcontroller  variant without a  FPU) only differ  in its peripheral

registers  from the 601 as long as I do  not know it better - [82] is a

bit   reluctant  in  this  respect...  The  RS6000  line  knows  a  few

instructions  more (that are  emulated on many  601-based systems), IBM

additionally  uses  different  mnemonics  for  their  pure  workstation

processors, as a reminiscence of 370 mainframes...

d) IBM5100, IBM5110, IBM5120

These   three  types  currently  all   reference  to  the  same  (PALM)

prozessor core.

e) MCORE

f) XGATE

g) 6800 -> 6801 -> 6301 -> 6811

While  the  6801  only  offers  a  few additional instructions (and the

6301  even a few more),  the 6811 provides a  second index register and

much more instructions.

h) 6809/6309 and 6805/68HC(S)08

These  processors  are  partially  source-code  compatible to the other

68xx  processors, but  they have  a different  binary code format and a

significantly  reduced  (6805)  resp.  enhanced (6809) instruction set.

The  6309  is  a  CMOS  version  of  the  6809 which is officially only

compatible  to the 6809,  but inofficially offers  more registers and a

lot of new instructions (see [56]).

i) 68HC12 --> 68HC12X

The  12X  core  offers  a  couple  of  new  instructions,  and existing

instructions were were enriched with new addressing modes.

j) S912ZVC19F0MKH, S912ZVC19F0MLF,

S912ZVCA19F0MKH, S912ZVCA19F0MLF,

S912ZVCA19F0WKH, S912ZVH128F2CLQ,

S912ZVH128F2CLL, S912ZVH64F2CLQ,

S912ZVHY64F1CLQ, S912ZVHY32F1CLQ,

S912ZVHY64F1CLL, S912ZVHY32F1CLL,

S912ZVHL64F1CLQ, S912ZVHL32F1CLQ,

S912ZVHL64F1CLL, S912ZVHL32F1CLL,

S912ZVFP64F1CLQ, S912ZVFP64F1CLL,

S912ZVH128F2VLQ, S912ZVH128F2VLL,

S912ZVH64F2VLQ, S912ZVHY64F1VLQ,

S912ZVHY32F1VLQ, S912ZVHY64F1VL,

S912ZVHY32F1VLL, S912ZVHL64F1VLQ

All  variants contain the same processor  core and the same instruction

set,  only the  on-chip peripherals  and the  amount of built-in memory

(RAM, Flash-ROM, EEPROM) vary from device to device.

k) 68HC16

l) 052001

This  chip is  an own  creation of  Konami and  similar to the Motorola

6809  in architecture  an instruction  set. However,  it is  not binary

compatible  and does not provide  all instructions and addressing modes

of its 'role model'.

m) HD6413308 -> HD6413309

These  both names represent the 300 and 300H variants of the H8 family;

the  H version  owns a  larger address  space (16  Mbytes instead of 64

Kbytes),  double-width  registers  (32  bits),  and  knows  a  few more

instructions   and  addressing   modes.  It   is  still  binary  upward

compatible.

n) HD6475328 -> HD6475348 -> HD6475368 -> HD6475388

These  processors all share the same  CPU core; the different types are

only  needed to  include the  correct subset  of registers  in the file

REG53X.INC.

o) SH7000 -> SH7600 --> SH7700

The  processor core  of the  7600 offers  a few  more instructions that

close  gaps  in  the  7000's  instruction  set (delayed conditional and

relative  and indirect jumps, multiplications  with 32-bit operands and

multiply/add  instructions).  The  7700  series  (also  known  as  SH3)

furthermore  offers a second register  bank, better shift instructions,

and instructions to control the cache.

p) HD614023 --> HD614043 --> HD614081

These  three variants of the  HMCS400 series differ by  the size of the

internal ROM and RAM.

q) HD641016

This is currently the only target with H16 core.

r) 6502 -> 65(S)C02

-> 65CE02 / W65C02S / 65C19 /

MELPS740 / HUC6280 / 6502UNDOC

The  CMOS version  defines some  additional instructions,  as well as a

number  of  some  instruction/addressing  mode  combinations were added

which  were not possible on  the 6502. The W65C02S  adds two opcodes to

the  65C02 instruction set  to give more  fine-grained control over how

to  stop  the  CPU  for  low  power  modes.  The  65SC02  lacks the bit

manipulation   instructions  of  the  65C02.  The  65CE02  adds  branch

instructions  with 16-bit  displacement, a  Z register,  a 16 bit stack

pointer, a programmable base page, and a couple of new instructions.

The  65C19 is not  binary upward compatible  to the original 6502! Some

addressing  modes  have  been  replaced  by  others.  Furthermore, this

processor  contains instruction set  extensions that facilitate digital

signal processing.

The   Mitsubishi  micro   controllers  in   opposite  expand  the  6502

instruction  set  primarily  to  bit  operations  and  multiplication /

division   instructions.   Except   for   the  unconditional  jump  and

instructions  to increment/decrement  the accumulator,  the instruction

extensions have nothing in common.

For  the  HuC  6280,  the  feature  that  sticks out most is the larger

address  space of 2 MByte  instead of 64 KBytes.  This is achieved with

a  buil-tin banking  mechanism. Furthermore,  it features  some special

instructions  to  communicate  with  a  video  processor (this chip was

used in video games) and to copy memory areas.

The  6502UNDOC  processor  type  enables  access  to the "undocumented"

6502  instructions, i.e. the  operations that result  from the usage of

bit  combinations in the  opcode that are  not defined as instructions.

The  variants supported  by AS  are listed  in the  appendix containing

processor-specific hints.

s) MELPS7700, 65816

Apart  from  a  '16-bit-version'  of  the 6502's instruction set, these

processors  both  offer  some  instruction  set  extensions.  These are

however   orthogonal   as   they   are   oriented   along  their  8-bit

predecessors  (65C02 resp.  MELPS-740). Partially,  different mnemonics

are used for the same operations.

t) PPS-4

u) MELPS4500

v) M16

w) M16C

x) PDP-11/03, PDP-11/04, PDP-11/05, PDP-11/10,

PDP-11/15, PDP-11/20, PDP-11/23, PDP-11/24,

PDP-11/34, PDP-11/35, PDP-11/40, PDP-11/44,

PDP-11/45, PDP-11/50, MicroPDP-11/53,

PDP-11/55, PDP-11/60, PDP-11/70, MicroPDP-11/73,

MicroPDP-11/83, PDP-11/84, MicroPDP-11/93,

PDP-11/94, T-11

The  various  models  of  the  PDP-11  series differ in instruction set

(both  in  built-in  instructions  as  well as in available extensions)

and the supported address space (64, 256, or 4096 KBytes).

y) WD16

The   WD16  uses  the  same  processor  as  the  LSI-11,  however  with

different  microcode.  As  a  consequence,  register set and addressing

modes  are the  same as  for a  PDP-11, however  the instruction set is

slightly  different and instructions also  available on the PDP-11 have

different machine codes.

z) MICROVAX-I, MICROVAX-II,

VAX-11/725, VAX-11/730, VAX-11/750,

VAX-11/780, VAX-11/782, VAX-11/785,

VAX-8200, VAX-8300, VAX-8500, VAX-8600

VAX-8650, VAX-8800

All  implementations  of  the  VAX  architecture  share  the  same core

instruction  set.  However,  certain  extensions,  like instructions to

process  strings,  packed  decimal  numbers  or  certain floating point

formats, are not available in hardware.

aa) CP-3F, LP8000, M380

The  chipset's processor element was sold  by AEG/Olympia, GI, and SGS-

Ates  under  the  respective  names.  There  are  no differences in the

instruction set and address spaces.

ab) 4004 -> 4040

In  comparison  to  its  predecessor,  the  4040 features about a dozen

additional machine instructions.

ac) 8008 -> 8008NEW

Intel  redefined the 8008's  mnemonics around 1975,  the second variant

reflects  this  new  instruction  set.  A  simultaneous support of both

sets was not possible due to mnemonic conflicts.

ad) 8021, 8022,

8401, 8411, 8421, 8461,

8039, (MSM)80C39, 8048, (MSM)80C48, 8041, 8042, 80C382

For  the  ROM-less  versions  8039  and  80C39,  the commands which are

using  the BUS (port  0) are forbidden.  The 8021 and  8022 are special

versions  with a strongly shrinked instruction  set, for which the 8022

has  two  A/D-  converters  and  the  necessary  control-commands.  The

instruction  set  of  the  MAB8401  to  8461  (designed  by Philips) is

somewhere   in  between   the  8021/8022   and  a   ''complete''  MC-48

instruction  set. On the  other hand, they  provide serial ports and up

to 8 KBytes of program memory.

It  is possible to  transfer the CMOS-versions  with the IDL resp. HALT

command  into a stop mode with  lower current consumption. The 8041 and

8042   have  some  additional  instructions  for  controlling  the  bus

interface,  but in  turn a  few other  commands were  omitted. The code

address  space of  8041, 8042,  84x1, 8021,  and 8022 is not externally

extendable,  and so AS  limits the code  segment of these processors to

the  size of the internal ROM.  The (SAB)80C382 is a variant especially

designed  by  Siemens  for  usage  in  telephones. It also knows a HALT

instruction,  plus ist supports  indirect addressing for  DJNZ and DEC.

In  turn, several  instructions of  the 'generic'  8048 were  left out.

The  OKI variants  (MSM...) also  feature indirect  addressing for DJNZ

and  DEC,  plus  enhanced  control  of  power-down modes, plus the full

basic MCS-48 instrucion set.

ae) 87C750 -> 8051, 8052, 80C320, 80C501, 80C502,

80C504, 80515, and 80517

-> 80C390

-> 80C251

The  87C750 can only access a maximum  of 2 Kbytes program memory which

is  why it lacks the LCALL and  LJMP instructions. AS does not make any

distinction  among  the  processors  in  the  middle,  instead  it only

stores  the different names  in the MOMCPU  variable (see below), which

allows  to query the setting with  IF instructions. An exception is the

80C504  that has a mask  flaw in its current  versions. This flaw shows

up  when  an  AJMP  or  ACALL  instruction  starts  at  the second last

address  of a 2K  page. AS will  automatically use long instructions or

issues  an error  message in  such situations.  The 80C251  in contrast

represents  a drastic progress in the  the direction 16/32 bits, larger

address  spaces, and a more orthogonal  instruction set. One might call

the   80C390  the  'small   solution':  Dallas  Semiconductor  modified

instruction  set and architecture  only as far  as it was necessary for

the 16 Mbytes large address spaces.

af) 8096 -> 80196 -> 80196N -> 80296

Apart  from a different  set of SFRs  (which however strongly vary from

version  to  version),  the  80196  knows  several new instructions and

supports  a 'windowing'  mechanism to  access the  larger internal RAM.

The   80196N  family  extends  the  address  space  to  16  Mbytes  and

introduces  a set of instructions  to access addresses beyond 64Kbytes.

The  80296 extends the  CPU core by  instructions for signal processing

and  a  second  windowing  register,  however  removes  the  Peripheral

Transaction  Server  (PTS)  and  therefore  looses  again  two  machine

instructions.

ag) 8080 -> V30EMU -> 8085 -> 8085UNDOC

The  8085 knows  the additional  commands RIM  and SIM  for controlling

the  interrupt mask  and the  two I/O-pins.  The type 8085UNDOC enables

additional  instructions  that  are  not  documented  by  Intel.  These

instructions are documented in section 4.23.

V30EMU  as  target  behaves  like  an  8080,  with  the addition of the

instructions  RETEM  and  CALLN.  These  allow  to end or interrupt the

8080 emulation on a V20/V30/V40/V50.

ah) 8088,8086

-> 80188,80186

-> V20,V30,V40,V50

-> V33,V53

-> V25,V35

-> V55

-> V55SC

-> V55PI

Processors  listed in the  same line feature  an identical CPU core and

therefore   identical  instruction  set.  Going  down  the  lines,  new

instructions  are added, with the  NEC CPUs going different 'branches',

coming from the 'V20 basic instruction set'.

ai) 80960

aj) 8X300 -> 8X305

The  8X305 features a  couple of additional  registers that miss on the

8X300.  Additionally,  it  can  do  new operations with these registers

(like direct writing of 8 bit values to peripheral addresses).

ak) XAG1, XAG2, XAG3

These  processors only differ  in the size  of their internal ROM which

is defined in STDDEFXA.INC.

al) AT90S1200, AT90S2313, AT90S2323, AT90S233,

AT90S2343, AT90S4414, AT90S4433, AT90S4434,

AT90S8515, AT90C8534, AT90S8535,

ATTINY4, ATTINY5, ATTINY9,

ATTINY10, ATTINY11, ATTINY12, ATTINY13, ATTINY13A,

ATTINY15, ATTINY20, ATTINY24(A), ATTINY25,

ATTINY26, ATTINY28, ATTINY40, ATTINY44(A),

ATTINY45, ATTINY48, ATTINY84(A), ATTINY85,

ATTINY87, ATTINY88, ATTINY102, ATTINY104,

ATTINY167, ATTINY261, ATTINY261A, ATTINY43U,

ATTINY441, ATTINY461, ATTINY461A, ATTINY828,

ATTINY841, ATTINY861, ATTINY861A, ATTINY1634,

ATTINY2313, ATTINY2313A, ATTINY4313, ATMEGA48,

ATMEGA8, ATMEGA8515, ATMEGA8535, ATMEGA88,

ATMEGA8U2, ATMEGA16U2, ATMEGA32U2,

ATMEGA16U4, ATMEGA32U4, ATMEGA32U6, AT90USB646,

AT90USB647, AT90USB1286, AT90USB1287, AT43USB355,

ATMEGA16, ATMEGA161, ATMEGA162, ATMEGA163,

ATMEGA164, ATMEGA165, ATMEGA168, ATMEGA169,

ATMEGA32, ATMEGA323, ATMEGA324, ATMEGA325,

ATMEGA3250, ATMEGA328, ATMEGA329, ATMEGA3290,

ATMEGA406, ATMEGA64, ATMEGA640, ATMEGA644,

ATMEGA644RFR2, ATMEGA645, ATMEGA6450,

ATMEGA649, ATMEGA6490, ATMEGA103, ATMEGA128,

ATMEGA1280, ATMEGA1281, ATMEGA1284,

ATMEGA1284RFR2, ATMEGA2560, ATMEGA2561

The  various AVR chip  variants mainly differ  in the amount of on-chip

memory  (flash,  SRAM,  EEPROM)  an  the  set  of  built-in peripherals

(GPIO,  timers,  UART,  A/D  converter...).  Compared  to  the  AT90...

predecessors,  the  ATmega  chip  also provide additional instructions,

while the ATtinys do not support the multiplication instructions.

am) AM29245 -> AM29243 -> AM29240 -> AM29000

The  further  one  moves  to  the  right  in  this  list, the fewer the

instructions  become that have  to be emulated  in software. While e.g.

the  29245 not  even owns  a hardware  multiplier, the two representors

in  the middle  only lack  the floating  point instructions.  The 29000

serves  as  a  'generic'  type  that  understands  all  instructions in

hardware.

an) 80C166 -> 80C167,80C165,80C163

80C167  and 80C165/163  have an  address space  of 16 Mbytes instead of

256  Kbytes,  and  furthermore  they  know some additional instructions

for  extended addressing  modes and  atomic instruction sequences. They

are  'second generation' processors and differ  from each other only in

the amount of on-chip peripherals.

ao) LR35902/GBZ80 -> Z80 -> Z80UNDOC

-> Z180

-> eZ80190, eZ80L92, eZ80F91,

eZ80F92, eZ80F93,

-> Z380

While  there are only  a few additional  instructions for the Z180, the

Z380  owns  32-bit  registers,  a  linear  address space of 4 Gbytes, a

couple   of   instruction   set   extensions   that  make  the  overall

instruction  set  considerably  more  orthogonal,  and  new  addressing

modes  (referring  to  index  register  halves,  stack relative). These

extensions   partially  already  exist  on   the  Z80  as  undocumented

extensions  and may  be switched  on via  the Z80UNDOC  variant. A list

with  the  additional  instructions  can  be  found in the chapter with

processor specific hints.

The  processor built  into the  Gameboy (official  designation LR35092,

commonly  referred to as ''Gameboy  Z80'') is a mixture  of an 8080 and

Z80.  It lacks the  IX/IY registers, the  I/O address space, the second

register bank and a couple of 16 bit instructions.

The  Zilog eZ80  variants extend  the Z80  architecture with a 16 MByte

address   space,  24  bit  registers  and  moderate  additions  to  the

instruction  set. Some  variants feature  an I  register that is only 8

bits  wide,  and  the  eZ80190  additionally  lacks  a  few  string I/O

instructios.  Otherwise,  they  only  differ  in  the amonut of on-chip

memory and peripherals.

ap) Z8601, Z8603, z86C03, z86E03, Z86C06, Z86E06,

Z86C08, Z86C21, Z86E21, Z86C30, Z86C31, Z86C32 Z86C40

-> Z88C00, Z88C01

-> eZ8, Z8F0113, Z8F011A, Z8F0123, Z8F012A,

Z8F0130, Z8F0131, Z8F0213, Z8F021A, Z8F0223, Z8F022A,

Z8F0230, Z8F0231, Z8F0411, Z8F0412, Z8F0413, Z8F041A,

Z8F0421, Z8F0422, Z8F0423, Z8F042A, Z8F0430, Z8F0431,

Z8F0811, Z8F0812, Z8F0813, Z8F081A, Z8F0821, Z8F0822,

Z8F0823, Z8F082A, Z8F0830, Z8F0831, Z8F0880, Z8F1232,

Z8F1233, Z8F1621, Z8F1622, Z8F1680, Z8F1681, Z8F1682,

Z8F2421, Z8F2422, Z8F2480, Z8F3221, Z8F3222, Z8F3281,

Z8F3282, Z8F4821, Z8F4822, Z8F4823, Z8F6081, Z8F6082,

Z8F6421, Z8F6422, Z8F6423, Z8F6481, Z8F6482

The  variants with Z8 core only differ  in internal memory size and on-

chip  peripherals,  i.e.  the  choice  does  not  have an effect on the

supported   instruction   set.   Super8   and   eZ8  are  substantially

different,  each  with  an  instruction  set  that  was vastly extended

(into  different directions), and they  are not fully upward-compatible

on source code level as well.

aq) Z8001, Z8002, Z8003, Z8004

The  operation mode (segmented  for Z8001 and  Z8003, non-segmented for

Z8002  and  Z8004)  is  selected  via  the  processor  type.  There  is

currently  no  further  differentiation  between  Z8001/8002 and Z8003/

8004.

ar) KCPSM, KCPSM3

Both  processor cores are not  available as standalone components, they

are  provided as  logic cores  for gate  arrays made  by Xilinx  The -3

variant   offers   a   larger   address   space   and  some  additional

instructions. Note that it is not binary upward-compatible!

as) MICO8_05, MICO8_V3, MICO8_V31

Lattice  unfortunately  changed  the  machine  instructions  more  than

once,  so  different  targets  became  necessary  to  provide continued

support  for older projects. The first  variant is the one described in

the 2005 manual, the two other ones represent versions 3.0 resp. 3.1.

at) 96C141, 93C141

These  two  processors  represent  the  two variations of the processor

family:   TLCS-900  and   TLCS-900L.  The   differences  of  these  two

variations will be discussed in detail in section 4.34.

au) 90C141

av) 87C00, 87C20, 87C40, 87C70

The  processors of the TLCS-870 series  have an identical CPU core, but

different  peripherals depending  on the  type. In  part registers with

the   same  name   are  located   at  different   addresses.  The  file

STDDEF87.INC  uses,  similar  to  the  MCS-51-family,  the  distinction

possible   by  different  types  to  provide  the  correct  symbol  set

automatically.

aw) TLCS-870/C

Currently,  only  the  processor  core  of  the  TLCS-870/C  family  is

implemented.

ax) 47C00 -> 470C00 -> 470AC00

These  three variations of the TLCS-47-family  have on-chip RAM and ROM

of  different size, which leads  to several bank switching instructions

being added or suppressed.

ay) 97C241

az) TC9331

ba) 16C54 -> 16C55 -> 16C56 -> 16C57

These  processors  differ  by  the  available  code  area,  i.e. by the

address limit after which AS reports overruns.

bb) 16C84, 16C64

Analog  to  the  MCS-51  family,  no  distinction  is  made in the code

generator,  the  different  numbers  only  serve to include the correct

SFRs in STDDEF18.INC.

bc) 17C42

bd) SX20, SX28

The SX20 uses a smaller housing and lacks port C.

be) ST6200, ST6201, ST6203, ST6208, ST6209,

ST6210, ST6215, ST6218, ST6220, ST6225,

ST6228, ST6230, ST6232, ST6235, ST6240,

ST6242, ST6245, ST6246, ST6252, ST6253,

ST6255, ST6260, ST6262, ST6263, ST6265,

ST6280, ST6285

The  various ST6 derivates differ in  the amount of on-chip peripherals

and built-in memory.

bf) ST7

ST72251G1, ST72251G2, ST72311J2, ST72311J4,

ST72321BR6, ST72321BR7, ST72321BR9, ST72325S4,

ST72325S6, ST72325J7, ST72325R9, ST72324J6,

ST72324K6, ST72324J4, ST72324K4, ST72324J2,

ST72324JK21, ST72325S4, ST72325J7, ST72325R9,

ST72521BR6, ST72521BM9, ST7232AK1, ST7232AK2,

ST7232AJ1, ST7232AJ2, ST72361AR4, ST72361AR6,

ST72361AR7, ST72361AR9, ST7FOXK1, ST7FOXK2,

ST7LITES2Y0, ST7LITES5Y0, ST7LITE02Y0,

ST7LITE05Y0, ST7LITE09Y0

ST7LITE10F1, ST7LITE15F1, ST7LITE19F1,

ST7LITE10F0, ST7LITE15F0, ST7LITE15F1,

ST7LITE19F0, ST7LITE19F1,

ST7LITE20F2, ST7LITE25F2, ST7LITE29F2,

ST7LITE30F2, ST7LITE35F2, ST7LITE39F2,

ST7LITE49K2,

ST7MC1K2, ST7MC1K4, ST7MC2N6, ST7MC2S4,

ST7MC2S6, ST7MC2S7, ST7MC2S9, ST7MC2R6,

ST7MC2R7, ST7MC2R9, ST7MC2M9,

STM8

STM8S001J3, STM8S003F3, STM8S003K3, STM8S005C6,

STM8S005K6, STM8S007C8, STM8S103F2, STM8S103F3,

STM8S103K3, STM8S105C4, STM8S105C6, STM8S105K4,

STM8S105K6, STM8S105S4, STM8S105S6, STM8S207MB,

STM8S207M8, STM8S207RB, STM8S207R8, STM8S207R6,

STM8S207CB, STM8S207C8, STM8S207C6, STM8S207SB,

STM8S207S8, STM8S207S6, STM8S207K8, STM8S207K6,

STM8S208MB, STM8S208RB, STM8S208R8, STM8S208R6,

STM8S208CB, STM8S208C8, STM8S208C6, STM8S208SB,

STM8S208S8, STM8S208S6, STM8S903K3, STM8S903F3,

STM8L050J3, STM8L051F3, STM8L052C6, STM8L052R8,

STM8L001J3, STM8L101F1, STM8L101F2, STM8L101G2,

STM8L101F3, STM8L101G3, STM8L101K3, STM8L151C2,

STM8L151K2, STM8L151G2, STM8L151F2, STM8L151C3,

STM8L151K3, STM8L151G3, STM8L151F3, STM8L151C4,

STM8L151C6, STM8L151K4, STM8L151K6, STM8L151G4,

STM8L151G6, STM8L152C4, STM8L152C6, STM8L152K4,

STM8L152K6, STM8L151R6, STM8L151C8, STM8L151M8,

STM8L151R8, STM8L152R6, STM8L152C8, STM8L152K8,

STM8L152M8, STM8L152R8, STM8L162M8, STM8L162R8,

STM8AF6366, STM8AF6388, STM8AF6213, STM8AF6223,

STM8AF6226, STM8AF6246, STM8AF6248, STM8AF6266,

STM8AF6268, STM8AF6269, STM8AF6286, STM8AF6288,

STM8AF6289, STM8AF628A, STM8AF62A6, STM8AF62A8,

STM8AF62A9, STM8AF62AA, STM8AF5268, STM8AF5269,

STM8AF5286, STM8AF5288, STM8AF5289, STM8AF528A,

STM8AF52A6, STM8AF52A8, STM8AF52A9, STM8AF52AA,

STM8AL3136, STM8AL3138, STM8AL3146, STM8AL3148,

STM8AL3166, STM8AL3168, STM8AL3L46, STM8AL3L48,

STM8AL3L66, STM8AL3L68, STM8AL3188, STM8AL3189,

STM8AL318A, STM8AL3L88, STM8AL3L89, STM8AL3L8A,

STM8TL52F4, STM8TL52G4, STM8TL53C4, STM8TL53F4,

STM8TL53G4

The  STM8 core extends the address space  to 16 Mbytes and introduces a

couple  of  new  instructions.  Though  many instructions have the same

machine code as for ST7, it is not binary upward compatible.

bg) ST9020, ST9030, ST9040, ST9050

These  4 names represent  the four ''sub-families''  of the ST9 family,

which  only differ in their  on-chip peripherals. Their processor cores

are  identical, which  is why  this distinction  is again  only used in

the include file containing the peripheral addresses.

bh) 6804

bi) 32010->32015

The  TMS32010 owns  just 144  bytes of  internal RAM,  and so AS limits

addresses   in  the  data   segment  just  up   to  this  amount.  This

restriction  does not apply  for the 32015,  the full range from 0..255

can be used.

bj) 320C25 -> 320C26 -> 320C28

These  processors  only  differ  slightly  in their on-chip peripherals

and in their configuration instructions.

bk) 320C30, 320C31 -> 320C40, 320C44

The  320C31  is  a  reduced  version  with  the  same  instruction set,

however   fewer   peripherals.   The   distinction   is   exploited  in

STDDEF3X.INC.  The C4x  variants are  sourcecode upward compatible, the

machine  codes  of  some  instructions  are however slightly different.

Once  again, the C44 is  a stripped-down version of  the C40, with less

peripherals and a smaller address space.

bl) 320C203 -> 320C50, 320C51, 320C53

The  first one  represents the  C20x family  of signal processors which

implement  a subset of  the C5x instruction  set. The distinction among

the C5x processors is currently not used by AS.

bm) 320C541

This one at the moment represents the TMS320C54x family...

bn) TI990/4, TI990/10, TI990/12

TMS9900, TMS9940, TMS9995, TMS99105, TMS99110

The    TMS99xx/99xxx    processors    are    basically    single   chip

implementations  of  the  TI990  minicomputers.  Some  TI990 models are

even  based  on  such  a  processor  instead  of  a  discrete  CPU. The

individual  models differ  in their  instruction set  (the TI990/12 has

the largest one) and the presence of a privileged mode.

bo) TMS70C00, TMS70C20, TMS70C40,

TMS70CT20, TMS70CT40,

TMS70C02, TMS70C42, TMS70C82,

TMS70C08, TMS70C48

All  members of this family share the  same CPU core, they therefore do

not  differ in their instruction set.  The differences manifest only in

the  file  REG7000.INC  where  address  ranges and peripheral addresses

are  defined. Types  listed in  the same  row have  the same  amount of

internal  RAM and  the same  on-chip peripherals,  they differ  only in

the amount of integrated ROM.

bp) 370C010, 370C020, 370C030, 370C040 and 370C050

Similar  to the  MCS-51 family,  the different  types are  only used to

differentiate   the   peripheral   equipment   in   STDDEF37.INC;   the

instruction set is always the same.

bq) MSP430 -> MSP430X

The  X  variant  of  the  CPU  core  extends  the address space from 64

KiBytes  to  1  MiByte  and  augments  the  instruction  set,  e.g.  by

prefixed to repeat instructions.

br) TMS1000, TMS1100, TMS1200, TMS1300

TMS1000  and TMS1200  each provide  1 KByte  of ROM  and 64  nibbles of

RAM,  while TMS1100  and TMS1300  provide twice  the amount  of RAM and

ROM.  Furthermore,  TI  has  defined  a significantly different default

instruction  set  fot  TMS1100  and  TMS1300(AS  only knows the default

instruction sets!)

bs) IMP-16C/200, IMP-16C/300, IMP-16P/200, IMP-16P/300, IMP-16L

The  IMP-16L  defines  a  few  additional  bits in its status register,

plus  more branch conditions. It  supports the extended instruction set

just like the /300 variants.

bt) IPC-16, INS8900

The  INS8900 is just a re-implementation of  PACE in a more modern NMOS

manufacturing process; there are no differences in instruction set.

bu) SC/MP

bv) 8070

This  processor  represents  the  whole  807x family (which consists at

least  of the  8070, 8072,  and 8073),  which however  shares identical

CPU cores.

bw) COP87L84

This  is the only  member of National  Semiconductor's COP8 family that

is  currently  supported.  I  know  that  the  family  is substantially

larger   and  that  there  are   representors  with  differently  large

instruction  sets  which  will  be  added  when  a need occurs. It is a

beginning, and National's documentation is quite extensive...

bx) COP410 -> COP420 -> COP440 -> COP444

The  COP42x derivates  offer some  additional instructions,  plus other

instructions have an extended operand range.

by) SC14400, SC14401, SC14402, SC14404, SC14405,

SC14420, SC14421, SC14422, SC14424

This  series of  DECT controllers  differentiates itself  by the amount

of  instructions,  since  each  of  them  supports  different  B  field

formats and their architecture has been optimized over time.

bz) NS16008, NS32008, NS08032, NS16032, NS32016, NS32032,

NS32332, NS32CG16, NS32532

National  renamed the first-generation CPUs  several times in the early

years,  NS16008/NS32008/NS08032  resp.  NS16032/NS32016  are  the  same

chips.  NS32332  and  NS32532  support  an  address  space  of 4 GBytes

instead  of 16  MBytes, and  the NS32CG16  is an  embedded variant with

additional instructions for bit block transfers.

ca) ACE1101, ACE1202

cb) F3850, MK3850,

MK3870, MK3870/10, MK3870/12, "MK3870/20, MK3870/22,

MK3870/30, MK3870/32, MK3870/40, MK3870/42,

MK3872, MK3873, MK3873/10, MK3873/12, MK3873/20,

MK3873/22, MK3874, MK3875, MK3875/22, MK3875/42,

MK3876, MK38P70/02, MK38C70, MK38C70/10,

MK38C70/20, MK97400, MK97410, MK97500, MK97501,

MK97503

This  huge  amount  of  variants  partially  results from the fact that

Mostek  renamed some variants  in the early  80s. The new naming scheme

allows  to  deduce  the  amount  of  internal  ROM  (0  to 4 for 0 to 4

Kbytes)  and  executable  RAM  (0  or  2  for  0  or 64 bytes) from the

suffix.  3850 and MK975xx support a 64K  address space, which is only 4

Kbytes  for all  other variants.  P variants  have an  EEPROM piggyback

socket  for prototyping, C  variants are fabricated  in CMOS technology

and  feature two new  machine instructions (HET  and HAL). The MK3873's

feature  is a  built-in serial  port, while  the MK3875 offers a second

supply voltage pin to buffer the internal memory in standby mode.

cc) 7800, 7801, 7802

78C05, 78C06

7807, 7808, 7809

7810->78C10, 78C11, 78C12, 78C14, 78C17, 78C18

�PD7800  to �PD7802  represent the  ''first generation''  of the uCOM87

family  from  NEC.  �PD78C05  and  �PD78C06  are  reduced variants that

implement   only  a  subset  of  the  instruction  set.  7807  to  7809

represent   the  uCOM87  series,  whose   instruction  set  was  vastly

extended.  All �PD781x  variants belong  to the  uCOM87AD series, which

adds  an A/D converter - however,  instructions for bit processing were

again  removed. NOTE: The instruction set  is in general only partially

binary  upward compatible! The  NMOS version �PD7810  has no stop-mode;

the  respective  command  and  the  ZCM  register are omitted. CAUTION!

NMOS  and CMOS  version partially  differ in  the reset  values of some

registers!

cd) uPD550, uPD554, uPD652,

uPD547, uPD552, uPD651,

uPD546, uPD553, uPD556, uPD557, uPD650

These  three groups of controllers belong  to the �COM-45, �COM-44, amd

�COM-43  family. The first two families implement a subset of the �COM-

43  instruction set. Otherwise,  the chips differ  by the amount of on-

chip ROM and RAM.

ce) 7500 <-> 7508

There  are two different types of CPU  cores in the �PD75xx family: the

7566  represents  the  the  'instruction  set  B',  which provides less

instructions,  less  registers  and  smaller  address  spaces. The 7508

represents  the 'full'  instruction set  A. CAUTION!  These instruction

sets are not 100% binary compatible!

cf) 75402,

75004, 75006, 75008,

75268,

75304, 75306, 75308, 75312, 75316,

75328,

75104, 75106, 75108, 75112, 75116,

75206, 75208, 75212, 75216,

75512, 75516

This  'cornucopia' of  processors differs  only by  the RAM size in one

group;   the   groups   themselves   again   differ  by  their  on-chip

peripherals  on the  one hand  and by  their instruction set's power on

the other hand.

cg) 78070

This  is currently the only  member of NEC's 78K0  family I am familiar

with. Similar remarks like for the COP8 family apply!

ch) 78214

This is currently the representor of NEC's 78K2 family.

ci) 78310

This is currently the representor of NEC's 78K3 family.

cj) 784026

This is currently the representor of NEC's 78K4 family.

ck) 7720 -> 7725

The  �PD7725  offers  larger  address  spaces and som more instructions

compared  to his  predecessor. CAUTION!  The processors  are not binary

compatible to each other!

cl) 77230

cm) 70616

This is currently the representor of NEC's V60 family.

cn) SYM53C810, SYM53C860, SYM53C815, SYM53C825,

SYM53C875, SYM53C895

The  simpler  members  of  this  family  of  SCSI  processors lack some

instruction  variants, furthermore they  are different in  their set of

internal registers.

co) MB89190

This processor type represents Fujitsu's F^2MC8L series...

cp) MB9500

...just  like this one  does it currently  for the 16-bit variants from

Fujitsu!

cq) MSM5840, MSM5842, MSM58421, MSM58422, MSM5847

These  variants of the  OLMS-40 family differ  in their instruction set

and in the amount of internal program and data memory.

cr) MSM5054, MSM5055, MSM5056, MSM6051, MSM6052

The  as for the OLMS-40 family:  differences in instruction set and the

amount of internal program and data memory.

cs) MN1610[ALT] -> MN1613[ALT]

In  addition to its predecessor's features,  the MN1613 offers a larger

address  space,  a  floating  point  unit  and  a couple of new machine

instructions.

ct) RXV1, RX110, RX111, RX113, RX130, RX210,

RX21A, RX220, RX610, RX621, RX62N, RX630,

RX631 -->

RXV2, RX140, RX230, RX231, RX64M,

RX651 -->

RXV3, RX660, RX671, RX72M, RX72N

Controllers  of the  RX series  can coarsely  be classified  into three

groups  or  generations.  From  generation  to  generation (RXv1, RXv2,

RXv3), new machine instructions were added.

cu) PMC150, PMS150, PFS154, PMC131, PMS130, PMS131

PMS132, PMS132B, PMS152, PMS154B, PMS154C, PFS173

PMS133, PMS134, DF69, MCS11, PMC232, PMC234, PMC251

PMC271,PMC884, PMS232, PMS234, PMS271

The  Padauk controllers  differ in  the size  of the internal (ROM/RAM)

memory,  the  type  of  internal  ROM  (erasable  or OTP), the built-in

peripherals,   and  their  instruction  set  (both  extent  and  binary

coding).

cv) 1802 -> 1804, 1805, 1806 -> 1804A, 1805A, 1806A

1804,  1805,  and  1806  feature  an  instruction  set that is slightly

enhanced,  compared to  the 'original'  1802, plus  on-chip RAM  and an

integrated  timer. The A  variants extend the  instruction set by DSAV,

DBNZ, and instructions for addition and subtraction in BCD format.

cw) XS1

This type represents the XCore-"family".

cx) 1750

MIL  STD 1750  is a  standard, therefore  there is  only one (standard)

variant...

cy) KENBAK

Since there has never been a KENBAK-2, the target is simply KENBAK...

cz) CP-1600

da) HPNANO

db) 6100 -> 6120

The  IM6120 supports  a larger  address space  (32K instead  of 4K) and

additional machine instructions.

dc) SC61860

This  is  the  processor  used  in  the  Sharp PC-12xx...PC-15xx pocket

computers.

dd) SC62015

This is the processor used in the Sharp PC-E500.

NONE  is a special target  that has not been  mentioned so far. This is

the  default target if no  target has been defined  on the command line

via  -cpu, and if no CPU statement  has been encountered so far. target

independent  pseudo instructions are still  possible in this situation,

however  it is  not possible  to create  any code,  neither via machine

instructions,  nor  via  placing  data  in  memory. In principle, it is

also  possible to  explicitly select  this target  via -cpu or CPU. The

practical use of this is however limited.

The  CPU instruction  needs the  processor type  as a simple literal, a

calculation like:

CPU     68010+10

is not allowed. Valid calls are e.g.

CPU     8051

or

CPU     6800

Regardless  of the processor  type currently set,  the integer variable

MOMCPU  contains  the  current  status  as  a  hexadecimal  number. For

example,  MOMCPU=$68010 for the  68010 or MOMCPU=80C48H  for the 80C48.

As  one cannot  express all  letters as  hexadecimal digits  (only A..F

are  possible),  all  other  letters  must  must  be omitted in the hex

notation; for example, MOMCPU=80H for the Z80.

You  can  take  advantage  of  this  feature to generate different code

depending  on the processor type. For  example, the 68000 does not have

a  machine instruction for  a subroutine return  with stack correction.

With  the variable MOMCPU you can define  a macro that uses the machine

instruction or emulates it depending on the processor type:

myrtd   macro   disp

if      MOMCPU<$68010 ; emulate for 68008 & 68000

move.l (sp),disp(sp)

lea    disp(sp),sp

rts

elseif

rtd    #disp         ; direct use on >=68010

endif

endm

cpu     68010

myrtd   12            ; results in RTD #12

cpu     68000

myrtd   12            ; results in MOVE../LEA../RTS

As  not all processor names  are built only out  of numbers and letters

from  A..F,  the  full  name  is  additionally  stored  in  the  string

variable named MOMCPUNAME.

The  assembler implicitly switches back to  the CODE segment when a CPU

instruction  is  executed.  This  is  done  because  CODE  is  the only

segment all processors support.

Note  that 68008 is  no longer the  default target. If  no -cpu command

line  argument has been given, the target  is set to the reserved value

NONE   up  to  the  first   CPU  statement.  Target-independent  pseudo

instructions  are  still  allowed  in  this  situation,  like  defining

constants  or macros. It is however  not possible to generate any code,

neither by machine instructions nor by disposing data in memory.

Some  targets define  options or  variants that  are so fundamental for

operation,  that they  have to  be selected  with the  CPU instruction.

Such  options  are  appended  to  the  argument,  separated  by  double

colons:

CPU <CPU Name>:<var1>=<val1>:<var2>=<val2>:...

See  the  respective  section  with  processor-specific  hints to check

whether a certain target supports such options.

3.2.4. SUPMODE, FPU, PMMU, CUSTOM

- - - - - - - - - - - - - - - - -

- SUPMODE   valid  for:  680x0,   NS32xxx,  PDP-11,  i960,  TLCS-900,

SH7000, i960, 29K, XA, PowerPC, M*Core, V60, and TMS9900

- FPU valid for: 680x0, NS32xxx, 80x86

- PMMU valid for: 680x0, NS32xxx

- CUSTOM valid for: NS32xxx

These  three switches  allow to  define which  parts of the instruction

set  shall  be  disabled  because  the  necessary preconditions are not

valid  for  the  following  piece  of  code.  The  parameter  for these

instructions  may be either ON  or OFF, the current  status can be read

out of a variable which is either TRUE or FALSE.

The commands have the following meanings in detail:

- SUPMODE:  allows  or  prohibits  commands,  for whose execution the

processor  has  to  be  within  the  supervisor  mode.  The  status

variable is called INSUPMODE.

- FPU:   allows   or   prohibits   the   commands  of  the  numerical

coprocessors  8087, NS32081/32381 resp. 68881  or 68882. The status

variable  is  called  FPUAVAIL.  For  NS32xxx as target, specifying

the  explicit FPU type  (NS32081, NS32181, NS32381,  or NS32580) is

also  possible, to enable  or disable the  additional registers and

instructions.

- PMMU:  allows or  prohibits the  commands of  the memory management

unit  68851 resp.  of the  built-in MMU  of the 68030. CAUTION! The

68030-MMU  supports  only  a  relatively  small subset of the 68851

instructions.  This is  controlled via  the FULLPMMU statement. The

status  variable  is  called  PMMUAVAIL.  For  NS32xxx  as  target,

specifying  the explicit MMU  type as target  (NS32082, NS32381, or

NS32352)  is  also  possible,  to  enable  access  to the MMU-type-

specific register set.

- CUSTOM:  allows  or  prohibits  the  commands  reserved  for custom

slave processors.

The  usage of of  instructions prohibited in  this manner will generate

a warning at SUPMODE, at PMMU and FPU a real error message.

3.2.5.  ACCMODE

- - - - - - - -

- valid for: VAX

The  VAX not only  knows a user  and supervisor mode,  it supports four

privilege  levels of this  type. Going from  more to less rights, these

are   named  Kernel,  Executive,  Supervisor,  and  User.  The  ACCMODE

instruction  informs the assembler about  the access mode the following

code  is executed within.  Not all machine  instructions are allowed in

all  modes.  Valid  arguments  are  either  the  mode  names  mentioned

before,  or a number  between zero (kernel  mode) to three (user mode).

As  default, user mode is assumed,  and the current setting (as numeric

value) may be read from a symbol of same name.

3.2.6. CIS, EIS, FIS and FP11

- - - - - - - - - - - - - - -

- valid for: PDP-11

These  statements enable or disable  the availibility of certain PDP-11

instruction   set  extensions.  For  one  of  these  statements  to  be

available,  the  respective  instructions  must  not  be  part  of  the

machine's  basic instruction set,  and there must  have been an upgrade

option to add them. In detail:

-  CIS:   ,,Commercial  Instruction   Set'',  i.e.   instructions  to

operate   on  packed  and  non-packed  BCD  numbers  with  variable

length.  They were  available as  an option  on the  LSI-11 and the

PDP-11/44.

-  EIS:  The  instructions  MUL,  DIV,  ASH  und ASHC, which were not

part  of  the  base  instruction  set  on  older  or smaller PDP-11

systems.  They were available as an  option on the LSI-11 resp. the

PDP-11/35 and PDP-11/40.

-  FIS:   Stack   oriented   instructions   implementing   the  basic

mathematic  operations on  floating point  numbers in  F format (32

bits).  They were  available as  an option  on the LSI-11 resp. the

PDP-11/35 and PDP-11/40.

-  FP11:  Full floating point support  with separate FPU registers in

F and D format (32/64 bits).

3.2.7. FULLPMMU

- - - - - - - -

valid for: 680x0

Motorola  integrated  the  MMU  into  the  processor  starting with the

68030,  but the built-in  FPU is equipped  only with a relatively small

subset  of the  68851 instruction  set. AS  will therefore  disable all

extended  MMU  instructions  when  the  target  processor  is  68030 or

higher.  It  is  however  possible  that  the  internal  MMU  has  been

disabled  in a  68030-based system  and the  processor operates with an

external  68851. One  can the  use a  FULLPMMU ON  to tell  AS that the

complete  MMU instruction  set is  allowed. Vice  versa, one  may use a

FULLPMMU  OFF  to  disable  all  additional  instruction  in spite of a

68020  target platform  to assure  that portable  code is  written. The

switch  between full and  reduced instruction set  may be done as often

as  needed, and the current setting may  be read from a symbol with the

same  name.  CAUTION!  The  CPU  instruction  implicitly sets or resets

this   switch  when  its  argument   is  a  68xxx  processor!  FULLPMMU

therefore has to be written after the CPU instruction!

3.2.8.  PADDING

- - - - - - - -

valid for: 680x0, 68xx, M*Core, XA, H8, SH7000, MSP430(X), TMS9900,

ST7/STM8, AVR (only if code segment granularity is 8 bits)

Various  processor  families  have  a  requirement that objects of more

than  one byte length must  be located on a  n even address. Aside from

data  objects, this may  also include instruction  words. For instance,

word  accesses to  an odd  address result  in an  exception on a 68000,

while  other processors  like the  H8 force  the lowest  address bit to

zero.

The  PADDING instruction allows  to activate a  mechanism that tries to

avoid  such misalignments. If the  situation arises that an instruction

word,  or a data object of 16 bits  or more (created e.g. via DC) would

be  stored on an odd address,  a padding byte is automatically inserted

before.  Such a padding byte is displayed  in the listing in a separate

line that contains the remark

<padding>

If  the source line also  contained a label, the  label still points to

the  address of  the code  or data  object, i.e.  right behind  the pad

byte.  The  same  is  true  for  a  label  in a source line immediately

before,  as  long  as  this  line  only  holds  the  label and no other

instruction. So, in the follwing example:

padding  on

org      $1000

dc.b     1

adr1:  nop

dc.b     1

adr2:

nop

dc.b     1

adr3:  equ      *

nop

the  labels  adr1  and  adr2  hold  the addresses of the respective NOP

instructions,  which were  made even  by inserting  a pad byte. adr3 in

contrast holds the address of the pad byte preceding the third NOP.

Similar  to the previous  instructions, the argument  to PADDING may be

either  ON or OFF,  and the current  setting may be  read from a symbol

with  the same name. PADDING  is by default only  enabled for the 680x0

family, it has to be turned on explicitly for all other families.

3.2.9.  PACKING

- - - - - - - -

valid  for: 56000, AVR, TMS3203x/4x, TMS3206x, MN1610, CP1600, �PD7720/

7725, �PD77230

In  some way,  PACKING is  similar to  PADDING, it  just has a somewhat

opposite  effect: While PADDING  extends the disposed  data to get full

words  and keep a  possible alignment, PACKING  squeezes several values

into  a single word. This makes sense  for the AVR's code segment since

the  CPU  has  a  special  instruction  (  LPM)  to access single bytes

within  a 16-bit word. In case this  option is turned on (argument ON),

two  byte values are packed into a  single word by DATA, similar to the

single  characters  of  string  arguments.  The  value  range of course

reduces  to -128...+255. If  this option is  turned off (argument OFF),

each  integer argument obtains its own word  and may take values from -

32768...+65535.

This  distinction is only  made for integer  arguments of DATA, strings

will  always be  packed.. Keep  further in  mind that packing of values

only  works  within  the  arguments  of  a  DATA  statement; if one has

subsequent  DATA  statements,  there  will  still  be half-filled words

when the argument count is odd!

3.2.10.  WARNRELATIVE

- - - - - - - - - - -

valid for: Zx80

This  switch instructs the  assembler whether to  issue warnings when a

relative  jump  instead  of  an  absolute  one  would  be possible. The

default  is  OFF  respectively  what  was  defined via the command line

arguments -wrelative and -wno-relative.

The current setting may be read from a symbol of same name.

3.2.11. MAXMODE

- - - - - - - -

valid for: TLCS-900, H8

The  processors of  the TLCS-900-family  are able  to work  in 2 modes,

the  minimum  and  maximum  mode.  Depending  on  the  actual mode, the

execution  environment and  the assembler  are a  little bit different.

Along  with  this  instruction  and  the  parameter  ON  or  OFF, AS is

informed  that the  following code  will run  in maximum  resp. minimum

mode.  The  actual  setting  can  be  read from the variable INMAXMODE.

Presetting is OFF, i.e. minimum mode.

Similarly,  one uses  this instruction  to tell  AS in  H8 mode whether

the  address space is 64K or 16  Mbytes. This setting is always OFF for

the 'small' 300 version and cannot be changed.

3.2.12. EXTMODE and LWORDMODE

- - - - - - - - - - - - - - -

valid for: Z380

The  Z380 may operate  in altogether 4  modes, which are  the result of

setting  two  flags:  The  XM  flag  rules  whether the processor shall

operate  wit an address space of 64 Kbytes  or 4 Gbytes and it may only

be  set to 1 (after a reset, it  is set to 0 for compatibility with the

Z80).  The LW  flag in  turn rules  whether word  operations shall work

with  a word  size of  16 or  32 bits.  The setting  of these two flags

influences  range checks of  constants and addresses,  which is why one

has  to tell AS the setting of  these two flags via these instructions.

The  default assumption is  that both flags  are 0, the current setting

(ON  or OFF)  may be  read from  the predefined symbols INEXTMODE resp.

INLWORDMODE.

3.2.13. SRCMODE

- - - - - - - -

valid for: MCS-251

Intel   substantially  extended  the  8051  instruction  set  with  the

80C251,  but unfortunately there was only  a single free opcode for all

these  new instructions.  To avoid  a processor  that will be eternally

crippled  by a prefix,  Intel provided two  operating modes: the binary

and  the source mode.  The new processor  is fully binary compatible to

the  8051 in binary mode, all  new instructions require the free opcode

as  prefix. In source mode, the  new instructions exchange their places

in  the code tables with the  corresponding 8051 instructions, which in

turn  then need a  prefix. One has  to inform AS  whether the processor

operates  in source mode (ON) or binary  mode (OFF) to enable AS to add

prefixes  when  required.  The  current  setting  may  be read from the

variable INSRCMODE. The default is OFF.

3.2.14. PLAINBASE

- - - - - - - - -

valid for: 6809

Historically,  AS allows  to omit  an empty  first argument  on indexed

address expressions. An

lda  x

for instance was equivalent to

lda  ,x

Though  meant as a feature, this wa  occasionally rather seen as a bug.

Current  versios  therefore  no  longer  allow  to  omit an empty index

argument  and will instead  emit a wrong  argument count error message.

If  this feature is still  desired or needed for  existing code, it may

be enabled via a

plainbase on

statement.  The  current  setting  may  be  read  from a symbol of same

name.

3.2.15. BIGENDIAN

- - - - - - - - -

valid for: MCS-51/251, PowerPC, SC/MP, 2650, NS32000

Intel   broke  with  its  own  principles  when  the  8051  series  was

designed:  in  contrast  to  all  traditions,  the  processor uses big-

endian  ordering for  all multi-byte  values! While  this was not a big

deal  for MCS-51 processors (the processor  could access memory only in

8-bit  portions, so  everyone was  free to  use whichever endianess one

wanted),  it may be a problem for the  251 as it can fetch whole (long-

)words  from memory  and expects  the MSB  to be  first. As this is not

the  way of constant disposal earlier versions  of AS used, one can use

this  instruction to toggle between big  and little endian mode for the

instructions  DB, DW, DD,  DQ, DT, and  DO. BIGENDIAN OFF (the default)

puts  the LSB first  into memory as  it used to  be on earlier versions

of  AS, BIGENDIAN ON engages the big-endian mode compatible to the MCS-

251.  One may of course change this  setting as often as one wants; the

current setting can be read from the symbol with the same name.

The  Renesas RX  as target  also supports  a selectable  endianess. For

compatibility  to  the  original  assembler,  the  statement  is  named

ENDIAN and accepts LITTLE or BIG as argument.

3.2.16.  WRAPMODE

- - - - - - - - -

valid for: Atmel AVR

After  this  switch  has  been  set  to  ON,  AS  will  assume that the

processor's  program counter does  not have the  full length of 16 bits

given  by  the  architecture,  but  instead  a  length  that is exactly

sufficient  to address  the internal  ROM. For  example, in case of the

AT90S8515,  this means 12 bits, corresponding  to 4 Kwords or 8 Kbytes.

This  assumption allows relative  branches from the  ROM's beginning to

the  end and  vice versa  which would  result in an out-of-branch error

when  using strict arithmetics. Here, they  work because the carry bits

resulting  from the  target address  computation are  discarded. Assure

that  the  target  processor  you  are  using works in the outlined way

before   you  enable  this  option!   In  case  of  the  abovementioned

AT90S8515,  this option  is even  necessary because  it is the only way

to perform a direct jump through the complete address space...

This  switch is set to  OFF by default, and  its current setting may be

read from a symbol with same name.

3.2.17. PANEL

- - - - - - -

valid for: IM61x0

This  switch  is  used  to  inform  the assembler whether the following

code  is  executet  with  a  set  or cleared Control Panel Flip-Flop. A

couple  of IOT  instructions are  only allowed  if the  flip-flop has a

certain  state. Usage of these instructions  in the other state will be

reported as an error by the assembler.

The current setting may be read from the symbol INPANEL.

3.2.18. SEGMENT

- - - - - - - -

valid for: all processors

Some  microcontrollers  and  signal  processors  know  various  address

ranges,  which  do  not  overlap  with  each  other  and  require  also

different  instructions  and  addressing  modes  for  access. To manage

these  ones also, the assembler  provides various program counters, you

can  switch  among  them  to  and  from  by  the  use  of  the  SEGMENT

instruction.  For subroutines  included with  INCLUDE, this e.g. allows

to  define data  used by  the main  program or  subroutines near to the

place  they  are  used.  In  detail,  the  following  segments with the

following names are supported:

- CODE: program code;

- DATA: directly addressable data (including SFRs);

- XDATA:  data in externally  connected RAM or  X-addressing space of

the DSP56xxx or ROM data for the �PD772x;

- YDATA: Y-addressing space of the DSP56xxx;

- IDATA: indirectly addressable (internal) data;

- BITDATA:  the  part  of  the  8051-internal  RAM  that  is  bitwise

addressable;

- IO: I/O-address range;

- REG: register bank of the ST9;

- ROMDATA: constant ROM of the NEC signal processors;

- EEDATA: built-in EEPROM.

See  also section  3.2.1 (ORG)  for detailed  information about address

ranges  and initial values of the  segments. Depending on the processor

family, not all segment types will be permitted.

The  bit segment is managed as if it  would be a byte segment, i.e. the

addresses will be incremented by 1 per bit.

Labels  get the same type  as attribute as the  segment that was active

when  the label was defined. So the  assembler has a limited ability to

check  whether  you  access  symbols  of  a  certain segment with wrong

instructions. In such cases the assembler issues a warning.

Example:

CPU     8051    ; MCS-51-code

segment code    ; test code

setb    flag    ; no warning

setb    var     ; warning : wrong segment

segment data

var     db      ?

segment bitdata

flag    db      ?

3.2.19. PHASE and DEPHASE

- - - - - - - - - - - - -

valid for: all processors

For  some applications  (especially on  Z80 systems),  the code must be

moved  to  another  address  range  before  execution. If the assembler

didn't  know about this, it would align  all labels to the load address

(not  the start address). The programmer  is then forced to write jumps

within  this  area  either  independent  of  location or has to add the

offset  at  each  symbol  manually.  The  first one is not possible for

some  processors,  the  last  one  is  extremely  error-prone. With the

commands  PHASE and DEPHASE, it is  possible to inform the assembler at

which address the code will really be executed on the target system:

phase   <address>

informs  the assembler  that the  following code  shall be  executed at

the   specified  address.   The  assembler   calculates  thereupon  the

difference  to the  real program  counter and  adds this difference for

the following operations:

- address values in the listing

- filing of label values

- program   counter   references   in   relative  jumps  and  address

expressions

- readout of the program counter via the symbols * or $

By using the instruction

DEPHASE

,  this  ''shifting''  is  reverted  to  the value previous to the most

recent  PHASE instruction.  PHASE und  DEPHASE may  be used in a nested

manner.

The  assembler keeps  phase values  for all  defined segments, although

this instruction pair only makes real sense in the code segment.

3.2.20.  SAVE and RESTORE

- - - - - - - - - - - - -

valid for: all processors

The  command  SAVE  forces  the  assembler  to  push  the  contents  of

following variables onto an internal stack:

- currently selected processor type (set by CPU);

- currently active memory area (set by SEGMENT);

- the flag whether listing is switched on or off (set by LISTING);

- the  flags  that  define  which  part  of  expanded macros shall be

printed in the assembly listing (set by /MACEXP_DFT/MACEXP_OVR).

- currently active character translation table (set by CODEPAGE).

The  counterpart RESTORE  pops the  values saved  last from this stack.

These  two  commands  were  primarily  designed  for  include files, to

change  the  above  mentioned  variables  in  any  way  inside of these

files,  without  loosing  their  original  content. This may be helpful

e.g.  in include files with own,  fully debugged subroutines, to switch

the listing generation off:

SAVE            ; save old status

LISTING OFF     ; save paper

.               ; the actual code

.

RESTORE         ; restore

In  opposite to  a simple  LISTING OFF  .. ON-pair,  the correct status

will  be  restored,  in  case  the  listing generation was switched off

already before.

The  assembler  checks  if  the  number  of  SAVE-and  RESTORE-commands

corresponds and issues error messages in the following cases:

- RESTORE, but the internal stack is empty;

- the stack not empty at the end of a pass.

In  case the currently used target  has machine instructions named SAVE

or  RESTORE,  this  functionality  may  be  reached  via  SAVEENV resp.

RESTOREENV.  As  an  alternative,  it  is always possible to explicitly

invoke  the  pseudo  instructions  by  prepending a period (.SAVE resp.

.RESTORE).

3.2.21.  ASSUME

- - - - - - - -

valid for: various

This  instruction  allows  to  tell  AS  the current setting of certain

registers  whose contents cannot be described  with a simple ON or OFF.

These  are  typically  registers  that  influence  addressing modes and

whose  contents  are  important  to  know  for  AS in order to generate

correct  addressing. It is  important to note  that ASSUME only informs

AS  about  these,  no  machine  code  is  generated that actually loads

these values into the appropriate registers!

A  value  defined  with  ASSUME  can  be  queried  or  integrated  into

expressions  via  the  built-in  function  ASSUMEDVAL. This is the case

for  all architectures listed in  the following sub-sections except for

the 8086.

65CE02

......

The  65CE02 features  a a  register named  'B' that  is used to set the

'base  page'.  In  comparison  to  the  original  6502, this allows the

programmer  to place  the memory  page addressable  with short  (8 bit)

addresses  anywhere in the  64K address space.  This register is set to

zero  after  a  reset,  so  the  65CE02 behaves like its predecessor. A

base  page at zero is also the  default assumption of the assembler. It

may   be  informed  about  its  actual   contents  via  a  ASSUME  B:xx

statement.  Addresses located in  this page will  then automatically be

addressed via short addressing modes.

6809

....

In  contrast  to  its  'predecessors'  like  the  6800  and  6502,  the

position  of  the  direct  page,  i.e.  the  page of memory that can be

reached  with single-byte  addresses, can  be set  freely. This is done

via  the 'direct page register'  that sets the page  number. One has to

assign  a  corresponding  value  to  this  register  via  ASSUME is the

contents   are  different  from  the  default  of  0,  otherwise  wrong

addresses will be generated!

68HC11K4

........

Also  for  the  HC11,  the  designers  finally  weren't  able  to avoid

banking,  to address  more than  64 Kbytes  with only 16 address lines.

The  registers MMSIZ, MMWBR,  MM1CR, and MM2CR  control whether and how

the  additional 512K address  ranges are mapped  into the CPU's address

space.  AS initially assumes  the reset state  of these registers, i.e.

all are set to $00 and windowing is disabled.

Furthermore,  the  settings  of  the  registers CONFIG, INIT, and INIT2

may  be specified. This enables the  assembler to deduce the mapping of

I/O  registers, internal RAM  and EEPROM into  CPU address space, which

has priority over mapping of memory via windowing.

68HC12X

.......

Similar  to its cousin  without the appended  'X', the HC12X supports a

short  direct addressing mode. In this case  however, it can be used to

address  more than just the  first 256 bytes of  the address space. The

DIRECT  register specifices  which 256  byte page  of the address space

is  addressed by this  addressing mode. ASSUME  is used to  tell AS the

current  value of this register, so  it is able to automatically select

the  most efficient address ing mode  when absolute addresses are used.

The default is 0, which corresponds to the reset state.

68HC16

......

The  68HC16 employs  a set  of bank  registers to  address a space of 1

Mbyte  with its registers  that are only  16 bits wide. These registers

supply  the upper 4 bits. Of these,  the EK register is responsible for

absolute  data  accesses  (not  jumps!).  AS  checks  for each absolute

address  whether  the  upper  4  bits  of  the address are equal to the

value  of EK specified via ASSUME. AS  issues a warning if they differ.

The default for EK is 0.

H8/500

......

In  maximum mode,  the extended  address space  of these  processors is

addressed  via  a  couple  of  bank  registers. They carry the names DP

(registers  from 0..3, absolute addresses), EP  (register 4 and 5), and

TP  (stack). AS  needs the  current value  of DP  to check  if absolute

addresses  are  within  the  currently  addressable bank; the other two

registers  are only used for indirect  addressing and can therefore not

be   monitored;  it  is  a  question  of  personal  taste  whether  one

specifies  their  values  or  not.  The  BR  register  is  in  contrast

important  because it  rules which  256-byte page  may be accessed with

short  addresses.  It  is  common  for  all  registers that AS does not

assume  any default value  for them as  they are undefined  after a CPU

reset.  Everyone  who  wants  to  use absolute addresses must therefore

assign values to at least DR and DP!

MELPS740

........

Microcontrollers  of  this  series  know  a ''special page'' addressing

mode  for the  JSR instruction  that allows  a shorter coding for jumps

into  the last  page of  on-chip ROM.  The size  of this ROM depends of

course  on the  exact processor  type, and  there are  more derivatives

than  it  would  be  meaningful  to  offer via the CPU instruction...we

therefore  have to rely on  ASSUME to define the  address of this page,

e.g.

ASSUME  SP:$1f

in case the internal ROM is 8K.

MELPS7700/65816

...............

These  processors contain a  lot of registers  whose contents AS has to

know  in  order  to  generate  correct  machine  code.  These  are  the

registers in question:

+--------+----------------------+-------------+---------+

| name   | function             | value range | default |

+--------+----------------------+-------------+---------+

+--------+----------------------+-------------+---------+

| DT/DBR | data bank            | 0-$ff       | 0       |

| PG/PBR | code Bank            | 0-$ff       | 0       |

| DPR    | directly addr. page  | 0-$ffff     | 0       |

| X      | index register width | 0 or 1      | 0       |

| M      | accumulator width    | 0 or 1      | 0       |

+--------+----------------------+-------------+---------+

To  avoid endless  repetitions, see  section 4.15  for instructions how

to  use  these  registers.  The  handling  is  otherwise similar to the

8086,  i.e.  multiple  values  may  be  set with one instruction and no

code  is generated  that actually  loads the  registers with  the given

values. This is again up to the programmer!

MCS-196/296

...........

Starting  with the  80196, all  processors of  the MCS-96 family have a

register  'WSR'  that  allows  to  map  memory  areas from the extended

internal  RAM or the  SFR range into  areas of the  register file which

may  then be  accessed with  short addresses.  If one  informs AS about

the  value of the  WSR register, it  can automatically find out whether

an  absolute address  can be  addressed with  a single-byte address via

windowing;   consequently,   long   addresses   will  be  automatically

generated  for registers  covered by  windowing. The  80296 contains an

additional  register WSR1 to  allow simultaneous mapping  of two memory

areas  into the  register file.  In case  it is  possible to  address a

memory cell via both areas, AS will always choose the way via WSR!

For  indirect addressing, displacements may be  either short (8 bits, -

128  to +127) or  long (16 bits).  The assembler will automatically use

the  shortest  possible  encoding  for  a  given  displacement.  It  is

however   possible  to  enforce  a   16-bit  coding  by  prefixing  the

displacement  argument  with  a  bigger  sign ((>). Similarly, absolute

addresses  in  the  area  from  0ff80h  to  0ffffh may be reached via a

short offset relative to the "null register".

8086

....

The   8086  is  able   to  address  data   from  all  segments  in  all

instructions,  but it  however needs  so-called ''segment prefixes'' if

another  segment  register  than  DS  shall  be used. In addition it is

possible  that the DS register is  adjusted to another segment, e.g. to

address  data in the code  segment for longer parts  of the program. As

AS  cannot  analyze  the  code's  meaning,  it has to informed via this

instruction  to  what  segments  the  segment  registers  point  at the

moment, e.g.:

ASSUME  CS:CODE, DS:DATA    .

It  is possible to assign assumptions  to all four segment registers in

this  way. This  instruction produces  no code,  so the  program itself

has to do the actual load of the registers with the values.

The  usage of this instruction  has on the one  hand the result that AS

is  able to automatically put ahead  prefixes at sporadic accesses into

the  code segment, or on the other hand, one can inform AS that the DS-

register was modified and you can save explicit CS:-instructions.

Valid  arguments  behind  the  colon  are  CODE,  DATA and NOTHING. The

latter  value informs  AS that  a segment  register contains  no usable

value (for AS). The following values are preinitialized:

CS:CODE, DS:DATA, ES:NOTHING, SS:NOTHING

Z180

....

The   Z180  contains  a  built-in  MMU   whicht  maps  the  CPU  core's

''logical''  address space of 64 KBytes  to a physical address space of

512  KBytes. The precise mappig is  defined via the the registers CBAR,

CBR  and BBR. Opposed  to the 68HC11K4,  the assembler will not perform

any  automatic translations of  physical to logical  addresses; only an

internal  mapping table is kept. It  is however possible to access this

table via the phys2cpu() function.

eZ80

....

The eZ80 mode can operate in two modes:

- In  Z80 mode, the BC, DE, HL, IX,  IY, and SP registers are 16 bits

wide,  and  only  the  64K  page  defined  by the MBASE register is

addressable.  For instructions with an absolute address or a non-8-

bit immediate value as argument, two bytes are fetched.

- In  ADL mode,  the mentioned  registers are  24 bits  wide, and the

whole  16 MByte  address space  can be  addressed. For instructions

with  an  absolute  address  or  a non-8-bit immediate value, three

bytes are fetched.

Since   instruction  encoding   and  range   checking  depends  on  the

operating  mode,  the  assembler  needs  to  know  which  mode  the CPU

currently  uses.  By  using  ASSUME  to  set  ADL either to 0 or 1, the

assembler  is told about  the default operating  mode, i.e. the mode if

no  suffixes are  given. Furthermore,  the assembler  can be told about

the  current  value  of  MBASE  (0  to  0ff  hex)  as well. The default

assumption  for both values is  zero, same as to  the values set in the

CPU after a reset.

XA

..

The  XA  family  has  a  data  address  space  of  16 Mbytes, a process

however  can always address within a 64K  segment only that is given by

the  DS register. One has to inform  AS about the current value of this

register   in  order  to  enable  it  to  check  accesses  to  absolute

addresses.

29K

...

The  processors of  the 29K  family feature  a register RBP that allows

to  protect banks  of 16  registers against  access from user mode. The

corresponding  bit  has  to  be  set  to achieve the protection. ASSUME

allows  to  tell  AS  which  value  RBP currently contains. AS can warn

this  way in case  a try to  access protected registers  from user mode

is made.

80C166/167

..........

Though  none  of  the  80C166/167's  registers  is  longer than sixteen

bits,  this  processor  has  18/24  address  lines  and  can  therefore

address  up to  256Kbytes/16Mbytes. To  resolve this  contradiction, it

neither   uses  the   well-known  (and   ill-famed)  Intel   method  of

segmentation  nor does it have  inflexible bank registers...no, it uses

paging!  To accomplish this, the logical  address space of 64 Kbytes is

split  into 4  pages of  16 Kbytes,  and for  each page there is a page

register  (named DPP0..DPP3) that  rules which of  the 16/1024 physical

pages  shall  be  mapped  to  this  logical  page.  AS  always tries to

present  the address  space with  a size  of 256Kbytes/16MBytes  in the

sight  of the programmer, i.e. the  physical page is taken for absolute

accesses  and  the  setting  of  bits  14/15  of the logical address is

deduced.  If no page register fits, a  warning is issued. AS assumes by

default  that the  four registers  linearly map  the first 64 Kbytes of

memory, in the following style:

ASSUME  DPP0:0,DPP1:1,DPP2:2,DPP3:3

The  80C167 knows  some additional  instructions that  can override the

page  registers'  function.  The  chapter with processor-specific hints

describes how these instructions influence the address generation.

Some  machine instructions  have a  shortened form  that can be used if

the argument is within a certain range:

- MOV Rn,#<0..15>

- ADD/ADDC/SUB/SUBC/CMP/XOR/AND/OR Rn, #<0..7>

- LOOP Rn,#<0..15>

The  assembler automatically  uses to  the shorter  coding if possible.

If  one wants to  enforce the longer  coding, one may  place a 'bigger'

character   right  before  the  expression  (behind  the  double  cross

character!).  Vice versa, a  'smaller' character can  be used to assure

the  shorter coding is used.  In case the operand  does not fulfill the

range  restrictions  for  the  shorter  coding,  an error is generated.

This  syntax may also be  used for branches and  calls which may either

have a short displacement or a long absolute argument.

TLCS-47

.......

The  direct  data  address  space  of  these  processors  (it  makes no

difference  whether you address directly or  via the HL register) has a

size  of only 256  nibbles. Because the  ''better'' family members have

up  to 1024 nibbles of  RAM on chip, Toshiba  was forced to introduce a

banking  mechanism via  the DMB  register. AS  manages the data segment

as  a continuous addressing  space and checks  at any direct addressing

if  the address is in the currently  active bank. The bank AS currently

expects can be set by means of

ASSUME  DMB:<0..3>

The default value is 0.

IPC-16/INS8900

..............

The  processor provides an input pin  named BPS to select which address

range  shall be  directly addressable:  either the  first 256 words, or

both  the  lowest  and  topmost  128  words.  The  first variant is the

default, an ASSUME BPS:1 switches to the second variant.

ST6

...

The  microcontrollers of  the ST62  family are  able to  map a part (64

bytes)  of the  code area  into the  data area,  e.g. to load constants

from  the ROM. This means also that at  one moment only one part of the

ROM  can be addressed.  A special register  rules which part  it is. AS

cannot  check the  contents of  this register  directly, but  it can be

informed  by this  instruction that  a new  value has  been assigned to

the  register.  AS  then  can  test  and  warn  if  necessary,  in case

addresses  of the code  segment are accessed,  which are not located in

the  ''announced'' window. If,  for example, the  variable VARI has the

value 456h, so

ASSUME  ROMBASE:VARI>>6

sets  the AS-internal variable to 11h,  and an access to VARI generates

an access to address 56h in the data segment.

It  is possible to assign a simple  NOTHING instead of a value, e.g. if

the  bank register is used temporarily as  a memory cell. This value is

also the default.

The  program counter of these  controller only has a  width of 12 bits.

This  means that some sort of banking  scheme had to be introduced if a

device  includes  more  than  4  KBytes  of program memory. The banking

scheme  splits  both  proram  space  and  program  memory in pages of 2

KBytes.  Page  one  of  the  program  space always accesses page one of

program  memory.  The  PRPR  register  present  on such devices selects

which  page of  program memory  is accessed  via addresses 000h to 7ffh

of  program  space.  As  an  initial  approcimation, AS regards program

space  to be  linear and  of the  size of  program memory. If a jump or

call  from page  one is  made to  code in  one of  the other  pages, it

checks  whether the  assumed contents  of the  PRPR register  match the

destination  address. If a jump  or call is done  from one of the other

pages  to  an  address  outside  of  page  one,  it  checks whether the

destination  address is  within the  same page.  IMPORTANT: The program

counter  itself is only 12  bits wide. It is  therefore not possible to

jump  from one  page to  another one,  without an  intermediate step of

jumping  back to page  one. Changing the  PRPR register while operating

outside  of  page  one  would  result  in ''pulling out'' the code from

under one's feet.

ST9

...

The  ST9 family uses exactly the  same instructions to address code and

data  area. It depends  on the setting  of the flag  register's DP flag

which  address space is referenced. To enable  AS to check if one works

with  symbols  from  the  correct  address  space  (this of course only

works  with absolute  accesses!), one  has to  inform AS whether the DP

flag  is currently  0 (code)  or 1  (data). The  initial value  of this

assumption is 0.

78K2

....

78K2  is an 8/16 bit  architecture, which has later  been extended to a

one-megabyte  addres space  via banking.  Banking is  realized with the

registers  PM6  (normal  case)  resp.  P6  (alternate  case  with  & as

prefix)  that supply the missing upper  four address bits. At least for

absolute  addresses, AS  can check  whether the  current, linear 20-bit

address is within the given 64K window.

78K3

....

Processors  witrh a  78K3 core  have register  banks that consist of 16

registers.  These registers may  be used via  their numbers (R0 to R15)

or  their  symbolic  names  (X=R0,  A=R1,  C=R2,  B=R3, VPL=R8, VPH=R9,

UPL=R10,  UPH=R11, E=R12, D=R13, L=R14,  H=R15). The processor core has

a  register select bit (RSS) to switch  the mapping of A/X and B/C from

R0..R3  to  R4..R7.  This  is  mainly  important  for instructions that

implicitly  use one  of these  registers (i.e.  instruction that do not

encode  the register number  in the machine  code). However, it is also

possible to inform the assembler about the changed mapping via a

assume rss:1

The  assmebler  will  then  insert  the alternate register numbers into

machine  instructions  that  explicitly  encode  the  register numbers.

Vice  versa, R5  will be  treated like  A instead  of R1  in the source

code.

78K4

....

78K4  was designed as  an 'upgrade path'  from 78K3, which  is why this

processor  core contains  the same  RSS bit  to control  the mapping of

registers AX and BC (though NEC discourages use of it in new code).

Aside  from  many  new  instructins  and  addressing  modes,  the  most

significant  extension is  the larger  address space  of 16  MBytes, of

which  only the  first MByte  may be  used for  program code.  The CPU-

internal  RAM  and  all  special  function  registers may be positioned

either  at the top of the first MByte  or the top of the first 64 KByte

page.  Choice is made via the  LOCATION machine instruction that either

takes  a 0  or 15  as argument.  Together with  remapping RAM and SFRs,

the  processor also  switches the  address ranges  that may  be reached

with  short (8 bit)  addresses. Parallel to  using LOCATION, one has to

inform  the  assembler  about  this  setting  via  a ASSUME LOCATION:..

statement.  It will  then use  short addressing  for the proper ranges.

The assembler will assume a default of 0 for LOCATION.

320C3x/C4x

..........

As  all instruction  words of  this processor  family are  only 32 bits

long  (of which  only 16  bits were  reserved for  absolute addresses),

the  missing upper 8/16 bits have to  be added from the DP register. It

is  however still  possible to  specify a  full 24/32-bit  address when

addressing,  AS will check then  whether the upper 8  bits are equal to

the  DP  register's  assumed  values.  ASSUME  is  different to the LDP

instruction  in the sense that one  cannot specify an arbitrary address

out  of the  bank in  question, one  has to  extract the  upper bits by

hand:

ldp     @addr

assume  dp:addr>>16

.

.

ldi     @addr,r2

uPD78(C)10

..........

These  processors have  a register  (V) that  allows to move the ''zero

page'',  i.e.  page  of  memory  that  is addressable by just one byte,

freely  in  the  address  space,  within  page  limits.  By  reasons of

comforts you don't want to work with expressions such as

inrw    Lo(counter)

so  AS  takes  over  this  job,  but  only under the premise that it is

informed  via the ASSUME-command about the  contents of the V register.

If  an instruction  with short  addressing is  used, it will be checked

if  the  upper  half  of  the  address  expression  corresponds  to the

expected content. A warning will be issued if both do not match.

75K0

....

As  the whole address space  of 12 bits could  not be addressed even by

the  help  of  register  pairs  (8  bits), NEC had to introduce banking

(like  many others too...):  the upper 4  address bits are fetched from

the  MBS register  (which can  be assigned  values from  0 to 15 by the

ASSUME  instruction), which  however will  only be  regarded if the MBE

flag  has been set to  1. If it is  0 (default), the lowest and highest

128  nibbles of the  address space can  be reached without banking. The

ASSUME  instruction is undefined  for the 75402  as it contains neither

a  MBE flag nor an  MBS register; the initial  values cannot be changed

therefore.

F�MC16L

.......

Similar  to  many  other  families  of  microcontrollers,  this  family

suffers  somewhat from its designers  miserliness: registers of only 16

bits  width are  faced with  an address  space of  24 bits. Once again,

bank  registers had to fill  the gap. In detail,  these are PCB for the

progam  code, DTB for all data  accesses, ADB for indirect accesses via

RW2/RW6,  and SSB/USB for the  stacks. They may all  take values from 0

to  255 and are by default assumed to  be 0, with the exception of 0ffh

for PCB.

Furthermore,  a DPR  register exists  that specifies  which memory page

within  the 64K bank given by DTB  may be reached with 8 bit addresses.

The  default for DPR is 1, resulting  in a default page of 0001xxh when

one takes DTB's default into account.

MN1613

......

The  MN1613 is an  extension of an  architecture with 16 bit addresses.

The  address extension is  done by a  set of "segment registers" (CSBR,

SSBR,  TSR0, and TSR1), each  of which is four  bits wide. The contents

of  a segment  register, left-shifted  by 14  bits, is  added to the 16

bit  addresses. This way,  a process may  access a memory  window of 64

KWords  within  the  address  space  of  256 KWords. The assembler uses

segment  register  values  reported  via  ASSUME  to  warn  whether  an

absolute  address is  outside the  window defined  by the  used segment

register.  If the  address is  within the  window, it  will compute the

correc  t16-bit offset.  Naturally, this  cannot be  done when indirect

addressing is used.

IM61x0

......

These  micro processors  implement the  instruction set  of a PDP/8 and

therefore  fundamentally support an address  space of 4 KWords. Banking

allows  to  extend  this  address  to  eight  ''fields'  of  4  KWords.

Addressing  of data and jumps are  principally only possible within the

same  field, with  one exception:  The IB  register allows to perform a

jump  to another 4K field.  It provides the upper  three bits of the 15

bit  target address, if IB  has been set to  a value unequal to NOTHING

via ASSUME.

3.2.22.  CKPT

- - - - - - -

valid for: TI990/12

Type  12  instructions  require  a  checkpoint  register for execution.

This  register may either  be specified explicitly  as fourth argument,

or   a  default  for   all  following  code   may  be  given  via  this

instruction.  If neither a CKPT  instruction nor an explicit checkpoint

register  was used,  an error  is reported.  The default  of no default

register may be restored by using NOTHING as argument to CKPT.

3.2.23.  EMULATED

- - - - - - - - -

valid for: 29K

AMD  defined  the  29000's  series  exception  handling  for  undefined

instructions  in a  way that  there is  a separate exception vector for

each  instruction.  This  allows  to  extend  the  instruction set of a

smaller  member of this  family by a  software emulation. To avoid that

AS  quarrels about these instructions  as being undefined, the EMULATED

instruction  allows to  tell AS  that certain  instructions are allowed

in  this  case.  The  check  if  the currently set processors knows the

instruction  is then skipped. For example,  if one has written a module

that  supports 32-bit  IEEE numbers  and the  processor does not have a

FPU, one writes

EMULATED FADD,FSUB,FMUL,FDIV

EMULATED FEQ,FGE,FGT,SQRT,CLASS

3.2.24. BRANCHEXT

- - - - - - - - -

valid for: XA

BRANCHEXT  with either  ON or  OFF as  argument tells  AS whether short

branches  that  are  only  available  with  an 8-bit displacement shall

automatically   be  'extended',  for  example  by  replacing  a  single

instruction like

bne     target

with  a  longer  sequence  of  same  functionality,  in  case the branc

target  is  out  of  reach  for  the  instruction's  displacement.  For

example, the replacement sequence for bne would be

beq     skip

jmp     target

skip:

In  case  there  is  no  fitting  'opposite'  for  an  instruction, the

sequence may become even longer, e.g. for jbc:

jbc     dobr

bra     skip

dobr:   jmp     target

skip:

This  feature however has the side  effect that there is no unambigious

assignment  between machine  and assembly  code any  more. Furthermore,

additional  passes may  be the  result if  there are  forward branches.

One should therefore use this feature with caution!

3.2.25. Z80SYNTAX

- - - - - - - - -

G"ultigkeit: 8008, 8080/8085, �PD78xx

With  ON  as  argument,  one  can  optionally  write  machine assembler

instructions   in  the  form  Zilog  defined  them  for  the  Z80.  For

instance,  you simply use  LD with self-explaining  operands instead of

MVI, LXI, MOV, STA, LDA, SHLD, LHLD, LDAX, STAX or SPHL.

Since  some mnemonics  have a  different meaning  in 8008/8080  and Z80

syntax,  it is  not possible  to program  in 'Z80  style' all the time,

unless  the '8080 syntax' is turned  off entirely by using EXCLUSIVE as

argument.  The  details  of  this  operation  mode  can be looked up in

section 4.22.

A  built-in symbol  of same  name allows  to query  the operation mode.

The mapping is 0=OFF, 1=ON, and 2=EXCLUSIVE.

3.2.26.  EXPECT and ENDEXPECT

- - - - - - - - - - - - - - -

This  pair of instructions  may be used  to frame a  piece of code that

is  expected to trigger one  or more error or  warning messages. If the

errors  or warnings  (identified by  their numbers,  see chapter  A) do

occur,  they are  suppressed and  assembly continues  without any error

(naturally,  without creating  code at  the erroneous places). However,

if  warnings or errors that were  expected do not occur, ENDEXPECT will

emit  errors about them. The main  usage scenario of these instructions

are  the self tests  in the tests/  subdirectory. For instance, one may

check this way if range checking of operands works as expected:

cpu      68000

expect   1320     ; immediate shift only for 1..8

lsl.l    #10,d0

endexpect

3.3. Data Definitions

---------------------

The  instructions described in this  section partially overlap in their

functionality,  but each processor  family defines other  names for the

same  function. To stay  compatible with the  standard assemblers, this

way of implementation was chosen.

If  not  explicitly  mentioned  otherwise,  all  instructions  for data

deposition  (not those for  reservation of memory!)  allow an arbitrary

number of parameters which are being processed from left to right.

3.3.1.  DC[.Size]

- - - - - - - - -

valid  for: 680x0,  M*Core, 68xx,  H8, SH7x00,  DSP56xxx, XA, ST7/STM8,

MN161x, IM61x0, CP-3F, SC61860

This   instruction  places  one  or   several  constants  of  the  type

specified  by the  attribute into  memory. The  attributes are the same

ones  as  defined  in  section  2.5,  and  there  is  additionally  the

possibility  for byte  constants to  place string  constants in memory,

like

String  dc.B "Hello world!\0"

The  parameter count may be  between 1 and 20.  A repeat count enclosed

in  brackets  may  additionally  be  prefixed  to  each  parameter; for

example,  one  can  for  example  fill  the  area  up  to the next page

boundary with zeroes with a statement like

dc.b    [(*+255)&$ffffff00-*]0

CAUTION!  This function easily allows to reach  the limit of 1 Kbyte of

generated code per line!

The  assembler can automatically  add another byte  of data in case the

byte   sum  should  become  odd,  to  keep  the  word  alignment.  This

behaviour may be turned on and off via the PADDING instruction.

Decimal  floating point numbers stored  with this instruction (DC.P...)

can  cover the  whole range  of extended  precision, one however has to

pay  attention to the detail  that the coprocessors currently available

from   Motorola  (68881/68882)  ignore  the   thousands  digit  of  the

exponent at the read of such constants!

The default attribute is W, that means 16-bit-integer numbers.

For  the  DSP56xxx,  the  data  type  is  fixed  to integer numbers (an

attribute  is therefore  neither necessary  nor allowed),  which may be

in  the range of  -8M up to  16M-1. String constants  are also allowed,

whereby three characters are packed into each word.

Opposed  to  the  standard  Motorola  assembler,  it  is  also valid to

reserve  memory space with this statement,  by using a question mark as

operand.  This is an extension added  by some third-party suppliers for

68K  assemblers, similar to what  Intel assemblers provide. However, it

should  be clear  that usage  of this  feature may  lead to portability

problems.  Furthermore, question  marks as  operands must  not be mixed

with 'normal' constants in a single statement.

3.3.2.  DS[.Size]

- - - - - - - - -

valid  for: 680x0,  M*Core, 68xx,  H8, SH7x00,  DSP56xxx, XA, ST7/STM8,

MN161x, IM61x0, CP-3F, PPS-4, SC61860

On  the one hand, this instruction  enables to reserve memory space for

the  specified count  of numbers  of the  type given  by the attribute.

Therefore,

DS.B    20

for example reserves 20 bytes of memory, but

DS.X    20

reserves 240 bytes!

The  other purpose  is the  alignment of  the program  counter which is

achieved by a count specification of 0. In this way, with a

DS.W    0  ,

the  program counter will be rounded up  to the next even address, with

a

DS.D 0

in  contrast to  the next  double word  boundary. Memory cells possibly

staying  unused thereby are  neither zeroed nor  filled with NOPs, they

simply stay undefined.

The default for the operand length is - as usual - W, i.e. 16 bits.

For  the  56xxx,  the  operand  length  is  fixed to words (of 24 bit),

attributes therefore do not exist just as in the case of DC.

3.3.3. BLKB, BLKW, BLKL, BLKD

- - - - - - - - - - - - - - -

valid for: Renesas RX

These  statements are used  to reserve memory  on Renesas RX. The total

amount  of  memory  reserved  is  the  instruction's argument times the

operand  size given by  the instruction (1  byte for BLKB,  2 bytes for

BLKW, 4 bytes for BLKL, and 8 bytes for BLKD).

3.3.4.  DN,DB,DW,DD,DQ,DT, and DO

- - - - - - - - - - - - - - - - -

valid for: Intel (except for 4004/4040), Zilog, Toshiba,

NEC, TMS370, Siemens, AMD, MELPS7700/65816,

M16(C), National, ST9, Atmel, TMS70Cxx, TMS1000,

Signetics, �PD77230, Fairchild, Intersil,

XS1, SC62015

These  commands are - one  could say - the  Intel counterpart to DS and

DC,  and as  expected, their  logic is  a little  bit different: First,

the specification of the operand length is moved into the mnemonic:

- DN: 4-bit integer

- DB: 8-bit byte or ASCII string similar to DC.B

- DW: 16-bit integer or half precision floating point

- DD: 32-bit integer or single precision floating point

- DQ: 64-bit integer or double precision floating point

- DT:  packed BCD  integer or  extended precision  floating point (80

bits)

- DO: quad precision floating point (128 bits)

Second,   the  distinction  between   constant  definition  and  memory

reservation  is done by the operand.  A reservation of memory is marked

by a ? :

db      ?       ; reserves a byte

dw      ?,?     ; reserves memory for 2 words (=4 byte)

dd      -1      ; places the constant -1 (FFFFFFFFH) !

Reserved  memory and constant definitions must  not be mixed within one

instruction:

db      "hello",?       ; --> error message

Additionally,   the  DUP  Operator  permits  the  repeated  placing  of

constant sequences or the reservation of whole memory blocks:

db      3 dup (1,2)     ; --> 1 2 1 2 1 2

dw      20 dup (?)      ; reserves 40 bytes of memory

As  you  can  see,  the  DUP-argument  must be enclosed in parentheses,

which  is  also  why  it  may  consist  of several components, that may

themselves be DUPs...the stuff therefore works recursively.

DUP  is however also  a place where  one can get  in touch with another

limit  of the assembler:  a maximum of  1024 bytes of  code or data may

be  generated in  one line.  This is  not valid  for the reservation of

memory, only for the definition of constant arrays!

The  DUP operator only gets recognized if  it is itself not enclosed in

parentheses,  and if  there is  a non-empty  argument to its left. This

way, it is possible to use a symbol of same name as argument.

Several  platforms define pseudo  instructions with same functionality,

but different names:

- In  order  to  be  compatible  to  the  M80,  DEFB/DEFW may be used

instead of DB/DW in Z80-mode.

- BYTE/ADDR  resp.  WORD/ADDRW  in  COP4/8  mode  are an alias for DB

resp.  DW,  with  the  pairs  differing in byte order: instructions

defined  by  National  for  address  storage  use  big endian, BYTE

resp. WORD in contrast use little endian.

- BYTE  resp. WORD  on PDP-11,  VAX and  WD16 work  like DB resp. DW,

however only accept integer arguments.

- LWORD  resp. QUAD on VAXwork like  DD resp. DQ, however only accept

integer arguments.

- The Renesas RX target provides:

- BYTE (like DB)

- WORD (like DW, only integer arguments)

- LWORD (like DD, only integer arguments)

- FLOAT (like DD, only floating point arguments)

- DOUBLE (like DQ, only floating point arguments)

If  DB is used in  an address space that  is not byte addressable (like

the  Atmel AVR's CODE segment),  bytes are packed in  pairs into 16 bit

words,  according  to  the  endianess  given  by  the architecture: for

little  endian, the LSB is  filled first. If the  total number of bytes

is  odd,  one  half  of  the  last  word  remains unused, just like the

argument  list had been padded. It will  also not be used if another DB

immediately  follows  in  source  code.  The  analogous is true for DN,

just  with the difference  that two or  four nibbles are  packet into a

byte or 16 bit word.

The  NEC 77230 is special  with its DW instruction:  It more works like

the  DATA statement of its smaller  brothers, but apart from string and

integer  arguments, it also  accepts floating point  values (and stores

them  in the  processor's proprietary  32-bit format).  There is no DUP

operator!

When  floating point  constants are  stored, they  cannot have a larger

precision  or  range  than  the  floating  point  type used on the host

system.  For instance, if the common 64  bit IEEE 754 format is used on

the  host side,  the value  range is  limited to  roughly +/-1.8*10^308

(see  the variable FLOATMAX),  and the lowest  two resp. eight bytes of

DT resp. DO are filled with zeros.

3.3.5. FLT2, FLT3, FLT4

- - - - - - - - - - - -

valid for: PDP-11 (FLT2, FLT4),

WD16 (FLT3)

FLT2  and FLT4  are similar  in function  to DD  bzw. DQ. However, they

only  store floating point  numbers (in DEC's  own F and  D formats) in

memory.  The  WD16  uses  an  own,  48  bits (three machine words) long

format.  Floating point numbers in this  format can be stored in memory

with the FLT3 instruction.

3.3.6. x_FLOATING

- - - - - - - - -

valid for: VAX

These  instructions  store  floating  point  constants in DEC format in

memory.  x represents one  of the four  supported formats (F,  D, G and

H).  FLOAT  is  an  alias  for  F_FLOATING,  and DOUBLE is an alias for

D_FLOATING. Otherwise, these instructions work like DD and DQ.

3.3.7.  DS, DS8

- - - - - - - -

valid for: Intel, Zilog, Toshiba, NEC, TMS370, Siemens, AMD,

M16(C), National, ST9, TMS7000, TMS1000, Intersil,

6502, 68xx

With this instruction, you can reserve a memory area:

DS      <count>

It is an abbreviation of

DB      <count> DUP (?)

Although  this could easily  be made by  a macro, some  people grown up

with   Motorola  CPUs  (Hi  Michael!)  suggest  DS  to  be  a  built-in

instruction...I hope they are satisfied now ;-)

DS8  is defined  as an  alias for  DS on  the National  SC14xxx. Beware

that  the code memory of  these processors is organized  in words of 16

bits,  it is therefore impossible to  reserve individual bytes. In case

the  argument of  DS is  odd, it  will be  rounded up  to the next even

number.

3.3.8. BLKx

- - - - - -

valid for: VAX

These  instructions  reserve  storage  space  for  the  given number of

elements,  with  the  element  size  coded  into the instruction's last

character.  Therefore, the size of the  reserved area in bytes is equal

to  the element count for BLKB, double  the count for BLKW, and sixteen

times the count for BLKO and BLKH.

3.3.9. BYT or FCB

- - - - - - - - -

valid for: 6502, 68xx, SC61860

By  this instruction,  byte constants  or ASCII  strings are  placed in

65xx/68xx-mode,  it therefore  corresponds to  DC.B on  the 68000 or DB

on  Intel  (which  ias  also  allowed).  Similarly  to DC, a repetition

factor  enclosed in  brackets ([..])  may be  prepended to every single

parameter.

3.3.10.  BYTE

- - - - - - -

valid for: ST6, 320C2(0)x, 320C5x, MSP, TMS9900, CP-1600

Ditto.  Note  that  when  in  320C2(0)x/5x  mode, the assembler assumes

that  a label on  the left side  of this instruction  has no type, i.e.

it  belongs to  no address  space. This  behaviour is  explained in the

processor-specific hints.

The  PADDING  instruction  allows  to  set  whether odd counts of bytes

shall be padded with a zero byte in MSP/TMS9900 mode.

The  operation of  BYTE on  CP-1600 is  somewhat different:  the 16-bit

integer  arguments  are  stored  byte-wise  in two consecutive words of

memory  (LSB  first).  If  individual  8-bit  values shall be stored in

memor (optionally packed), use the TEXT instruction!

3.3.11. DC8

- - - - - -

valid for: SC144xx

This  statement is an  alias for DB,  i.e. it may  be used to dump byte

constants or strings to memory.

3.3.12.  ADR or FDB

- - - - - - - - - -

valid for: 6502, 68xx, SC61860

ADR  resp.  FDB  stores  word  constants  when in 65xx/68xx mode. It is

therefore  the  equivalent  to  DC.W  on  the  68000  or  DW  on  Intel

platforms  (which  is  also  allowed).  Similarly  to  DC, a repetition

factor  enclosed in  brackets ([..])  may be  prepended to every single

parameter.

3.3.13. DDB

- - - - - -

valid for: 6502, MELPS-7700

This  instructions works similar to ADR,  just with the difference that

the 16 bit values are stored in big endian order.

3.3.14. DCM

- - - - - -

valid for: 6502

This  instructions  allows  to  dispose  floating  point  constants  in

memory,  in  the  format  that  described  in [3]: An exponent of eight

bits  and a  mantissa of  24 bits  in two's  complement representation,

stored in big-endian order.

3.3.15.  WORD

- - - - - - -

valid for: ST6, i960, 320C2(0)x, 320C3x/C4x/C5x, MSP, CP-1600,

IMP-16, IPC-16

If  assembling for the  320C3x/C4x or i960,  this command stores 32-bit

words,  16-bit  words  for  the  other  families.  Note  that  when  in

320C2(0)x/5x  mode,  the  assembler  assumes  that  a label on the left

side  of this instruction  has no type,  i.e. it belongs  to no address

space.  This  behaviour  is  explained  at the discussion on processor-

specific hints.

3.3.16.  DW16

- - - - - - -

valid for: SC144xx

This  instruction is for  SC144xx targets a  way to dump  word (16 bit)

constants to memory. CAUTION!! It is therefore an alias for DW.

3.3.17.  ACON

- - - - - - -

valid for: 2650

ACON  works the same way  as DW, the 16  bit numbers are however stored

in big endian format.

3.3.18.  LONG

- - - - - - -

valid for: 320C2(0)x, 320C5x

LONG  stores a 32-bit  integer to memory  with the order LoWord-HiWord.

Note  that  when  in  320C2(0)x/5x  mode,  the assembler assumes that a

label  on  the  left  side  of  this  instruction  has no type, i.e. it

belongs  to  no  address  space.  This  behaviour  is  explained in the

processor-specific hints.

3.3.19.  SINGLE, DOUBLE, and EXTENDED

- - - - - - - - - - - - - - - - - - -

valid for: 320C3x/C4x (not DOUBLE), 320C6x (not EXTENDED)

Both  commands  store  floating-point  constants  to memory. In case of

the  320C3x/C4x,  they  are  not  stored  in  IEEE-format.  Instead the

processor-specific  formats with  32 and  40 bit  are used.  In case of

EXTENDED  the resulting  constant occupies  two memory  words. The most

significant  8 bits (the exponent) are  written to the first word while

the other ones (the mantissa) are copied into the second word.

3.3.20.  FLOAT and DOUBLE

- - - - - - - - - - - - -

valid for: 320C2(0)x, 320C5x

These  two commands store floating-point  constants in memory using the

standard  IEEE 32-bit  and 64-bit  IEEE formats.  The least significant

byte  is copied to the first  allocated memory location. Note that when

in  320C2(0)x/5x  mode  the  assembler  assumes  that all labels on the

left  side  of  an  instruction  have  no  type, i.e. they belong to no

address  space. This  behaviour is  explained in the processor-specific

hints.

3.3.21. SINGLE and DOUBLE

- - - - - - - - - - - - -

valid for: TMS99xxx

These  two commands store floating-point  constants in memory using the

processor's  floating  point  format,  which  is  equal  to the IBM/360

floating point format.

3.3.22.  EFLOAT, BFLOAT, and TFLOAT

- - - - - - - - - - - - - - - - - -

valid for: 320C2(0)x, 320C5x

Another  three floating  point commands.  All of  them support non-IEEE

formats, which should be easily applicable on signal processors:

- EFLOAT: mantissa with 16 bits, exponent with 16 bits

- BFLOAT: mantissa with 32 bits, exponent with 16 bits

- TFLOAT: mantissa with 64 bits, exponent with 32 bits

The  three commands share  a common storage  strategy. In all cases the

mantissa  precedes  the  exponent  in  memory,  both  are stored as 2's

complement  with the  least significant  byte first.  Note that when in

320C2(0)x/5x  mode the  assembler assumes  that all  labels on the left

side  of an instruction  have no type,  i.e. they belong  to no address

space. This behaviour is explained in the processor-specific hints.

3.3.23.  Qxx and LQxx

- - - - - - - - - - -

valid for: 320C2(0)x, 320C5x

Qxx  and  LQxx  can  be  used  to  generate  constants in a fixed point

format.  xx denotes a  2-digit number. The  operand is first multiplied

by  2  ^xx  before  converting  it  to  binary notation. Thus xx can be

viewed  as  the  number  of  bits  which  should  be  reserved  for the

fractional  part  of  the  constant  in  fixed point format. Qxx stores

only one word (16 bit) while LQxx stores two words (low word first):

q05     2.5     ; --> 0050h

lq20    ConstPI ; --> 43F7h 0032h

Please  do not  flame me  in case  I calculated  something wrong  on my

HP28...

3.3.24.  DATA

- - - - - - -

valid  for: PIC,  320xx, AVR,  MELPS-4500, H8/500,  HMCS400, 4004/4040,

�PD772x, OLMS-40/50, Padauk

This  command stores data  in the current  segment. Both integer values

as  well as character  strings are supported.  On 16C5x/16C8x, 17C4x in

data  segment,  and  on  the  4500,  4004, and HMCS400 in code segment,

characters  occupy one word. On AVR,  17C4x in code segment, �PD772x in

the  data segments, and  on 3201x/3202x, in  general two characters fit

into  one  word  (LSB  first).  The  �PD77C25 can hold three bytees per

word  in the code segment. When  in 320C3x/C4x, mode the assembler puts

four  characters  into  one  word  (MSB  first).  In  contrast  to this

characters  occupy  two  memory  locations  in  the data segment of the

4500,  similar in  the 4004  and HMCS400.  The range  of integer values

corresponds  to  the  word  width  of  each  processor  in  a  specific

segment.  This  means  that  DATA  has  the  same result than WORD on a

320C3x/C4x  (and  that  of  SINGLE  if  AS  recognizes the operand as a

floating-point constant).

3.3.25. ZERO, CP-1600

- - - - - - - - - - -

valid for: PIC

Generates  a continuous string of zero  words in memory (which equals a

NOP on PIC).

3.3.26. FB and FW

- - - - - - - - -

valid for: COP4/8

These  instruction  allow  to  fill  memory  blocks with a byte or word

constant.  The first  operand specifies  the size  of the  memory block

while the second one sets the filling constant itself.

3.3.27. ASCII, ASCIC, and ASCIZ

- - - - - - - - - - - - - - - -

valid for: ST6, PDP-11, VAX, IMP-16, IPC-16 ( ASCII)

ST6, PDP-11, VAX ( ASCIZ)

PDP-11, VAX ( ASCIC)

These  commands store  string constants  to memory.  While ASCII writes

the  character information only,  ASCIZ additionally appends  a zero to

the end of the string, and ASCIC prepends a length byte.

3.3.28.  STRING and RSTRING

- - - - - - - - - - - - - -

valid for: 320C2(0)x, 320C5x

These  commands  are  functionally  equivalent  to  DATA,  but  integer

values  are  limited  to  the  range  of  byte values. This enables two

characters  or  numbers  to  be  packed  together  into  one word. Both

commands  only  differ  in  the  order  they use to write bytes: STRING

stores  the upper one first then the  lower one, RSTRING does this vice

versa.  Note that when in 320C2(0)x/5x  mode the assembler assumes that

a  label on  the left  side of  this instruction  has no  type, i.e. it

belongs  to  no  address  space.  This  behaviour  is  explained in the

processor-specific hints.

3.3.29.  PACKED

- - - - - - - -

valid for: PDP-11, VAX

This  instruction may  be use  to put  numbers in packed decimal format

into  mmemory. Ist argument  is either a  (signed) integer number, or a

string.  Of course, the latter may only  contain characters from 0 to 9

and  an  optional  plus  or  minus  sign  at the beginning. The maximum

number  of digits is  31, and the  sign is appended  as last digit. The

values  12 resp. 13  represent a plus  resp. minus sign.  If the number

of  digits  plus  sign  is  odd,  a  zero  digit  is  inserted  at  the

beginning.

Optionally,  a  symbol's  name  may  be  given  as second argument. The

number  of digits,  excluding the  sign and  the optional  null pad, is

stored in this symbol.

3.3.30. RADIX50

- - - - - - - -

valid for: diverse

This  instruction may be  used to put  strings in packed RADIX50 format

into   memory.  This   coding  was   common  for   file  names  in  DEC

environments  and packs three characters into  one 16 bit word. RADIX50

is  not a built-in  instruction. Instead, it  is defined as  a macro in

the radix50.inc include file.

3.3.31. FCC

- - - - - -

valid for: 6502, 68xx

When  in  65xx/68xx  mode,  string  constants  are generated using this

instruction.  In contrast to the  original assembler AS11 from Motorola

(this  is  the  main  reason  why  AS  understands  this  command,  the

functionality  is  contained  within  the  BYT  instruction)  you  must

enclose  the  string  argument  by  double  quotation  marks instead of

single  quotation  marks  or  slashes.  Similarly  to  DC, a repetition

factor  enclosed in  brackets ([..])  may be  prepended to every single

parameter.

3.3.32.  TEXT

- - - - - - -

In  CP-1600 mode,  This instruction  is used  to store string constants

in packed format, i.e. two characters per word.

3.3.33.  DFS or RMB

- - - - - - - - - -

valid for: 6502, 68xx

Reserves  a memory  block when  in 6502/68xx  mode. It is therefore the

equivalent to DS.B on the 68000 or DB ? on Intel platforms.

3.3.34. BLOCK

- - - - - - -

valid for: ST6

Ditto.

3.3.35. SPACE

- - - - - - -

valid for: i960

Ditto.

3.3.36. RES

- - - - - -

valid  for: PIC,  MELPS-4500, HMCS400,  3201x, 320C2(0)x,  320C5x, AVR,

�PD772x, OLMS-40/50, Padauk, CP-1600, PPS-4, 2650

This   command  allocates  memory.  When  used  in  code  segments  the

argument  counts words  (10/12/14/16 bit).  In data  segments it counts

bytes  for PICs, nibbles for 4500,  PPS-4, and OLMS-40/50 and words for

the TI devices.

3.3.37. BSS

- - - - - -

valid for: 320C2(0)x, 320C3x/C4x/C5x/C6x, MSP

BSS  works  like  RES,  but  when  in  320C2(0)x/5x mode, the assembler

assumes  that  a  label  on  the  left  side of this instruction has no

type,  i.e it belongs to no  address space. This behaviour is explained

in the processor-specific hints.

3.3.38. DSB and DSW

- - - - - - - - - -

valid for: COP4/8

Both  instructions allocate  memory and  ensure compatibility to ASMCOP

from  National. While  DSB takes  the argument  as byte count, DSW uses

it as word count (thus it allocates twice as much memory than DSB).

3.3.39.  DS16

- - - - - - -

valid for: SC144xx

This  instruction  reserves  memory  in  steps  of  full words, i.e. 16

bits. It is an alias for DW.

3.3.40. ALIGN

- - - - - - -

valid for: all processors

Takes  the argument to  align the program  counter to a certain address

boundary.  AS increments  the program  counter to  the next multiple of

the  argument.  So,  ALIGN  corresponds  to  DS.x on 68000, but is much

more flexible at the same time.

Example:

align     2

aligns  to an even  address (PC mod  2 = 0).  If Align is  used in this

form  with only one argument, the  contents of the skipped memory space

is  not defined. An optinal  second argument may be  used to define the

(byte) value used to fill the area.

3.3.41. LTORG

- - - - - - -

valid for: SH7x00, IM61x0, IMP-16, IPC-16

Although  the SH7000 processor  can do an  immediate register load with

8  bit only, AS  shows up with  no such restriction.  This behaviour is

instead  simulated  through  constants  in  memory. Storing them in the

code  segment (not far  away from the  register load instruction) would

require  an  additional  jump.  AS  Therefore  gathers the constants an

stores  them at  an address  specified by  LTORG. Details are explained

in the processor-specific section somewhat later.

3.4. Macro Instructions

-----------------------

valid for: all processors

Now  we finally reach the things  that make a macro assembler different

from  an ordinary assembler:  the ability to  define macros (guessed it

!?).

When  speaking about 'macros', I generally  mean a sequence of (machine

or  pseudo)  instructions  which  are  united  to  a  block  by special

statements  and  can  then  be  treated  in certain ways. The assembler

knows the following statements to work with such blocks:

3.4.1.  MACRO

- - - - - - -

is  probably the most important  instruction for macro programming. The

instruction sequence

<name>  MACRO   [parameter list]

<instructions>

ENDM

defines  the  macro  <name>  to  be  the enclosed instruction sequence.

This  definition by  itself does  not generate  any code! In turn, from

now  on the instruction sequence can simply  be called by the name, the

whole   construct  therefore   shortens  and   simplifies  programs.  A

parameter  list may  be added  to the  macro definition  to make things

even more useful.

While  a  macro's  name  only  has  to  conform  to the usual rules for

symbol  names (see  section 2.7),  there is  the additional restriction

that  parameter names may  not contain underscore  characters. This may

be  changed via  the -underscore-macroargs  command line  argument, but

sinve  it  has  additional  side  effects,  it  should  only be used if

absolutely necessary.

A  switch  to  case-sensitive  mode  influences  both  macro  names and

parameters.

Similar  to symbols, macros  are local, i.e.  they are only  known in a

section  and its subsections when the  definition is done from within a

section.  This behaviour however  can be controlled  in wide limits via

the options PUBLIC and GLOBAL described below.

A  default value  may be  provided for  each macro  parameter (appended

via  an equal  sign). This  value is  used if  there is no argument for

this  parameter  at  macro  call  or  if  the  positional argument (see

below) for this parameter is empty.

Apart  from  the  macro  parameters  themselves, the parameter list may

contain  control  parameters  which  influence  the  processing  of the

macro.  These parameters  are distinguished  from normal  parameters by

being  enclosed in curly  braces. The following  control parameters are

defined:

- EXPAND/NOEXPAND:  rule whether  the enclosed  code shall be written

to  the  listing  when  the  macro  is expanded. The default is the

value set by the pseudo instruction MACEXP_DFT.

- EXPIF/NOEXPIF:  rule whether instructions  for conditional assembly

and  code excluded by it  shall be written to  the listing when the

macro  is  expanded.  The  default  is  the value set by the pseudo

instruction MACEXP_DFT.

- EXPMACRO/NOEXPMACRO:  rule  whether  macros  defined in the macro's

body  shall be written  to the listing  when the macro is expanded.

The   default  is   the  value   set  by   the  pseudo  instruction

MACEXP_DFT.

- EXPREST/NOEXPREST  : rule whether a  macro body's lines not fitting

into  the  first  two  categories  shall  be written to the listing

when  the macro is  expanded. The default  is the value  set by the

pseudo instruction MACEXP_DFT.

- PUBLIC[:section  name]:  assigns  the  macro  to  a  parent section

instead   of  the  current  section.  A  section  can  make  macros

accessible   for  the   outer  code   this  way.   If  the  section

specification  is  missing,  the  macro  becomes completely global,

i.e. it may be referenced from everywhere.

- GLOBAL[:section   name]:  rules  that  in  addition  to  the  macro

itself,  another  macro  shall  be  generated  that  has  the  same

contents  but is  assigned to  the specified  section. Its  name is

constructed  by  concatenating  the  current  section's name to the

macro  name. The section specified must  be a parent section of the

current  section; if  the specification  is missing, the additional

macro  becomes  globally  visible.  For  example,  if  a macro A is

defined  in a section  B that is  a child section  of section C, an

additional   global  macro  named  C_B_A  would  be  generated.  In

contrast,  if C  had been  specified as  target section,  the macro

would  be named B_A  and be assigned  to section C.  This option is

turned  off by default  and it only  has an effect  when it is used

from   within  a  section.   The  macro  defined   locally  is  not

influenced by this option.

- EXPORT/NOEXPORT:  rules whether the definition  of this macro shall

be  written to a separate  file in case the  -M command line option

was  given.  This  way,  definitions  of  'private'  macros  may be

mapped  out selectively. The default  is FALSE, i.e. the definition

will  not be written  to the file.  The macro will  be written with

the  concatenated  name  if  the  GLOBAL  option  was  additionally

present.

- INTLABEL/NOINTLABEL  :  rules  whether  a  label  defined in a line

that  calls  this  macro  may  be  used  as an additional parameter

inside the label or not, instead of simply 'labeling' the line.

- GLOBALSYMBOLS/NOGLOBALSYMBOLS  :  rules  whether  labels defined in

the  macro's  body  shall  be  local  to  this  macro  or  also  be

available  outside the  macro. The  default is  to keep them local,

since using a macro multiple time would be difficult otherwise.

The  control parameters described above  are removed from the parameter

list  by AS, i.e.  they do not  have a further  influence on processing

and usage.

When  a  macro  is  called,  the  parameters  given  for  the  call are

textually  inserted  into  the  instruction  block  and  the  resulting

assembler  code  is  assembled  as  usual.  Zero  length parameters are

inserted  in case too few parameters  are specified. It is important to

note  that string  constants are  not protected  from macro expansions.

The old IBM rule:

It's not a bug, it's a feature!

applies  for  this  detail.  The  gap  was  left  to  allow checking of

parameters  via  string  comparisons.  For  example,  one can analyze a

macro parameter in the following way:

mul     MACRO   para,parb

IF      UpString("PARA")<>"A"

MOV    a,para

ENDIF

IF      UpString("PARB")<>"B"

MOV    b,parb

ENDIF

!mul     ab

ENDM

It  is important for the example  above that the assembler converts all

parameter  names  to  upper  case  when  operating  in case-insensitive

mode,   but  this  conversion  never   takes  place  inside  of  string

constants.  Macro  parameter  names  therefore  have  to  be written in

upper case when they appear in string constants.

Macro  parameter  expansion  furthermore  depends  on whether parameter

names   may  contain  underscores   or  not.  If   not  (the  default),

underscores  in the  macro body  also have  a function of 'limiters' to

detect parameter names. So in the following example:

setled  macro led,value

out   led,value

ld    led_shadow,value

endm

the  parameter 'led'  would be  replaced in  both source lines, whereas

only  in the first one if  the command line switch underscore-macroargs

is  used. Several  of the  include files  shipped with  AS rely  on the

default  behaviour.  This  switch  should  therefore  only  be  used if

absolutely necessary.

Macro  arguments may  be given  in either  of two  forms: positional or

keyword arguments.

For   positional  arguments,  the  assignment  of  arguments  to  macro

parameters  simply  results  from  the  position of arguments, i.e. the

first   argument  is  assigned  to  the  first  parameter,  the  second

argument  to  the  second  parameter  and  so  on.  If  the  number  of

arguments   is  smaller  than  the  number  of  parameters,  eventually

defined  default values  or simply  an empty  string are  inserted. The

same is valid for empty arguments in the argument list.

Keyword  arguments on the other  hand explicitly define which parameter

they relate to, by being prefixed with the parameter's name:

mul  para=r0,parb=r1

Again,  non-assigned parameters will use  an eventually defined default

or an empty string.

As  a difference  to positional  arguments, keyword  arguments allow to

assign an empty string to a parameter with a non-empty default.

Mixing  of  positional  and  keyword  arguments  in  one  macro call is

possible,  however it is not allowed  to use positional arguments after

the first keyword argument.

The  same  naming  rules  as  for  usual  symbols  also apply for macro

parameters,  with  the  exception  that  only  letters  and numbers are

allowed,  i.e. dots and underscores  are forbidden. This constraint has

its  reason in a  hidden feature: the  underscore allows to concatenate

macro parameter names to a symbol, like in the following example:

concat  macro   part1,part2

call    part1_part2

endm

The call

concat  module,function

will therefore result in

call    module_function

Apart  from the parameters  explicitly declared for  a macro, four more

'implicitly'   declared  parameters   exist.  Since   they  are  always

present, they cannot not be redeclared as explicit parameters:

-  ATTRIBUTE  refers to the attribute appended  to the macro call, in

case  the  currently  active  architecture  supports attributes for

machine instructions. See below for an example!

-  ALLARGS  refers to a comma-separated  list of all arguments passed

to a macro, usable e.g. to pass them on to a IRP statement.

-  ARGCOUNT  refers to  the actual  count of  parameters passed  to a

macro.  Note  however  that  this  number  is  never lower than the

formal  parameter  count  of  the  macro,  since  AS  will  fill up

missing arguments with empty strings!

-  __LABEL__  refers  to  a  label  present  in a line that calls the

macro.  This replacement  only takes  place if  the INTLABEL option

was set for this macro!

IMPORTANT:  the  names  of  these  implicit  parameters  are also case-

insensitive if AS was told to operate case-sensitive!

The  purpose of being  able to 'internally'  use a label  in a macro is

surely  not immediately obvious. There might  be cases where moving the

macro's  entry point  into its  body may  be useful. The most important

application  however are  TI signal  processors that  use a double pipe

symbol in the label's column to mark parallelism, like this:

instr1

||  instr2

(since  both instructions  merge into  a single  word of  machine code,

you  cannot  branch  to  the  second  instruction  -  so  occupying the

label's  position  doesn't  hurt).  The  problem  is  however that some

'convenience  instructions' are  realized as  macros. A parallelization

symbol  written in front of a macro  call normally would be assigned to

the  macro itself, not to the  macro body's first instruction. However,

things work with this trick:

myinstr    macro {INTLABEL}

__LABEL__  instr2

endm

instr1

||         myinstr

The  result  after  expanding  myinstr  is  identical  to  the previous

example without macro.

Recursion  of macros,  i.e. the  repeated call  of a  macro from within

its  own body is completely legal. However,  like for any other sort of

recursion,  one has to  assure that there  is an end  at someplace. For

cases  where one  forgot this,  AS keeps  an internal counter for every

macro  that is  incremented when  an expansion  of this  macro is begun

and  decremented  again  when  the  expansion  is completed. In case of

recursive  calls, this  counter reaches  higher and  higher values, and

at  a limit settable via NESTMAX, AS  will refuse to expand. Be careful

when  you turn off this emergency  brake: the memory consumption on the

heap may go beyond all limits and even shut down a Unix system...

A small example to remove all clarities ;-)

A  programmer  braindamaged  by  years  of programming Intel processors

wants  to have the instructions PUSH/POP  also for the 68000. He solves

the 'problem' in the following way:

push    macro   op

move.ATTRIBUTE op,-(sp)

endm

pop     macro   op

move.ATTRIBUTE (sp)+,op

endm

If one writes

push    d0

pop.l   a2    ,

this results in

move.   d0,-(sp)

move.l  (sp)+,a2

A macro definition must not cross include file boundaries.

Labels  defined in  macros always  are regarded  as being local, unless

the  GLOBALSYMBOLS  was  used  in  the  macro's definition. If a single

label  shall  be  made  public  in  a  macro  that  uses  local  labels

otherwise,  it  may  be  defined  with  a  LABEL statement which always

creates global symbols (similar to BIT, SFR...):

<Name>  label   $

When  parsing  a  line,  the  assembler  first  checks  the  macro list

afterwards  looks  for  processor  instructions,  which  is  why macros

allow  to  redefine  processor  instructions.  However,  the definition

should  appear previously  to the  first invocation  of the instruction

to avoid phase errors like in the following example:

bsr     target

bsr     macro   targ

jsr     targ

endm

bsr     target

In  the  first  pass,  the  macro  is  not  known  when  the  first BSR

instruction  is assembled;  an instruction  with 4  bytes of  length is

generated.  In  the  second  pass  however,  the  macro  definition  is

immediately  available (from the  first pass), a  JSR of 6 bytes length

is  therefore generated. As a result,  all labels following are too low

by  2 and phase errors occur for  them. An additional pass is necessary

to resolve this.

Because  a machine  or pseudo  instruction becomes  hidden when a macro

of  same name  is defined,  there is  a backdoor  to reach the original

meaning:  the search for  macros is suppressed  if the name is prefixed

with  an exclamation mark (!).  This may come in  handy if one wants to

extend  existing instructions  in their  functionality, e.g.  the TLCS-

90's shift instructions:

srl     macro   op,n            ; shift by n places

rept    n               ; n simple instructions

!srl   op

endm

endm

From now on, the SRL instruction has an additional parameter...

Macro Expansion in the Listing

..............................

If  a  macro  is  being  called,  the  macro's  body is included in the

assembly   listing,  after  arguemnts  have  been  expanded.  This  can

significantly  increase the  listing's size  and make  it hard to read.

It  is  therefore  possible  to  suppress  this expansion totally or in

parts.  Fundamentally,  AS  divides  the  source  lines  contained in a

macro's body into three classes:

- Macro  definitions,  i.e.  the  macro  is  used  to  define another

macro, or it contains REPT/IRP/IRPC/WHILE blocks.

- Instructions  for conditional  assembly plus  any source lines that

have   not  been  assembled  due  to  conditional  assembly.  Since

conditional  assembly may  depend on  macro arguments,  this subset

may also vary.

- All  remaining sourc  elines that  do not  fall under the first two

categories.

Which  parts  occur  in  the  listing  may  be defined individually for

every  macro. When defining a macro, the  default is the set defined by

the  most recent MACEXP_DFT instruction (3.7.3).  If one of the EXPAND/

NOEXPAND,   EXPIF/NOEXPIF  EXPMACRO/NOEXPMACRO,   or  EXPREST/NOEXPREST

directives  is used in  the macro's definition,  they act additionally,

but  with  higher  preference.  For  instance,  if  expansion  had been

disabled  completely  (MACEXP_DFT  OFF),  adding  the directive EXPREST

has  the effect that when  using this macro, only  lines are written to

the  listing that  remain after  conditional assembly  and are no macro

definitions themselves.

In  consequence,  changing  the  set  via  MACEXP_DFT  has no effect on

macros  that  have  been  defined  before this statement. The listing's

section  shows  for  defined  macros  the  effective  set  of expansion

directives.  The list given in curly braces  is shorted so that it only

conatins  the last (and therefore valid)  directive for a certain class

of  source lines. A  NOIF given via  MACEXP_DFT will therefore not show

up if the directive EXPIF had been given specifically for this macro.

There  might  be  cases  where  it  is useful to override the expansion

rules  for  a  certain  macro,  regardless  whether  they were given by

MACEXP_DFT  or individual directives.  The statement MACEXP_OVR (3.7.3)

exists  for such cases.  It only has  an effects on macros subsequently

being  expanded. Once again,  directives given by  this instruction are

regarded  in  addition  to  a  macro's  rules,  and they do with higher

priority.   A  MACEXP_OVR  without  any   arguments  disables  such  an

override.

3.4.2.  IRP

- - - - - -

is  a  simplified  macro  definition  for  the case that an instruction

sequence  shall be applied to a couple  of operands and the the code is

not  needed any more afterwards. IRP needs  a symbol for the operand as

its  first parameter,  and an  (almost) arbitrary  number of parameters

that  are sequentially  inserted into  the block  of code. For example,

one can write

irp     op, acc,b,dpl,dph

push    op

endm

to push a couple of registers to the stack, what results in

push    acc

push    b

push    dpl

push    dph

Analog  to  a  macro  definition,  the  argument  list  may contain the

control  parameters  GLOBALSYMBOLS  resp.  NOGLOBALSYMBOLS  (marked  as

such  by  being  enclosed  in  curly  braces).  This  allows to control

whether used labels are local for every pass or not.

3.4.3. IRPC

- - - - - -

IRPC  is a variant of IRP where  the first argument's occurences in the

lines  up  to  ENDM  are  successively  replaced by the characters of a

string  instead  of  further  parameters.  For  example,  an especially

complicated way of placing a string into memory would be:

irpc    char,"Hello World"

db      'CHAR'

endm

CAUTION!  As  the  example  already  shows,  IRPC only inserts the pure

character;  it  is  the  programmer's  task  to  assure that valid code

results  (in  this  example  by  inserting quotes, including the detail

that no automatic conversion to uppercase characters is done).

3.4.4. REPT

- - - - - -

is  the simplest way to employ  macro constructs. The code between REPT

and  ENDM  is  assembled  as  often  as  the  integer  argument of REPT

specifies.  This statement is  commonly used in  small loops to replace

a programmed loop to save the loop overhead.

An example for the sake of completeness:

rept    3

rr      a

endm

rotates the accumulator to the right by three digits.

The  optional  control  parameters  GLOBALSYMBOLS resp. NOGLOBALSYMBOLS

(marked  as such by  being enclosed in  curly braces) may  also be used

here   to   decide   whether   labels   are  local  to  the  individual

repetitions.

In  case REPT's argument  is equal to  or smaller than  0, no expansion

at  all is done. This  is different to older  versions of AS which used

to  be  a  bit  'sloppy'  in  this  respect  and  always  made a single

expansion.

3.4.5.  WHILE

- - - - - - -

WHILE  operates similarly to REPT, but  the fixed number of repetitions

given  as an  argument is  replaced by  a boolean  expression. The code

framed  by WHILE  and ENDM  is assembled  until the  expression becomes

logically  false. This may  mean in the  extreme case that the enclosed

code  is not assembled at all in  case the expression was already false

when  the construct was  found. On the  other hand, it  may happen that

the  expression  stays  true  forever  and AS will run infinitely...one

should   apply  therefore  a  bit  of   accuracy  when  one  uses  this

construct,  i.e. the code must contain  a statement that influences the

condition, e.g. like this:

cnt     set     1

sq      set     cnt*cnt

while   sq<=1000

dc.l    sq

cnt      set     cnt+1

sq       set     cnt*cnt

endm

This example stores all square numbers up to 1000 to memory.

Currently  there exists a  little ugly detail  for WHILE: an additional

empty  line that was not present in  the code itself is added after the

last  expansion. This  is a  'side effect'  based on  a weakness of the

macro  processor and it is  unfortunately not that easy  to fix. I hope

noone minds...

3.4.6.  EXITM

- - - - - - -

EXITM  offers  a  way  to  terminate  a  macro  expansion or one of the

instructions  REPT,  IRP,  or  WHILE  prematurely. Such an option helps

for  example to replace encapsulations  with IF-ENDIF-ladders in macros

by  something more readable.  Of course, an  EXITM itself always has to

be  conditional, what  leads us  to an  important detail: When an EXITM

is  executed, the stack  of open IF  and SWITCH constructs  is reset to

the  state  it  had  just  before  the macro expansion started. This is

imperative  for  conditional  EXITM's  as  the ENDIF resp. ENDCASE that

frames  the  EXITM  statement  will  not  be reached any more; AS would

print  an error  message without  this trick.  Please keep also in mind

that  EXITM  always  only  terminates  the innermost construct if macro

constructs  are  nested!  If  one  want  to  completely  break out of a

nested  construct,  one  has  to  use  additional EXITM's on the higher

levels!

3.4.7.  SHIFT

- - - - - - -

SHIFT  is a tool  to construct macros  with variable argument lists: it

discards   the  first  parameter,  with  the  result  that  the  second

parameter  takes its  place and  so on.  This way  one could  process a

variable  argument list...if you do it  the right way. For example, the

following does not work...

pushlist  macro reg

rept  ARGCOUNT

push  reg

shift

endm

endm

...because  the macro  gets expanded  once, its  output is  captured by

REPT  and  then  executed  n  times.  Therefore,  the first argument is

saved n times...the following approach works better:

pushlist  macro reg

if      "REG"<>""

push    reg

shift

pushlist ALLARGS

endif

endm

Effectively,  this is a recursion that  shortens the argument list once

per  step.  The  important  trick  is  that  a  new  macro expansion is

started in each step...

In  case SHIFT ist already a  machine instruction for a certain target,

SHFT  may  be  used  instead,  or  the pseudo instruction is referenced

explicitly by prepending a period (.SHIFT instead of SHIFT).

3.4.8.  MAXNEST

- - - - - - - -

MAXNEST  allows to  adjust how  often a  mcro may be called recursively

before  AS terminates  with an  error message.  The argument  may be an

arbitrary  positive  integer  value,  with  the special value 0 turning

the  this security brake completely off  (be careful with that...). The

default  value for the maximum nesting  level is 256; its current value

may be read from the integer symbol of same name.

3.4.9. FUNCTION

- - - - - - - -

Though  FUNCTION is not  a macro statement  in the inner  sense, I will

describe  this  instruction  at  this  place  because  it  uses similar

principles like macro replacements.

This  instruction  is  used  to  define  new functions that may then be

used  in formula expressions like  predefined functions. The definition

must have the following form:

<name>  FUNCTION <arg>,..,<arg>,<expression>

The  arguments are  the values  that are  'fed into'  the function. The

definition  uses symbolic names for  the arguments. The assembler knows

by  this that where  to insert the  actual values when  the function is

called. This can be seen from the following example:

isdigit FUNCTION ch,(ch>='0')&&(ch<='9')

This   function  checks   whether  the   argument  (interpreted   as  a

character)  is  a  number  in  the  currently  valid character set (the

character  set  can  be  modified  via  CHARSET,  therefore the careful

wording).

The  arguments' names  (CH in  this case)  must conform to the stricter

rules  for macro parameter  names, i.e. the  special characters . and _

are not allowed.

User-defined  functions  can  be  used  in  the  same  way  as  builtin

functions,  i.e.  with  a  list  of  parameters,  separated  by commas,

enclosed in parentheses:

IF isdigit(char)

message "\{char} is a number"

ELSEIF

message "\{char} is not a number"

ENDIF

When  the function  is called,  all parameters  are calculated once and

are  then inserted into the function's  formula. This is done to reduce

calculation   overhead  and  to  avoid  side  effects.  The  individual

arguments  have  to  be  separated  by  commas when a function has more

than one parameter.

CAUTION!  Similar  to  macros,  one  can  use user-defined functions to

override  builtin  functions.  This  is  a  possible  source  for phase

errors.  Such  definitions  therefore  should  be done before the first

call!

The  result's type  may depend  on the  type of  the input arguments as

the  arguments are textually inserted  into the function's formula. For

example, the function

double  function x,x+x

may  have an integer,  a float, or  even a string  as result, depending

on the argument's type!

When  AS  operates  in  case-sensitive  mode,  the  case  matters  when

defining   or  referencing  user-defined   functions,  in  contrast  to

builtin functions!

3.5. Structures

---------------

valid for: all processors

Even  in assembly language  programs, there is  sometimes the necessity

to  define composed  data structures,  similar to high-level languages.

AS  supports both the definition and  usage of structures with a couple

of  statements. These  statements shall  be explained  in the following

section.

3.5.1. Definition

- - - - - - - - -

The  definiton of  a structure  is begun  with the statement STRUCT and

ends  with ENDSTRUCT (lazy  people may also  write STRUC resp. ENDSTRUC

or  ENDS  instead).  A  optional  label preceding these instructions is

taken  as the name  of the structure  to be defined;  it is optional at

the  end  of  the  definition  and  may  be used to redefine the length

symbol's   name  (see  below).  The   remaining  procedure  is  simple:

Together  with STRUCT, the  cuurent program counter  is saved and reset

to  zero.  All  labels  defined  between STRUCT and ENDSTRUCT therefore

are  the offsets  of the  structure's data  fields. Reserving  space is

done  via  the  same  instructions  that  are  also  otherwise used for

reserving  space, like  e.g. DS.x  for Motorola  CPUs or  DB &  co. for

Intel-style  processors. The  rules for  rounding up  lengths to assure

certain  alignments also apply  here - if  one wants to define 'packed'

structures,  a  preceding  PADDING  OFF  may  be necessary. Vice versa,

alignments may be forced with ALIGN or similar instructions.

Since  such a  definition only  represents a  sort of 'prototype', only

instructions  that  reserve  memory  may  be used, no instructions that

dispose constants or generate code.

Labels  defined inside  structures (i.e.  the elements'  names) are not

stored  as-is.  Instead,  the  structure's  name  is prepended to them,

separated  with a special  character. By default,  this is the underbar

(_).  This behaviour however may be  modified with two arguments passed

to the STRUCT statement:

- NOEXTNAMES  suppressed the  prepending of  the structure's name. In

this  case, it  is the  programmer's responsibility  to assure that

field names are not used more than once.

- DOTS  instructs AS to  use the dot  as connecting character instead

of  the underbar. It should however  be pointed out that on certain

target  architectures,  the  dot  has  a  special  meaning  for bit

addressing, which may lead to problems!

It  is futhermore possible to turn the usage  of a dot on resp. off for

all following structures:

dottedstructs <on|off>

Aside  from the  element names,  AS also  defines a further symbol with

the  structure's overall length when  the definition has been finished.

This  symbol  has  the  name  LEN,  which  is  being  extended with the

structure's  name via the same rules  - or alternitavely with the label

name given with the ENDSTRUCT statement.

In practice, this may things may look like in this example:

Rec     STRUCT

Ident   db      ?

Pad     db      ?

Pointer dd      ?

Rec     ENDSTRUCT

In this example, the symbol REC_LEN would be assigned the value 6.

3.5.2.  Usage

- - - - - - -

Once  a structure  has been  assigned, usage  is as  simple as possible

and similar to a macro: a simple

thisrec Rec

reserves  as  much  memory  as  needed  to  hold  an  instance  of  the

structure,  and additionally defines a symbol  for every element of the

structure  with its  address, in  this case THISREC_IDENT, THISREC_PAD,

and  THISREC_POINTER.  A  label  naturally  must  not  be  omitted when

calling a structure; if it is missing, an error will be emitted.

Additional  arguments  allow  to  reserve  memory  for a whole array of

structures.  The dimensions (up to three)  are defined via arguments in

square brackets:

thisarray Rec [10],[2]

In  this example,  space for  2*10=20 structures  is reserved. For each

individual  structure in the  array, proper symbols  are generated that

have the array indices in their name.

3.5.3.  Nested Structures

- - - - - - - - - - - - -

Is  is perfectly valid to call  an already defined structure within the

definition  of another  structure. The  procedure that  is taking place

then  is a combination  of the definition  and calling described in the

previous   two  sections:  elements  of   the  substructure  are  being

defined,  the name of the instance is  being prepended, and the name of

the   super-structure   is   once   again   geing   prepended  to  this

concatenated name. This may look like the following:

TreeRec struct

left    dd         ?

right   dd         ?

data    Rec

TreeRec endstruct

It   is  also  allowed  to  define  one  structure  inside  of  another

structure:

TreeRec struct

left    dd         ?

right   dd         ?

TreeData struct

name      db         32 dup(?)

id        dw         ?

TreeData endstruct

TreeRec endstruct

3.5.4. Unions

- - - - - - -

A  union is a special  form of a structure  whose elements are not laid

out  sequentially  in  memory.  Instead  all  elements  occupy the same

memory  and are located  at offset 0  in the structure. Naturally, such

a  definition basically does  nothing more than  to assign the value of

zero  to a couple of  symbols. It may however  be useful to clarify the

overlap  in a  program and  therefore to  make it  more 'readable'. The

size of a union is the maximum of all elements' lengths.

3.5.5.  Nameless Structures

- - - - - - - - - - - - - -

The  name of a structure or union is  optional if it is part of another

(named)  structure  or  union.  Elements  of  this  structure will then

become part of of the 'next higher' named structure. For example,

TreeRec struct

left    dd         ?

right   dd         ?

struct

name      db         32 dup(?)

id        dw         ?

endstruct

TreeRec endstruct

generates the symbols TREEREC_NAME and TREEREC_ID.

Futhermore,  no symbol holding  its length is  generated for an unnamed

structure or union.

3.5.6.  Structures and Sections

- - - - - - - - - - - - - - - -

Symbols  that  are  created  in  the  course  of  defining  or usage of

structures  are  treated  just  like  normal  symbols,  i.e.  when used

within  a section, these symbols are local  to the section. The same is

however  also  true  for  the  structures  themselves, i.e. a structure

defined within a section cannot be used outside of the section.

3.5.7.  Structures and Macros

- - - - - - - - - - - - - - -

If  one wants to instantiate structures via  macros, one has to use the

GLOBALSYMBOLS  options  when  defining  the  macro  to make the defined

symbols  visible outside the macro. For  instance, a list of structures

can be defined in the following way:

irp     name,{GLOBALSYMBOLS},rec1,rec2,rec3

name    Rec

endm

3.6. Conditional Assembly

-------------------------

valid for: all processors

The   assembler  supports   conditional  assembly   with  the  help  of

statements  like  IF...  resp.  SWITCH...  .  These  statements work at

assembly  time allowing  or disallowing  the assembly  of program parts

based  on conditions.  They are  therefore not  to be  compared with IF

statements  of  high-level  languages  (though  it would be tempting to

extend  assembly  language  with  structurization  statements of higher

level languages...).

The  following  constructs  may  be  nested arbitrarily (until a memory

overflow occurs).

3.6.1.  IF / ELSEIF / ENDIF

- - - - - - - - - - - - - -

IF  is the most common and  most versatile construct. The general style

of an IF statement is as follows:

IF      <expression 1>

.

.

<block 1>

.

.

ELSEIF  <expression 2>

.

.

<block 2>

.

.

(possibly more ELSEIFs)

.

.

ELSEIF

.

.

<block n>

.

.

ENDIF

IF  serves as an  entry, evaluates the  first expression, and assembles

block  1 if the  expression is true  (i.e. not 0).  All further ELSEIF-

blocks  will  then  be  skipped.  However,  if the expression is false,

block  1  will  be  skipped  and  expression  2  is  evaluated. If this

expression  turns out to be  true, block 2 is  assembled. The number of

ELSEIF  parts is variable  and results in  an IF-THEN-ELSE ladder of an

arbitrary  length.  The  block  assigned  to  the  last ELSEIF (without

argument)  only gets  assembled if  all previous  expressions evaluated

to  false; it  therefore forms  a 'default'  branch. It is important to

note  that only  one of  the blocks  will be  assembled: the  first one

whose IF/ELSEIF had a true expression as argument.

The  ELSEIF parts are optional, i.e. IF  may directly be followed by an

ENDIF. An ELSEIF without parameters must be the last branch.

ELSEIF  always  refers  to  the  innermost,  unfinished IF construct in

case IF's are nested.

In  addition to  IF, the  following further  conditional statements are

defined:

- IFDEF  <expressionl>: true if  the expression does  not contain any

symbols that are so far undefined.

- IFNDEF <expression>: counterpart to IFDEF.

- IFUSED  <symbol>: true if  if the given  symbol has been referenced

at least once up to now.

- IFNUSED <symbol>: counterpart to IFUSED.

- IFEXIST  <name>: true if the given  file exists. The same rules for

search  paths and syntax apply as  for the INCLUDE instruction (see

section 3.9.2).

- IFNEXIST <name>: counterpart to IFEXIST.

- IFB  <arg-list>: true  if all  arguments of  the parameter list are

empty strings.

- IFNB <arg-list>: counterpart to IFB.

It  is valid to write  ELSE instead of ELSEIF  since everybody seems to

be used to it...

For  everyIF...  statement,  there  has  to  be  a corresponding ENDIF.

'Open'  constructs  will  lead  to  an  error  message at the end of an

assembly  path. The way  AS has 'paired'  ENDIF statements with IFs may

be  deduced from  the assembly  listing: for  ENDIF, the line number of

the corresponding IF... will be shown.

3.6.2. SWITCH / CASE / ELSECASE / ENDCASE

- - - - - - - - - - - - - - - - - - - - -

CASE  is a special  case of IF  and is designed  for situations when an

expression  has to be compared  with a couple of  values. This could of

course also be done with a series of ELSEIFs, but the following form

SWITCH  <expression>

.

.

CASE    <value 1>

.

<block 1>

.

CASE    <value 2>

.

<block 2>

.

(further CASE blocks)

.

CASE    <value n-1>

.

<block n-1>

.

ELSECASE

.

<block n>

.

ENDCASE

has  the advantage  that the  expression is  only written once and also

only  gets  evaluated  once.  It  is  therefore  less  error-prone  and

slightly faster than an IF chain, but obviously not as flexible.

It  is possible  to specify  multiple values  separated by  commas to a

CASE  statement in  order to  assemble the  following block in multiple

cases.  The ELSECASE branch again serves as  a 'trap' for the case that

none  of the CASE conditions  was met. AS will  issue a warning in case

it is missing and all comparisons fail.

Even  when value  lists of  CASE branches  overlap, only  one branch is

executed, which is the first one in case of ambiguities.

SWITCH  only serves  to open  the whole  construct; an arbitrary number

of  statements  may  be  between  SWITCH  and the first CASE (but don't

leave  other  IFs  open!),  for  the  sake  of  better readability this

should however not be done.

In  case that SWITCH  is already a  machine instruction on the selected

processor  target, the  construct may  be openend  via SELECT,  or by a

leading  period to  explicitly invoke  the pseudo  instruction (.SWITCH

instead of SWITCH).

Similarly  to  IF  constructs,  there  must  be exactly one ENDCASE for

every  SWITCH. Analogous to  ENDIF, for ENDCASE  the line number of the

corresponding SWITCH is shown in the listing.

3.7. Listing Control

--------------------

valid for: all processors

3.7.1. PAGE, PAGESIZE

- - - - - - - - - - -

PAGE  is used to  tell AS the  dimensions of the  paper that is used to

print  the assembly listing. The first  parameter is thereby the number

of  lines after  which AS  shall automatically  output a form feed. One

should  however  take  into  account  that  this value does not include

heading  lines  including  an  eventual  line specified with TITLE. The

minimum  number  of  lines  is  5,  and  the  maximum  value  is 255. A

specification  of 0 has the  result that AS will  not do any form feeds

except  those triggered  by a  NEWPAGE instruction  or those implicitly

engaged  at the end of  the assembly listing (e.g.  prior to the symbol

table).

The   specification  of  the  listing's  length  in  characters  is  an

optional  second parameter  and serves  two purposes:  on the one hand,

the  internal line counter of AS will  continue to run correctly when a

source  line has  to be  split into  several listing  lines, and on the

other  hand there are  printers (like some  laser printers) that do not

automatically  wrap  into  a  new  line  at line end but instead simply

discard  the  rest.  For  this  reason,  AS does line breaks by itself,

i.e.  lines that are too  long are split into  chunks whose lengths are

equal  to or smaller than the specified  width. This may lead to double

line  feeds on  printers that  can do  line wraps  on their  own if one

specifies  the  exact  line  width  as  listing width. The solution for

such  a  case  is  to  reduce  the  assembly  listing's width by 1. The

specified  line  width  may  lie  between  5 and 255 characters; a line

width  of 0  means similarly  to the  page length  that AS shall not do

any  splitting  of  listing  lines;  lines  that are too long of course

cannot be taken into account of the form feed then any more.

The  default setting for the  page length is 60  lines, the default for

the  line width  is 0;  the latter  value is  also assumed when PAGE is

called with only one parameter.

In  case  PAGE  is  already  a  machine  instruction  on  the  selected

processor  target, use  instead PAGESIZE  to define  the paper size. As

an  alternative, it is always possible  to explicitly invoke the pseudo

instruction by prepending a period (.PAGE instead of PAGE).

CAUTION!  There  is  no  way  for  AS  to  check  whether the specified

listing length and width correspond to the reality!

3.7.2.  NEWPAGE

- - - - - - - -

NEWPAGE  can be used  to force a  line feed though  the current line is

not  full up to now. This might  be useful to separate program parts in

the  listing that  are logically  different. The  internal line counter

is  reset  and  the  page  counter  is incremented by one. The optional

parameter  is  in  conjunction  with  a  hierarchical page numbering AS

supports  up to  a chapter  depth of  4. 0  always refers to the lowest

depth,  and the  maximum value  may vary  during the assembly run. This

may look a bit puzzling, as the following example shows:

page 1,     instruction NEWPAGE 0 -> page 2

page 2,     instruction NEWPAGE 1 -> page 2.1

page 2.1,   instruction NEWPAGE 1 -> page 3.1

page 3.1,   instruction NEWPAGE 0 -> page 3.2

page 3.2,   instruction NEWPAGE 2 -> page 4.1.1

NEWPAGE  <number> may therefore result  in changes in different digits,

depending  on the current chapter depth.  An automatic form feed due to

a  line counter  overflow or  a NEWPAGE  without parameter  is equal to

NEWPAGE  0. Previous  to the  output of  the symbol  table, an implicit

NEWPAGE <maximum up to now> is done to start a new 'main chapter'.

3.7.3.  MACEXP_DFT and MACEXP_OVR

- - - - - - - - - - - - - - - - -

Once  a macro  is tested  and 'done',  one might  not want to see it in

the  listing  when  it  is  used.  As  described  in  the section about

defining  and using macros  (3.4.1), additional arguments  to the MACRO

statement  allow to  control whether  a macro's  body is  expanded upon

its  usage and if yes,  which parts of it.  In case that several macros

are  defined in  a row,  it is  not necessary  to give these directives

for  every single macro. The  pseudo instruction MACEXP_DFT defines for

all  following macros which parts shall  be expanded upon invocation of

the macro:

- ON resp. OFF enable or disable expansion completely.

- The  arguments  IF  resp.  NOIF  enable  or  disable  expansion  of

instructions  for conditional assembly, plus  the expansion of code

parts the were excluded because of conditional assembly.

- Macro  definitions (which includes  REPT, WHILE and  IRP(C)) may be

excluded  from or included in the  expanded parts via the arguments

MACRO resp. NOMACRO.

- All  other lines not  fitting into the  first two categories may be

excluded  from or included in the  expanded parts via the arguments

REST resp. NOREST.

The  default is  ON, i.e.  defined macros  will be expanded completely,

of   course  unless   specific  expansion   arguments  were   given  to

individual  macros.  Furthermore,  arguments  given  to MACEXP_DFT work

relative  to the current setting: for  instance, if expansion is turned

on completely initially, the statement

MACEXP_DFT  noif,nomacro

has  the result that for macros  defined in succession, only code parts

that   are  no  macro   definition  and  that   are  not  excluded  via

conditional assembly will be listed.

This  instruction  plus  the  per-macro directives provide fine-grained

per-macro  over the parts  being expanded. However,  there may be cases

in  practice where  one wants  to see  the expanded  code of a macro at

one  place  and  not  at  the  other.  This  is  possible  by using the

statement  MACEXP_OVR: it  accepts the  same arguemnts like MACEXP_DFT,

these  however act  as overrides  for all  macros being expanded in the

following  code.  This  is  in  contrast to MACEXP_DFT which influences

macros  being  defined  in  the  following  code.  For instance, if one

defined  for  a  macro  that  neither macro definitions nor conditional

assembly shall be expanded in the listing, a

MACEXP_OVR  MACRO

re-enables  expansion of  macro definitions  for its  following usages,

while a

MACEXP_OVR  ON

forces   expansion  of  the   complete  macro  body   in  the  listing.

MACEXP_OVR  without  arguments  again  disables  all  overrides, macros

will again behave as individually specified upon definition.

Both  statements  also  have  an  effect on other macro-like constructs

(REPT,  IRP, IRPC  WHILE). However,  since these  are expanded only one

and  ,,in-place'', the  functional difference  of these  two statements

becomes   minimal.  In  case  of  differences,  the  override  set  via

MACEXP_OVR has a higher priority.

The  Setting  currently  made  via  MACEXP_DFT  may  be  read  from the

predefined  symbol MACEXP.  For backward  compatibility reasons,  it is

possible  to use the  statement MACEXP instead  of MACEXP_DFT. However,

one should not make use of this in new programs.

3.7.4.  LISTING

- - - - - - - -

works  like MACEXP  and accepts  the same  parameters, but is much more

radical: After a

listing off   ,

nothing  at all  will be  written to  the listing. This directive makes

sense  for  tested  code  parts  or  include  files  to  avoid  a paper

consumption  going beyond all bounds. CAUTION!  If one forgets to issue

the  counterpart somewhere  later, even  the symbol  table will  not be

written  any  more!  In  addition  to  ON and OFF, LISTING also accepts

NOSKIPPED  and  PURECODE  as  arguments.  Program  parts  that were not

assembled  due  to  conditional  assembly  will  not  be written to the

listing  when NOSKIPPED is set, while  PURECODE - as the name indicates

-  even suppresses the  IF directives themselves  in the listing. These

options  are useful if  one uses macros  that act differently depending

on  parameters  and  one  only  wants  to  see  the  used  parts in the

listing.

The  current setting may be read  from the symbol LISTING (0=OFF, 1=ON,

2=NOSKIPPED, 3=PURECODE).

3.7.5.  PRTINIT and PRTEXIT

- - - - - - - - - - - - - -

Quite  often it  makes sense  to switch  to another printing mode (like

compressed  printing)  when  the  listing  is  sent to a printer and to

deactivate  this mode again  at the end  of the listing.  The output of

the  needed control sequences can  be automated with these instructions

if  one specifies the sequence that shall  be sent to the output device

prior   to  the  listing  with   PRTINIT  <string>  and  similarly  the

deinitialization  string with  PRTEXIT <string>.  <string> has  to be a

string   expression  in  both  cases.   The  syntax  rules  for  string

constants  allow to insert  control characters into  the string without

too much tweaking.

When  writing the listing,  the assembler does  not differentiate where

the  listing actually  goes, i.e.  printer control  characters are sent

to the screen without mercy!

Example:

For  Epson  printers,  it  makes  sense  to  switch  them to compressed

printing because listings are so wide. The lines

prtinit "\15"

prtexit "\18"

assure  that the compressed mode  is turned on at  the beginning of the

listing and turned off afterwards.

3.7.6.  TITLE

- - - - - - -

The  assembler normally adds a header line  to each page of the listing

that  contains the source  file's name, date,  and time. This statement

allows  to extend the page header  by an arbitrary additional line. The

string that has to be specified is an arbitrary string expression.

Example:

For  the Epson printer  already mentioned above,  a title line shall be

written  in  wide  mode,  which  makes  it  necessary  to  turn off the

compressed mode before:

title   "\18\14Wide Title\15"

(Epson  printers automatically turn off  the wide mode at  the end of a

line.)

3.7.7.  RADIX

- - - - - - -

RADIX  with  a  numerical  argument  between  2 and 36 sets the default

numbering  system  for  integer  constants,  i.e.  the numbering system

used  if nothing  else has  been stated  explicitly. The default is 10,

and  there  are  some  possible  pitfalls  to  keep  in  mind which are

described in section 2.10.1.

Independent  of the  current setting,  the argument  of RADIX is always

decimal;  furthermore, no symbolic  or formula expressions  may be used

as argument. Only use simple constant numbers!

If  the  IM61x0  is  the  current  target, the instructions DECIMAL and

OCTAL are available as shortforms for RADIX 10 respectively RADIX 8.

3.7.8. OUTRADIX

- - - - - - - -

OUTRADIX  can in a  certain way be  regarded as the  opposite to RADIX:

This  statement allows to  configure which numbering  system to use for

integer  results when  \{...} constructs  are used  in string constants

(see  section 2.10.3). Valid arguments range  again from 2 to 36, while

the default is 16.

3.8. Local Symbols

------------------

valid for: all processors

local  symbols  and  the  section  concept  introduced  with them are a

completely  new  function  that  was  introduced with version 1.39. One

could  say that  this part  is version  ''1.0'' and  therefore probably

not  the  optimum.  Ideas  and  (constructive)  criticism are therefore

especially  wanted. I admittedly described the  usage of sections how I

imagined  it.  It  is  therefore  possible  that  the  reality  is  not

entirely  equal to  the model  in my  head. I  promise that  in case of

discrepancies,  changes  will  occur  that  the reality gets adapted to

the  documentation  and  not  vice  versa  (I  was told that the latter

sometimes takes place in larger companies...).

AS  does not generate linkable code  (and this will probably not change

in  the near  future :-().  This fact  forces one  to always assemble a

program  in a whole.  In contrast to  this technique, a separation into

linkable modules would have several advantages:

- shorter  assembly times  as only  the modified  modules have  to be

reassembled;

- the   option  to  set  up   defined  interfaces  among  modules  by

definition of private and public symbols;

- the  smaller length  of the  individual modules  reduces the number

of  symbols per module  and therefore allows  to use shorter symbol

names that are still unique.

Especially  the last  item was  something that  always nagged  me: once

there  was  a  label's  name  defined  at the beginning of a 2000-lines

program,  there  was  no  way  to  reuse  it  somehow - even not at the

file's  other end  where routines  with a  completely different context

were placed. I was forced to use concatenated names in the style of

<subprogram name>_<symbol name>

that  had  lengths  ranging  from  15  to  25  characters  and made the

program  difficult  to  overlook.  The  concept of section described in

detail  in the following text was designed  to cure at least the second

and  third item of  the list above.  It is completely  optional: if you

do  not want to use  sections, simply forget them  and continue to work

like you did with previous versions of AS.

3.8.1.  Basic Definition (SECTION/ENDSECTION)

- - - - - - - - - - - - - - - - - - - - - - -

A  section  represents  a  part  of  the  assembler program enclosed by

special statements and has a unique name chosen by the programmer:

.

.

<other code>

.

.

SECTION <section's name>

.

.

<code inside of the section>

.

.

ENDSECTION [section's name]

.

.

<other code>

.

.

The  name of  a section  must conform  to the  conventions for s symbol

name;  AS stores section  and symbol names  in separate tables which is

the  reason why a  name may be  used for a  symbol and a section at the

same  time. Section  names must  be unique  in a  sense that there must

not  be more than one  section on the same  level with the same name (I

will  explain in the  next part what  ''levels'' mean). The argument of

ENDSECTION  is optional, it may  also be omitted; if  it is omitted, AS

will   show  the  section's  name  that   has  been  closed  with  this

ENDSECTION.  Code inside a  section will be  processed by AS exactly as

if it were outside, except for three decisive differences:

- Symbols  defined within  a section  additionally get  an internally

generated  number that  corresponds to  the section.  These symbols

are  not  accessible  by  code  outside  the  section  (this can be

changed by pseudo instructions, later more about this).

- The  additional  attribute  allows  to  define  symbols of the same

name  inside  and  outside  the  section;  the  attribute  makes it

possible  to  use  a  symbol  name  multiple  times without getting

error messages from AS.

- If  a symbol of a certain name  has been defined inside and outside

of  a  section,  the  ''local''  one  will  be preferred inside the

section,  i.e. AS first  searches the symbol  table for a symbol of

the  referenced  name  that  also  was  assigned  to the section. A

search  for a global  symbol of this  name only takes  place if the

first search fails.

This  mechanism  e.g.  allows  to  split  the  code into modules as one

might  have done  it with  linkable code.  A more fine-grained approach

would  be to pack  every routine into  a separate section. Depending on

the  individual  routines'  lengths,  the  symbols for internal use may

obtain very short names.

AS  will by default  not differentiate between  upper and lower case in

section  names;  if  one  however  switches to case-sensitive mode, the

case will be regarded just like for symbols.

The  organization described  up to  now roughly  corresponds to what is

possible  in  the  C  language  that  places  all functions on the same

level.  However, as  my ''high-level''  ideal was  Pascal and  not C, I

went one step further:

3.8.2.  Nesting and Scope Rules

- - - - - - - - - - - - - - - -

It  is  valid  to  define  further  sections  within a section. This is

analog  to the  option given  in Pascal  to define  procedures inside a

procedure or function. The following example shows this:

sym     EQU        0

SECTION    ModuleA

SECTION    ProcA1

sym       EQU        5

ENDSECTION ProcA1

SECTION    ProcA2

sym       EQU        10

ENDSECTION ProcA2

ENDSECTION ModuleA

SECTION    ModuleB

sym      EQU        15

SECTION    ProcB

ENDSECTION ProcB

ENDSECTION ModuleB

When  looking up a symbol,  AS first searches for  a symbol assigned to

the  current  section,  and  afterwards  traverses  the  list of parent

sections  until the  global symbols  are reached.  In our  example, the

individual  sections see the  values given in  table 3.1 for the symbol

sym:

+---------+-------+-----------------+

| section | value | from section... |

+---------+-------+-----------------+

+---------+-------+-----------------+

| Global  | 0     | Global          |

+---------+-------+-----------------+

| ModuleA | 0     | Global          |

+---------+-------+-----------------+

| ProcA1  | 5     | ProcA1          |

+---------+-------+-----------------+

| ProcA2  | 10    | ProcA2          |

+---------+-------+-----------------+

| ModuleB | 15    | ModuleB         |

+---------+-------+-----------------+

| ProcB   | 15    | ModuleB         |

+---------+-------+-----------------+

Table 3.1: Valid values for the Individual Sections

This  rule can be  overridden by explicitly  appending a section's name

to  the  symbol's  name.  The  section's  name  has  to  be enclosed in

brackets:

move.l  #sym[ModulB],d0

Only  sections  that  are  in  the  parent  section path of the current

section  may be used.  The special values  PARENT0..PARENT9 are allowed

to  reference the  n-th ''parent''  of the  current section; PARENT0 is

therefore  equivalent  to  the  current  section  itself,  PARENT1  the

direct  parent and so on.  PARENT1 may be abbreviated  as PARENT. If no

name is given between the brackets, like in this example:

move.l  #sym[],d0 ,

one  reaches the global  symbol. CAUTION! If  one explicitly references

a  symbol from a  certain section, AS  will only seek  for symbols from

this  section,  i.e.  the  traversal  of  the  parent  sections path is

omitted!

Similar   to  Pascal,  it  is  allowed  that  different  sections  have

subsections  of  the  same  name;  the  principle  of  locality  avoids

irritations.  One  should  IMHO  still  use  this  feature as seldom as

possible:  Symbols listed in the symbol  resp. cross reference list are

only  marked  with  the  section  they  are  assigned  to, not with the

''section  hierarchy'' lying above them  (this really would have busted

the  available space);  a differentiation  is made  very difficult this

way.

As  a  SECTION  instruction  does  not  define  a  label by itself, the

section  concept  has  an  important  difference to Pascal's concept of

nested  procedures: a  pascal procedure  can automatically  ''see'' its

subprocedures(functions),  AS  requires  an  explicit  definition of an

entry point. This can be done e.g. with the following macro pair:

proc    MACRO   name

SECTION name

name    LABEL   $

ENDM

endp    MACRO   name

ENDSECTION name

ENDM

This  example also shows  that the locality  of labels inside macros is

not  influenced  by  sections.  It  makes  the  trick  with  the  LABEL

instruction necessary.

This  does of  course not  solve the  problem completely.  The label is

still  local and  not referencable  from the  outside. Those  who think

that  it  would  suffice  to  place  the  label in front of the SECTION

statement  should be quiet  because they would  spoil the bridge to the

next theme:

3.8.3.  PUBLIC and GLOBAL

- - - - - - - - - - - - -

The  PUBLIC statement allows to change the  assignment of a symbol to a

certain  section. It  is possible  to treat  multiple symbols  with one

statement,  but  I  will  use  an  example  with only one symbol in the

following  (not  hurting  the  generality  of  this discussion). In the

simplest  case, one  declares a  symbol to  be global,  i.e. it  can be

referenced from anywhere in the program:

PUBLIC  <name>

As  a  symbol  cannot  be  moved  in  the symbol table once it has been

sorted  in, this  statement has  to appear  before the symbol itself is

defined.  AS stores  all PUBLICs  in a  list and  removes an entry from

this  list when the  corresponding symbol is  defined. AS prints errors

at  the  end  of  a  section  in  case  that  not all PUBLICs have been

resolved.

Regarding  the hierarchical section  concept, the method  of defining a

symbol  as purely global looks extremely  brute. There is fortunately a

way  to  do  this  in  a  bit  more  differentiated way: by appending a

section name:

PUBLIC  <name>:<section>

The  symbol will  be assigned  to the  referenced section and therefore

also  becomes accessible for all its  subsections (except they define a

symbol  of the  same name  that hides  the ''more  global'' symbol). AS

will  naturally protest if  several subsections try  to export a symbol

of  same name to  the same level.  The special PARENTn values mentioned

in  the  previous  section  are  also  valid  for <section> to export a

symbol  exactly n  levels up  in the  section hierarchy. Otherwise only

sections  that are  parent sections  of the  current section  are valid

for  <section>. Sections that  are in another  part of the section tree

are  not  allowed.  If  several  sections  in  the  parent section path

should  have the  same name  (this is  possible), the lowest level will

be taken.

This tool lets the abovementioned macro become useful:

proc    MACRO   name

SECTION name

PUBLIC  name:PARENT

name    LABEL   $

ENDM

This  setting is equal  to the Pascal  model that also  only allows the

''father'' to see its children, but not the ''grandpa''.

AS  will quarrel about double-defined symbols  if more than one section

attempts  to  export  a  symbol  of  a  certain  name to the same upper

section.  This  is  by  itself  a  correct  reaction,  and one needs to

''qualify''  symbols  somehow  to  make  them  distinguishable if these

exports  were  deliberate.  A  GLOBAL  statement  does  just  this. The

syntax  of GLOBAL  is identical  to PUBLIC,  but the symbol stays local

instead  of being assigned to a  higher section. Instead, an additional

symbol  of the  same value  but with  the subsection's name appended to

the  symbol's  name  is  created,  and  only this symbol is made public

according  to the section specification. If  for example two sections A

and  B  both  define  a  symbol  named  SYM and export it with a GLOBAL

statement  to their  parent section,  the symbols  are sorted  in under

the names A_SYM resp. B_SYM .

In  case that source and target section  are separated by more than one

level, the complete name path is prepended to the symbol name.

3.8.4.  FORWARD

- - - - - - - -

The  model  described  so  far  may  look  beautiful,  but  there is an

additional  detail not present in Pascal  that may spoil the happiness:

Assembler  allows forward  references. Forward  references may  lead to

situations  where AS  accesses a  symbol from  a higher  section in the

first  pass. This is  not a disaster  by itself as  long as the correct

symbol  is  used  in  the  second  pass, but accidents of the following

type may happen:

loop:   .

<code>

.

.

SECTION sub

.               ; ***

.

bra.s   loop

.

.

loop:   .

.

ENDSECTION

.

.

jmp     loop    ; main loop

AS  will take the global label loop  in the first pass and will quarrel

about  an  out-of-branch  situation  if  the  program part at <code> is

long  enough. The second  pass will not  be started at  all. One way to

avoid  the  ambiguity  would  be  to  explicitly  specify  the symbol's

section:

bra.s   loop[sub]

If  a local  symbol is  referenced several  times, the  brackets can be

saved  by using a  FORWARD statement. The  symbol is thereby explicitly

announced  to  be  local,  and  AS  will  only look in the local symbol

table  part  when  this  symbol  is  referenced.  For  our example, the

statement

FORWARD loop

should be placed at the position marked with ***.

FORWARD  must not  only be  stated prior  to a symbol's definition, but

also  prior to its first usage in a  section to make sense. It does not

make  sense  to  define  a  symbol  private  and  public;  this will be

regarded as an error by AS.

3.8.5.  Performance Aspects

- - - - - - - - - - - - - -

The  multi-stage lookup in  the symbol table  and the decision to which

section  a symbol  shall be  assigned of  course cost  a bit of time to

compute.  An 8086  program of  1800 lines  length for example took 34.5

instead  of 33 seconds after a  modification to use sections (80386 SX,

16MHz,  3  passes).  The  overhead  is  therefore  limited.  As  it has

already  been stated at  the beginning, is  is up to  the programmer if

(s)he wants to accept it. One can still use AS without sections.

3.9. Miscellaneous

------------------

3.9.1. SHARED

- - - - - - -

valid for: all processors

This  statement  instructs  AS  to  write  the  symbols  given  in  the

parameter  list  (regardless  if  they  are  integer,  float  or string

symbols)  together with  their values  into the  share file. It depends

upon  the  command  line  parameters  described  in section 2.4 whether

such  a file is generated at all and  in which format it is written. If

AS  detects this instruction and no  share file is generated, a warning

is the result.

CAUTION!  A comment possibly  appended to the  statement itself will be

copied  to  the  first  line  outputted  to the share file (if SHARED's

argument  list is empty, only  the comment will be  written). In case a

share  file is written  in C or  Pascal format, one  has to assure that

the  comment  itself  does  not  contain character sequences that close

the comment (''*/'' resp. ''*)''). AS does not check for this!

3.9.2.  INCLUDE

- - - - - - - -

valid for: all processors

This  instruction inserts the  file given as  a parameter into the just

as  if it would  have been inserted  with an editor  (the file name may

optionally  be  enclosed  with  ''  characters).  This  instruction  is

useful  to split  source files  that would  otherwise not  fit into the

editor or to create ''tool boxes''.

In  case that the  file name does  not have an  extension, an extension

of  INC is assumed in a firstg step.  Only if no sch file exists, or if

the  specified name contains  a period (and  therefore an extension), a

file of exactly this name is searched for.

The  assmebler  primarily  tries  to  open  the  file  in the directory

containing  the  source  file  with  the  INCLUDE statenemt. This means

that  a path  contained in  the file  specification is relative to this

file's  directory, not to the directory  the assembler was called from.

Via  the -i <path list>  option, one can specify  a list of directories

that  will automatically be searched  for the file. If  the file is not

found, a fatal error occurs, i.e. assembly terminates immediately.

For  compatibility reasons, it is valid to  enclose the file name in ''

characters, i.e.

include stddef51

and

include "stddef51.inc"

are  equivalent.  CAUTION!  This  freedom  of  choice is the reason why

only a string constant but no string expression is allowed!

3.9.3. BINCLUDE

- - - - - - - -

valid for: all processors

BINCLUDE  can be used to embed  binary data generated by other programs

into  the code generated  by AS (this  might theoretically even be code

created by AS itself...). BINCLUDE has three forms:

BINCLUDE <file>

This way, the file is completely included.

BINCLUDE <file>,<offset>

This  way, the file's contents are  included starting at <offset> up to

the file's end.

BINCLUDE <file>,<offset>,<length>

This way, <length> bytes are included starting at <offset>.

The  same rules  regarding search  paths and  assumed suffixes apply as

for INCLUDE.

3.9.4. MESSAGE, WARNING, ERROR, and FATAL

- - - - - - - - - - - - - - - - - - - - -

valid for: all processors

Though  the assembler  checks source  files as  strict as  possible and

delivers  differentiated  error  messages,  it  might be necessary from

time  to  time  to  issue  additional  error  messages  that  allow  an

automatic  check for  logical error.  The assembler distinguishes among

three  different types  of error  messages that  are accessible  to the

programmer via the following three instructions:

- WARNING:  Errors that hint  at possibly wrong  or inefficient code.

Assembly continues and a code file is generated.

- ERROR:  True  errors  in  a  program.  Assembly  continues to allow

detection  of  possible  further  errors  in  the same pass. A code

file is not generated.

- FATAL:  Serious  errors  that  force  an  immediate  termination of

assembly. A code file may be generated but will be incomplete.

All  three  instructions  have  the  same  format  for the message that

shall  be  issued:  an  arbitrary  string  expression,  which  may be a

simple   string  constant,  but  as  well  a  complex  expression  that

evaluates  to a  string. It  also includes  the feature to embed symbol

values in strings, which is described in 2.7:

message "Start Address is \{start_address}"

Instructions  generating warnings  or errors  typically only make sense

in  conjunction  wit  conditional  assembly.  For  example, if there is

only  a limited address space for a  program, one can test for overflow

in the following way:

ROMSize equ     8000h   ; 27256 EPROM

ProgStart:

.

.

<the program itself>

.

.

ProgEnd:

if      ProgEnd-ProgStart>ROMSize

error  "\athe program is too long!"

endif

Apart  from  the  instructions  generating  errors,  there  is  also an

instruction  MESSAGE  that  simply  prints  a  message  to the assembly

listing  and  th  ecosole  (the  latter  only  if the quiet mode is not

used). Its usage is equal to the other three instructions.

3.9.5. READ

- - - - - -

valid for: all processors

One   could  say  that   READ  is  the   counterpart  to  the  previous

instruction  group: it allows  to read values  from the keyboard during

assembly.  You might ask what  this is good for.  I will break with the

previous  principles and  put an  example before  the exact description

to outline the usefulness of this instruction:

A  program needs for data  transfers a buffer of  a size that should be

set  at assembly time.  One could store  this size in  a symbol defined

with EQU, but it can also be done interactively with READ:

IF      MomPass=1

READ    "buffer size",BufferSize

ENDIF

Programs   can  this   way  configure   themselves  dynamically  during

assembly  and  one  could  hand  over  the  source  to  someone who can

assemble  it  without  having  to  dive  into  the  source code. The IF

conditional  shown  in  the  example  should  always  be  used to avoid

bothering the user multiple times with questions.

READ  is quite  similar to  SET with  the difference  that the value is

read  from the  keyboard instead  of the  instruction's arguments. This

for  example also implies  that AS will  automatically set the symbol's

type  (integer, float or string)  or that it is  valid to enter formula

expressions instead of a simple constant.

READ  may  either  have  one  or  two  parameters because the prompting

message  is  optional.  AS  will  print  a message constructed from the

symbol's name if it is omitted.

3.9.6.  INTSYNTAX

- - - - - - - - -

valid for: all processors

This  instruction allows  to modify  the set  of notations  for integer

constants  in  various  number  systems.  -  After  selection  of a CPU

target,  a certain default  set is installed  (see section E). This set

may  be augmented  with other  notations, or  notations may  be removed

from  it. INTSYNTAX takes  an arbitrary list  of arguments which either

begin  with  a  plus  or  minus  character,  followed by the notation's

identifier. For instance, the following statement

INTSYNTAX    -0oct,+0hex

has  the result that a  leading zero marks a  hexadecimal instead of an

octal  constant, a common  usage on some  assemblers for the SC/MP. The

identifiers  for all notations can  be found in table  2.7. There is no

limit  on combining notations, except  when they contradict each other.

For  instance, it would not  be allowed to enable  0oct and 0hex at the

same time.

3.9.7.  RELAXED

- - - - - - - -

valid for: all processors

By  default, AS  assigns a  distinct syntax  for integer constants to a

processor  family  (which  is  in  general  equal to the manufacturer's

specifications,   as  long  as  the  syntax  is  not  too  bizarre...).

Everyone  however has  his own  preferences for  another syntax and may

well  live with  the fact  that his  programs cannot  be translated any

more with the standard assembler. If one places the instruction

RELAXED ON

right  at the  program's beginning,  one may  furtherly use  any syntax

for  integer  constants,  even  mixed  in  a program. AS tries to guess

automatically  for  every  expression  the  syntax  that was used. This

automatism  does not always deliver the  result one might have in mind,

and  this  is  also  the  reason  why  this  option  has  to  be enable

explicitly:  if there are  no prefixes or  postfixes that unambiguously

identify  either Intel  or Motorola  syntax, the  C mode  will be used.

Leading  zeroes that are  superfluous in other  modes have a meaning in

this mode:

move.b  #08,d0

This  constant will be understood as  an octal constant and will result

in  an error message  as octal numbers  may only contain  digits from 0

to  7.  One  might  call  this  a  lucky  case; a number like 077 would

result  in trouble  without getting  a message  about this. Without the

relaxed   mode,   both   expressions   unambiguously  would  have  been

identified as decimal constants.

The current setting may be read from a symbol with the same name.

3.9.8. COMPMODE

- - - - - - - -

valid for: various processors

Though  the assember strives to  behave like the correspondig "original

assemblers",  there are  cases when  emulating the original assembler's

behaviour  would forbid code  optimizations which are  valid and useful

in my opinion. Use the statement

compmode on

to  switch  to  a  'compatibility  mode'  which  prioritizes  'original

behaviour'  to  most  efficient  code.  See the respective section with

processor-specific  hints  whether  there  are  any  situations for the

specific target.

Compatibility  mode is disabled by default,  unless it was activated by

the  command line switch of same name.  The current setting may be read

from a symbol with the same name.

3.9.9.  END

- - - - - -

valid for: all processors

END  marks  the  end  of  an  assembler  program. Lines that eventually

follow  in  the  source  file  will  be  ignored. IMPORTANT: END may be

called  from within a macro, but  the IF-stack for conditional assembly

is   not  cleared  automatically.  The  following  construct  therefore

results in an error:

IF      DontWantAnymore

END

ELSEIF

END  may optionally have  an integer expression  as argument that marks

the  program's entry  point. AS  stores this  in the  code file  with a

special record and it may be post-processed e.g. with P2HEX.

END  has always been  a valid instruction  for AS, but  the only reason

for  this  in  earlier  releases  of  AS  was compatibility; END had no

effect.

4. Processor-specific Hints

===========================

When  writing the individual  code generators, I  strived for a maximum

amount  of compatibility  to the  original assemblers.  However, I only

did  this as long as it did  not mean an unacceptable additional amount

of  work. I listed  important differences, details  and pitfalls in the

following chapter.

4.1. 6811

---------

''Where  can I buy such  a beast, a HC11  in NMOS?'', some of you might

ask.   Well,  of  course  it  does  not  exist,  but  an  H  cannot  be

represented  in a  hexadecimal number  (older versions  of AS would not

have  accepted such a name  because of this), and  so I decided to omit

all the letters...

''Someone  stating that something is  impossible should be at least

as cooperative as not to hinder the one who currently does it.''

From  time  to  time,  one  is  forced  to  revise one's opinions. Some

versions  earlier,  I  stated  at  his  place  that I couldn't use AS's

parser  in a way that it is  also possible to to separate the arguments

of  BSET/BCLR resp. BRSET/BRCLR with spaces.  However, it seems that it

can  do more than I wanted to  believe...after the n+1th request, I sat

down  once again  to work  on it  and things  seem to work now. You may

use  either  spaces  or  commas,  but  not  in  all  variants, to avoid

ambiguities:  for every  variant of  an instruction,  it is possible to

use  only commas or  a mixture of  spaces and commas  as Motorola seems

to  have defined it  (their data books  do not always  have the quality

of the corresponding hardware...):

Bxxx  abs8 #mask         is equal to Bxxx  abs8,#mask

Bxxx  disp8,X #mask      is equal to Bxxx  disp8,X,#mask

BRxxx abs8 #mask addr    is equal to BRxxx abs8,#mask,addr

BRxxx disp8,X #mask addr is equal to BRxxx disp8,X,#mask,addr

In  this list,  xxx is  a synonym  either for  SET or CLR; #mask is the

bit  mask to be applied  (the # sign is  optional). Of course, the same

statements are also valid for Y-indexed expression (not listed here).

With  the K4  version of  the HC11,  Motorola has  introduced a banking

scheme,  which  one  one  hand  easily  allows  to once again extend an

architecture  that has  become 'too  small', but  on the other hand not

really  makes  programmers'  and  tool  developers' lifes simpler...how

does   one  sensibly  map  something  like   this  on  a  model  for  a

programmer?

The  K4 architecture  extends the  HC11 address  space by 2x512 Kbytes,

which  means that  we now  have a  total address  space of 64+1024=1088

Kbytes.  AS acts  like this  were one  large unified addres space, with

the following layout:

- $000000...$00ffff: the old HC11 address space

- $010000...$08ffff: Window 1

- $090000...$10ffff: Window 2

Via  the ASSUME statement,  one tells AS  how the banking registers are

set  up, which  in turn  describes which  extended areas  are mapped to

which  physical addresses. For absolute  addresses modes with addresses

beyond  $10000, AS automatically computes  the address within the first

64K  that  is  to  be  used.  Of  course  this  only  works  for direct

addressing  modes, it  is the  programmer's responsibility  to keep the

overview for indirect or indexed addressing modes!

In  case one is  not really sure  if the current  mapping is really the

desired  one, the pseudo  instruction PRWINS may  be used, which prints

the  assumes MMxxx register contents  plus the current mapping(s), like

this:

MMSIZ $e1 MMWBR $84 MM1CR $00 MM2CR $80

Window 1: 10000...12000 --> 4000...6000

Window 1: 90000...94000 --> 8000...c000

An instruction

jmp     *+3

located  at  $10000  would  effectively  result  in  a  jump to address

$4003.

4.2. PowerPC

------------

Of  course,  it  is  a  bit  crazy  idea  to  add  support  in AS for a

processor  that  was  mostly  designed  for  usage  in  work  stations.

Remember  that AS  mainly is  targeted at  programmers of  single board

computers.  But things  that today  represent the  absolute high end in

computing  will be  average tomorrow  and maybe  obsolete the next day,

and  in the  meantime, the  Z80 as  the 8088  have been retired as CPUs

for   personal  computers  and  been  moved  to  the  embedded  market;

modified   versions   are   marketed   as  microcontrollers.  With  the

appearance  of the  MPC505 and  PPC403, my  suspicion has  proven to be

true  that  IBM  and  Motorola  try  to promote this architecture in as

many fields as possible.

However,  the  current  support  is  a bit incomplete: Temporarily, the

Intel-style  mnemonics are used  to allow storage  of data and the more

uncommon  RS/6000 machine  instructions mentioned  in [81]  are missing

(hopefully  noone  misses  them!).  I  will  finish  this  as  soon  as

information about them is available!

4.3. IBM PALM

-------------

IBM's  PALM  processor  has  been  ''Terra  Incognita''  for long time,

because  it never  has been  used outside  of IBM. Furthermore, the IBM

5100  to 5120  that were  equipped with  it were  exotic and expensive,

and  were  quickly  forgotten  over  the  success  of  the IBM PC. Only

Christian  Corti's extensive  reverse engineering  made it  possible to

implement this target [43].

When  Christian began  to reverse  engineer the  PALM processor, he did

not  know the assembler mnemonics  defined by IBM, so  he had to choose

his  own  ones.  He  of  course  did this with the background knowledge

about  decades  of  other  processor  architectures that were developed

from 1973 (when PALM was constructed) until today.

If  you compare his mnemonics with the  ones from IBM (a document about

them  was finally published  in [40]), I  see parallels to the assembly

language  of the Intel 8080/8085 on the  one side, and the Zilog Z80 on

the  other side. ''Intel Mnemonics'' pack  the addressing mode into the

mnemonic's  name (like  MVI for  ''MoVe Immediate''  or LDHD for ''LoaD

Halfword   Direct'').  This  is  significantly   easier  to  parse  and

transform into machine language for an assembler.

The  other  mnemonics  group  all  machine instructions doing a certain

operation  under the same mnemonic, like LD  for ''LoaD'' or MOVE for a

(16  bit) data  move. This  makes usage  for a programmer much simpler,

parsing  the different  addressing modes  however results  in some more

work for an assembler.

So,  both  sets  of  mnemonics  have  their justification: The IBM ones

simply  because  they  are  ''the  original''  ones  and are use in all

vendor  documentation, and  the new  ones because  they are simply more

understandable  and easier to use. I  therefore decided to support both

sets  in  my  assembler,  and  this  was  fortunately  possible without

creating  any  conflicts.  Support  includes the ''macro instructions''

CALL,  RCALL,  JMP,  BRA,  LWI  and  RET.  And there are a few things I

added myself:

-  AND  and OR also  accept an immediate  operand as second argument.

This  is mapped  to the  SET resp.  CLR instructions, and of course

the value gets inverted for AND.

-  MOVE  also accepts an  immediate argument as  source and generates

the same machine code as LWI. The same is true for MOVB and LBI.

Macro  instructions  consisting  of  more  than one (half) word however

create   a  new  problem:  The   only  form  of  conditional  execution

supported   by  the  PALM  processor  is  a  conditional  skip  of  the

following  instruction  word.  If  such  a  skip is followed by a macro

instruction,  it  would  only  partially  be  skipped. I have therefore

added  a small  state machine  that attempts  to detect  such sequences

and will issue a warning.

The  IBM 5110 and 5120 do not  use the ASCII character set, but instead

EBCDIC  as known from IBM mainframes.  The include subdirectory holds a

file  that  may  be  used  to  convert from ASCII to EBCDIC. IMPORTANT:

This  file defines  EBCDIC as  an extra  code page,  so the translation

has to be activated with the statement

codepage        cp037

One  more  word  about  integer  constant  syntax:  Christian Corti had

decided  to  use  the  ''Motorola  Syntax'', i.e. hexadecimal constants

must  be prefixed with a  dollar sign. As the  PALM is an IBM design, I

decided  to  use  the  ''IBM  Syntax''  by  default,  which  means that

numeric  constants are enclosed  in apostrophes and  prefixed with an X

for   hexadecimal   values.   To   assemble   the  code  examples  from

Christian's  pages without modifying them,  add the following statement

at the program's beginning:

intsyntax       +$hex,-x'hex'

4.4. DSP56xxx

-------------

Motorola,  which devil rode  you! Which person  in your company had the

''brilliant''  idea  to  separate  the  parallel  data  transfers  with

spaces!  In result,  everyone who  wants to  make his  code a  bit more

readable, e.g. like this:

move    x:var9 ,r0

move    y:var10,r3   ,

is  p****ed  because  the  space  gets  recognized  as  a separator for

parallel data transfers!

Well...Motorola  defined it  that way,  and I  cannot change  it. Using

tabs  instead of  spaces to  separate the  parallel operations  is also

allowed,  and  the  individual  operations'  parts  are again separated

with commas, as one would expect it.

[76]  states that  instead of  using MOVEC,  MOVEM, ANDI  or ORI, it is

also  valid  to  use  the  more  general  Mnemonics MODE, AND or OR. AS

(currently) does not support this.

4.5. H8/300

-----------

Regarding   the   assembler   syntax   of   these  processors,  Hitachi

generously   copied  from  Motorola  (that  wasn't  by  far  the  worst

choice...),  unfortunately  the  company  wanted  to  introduce its own

format  for hexadecimal numbers. To make it  even worse, it is a format

that uses unbalanced single quotes, just like Microchip does:

mov.w #h'ff,r0

This  format is  not supported  by default.  Instead, one  has to write

hexadecimal  numbers in the well-known  Motorola syntax: with a leading

dollar  sign.  If  you  really  need  the  'Hitachi  Syntax',  e.g.  to

assemble  existing code,  enable the  RELAXED mode.  Bear in  mind that

this  syntax  has  received  few  testing  so  far. I can therefore not

guarantee that it will work in all cases!

4.6. H8/500

-----------

The  H8/500's  MOV  instruction  features  an  interesting and uncommon

optimization:  If  the  target  operand  has  a  size of 16 bits, it is

still  possible  to  use  an  8-bit  (immediate)  source  operand.  For

example, for an instruction like this:

mov.w #$ffff,@$1234

it  is possible to encode  the immediate source code  operand just as a

single   $ff  and  to  save  one  byte  in  code  size.  The  processor

automatically  performs  a  sign  extension,  which  turns $ff into the

desired  value $ffff.  AS is  aware of  this optimization  and will use

it,  unless  it  was  explicitly  forbidden  via  a  :16  suffix at the

immediate operand.

Feedback  from  users  trying  to  assemble  existing code has revealed

that  the original Hitachi assembler  implements this optimization in a

different  way: it  assumes a  zero instead  of a  sign extension. This

means  that immediate  values from  0 to  255 ($0000  to $00ff) and not

from  -128 to  +127 ($ff80  to $007f)  are encoded  as one  byte. Tests

with  physical hardware brought the  result that the Programmers Manual

is   correct:  The  processor  performs   a  sign  extension.  AS  will

therefore  by default only use the  shorter encoding if a value ranging

from  -128 to  +127 respectively  $ff80 to  $007f is  used. If you have

existing  code that assumes values  from $80 to $ff  are encoded as one

byte,   you  may  activate  a   'compatibility  mode',  either  by  the

statement

compmode on

in the source code or by the command line switch of same name.

Aside  from this, the same  remarks regarding hexadecimal number syntax

apply as for H8/500.

4.7. SH7000/7600/7700

---------------------

Unfortunately,   Hitachi  once   again  used   their  own   format  for

hexadecimal  numbers, and once  again I was  not able to reproduce this

with AS...please use Motorola syntax!

When  using literals  and the  LTORG instruction,  a few things have to

be  kept in  mind if  you do  not want  to suddenly get confronted with

strange error messages:

Literals  exist due to  the fact that  the processor is  unable to load

constants  out of a range of -128  to 127 with immediate addressing. AS

(and  the  Hitachi  assembler)  hide  this  inability  by the automatic

placement  of constants  in memory  which are  then referenced  via PC-

relative  addressing. The question  that now arises  is where to locate

these  constants in memory. AS does  not automatically place a constant

in  memory  when  it  is  needed;  instead, they are collected until an

LTORG  instruction occurs. The  collected constants are  then dumped en

bloc,  and  their  addresses  are  stored  in ordinary labels which are

also visible in the symbol table. Such a label's name is of the form

LITERAL_s_xxxx_n  .

In  this name, s  represents the literal's  type. Possible values are W

for  16-bit  constants,  L  for  32-bit  constants  and  F  for forward

references  where  AS  cannot  decide  in  anticipation  which  size is

needed.  In case of  s=W or L,  xxxx denotes the  constant's value in a

hexadecimal  notation,  whereas  xxxx  is  a  simple running number for

forward  references  (in  a  forward  reference,  one does not know the

value  of a  constant when  it is  referenced, so  one obviously cannot

incorporate  its value  into the  name). n  is a counter that signifies

how  often a literal  of this value  previously occurred in the current

section.  Literals  follow  the  standard  rules  for  localization  by

sections.  It is therefore absolutely  necessary to place literals that

were generated in a certain section before the section is terminated!

The  numbering  with  n  is  necessary  because  a  literal  may  occur

multiple  times in a section. One reason for this situation is that PC-

relative  addressing only  allows positive  offsets; Literals that have

once  been placed with an LTORG can  therefore not be referenced in the

code  that  follows.  The  other  reason  is  that  the displacement is

generally limited in length (512 resp. 1024 bytes).

An  automatic LTORG at the end of  a program or previously to switching

to  a  different  target  CPU  does  not  occur; if AS detects unplaced

literals in such a situation, an error message is printed.

As  the PC-relative  addressing mode  uses the  address of  the current

instruction  plus 4,  it is  not possible  to access  a literal that is

stored directly after the instruction, like in the following example:

mov     #$1234,r6

ltorg

This  is a  minor item  since the  CPU anyway  would try to execute the

following  data as code.  Such a situation  should not occur  in a real

program...another  pitfall is far more  real: if PC-relative addressing

occurs  just behind  a delayed  branch, the  program counter is already

set  to  the  destination  address,  and  the  displacement is computed

relative  to the  branch target  plus 2.  Following is an example where

this detail leads to a literal that cannot be addressed:

bra     Target

mov     #$12345678,r4        ; is executed

.

.

ltorg                        ; here is the literal

.

.

Target: mov     r4,r7                ; execution continues here

As   Target+2  is  on  an  address   behind  the  literal,  a  negative

displacement  would result. Things become  especially hairy when one of

the  branch instructions JMP, JSR, BRAF, or  BSRF is used: as AS cannot

calculate  the  target  address  (it  is  generated  at  runtime from a

register's  contents), a  PC value  is assumed  that should  never fit,

effectively disabling any PC-relative addressing at this point.

It  is not possible to deduce the  memory usage from the count and size

of  literals. AS might  need to insert  a padding word  to align a long

word  to an address that  is evenly divisible by  4; on the other hand,

AS  might reuse  parts of  a 32-bit  literal for other 16-bit literals.

Of  course multiple use of  a literal with a  certain value will create

only  one entry. However, such  optimizations are completely suppressed

for  forward  references  as  AS  does  not  know  anything about their

value.

As  literals use the  PC-relative addressing which  is only allowed for

the  MOV  instruction,  the  usage  of  literals is also limited to MOV

instructions.  The way  AS uses  the operand  size is  a bit  tricky: A

specification  of a  byte or  word move  means to generate the shortest

possible  instruction that results  in the desired  value placed in the

register's  lowest 8  resp. 16  bits. The  upper 24  resp. 16  bits are

treated  as ''don't care''.  However, if one  specifies a longword move

or  omits  the  size  specification  completely,  this  means  that the

complete   32-bit  register  should  contain  the  desired  value.  For

example, in the following sequence

mov.b   #$c0,r0

mov.w   #$c0,r0

mov.l   #$c0,r0   ,

the  first instruction  will result  in true  immediate addressing, the

second  and third instruction will use a  word literal: As bit 7 in the

number  is set, the byte instruction  will effectively create the value

$FFFFFFC0  in the register. According  to the convention, this wouldn't

be  the desired value in the second  and third example. However, a word

literal  is also  sufficient for  the third  case because the processor

will copy a cleared bit 15 of the operand to bits 16..31.

As  one can see, the  whole literal stuff is  rather complex; I'm sorry

but  there was no chance of  making things simpler. It is unfortunately

a  part  of  its  nature  that  one sometimes gets error messages about

literals  that  were  not  found,  which  logically  should  not  occur

because   AS  does  the  literal  processing  completely  on  his  own.

However,  if  other  errors  occur  in  the  second pass, all following

labels  will move because  AS does not  generate any code  any more for

statements  that have  been identified  as erroneous.  As literal names

are  partially  built  from  other  symbols' values, other errors might

follow  because literal names  searched in the  second pass differ from

the  names stored  in the  first pass  and AS  quarrels about undefined

symbols...if  such  errors  should  occur,  please  correct  all  other

errors first before you start cursing on me and literals...

People  who come out of the Motorola  scene and want to use PC-relative

addressing  explicitly  (e.g.  to  address  variables  in  a  position-

independent  way) should know  that if this  addressing mode is written

like in the programmer's manual:

mov.l   @(Var,PC),r8

no  implicit conversion  of the  address to  a displacement will occur,

i.e.  the operand  is inserted  as-is into  the machine code (this will

probably  generate  a  value  range  error...).  If you want to use PC-

relative  addressing on the SH7x00,  simply use ''absolute'' addressing

(which does not exist on machine level):

mov.l   Var,r8

In  this  example,  the  displacement  will be calculated correctly (of

course,  the same limitations apply for  the displacement as it was the

case for literals).

4.8. HMCS400

------------

The  instruction set of  these 4 bit  processors spontaneously reminded

me  of  the  8080/8085  -  many  mnemonics,  the  addressing mode (e.g.

direct   or  indirect)   is  coded   into  the   instruction,  and  the

instructions  are  sometimes  hard  to  memorize. AS or course supports

this  syntax  as  Hitachi  defined  it.  I  however implemented another

variant  for  most  instructions  that  is  -  in  my  opinion  -  more

beautiful  and better  to read.  The approach  is similar to what Zilog

did  back  then  for  the  Z80.  For instance, all machine instructions

that  transfer  data  in  some  form,  may  the  operands be constants,

registers,  or  memory  cells,  may  be  used  via  the LD instruction.

Similar   'meta   instructions'   exist   for  arithmetic  and  logical

instructions.  A  complete  list  of  all  meta  instructions and their

operands  can be found in  the tables 4.8 and  4.8, their practical use

can be seen in the file t_hmcs4x.asm.

+--------------------+------------------------------------+

| Meta Instruction   | Replaces                           |

+--------------------+------------------------------------+

| LD src, dest       | LAI, LBI, LMID, LMIIY,             |

|                    | LAB, LBA, LAY, LASPX, LASPY, LAMR, |

|                    | LWI, LXI, LYI, LXA, LYA, LAM, LAMD |

|                    | LBM, LMA, LMAD, LMAIY, LMADY       |

| XCH src, dest      | XMRA, XSPX, XSPY, XMA, XMAD, XMB   |

| ADD src, dest      | AYY, AI, AM, AMD                   |

| ADC src, dest      | AMC, AMCD                          |

| SUB src, dest      | SYY                                |

| SBC src, dest      | SMC, SMCD                          |

| OR src, dest       | OR, ORM, ORMD                      |

| AND src, dest      | ANM, ANMD                          |

| EOR src, dest      | EORM, EORMD                        |

| CP cond, src, dest | INEM, INEMD, ANEM, ANEMD, BNEM,    |

|                    | YNEI, ILEM, ILEMD, ALEM, ALEMD,    |

|                    | BLEM, ALEI                         |

| BSET bit           | SEC, SEM, SEMD                     |

| BCLR bit           | REC, REM, REMD                     |

| BTST bit           | TC, TM, TMD                        |

+--------------------+------------------------------------+

Table 4.1: Meta Instructions HMCS400

+-----------+------------------------------------+

| Operand   | Types                              |

+-----------+------------------------------------+

| src, dest | A, B, X, Y, W, SPX, SPY (register) |

|           | M (memory addressed by X/Y/W)      |

|           | M+ (ditto, with auto increment)    |

|           | M- (ditto, with auto decrement)    |

|           | #val (2/4 bits immediate)          |

|           | addr10 (memory direct)             |

|           | MRn (memory register 0..15)        |

| cond      | NE (unequal)                       |

|           | LE (less or equal)                 |

| bit       | CA (carry)                         |

|           | bitpos,M                           |

|           | bitpos,addr10                      |

| bitpos    | 0..3                               |

+-----------+------------------------------------+

Table 4.2: Operand Types for HMCS400 Meta Instructions

4.9. H16

--------

The  instruction  set  of  the  H16's  core  well  deserves  the  label

''CISC'':   complex   addressing   modes,   instructions  of  extremely

variable  length, and there  are many shortforms  for instructions with

common  operands.  For  instance,  several  instructions know different

''formats'',  depending on the type  of source and destination operand.

The  general  rule  is  that  AS  will always use the shortest possible

format, unless it was specified explicitly: angegeben:

mov.l     r4,r7     ; uses R format

mov.l     #4,r7     ; uses RQ format

mov.l     #4,@r7    ; uses Q format

mov.l     @r4,@r7   ; uses G format

mov:q.l   #4,r7     ; forces Q instead of RQ format

mov:g.l   #4,r7     ; forces G instead of RQ format

For  immediate arguments, the ''natural'  argument length is used, e.g.

2  bytes for 16 bits.  Shorter or longer arguments  may be forced by an

appended  operand size (.b, .w, .l or  :8, :16, :32). However, the rule

for  displacements  and  absolute  addresses  is that the shortest form

will  be used  if no  explicit size  is given. This includes exploiting

that  the  processor  does  not  output  the uppermost eight bits of an

address.  Therefore, an absolute  address of $ffff80  can be coded as a

single byte ($80).

Furthermore,   AS  knows  the  ''accumulator  bit'',  i.e.  the  second

operand   of  a  two-operand  instruction  my   be  left  away  if  the

destination  is  register  zero.  There  is  currently no override this

behaviour.

Additionally, the following optimizations are performed:

- MOV  R0,<ea>  gets  optimized  to  MOVF  <ea>, unless <ea> is a PC-

relative   expression  and  the  size  of  the  displacement  would

change.   This  optimization  may  be  disabled  by  specifying  an

explicit format.

- SUB  does not support the  Q format, however it  may be replaced by

ADD:Q  with a negated immediate argument,  given the argument is in

the  range -127...+128. This  optimization may as  well be disabled

by specifying an explicit format.

4.10. OLMS-40

-------------

Similar  to  the  HMCS400,  addressing  modes  are  largely encoded (or

rather  encrypted..) into into  the mnemonics, and  also here I decided

to  provide an  alternate notation  that is  more modern  and better to

read.  A complete list of all  meta instructions and their operands can

be  found in the tables 4.10 and  4.10, their practical use can be seen

in the file t_olms4.asm.

+------------------+----------------------------------------+

| Meta Instruction | Replaces                               |

+------------------+----------------------------------------+

| LD dest, src     | LAI, LLI, LHI, L,                      |

|                  | LAL, LLA, LAW, LAX, LAY, LAZ,          |

|                  | LWA, LXA, LYA, LPA, LTI, RTH, RTL      |

| DEC dest         | DCA, DCL, DCM, DCW, DCX, DCY, DCZ, DCH |

| INC dest         | INA, INL, INM, INW, INX, INY, INZ      |

| BSET bit         | SPB, SMB, SC                           |

| BCLR bit         | RPB, RMB, RC                           |

| BTST bit         | TAB, TMB, Tc                           |

+------------------+----------------------------------------+

Table 4.3: Meta Instructions OLMS-40

+-----------+---------------------------------------------+

| Operand   | Types                                       |

+-----------+---------------------------------------------+

| src, dest | A, W, X, Y, Z, DPL, DPH (Register)          |

|           | T, TL, TH (Timer, obere/untere H"alfte)     |

|           | (DP), M (Speicher adressiert durch DPH/DPL) |

|           | #val (4/8 bit immediate)                    |

|           | PP (Port-Pointer)                           |

| bit       | C (Carry)                                   |

|           | (PP), bitpos                                |

|           | (DP), bitpos                                |

|           | (A), bitpos                                 |

| bitpos    | 0..3                                        |

+-----------+---------------------------------------------+

Table 4.4: Operand Types for OLMS-40 Meta Instructions

4.11. OLMS-50

-------------

The  data  memory  of  these  4  bit  controllers consists of up to 128

nibbles.   However,   only   a   very   small  subset  of  the  machine

instructions  have  enough  space  to  accomodate  seven  address bits,

which  menas that -  once again -  banking must help  out. The majority

of  instructions that address  memory only contain  the lower four bits

of  the RAM  address, and  unless the  lowest 16  nibbles of the memory

shall  be  addressed,  the  P  register  delivers  the  necessary upper

address bits. The assembler is told about its current value via an

assume  p:<value>

statement, e.g. directly after a PAGE instruction.

Speaking  of PAGE:  both PAGE  and SWITCH  are machine  instructions on

these  controllers, i.e. the do not  have the function known from other

targets.  The pseudo  instruction to  start a  SWITCH/CASE construct is

SELECT  in  OLMS-50  mode,  and  the  listing's  page  size  is set via

PAGESIZE.

4.12. MELPS-4500

----------------

The  program memory of these microcontrollers  is organized in pages of

128  words. Honestly said, this  organization only exists because there

are  on the one  hand branch instructions  with a target  that must lie

within  the same  page, and  on the  other hand  ''long'' branches that

can  reach  the  whole  address  space.  The standard syntax defined by

Mitsubishi  demands that page  number and offset  have to be written as

two  distinct arguments for  the latter instructions.  As this is quite

inconvenient  (except  for  indirect  jumps,  a programmer has no other

reason  to  deal  with  pages),  AS  also  allows  to  write the target

address in a ''linear'' style, for example

bl      $1234

instead of

bl      $24,$34 .

4.13. 6502UNDOC

---------------

Since  the 6502's undocumented instructions  naturally aren't listed in

any  data book, they shall be listed  shortly at this place. Of course,

you  are using them  on your own  risk. There is  no guarantee that all

mask  revisions will support all variants!  They anyhow do not work for

the   CMOS   successors   of   the   6502,  since  they  allocated  the

corresponding bit combinations with "official" instructions...

The following symbols are used:

&         binary AND

|         binary OR

^        binary XOR

<<        logical shift left

>>        logical shift right

<<<       rotate left

>>>       rotate right

<-        assignment

(..)      contents of ..

..        bits ..

A         accumulator

X,Y       index registers X,Y

S         stack pointer

An        accumulator bit n

M         operand

C         carry

PCH       upper half of program counter

Instruction      : JAM or KIL or CRS

Function         : none, prozessor is halted

Addressing Modes : implicit

Instruction      : SLO

Function         : M<-((M)<<1)|(A)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : ANC

Function         : A<-(A)&(M), C<- A7

Addressing Modes : immediate

Instruction      : RLA

Function         : M<-((M)<<1)&(A)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : SRE

Function         : M<-((M)>>1)^(A)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : ASR

Function         : A<-((A)&(M))>>1

Addressing Modes : immediate

Instruction      : RRA

Function         : M<-((M)>>>1)+(A)+(C)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : ARR

Function         : A<-((A)&(M))>>>1

Addressing Modes : immediate

Instruction      : SAX

Function         : M<-(A)&(X)

Addressing Modes : absolute long/short, Y-indexed short,

Y-indirect

Instruction      : ANE

Function         : M<-((A)&$ee)|((X)&(M))

Addressing Modes : immediate

Instruction      : SHA

Function         : M<-(A)&(X)&(PCH+1)

Addressing Modes : X/Y-indexed long

Instruction      : SHS

Function         : X<-(A)&(X), S<-(X), M<-(X)&(PCH+1)

Addressing Modes : Y-indexed long

Instruction      : SHY

Function         : M<-(Y)&(PCH+1)

Addressing Modes : Y-indexed long

Instruction      : SHX

Function         : M<-(X)&(PCH+1)

Addressing Modes : X-indexed long

Instruction      : LAX

Function         : A,X<-(M)

Addressing Modes : absolute long/short, Y-indexed long/short,

X/Y-indirect

Instruction      : LXA

Function         : X04<-(X)04&(M)04,

A04<-(A)04&(M)04

Addressing Modes : immediate

Instruction      : LAE

Function         : X,S,A<-((S)&(M))

Addressing Modes : Y-indexed long

Instruction      : DCP

Function         : M<-(M)-1, Flags<-((A)-(M))

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : SBX

Function         : X<-((X)&(A))-(M)

Addressing Modes : immediate

Instruction      : ISB

Function         : M<-(M)+1, A<-(A)-(M)-(C)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

4.14. MELPS-740

---------------

Microcontrollers  of  this  family  have  a  quite  nice, however well-

hidden  feature: If one sets bit 5  of the status register with the SET

instruction,  the  accumulator  will  be  replaced with the memory cell

addressed   by  the  X  register  for  all  load/store  and  arithmetic

instructions.  An attempt  to integrate  this feature  cleanly into the

assembly  syntax has not  been made so  far, so the  only way to use it

is  currently  the  ''hard''  way  (SET...instructions with accumulator

addressing...CLT).

Not  all  MELPS-740  processors  implement  all instructions. This is a

place  where  the  programmer  has  to  watch  out  for himself that no

instructions  are used that are unavailable for the targeted processor;

AS  does  not  differentiate  among  the  individual processors of this

family.  For  a  description  of  the  details  regarding  special page

addressing, see the discussion of the ASSUME instruction.

4.15. MELPS-7700/65816

----------------------

As  it seems,  these two  processor families  took disjunct development

paths,  starting from  the 6502  via their  8 bit predecessors. Shortly

listed, the following differences are present:

- The 65816 does not have a B accumulator.

- The 65816 does not have instructions to multiply or divide.

- The  65816 misses  the instructions  SEB, CLB,  BBC, BBS, CLM, SEM,

PSH,  PUL and  LDM. Instead,  the instructions  TSB, TRB, BIT, CLD,

SED, XBA, XCE and STZ take their places in the opcode table.

The  following  instructions  have  identical  function,  yet different

names:

+-------+------------++-------+------------+

| 65816 | MELPS-7700 || 65816 | MELPS-7700 |

+-------+------------++-------+------------+

+-------+------------++-------+------------+

|  REP  |     CLP    ||  PHK  |     PHG    |

|  TCS  |     TAS    ||  TSC  |     TSA    |

|  TCD  |     TAD    ||  TDC  |     TDA    |

|  PHB  |     PHT    ||  PLB  |     PLT    |

|  WAI  |     WIT    ||       |            |

+-------+------------++-------+------------+

Especially  tricky  are  the  instructions  PHB,  PLB  and  TSB:  these

instructions  have  a  totally  different  encoding and meaning on both

processors!

Unfortunately,  these processors address their memory  in a way that is

IMHO  even one level higher on  the open-ended chart of perversity than

the  Intel-like segmentation: They  do banking! Well,  this seems to be

the  price for the 6502 upward-compatibility;  before one can use AS to

write  code  for  these  processors,  one  has  to  inform AS about the

contents of several registers (using the ASSUME instruction):

The  M flag rules whether the accumulators  A and B should be used with

8  bits (1) or 16  bits (0) width. Analogously,  the X flag decides the

width  of the X  and Y index  registers. AS needs  this information for

the  decision  about  the  argument's  width  when immediate addressing

(#<constant>) occurs.

The  memory is organized  in 256 banks  of 64 KBytes.  As all registers

in  the CPU  core have  a maximum  width of  16 bits,  the upper 8 bits

have  to  be  fetched  from  2  special bank registers: DT delivers the

upper  8  bits  for  data  accesses,  and PG extends the 16-bit program

counter  to 24  bits. A  16 bits  wide register  DPR allows to move the

zero  page known from  the 6502 to  an arbitrary location  in the first

bank.  If AS  encounters an  address (it  is irrelevant if this address

is   part  of  an  absolute,  indexed,  or  indirect  expression),  the

following addressing modes will be tested:

1 Is  the address  in the  range of  DPR..DPR+$ff? If yes, use direct

addressing with an 8-bit address.

2 Is  the address contained in the  page addressable via DT (resp. PG

for  branch instructions)? If  yes, use absolute  addressing with a

16-bit address.

3 If nothing else helps, use long addressing with a 24-bit address.

As  one can see from this  enumeration, the knowledge about the current

values  of DT, PG and  DPR is essential for  a correct operation of AS;

if  the  specifications  are  incorrect,  the  program will probably do

wrong  addressing at  runtime. This  enumeration also  implied that all

three  address  lengths  are  available;  if  this is not the case, the

decision chain will become shorter.

The  automatic determination of the  address length described above may

be  overridden by  the usage  of prefixes.  If one prefixes the address

by  a <, >, or >> without a  separating space, an address with 1, 2, or

3  bytes of  length will  be used,  regardless if  this is  the optimal

length.  If one uses an  address length that is  either not allowed for

the  current  instruction  or  too  short  for  the  address,  an error

message is the result.

To  simplify porting of 6502 programs,  AS uses the Motorola syntax for

hexadecimal  constants  instead  of  the  Intel/IEEE syntax that is the

format  preferred by Mitsubishi for their  740xxx series. I still think

that  this is the  better format, and  it looks as  if the designers of

the  65816 were of the same  opinion (as the RELAXED instruction allows

the  alternative use of  Intel notation, this  decision should not hurt

anything).  Another  important  detail  for  the porting of programs is

that  it is valid to  omit the accumulator A  as target for operations.

For  example, it  is possible  to simply  write LDA  #0 instead  of LDA

A,#0.

A  real goodie  in the  instruction set  are the instructions MVN resp.

MVP  to do block transfers.  However, their address specification rules

are  a bit strange: bits 0--15 are stored in index registers, bits 16--

23  are  part  of  the  instruction.  When  one  uses  AS,  one  simply

specifies  the  full  destination  and  source  addresses. AS will then

automatically  grab  the  correct  bits.  This  is a fine yet important

difference  Mitsubishi's assembler where you  have to extract the upper

8  bits on your own. Things become  really convenient when a macro like

the following is used:

mvpos   macro   src,dest,len

if      MomCPU=$7700

lda    #len

elseif

lda    #(len-1)

endif

ldx     #(src&$ffff)

ldy     #(dest&$ffff)

mvp     dest,src

endm

Caution,  possible pitfall:  if the  accumulator contains  the value n,

the  Mitsubishi  chip  will  transfer  n  bytes,  but  the  65816  will

transfer n+1 bytes!

The  PSH and  PUL instructions  are also  very handy because they allow

to  save  a  user-defined  set  to  be  saved  to the stack resp. to be

restored  from the stack.  According to the  Mitsubishi data book [62],

the  bit  mask  has  to  be  specified  as an immediate operand, so the

programmer  either has to  keep all bit<->register  assignments in mind

or  he has to define some  appropriate symbols. To make things simpler,

I  decided to  extend the  syntax at  this point:  It is valid to use a

list  as argument which  may contain an  arbitrary sequence of register

names or immediate expressions. Therefore, the following instructions

psh     #$0f

psh     a,b,#$0c

psh     a,b,x,y

are  equivalent.  As  immediate  expressions  are still valid, AS stays

upward compatible to the Mitsubishi assemblers.

One  thing I  did not  fully understand  while studying  the Mitsubishi

assembler  is the  treatment of  the PER  instruction: this instruction

allows  to  push  a  16-bit  variable  onto  the stack whose address is

specified  relative  to  the  program  counter.  Therefore,  it  is  an

absolute   addressing  mode  from  the   programmer's  point  of  view.

Nevertheless,  the Mitsubishi assembler  requests immediate addressing,

and  the instructions argument is placed  into the code just as-is. One

has  to calculate the  address in his  own, which is something symbolic

assemblers   were  designed  for   to  avoid...as  I   wanted  to  stay

compatible,   AS  contains  a  compromise:  If  one  chooses  immediate

addressing  (with a leading  # sign), AS  will behave like the original

from  Mitsubishi. But if the  # sign is omitted,  as will calculate the

difference  between  the  argument's  value  and  the  current  program

counter and insert this difference instead.

A  similar situation  exists for  the PEI  instruction that  pushes the

contents  of a  16-bit variable  located in  the zero  page: Though the

operand  represents  an  address,  once  again  immediate addressing is

required.  In this case, AS will  simply allow both variants (i.e. with

or without a # sign).

4.16. M16

---------

The  M16 family is a  family of highly complex  CISC processors with an

equally  complicated  instruction  set.  One  of  the instruction set's

properties  is the  detail that  in an  instruction with  two operands,

both  operands may be  of different sizes.  The method of appending the

operand  size as an  attribute of the  instruction (known from Motorola

and  adopted  from  Mitsubishi)  therefore  had  to  be extended: it is

valid  to append  attributes to  the operands  themselves. For example,

the following instruction

mov     r0.b,r6.w

reads  the lowest 8  bits of register  0, sign-extends them  to 32 bits

and  stores the result into  register 6. However, as  one does not need

this  feature in  9 out  of 10  cases, it  is still valid to append the

operand size to the instruction itself, e.g.

mov.w   r0,r6

Both  variants may be mixed;  in such a case,  an operand size appended

to   an   operand   overrules   the   ''default''.   An  exception  are

instructions  with two  operands. For  these instructions,  the default

for   the  source  operand  is  the  destination  operand's  size.  For

example, in the following example

mov.h   r0,r6.w

register  0 is accessed  with 32 bits,  the size specification appended

to  the instruction  is not  used at  all. If  an instruction  does not

contain   any  size  specifications,  word   size  (w)  will  be  used.

Remember:  in contrast to the 68000  family, this means 32 bits instead

of 16 bits!

The  chained addressing modes  are also rather  complex; the ability of

AS  to automatically  assign address  components to  parts of the chain

keeps  things at least halfway manageable.  The only way of influencing

AS  allows (the  original assembler  from Mitsubishi/Green Hills allows

a  bit more  in this  respect) is  the explicit setting of displacement

lengths by appending :4, :16 and :32.

4.17. CP-3F

-----------

The  CP-3F was  developed in  the early  1970, a  time when development

systems  were also much less powerful  than today. So when the assembly

language  for  a  new  processor  was  designed,  it  was also a design

target  that it  should be  easily translatable  into machine language.

So  the  CP-3F's  original  assembler  mnemonics group into few classes

that  can be  treated the  same by  an assembler:  instructions with an

immediate  argument of  3, 4,  or 8  bits, instructions with a register

operand,  branches and  instructions with  no argument  at all. This is

simple  to translate into  machine code, the  readability of the source

code  however leaves  to be  desired by  today's standards - comparable

to  Intel's  assembly  language  for  the  8080. I therefore decided to

provide  an  alternate  syntax  which  more  clearly  expresses what an

instruction does:

Table 4.5: Alternate Instruction Syntax for CP-3F

+----------+---------------+----------------------------+

| Original | Alternate     | Function                   |

+----------+---------------+----------------------------+

+----------+---------------+----------------------------+

| las imm4 | ld a,#<imm4   | A <-- 0:imm4               |

+----------+---------------+----------------------------+

| lal imm8 | ld a,#>imm8   | A <-- imm8                 |

+----------+---------------+----------------------------+

| lss imm3 | ld s,#imm3    | S <-- imm3                 |

+----------+---------------+----------------------------+

| lts imm3 | ld t,#imm3    | T <-- imm3                 |

+----------+---------------+----------------------------+

| anl imm8 | and [a,]#imm8 | A <-- A & imm8             |

+----------+---------------+----------------------------+

| eol imm8 | xor [a,]#imm8 | A <-- A ^ imm8             |

+----------+---------------+----------------------------+

| orl imm8 | or [a,]#imm8  | A <-- A | imm8             |

+----------+---------------+----------------------------+

| adl imm8 | add [a,]#imm8 | A <-- A + imm8             |

+----------+---------------+----------------------------+

| cml imm8 | cp [a,]#imm8  | Flags <-- A - imm8         |

+----------+---------------+----------------------------+

| lav      | ld a,v        | A <-- V                    |

+----------+---------------+----------------------------+

| law      | ld a,w        | A <-- W                    |

+----------+---------------+----------------------------+

| lax      | ld a,x        | A <-- X                    |

+----------+---------------+----------------------------+

| lay      | ld a,y        | A <-- Y                    |

+----------+---------------+----------------------------+

| sav      | ld v,a        | V <-- A                    |

+----------+---------------+----------------------------+

| saw      | ld w,a        | W <-- A                    |

+----------+---------------+----------------------------+

| sax      | ld x,a        | X <-- A                    |

+----------+---------------+----------------------------+

| say      | ld y,a        | Y <-- A                    |

+----------+---------------+----------------------------+

| sat      | ld t,a        | T <-- A                    |

+----------+---------------+----------------------------+

| sst      | ld st,a       | S|T <-- A                  |

+----------+---------------+----------------------------+

| als      | sla [a]       | A(7..1) <-- A(6..0),       |

|          | sla [a],1     | A(0) <-- 0                 |

+----------+---------------+----------------------------+

| ars      | srl [a]       | A(6..0) <-- A(7..1),       |

|          | srl [a],1     | A(7) <-- 0                 |

+----------+---------------+----------------------------+

| alf      | sla [a],4     | A(7..4) <-- A(3..0),       |

|          |               | A(3..0) <-- 0              |

+----------+---------------+----------------------------+

| arf      | srl [a],4     | A(3..0) <-- A(7..4),       |

|          |               | A(7..4) <-- 0              |

+----------+---------------+----------------------------+

| lar n    | ld a,n        | A <-- R(n), n=0..11        |

+----------+---------------+----------------------------+

| lar 12   | ld a,(st)     | A <-- R(S,T)               |

+----------+---------------+----------------------------+

| lar 13   | ld a,(st)-    | A <-- R(S,T),              |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| lar 14   | ld a,(st)+    | A <-- R(S,T),              |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| sar n    | ld n,a        | R(n) <-- A, n=0..11        |

+----------+---------------+----------------------------+

| sar 12   | ld (st),a     | R(S,T) <-- A               |

+----------+---------------+----------------------------+

| sar 13   | ld (st)-,a    | R(S,T) <-- A,              |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| sar 14   | ld (st)+,a    | R(S,T) <-- A,              |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| adr n    | add a,n       | A <-- A + R(n), n=0..11    |

+----------+---------------+----------------------------+

| adr 12   | add a,(st)    | A <-- A + R(S,T)           |

+----------+---------------+----------------------------+

| adr 13   | add a,(st)-   | A <-- A + R(S,T),          |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| adr 14   | add a,(st)+   | A <-- A + R(S,T),          |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| anr n    | and a,n       | A <-- A & R(n), n=0..11    |

+----------+---------------+----------------------------+

| anr 12   | and a,(st)    | A <-- A & R(S,T)           |

+----------+---------------+----------------------------+

| anr 13   | and a,(st)-   | A <-- A & R(S,T),          |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| anr 14   | and a,(st)+   | A <-- A & R(S,T),          |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| eor n    | xor a,n       | A <-- A ^ R(n), n=0..11    |

+----------+---------------+----------------------------+

| eor 12   | xor a,(st)    | A <-- A ^ R(S,T)           |

+----------+---------------+----------------------------+

| eor 13   | xor a,(st)-   | A <-- A ^ R(S,T),          |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| eor 14   | xor a,(st)+   | A <-- A ^ R(S,T),          |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| dec n    | dec a,n       | R(n) <-- R(n) - 1, n=0..11 |

+----------+---------------+----------------------------+

| dec 12   | dec a,(st)    | R(S,T) <-- R(S,T) - 1      |

+----------+---------------+----------------------------+

| dec 13   | dec a,(st)-   | R(S,T) <-- R(S,T) - 1,     |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| dec 14   | dec a,(st)+   | R(S,T) <-- R(S,T) - 1,     |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| six      | ld (z(x)),a   | (Z(module X)) <-- A        |

+----------+---------------+----------------------------+

| lix      | ld a,(z(x))   | A <-- (Z(module X))        |

+----------+---------------+----------------------------+

| liy      | ld a,(z(y))   | A <-- (Z(module Y))        |

+----------+---------------+----------------------------+

| sqx      | ld q(x),a     | Q(module X) <-- X|A        |

+----------+---------------+----------------------------+

| sqy      | ld q(y),a     | Q(module Y) <-- Y|A        |

+----------+---------------+----------------------------+

| szx      | ld z(x),a     | Z(module X) <-- X|A        |

+----------+---------------+----------------------------+

| szy      | ld z(y),a     | Z(module Y) <-- Y|A        |

+----------+---------------+----------------------------+

4.18. 4004/4040

---------------

Thanks  to John  Weinrich, I  now have  the official  Intel data sheets

describing   these  'grandfathers'  of  all  microprocessors,  and  the

questions  about the  syntax of  register pairs  (for 8-bit operations)

have  been weeded out for  the moment: It is  RnRm with n resp. m being

even  integers in the range from 0 to E  resp. 1 to F. The equation m =

n + 1 must be fulfilled.

4.19. MCS-48

------------

The  maximum address  space of  these processors  is 4 Kbytes, resp. up

to  8  Kbytes  on  some  Philips  varaints.  This  address space is not

organized  in a  linear way  (how could  this be  on an  Intel CPU...).

Instead,  it is split into 2 banks of  2 Kbytes. The only way to change

the  program counter  from one  bank to  the other are the instructions

CALL  and JMP, by setting the most  significant bit of the address with

the instructions SEL MB0 to SEL MB3.

The  assembler may be informed about  the bank currently being selected

for jumps and calls, via an ASSUME statement:

\asname{}SUME MB:<0..3>

If  one tries to  jump to an  address in a  different bank, a warnig is

issued.

If  the  special  value  NOTHING  is  used  (this  is  by  the  way the

default),  an automatism uilt  into JMP and  CALL is activated. It will

insert  a SEL  MBx instruction  if the  current program counter and the

target  address are located  in different banks.  Explicit usage of SEL

MBx  instructions is no longer  necessary (though it remains possible),

and  it  might  interfere  with  this  mechanism, like in the following

example:

000:  SEL      MB1

JMP      200h

AS  assumes that the MB  flag is 0 and  therefore does not insert a SEL

MB0 instruction, with the result that the CPU jumps to address A00h.

Furthermore,  one should  keep in  mind that  a jump  instruction might

become longer (3 instead of 2 bytes).

4.20. MCS-51

------------

The   assembler  is   accompanied  by   the  files  STDDEF51.INC  resp.

80C50X.INC  that  define  all  bits  and  SFRs  of the processors 8051,

8052,  and 80515  resp. 80C501,  502, and  504. Depending on the target

processor  setting (made  with the  CPU statement),  the correct subset

will  be included.  Therefore, the  correct order  for the instructions

at the beginning of a program is

CPU     <processor type>

INCLUDE stddef51.inc   .

Otherwise,   the  MCS-51   pseudo  instructions   will  lead  to  error

messages.

As  the 8051 does not  have instructions to to  push the registers 0..7

onto  the  stack,  one  has  to  work with absolute addresses. However,

these  addresses depend on which register  bank is currently active. To

make  this situation a little bit  better, the include files define the

macro  USING that  accepts the  symbols Bank0...Bank3  as arguments. In

response,  the  macro  will  assign  the  registers'  correct  absolute

addresses  to the  symbols AR0..AR7.  This macro  should be  used after

every  change  of  the  register  banks.  The  macro  itself  does  not

generate any code to switch to the bank!

The  macro also  makes bookkeeping  about which  banks have  been used.

The   result  is  stored  in  the  integer  variable  RegUsage:  bit  0

corresponds  to bank  0, bit  1 corresponds  to bank  1. and  so on. To

output   its  contents  after  the   source  has  been  assembled,  use

something like the following piece of code:

irp       BANK,Bank0,Bank1,Bank2,Bank3

if        (RegUsage&(2^BANK))<>0

message   "bank \{BANK} has been used"

endif

endm

The   multipass  feature  introduced  with   version  1.38  allowed  to

introduce  the additional  instructions JMP  and CALL.  If branches are

coded  using these instructions, AS  will automatically use the variant

that  is optimal  for the  given target  address. The options are SJMP,

AJMP,  or LJMP for JMP  resp. ACALL or LCALL  for CALL. Of course it is

still  possible to  use these  variants directly,  in case one wants to

force a certain coding.

4.21. MCS-251

-------------

When  designing the 80C251, Intel really tried  to make the move to the

new  family as smooth  as possible for  programmers. This culminated in

the  fact that  old applications  can run  on the new processor without

having  to recompile  them. However,  as soon  as one  wants to use the

new  features, some  details have  to be  regarded which  may turn into

hidden pitfalls.

The  most important  thing is  the absence  of a distinct address space

for  bits  on  the  80C251.  All  SFRs  can  now  be addressed bitwise,

regardless  of their address.  Furthermore, the first  128 bytes of the

internal  RAM  are  also  bit  addressable.  This  has  become possible

because  bits  are  not  any  more  handled by a separate address space

that   overlaps  other  address  spaces.   Instead,  similar  to  other

processors,  bits  are  addressed  with  a two-dimensional address that

consists  of  the  memory  location  containing  the  bit and the bit's

location  in the byte. One result is  that in an expression like PSW.7,

AS  will do the  separation of address  and bit position itself. Unlike

to  the 8051,  it is  not any  more necessary  to explicitly generate 8

bit  symbols. This has the other  result that the SFRB instruction does

not  exist any more. If  it is used in  a program that shall be ported,

it may be replaced with a simple SFR instruction.

Furthermore,  Intel  cleaned  up  the  cornucopia  of different address

spaces  on the  8051: the  internal RAM  (DATA resp.  IDATA), the XDATA

space  and the former  CODE space were  unified to a  single CODE space

that  is now  16 Mbytes  large. The  internal RAM  starts at address 0,

the  internal ROM starts at address  ff0000h, which is the address code

has  to  be  relocated  to.  In  contrast,  the  SFRs  were  moved to a

separate  address  space  (which  AS  refers  to  as  the  IO segment).

However,  they have  the same  addresses in  this new  address space as

they  used to  have on  the 8051.  The SFR  instructions knows  of this

difference  and automatically assigns symbols to  either the DATA or IO

segment,  depending  on  the  target  processor.  As  there  is  no BIT

segment  any more,  the BIT  instruction operates completely different:

Instead  of  a  linear  address  ranging  from 0..255, a bit symbol now

contains  the byte's address in bit 0..7,  and the bit position in bits

24..26.  Unfortunately,  creating  arrays  of  flags  with  a  symbolic

address is not that simple any more: On an 8051, one simply wrote:

segment bitdata

bit1    db      ?

bit2    db      ?

or

defbit  macro   name

name    bit     cnt

cnt     set     cnt+1

endm

On a 251, only the second way still works, like this:

adr     set     20h     ; start address of flags

bpos    set     0       ; in the internal RAM

defbit  macro   name

name    bit     adr.bpos

bpos    set     bpos+1

if      bpos=8

bpos     set     0

adr      set     adr+1

endif

endm

Another  small detail: Intel now prefers CY  instead of C as a symbolic

name  for the carry,  so you might  have to rename  an already existing

variable  of the same  name in your  program. However, AS will continue

to  understand also  the old  variant when  using the instructions CLR,

CPL,  SETB, MOV,  ANL, or  ORL. The  same is  conceptually true for the

additional  registers R8..R15,  WR0..WR30, DR0..DR28,  DR56, DR60, DPX,

and SPX.

Intel  would like everyone  to write absolute  addresses in a syntax of

XX:YYYY,  where XX is a  64K bank in the  address space resp. signifies

addresses  in the  I/O space  with an  S. As  one might guess, I am not

amused  about  this,  which  is  why  it  is legal to alternitavely use

linear  addresses  in  all  places.  Only  the  S  for I/O addresses is

incircumventable, like in this case:

Carry   bit     s:0d0h.7

Without  the prefix,  AS would  assume an  address in the CODE segment,

and only the first 128 bits in this space are bit-addressable...

Like  for the 8051, the generic  branch instructions CALL and JMP exist

that  automatically choose the  shortest machine code  depending on the

address  layout. However, while JMP also may use the variant with a 24-

bit  address, CALL will not  do this for a  good reason: In contrast to

ACALL  and LCALL,  ECALL places  an additional  byte onto  the stack. A

CALL  instruction would  result where  you would  not know what it will

do. This problem does not exist for the JMP instructions.

There  is one  thing I  did not  understand: The  80251 is also able to

push  immediate operands onto the stack,  and it may push either single

bytes   or  complete  words.  However,  the  same  mnemonic  (PUSH)  is

assigned  to both variants -  how on earth should  an assembler know if

an instruction like

push    #10

shall  push a byte  or a word  containing the value  10? So the current

rule  is that PUSH always  pushes a byte; if  one wants to push a word,

simply use PUSHW instead of PUSH.

Another  well-meant advise:  If you  use the  extended instruction set,

be  sure  to  operate  the  processor  in  source  mode; otherwise, all

instructions  will become  one byte  longer! The  old 8051 instructions

that  will in  turn become  one byte  longer are  not a  big matter: AS

will   either  replace  them  automatically   with  new,  more  general

instructions  or  they  deal  with  obsolete addressing modes (indirect

addressing via 8 bit registers).

4.22. 8080/8085

---------------

As mentioned before, the statement

Z80SYNTAX <ON|OFF|EXCLUSIVE>

makes   it   possible   to   write   the  vast  majority  of  8080/8085

instructions  in  'Z80  style',  i.e.  with  less  mnemonics  but  with

operands  that  are  easier  to  understand. In non-exclusive mode, the

Z80  syntax  is  not  allowed  for  the following instructions, because

they conflict with existing 8080 mnemonics:

- CP  in 'Intel  syntax' means  'Call on  Positive', in  Zilog syntax

however  it means  'Compare'. If  you use  CP with a numeric value,

it  is not possible  for the assembler  to recognize whether a jump

to  an absolute  address or  a compare  with an  immediate value is

meant.  The assembler will generate a  jump in this case, since the

Intel  syntax has precedence  in case of  ambiguities. If one wants

the  comparison, one may  explicitly write down  the accumulator as

destination operand, e.g. CP A,12h instead of CP 12h.

- JP  in  Intel  syntax  means  'Jump  on  Positive', in Zilog syntax

however,  this  is  the  jump  instruction  in general. Conditional

jumps  in Zilog  syntax (JP  cond,addr) are  unambigious because of

the  two  arguments.  With  only  one  argument, the assembler will

however  always  generate  the  conditional  jump.  If  you want an

unconditional  jump to an  absolute address, you  still have to use

the Intel syntax ((JMP addr).

The  8085 supports the  instructions RIM and  SIM that are  not part of

the  Z80 instruction  set. They  may be  written in  'Z80 style'  as LD

A,IM resp. LD IM,A.

The  'generic jump' J  known from the  Z80 target is  also available if

Z80  syntax  is  activated.  However,  since  the  8080/8085  does  not

support relative jumps, J is always translated to JP.

4.23. 8085UNDOC

---------------

Similarly  to  the  Z80  or  6502,  Intel  did  not further specify the

undocumented   8085  instructions.   This  however   means  that  other

assemblers  might  use  different  mnemonics  for  the  same  function.

Therefore,  I will list the instructions  in the following. Once again,

usage  of these instructions is at one's  own risk - even the Z80 which

is  principally  upward  compatible  to  the  8085 uses the opcodes for

entirely different functions...

Instruction : DSUB [reg]

Z80 Syntax : SUB HL,reg

Function  : HL <- HL - reg

Flags     : CY, S, X5, AC, Z, V, P

Arguments : reg = B for BC (optional for non-Z80 syntax)

Instruction : ARHL

Z80 Syntax : SRA HL

Function  : HL,CY <- HL >> 1 (arithmetisch)

Flags     : CY

Arguments : none resp. fixed for Z80 syntax

Instruction : RDEL

Z80 Syntax : RLC DE

Function  : CY,DE <- DE << 1

Flags     : CY, V

Arguments : none resp. fixed for Z80 syntax

Instruction : LDHI d8

Z80 Syntax : ADD DE,HL,d8

Function  : DE <- HL + d8

Flags     : none

Arguments : d8 = 8-bit constant, registers fixed for Z80 syntax

Instruction : LDSI d8

Z80 Syntax : ADD DE,SP,d8

Function  : DE <- SP + d8

Flags     : none

Arguments : d8 = 8-bit constant, registers fixed for Z80 syntax

Instruction : RSTflag

Z80 Syntax : RST flag

Function  : restart to 40h if flag=1

Flags     : none

Arguments : flag = V for overflow bit

Instruction : SHLX [reg]

Z80 Syntax : LD (reg),HL

Function  : [reg] <- HL

Flags     : none

Arguments : reg = D/DE for DE (optional for non-Z80 syntax)

Instruction : LHLX [reg]

Z80 Syntax : LD HL,(reg)

Function  : HL <- [reg]

Flags     : none

Arguments : reg = D/DE for DE (optional for non-Z80 syntax)

Instruction : JNX5 addr

Z80 Syntax : JP NX5, addr

Function  : jump to addr if X5=0

Flags     : none

Arguments : addr = absolute 16-bit address

Instruction : JX5 addr

Z80 Syntax : JP X5,addr

Function  : jump to addr if X5=1

Flags     : none

Arguments : addr = absolute 16-bit address

X5  refers to the otherwise  unused bit 5 in  the processor status word

(PSW).

4.24. 8086..V35

---------------

Actually,  I had  sworn myself  to keep  the segment disease of Intel's

8086  out of  the assembler.  However, as  there was  a request  and as

students  are  more  flexible  than  the  developers  of this processor

obviously   were,  there  is   now  a  rudimentary   support  of  these

processors  in AS.  When saying,  'rudimentary', it  does not mean that

the  instruction  set  is  not  fully  covered. It means that the whole

pseudo  instruction stuff that  is available when  using MASM, TASM, or

something  equivalent does not exist. To put  it in clear words, AS was

not  primarily designed  to write  assembler programs  for PC's (heaven

forbid,   this  really  would  have   meant  reinventing  the  wheel!);

instead,  the development  of programs  for single-board  computers was

the main goal (which may also be equipped with an 8086 CPU).

For  die-hards who still want to write  DOS programs with AS, here is a

small list of things to keep in mind:

- Only COM files may be created.

- Only  use the  CODE segment,  and place  also all variables in this

segment.

- DOS  initializes  all  segment  registers  to  the code segment. An

ASSUME  DS:DATA,  SS:DATA  right  at  the  program's  beginning  is

therefore necessary.

- DOS  loads the  code to  a start  address of  100h. An  ORG to this

address is absolutely necessary.

- The  conversion to a binary  file is done with  P2BIN (see later in

this document), with an address filter of $-$.

For  these  processors,  AS  only  supports  a small programming model,

i.e.  there is one code segment with a  maximum of 64 Kbytes and a data

segment  of equal size for data (which  cannot be set to initial values

for  COM  files).  The  SEGMENT  instruction  allows  to switch between

these  two segments. From  this facts results  that branches are always

intrasegment  branches if  they refer  to targets  in this  single code

segment.  In  case  that  far  jumps  should  be  necessary,  they  are

possible  via CALLF or  JMPF with a  memory address or a Segment:Offset

value as argument.

Another  big  problem  of  these  processors is their assembler syntax,

which  is sometimes ambiguous and whose  exact meaning can then only be

deduced  by looking at  the current context.  In the following example,

either  absolute  or  immediate  addressing  may be meant, depending on

the symbol's type:

mov     ax,value

When  using AS,  an expression  without brackets  always is interpreted

as   immediate  addressing.  For  example,  when  either  a  variable's

address  or its  contents shall  be loaded,  the differences  listed in

table 4.5 are present between MASM and AS:

+-----------+--------------------+---------------+

| assembler | address            | contents      |

+-----------+--------------------+---------------+

+-----------+--------------------+---------------+

| MASM      | mov ax,offset vari | mov ax,vari   |

|           | lea ax,vari        | mov ax,[vari] |

|           | lea ax,[vari]      |               |

|           |                    |               |

| AS        | mov ax,vari        | mov ax,[vari] |

|           | lea ax,[vari]      |               |

+-----------+--------------------+---------------+

Table 4.5: Differences AS<->MASM Concerning Addressing Syntax

When  addressing via  a symbol,  the assembler  checks whether they are

assigned  to  the  data  segment  and  tries to automatically insert an

appropriate  segment  prefix.  This  happens  for  example when symbols

from  the code  segment are  accessed without  specifying a  CS segment

prefix.   However,  this  mechanism   can  only  work   if  the  ASSUME

instruction (see there) has previously been applied correctly.

The  Intel syntax  also requires  to store  whether bytes or words were

stored  at a symbol's address.  AS will do this  only when the DB resp.

DW  instruction is in the same source  line as the label. For any other

case,  the operand  size has  to be  specified explicitly with the BYTE

PTR,  WORD  PTR,...  operators.  As  long  as  a  register is the other

operator,  this may  be omitted,  as the  operand size  is then clearly

given by the register's name.

In  an 8086-based system,  the coprocessor is  usually synchronized via

via  the  processor's  TEST  input  line  which  is  connected  to  toe

coprocessor's  BUSY output line.  AS supports this  type of handshaking

by  automatically  inserting  a  WAIT  instruction  prior to every 8087

instruction.  If  this  is  undesired  for  any  reason, an N has to be

inserted after the F in the mnemonic; for example,

FINIT

FSTSW   [vari]

becomes

FNINIT

FNSTSW  [vari]

This variant is valid for all coprocessor instructions.

4.25. 8X30x

-----------

The  processors  of  this  family  have  been  optimized  for  an  easy

manipulation  of bit  groups at  peripheral addresses. The instructions

LIV  and RIV were  introduced to deal  with such objects  in a symbolic

fashion.   They  work   similar  to   EQU,  however   they  need  three

parameters:

1 the  address of  the peripheral  memory cell  that contains the bit

group (0..255);

2 the number of the group's first bit (0..7);

3 the length of the group, expressed in bits (1..8).

CAUTION!  The 8X30x  does not  support bit  groups that  span over more

than  one  memory  address.  Therefore,  the  valid value range for the

length  can be  stricter limited,  depending on  the start position. AS

does  not  perform  any  checks  at  this point, you simply get strange

results at runtime!

Regarding  the machine code, length and  position are expressed vis a 3

bit  field in the  instruction word and  a proper register number (LIVx

resp.  RIVx).  If  one  uses  a  symbolic object, AS will automatically

assign  correct  values  to  this  field,  but  it  is  also allowed to

specify  the length explicitly as a third  operand if one does not work

with  symbolic  objects.  If  AS  finds  such a length specification in

spite  of a  symbolic operand,  it will  compare both lengths and issue

an  error  if  they  do  not  match  (the same will happen for the MOVE

instruction  if two symbolic operands with different lengths are used -

the instruction simply only has a single length field...).

Apart  from the real machine instructions,  AS defines similarly to its

''idol''  MCCAP  some  pseudo  instructions  that  are  implemented  as

builtin macros:

- NOP is a shortform for MOVE AUX,AUX

- HALT is a shortform for JMP *

- XML ii is a shortform for XMIT ii,R12 (only 8X305)

- XMR ii is a shortform for XMIT ii,R13 (only 8X305)

- SEL  <busobj>  is  a  shortform  for  XMIT  <adr>,IVL/IVR,  i.e. it

performs the necessary preselection to access <busobj>.

The  CALL  and  RTN  instructions  MCCAP  also implements are currently

missing  due to  sufficient documentation.  The same  is true for a set

of  pseudo instructions to  store constants to  memory. Time may change

this...

4.26. XA

--------

Similar   to   its   predecessor   MCS/51,   but  in  contrast  to  its

'competitor'  MCS/251, the Philips XA has  a separate address space for

bits,  i.e. all  bits that  are accessible  via bit instructions have a

certain,  one-dimensional address which is  stored as-is in the machine

code.  However, I could not take  the obvious opportunity to offer this

third  address space (code  and data are  the other two)  as a separate

segment.  The reason  is that  - in  contrast to  the MCS/51 - some bit

addresses  are ambiguous:  bits with  an address  from 256 to 511 refer

to  the  bits  of  memory  cells  20h..3fh in the current data segment.

This  means that these  addresses may correspond  to different physical

bits,  depending on the  current state. Defining  bits with the help of

DC  instructions -  something that  would be  possible with  a separate

segment  - would not make too  much sense. However, the BIT instruction

still  exists  to  define  individual  bits  (regardless  if  they  are

located  in  a  register,  the  RAM  or  SFR  space)  that  can then be

referenced  symbolically. If the bit is  located in RAM, the address of

the  64K-bank is  also stored.  This way,  AS can  check whether the DS

register  has previously  be assigned  a correct  value with  an ASSUME

instruction.

In  contrast, nothing can  stop AS's efforts  to align potential branch

targets  to even addresses.  Like other XA  assemblers, AS does this by

inserting NOPs right before the instruction in question.

4.27. AVR

---------

In  contrast to the AVR assembler, AS  by default uses the Intel format

to  write hexadecimal  contants instead  of the  C syntax. All right, I

did  not look into the (free) AVR  assembler before, but when I started

with  the AVR  part, there  was hardly  mor einformation  about the AVR

than  a preliminary manual  describing processor types  that were never

sold...this problem can be solved with a simple RELAXED ON.

Optionally,  AS can generate so-called "object  files" for the AVRs (it

also  works  for  other  CPUs,  but  it  does  not  make  any sense for

them...).  These are  files containing  code and  source line info what

e.g.  allows a step-by-step execution on  source level with the WAVRSIM

simulator  delivered by  Atmel. Unfortunately,  the simulator  seems to

have   trouble  with   source  file   names  longer   than  approx.  20

characters:  Names  are  truncated  and/or  extended by strange special

characters  when the  maximum length  is exceeded.  AS therefore stores

file   name   specifications   in   object   files   without   a   path

specification.  Therefore, problems may arise  when files like includes

are not in the current directory.

A  small  specialty  are  machine  instructions  that have already been

defined  by Atmel as  part of the  architecture, but up  to now haven't

been  implemented in any  of the family's  members. The instructions in

question  are MUL, JMP, and CALL.  Considering the latter ones, one may

ask  himself  how  to  reach  the  4  Kwords large address space of the

AT90S8515  when the  'next best'  instructions RJMP  and RCALL can only

branch  up  to  2  Kwords  forward  or  backward.  The  trick  is named

'discarding  the upper address  bits' and described  in detail with the

WRAPMODE statement.

All  AVR targets  support the  optional CPU  argument CODESEGSIZE. Like

in this example,

cpu atmega8:codesegsize=0

it  may be  used to  instruct the  assembler to  treat the code segment

(i.e.  the internal flash  ROM) as being  organized in bytes instead of

16  bit words. This is  the view when the  LPM instruction is used, and

which  some other  (non Atmel)  assemblers use  in general.  It has the

advantage  that addresses  in the  CODE segment  need not be multiplied

by  two if used  for data accesses.  On the other  hand, care has to be

taken  that instructions do  not start on  an odd address  - this would

be  the  equivalent  of  an  instruction  occupying  fractions of flash

words.  The PADDING  option is  therefore enabled  by default, while it

remains  possible to define arrays of bytes  via multiple uses of DB or

DATA  without the  risk of  padding bytes  inserted in  between. Target

addresses   for  relative  and   absolute  branches  automatically  get

divided  by two in this ''byte  mode''. The default is the organizazion

in  16  bit  word  as  used  by  the original Atmel assembler. This may

explicitly be selected by using the argument codesegsize=1.

4.28. Z80, Z380

---------------

The  Z80  supports  two  types  of  jump  instructions:  relative ( JR)

supports  jump distances from -128 to  +127 bytes, while absolute jumps

(  JP)  allow  to  reach  the  complete  address space. AS additionally

supports  a  pseudo  instruction  J  which  automatically  selects  the

shortest  possible  variant,  depending  on  the target address and the

condition  (relative jumps  do not  support all  conditions). This also

applies  to  all  targets  'derived'  from  Z80,  like  Z80UNDOC, Z180,

RABBIT2000, and LR35902.

J  is also  offered for  the Z380,  but since  the Z380 supports larger

jump  distances,  absolute  jumps  will  only  be  used  if the largest

possible jump distance (+/- 8 MByte) no longer suffices.

This  instruction should  be used  with thought  and care, since JR and

JP  are  not  entirely  equivalent  in  function: code that exclusively

uses  relative  jumps  is  position-independent,  while  absolute jumps

require relocation.

4.29. Z80UNDOC

--------------

As  one might guess, Zilog did not  make any syntax definitions for the

undocumented  instructions;  furthermore,  not  everyone might know the

full  set. It  might therefore  make sense  to list all instructions at

this place:

Similar  to  the  Z380  and  eZ80,  it  is  possible to access the byte

halves  of IX and IY separately.  In detail, these are the instructions

that allow this:

INC Rx              LD R,Rx             LD  Rx,n

DEC Rx              LD Rx,R             LD  Rx,Ry

ADD/ADC/SUB/SBC/AND/XOR/OR/CP A,Rx

Rx  and Ry are synonyms for IXL, IXU,  IYL or IYU. Keep however in mind

that  in the case of LD Rx,Ry, both  registers must be part of the same

index register.

The  coding of shift  instructions leaves an  undefined bit combination

which  is now accessible as the  ttySL1, SLI, SLIA, or SLS instruction.

It  works  like  SLA  with  the  difference  of  entering  a 1 into bit

position  0. CAUTION!  Some sources  also name  this operation  SLL. It

decided  to  not  offer  this,  since  it is misleading: SLL translates

into  "shift  left  logically",  and  the  operation  performed by this

instruction  is  no  logical  left  shift.  If one should define SLL at

all,  then as  an alias  for SLA.  If you  have existing code that uses

SLL in the meaning of SL1/SLI, define it via a macro.

Like  all other  (docummented) shift  instructions, this  also works in

another undocumented variant:

SLIA    R,(XY+d)

SLIA    (XY+d),R

In  this  case,  R  is  an  arbitrary  8-bit  register (excluding index

register  halves...),  and  (XY+d)  is  a  normal indexed address. This

operation  has the  additional effect  of copying  the result  into the

register. This also works for the RES and SET instructions:

SET/RES R,n,(XY+d)

SET/RES n,(XY+d),R

Furthermore, two hidden I/O instructions exist:

IN      (C) resp. TSTI

OUT     (C),0

Their  operation  should  be  clear.  CAUTION! Noone can guarantee that

all  mask  revisions  of  the  Z80  execute these instructions, and the

Z80's  successors  will  react  with  traps  if  they find one of these

instructions. Use them on your own risk...

4.30. GB_Z80 resp. LR35902

--------------------------

The  LR35902 SoC used  in the original  Gameboy was developed by Sharp,

and   the   CPU   core   is   (probably)   the  same  as  in  the  SM83

microcontrollers.  Regarding  its  instruction  set,  it  is  somewhere

''half  way'' between 8080 and Z80,  however with its own omissions and

extensions.  Sharp of  course defined  an assembler  syntax for the new

instructions.  However,  variations  have  established  itself  in  the

''Gameboy  scene''. I  tried to  regard those  as well  (as far as I am

aware of them):

+---------------+---------------+------------------+

| Sharp         | Alternate     | Function         |

+---------------+---------------+------------------+

+---------------+---------------+------------------+

| LD A,(HLD)    | LD A,(HL-)    | A <-- (HL),      |

|               | LDD A,(HL)    | HL <-- HL-1      |

+---------------+---------------+------------------+

| LD A,(HLI)    | LD A,(HL+)    | A <-- (HL),      |

|               | LDI A,(HL)    | HL <-- HL+1      |

+---------------+---------------+------------------+

| LD (HLD),A    | LD (HL-),A    | (HL) <-- A,      |

|               | LDD (HL),A    | HL <-- HL-1      |

+---------------+---------------+------------------+

| LD (HLI),A    | LD (HL+),A    | (HL) <-- A,      |

|               | LDI (HL),A    | HL <-- HL+1      |

+---------------+---------------+------------------+

| LD A,(C)      | LD A,(FF00+C) | A <-- (0ff00h+C) |

|               | LDH A,(C)     |                  |

+---------------+---------------+------------------+

| LD (C),A      | LD (FF00+C),A | (0ff00h+C) <-- A |

|               | LDH (C),A     |                  |

+---------------+---------------+------------------+

| LD (FF00+n),A | LDH (n),A     | (0ff00h+n) <-- A |

+---------------+---------------+------------------+

| LD A,(FF00+n) | LDH A,(n)     | A <-- (0ff00h+n) |

+---------------+---------------+------------------+

| LDHL SP,d     | LD HL,SP+d    | HL <-- SP + d    |

+---------------+---------------+------------------+

| LDX A,(nn)    | LD A,(nn)     | A <-- (nn) ^1    |

+---------------+---------------+------------------+

| LDX (nn),A    | LD (nn),A     | (nn) <-- A ^1    |

+---------------+---------------+------------------+

| ^1 enforces 16 bit addressing                    |

+---------------+---------------+------------------+

4.31. Z380

----------

As  this  processor  was  designed  as  a  grandchild of the still most

popular  8-bit microprocessor, it  was a sine-qua-non  design target to

execute  existing Z80  programs without  modification (of  course, they

execute  a bit  faster, roughly  by a  factor of 10...). Therefore, all

extended  features can  be enabled  after a  reset by  setting two bits

which  are named XM (eXtended  Mode, i.e. a 32-bit  instead of a 16-bit

address  space) respectively  LW (long  word mode,  i.e. 32-bit instead

of  16-bit operands). One has to  inform AS about their current setting

with  the instructions EXTMODE  resp. LWORDMODE, to  enable AS to check

addresses  and constants against  the correct upper  limits. The toggle

between   32-  and   16-bit  instruction   of  course  only  influences

instructions  that are  available in  a 32-bit  variant. Unfortunately,

the  Z380  currently  offers  such  variants  only  for  load and store

instructions;  arithmetic can  only be  done in  16 bits.  Zilog really

should   do  something   about  this,   otherwise  the   most  positive

description  for  the  Z380  would  be  ''16-bit  processor with 32-bit

extensions''...

The  whole thing  becomes complicated  by the  ability to  override the

operand  size  set  by  LW  with  the instruction prefixes DDIR W resp.

DDIR  LW. AS  will note  the occurrence  of such  instructions and will

toggle  setting  for  the  instruction  following directly. By the way,

one  should never explicitly  use other DDIR  variants than W resp. LW,

as  AS will introduce them automatically  when an operand is discovered

that  is too  long. Explicit  usage might  puzzle AS. The automatism is

so powerful that in a case like this:

DDIR    LW

LD      BC,12345678h   ,

the   necessary  IW  prefix  will  automatically  be  merged  into  the

previous instruction, resulting in

DDIR    LW,IW

LD      BC,12345668h   .

The  machine code that was  first created for DDIR  LW is retracted and

replaced, which is signified with an R in the listing.

4.32. Z8, Super8, and eZ8

-------------------------

The  CPU core  contained in  the Z8  microcontrollers does  not contain

any  specific registers.  Instead, a  block of  16 consecutive cells of

the  internal address  space (contains  RAM and  I/O registers)  may be

used  as 'work  registers' and  be addressed  with 4-bit addresses. The

RP  registers define which  memory block is  used as work registers: on

a  classic Z8, bits 4 to  7 of RP define the  'offset' that is added to

a  4-bit work  register address  to get  a complete  8-bit address. The

Super8  core features two register pointers  (RP0 and RP1), which allow

mapping  the  lower  and  upper  half  of  work  registers  to separate

places.

Usually,   one  refers  to  work   registers  as  R0..R15  in  assembly

statements.  It is however  also posssible to  regard work registers as

an  efficient way  to address  a block  of memory addresses in internal

RAM.

The  ASSUME statement is used  to inform AS about  the current value of

RP.  AS is then  capable to automatically  decide whether an address in

internal  RAM may be  reached with a  4-bit or 8-bit  address. This may

be used to assign symbolic names to work registers:

op1     equ     040h

op2     equ     041h

srp     #040h

assume  rp:040h

ld      op1,op2         ; equal to ld r0,r1

Note  that though  the Super8  does not  have an  RP register (only RP0

and  RP1), RP as argument to ASSUME is  still allowed - it will set the

assumed  values  of  RP0  and  RP1  to  value resp. value+8, as the SRP

machine instruction does on the Super 8 core.

Opposed  to  the  original  Zilog  assembler,  it  is  not necessary to

explicitly   specify  'work   register  addressing'   with  a  prefixed

exclamation   mark.  AS  however  also  understands  this  syntax  -  a

prefixed  exclamation  mark  enforces  4-bit  addressing, even when the

address  does not  lie within  the 16-address  block defined  by RP (AS

will  issue  a  warning  in  that  case).  Vice  versa,  a  prefixed  >

character  enforces 8-bit  addressing even  when the  address is within

the current 16-address block.

The  eZ8  takes  this  'game'  to  the next level: the internal address

space  now has 12 instead  of 8 bits. To  assure compatibility with the

old  Z8 core,  Zilog placed  the additional  4 bits  in the  lower four

bits  of  RP.  For  instance,  an  RP  value  of 12h defines an address

window from 210h to 21fh.

At  the same  time, the  lower four  bits of  RP define a window of 256

addresses  that can  be addressed  with 8-bit  addresses. The mechanism

to  automatically select between 8-  and 12-bit addresses is analogous.

'Long'   12-bit  addresses   may  be   enforced  by   prefixing  two  >

characters.

4.33. Z8000

-----------

A Z8001/8003 may be operated in one of two modes:

-  Non-Segmented:  The memory address space  is limited to 64 KBytes,

and  all addresses  are 'simple'  linear 16  bit addresses. Address

registers   are  single   16  bit   registers  (Rn),  and  absolute

addresses within instructions are one byte long.

-  Segmented:  Memory is structured into up  to 128 segments of up to

64  KBytes size. Addresses consist of a  7 bit segment number and a

16   bit  offset.  Address  registers  are  register  pairs  (RRn).

Absolute  addresses  in  instructions  occupy  two  16  bit  words,

unless the offset is smaller than 256.

The  operation  mode  (segmented  or  non-segmented)  therefore  has an

influence  on the  generated code  and is  selected implicitly  via the

selected  processor type.  For instance,  if the  target is  a Z8001 in

non-segmented mode, use Z8002 as target.

However,  similar  to  the  8086,  there  is  no  'real'  support for a

segmented  memory model  in AS.  In segmented  mode, the segment number

is  simply interpreted  as the  upper seven  bits of a virtually linear

address  space. Though this is  not what Zilog intended,  it is the way

the segment number was used on the Z8001 if the system had no MMU.

AS  in general  implements the  Z8000 machine  instruction syntax as it

is  specified by  Zilog in  its manuals.  However, there are assemblers

that  support extensions or  variations of the  syntax. AS implements a

few of them as well:

4.33.1.  Conditions

- - - - - - - - - -

In   addition  to  the  conditions  defined  by  Zilog,  the  following

alternative names are defined:

+-----------+-------+---------------------------+

| Alternate | Zilog | Meaning                   |

+-----------+-------+---------------------------+

+-----------+-------+---------------------------+

| ZR        | Z     | Z = 1                     |

| CY        | C     | C = 1                     |

| LLE       | ULE   | (C OR Z) = 1              |

| LGE       | UGE   | C = 0                     |

| LGT       | UGT   | ((C = 0) AND (Z = 0)) = 1 |

| LLT       | ULT   | C = 1                     |

+-----------+-------+---------------------------+

4.33.2. Flags

- - - - - - -

SETFLG,  COMFLG  und  RESFLG  accept  the  following alternate names as

arguments:

+-----------+-------+------------+

| Alternate | Zilog | Meaning    |

+-----------+-------+------------+

+-----------+-------+------------+

| ZR        | Z     | Zero Flag  |

| CY        | C     | Carry Flag |

+-----------+-------+------------+

4.33.3. Indirect Addressing

- - - - - - - - - - - - - -

It  is valid  to write  Rn^ instead  of @Rn,  if the option AMDSyntax=1

was  given  to  the  CPU  statement.  If  an  I/O  address is addressed

indirectly, this option even allows to write just Rn.

4.33.4.  Direct versus Immediate Addressing

- - - - - - - - - - - - - - - - - - - - - -

The  Zilog syntax mandates that immediate  addressing has to be done by

prefixing   the  argument  with  a  hash  character.  However,  if  the

AMDSyntax=1  option  was  given  to  the  CPU  statement,  the  type of

argument  (label  or  constant)  decides  whether  immediate  or direct

addressing  is  to  be  used.  Immediate  addressing  may  be forced by

prefixing  the argument with a circumflex,  i.e. to load the address of

a label into a register.

4.34. TLCS-900(L)

-----------------

These  processors may run in  two operating modes: on  the one hand, in

minimum  mode, which  offers almost  complete source code compatibility

to  the Z80 and TLCS-90,  and on the other  hand in maximum mode, which

is  necessary to  make full  use of  the processor's  capabilities. The

main differences between these two modes are:

- width of the registers WA, BC, DE, and HL: 16 or 32 bits;

- number of register banks: 8 or 4;

- code address space: 64 Kbytes or 16 Mbytes;

- length of return addresses: 16 or 32 bits.

To  allow AS  to check  against the  correct limits,  one has to inform

him  about the current execution mode  via the MAXMODE instruction (see

there). The default is the minimum mode.

From  this follows  that, depending  on the  operating mode, the 16-bit

resp.  32-bit  versions  of  the  bank  registers  have  to be used for

addressing,  i.e. WA,  BC, DE  and HL  for the  minimum mode resp. XWA,

XBC,  XDE and XHL for the maximum  mode. The registers XIX..XIZ and XSP

are  always 32  bits wide  and therefore  always have  to to be used in

this   form  for  addressing;   in  this  detail,   existing  Z80  code

definitely  has  to  be  adapted  (not  including  that there is no I/O

space and all I/O registers are memory-mapped...).

Absolute   addresses  and  displacements  may  be  coded  in  different

lengths.  Without  an  explicit  specification,  AS will always use the

shortest   possible   coding.   This   includes   eliminating   a  zero

displacement,  i.e.  (XIX+0)  becomes  (XIX).  If  a  certain length is

needed,  it may be forced  by appending a suffix  (:8, :16, :24) to the

displacmenet resp. the address.

The  syntax chosen by  Toshiba is a  bit unfortunate in  the respect of

choosing  an single quote (') to  reference the previous register bank.

The  processor independent  parts of  AS already  use this character to

mark character constants. In an instruction like

ld      wa',wa   ,

AS  will  not  recognize  the  comma  for  parameter  separation.  This

problem  can be circumvented  by usage of  an inverse single quote (`),

for example

ld      wa`,wa

Toshiba  delivers an  own assembler  for the  TLCS-900 series (TAS900),

which is different from AS in the following points:

Symbol Conventions

..................

- TAS900   differentiates  symbol   names  only   on  the   first  32

characters.  In contrast,  AS always  stores symbol  names with the

full   length  (up  to  255  characters)  and  uses  them  all  for

differentiation.

- TAS900  allows  to  write  integer  constants  either in Intel or C

notation   (with  a  0  prefix  for   octal  or  a  0x  prefix  for

hexadecimal  constants).  By  default,  AS  only supports the Intel

notation.  With the help of the  RELAXED instruction, one also gets

the C notation (among other).

- AS   does  not  distinguish  between   upper  and  lower  case.  In

contrast,   TAS900  differentiates  between  upper-  and  lowercase

letters  in symbol names.  One needs to  engage the -u command line

option to force AS to do this.

Syntax

......

For  many instructions, the  syntax checking of  AS is less strict than

the  checking of TAS900.  In some (rare)  cases, the syntax is slightly

different.  These extensions  and changes  are on  the one hand for the

sake  of a better portability of existing  Z80 codes, on the other hand

they   provide  a  simplification  and   better  orthogonality  of  the

assembly syntax:

- In  the  case  of  LDA,  JP,  and  CALL,  TAS requires that address

expressions  like XIX+5  must not  be placed  in parentheses, as it

is  usually  the  case.  For  the  sake of better orthogonality, AS

requires  parentheses for LDA.  They are optional  if JP resp. CALL

are used with a simple, absolute address.

- In  the case of JP, CALL, JR, and  SCC, AS leaves the choice to the

programmer  whether to  explicitly write  out the default condition

T  (=  true)  as  first  parameter  or not. TAS900 in contrast only

allows  to use  the default  condition implicitly  (e.g. jp (xix+5)

instead of jp t,(xix+5)).

- For  the EX instruction,  AS allows operand  combinations which are

not  listed in [166]  but can be  reduced to a standard combination

by   swapping  the  operands.  Combinations  like  EX  f`,f  or  EX

wa,(xhl)  become  possible.  In  contrast,  TAS900  limits  to  the

'pure' combinations.

- AS  allows to  omit an  increment resp.  decrement of  1 when using

the   instructions  INC   and  DEC.   TAS900  instead   forces  the

programmer to explicit usage of '1'.

- The  similar is true for the  shift instructions: If the operand is

a  register, TAS900 requires  that even a  shift count of  1 has to

be  written  explicitly;  however,  when  the operand is in memory,

the  hardware  limits  the  shift  count  to  1  which  must not be

written  in  this  case.  With  AS,  a  shift  count of 1 is always

optional and valid for all types of operands.

Macro Processor

...............

The  macro processor  of TAS900  is an  external program  that operates

like  a preprocessor. It  consists of two  components: The first one is

a  C-like preprocessor, and the second  one is a special macro language

(MPL)  that reminds of high level  languages. The macro processor of AS

instead  is oriented towards ''classic''  macro assemblers like MASM or

M80 (both programs from Microsoft). It is a fixed component of AS.

Output Format

.............

TAS900  generates  relocatable  code  that  allows  to  link separately

compiled  programs  to  a  single  application.  AS  instead  generates

absolute  machine code  that is  not linkable.  There are  currently no

plans to extend AS in this respect.

Pseudo Instructions

...................

Due  to the  missing linker,  AS lacks  a couple of pseudo instructions

needed   for   relocatable   code   TAS900  implements.  The  following

instructions are available with equal meaning:

EQU, DB, DW, ORG, ALIGN, END, TITLE, SAVE, RESTORE

The  latter  two  have  an  extended  functionality for AS. Some TAS900

pseudo  instructions can  be replaced  with equivalent  AS instructions

(see table 4.6).

+--------------+---------------------+-----------------------------+

| TAS900       | AS                  | meaning/function            |

+--------------+---------------------+-----------------------------+

+--------------+---------------------+-----------------------------+

| DL <Data>    | DD <Data>           | define longword constants   |

+--------------+---------------------+-----------------------------+

| DSB <number> | DB <number> DUP (?) | reserve bytes of memory     |

+--------------+---------------------+-----------------------------+

| DSW <number> | DW <number> DUP (?) | reserve words of memory     |

+--------------+---------------------+-----------------------------+

| DSD <number> | DD <number> DUP (?) | reserve longwords of memory |

+--------------+---------------------+-----------------------------+

| $MIN[IMUM]   | MAXMODE OFF         | following code runs         |

|              |                     | in minimum mode             |

+--------------+---------------------+-----------------------------+

| $MAX[IMUM]   | MAXMODE ON          | following code runs         |

|              |                     | in maximum mode             |

+--------------+---------------------+-----------------------------+

| $SYS[TEM]    | SUPMODE ON          | following code runs         |

|              |                     | in system mode              |

+--------------+---------------------+-----------------------------+

| $NOR[MAL]    | SUPMODE OFF         | following code runs         |

|              |                     | in user mode                |

+--------------+---------------------+-----------------------------+

| $NOLIST      | LISTING OFF         | turn off assembly listing   |

+--------------+---------------------+-----------------------------+

| $LIST        | LISTING ON          | turn on assembly listing    |

+--------------+---------------------+-----------------------------+

| $EJECT       | NEWPAGE             | start new page in listing   |

+--------------+---------------------+-----------------------------+

Table 4.6: equivalent instructions TAS900<->AS

Toshiba  manufactures two  versions of  the processor  core, with the L

version  being  an  ''economy  version''.  AS  will  make the following

differences between TLCS-900 and TLCS-900L:

- The  instructions MAX and NORMAL are not allowed for the L version;

the MIN instruction is disabled for the full version.

- The  L version  does not  know the  normal stack  pointer XNSP/NSP,

but instead has the interrupt nesting register INTNEST.

The  instructions  SUPMODE  and  MAXMODE  are  not  influenced, just as

their  initial  setting  OFF.  The  programmer  has to take care of the

fact  that the  L version  starts in  maximum mode  and does not have a

normal  mode. However, AS  shows a bit  of mercy against  the L variant

by suppressing warnings for privileged instructions.

4.35. TLCS-90

-------------

Maybe  some  people  might  ask  themselves  if  I mixed up the order a

little  bit, as Toshiba  first released the  TLCS-90 as an extended Z80

and  afterwards  the  16-bit  version  TLCS-900. Well, I discovered the

'90  via  the  '900  (thank  you  Oliver!).  The two families are quite

similar,   not  only   regarding  their   syntax  but   also  in  their

architecture.  The hints for the  '90 are therefore a  subset of of the

chapter  for the '900:  As the '90  only allows shifts, increments, and

decrements  by one, the count  need not and must  not be written as the

first  argument.  Once  again,  Toshiba  wants  to omit parentheses for

memory  operands of LDA, JP, and CALL,  and once again AS requires them

for  the  sake  of  orthogonality  (the  exact reason is of course that

this  way, I  saved an  extra in  the address  parser, but one does not

say such a thing aloud).

Principally,  the  TLCS-90  series  already  has  an address space of 1

Mbyte  which is  however only  accessible as  data space  via the index

registers.  AS therefore does not regard  the bank registers and limits

the  address space  to 64  Kbytes. This  should not  limit too  much as

this area above is anyway only reachable via indirect addressing.

4.36. TLCS-870

--------------

Once   again  Toshiba...a  company  quite  productive  at  the  moment!

Especially  this  branch  of  the  family (all Toshiba microcontrollers

are  quite similar in their binary  coding and programming model) seems

to  be targeted towards  the 8051 market:  the method of separating the

bit  position from the  address expression with  a dot had  its root in

the  8051.  However,  it  creates  now  exactly  the sort of problems I

anticipated  when working on  the 8051 part:  On the one  hand, the dot

is  a legal part of symbol names, but  on the other hand, it is part of

the  address syntax.  This means  that AS  has to  separate address and

bit  position and must process  them independently. Currently, I solved

this   conflict  by  seeking  the  dot  starting  at  the  end  of  the

expression.  This way, the  last dot is  regarded as the separator, and

further  dots stay  parts of  the address.  I continue to urge everyone

to omit dots in symbol names, they will lead to ambiguities:

LD      CF,A.7  ; accumulator bit 7 to carry

LD      C,A.7   ; constant 'A.7' to accumulator

4.37. TLCS-47

-------------

This  family of 4-bit microcontrollers should  mark the low end of what

is  supportable by AS.  Apart from the  ASSUME instruction for the data

bank  register  (see  there),  there  is  only  one thing that is worth

mentioning:  In the data and I/O  segment, nibbles are reserved instead

of  byte  (it's  a  4-bitter...).  The  situation is similar to the bit

data  segment of the 8051,  where a DB reserves  a single bit, with the

difference that we are dealing with nibbles.

Toshiba  defined  an  ''extended  instruction  set'' for this processor

family  to facilitate the  work with their  limited instruction set. In

the  case  of  AS,  it  is  defined  in  the include file STDDEF47.INC.

However,  some instructions  that could  not be  realized as macros are

''builtins''  and  are  therefore  also  available  without the include

file:

- the  B instruction  that automatically  chooses the optimal version

of the jump instruction (BSS; BS, or BSL);

- LD in the variant of HL with an immediate operand;

- ROLC and RORC with a shift amplitude higher than one.

4.38. TLCS-9000

---------------

This  was the first  time that I  implemented a processor  for AS which

was  not  yet  available  at  that  point of time. And unfortunately, I

received  back then  information that  Toshiba had  decided no to maket

this  processor at all.  This of course  had the result  that the TLCS-

9000 part of the assembler

1 was  a ''paper design'',  i.e. there was  so far no  chance to test

it on real hardware and

2 the  documentation  for  the  '9000  I  could get hold of [169] was

preliminary and was unclear in a couple of detail issues.

So i effect, this target went into 'dormant mode'...

...cut,  20 years have  passed: all of  a sudden, people are contacting

me  and  tell  me  that  Toshiba  actually  did sell TLCS-9000 chips to

customers,  and they ask  for documentation to  do reverse engineering.

Maybe   this  will  shed  some  light  on  the  remaining  unclarities.

Nevertheless,  errors in  this code  generator are  quite possible (and

will  of course be fixed!).  At least the few  examples listed in [169]

are assembled correctly.

Displacements   included  in  machine  instructions  may  only  have  a

certain  maximum length (e.g.  9 or 13  bits). In case the displacement

is  longer, a prefix  containing the 'upper  bits' must be prepended to

the  instruction.  AS  will  automatically  insert  such  prefixes when

necessary,  however it is also  possible to force usage  of a prefix by

adding a leading '>'. An example for this:

ld:g.b  (0h),0       ; no prefix

ld:g.b  (400000h),0  ; prefix added automatically

ld:g.b  (>0h),0      ; forced prefix

4.39. TC9331

------------

Toshiba  supplied a (DOS-based) assembler  for this processor which was

named  ASM31T.  This  assembler  supports  a  number of syntax elements

which  could not  be mapped  on the  capabilities of AS without risking

incompatibilities  for  existing  source  files  for other targets. The

following   issues  might  require  changes  on  programs  written  for

ASM31T:

- ASM31T  supports C-like  comments (/*  ... */)  which may also span

multiple  lines. Such comments are not  supported by AS and have to

be replaced by comments beginning with a semicolon.

- Similar  to ASM31T, AS supports  comments with round parentheses ((

...  )),  however  only  within  a  single command argument. Should

such  a comment  contain a  comma, this  comma will be treated like

an  argument separator  and the  comment will  not be  skipped when

parsing the arguments.

- ASM31T  allows symbol  and label  names containing  a dash. AS does

not   allow  this,  because   the  dash  is   regarded  to  be  the

subtraction  operator. It  would be  unclear whether  an expression

like  end-start  represents  a  single  symbol or the difference of

two symbols.

- ASM31T  requires  an  END  statement  as  the last statement of the

program; this is optional for AS.

Furthermore,   AS   currently   lacks   the   capabilities   to  detect

conflicting  uses  of  functional  units  in  a  machine  instructions.

Toshiba's  documentation  is  a  bit  difficult  to  understand in this

respect...

4.40. 29xxx

-----------

As   it  was  already  described  in   the  discussion  of  the  ASSUME

instruction,  AS can use  the information about  the current setting of

the  RBP register  to detect  accesses to  privileged registers in user

mode.  This  ability  is  of  course  limited  to direct accesses (i.e.

without   using  the  registers  IPA...IPC),  and  there  is  one  more

pitfall:  as  local  registers  (registers  with  a  number  >127)  are

addressed  relative to  the stack  pointer, but  the bits in RBP always

refer  to absolute numbers, the check  is NOT done for local registers.

An  extension would  require AS  to know  always the  absolute value of

SP, which would at least fail for recursive subroutines...

4.41. 80C16x

------------

As   it  was  already  explained  in   the  discussion  of  the  ASSUME

instruction,  AS tries  to hide  the fact  that the  processor has more

physical  than  logical  RAM  as  far  as possible. Please keep in mind

that  the DPP registers are valid only  for data accesses and only have

an  influence  on  absolute  addressing,  neither  on  indirect  nor on

indexed  addresses.  AS  cannot  know  which value the computed address

may  take at runtime... The paging  unit unfortunately does not operate

for  code  accesses  so  one  has  to  work with explicit long or short

CALLs,  JMPs,  or  RETs.  At  least  for the ''universal'' instructions

CALL  and JMP, AS  will automatically use  the shortest variant, but at

least  for the RET one  should know where the  call came from. JMPS and

CALLS  principally  require  to  write  segment and address separately,

but  AS is written  in a way  that it can  split an address on its own,

e.g. one can write

jmps    12345h

instead of

jmps    1,2345h

Unfortunately,  not  all  details  of  the  chip's internal instruction

pipeline  are hidden:  if CP  (register bank  address), SP  (stack), or

one   of  the  paging  registers  are  modified,  their  value  is  not

available  for  the  instruction  immediately  following.  AS  tries to

detect  such situations  and will  issue a  warning in such cases. Once

again, this mechanism only works for direct accesses.

Bits  defined with the  BIT instruction are  internally stored as a 12-

bit  word, containing  the address  in bits  4..11 and the bit position

in  the four LSBs. This  order allows to refer  the next resp. previous

bit  by  incrementing  or  decrementing  the address. This will however

not  work  for  explicit  bit  specifications  when  a word boundary is

crossed.  For example, the following expression  will result in a range

check error:

bclr    r5.15+1

We need a BIT in this situation:

msb     bit     r5.15

.

.

bclr    msb+1

The  SFR area was doubled  for the 80C167/165/163: bit  12 flags that a

bit  lies in  the second  part. Siemens  unfortunately did  not foresee

that  256  SFRs  (128  of  them  bit addressable) would not suffice for

successors  of  the  80C166.  As  a  result,  it would be impossible to

reach  the second  SFR area  from F000H..F1DFH  with short addresses or

bit   instructions  if  the  developers   had  not  included  a  toggle

instruction:

EXTR    #n

This  instruction  has  the  effect  that  for  the next n instructions

(0<n<5),  it is possible to address  the alternate SFR space instead of

the  normal  one.  AS  does  not  only generate the appropriate machine

code  when it  encounters this  instruction. It  also sets  an internal

flag  that will only allow accesses to  the alternate SFR space for the

next  n  instructions.  Of  course,  they  may  not contain jumps... Of

course,  it is always possible  to define bits from  either area at any

place,  and it is always possible  to reach all registers with absolute

addresses.  In contrast,  short and  bit addressing  only works for one

area  at  a  time,  attempts  contradicting  to  this will result in an

error message.

The  situation is  similar for  prefix instructions  and absolute resp.

indirect   addressing:  as   the  prefix   argument  and   the  address

expression  cannot always  be evaluated  at assembly  time, chances for

checking  are  limited  and  AS  will  limit  itself  to  warnings...in

detail, the situation is as follows:

- fixed  specification of a 64K bank  with EXTS or EXTSR: the address

expression  directly  contains  the  lower  16  bits  of the target

address.  If  the  prefix  and  the  following  instruction  have a

constant  operand, AS  will check  if the  the prefix  argument and

bits 16..23 of the target address are equal.

- fixed  specification of a 16K page  with EXTP or EXTPR: the address

expression  directly  contains  the  lower  14  bits  of the target

address.  Bits 14 and 15  are fixed to 0,  as the processor ignores

them  in this  mode. If  the prefix  and the  following instruction

have  a constant operand, AS will  check if the the prefix argument

and bits 14..23 of the target address are equal.

An  example  to  clarify  things  a  bit  (the DPP registers have their

reset values):

extp    #7,#1      ; range from 112K..128K

mov     r0,1cdefh  ; results in address 0defh in code

mov     r0,1cdefh  ; -->warning

exts    #1,#1      ; range from 64K..128K

mov     r0,1cdefh  ; results in address 0cdefh in code

mov     r0,1cdefh  ; -->warning

4.42. PIC16C5x/16C8x

--------------------

Similar  to  the  MCS-48  family,  the  PICs split their program memory

into  several banks because the opcode  does not offer enough space for

a  complete address. AS  uses the same  automatism for the instructions

CALL  and GOTO, i.e. the  PA bits in the  status word are set according

to  the start and  target address. However,  this procedure is far more

problematic compared to the 48's:

1 The  instructions  are  not  any  more  one  word long (up to three

words).  Therefore, it is  not guaranteed that  they can be skipped

with a conditional branch.

2 It  is possible  that the  program counter  crosses a page boundary

while  the program sequence is executed.  The setting of PA bits AS

assumes may be different from reality.

The  instructions  that  operate  on  register  W  and another register

normally  require a second parameter  that specifies whether the result

shall  be stored in  W or the  register. Under AS,  it is valid to omit

the  second  parameter.  The  assumed  target  then  depends  upon  the

operation's  type:  For  unary  operations,  the  result  is by default

stored back into the register. These instructions are:

COMF, DECF, DECFSZ, INCF, INCFSZ, RLF, RRF, and SWAPF

The other operations by default regard W as an accumulator:

ADDWF, ANDWF, IORWF, MOVF, SUBWF, and XORWF

The  syntax defined  by Microchip  to write  literals is  quite obscure

and  reminds of the syntax used  on IBM 360/370 systems (greetings from

the  stone-age...).  To  avoid  introducing  another  branch  into  the

parser,  with  AS  one  has  to  write constants in the Motorola syntax

(optionally Intel or C in RELAXED mode).

4.43. PIC 17C4x

---------------

With  two exceptions, the same  hints are valid as  for its two smaller

brothers:   the  corresponding  include  file  only  contains  register

definitions,  and the  problems concerning  jump instructions  are much

smaller.  The only exception  is the LCALL  instruction, which allows a

jump  with  a  16-bit  address.  It  is  translated  with the following

''macro'':

MOVLW   <addr15..8>

MOWF    3

LCALL   <addr0..7>

4.44. SX20/28

-------------

The  limited length of the instruction  word does not permit specifying

a  complete program memory address (11  bits) or data memory address (8

bits).   The  CPU  core   augments  the  truncated   address  from  the

instruction   word  with  the  PA   bits  from  the  STATUS  registers,

respectively  with the upper  bits of the  FSR register. It is possible

to  inform the assembler via ASSUME  instructions about the contents of

these  two  registers.  In  case  that  addresses  are  used  that  are

inaccessible with th current values, a warning is issued.

4.45. ST6

---------

These  processors have the ability to  map their code ROM pagewise into

the  data area. I am not keen  on repeating the whole discussion of the

ASSUME  instruction  at  this  place,  so  I refer to the corresponding

section  (3.2.21) for an  explanation how to  read constants out of the

code ROM without too much headache.

Some  builtin ''macros'' show up when  one analyzes the instruction set

a  bit more  in detail.  The instructions  I found  are listed in table

4.7 (there are probably even more...):

+-------------+------------+

| instruction | in reality |

+-------------+------------+

+-------------+------------+

| CLR A       | SUB A,A    |

| SLA A       | ADD A,A    |

| CLR addr    | LDI addr,0 |

| NOP         | JRZ PC+1   |

+-------------+------------+

Table 4.7: Hidden Macros in the ST62's Instruction Set

Especially  the last  case is  a bit  astonishing...unfortunately, some

instructions   are  really  missing.  For  example,  there  is  an  AND

instruction  but no OR...not to  speak of an XOR.  For this reason, the

include   file   STDDEF62.INC   contains   also   some  helping  macros

(additionally to register definitions).

The  original assembler  AST6 delivered  by SGS-Thomson  partially uses

different  pseudo instructions  than AS.  Apart from  the fact  that AS

does  not mark  pseudo instructions  with a  leading dot, the following

instructions are identical:

ASCII, ASCIZ, BLOCK, BYTE, END, ENDM, EQU, ERROR, MACRO,

ORG, TITLE, WARNING

Table  4.8  shows  the  instructions  which  have  AS counterparts with

similar function.

+----------+---------------------+------------------------------+

| AST6     | AS                  | meaning/function             |

+----------+---------------------+------------------------------+

+----------+---------------------+------------------------------+

| .DISPLAY | MESSAGE             | output message               |

+----------+---------------------+------------------------------+

| .EJECT   | NEWPAGE             | new page in assembly listing |

+----------+---------------------+------------------------------+

| .ELSE    | ELSEIF              | conditional assembly         |

+----------+---------------------+------------------------------+

| .ENDC    | ENDIF               | conditional assembly         |

+----------+---------------------+------------------------------+

| .IFC     | IF...               | conditional assembly         |

+----------+---------------------+------------------------------+

| .INPUT   | INCLUDE             | insert include file          |

+----------+---------------------+------------------------------+

| .LIST    | LISTING, MACEXP_DFT | settings for listing         |

+----------+---------------------+------------------------------+

| .PL      | PAGE                | page length of listing       |

+----------+---------------------+------------------------------+

| .ROMSIZE | CPU                 | set target processor         |

+----------+---------------------+------------------------------+

| .VERS    | VERSION (symbol)    | query version                |

+----------+---------------------+------------------------------+

| .SET     | EVAL                | redefine variables           |

+----------+---------------------+------------------------------+

Table 4.8: Equivalent Instructions AST6<->AS

4.46. ST7

---------

In  [133], the .w  postfix to signify  16-bit addresses is only defined

for  memory  indirect  operands.  It  is  used  to  mark  that a 16-bit

address  is stored at a zero  page address. AS additionally allows this

postfix  for absolute  addresses or  displacements of  indirect address

expressions  to force 16-bit  displacements in spite  of an 8-bit value

(0..255).

4.47. ST9

---------

The  ST9's bit  addressing capabilities  are quite  limited: except for

the  BTSET instruction,  only bits  within the  current set  of working

registers  are accessible. A bit address  is therefore of the following

style:

rn.[!]b   ,

whereby  ! means an optional  complement of a source  operand. If a bit

is  defined symbolically, the  bit's register number  is stored in bits

7..4,  the  bit's  position  is  stored  in  bits 3..1 and the optional

complement  is kept  in bit  0. AS  distinguishes explicit and symbolic

bit  addresses  by  the  missing  dot.  A bit's symbolic name therefore

must  not contain a  dot, thought it  would be legal  in respect to the

general  symbol  name  conventions.  It  is  also  valid  to  invert  a

symbolically referred bit:

bit2    bit     r5.3

.

.

bld     r0.0,!bit2

This  opportunity also  allows to  undo an  inversion that  was done at

definition of the symbol.

The  include file REGST9.INC defines the  symbolic names of all on-chip

registers  and their  associated bits.  Keep however  in mind  that the

bit   definitions  only  work  after  previously  setting  the  working

register bank to the address of these peripheral registers!

In  contrast to the  definition file delivered  with the AST9 assembler

from  SGS-Thomson,  the  names  of  peripheral  register names are only

defined  as  general  registers  (R...),  not also as working registers

(r...).  The  reason  for  this  is  that  AS does not support register

aliases; a tribute to assembly speed.

4.48. 6804

----------

To  be  honest:  I  only  implemented  this  processor in AS to quarrel

about  SGS-Thomson's peculiar behaviour.  When I first  read the 6804's

data  book, the ''incomplete'' instruction  set and the built-in macros

immediately  reminded me  of the  ST62 series  manufactured by the same

company.  A  more  thorough  comparison  of the opcodes gave surprising

insights:  A  6804  opcode  can  be  generated by taking the equivalent

ST62  opcode and  mirroring all  the bits!  So Thomson  obviously did a

bit  of processor  core recycling...which  would be  all right  if they

would  not try  to hide  this: different  peripherals, motorola instead

of  Zilog-style syntax, and  the awful detail  of not mirroring operand

fields  in the opcode  (e.g. bit fields  containing displacements). The

last  item is also the reason that  finally convinced me to support the

6804  in AS.  I personally  can only  guess which department at Thomson

did the copy...

In  contrast to  its ST62  counterpart, the  include file  for the 6804

does  not contain instruction macros  that help a bit  to deal with the

limited  machine instruction  set. This  is left  as an exercise to the

reader!

4.49. TMS3201x

--------------

It  seems  that  every  semiconductor's  ambition  is  to invent an own

notation  for hexadecimal numbers. Texas  Instrument took an especially

eccentric  approach  for  these  processors:  a  >  sign as prefix! The

support  of such a  format in AS  would have lead  to extreme conflicts

with  AS's compare and shift operators.  I therefore decided to use the

Intel  notation, which is  what TI also  uses for the  340x0 series and

the 3201x's successors...

The  instruction word of  these processors unfortunately  does not have

enough  bits to  store all  8 bits  for direct  addressing. This is why

the  data  address  space  is  split  into  two  banks of 128 words. AS

principally  regards the data address space  as a linear segment of 256

words  and automatically clears bit 7  on direct accesses (an exception

is  the SST  instruction that  can only  write to  the upper bank). The

programmer  has to take care that the  bank flag always has the correct

value!

Another   hint  that  is  well  hidden  in  the  data  book:  The  SUBC

instruction  internally needs more  than one clock  for completion, but

the  control unit  already continues  to execute  the next instruction.

An   instruction   following   SUBC   therefore   may  not  access  the

accumulator. AS does not check for such conditions!

4.50. TMS320C2x

---------------

As  I did  not write  this code  generator myself  (that does not lower

its  quality by any  standard), I can  only roughly line  out why there

are  some instructions that force a  prefixed label to be untyped, i.e.

not  assigned  to  any  specific  address  space:  The 2x series of TMS

signal  processors has  a code  and a  data segment  which are  both 64

Kbytes  large.  Depending  on  external  circuitry, code and data space

may  overlap, e.g. to allow  storage of constants in  the code area and

access  them  as  data.  Data  storage  in  the  code  segment  may  be

necessary  because older  versions of  AS assume  that the data segment

only  consists of RAM  that cannot have  a defined power-on  state in a

single  board  system.  They  therefore  reject  storage of contents in

other  segments  than  CODE.  Without  the  feature  of  making symbols

untyped,  AS  would  punish  every  access  to a constant in code space

with  a  warning  (''symbol  out  of  wrong  segment'').  To  say it in

detail, the following instructions make labels untyped:

BSS, STRING, RSTRING, BYTE, WORD , LONG

FLOAT, DOUBLE, EFLOAT, BFLOAT and TFLOAT

If  one needs a typed label in  front of one of these instructions, one

can  work around  this by  placing the  label in  a separate  line just

before  the  pseudo  instruction  itself.  On  the  other  hand,  it is

possible  to  place  an  untyped  label  in  front  of  another  pseudo

instruction by defining the label with EQU, e.g.

<name>  EQU     $        .

4.51. TMS320C3x/C4x

-------------------

The  syntax detail that  created the biggest  amount of headache for me

while  implementing this processor family  is the splitting of parallel

instructions  into two  separate source  code lines.  Fortunately, both

instructions  of such a  construct are also  valid single instructions.

AS  therefore first  generates the  code for  the first instruction and

replaces  it by the parallel machine  code when a parallel construct is

encountered  in the second  line. This operation  can be noticed in the

assembly  listing by  the machine  code address  that does  not advance

and the double dot replaced with a R.

Compared  to  the  TI  assembler,  AS  is not as flexible regarding the

position  of the double  lines that signify  a parallel operation (||):

One  either  has  to  place  them  like  a label (starting in the first

column)  or to prepend them to the  second mnemonic. The line parser of

AS will run into trouble if you do something else...

4.52. TMS9900

-------------

Similar  to  most  older  TI  microprocessor  families,  TI used an own

format  for hexadecimal  and binary  constants. AS  instead favours the

Intel  syntax which  is also  common for  newer processor  designs from

TI.

The  TI  syntax  for  registers  allows  to use a simple integer number

between  0 and  15 instead  of a  real name  (Rx or  WRx). This has two

consequences:

- R0...R15  resp.  WR0..WR15  are  simple  predefined integer symbols

with  values from 0  to 15, and  the definition of register aliases

is a simple matter of EQU.

- In  contrast  to  several  other  processors,  I  cannot  offer the

additional  AS feature that allows  to omit the character sigifying

absolute  addressing  (a  @  sign  in  this  case).  As  a  missing

character  would  mean  register  numbers  (from  0  to 15) in this

case, it was not possible to offer the optional omission.

Furthermore,  TI sometimes uses  Rx to name  registers and WRx at other

places...currently both variants are recognized by AS.

4.53. TMS70Cxx

--------------

This  processor family  belongs to  the older  families developed by TI

and   therefore  TI's  assemblers  use  their  proprietary  syntax  for

hexadecimal  resp. binary constants  (a prefixed <  resp. ? character).

As  this format could not be realized  for AS, the Intel syntax is used

by  default. This  is the  format TI  to which  also switched over when

introducing   the  successors,  of  this  family,  the  370  series  of

microcontrollers.   Upon   a   closer   inspection  of  both's  machine

instruction  set, one discovers  that about 80%  of all instruction are

binary  upward compatible, and that also  the assembly syntax is almost

identical  - but unfortunately only almost.  TI also took the chance to

make  the syntax more  orthogonal and simple.  I tried to introduce the

majority of these changes also into the 7000's instruction set:

- It   is  valid  to  use  the  more  common  #  sign  for  immediate

addressing instead of the percent sign.

- If  a port  address (P...)  is used  as source  or destination in a

AND,  BTJO, BTJZ, MOV, OR, or  XOR instruction, it is not necessary

to  use the mnemonic variant with an  appended P - the general form

is sufficient.

- The  prefixed @ sign  for absolute or  B-relative addressing may be

omitted.

- Instead of CMPA, CMP with A as target may be written.

- Instead  of LDA resp.  STA, one can  simply use the MOV instruction

with A as source resp. destination.

- One can write MOVW instead of MOVD.

- It is valid to abbreviate RETS resp. RETI as RTS resp. RTI.

- TSTA resp. TSTB may be written as TST A resp. TST B.

- XCHB B is an alias for TSTB.

An  important note: these variants are  only allowed for the TMS70Cxx -

the corresponding 7000 variants are not allowed for the 370 series!

4.54. TMS370xxx

---------------

Though  these processors do  not have specialized  instructions for bit

manipulation,  the  assembler  creates  (with  the  help  of  the  DBIT

instruction   -  see  there)  the  illusion  as  if  single  bits  were

addressable.  To achieve this, the  DBIT instructions stores an address

along  with a  bit position  into an  integer symbol  which may then be

used  as an argument  to the pseudo  instructions SBIT0, SBIT1, CMPBIT,

JBIT0,  and JBIT1. These are translated  into the instructions OR, AND,

XOR, BTJZ, and BTJO with an appropriate bit mask.

There  is  nothing  magic  about  these  bit  symbols,  they are simple

integer  values that  contain the  address in  their lower  and the bit

position  in their upper half. One  could construct bit symbols without

the DBIT instruction, like this:

defbit  macro   name,bit,addr

name    equ     addr+(bit<<16)

endm

but  this technique would  not lead to  the EQU-style syntax defined by

TI  (the symbol  to be  defined replaces  the label  field in  a line).

CAUTION!   Though  DBIT   allows  an   arbitrary  address,  the  pseudo

instructions  can only operate with addresses  either in the range from

0..255  or  1000h..10ffh.  The  processor  does  not  have  an absolute

addressing mode for other memory ranges...

4.55. MSP430(X)

---------------

The  MSP  was  designed  to  be  a  RISC processor with a minimal power

consumption.  The set of machine  instructions was therefore reduced to

the  absolute minimum (RISC  processors do not  have a microcode ROM so

every  additional  instruction  has  to  be implemented with additional

silicon  that increases  power consumption).  A number  of instructions

that  are hardwired  for other  processors are  therefore emulated with

other   instructions.   Older   versions   of   AS   implemented  these

instructions  via  macros  in  the  file  REGMSP.INC.  If  one  did not

include  this file, you  got error messages  for more than  half of the

instructions  defined by TI. This has  been changed in recent versions:

as  part of  adding the  430X instruction  set, implementation of these

instructions  was moved into the  assmebler's core. REGMSP.INC now only

contains  addresses of  I/O registers.  If you  need the old macros for

some reason, they have been moved to the file EMULMSP.INC.

Instruction  emulation also  covers some  special cases  not handled by

the original TI assembler. For instance,

rlc  @r6+

is automatically assembled as

addc @r6+,-2(r6)

4.56. TMS1000

-------------

At  last, world's first microcontroller finally  also supported in AS -

it  took long  to fill  this gap,  but now  it is done. This target has

some pitfalls that will be discussed shortly in this section.

First,  the instruction set  of these controllers  is partially defined

via  the ROM  mask, i.e.  the function  of some  opcodes may  be freely

defined  to some  degree. AS  only knows  the instructions  and codings

that  are described as default codings in  [163]. If you have a special

application  with  an  instruction  set  deviating  from  this, you may

define and modify instructions via macros and the DB instruction.

Furthermore,  keep  in  mind  that  branches  and subroutine calls only

contain  the lower 6  bits of the  target address. The  upper 4 resp. 5

bits  are fetched from page  and chapter registers tha  thave to be set

beforehand.  AS  cannot  check  whether  these  registers have been set

correctly  by the programmer!  At least for  the cas of  staying in the

same  chapter, there are the  assmebler pseudo instructions CALLL resp.

BL  that combine an LDP and  CALL/BR instruction. Regarding the limited

amount  of  program  memory,  this  is  a  convenient  yet  inefficient

variant.

4.57. COP8

----------

National  unfortunately also decided to use  the syntax well known from

IBM  mainframes (and much  hated by me..)  to write non-decimal integer

constants.  Just like  with other  processors, this  does not work with

AS's  parser.  ASMCOP  however  fortunately  also  seems to allow the C

syntax,  which is why  this became the  default for the  COP series and

the SC/MP...

4.58. SC/MP

-----------

If  indirect addressing with displacement is  used on the SC/MP, and if

the  base or pointer register is not  P0/PC, a displacement of -128 (80

hex)  has a special  meaning: the contents  of the E  register are used

as  displacement  instead  of  this  value.  If  using  the 'classic NS

assembler',   the  programmer  has   to  know  about   this,  and  even

explicitly use it:

ereg   equ -128

ld  ereg(p1)

This  however bears the  risk that -128  may accidentally be the result

of  a computed  displacement, and  you might  have a  hard time finding

out  why  the  program  does  not  do  what  was  indented. I therefore

decided to make this special value more explicit:

If  a  displacement  of  -128  is  used,  a  warning is issued. One may

simply  ignore this  warning. If  you want  to get  rid of  it, use the

built-in  literal E, which  explicitly references the  register of same

name:

ld e(p1)

Since  the SC/MP target supports register  symbols, it is also possible

to define the 'own symbol' in a proper way:

ereg   reg e

ld  ereg(p1)

This  should reduce  the amount  of necessary  changes in existing code

to a minimum.

4.59. SC144xxx

--------------

Originally,  National offered  a relatively  simple assembler  for this

series  of DECT controllers.  An much more  powerful assembler has been

announced  by IAR, but  it is not  available up to  now. However, since

the  development tools  made by  IAR are  as much target-independent as

possible,  one  can  roughly  estimate  the pseudo instructions it will

support  by looking at  other available target  platforms. With this in

mind,  the (few) SC144xx-specific instructions  DC, DC8, DW16, DS, DS8,

DS16,  DW  were  designed.  Of  course,  I  didn't want to reinvent the

wheel  for pseudo instructions  whose functionality is  already part of

the  AS core. Therefore, here is  a little table with equivalences. The

statements  ALIGN, END, ENDM,  EXITM, MACRO, ORG,  RADIX, SET, and REPT

both  exist for the  IAR assembler and  AS and have same functionality.

Changes are needed for the following instructions:

+---------------------+-----------------+---------------------------+

| IAR                 | AS              | Funktion                  |

+---------------------+-----------------+---------------------------+

+---------------------+-----------------+---------------------------+

| #include            | include         | include file              |

| #define             | SET, EQU        | define symbol             |

| #elif, ELIF, ELSEIF | ELSEIF          | start another             |

|                     |                 | IF branch                 |

| #else, ELSE         | ELSE            | last branch of an IF      |

|                     |                 | construct                 |

| #endif, ENDIF       | ENDIF           | ends an IF construct      |

| #error              | ERROR, FATAL    | create error message      |

| #if, IF             | IF              | start an IF construct     |

| #ifdef              | IFDEF           | symbol defined ?          |

| #ifndef             | IFNDEF          | symbol not defined ?      |

| #message            | MESSAGE         | output message            |

| =, DEFINE, EQU      | =, EQU          | fixed value assignment    |

| EVEN                | ALIGN 2         | force PC to be equal      |

| COL, PAGSIZ         | PAGE            | set page size for listing |

| ENDR                | ENDM            | end REPT construct        |

| LSTCND, LSTOUT      | LISTING         | control amount of listing |

| LSTEXP, LSTREP      | MACEXP          | list expanded macros?     |

| LSTXRF              |  <command line> | generate cross reference  |

| PAGE                | NEWPAGE         | new page in listing       |

| REPTC               | IRPC            | repetition with character |

|                     |                 | replacement               |

+---------------------+-----------------+---------------------------+

There  is  no  direct  equivalent  for  CASEON, CASEOFF, LOCAL, LSTPAG,

#undef, and REPTI.

A  100% equivalent is  of course impossible  as long as  there is no C-

like  preprocessor  in  AS.  C-like  comments  unfortunately  are  also

impossible  at the  moment. Caution:  When modifying  IAR codes for AS,

do  not forget to move converted  preprocessor statements out of column

1 as AS reserves this column exclusively for labels!

4.60. NS32xxx

-------------

As  one  might  expect  from  a  CISC  processor,  the  NS32xxx  series

provides  sophisticated and  complex addressing  modes. National defied

the  assembly syntax for each of them  in its manuals, and this is also

the  syntax AS implements. However, as  for every architecture that was

supported  by third-party  tools, there  are deviations and extensions,

and I added a few of them to AS:

The syntax to use PC-relative addressing, as defined by National, is:

movb r0,*+disp

This  of course quite  clearly expresses what  is happening at runtime,

one  however has  to compute  the distance  himself if a certain memory

location is to be addressed:

movb r0,*+(addr-*)

The  first  simplification  is  that  under  certain  conditions, it is

sufficient to just write:

movb r0,addr

since  absolute addressierung is marked by  a @ prefix. This is allowed

under the following conditions:

- Immediate  addressierung is  not allowed,  e.g. because the operand

is the destination and there is no risk os ambiguities.

- An  index extension  is used  (appended in  square brackets), which

must not be combined with immediate addressing.

As  an  alterntative,  AS  also  supports  the following way to use PC-

relative addressing:

movb r0,addr(pc)

Analog to the 68000, the distance is computed automatically.

The external mode, whis written this way in National syntax:

movb r0,ext(disp1)+disp2

there is another supported syntax variant:

movb r0,disp2(disp1(ext))

which used to be common in UNIX environments.

4.61. uPD78(C)1x

----------------

For  relative, unconditional instructions, there  is the JR instruction

branch  distance -32...+31, one byte),  and the JRE instruction (branch

distance  -256...+255, two  bytes). AS  furthermore knows  the J pseudo

instruction,   which  automatically   selects  the   shortest  possible

variant.

Architecture  and  instructon  set  of  these  processors  are coarsely

related  to the Intel 8080/8085  - thi is also  true for the mnemonics.

The  adressing mode  (direct, indirect,  immediate) is  packed into the

mnemonic,  and 16 bit registers (BC, DE,  HL) are written with just one

letter.  However, since  NEC itself  also uses  at some places written-

out  register names and  parentheses to signify  indirect addressing, I

decided  to support some  alternative notations next  to the 'official'

ones.   Some  non-NEC  tools  like  disassemblers  seem  to  use  these

notations either:

- It is allowed to use BC, (B), or (BC) instead of B.

- It is allowed to use DE, (D), or (DE) instead of D.

- It is allowed to use HL, (H), or (HL) instead of H.

- It is allowed to use DE+, (D+), (DE+), or (DE)+ instead of D+.

- It is allowed to use HL+, (H+), (HL+), or (HL)+ instead of H+.

- It is allowed to use DE-, (D-), (DE-), or (DE)- instead of D-.

- It is allowed to use HL-, (H-), (HL-), or (HL)- instead of H-.

- It  is allowed  to use  DE++, (D++),  (DE++), or  (DE)++ instead of

D++.

- It  is allowed  to use  HL++, (H++),  (HL++), or  (HL)++ instead of

H++.

- It  is allowed to use DE--, (D--), (DE--), or (DE)-- instead of D--

.

- It  is allowed to use HL--, (H--), (HL--), or (HL)-- instead of H--

.

- It  is allowed  to use  HL+A, A+H,  A+HL, (H+A),  (HL+A), (A+H), or

(A+HL) instead of H+A.

- It  is allowed  to use  HL+B, B+H,  B+HL, (H+B),  (HL+B), (B+H), or

(B+HL) instead of H+B.

- It  is allowed to use HL+EA,  EA+H, EA+HL, (H+EA), (HL+EA), (EA+H),

or (EA+HL) instead of H+EA.

Since  architecture and  instruction set  are so  ''8080-like'', it was

straightforward  to support  the Z80SYNTAX  statement, which  allows to

write  many machine instructions  in a more  intuitive and better-known

way.   However,  since  both  the   uCON87  family's  architecture  and

instruction  set differ  from the  8080 in  a couple  of details, it is

not  possible  to  provide  a  complete  one-to-one  mapping.  Not  all

original   instructions   have   a   ''Z80   equivalent'',   and   some

instructions  known from 8080  and Z80 do  not exist on  the uCOM87. It

therefore does not make sense to support Z80SYNTAX EXCLUSIVE.

The  following table lists  all instructions defined  in Z80SYNTAX mode

and their equivalents in original syntax:

Table 4.10: Instruction Variants in Z80SYNTAX Mode

+-----------------+----------------+-----------------------+-----------+

| Z80SYNTAX       | Original       | Operation             | CPUs      |

+-----------------+----------------+-----------------------+-----------+

+-----------------+----------------+-----------------------+-----------+

| LD r1,A         | MOV r1,A       | r1<-A                 | 1,2,3,4   |

|                 |                |                       | (r1!=EAx) |

|                 |                |                       | 3,4       |

|                 |                |                       | (r1=EAx)  |

| LD A,r1         | MOV A,r1       | A<-r1                 | 1,2,3,4   |

|                 |                |                       | (r1!=EAx) |

|                 |                |                       | 3,4       |

|                 |                |                       | (r1=EAx)  |

| LD sr,A         | MOV sr,A       | sr<-A                 | 1,2,3,4   |

| LD A,sr1        | MOV A,sr1      | A<-sr1                | 1,2,3,4   |

| LD r,(word)     | MOV r,word     | r<-(word)             | 1,2,3,4   |

|                 |                |                       | (r1!=V)   |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r1=V)    |

| LD (word),r     | MOV word,r     | (word)<-r             | 1,2,3,4   |

|                 |                |                       | (r1!=V)   |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r1=V)    |

| LD r,byte       | MVI r,byte     | r<-byte               | 1,2,3,4   |

|                 |                |                       | (r1!=V)   |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r1=V)    |

| LD sr2,byte     | MVI sr2,byte   | sr2<-byte             | 2,3,4     |

| LD (wa),byte    | MVIW wa,byte   | (wa)<-byte            | 2,3,4     |

| LD (rpa1),byte  | MVIX rpa1,byte | (rpa1)<-byte          | 2,3,4     |

| LD (wa),a       | STAW wa        | (wa)<-A               | 1,2,3,4   |

| LD A,(wa)       | LDAW wa        | A<-(wa)               | 1,2,3,4   |

| LD (rpa),a      | STAX rpa       | (rpa)<-(wa)           | 1,2       |

| LD (rpa2),a     | STAX rpa2      | (rpa2)<-(wa)          | 3,4       |

| LD A,(rpa)      | LDAX rpa       | (wa)<-(rpa)           | 1,2       |

| LD A,(rpa2)     | LDAX rpa2      | (wa)<-(rpa2)          | 3,4       |

| LD rp3,EA       | DMOV rp3,EA    | rp3<-EA               | 3,4       |

| LD sr3,EA       | DMOV sr3,EA    | sr3<-EA               | 3,4       |

| LD EA,sr4       | DMOV EA,sr4    | EA<-sr4               | 3,4       |

| LD EA,rp3       | DMOV EA,rp3    | EA<-rp3               | 3,4       |

| LD (word),BC    | SBCD word      | (word)<-BC            | 1,2,3,4   |

| LD (word),DE    | SDED word      | (word)<-DE            | 1,2,3,4   |

| LD (word),HL    | SHLD word      | (word)<-HL            | 1,2,3,4   |

| LD (word),SP    | SSPD word      | (word)<-SP            | 1,2,3,4   |

| LD (rpa3),EA    | STEAX rpa3     | (rpa3)<-EA            | 3,4       |

| LD BC,(word)    | LBCD word      | BC<-(word)            | 1,2,3,4   |

| LD DE,(word)    | LDED word      | DE<-(word)            | 1,2,3,4   |

| LD HL,(word)    | LHLD word      | HL<-(word)            | 1,2,3,4   |

| LD SP,(word)    | LSPD word      | SP<-(word)            | 1,2,3,4   |

| LD EA,(rpa3)    | LDEAX rpa3     | EA<-(word)            | 3,4       |

| LD rp,word      | LXI rp,word    | rp<-word              | 1,2,3,4   |

| LD EA,word      | LXI EA,word    | EA<-word              | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| ADD A,(rpa)     | ADDX rpa       | A<-A+(rpa)            | 1,2,3,4   |

| ADD A,byte      | ADI A,byte     | A<-A+byte             | 1,2,3,4   |

| ADD r,byte      | ADI r,byte     | r<-r+byte             | 2,3,4     |

| ADD sr2,byte    | ADI sr2,byte   | sr2<-A+sr2            | 2,3,4     |

| ADD A,(wa)      | ADDW wa        | A<-A+(wa)             | 2,3,4     |

| ADD EA,r2       | EADD EA,r2     | EA<-EA+r2             | 3,4       |

| ADD EA,rp3      | DADD EA,rp3    | EA<-EA+rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| ADC A,(rpa)     | ADCX rpa       | A<-A+(rpa)+CY         | 1,2,3,4   |

| ADC A,byte      | ACI A,byte     | A<-A+byte+CY          | 1,2,3,4   |

| ADC r,byte      | ACI r,byte     | r<-r+A+CY             | 2,3,4     |

| ADC sr2,byte    | ACI sr2,byte   | sr2<-A+sr2+CY         | 2,3,4     |

| ADC A,(wa)      | ADCW wa        | A<-A+(wa)+CY          | 2,3,4     |

| ADC EA,rp3      | DADC EA,rp3    | EA<-EA+rp3+CY         | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| ADDNC A,(rpa)   | ADDNCX rpa     | A<-A+(rpa)            | 1,2,3,4   |

|                 |                | skip if !CY           |           |

| ADDNC A,byte    | ADINC A,byte   | A<-A+byte             | 1,2,3,4   |

|                 |                | skip if !CY           |           |

| ADDNC r,byte    | ADINC r,byte   | r<-r+A                | 2,3,4     |

|                 |                | skip if !CY           |           |

| ADDNC sr2,byte  | ADINC sr2,byte | sr2<-A+sr2            | 2,3,4     |

|                 |                | skip if !CY           |           |

| ADDNC A,(wa)    | ADDNCW wa      | A<-A+(wa)             | 2,3,4     |

|                 |                | skip if !CY           |           |

| ADDNC EA,rp3    | DADDNC EA,rp3  | EA<-EA+rp3            | 3, 4      |

|                 |                | skip if !CY           |           |

+-----------------+----------------+-----------------------+-----------+

| SUB A,(rpa)     | SUBX rpa       | A<-A-(rpa)            | 1,2,3,4   |

| SUB A,byte      | SUI A,byte     | A<-A-byte             | 1,2,3,4   |

| SUB r,byte      | SUI r,byte     | r<-r-A                | 2,3,4     |

| SUB sr2,byte    | SUI sr2,byte   | sr2<-A-sr2            | 2,3,4     |

| SUB A,(wa)      | SUBW wa        | A<-A-(wa)             | 2,3,4     |

| SUB EA,r2       | ESUB EA,r2     | EA<-EA-r2             | 3,4       |

| SUB EA,rp3      | DSUB EA,rp3    | EA<-EA-rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SBB A,(rpa)     | SBBX rpa       | A<-A-(rpa)-CY         | 1,2,3,4   |

| SBB A,byte      | SBI A,byte     | A<-A-byte-CY          | 1,2,3,4   |

| SBB r,byte      | SBI r,byte     | r<-r-A-CY             | 2,3,4     |

| SBB sr2,byte    | SBI sr2,byte   | sr2<-A-sr2-CY         | 2,3,4     |

| SBB A,(wa)      | SBBW wa        | A<-A-(wa)-CY          | 2,3,4     |

| SBB EA,rp3      | DSBB EA,rp3    | EA<-EA-rp3-CY         | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SUBNB A,(rpa)   | SUBNBX rpa     | A<-A-(rpa)            | 1,2,3,4   |

|                 |                | skip if !CY           |           |

| SUBNB A,byte    | SUINB A,byte   | A<-A-byte             | 1,2,3,4   |

|                 |                | skip if !CY           |           |

| SUBNB r,byte    | SUINB r,byte   | r<-r-A                | 2,3,4     |

|                 |                | skip if !CY           |           |

| SUBNB sr2,byte  | SUINB sr2,byte | sr2<-A-sr2            | 2,3,4     |

|                 |                | skip if !CY           |           |

| SUBNB A,(wa)    | SUBNBW wa      | A<-A-(wa)             | 2,3,4     |

|                 |                | skip if !CY           |           |

| SUBNB EA,rp3    | DSUBNB EA,rp3  | EA<-EA-rp3            | 3,4       |

|                 |                | skip if !CY           |           |

+-----------------+----------------+-----------------------+-----------+

| AND A,r         | ANA A,r        | A<-A&r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| AND r,A         | ANA r,A        | r<-A&r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| AND A,(rpa)     | ANAX rpa       | A<-A&(rpa)            | 1,2,3,4   |

| AND A,byte      | ANI A,byte     | A<-A&byte             | 1,2,3,4   |

| AND r,byte      | ANI r,byte     | r<-r&byte             | 2,3,4     |

| AND sr2,byte    | ANI sr2,byte   | sr2<-sr2&byte         | 1,2,3,4   |

| AND A,(wa)      | ANAW wa        | A<-A&(wa)             | 2,3,4     |

| AND (wa),byte   | ANIW wa,byte   | (wa)<-(wa)&byte       | 1,2,3,4   |

| AND EA,rp3      | DAN EA,rp3     | EA<-EA&rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| OR A,r          | ORA A,r        | A<-A|r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| OR r,A          | ORA r,A        | r<-A|r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| OR A,(rpa)      | ORAX rpa       | A<-A|(rpa)            | 1,2,3,4   |

| OR A,byte       | ORI A,byte     | A<-A|byte             | 1,2,3,4   |

| OR r,byte       | ORI r,byte     | r<-r|byte             | 2,3,4     |

| OR sr2,byte     | ORI sr2,byte   | sr2<-sr2|byte         | 1,2,3,4   |

| OR A,(wa)       | ORAW wa        | A<-A|(wa)             | 2,3,4     |

| OR (wa),byte    | ORIW wa,byte   | (wa)<-(wa)|byte       | 1,2,3,4   |

| OR EA,rp3       | DOR EA,rp3     | EA<-EA|rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| XOR A,r         | XRA A,r        | A<-A^r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| XOR r,A         | XRA r,A        | r<-A^r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| XOR A,(rpa)     | XRAX rpa       | A<-A^(rpa)            | 1,2,3,4   |

| XOR A,byte      | XRI A,byte     | A<-A^byte             | 2,3,4     |

| XOR r,byte      | XRI r,byte     | r<-r^byte             | 2,3,4     |

| XOR sr2,byte    | XRI sr2,byte   | sr2<-sr2^byte         | 2,3,4     |

| XOR A,(wa)      | XRAW wa        | A<-A^(wa)             | 2,3,4     |

| XOR EA,rp3      | DXR EA,rp3     | EA<-EA^rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKGT A,r        | GTA A,r        | skip if A>r           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKGT r,A        | GTA r,A        | skip if r>A           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKGT A,(rpa)    | GTAX rpa       | skip if A>(rpa)       | 1,2,3,4   |

| SKGT A,byte     | GTI A,byte     | skip if A>byte        | 1,2,3,4   |

| SKGT r,byte     | GTI r,byte     | skip if r>byte        | 2,3,4     |

| SKGT sr2,byte   | GTI sr2,byte   | skip if sr2>byte      | 2,3,4     |

| SKGT A,(wa)     | GTAW wa        | skip if A>(wa)        | 2,3,4     |

| SKGT (wa),byte  | GTIW wa,byte   | skip if (wa)>byte     | 1,2,3,4   |

| SKGT EA,rp3     | DGT EA,rp3     | skip if EA>rp3        | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKLT A,r        | LTA A,r        | skip if A<r           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKLT r,A        | LTA r,A        | skip if r<A           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKLT A,(rpa)    | LTAX rpa       | skip if A<(rpa)       | 1,2,3,4   |

| SKLT A,byte     | LTI A,byte     | skip if A<byte        | 1,2,3,4   |

| SKLT r,byte     | LTI r,byte     | skip if r<byte        | 2,3,4     |

| SKLT sr2,byte   | LTI sr2,byte   | skip if sr2<byte      | 2,3,4     |

| SKLT A,(wa)     | LTAW wa        | skip if A<(wa)        | 2,3,4     |

| SKLT (wa),byte  | LTIW wa,byte   | skip if (wa)<byte     | 1,2,3,4   |

| SKLT EA,rp3     | DLT EA,rp3     | skip if EA<rp3        | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKNE A,r        | NEA A,r        | skip if A!=r          | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKNE r,A        | NEA r,A        | skip if r!=A          | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKNE A,(rpa)    | NEAX rpa       | skip if A!=(rpa)      | 1,2,3,4   |

| SKNE A,byte     | NEI A,byte     | skip if A!=byte       | 1,2,3,4   |

| SKNE r,byte     | NEI r,byte     | skip if r!=byte       | 2,3,4     |

| SKNE sr2,byte   | NEI sr2,byte   | skip if sr2!=byte     | 2,3,4     |

| SKNE A,(wa)     | NEAW wa        | skip if A!=(wa)       | 2,3,4     |

| SKNE (wa),byte  | NEIW wa,byte   | skip if (wa)!=byte    | 1,2,3,4   |

| SKNE EA,rp3     | DNE EA,rp3     | skip if EA!=rp3       | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKEQ A,r        | EQA A,r        | skip if A=r           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKEQ r,A        | EQA r,A        | skip if r=A           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKEQ A,(rpa)    | EQAX rpa       | skip if A=(rpa)       | 1,2,3,4   |

| SKEQ A,byte     | EQI A,byte     | skip if A=byte        | 1,2,3,4   |

| SKEQ r,byte     | EQI r,byte     | skip if r=byte        | 2,3,4     |

| SKEQ sr2,byte   | EQI sr2,byte   | skip if sr2=byte      | 2,3,4     |

| SKEQ A,(wa)     | EQAW wa        | skip if A=(wa)        | 2,3,4     |

| SKEQ (wa),byte  | EQIW wa,byte   | skip if (wa)=byte     | 1,2,3,4   |

| SKEQ EA,rp3     | DEQ EA,rp3     | skip if AEA=rp3       | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKON A,r        | ONA A,r        | skip if (A&r)!=0      | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKON r,A        | ONA r,A        | skip if (r&A)!=0      | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKON A,(rpa)    | ONAX rpa       | skip if (A&(rpa))!=0  | 1,2,3,4   |

| SKON A,byte     | ONI A,byte     | skip if (A&byte)!=0   | 1,2,3,4   |

| SKON r,byte     | ONI r,byte     | skip if (r&byte)!=0   | 2,3,4     |

| SKON sr2,byte   | ONI sr2,byte   | skip if (sr2&byte)!=0 | 2,3,4     |

| SKON A,(wa)     | ONAW wa        | skip if (A&(wa))!=0   | 2,3,4     |

| SKON (wa),byte  | ONIW wa,byte   | skip if (wa)&byte!=0  | 1,2,3,4   |

| SKON EA,rp3     | DON EA,rp3     | skip if (EA&rp3)!=0   | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKOFF A,r       | OFFA A,r       | skip if (A&r)=0       | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKOFF r,A       | OFFA r,A       | skip if (r&A)=0       | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKOFF A,(rpa)   | OFFAX rpa      | skip if (A&(rpa))=0   | 1,2,3,4   |

| SKOFF A,byte    | OFFI A,byte    | skip if (A&byte)=0    | 1,2,3,4   |

| SKOFF r,byte    | OFFI r,byte    | skip if (r&byte)=0    | 2,3,4     |

| SKOFF sr2,byte  | OFFI sr2,byte  | skip if (sr2&byte)=0  | 2,3,4     |

| SKOFF A,(wa)    | OFFAW wa       | skip if (A&(wa))=0    | 2,3,4     |

| SKOFF (wa),byte | OFFIW wa,byte  | skip if (wa)&byte=0   | 1,2,3,4   |

| SKOFF EA,rp3    | DOFF EA,rp3    | skip if (EA&rp3)=0    | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| INC r2          | INR r2         | r2<-r2+1              | 1,2,3,4   |

| INC (wa)        | INRW wa        | (wa)<-(wa)+1          | 1,2,3,4   |

| INC rp          | INX rp         | rp<-rp+1              | 1,2,3,4   |

+-----------------+----------------+-----------------------+-----------+

| DEC r2          | DCR r2         | r2<-r2-1              | 1,2,3,4   |

| DEC (wa)        | DCRW wa        | (wa)<-(wa)-1          | 1,2,3,4   |

| DEC rp          | DCX rp         | rp<-rp-1              | 1,2,3,4   |

+-----------------+----------------+-----------------------+-----------+

| CPU Group 1: 78C05, 78C06                                            |

| CPU Group 2: 7800, 7801, 7802                                        |

| CPU Group 3: 7807, 7808, 7809, 7810                                  |

| CPU Group 4: 7810, 78C1x                                             |

+-----------------+----------------+-----------------------+-----------+

4.62. 75K0

----------

Similar  to other  processors, the  assembly language  of the 75 series

also  knows  pseudo  bit  operands,  i.e.  it  is  possible to assign a

combination  of address  and bit  number to  a symbol  that can then be

used  as an argument  for bit oriented  instructions just like explicit

expressions.  The  following  three  instructions  for example generate

the same code:

ADM     sfr     0fd8h

SOC     bit     ADM.3

skt     0fd8h.3

skt     ADM.3

skt     SOC

AS  distinguishes direct and  symbolic bit accesses  by the missing dot

in  symbolic names;  it is  therefore forbidden  to use  dots in symbol

names to avoid misunderstandings in the parser.

The  storage format of bit symbols  mostly accepts the binary coding in

the  machine instructions themselves: 16 bits  are used, and there is a

''long''  and  a  ''short''  format.  The  short  format  can store the

following variants:

- direct accesses to the address range from 0FBxH to 0FFxH

- indirect accesses in the style of Addr.@L (0FC0H <= Addr <=0FFFH)

- indirect accesses in the style of @H+d4.bit

The  upper  byte  is  set  to  0,  the  lower  byte  contains  the  bit

expression  coded according to [106]. The  long format in contrast only

knows  direct  addressing,  but  it  can  cover the whole address space

(given  a correct  setting of  MBS and  MBE). A  long expression stores

bits  0..7 of the address  in the lower byte,  the bit position in bits

8  and 9, and  a constant value  of 01 in  bits 10 and  11. The highest

bits  allow to distinguish easily between  long and short addresses via

a  check if the upper byte is 0.  Bits 12..15 contain bits 8..11 of the

address;  they are not  needed to generate  the code, but  they have to

be  stored somewhere  as the  check for  correct banking  can only take

place when the symbol is actually used.

4.63. 78K0

----------

NEC  uses  different  ways  to  mark  absolute  addressing  in its data

books:

- absolute short: no prefix

- absolute long: prefix of !

- PC relative: prefix of $

Under  AS, these prefixes  are only necessary  if one wants  to force a

certain   addressing   mode   and   the  instruction  allows  different

variants.  Without a prefix, AS  will automatically select the shortest

variant.  It should  therefore rarely  be necessary  to use a prefix in

practice.

4.64. 78K2/78K3/78K4

--------------------

Analogous   to  the  78K0,   NEC  here  also   uses  dollar  signs  and

exclamation   marks   to   specify   different   lengths   of   address

expressions.  The selection  between long  and short  addresses is done

automatically  (both in  RAM and  SFR areas),  only relative addressing

has  to  be  selected  explicitly,  if  an  instruction  supports  both

variants (like BR).

An  additional  remark  (which  is  also  true for the 78K0): Those who

want   to  use  Motorola   syntax  via  RELAXED,   might  have  to  put

hexadecimal  constants in  parentheses, since  the leading  dollar sign

might be misunderstood as relative addressing...

4.65. uPD772x

-------------

Both  the 7720  and 7725  are provided  by the  same code generator and

are  extremely  similar  in  their  instruction set. One should however

not  beleive  that  they  are  binary  compatible: To get space for the

longer  address fields  and additional  instructions, the bit positions

of   some  fields  in  the  instruction  word  have  changed,  and  the

instruction  length has  changed from  23 to  24 bits.  The code format

therefore uses different header ids for both CPUs.

They  both  have  in  common  that  in  addition  to  the code and data

segment,  there is also a ROM for  storage of constants. In the case of

AS, it is mapped onto the ROMDATA segment!

4.66. uCOM-43

-------------

The  uCOM-43 instruction set contains  an instruction CZP that performs

a  single  byte  subroutine  call  to  the  addresses 0, 4, 8, 12...60.

However,   the   available   documentation   is   unclear   about   the

instruction's  argument  in  source  code:  should  it  be  the  target

address  (i.e.,  a  multiple  of  four)  or  th vector contained in the

opcode  (i.e.,  a  number  from  0  to  15).  To  decide, the assembler

performs some guessing:

- If  the  argument  is  larger  than  15,  it  must  be  an address,

otherwise:

- If  the argument is  not a multiple  of four, it  must be a vector,

otherwise:

- If the argument is zero, no distinction is possible, otherwise:

- If  the argument  is a  symbol from  CODE space  (e.g. a label), it

must be an address, otherwise:

- The argument is a vector.

The  final two rules only  apply to the values  4, 8, and 12, and since

the  decision is  not intuitively  clear, the  decision for a vector is

accompanied  with a warning.  In case an  address was meant, define the

argument as symbol from CODe space:

dest   label  4

.

.

.

czp dest

In  case a  vector was  meant, and  the warning  bothers you, it may be

suppressed this way:

vector equ    4

.

.

.

expect 480

czp    vector

endexpect

I  admit that  this situation  is not  100% satisfactory. If anyone has

information  about the behaviour of the  original NEC assembler, I'd be

grateful for contacting me.

4.67. F2MC16L

-------------

Along  with  the  discussion  of  the  ASSUME statement, it has already

been  mentioned that  it is  important to  inform AS  about the correct

current  values of all bank registers -  if your program uses more than

64K  RAM or 64K  ROM. With these  assumptions in mind,  AS checks every

direct  memory access for attempts to  access a memory location that is

currently  not in  reach. Of  course, standard  situations only require

knowledge  of DTB  and DPR  for this  purpose, since  ADB resp. SSB/USB

are  only  used  for  indirect  accesses  via RW2/RW6 resp. RW3/RW7 and

this  mechanism  anyway  doesn't  work  for indirect accesses. However,

similar  to the 8086, it  is possible to place  a prefix in front of an

instruction  to  replace  DTB  by  a  different  register. AS therefore

keeps  track  of  used  segment  prefixes and toggles appropriately for

the  next machine instruction. A  pseudo instruction placed between the

prefix  and the machine instruction does  not reset the toggle. This is

also  true  for  pseudo  instructions  that  store  data  or modify the

program counter. Which doesn't make much sense anyway...

4.68. MN161x

------------

This   target  is  special   because  there  are   two  different  code

generators  one may choose  from. The first  one was kindly provided by

Haruo  Asano  and  that  may  be  reached  via  the  CPU  names  MN1610

resp.MN1613.  The other one was written by  me and is activated via the

CPU  names MN1610ALT resp.  MN1613ALT. If you  want to use the MN1613's

extended  address space  of 256  KWords, or  if you  want to experiment

with  the  MN1613's  floating  point  formant,  you have to use the ALT

target.

4.69. CDP180x

-------------

This  family of  processors supports  both long  and short  branches: a

short  branch is  only possible  within the  same 256 byte memory page,

and  a long branch is possible to  any target in the 64K address space.

The  assembly  syntax  provides  different  mnemonics for both variants

(the  long variant with  a leading 'L'),  but there is  no variant that

would  let  the  assembler  decide  itself  between  long  or short. AS

supports such 'pseudo instructions' as an extension:

- JMP becomes BR oder LBR.

- JZ becomes BZ oder LBZ.

- JNZ becomes BNZ oder LBNZ.

- JDF becomes BDF oder LBDF.

- JPZ becomes BPZ oder LBPZ.

- JGE becomes BGE oder LBGE.

- JNF becomes BNF oder LBNF.

- JM becomes BM oder LBM.

- JL becomes BL oder LBL.

- JQ becomes BQ oder LBQ.

- JNQ becomes BNQ oder LBNQ.

4.70. KENBAK

------------

The  KENBAK-1  was  developed  in  1970,  at  a  time  when  the  first

microprocessor  was still  three years  away. One  may assume  that for

the  few hobbyists that could afford the  kit back then, this was their

first  and  only  computer.  As  a  consequence,  they had nothing they

could  run an assembler on,  the KENBAK-1 itself with  its 256 bytes of

memory  was way too small for such  a task. The preferred method was to

use  pre-printed tables, which  had fields to  fill in instructions and

machine  codes.  Once  this  ''programming  job''  was  done, one would

enter the machine code manually via the computer's switch row.

The  effect of  this is  that though  the KENBAK's assembly language is

described  in the  manual, there  is no  real formal  definition of it.

When  Grant Stockly released new KENBAK kits  a few years ago, he did a

first  implementation  of  the  KENBAK  on my assembler. Unfortunately,

this  never  went  upstream.  I  tried  to  take  up  his  ideas  in my

implementation,  but on the other  hand I also tried  to offer a syntax

that  should  be  familiar  to  programmers  of  6502,  Z80  or similar

processors. The following table lists the syntax differences:

Table 4.10: KENBAK-Befehlssyntax

+------------------------------------+------------------------+------------------------+

| Stockly                            | Alternativ             | Bemerkung              |

+------------------------------------+------------------------+------------------------+

+------------------------------------+------------------------+------------------------+

| Arithmetic/Logic (ADD/SUB/LOAD/STORE/AND/OR/LNEG)                                    |

+------------------------------------+------------------------+------------------------+

| instr Constant, Reg, Wert,         | instr Reg, #Wert       | immediate              |

| instr Memory, Reg, Addr,           | instr Reg, Addr        | direct                 |

| instr Indirect, Reg, Addr,         | instr Reg, (Addr)      | direct                 |

| instr Indexed, Reg, Addr,          | instr Reg, Addr,X      | indexed                |

| instr Indirect-Indexed, Reg, Addr, | instr Reg, (Addr),X    | indirect-indexed       |

+------------------------------------+------------------------+------------------------+

| Jumps                                                                                |

+------------------------------------+------------------------+------------------------+

| JPD Reg, Cond, Addr                | JP Reg, Cond, Addr     | conditional-direct     |

| JPI Reg, Cond, Addr                | JP Reg, Cond, (Addr)   | conditional-indirect   |

| JMD Reg, Cond, Addr                | JM Reg, Cond, Addr     | conditional-direct     |

| JMI Reg, Cond, Addr                | JM Reg, Cond, (Addr)   | conditional-indirect   |

| JPD Unconditional, Cond, Addr      | JP Addr                | unconditional-direct   |

| JPI Unconditional, Cond, Addr      | JP (Addr)              | unconditional-indirect |

| JMD Unconditional, Cond, Addr      | JM Addr                | unconditional-direct   |

| JMI Unconditional, Cond, Addr      | JM (Addr)              | unconditional-indirect |

+------------------------------------+------------------------+------------------------+

| Jump Conditions                                                                      |

+------------------------------------+------------------------+------------------------+

| Non-zero                           | NZ                     | != 0                   |

| Zero                               | Z                      | = 0                    |

| Negative                           | N                      | < 0                    |

| Positive                           | P                      | >= 0                   |

| Positve-Non-zero                   | PNZ                    | > 0                    |

+------------------------------------+------------------------+------------------------+

| Skips                                                                                |

+------------------------------------+------------------------+------------------------+

| SKP 0, bit, Addr                   | SKP0 bit, Addr [,Dest] |                        |

| SKP 1, bit, Addr                   | SKP1 bit, Addr [,Dest] |                        |

+------------------------------------+------------------------+------------------------+

| Bit Manipulation                                                                     |

+------------------------------------+------------------------+------------------------+

| SET 0, bit, Addr                   | SET0 bit, Addr         |                        |

| SET 1, bit, Addr                   | SET1 bit, Addr         |                        |

+------------------------------------+------------------------+------------------------+

| Shifts/Rotates                                                                       |

+------------------------------------+------------------------+------------------------+

| SHIFT LEFT, cnt, Reg               | SFTL [cnt,] Reg        |                        |

| SHIFT RIGHT, cnt, Reg              | SFTR [cnt,] Reg        | arithm. Shift          |

| ROTATE LEFT, cnt, Reg              | ROTL [cnt,] Reg        |                        |

| ROTATE RIGHT, cnt, Reg             | ROTR [cnt,] Reg        |                        |

+------------------------------------+------------------------+------------------------+

There   is  no  pseudo  instruction  to  switch  between  these  syntax

variants. They may both be used anytime and in an arbitrary mix.

The  target  address  [Dest]  that  may  optionally  be  added  to skip

instructions  will not become  part of the  machine code. The assembler

only  checks  whether  the  processor  wil  actually  skip to the given

address.  This allows for instance to  check whether one actually tries

to  skip a one-byte  instruction. If the  shift count argument [cnt] is

omitted, a one-bit shift/rotate is coded.

4.71. HP Nanoprocessor

----------------------

The  HP Nanoprocessor  does not  provide any  instructions to read data

from  the ROM  address space.  The respective  instructions LDR and STR

rather  represent  what  is  called  ''immediate  addressing'' on other

processors.  For this reson,  there are pseudo  instructions that would

allow placing data constants im ROM memory or to reserve space in it.

4.72. IM61x0

------------

This  microprocessor  is  effectively  a  single chip implementation of

the  PDP8/E, which  is why  Digital Equipment's  PAL-II is  usually the

''reference    assembler''   for   source    code   samples.   The   AS

implementation  deviates from the PAL-III syntax  in a couple of areas,

among  other reasons also  because several things  only could have been

provided  with huge  efforts. Here  are some  hints about  how to adapt

existing code:

- PAL-III  marks  labels  by  an  appended  comma. AS instead uses an

appended  double  colon,  or  no  special  character  at all if the

label begins in the first column of a line.

- Placing  constants  in  memory  is  done  with  PAL-III  simply  by

writing  the numeric  constant instead  of a  mnemonic. AS uses the

DC  instruction to  place data,  which will  also accept  more than

one  word as argument. However, the  LTORG mechanism may be used in

some  cases to  get around  explicitly placing  constants in memory

at all.

- The program counter is set by ORG address instead of*address.

5. File Formats

===============

In  this chapter, the formats of  files AS generates shall be explained

whose formats are not self-explanatory.

5.1. Code Files

---------------

The  format for code files  generated by the assembler  must be able to

separate   code  parts   that  were   generated  for  different  target

processors;  therefore, it is a bit  different from most other formats.

Though  the assembler package  contains tools to  deal with code files,

I think is a question of good style to describe the format in short:

If  a code  file contains  multibyte values,  they are stored in little

endian  order. This  rule is  already valid  for the  16-bit magic word

$1489, i.e. every code file starts with the byte sequence $89/$14.

This  magic word is  followed by an  arbitrary number of ''records''. A

record  may either  contain a  continuous piece  of the code or certain

additional  information. Even without  switching to different processor

types,  a  file  may  contain  several code-containing records, in case

that  code or  constant data  areas are  interrupted by reserved memory

areas  that should  not be  initialized. This  way, the assembler tries

to keep the file as short as possible.

Common  to  all  records  is  a  header byte which defines the record's

type  and  its  contents.  Written  in  a  PASCALish  way,  the  record

structure can be described in the following way:

FileRecord = RECORD CASE Header:Byte OF

$00:(Creator:ARRAY[] OF Char);

$01..

$7f:(StartAdr : LongInt;

Length   : Word;

Data     : ARRAY[0..Length-1] OF Byte);

$80:(EntryPoint:LongInt);

$81:(Header   : Byte;

Segment  : Byte;

Gran     : Byte;

StartAdr : LongInt;

Length   : Word;

Data     : ARRAY[0..Length-1] OF Byte);

END

This  description  does  not  express  fully  that  the  length of data

fields is variable and depends on the value of the Length entries.

A  record with a header  byte of $81 is  a record that may contain code

or  data  from  arbitrary  segments.  The first byte (Header) describes

the  processor family the  following code resp.  data was generated for

(see table 5.1).

Table 5.1: Header Bytes for the Different Processor Families

+--------+--------------++--------------------------+--------+

| Header | Family       ||          Header          | Family |

+--------+--------------++--------------------------+--------+

+--------+--------------++--------------------------+--------+

|   $01  | 680x0, 6833x ||                          |        |

|        | $02          ||       ATARI_VECTOR       |        |

|        | $03          ||          M*Core          |        |

|        | $04          ||           XGATE          |        |

|        | $05          ||          PowerPC         |        |

|        | $06          ||           XCore          |        |

|        | $07          ||          TMS1000         |        |

|        | $08          ||          NS32xxx         |        |

|        | $09          ||         DSP56xxx         |        |

|        | $0a          ||          CP-1600         |        |

|        | $0b          ||     HP Nano Processor    |        |

|        | $0c          ||        IM6100/6120       |        |

|        | $0d          ||          NEC V60         |        |

|        | $0e          ||         IBM PALM         |        |

|        | $0f          ||           CP-3F          |        |

|        | $10          ||      Rockwell PPS-4      |        |

|        | $11          ||      65xx/MELPS-740      |        |

|        | $12          ||        MELPS-4500        |        |

|        | $13          ||            M16           |        |

|        | $14          ||           M16C           |        |

|        | $15          ||          F^2MC8L         |        |

|        | $16          ||         F^2MC16L         |        |

|        | $17          ||          IMP-16          |        |

|        | $18          ||      IPC-16/INS8900      |        |

|        | $19          ||     65816/MELPS-7700     |        |

|        | $1a          ||           PDK13          |        |

|        | $1b          ||           PDK14          |        |

|        | $1c          ||           PDK15          |        |

|        | $1d          ||           PDK16          |        |

|        | $1e          ||        Renesas RX        |        |

|        | $1f          ||          SC61860         |        |

|        | $20          ||          SC62015         |        |

|        | $21          ||          MCS-48          |        |

|        | $22          ||       Konami 052001      |        |

|        | $23          ||          PDP-11          |        |

|        | $24          ||           WD16           |        |

|        | $25          ||         SYM53C8xx        |        |

|        | $26          ||            VAX           |        |

|        | $27          ||          KENBAK          |        |

|        | $28          ||          �COM-43         |        |

|        | $29          ||           29xxx          |        |

|        | $2a          ||           i960           |        |

|        | $31          ||          MCS-51          |        |

|        | $32          ||            ST9           |        |

|        | $33          ||            ST7           |        |

|        | $35          ||           Z8000          |        |

|        | $35          ||          Super8          |        |

|        | $36          ||          MN161x          |        |

|        | $37          ||           2650           |        |

|        | $38          ||         1802/1805        |        |

|        | $39          ||      MCS-96/196/296      |        |

|        | $3a          ||           8X30x          |        |

|        | $3b          ||            AVR           |        |

|        | $3c          ||            XA            |        |

|        | $3d          || AVR (8-Bit Code-Segment) |        |

|        | $3e          ||           8008           |        |

|        | $3f          ||         4004/4040        |        |

|        | $40          ||            H16           |        |

|        | $41          ||         8080/8085        |        |

|        | $42          ||         8086..V35        |        |

|        | $43          ||           SX20           |        |

|        | $44          ||            F8            |        |

|        | $45          ||           S12Z           |        |

|        | $46          ||           78K4           |        |

|        | $47          ||         TMS320C6x        |        |

|        | $48          ||          TMS9900         |        |

|        | $49          ||         TMS370xxx        |        |

|        | $4a          ||          MSP430          |        |

|        | $4b          ||        TMS320C54x        |        |

|        | $4c          ||        80C166/167        |        |

|        | $4d          ||          OLMS-50         |        |

|        | $4e          ||          OLMS-40         |        |

|        | $4f          ||       MIL STD 1750       |        |

|        | $50          ||         HMCS-400         |        |

|        | $51          ||     Z80/180/380/eZ80     |        |

|        | $52          ||         TLCS-900         |        |

|        | $53          ||          TLCS-90         |        |

|        | $54          ||         TLCS-870         |        |

|        | $55          ||          TLCS-47         |        |

|        | $56          ||         TLCS-9000        |        |

|        | $57          ||        TLCS-870/C        |        |

|        | $58          ||         NEC 78K3         |        |

|        | $59          ||            eZ8           |        |

|        | $5a          ||          TC9331          |        |

|        | $5b          ||          KCPSM3          |        |

|        | $5c          ||       LatticeMico8       |        |

|        | $5d          ||         NEC 75xx         |        |

|        | $5e          ||          68RS08          |        |

|        | $5f          ||           COP4           |        |

|        | $60          ||           78K2           |        |

|        | $61          ||     6800, 6301, 6811     |        |

|        | $62          ||         6805/HC08        |        |

|        | $63          ||           6809           |        |

|        | $64          ||           6804           |        |

|        | $65          ||          68HC16          |        |

|        | $66          ||          68HC12          |        |

|        | $67          ||            ACE           |        |

|        | $68          ||         H8/300(H)        |        |

|        | $69          ||          H8/500          |        |

|        | $6a          ||           807x           |        |

|        | $6b          ||           KCPSM          |        |

|        | $6c          ||          SH7000          |        |

|        | $6d          ||          SC14xxx         |        |

|        | $6e          ||           SC/MP          |        |

|        | $6f          ||           COP8           |        |

|        | $70          ||         PIC16C8x         |        |

|        | $71          ||         PIC16C5x         |        |

|        | $72          ||         PIC17C4x         |        |

|        | $73          ||         TMS-7000         |        |

|        | $74          ||         TMS3201x         |        |

|        | $75          ||         TMS320C2x        |        |

|        | $76          ||       TMS320C3x/C4x      |        |

|        | $77          ||      TMS320C20x/C5x      |        |

|        | $78          ||            ST6           |        |

|        | $79          ||            Z8            |        |

|        | $7a          ||        �PD78(C)10        |        |

|        | $7b          ||           75K0           |        |

|        | $7c          ||           78K0           |        |

|        | $7d          ||          �PD7720         |        |

|        | $7e          ||          �PD7725         |        |

|        | $7f          ||         �PD77230         |        |

|        |              ||                          |        |

+--------+--------------++--------------------------+--------+

The  Segment  field  signifies  the  address  space  the following code

belongs to. The assignment defined in table 5.1 applies.

+--------+-------------++--------+---------+

| number | segment     || number | segment |

+--------+-------------++--------+---------+

+--------+-------------++--------+---------+

|   $00  | <undefined> ||   $01  | CODE    |

|   $02  | DATA        ||   $03  | IDATA   |

|   $04  | XDATA       ||   $05  | YDATA   |

|   $06  | BDATA       ||   $07  | IO      |

|   $08  | REG         ||   $09  | ROMDATA |

+--------+-------------++--------+---------+

Table 5.1: Codings of the Segment Field

The  Gran field describes the code's  ''granularity'', i.e. the size of

the  smallest  addressable  unit  in  the  following  set of data. This

value  is a function of processor type  and segment and is an important

parameter  for  the  interpretation  of  the  following two fields that

describe  the block's  start address  and its  length: While  the start

address   refers  to  the  granularity,  the  Length  value  is  always

expressed  in bytes! For example, if the  start address is $300 and the

length   is  12,  the  resulting  end  address  would  be  $30b  for  a

granularity  of 1, however  $303 for a  granularity of 4! Granularities

that  differ from 1  are rare and  mostly appear in  DSP CPU's that are

not  designed  for  byte  processing.  For  example, a DSP56K's address

space  is  organized  in  64  Kwords  of 16 bits. The resulting storage

capacity  is 128 Kbytes,  however it is  organized as 2  ^16 words that

are addressed with addresses 0,1,2,...65535!

The  start  address  is  always  32  bits  in  size, independent of the

processor  family. In  contrast, the  length specification  has only 16

bits,  i.e.  a  record  may  have  a  maximum length of 4+4+2+(64K-1) =

65545 bytes.

Data  records with a Header ranging from  $01 to $7f present a shortcut

and  preserve  backward  compatibility  to  earlier  definitions of the

file  format: in their case, the  Header directly defines the processor

type,  the  target  segment  is  fixed  to  CODE and the granularity is

implicitly  given by the  processor type, rounded  up to the next power

of  two. AS prefers to  use these records whenever  data or code should

go into the CODE segment.

A  record  with  a  Header  of  $80  defines  an  entry point, i.e. the

address  where execution of the program  should start. Such a record is

the  result  of  an  END  statement  with  a  corresponding  address as

argument.

The  last record in a  file bears the Header  $00 and has only a string

as   data  field.  This  string  does   not  have  an  explicit  length

specification;  its  end  is  equal  to  the  file's  end.  The  string

contains  only the name  of the program  that created the  file and has

no further meaning.

5.2. Debug Files

----------------

Debug  files may optionally be generated  by AS. They deliver important

information  for  tools  used  after  assembly,  like  disassemblers or

debuggers.  AS can  generate debug  files in  one of  three formats: On

the  one  hand,  the  object  format  used  by the AVR tools from Atmel

respectively  a NoICE-compatible  command file,  and on  the other hand

an  own format. The first two are  described in detail in [5] resp. the

NoICE  documentations, which  is why  the following  description limits

itself to the AS-specific MAP format:

The information in a MAP file is split into three groups:

- symbol table

- memory usage per section

- machine addresses of source lines

The  second item is  listed first in  the file. A  single entry in this

list consists of two numbers that are separated by a : character:

<line number>:<address>

Such  an entry  states that  the machine  code generated for the source

statement  in  a  certain  line  is  stored  at  the  mentioned address

(written   in  hexadecimal  notation).  With  such  an  information,  a

debugger  can  display  the  corresponding  source lines while stepping

through  a program. As a program  may consist of several include files,

and  due  to  the  fact  that  a  lot  of processors have more than one

address  space (though  admittedly only  one of  them is  used to store

executable  code), the  entries described  above have  to be sorted. AS

does  this sorting in  two levels: The  primary sorting criteria is the

target  segment, and  the entries  in one  of these sections are sorted

according  to files.  The sections  resp. subsections  are separated by

special lines in the style of

Segment <segment name>

resp.

File <file name>   .

The  source line info is  followed by the symbol  table. Similar to the

source  line  info,  the  symbol  table  is  primarily  sorted  by  the

segments  individual  symbols  are  assigned  to.  In  contrast  to the

source  line info, an additional  section NOTHING exists which contains

the  symbols  that  are  not  assigned  to  any  specific segment (e.g.

symbols  that  have  been  defined  with  a  simple  EQU  statement). A

section  in the symbol  table is started  with a line  of the following

type:

Symbols in Segment <segment name>

The  symbols  in  a  section  are  sorted according to the alphabetical

order  of their  names, and  one symbol  entry consists  of exactly one

line.  Such a  line consists  of six  fields witch  are separated by at

least a single space:

The  first field is  the symbol's name,  possibly extended by a section

number  enclosed in  brackets. Such  a section  number limits the range

of  validity for  a symbol.  The second  field designates  the symbol's

type:   Int  stands  for  integer  values,  Float  for  floating  point

numbers,  and  String  for  character  arrays.  The third field finally

contains  the symbol's  value. If  the symbol  contains a string, it is

necessary  to  use  a  special  encoding  for  control  characters  and

spaces.   Without  such  a   coding,  spaces  in   a  string  could  be

misinterpreted  as  delimiters  to  the  next  field.  AS uses the same

syntax  that is  also valid  for assembly  source files: Instead of the

character,  its ASCII value  with a leading  backslash (\) is inserted.

For example, the string

This is a test

becomes

This\032is\032\a\032test   .

The  numerical value always has three  digits and has to be interpreted

as  a decimal  value. Naturally,  the backslash  itself also  has to be

coded this way.

The  fourth  field  specifies  -  if  available  - the size of the data

structure  placed at  the address  given by  the symbol. A debugger may

use  this  information  to  automatically  display  variables  in their

correct  length when  they are  referred symbolically.  In case AS does

not  have  any  information  about  the  symbol size, this field simply

contains the value -1.

The  fifth field states  via the values  0 or 1  if the symbol has been

used  during assembly.  A program  that reads  the symbol table can use

this  field to skip  unused symbols as  they are probably unused during

the following debugging/disassembly session.

Finally,  the sixth field  states via the  values 0 or  1 if the symbol

is  a constant (0) or variable(1).  Constant symbols are set once, e.g.

via  the  EQU  statement  or  a  label,  while variables are allowed to

change  their value during  the course of  assembly. The MAP file lists

the final value.

The  third section in a debug  file describes the program's sections in

detail.  The  need  for  such  a  detailed  description arises from the

sections'  ability to limit  the validity range  of symbols. A symbolic

debugger   for  example  cannot  use  certain  symbols  for  a  reverse

translation,  depending on  the current  PC value.  It may also have to

regard  priorities  for  symbol  usage  when  a value is represented by

more  than one symbol. The  definition of a section  starts with a line

of the following form:

Info for Section nn ssss pp

nn  specifies the  section's number  (the number  that is  also used in

the  symbol table as a  postfix for symbol names),  ssss gives its name

and  pp  the  number  of  its  parent  section. The last information is

needed  by a  retranslator to  step upward  through a  tree of sections

until  a fitting  symbol is  found. This  first line  is followed  by a

number  of  further  lines  that  describe  the code areas used by this

section.  Every  single  entry  (exactly  one  entry  per  line) either

describes  a single address  or an address  range given by  a lower and

an  upper bound (separation of lower and  upper bound by a minus sign).

These  bounds  are  ''inclusive'',  i.e.  the  bounds  themselves  also

belong  to the area. Is is important  to note that an area belonging to

a   section  is  not  additionally  listed  for  the  section's  parent

sections  (an exception is  of course a  deliberate multiple allocation

of  address areas, but you  would not do this,  would you?). On the one

hand,   this  allows  an  optimized  storage  of  memory  areas  during

assembly.  On  the  other  hand,  this  should  not  be an obstacle for

symbol   backtranslation  as   the  single   entry  already   gives  an

unambiguous  entry point  for the  symbol search  path. The description

of a section is ended by an empty line or the end of the debug file.

Program  parts that lie  out of any  section are not listed separately.

This  implicit ''root section'' carries the  number -1 and is also used

as  parent  section  for  sections  that  do  not  have  a  real parent

section.

It  is possible  that the  file contains  empty lines or comments (semi

colon  at line start).  A program reading  the file has  to ignore such

lines.

6. Utility Programs

===================

To  simplify the work with  the assembler's code format  a bit, I added

some  tools  to  aid  processing  of  code  files.  These  programs are

released under the same license terms as stated in section 1.1!

Common  to all programs are the  possible return codes they may deliver

upon completion (see table 6.1).

+-------------+----------------------------------+

| return code | error condition                  |

+-------------+----------------------------------+

+-------------+----------------------------------+

|      0      | no errors                        |

|      1      | error in command line parameters |

|      2      | I/O error                        |

|      3      | file format error                |

+-------------+----------------------------------+

Table 6.1: Return Codes of the Utility Programs

Just  like  AS,  all  programs  take  their  input from STDIN and write

messages  to STDOUT (resp. error  messages to STDERR). Therefore, input

and output redirections should not be a problem.

In  case that  numeric or  address specifications  have to  be given in

the  command line,  they may  also be  written in hexadecimal notation,

either  ba allending a h or prepending  a dollar character or a 0x like

in C. (e.g. 10h, $10, or 0x10 instead of 16).

Unix  shells  however  assign  a  special  meaning  to the dollar sign,   UNIX

which  makes it  necessary to  escape a  dollar sign  with a backslash.

The 0x variant is definitely more comfortable in this case.

Otherwise,  calling conventions and variations  are equivalent to those

of  AS (except  for PLIST  and AS2MSG);  i.e. it  is possible  to store

frequently  used parameters in  an environment variable  (whose name is

constructed  by appending CMD  to the program's  name, i.e. BINDCMD for

BIND),  to  negate  options,  and  to  use  all upper- resp. lower-case

writing (for details on this, see section 2.4).

Address   specifications  always  relate  to  the  granularity  of  the

processor  currently in  question; for  example, on  a PIC,  an address

difference of 1 means a word and not a byte.

6.1. PLIST

----------

PLIST  is the simplest  one of the  five programs supplied: its purpose

is  simply to list all  records that are stored  in a code file. As the

program does not do very much, calling is quite simple:

PLIST <file name>

The  file name will  automatically be extended  with the extension P if

it doesn't already have one.

CAUTION!  At  this  place,  no  wildcards  are  allowed!  If there is a

necessity  to list  several files  with one  command, use the following

''mini batch'':

for %n in (*.p) do plist %n

PLIST  prints  the  code  file's  contents  in  a  table style, whereby

exactly  one line will be printed  per record. The individual rows have

the following meanings:

- code type: the processor family the code has been generated for.

- start  address:  absolute  memory  address  that expresses the load

destination for the code.

- length: length of this code chunk in bytes.

- end  address:  last  address  of  this  code chunk. This address is

calculated as start address+length-1.

All outputs are in hexadecimal notation.

Finally,  PLIST will print a  copyright remark (if there  is one in the

file), together with a summaric code length.

Simply  said, PLIST is a sort of DIR  for code files. One can use it to

examine a file's contents before one continues to process it.

6.2. BIND

---------

BIND  is a  program that  allows to  concatenate the records of several

code  files into  a single  file. A  filter function  is available that

can  be used to copy  only records of certain  types. Used in this way,

BIND can also be used to split a code file into several files.

The general syntax of BIND is

BIND <source file(s)> <target file> [options]

Just  like  AS,  BIND  regards  all  command line arguments that do not

start  with a +, -  or / as file  specifications, of which the last one

must  designate  the  destination  file.  All other file specifications

name sources, which may again contain wildcards.

Currently, BIND defines only one command line option:

- f  <Header[,Header]>: sets a list of  record headers that should be

copied.  Records with other header IDs  will not be copied. Without

such  an option, all  records will be  copied. The headers given in

the  list correspond to the HeaderID  field of the record structure

described  in  section  5.1.  Individual  headers  in this list are

separated with commas.

For  example, to filter  all MCS-51 code  out of a  code file, use BIND

in the following way:

BIND <source name> <target name> -f $31

If  a file  name misses  an extension,  the extension  P will  be added

automatically.

6.3. P2HEX

----------

P2HEX  is an  extension of  BIND. It  has all  command line  options of

BIND  and  uses  the  same  conventions  for file names. In contrary to

BIND,  the target file is written as a  Hex file, i.e. as a sequence of

lines which represent the code as ASCII hex numbers.

P2HEX  knows nine different  target formats, which  can be selected via

the command line parameter F:

- Motorola S-Records (-F Moto))

- MOS Hex (-F MOS)

- Intel Hex (Intellec-8, -F Intel)

- 16-Bit Intel Hex (MCS-86, -F Intel16)

- 32-Bit Intel Hex (-F Intel32))

- Tektronix Hex (-F Tek)

- Texas Instruments DSK (-F DSK)

- Atmel AVR Generic (-F Atmel, see [5])

- Lattice Mico8 prom_init (-F Mico8)

- C arrays, for inclusion into C(++) source files (-F C)

If  no target format is  explicitly specified, P2HEX will automatically

choose  one  depending  in  the  processor type: S-Records for Motorola

CPUs,  Hitachi, and  TLCS-900, MOS  for 65xx/MELPS,  DSK for the 16 bit

signal  processors from  Texas, Atmel  Generic for  the AVRs, and Intel

Hex  for  the  rest.  Depending  on  the  start addresses width, the S-

Record  format will use  Records of type  1, 2, or  3, however, records

in  one group will always  be of the same  type. This automatism can be

partially suppressed via the command line option

-M <1|2|3>

A  value of 2 resp.  3 assures that that  S records with a minimum type

of  2 resp. 3 will be used, while  a value of 1 corresponds to the full

automatism.

Normally,  the AVR  format always  uses an  address length  of 3 bytes.

Some  programs  however  do  not  like  that...which  is why there is a

switch

-avrlen <2|3>

that  allows  to  reduce  the  address  length  to two bytes in case of

emergency.

The  Mico8 format is different from all  the other formats in having no

address  fields - it is plain list  of all instruction words in program

memory.  When  using  it,  be  sure  that  the  used  address range (as

displeyed e.g. by PLIS) starts at zero and is continuous.

The  Intel, MOS and Tektronix formats  are limited to 16 bit addresses,

the  16-bit Intel format reaches 4  bits further. Addresses that are to

long  for a  given format  will be  reported by  P2HEX with  a warning;

afterwards, they will be truncated (!).

For the PIC microcontrollers, the switch

-m <0..3>

allows  to  generate  the  three  different  variants  of the Intel Hex

format.  Format 0 is INHX8M which  contains all bytes in a Lo-Hi-Order.

Addresses  become  double  as  large  because  the  PICs  have  a word-

oriented  address  space  that  increments  addresses  only  by one per

word.  This format is also the  default. With Format 1 (INHX16M), bytes

are  stored in their  natural order. This  is the format Microchip uses

for  its own  programming devices.  Format 2  (INHX8L) resp. 3 (INHX8H)

split  words into  their lower  resp. upper  bytes. With these formats,

P2HEX  has to be called twice to  get the complete information, like in

the following example:

p2hex test -m 2

rename test.hex test.obl

p2hex test -m 3

rename test.hex test.obh

For  the Motorola  format, P2HEX  additionally uses  the S5 record type

mentioned  in [12].  This record  contains the  number of  data records

(S1/S2/S3)  to  follow.  As  some  programs  might not know how to deal

with this record, one can suppress it with the option

+5  .

The  C  format  is  different  in  the  sense  that it always has to be

selected  explicitly. The output file is  basically a complete piece of

C  or  C++  code  that  contains  the  data  as  a  list  of  C arrays.

Additionally  to  the  data  itself,  a  list of descriptors is written

that  describes the start, length, and  end address of each data block.

The contents of these descriptors may be configured via the option

-cformat <format>

Each letter in format defines an element of the descriptor:

- A  d or D  defines a pointer  to the data  itself. Usage of a lower

or  upper  case  letter  defines  whether  lowercase  or  uppercase

letters are used for hexadecimal constants.

- An  s  or  S  defines  the  start  address  of  the data, either as

unsigned or unsigned long.

- An  l or L  defines the length  of the data,  either as unsigned or

unsigned long.

- An  e or E  defines the end  address of the  data, specifically the

last  address  used  by  the  data,  either as unsigned or unsigned

long.

In  case a source  file contains code  record for different processors,

the  different hex formats  will also show  up in the  target file - it

is therefore strongly advisable to use the filter function.

Apart  form  this  filter  function,  P2HEX  also  supports  an address

filter,  which is  useful to  split the  code into  several parts (e.g.

for a set of EPROMs):

-r <start address>-<end address>

The  start address  is the  first address  in the  window, and  the end

address  is the last address in the  window, not the first address that

is  out of  the window.  For example,  to split  an 8051 program into 4

2764 EPROMs, use the following commands:

p2hex <source file> eprom1 -f $31 -r $0000-$1fff

p2hex <source file> eprom2 -f $31 -r $2000-$3fff

p2hex <source file> eprom3 -f $31 -r $4000-$5fff

p2hex <source file> eprom4 -f $31 -r $6000-$7fff

It  is allowed to specifiy  a single dollar character  or '0x' as start

or  stop  address.  This  means  that  the lowest resp. highest address

found  in the source file  shall be taken as  start resp. stop address.

The  default range is  '0x-0x', i.e. all  data from the  source file is

transferred.

CAUTION!   This  type  of  splitting   does  not  change  the  absolute

addresses  that will  be written  into the  files! If  the addresses in

the  individual hex files should rather start  at 0, one can force this

with the additional switch

-a     .

On  the other hand, to move the  addresses to a different location, one

may use the switch

-R <value> .

The  value given is an offset, i.e.  it is added to the addresses given

in the code file.

By  using an  offset, it  is possible  to move  a file's contents to an

arbitrary  position. This offset  is simply appended  to a file's name,

surrounded  with  parentheses.  For  example,  if  the  code  in a file

starts  at address  0 and  you want  to move  it to address 1000 hex in

the hex file, append ($1000) to the file's name (without spaces!).

In  case  the  source  file(s)  not  only  contain  data  for  the code

segment, the switch

-segment <name>

allows  to select the  segment data is  extracted from and converted to

HEX  format. The segment names  are the same as  for the SEGMENT pseudo

instruction  (3.2.18). The TI  DSK is a  special case since  it has the

ability  to distinguish between  data and code  in one file.  If TI DSK

is  the output format, P2HEX will  automatically extract data from both

segments if no segment was specified explicitly.

Similar to the -r option, the argument

-d <start>-<end>

allows  to designate the  address range that  should be written as data

instead of code.

The option

-e <address>

is  valid for the DSK,  Intel, and Motorola formats.  Its purpose is to

set  the entry  address that  will be  inserted into  the hex  file. If

such  a  command  line  parameter  is  missing,  P2HEX  will  search  a

corresponding  entry in  the code  file. If  even this  fails, no entry

address  will  be  written  to  the  hex  file (DSK/Intel) or the field

reserved for the entry address will be set to 0 (Motorola).

Unfortunately,  one finds different  statements about the  last line of

an   Intel-Hex  file  in  literature.   Therefore,  P2HEX  knows  three

different   variants  that   may  be   selected  via  the  command-line

parameter i and an additional number:

0  :00000001FF

1  :00000001

2  :0000000000

By default, variant 0 is used which seems to be the most common one.

If  the target file  name does not  have an extension,  an extension of

HEX is supposed.

By  default, P2HEX  will print  a maximum  of 16  data bytes  per line,

just  as most other tools that output  Hex files. If you want to change

this, you may use the switch

-l <count>   .

The  allowed range of values goes from  2 to 254 data bytes; odd values

will implicitly be rounded down to an even count.

In  most cases,  the temporary  code files  generated by  AS are not of

any further need after P2HEX has been run. The command line option

-k

allows   to   instruct   P2HEX   to   erase  them  automatically  after

conversion.

In  contrast to BIND,  P2HEX will not  produce an empty  target file if

only  one file  name (i.e.  the target  name) has  been given. Instead,

P2HEX  will use the corresponding code  file. Therefore, a minimal call

in the style of

P2HEX <name>

is possible, to generate <name>.hex out of <name>.p.

6.4. P2BIN

----------

P2BIN  works similar to  P2HEX and offers  the same options (except for

the  a  and  i  options  that  do  not  make  sense  for binary files),

however,  the result  is stored  as a  simple binary  file instead of a

hex  file.  Such  a  file  is  for  example suitable for programming an

EPROM.

P2BIN  knows  three  additional  options  to  influence  the  resulting

binary file:

- l  <8  bit  number>:  sets  the  value  that should be used to fill

unused  memory  areas.  By  default,  the  value  $ff is used. This

value  assures that  every half-way  intelligent EPROM  burner will

skip  these areas. This option allows  to set different values, for

example  if you  want to  generate an  image for the EPROM versions

of  MCS-48  microcontrollers  (empty  cells  of  their  EPROM array

contain zeroes, so $00 would be the correct value in this case).

- s:  commands  the  program  to  calculate  a checksum of the binary

file.  This  sum  is  printed  as  a  32-bit  value,  and the two's

complement  of  the  least  significant  bit  will be stored in the

file's  last byte. This way, the  modulus- 256-sum of the file will

become zero.

- m:  is designed for the  case that a CPU  with a 16- or 32-bit data

bus  is used and the  file has to be  split for several EPROMs. The

argument may have the following values:

- ALL: copy everything

- ODD: copy all bytes with an odd address

- EVEN: copy all bytes with an even address

- BYTE0..BYTE3: copy only bytes with an address of 4n+0 .. 4n+3

- WORD0,  WORD1: copy only the lower  resp. upper 16- bit word of

a 32-bit word

To  avoid  confusions:  If  you  use  this option, the resulting binary

file  will become  smaller because  only a  part of  the source will be

copied.  Therefore, the resulting  file will be  smaller by a factor of

2 or 4 compared to ALL. This is just natural...

In  case the code file  does not contain an  entry address, one may set

it  via the -e command line option  just like with P2HEX. Upon request,

P2BIN  prepends  the  resulting  image  with  this address. The command

line option

-S

activates  this function.  It expects  a numeric  specification ranging

from  1 to  4 as  parameter which  specifies the  length of the address

field  in bytes. This number  may optionally be prepended  wit a L or B

letter  to  set  the  endian  order  of  the  address. For example, the

specification  B4  generates  a  4  byte  address  in big endian order,

while  a specification of  L2 or simply  2 creates a  2 byte address in

little endian order.

6.5. AS2MSG

-----------

AS2MSG  is  not  a  tool  in  the  real  sense, it is a filter that was

designed  to  simplify  the  work  with  the  assembler for (fortunate)

users  of Borland Pascal 7.0. The DOS  IDEs feature a 'tools' menu that

can  be  extended  with  own  programs  like  AS.  The filter allows to

directly  display the error  messages paired with  a line specification

delivered  by AS in the  editor window. A new  entry has to be added to

the   tools  menu  to  achieve   this  (Options/Tools/New).  Enter  the

following values:

- Title: ~m~acro assembler

- Program path: AS

- Command line:

-E !1 $EDNAME $CAP MSG(AS2MSG) $NOSWAP $SAVE ALL

- assign a hotkey if wanted (e.g. Shift-F7)

The  -E option  assures that  Turbo Pascal  will not  become puzzled by

STDIN and STDERR.

I  assume that AS and  AS2MSG are located in  a directory listed in the

PATH  variable. After pressing the  appropriate hotkey (or selecting AS

from  the tools  menu), as  will be  called with  the name  of the file

loaded  in the  active editor  window as  parameter. The error messages

generated  during  assembly  are  redirected  to  a special window that

allows  to browse through the errors.  Ctrl-Enter jumps to an erroneous

line.  The  window  additionally  contains  the statistics AS prints at

the end of an assembly. These lines obtain the dummy line number 1.

TURBO.EXE  (Real  Mode)  and  BP.EXE  (Protected  Mode) may be used for

this  way of working with  AS. I recommend however  BP, as this version

does  not have  to 'swap'  half of  the DOS  memory before before AS is

called.

A. Error Messages of AS

=======================

Here  is  a  list  of  all  error  messages  emitted  by AS. Each error

message is described by:

- the  internal error number  (it is displayed  only if AS is started

with the -n option)

- the text of the error message

- error type:

- Warning:  informs the user that a  possible error was found, or

that  some  inefficient  binary  code  could  be generated. The

assembly process is not stopped.

- Error:  an error was detected.  The assembly process continues,

but no binary code is emitted.

- Fatal:   unrecoverable   error.   The   assembly   process   is

terminated.

- reason of the error: the situation originating the error.

- argument: a further explanation of the error message.

5 useless displacement

Type:

warning

Reason:

680x0,  6809 and  COP8 CPUs:  an address  displacement of 0 was

given.   An   address   expression   without   displacement  is

generated,  and  a  convenient  number  of  NOPs are emitted to

avoid phasing errors.

Argument:

none

10 short addressing possible

Type:

warning

Reason:

680x0-,  6502 and  68xx CPUs:  a given  memory location  can be

reached    using   short   addressing.   A   short   addressing

instruction  is emitted,  together with  the required number of

NOPs to avoid phasing errors.

Argument:

none

20 short jump possible

Type:

warning

Reason:

680x0-  and 8086 CPUs  can execute jumps  using a short or long

displacement.  If a shorter jump  was not explicitly requested,

in  the first  pass room  for the  long jump  is reserved. Then

the  code for  the shorter  jump is  emitted, and the remaining

space is filled with NOPs t

Argument:

none

25 relative jump possible

Type:

warning

Reason:

Z80  jumps  may  be  either  relative  or absolute. An absolute

jump  was requested in this  case, however a (shorter) relative

jump would be possible as well.

Argument:

the jump instruction's argument

30 no sharefile created, SHARED ignored

Type:

warning

Reason:

A  SHARED  directive  was  found,  but  on  the command line no

options were specified, to generate a shared file.

Argument:

none

40 FPU possibly cannot read this value (>=1E1000)

Type:

warning

Reason:

The  BCD-floating  point  format  used  by the 680x0-FPU allows

such  a large exponent, but  according to the latest databooks,

this  cannot be  fully interpreted.  The corresponding  word is

assembled,  but  the  associated  function  is  not expected to

produce the correct result.

Argument:

none

50 privileged instruction

Type:

warning

Reason:

A  Supervisor-mode directive  was used,  that was  not preceded

by an explicit SUPMODE ON directive

Argument:

none

60 distance of 0 not allowed for short jump (NOP created instead)

Type:

warning

Reason:

A  short jump with  a jump distance  equal to 0  is not allowed

by  680x0  resp.  COP8  processors,  since  the associated code

word  is used to  identify long jump  instruction. Instead of a

jump instruction, AS emits a NOP

Argument:

none

70 symbol out of wrong segment

Type:

warning

Reason:

The  symbol  used  as  an  operand  comes from an address space

that cannot be addressed together with the given instruction

Argument:

none

75 segment not accessible

Type:

warning

Reason:

The  symbol  used  as  an  operand  belongs to an address space

that  cannot be accessed  with any of  the segment registers of

the 8086

Argument:

The name of the inaccessible segment

80 change of symbol values forces additional pass

Type:

warning

Reason:

A  symbol changed  value, with  respect to  previous pass. This

warning is emitted only if the -r option is used.

Argument:

name of the symbol that changed value.

90 overlapping memory usage

Type:

warning

Reason:

The  analysis of the usage list  shows that part of the program

memory   was  used  more  than  once.  The  reason  can  be  an

excessive usage of ORG directives.

Argument:

none

95 overlapping register usage

Type:

warning

Reason:

The  instruction  uses  whole  registers  or parts thereof in a

non-allowed way.

Argument:

The offending argument

100 none of the CASE conditions was true

Type:

warning

Reason:

A   SWITCH...CASE   directive   without   ELSECASE  clause  was

executed,  and  none  of  the  CASE  conditions was found to be

true.

Argument:

none

110 page might not be addressable

Type:

warning

Reason:

The  symbol  used  as  an  operand  was not found in the memory

page defined by an ASSUME directive (ST6, 78(C)10).

Argument:

none

120 register number must be even

Type:

warning

Reason:

The  CPU  allows  to  concatenate  only  register  pairs, whose

start address is even (RR0, RR2, ..., only for Z8).

Argument:

none

130 obsolete instruction, usage discouraged

Type:

warning

Reason:

The  instruction used, although supported,  was superseded by a

new  instruction.  Future  versions  of  the  CPU could no more

implement the old instruction.

Argument:

none

140 unpredictable execution of this instruction

Type:

warning

Reason:

The  addressing  mode  used  for  this  instruction is allowed,

however  a register  is used  in such  a way  that its contents

cannot be predicted after the execution of the instruction.

Argument:

none

150 localization operator senseless out of a section

Type:

warning

Reason:

An  aheaded @ must  be used, so  that it is explicitly referred

to  the local  symbols used  in the  section. When the operator

is  used out of a section,  there are no local symbols, because

this operator is useless in this context.

Argument:

none

160 senseless instruction

Type:

warning

Reason:

The  instruction used has no meaning,  or it can be substituted

by an other instruction, shorter and more rapidly executed.

Argument:

none

170 unknown symbol value forces additional pass

Type:

warning

Reason:

AS  expects a  forward definition  of a  symbol, i.e.  a symbol

was  used  before  it  was  defined.  A  further  pass  must be

executed.  This warning  is emitted  only if  the -r option was

used.

Argument:

none

180 address is not properly aligned

Type:

warning

Reason:

An  address  was  used  that  is  not  an exact multiple of the

operand  size.  Although  the  CPU  databook  forbids this, the

address  could be stored in the  instruction word, so AS simply

emits a warning.

Argument:

none.

190 I/O-address must not be used here

Type:

warning

Reason:

The  addressing mode or  the address used  are correct, but the

address  refers to the  peripheral registers, and  it cannot be

used in this circumstance.

Argument:

none.

200 possible pipelining effects

Type:

warning

Reason:

A  register  is  used  in  a  series of instructions, so that a

sequence   of  instructions  probably  does  not  generate  the

desired  result. This usually  happens when a  register is used

before its new content was effectively loaded in it.

Argument:

the register probably causing the problem.

210 multiple use of address register in one instruction

Type:

warning

Reason:

A  register used  for the  addressing is  used once more in the

same  instruction, in a  way that results  in a modification of

the  register  value.  The  resulting  address  does not have a

well defined value.

Argument:

the register used more than once.

220 memory location is not bit addressable

Type:

warning

Reason:

Via  a SFRB  statement, it  was tried  to declare a memory cell

as  bit addressable  which is  not bit  addressable due  to the

8051's architectural limits.

Argument:

none

230 stack is not empty

Type:

warning

Reason:

At  the end of  a pass, a  stack defined by  the program is not

empty.

Argument:

the name of the stack and its remaining depth

240 NUL character in string, result is undefined

Type:

warning

Reason:

A  string constant contains a  NUL character. Though this works

with  the Pascal version, it is a  problem for the C version of

AS  since  C  itself  terminates  strings with a NUL character.

i.e.  the  string  would  have  its  end  for  C  just  at this

point...

Argument:

none

250 instruction crosses page boundary

Type:

warning

Reason:

The  parts of a  machine statement partiallly  lie on different

pages.   As  the   CPU's  instruction   counter  does  not  get

incremented  across page boundaries,  the processor would fetch

at  runtime  the  first  byte  of  the  old page instead of the

instruction's following byte; the program

Argument:

none

255 range underflow

Type:

warning

Reason:

A  numeric value  was below  the allowed  range. AS brought the

value  back into  the allowed  range by  truncating upper bits,

but  it is not  guaranteed that meaningful  and correct code is

generated by this.

Argument:

none

260 range overflow

Type:

warning

Reason:

A  numeric value  was above  the allowed  range. AS brought the

value  back into  the allowed  range by  truncating upper bits,

but  it is not  guaranteed that meaningful  and correct code is

generated by this.

Argument:

none

270 negative argument for DUP

Type:

warning

Reason:

The  repetition argument  of a  DUP directive  was smaller than

0. Analogous to a count of exactly 0, no data is stored.

Argument:

none

280 single   X  operand   interpreted  as   indexed  and  not  implicit

addressing

Type:

warning

Reason:

A  single X operand may be  interpreted either as register X or

x-indexed  addressing  with  zero  displacement, since Motorola

does  not specify  this variant.  AS chooses  the latter, which

may not be the desired one.

Argument:

none

300 bit number will be truncated

Type:

warning

Reason:

This   instruction  only   operates  on   byte  resp.  longword

operands.  bit  numbers  beyond  7  resp.  31  will  be treated

modulo-8 resp. modulo-32 by the CPU.

Argument:

none

310 invalid register pointer value

Type:

warning

Reason:

Valid  values  for  the  RP  register  range  from 0x00 to 0x70

resp. 0xf0, because all other areas are unused on the Z8.

Argument:

none

320 macro argument redefined

Type:

warning

Reason:

A  macro parameter was  assigned two or  more different values.

This  may  happen  by  usage  of  keyword  arguments.  The last

argument is actually used.

Argument:

name of the macro parameter

330 deprecated instruction

Type:

warning

Reason:

This  instruction  is  deprecated  and  should  not be used any

more in new programs.

Argument:

the instruction that should be used instead.

340 source operand is longer or same size as destination operand

Type:

warning

Reason:

The  source  operand's  size  is  larger  than  the destination

operand's  size,  expressed  in  bits.  Sign  or zero extension

does  not  make  sense  with  these  arguments.  See  the CPU's

reference manual for its behaviour in this situation.

Argument:

none

350 TRAP number represents valid instruction

Type:

warning

Reason:

A  TRAP  with  this  number  uses  the  same  machine code as a

machine instruction supported by the CPU.

Argument:

none

360 Padding added

Type:

warning

Reason:

The  amount of bytes placed  in memory is odd;  one half of the

last 16 bit word remains unused.

Argument:

none

370 register number wraparound

Type:

warning

Reason:

The  start register number plus  the count of registers results

in a last register beyond the end of the register bank.

Argument:

the argument holding the register count

380 using indexed instead of indirect addressing

Type:

warning

Reason:

Indirect  addressing  is  not  allowed  at this place. Instead,

indexed  addressing with a  dummy displacement of  zero will be

used.

Argument:

the argument holding the indirect addressing expression

390 not allowed in normal mode

Type:

warning

Reason:

This  machine instruction  is only  allowed in  panel mode, not

during ''normal operation''.

Argument:

the machine instruction in question

400 not allowed in panel mode

Type:

warning

Reason:

This  machine  instruction  is  only  allowed  during  ''normal

operation'', not in panel mode.

Argument:

the machine instruction in question

410 argument out of range

Type:

warning

Reason:

The  argument or the sum of  two arguments is outside the range

allowed   for   this   instruction,   though   the  instruction

principally provides room for larger values.

Argument:

the argument in question

420 attempt to skip multiword instruction

Type:

warning

Reason:

The  previous  instruction  was  a  skip instruction, which can

only  skip  a  single  (half)  word. The current instruction is

longer  than one word, so a skip  would jump into the middle of

it.

Argument:

the multi-word instruction in question

430 implicit sign extension

Type:

warning

Reason:

As  part  of  executing  this  instruction,  the processor will

perform  a sign extension  to the full  register width. For the

given  argument, this means  the register's upper  bits will be

filled  with ones  and not  zeros. Depending  on the usage that

follows, this may be irrelevant or not.

Argument:

the value in question

440 numeric  value  -128  means  usage  of  E  register's  content (use

literal 'E' to avoid this warning)

Type:

warning

Reason:

On  the SC/MP, a  displacement of -128  means in this case that

the  actual displacement  is not  -128, but  instead taken from

the  E register. The  assembler cannot decide  for sure whether

this  was intended or  the accidental result  of a computation,

and therefore warns. Use the literal val

Argument:

the displacement argument in question

450 I/O address must be accessed via INS/OUTS

Type:

warning

Reason:

I/O  addresses  in  the  range  of  0  to  3 are located in the

processor  module  and  can  only  be  accessed via the INS and

OUTS instructios, not via IN or OUT.

Argument:

the address argument in question

460 CASE limit does not match number of branch addresses

Type:

warning

Reason:

The  CASE  instruction  expects  a  branch  table  with as many

entries,  as the limit  argument says, plus  one. The limit and

the number of branch addresses do not fit together.

Argument:

the limit argument

470 instruction assembled as NOP

Type:

warning

Reason:

The  machinne  code  assigned  to  this instruction is used for

different  purposes.  Since  this  instruction  anyway does not

perform any operation, it is assembled as a NOP.

Argument:

none

480 argument treated as vector

Type:

warning

Reason:

There  is no way  to umambiguously decide  whether the argument

is  an  address  or  a  vector.  Since  the argument is a plain

numeric  value not  assigned to  any segment,  it is assumed to

be a vector.

Argument:

the argument in question

1000 symbol double defined

Type:

error

Reason:

A  new value is assigned  to a symbol, using  a label or a EQU,

PORT,  SFR, LABEL,  SFRB or  BIT instruction:  however this can

be done only using SET/EVAL.

Argument:

the  name of  the offending  symbol, and  the line number where

it  was defined  for the  first time,  according to  the symbol

table.

1010 symbol undefined

Type:

error

Reason:

A  symbol is still not defined  in the symbol table, also after

a second pass.

Argument:

the name of the undefined symbol.

1020 invalid symbol name

Type:

error

Reason:

A  symbol does not  fulfill the requirements  that symbols must

have  to be considered  valid by AS.  Please pay attention that

more  stringent  syntax  rules  exist  for  macros and function

parameters.

Argument:

the wrong symbol name

1030 reserved symbol name

Type:

error

Reason:

A  symbol  is  valid  by  itself,  but  this  specific  name is

reserved  for other purposes.  It therefore cannot  be used for

user-defined symbols.

Argument:

the symbol name in question

1090 invalid format

Type:

error

Reason:

The   instruction  format   used  does   not  exist   for  this

instruction.

Argument:

the known formats for this command

1100 useless attribute

Type:

error

Reason:

The  instruction (processor  or pseudo)  cannot be  used with a

point-suffixed attribute.

Argument:

none

1105 attribute may only be one character long

Type:

error

Reason:

The  attribute following a point  after an instruction must not

be longer or shorter than one character.

Argument:

none

1107 undefined attribute

Type:

error

Reason:

This instruction uses an invalid attribute.

Argument:

none

1110 wrong number of operands

Type:

error

Reason:

The  number of arguments issued  for the instruction (processor

or  pseudo)  does  not  conform  with  the  accepted  number of

operands.

Argument:

the expected number of arguments resp. operands

1112 failed splitting argument into parts

Type:

error

Reason:

For  some targets (e.g. DSP56000),  the comma-separated have to

be split into individual operands, which failed.

Argument:

none

1115 wrong number of operations

Type:

error

Reason:

The number of options given with this command is not correct.

Argument:

none

1120 addressing mode must be immediate

Type:

error

Reason:

The  instruction  can  be  used  only  with  immediate operands

(preceded by #).

Argument:

none

1130 invalid operand size

Type:

error

Reason:

Although  the operand  is of  the right  type, it does not have

the correct length (in bits).

Argument:

none

1131 conflicting operand sizes

Type:

error

Reason:

The operands used have different length (in bits)

Argument:

none

1132 undefined operand size

Type:

error

Reason:

It  is not possible  to estimate, from  the opcode and from the

operands,  the  size  of  the  operand  (a  trouble  with  8086

assembly). You must define it with a BYTE or WORD PTR prefix.

Argument:

none

1133 expected integer or string, but got floating point number

Type:

error

Reason:

A  floating point  number cannot  be used  as argument  at this

place.

Argument:

the argument in question

1134 expected integer, but got floating point number

Type:

error

Reason:

A  floating point  number cannot  be used  as argument  at this

place.

Argument:

the argument in question

1136 expected floating point number, but got string

Type:

error

Reason:

A string cannot be used as argument at this place.

Argument:

the argument in question

1137 operand type mismatch

Type:

Error

Reason:

The  two arguments  of an  operator are  not of  same data type

(integer/float/string).

Argument:

keines

1138 expected string, but got integer

Type:

error

Reason:

An integer cannot be used as argument at this place.

Argument:

the argument in question

1139 expected string, but got floating point number

Type:

error

Reason:

An  floating point  number cannot  be used  as argument at this

place.

Argument:

the argument in question

1140 too many arguments

Type:

error

Reason:

No more than 20 arguments can be given to any instruction

Argument:

none

1141 expected integer, but got string

Type:

error

Reason:

A string cannot be used as argument at this place.

Argument:

the argument in question

1142 expected integer or floating point number, but got string

Type:

error

Reason:

A string cannot be used as argument at this place.

Argument:

the argument in question

1143 expected string

Type:

error

Reason:

Only  a  string  (enclosed  in  single  quotes)  may be used as

argument at this place.

Argument:

the argument in question

1144 expected integer

Type:

error

Reason:

Only an integer number may be used as argument at this place.

Argument:

the argument in question

1145 expected   integer,  floating  point  number  or  string  but  got

register

Type:

error

Reason:

A register symbol may not be used as argument at this place.

Argument:

the argument in question

1146 expected integer or string

Type:

error

Reason:

A  floating point number or register  symbol may not be used as

argument at this place.

Argument:

the argument in question

1147 expected register

Type:

error

Reason:

Only an register may be used as argument at this place.

Argument:

the argument in question

1148 register symbol for different target

Type:

error

Reason:

The  used register  symbol was  defined for  a target different

from the current one and is not compatible.

Argument:

the argument in question

1149 expected floating point argument but got integer

Type:

error

Reason:

Only  a floating point argument may  be used at this place, but

an integer argument was given.

Argument:

the argument in question

1151 expected integer or floating point number but got register

Type:

error

Reason:

Only  an integer or floating point  number argument may be used

at this place, but a register was given.

Argument:

the argument in question

1152 expected integer or string but got register

Type:

error

Reason:

Only  an integer or string argument  may be used at this place,

but a register was given.

Argument:

the argument in question

1153 expected integer but got register

Type:

error

Reason:

Only  an  integer  argument  may  be  used at this place, but a

register was given.

Argument:

the argument in question

1154 string too long

Type:

error

Reason:

The  string is too  long to be  represented with leading length

byte.

Argument:

the argument in question

1200 unknown instruction

Type:

error

Reason:

An  instruction  was  used  that  is neither an AS instruction,

nor  a  known  macine  instruction  for  the  current processor

type.

Argument:

none

1300 number of opening/closing brackets does not match

Type:

error

Reason:

The   expression  parser   found  an   expression  enclosed  by

parentheses,   where   the   number   of  opening  and  closing

parentheses does not match.

Argument:

the wrong expression

1310 division by 0

Type:

error

Reason:

An  expression  on  the  right  side  of  a division or modulus

operation was found to be equal to 0.

Argument:

none

1315 range underflow

Type:

error

Reason:

An integer word underflowed the allowed range.

Argument:

the  value of the word and  the allowed minimum (in most cases,

maybe I will complete this one day...)

1320 range overflow

Type:

error

Reason:

An integer word overflowed the allowed range.

Argument:

the  value  of  the  world,  and  the  allowed maximum (in most

cases, maybe I will complete this one day...)

1322 not a power of two

Type:

error

Reason:

only powers of two (1,2,4,8,...) are allowed at this place.

Argument:

The value in question

1323 invalid decimal digit

Type:

error

Reason:

A   string  as  argument   to  PACKED  may   only  contain  the

characters  from 0  to 9,  and optionally  a plus or minus sign

at the beginning.

Argument:

The argument in question

1324 decimal string too long

Type:

error

Reason:

A packed decimal number must not be longer than 31 digits.

Argument:

The argument in question

1325 address is not properly aligned

Type:

error

Reason:

The  given address does not correspond  with the size needed by

the  data transfer, i.e. it is  not an integral multiple of the

operand size. Not all processor types can use unaligned data.

Argument:

none

1330 distance too big

Type:

error

Reason:

The displacement used for an address is too large.

Argument:

none

1331 target not on same page

Type:

error

Reason:

Instruction  and operand  address must  be located  in the same

memory page.

Argument:

the address argument in question

1340 short addressing not allowed

Type:

error

Reason:

The  address of  the operand  is outside  of the  address space

that can be accessed using short-addressing mode.

Argument:

none

1350 addressing mode not allowed here

Type:

error

Reason:

the  addressing  mode  used,  although usually possible, cannot

be used here.

Argument:

none

1351 address must be even

Type:

error

Reason:

At  this point, only even addresses  are allowed, since the low

order bits are used for other purposes or are reserved.

Argument:

the argument in question

1352 address must be aligned

Type:

error

Reason:

At  this  point,  only  aligned  (i.e.  a mulitple of 2,4,8...)

addresses  are allowed, since  the low order  bits are used for

other purposes or are reserved.

Argument:

the argument in question

1355 addressing mode not allowed in parallel operation

Type:

error

Reason:

The  addressing  mode(s)  used  are  allowed in sequential, but

not in parallel instructions

Argument:

none

1360 undefined condition

Type:

error

Reason:

The  branch  condition  used  for  a  conditional jump does not

exist.

Argument:

none

1365 incompatible conditions

Type:

error

Reason:

The  used  combination  of  conditions  is  not  possible  in a

single instruction.

Argument:

the condition where the incompatibility was detected.

1366 unknown flag

Type:

error

Reason:

The given flag does not exist.

Argument:

the argument using the flag in question

1367 duplicate flag

Type:

error

Reason:

The given flag has already been used in the list of flags.

Argument:

the argument duplicating the flag

1368 unknown interrupt

Type:

error

Reason:

The given interrupt does not exist.

Argument:

the argument using the interrupt in question

1369 duplicate interrupt

Type:

error

Reason:

The  given  interrupt  has  already  been  used  in the list of

interrupt.

Argument:

the argument duplicating the interrupt

1370 jump distance too big

Type:

error

Reason:

the  jump instruction and destination  are too apart to execute

the jump with a single step

Argument:

none

1371 jump distance is zero

Type:

error

Reason:

the  jump  destination  is  right  behind the jump instruction,

and a jump distance of zero cannot be encoded.

Argument:

the target address in source code

1375 jump distance is odd

Type:

error

Reason:

Since  instruction must only be  located at even addresses, the

jump  distance between  two instructions  must always  be even,

and  the  LSB  of  the  jump  distance  is used otherwise. This

issue  was  not  verified  here.  The  reason  is  usually  the

presence of an odd number of data in bytes or a wro

Argument:

none

1376 skip target mismatch

Type:

error

Reason:

The  gien branch target is not  the address the processor would

jump to if the skip instruction were executed.

Argument:

the given (intended) jump target

1380 invalid argument for shifting

Type:

error

Reason:

only  a constant  or a  data register  can be used for defining

the shift size. (only for 680x0)

Argument:

none

1390 operand must be in range 1..8

Type:

error

Reason:

constants  for shift size  or ADDQ argument  can be only within

the 1..8 range (only for 680x0)

Argument:

none

1400 shift amplitude too big

Type:

error

Reason:

(no more used)

Argument:

none

1410 invalid register list

Type:

error

Reason:

The  register  list  argument  of  MOVEM  or FMOVEM has a wrong

format (only for 680x0)

Argument:

none

1420 invalid addressing mode for CMP

Type:

error

Reason:

The  operand combination used  with the CMP  instruction is not

allowed (only for 680x0)

Argument:

none

1430 invalid CPU type

Type:

error

Reason:

The  processor  type  used  as  argument  for  CPU  command  is

unknown to AS.

Argument:

the unknown processor type

1431 invalid FPU type

Type:

error

Reason:

The  co-processor  type  used  as  argument  for FPU command is

unknown to AS.

Argument:

the unknown co-processor type

1432 invalid PMMU type

Type:

error

Reason:

The  MMU type used  as argument for  PMMU command is unknown to

AS.

Argument:

the unknown MMU type

1440 invalid control register

Type:

error

Reason:

The  control register  used by  a MOVEC  is not (yet) available

for the processor defined by the CPU command.

Argument:

none

1445 invalid register

Type:

error

Reason:

The  register  used,  although  valid,  cannot  be used in this

context.

Argument:

none

1446 register(s) listed more than once

Type:

error

Reason:

A  register appears more than once  in the list of registers to

be saved or restored.

Argument:

none

1447 register bank mismatch

Type:

error

Reason:

An address expression uses registers from different banks.

Argument:

the register in question

1448 undefined register length

Type:

error

Reason:

Registers  of different  size may  be used  at this  place, and

the register length cannot be deduced from the address alone.

Argument:

the argument in question

1449 invalid operation on register

Type:

error

Reason:

This  operation  may  not  be  applied  to  this register, e.g.

because the register is read-only or write-only.

Argument:

the register in question

1450 RESTORE without SAVE

Type:

error

Reason:

A  RESTORE command  was found,  that cannot  be coupled  with a

corresponding SAVE.

Argument:

none

1460 missing RESTORE

Type:

error

Reason:

After the assembling pass, a SAVE command was missing.

Argument:

none.

1465 unknown macro control instruction

Type:

error

Reason:

A macro option parameter is unknown to AS.

Argument:

the dubious option.

1470 missing ENDIF/ENDCASE

Type:

error

Reason:

after  the  assembling,  some  of  the  IF- or CASE- constructs

were found without the closing command

Argument:

none

1480 invalid IF-structure

Type:

error

Reason:

The command structure in a IF- or SWITCH- sequence is wrong.

Argument:

none

1483 section name double defined

Type:

error

Reason:

In  this  program  module  a  section  with the same name still

exists.

Argument:

the multiple-defined name

1484 unknown section

Type:

error

Reason:

In the current scope, there are no sections with this name

Argument:

the unknown name

1485 missing ENDSECTION

Type:

error

Reason:

Not all the sections were properly closed.

Argument:

none

1486 wrong ENDSECTION

Type:

error

Reason:

The  given ENDSECTION does not refer  to the most deeply nested

one.

Argument:

none

1487 ENDSECTION without SECTION

Type:

error

Reason:

An  ENDSECTION command  was found,  but the  associated section

was not defined before.

Argument:

none

1488 unresolved forward declaration

Type:

error

Reason:

A  symbol  declared  with  a  FORWARD or PUBLIC statement could

not be resolved.

Argument:

the  name of  the unresolved  symbol, plus  the position of the

forward declaration in the source.

1489 conflicting FORWARD <-> PUBLIC-declaration

Type:

error

Reason:

A symbol was defined both as public and private.

Argument:

the name of the symbol.

1490 wrong numbers of function arguments

Type:

error

Reason:

The  number of arguments  used for referencing  a function does

not  match  the  number  of  arguments  defined in the function

definition.

Argument:

none

1495 unresolved literals (missing LTORG)

Type:

error

Reason:

At  the  end  of  the  program,  or  just  before  switching to

another processor type, unresolved literals still remain.

Argument:

none

1500 instruction not allowed on

Type:

error

Reason:

Although  the instruction  is correct,  it cannot  be used with

the selected member of the CPU family.

Argument:

The processor variants that would support this instruction.

1501 FPU instructions are not enabled

Type:

error

Reason:

FPU  instruction  set  extensions  must  be enabled to use this

instruction.

Argument:

none

1502 PMMU instructions are not enabled

Type:

error

Reason:

PMMU  instruction set  extensions must  be enabled  to use this

instruction.

Argument:

none

1503 full PMMU instruction set is not enabed

Type:

error

Reason:

This  instrction is  only contained  in the 68851's instruction

set,  not  in  the  reduced  instruction  set of the integrated

PMMU.

Argument:

none

1504 Z80 syntax was not allowed

Type:

error

Reason:

This  instruction is only  allowed if Z80  syntax for 8080/8085

instructions has been enabled.

Argument:

none

1505 addressing mode not allowed on

Type:

error

Reason:

Although  the  addressing  mode  used  is correct, it cannot be

used with the selected member of the CPU family.

Argument:

The  processor  variants  that  would  support  this addressing

mode.

1506 not allowed in exclusive Z80 syntax mode

Type:

error

Reason:

This  instrction is no  longer allowed if  exclusive Z80 syntax

mode for 8080/8085 instructions has been set.

Argument:

none

1507 FPU instruction not supported on ...

Type:

error

Reason:

Although  this  FPU  instruction  exists,  it cannot be used on

the selected type of FPU.

Argument:

The instruction in question

1508 Custom instructions are not enabled

Type:

error

Reason:

Custom  instruction set extensions must  be enabled to use this

instruction.

Argument:

The instruction in question

1509 instruction extension not enabled

Type:

error

Reason:

This  instruction is part  of an extension  whose usage has not

been enabled.

Argument:

The extension's name

1510 invalid bit position

Type:

error

Reason:

Either  the number  of bits  specified is  not allowed,  or the

command is not completely specified.

Argument:

none

1520 only ON/OFF allowed

Type:

error

Reason:

This pseudo command accepts as argument either ON or OFF

Argument:

none

1530 stack is empty or undefined

Type:

error

Reason:

It  was tried  to access  a stack  via a  POPV instruction that

was either never defined or already emptied.

Argument:

the name of the stack in question

1540 not exactly one bit set

Type:

error

Reason:

Not  exactly one  bit was  set in  a mask  passed to the BITPOS

function.

Argument:

none

1550 ENDSTRUCT without STRUCT

Type:

error

Reason:

An  ENDSTRUCT instruction  was found  though there is currently

no structure definition in progress.

Argument:

none

1551 open structure definition

Type:

error

Reason:

After  end of assembly,  not all STRUCT  instructions have been

closed with appropriate ENDSTRUCTs.

Argument:

the innermost, unfinished structure definition

1552 wrong ENDSTRUCT

Type:

error

Reason:

the  name  parameter  of  an  ENDSTRUCT  instruction  does  not

correspond to the innermost open structure definition.

Argument:

none

1553 phase definition not allowed in structure definition

Type:

error

Reason:

What  should I  say about  that? PHASE  inside a  record simply

does not make sense and only leads to confusion...

Argument:

none

1554 invalid STRUCT directive

Type:

error

Reason:

Only  EXTNAMES,  NOEXTNAMES,  DOTS,  and  NODOTS are allowed as

directives of a STRUCT statement.

Argument:

the unknown directive

1555 structure re-defined

Type:

error

Reason:

A structure of this name has already been defined.

Argument:

the name of the structure

1556 unresolvable structure element reference

Type:

error

Reason:

An  element  in  a  structure  references  to  another element,

however  this referenced element was  not defined or itself has

an unresolvable reference.

Argument:

the name of the element itself and the referenced one

1557 duplicate structure element

Type:

error

Reason:

The structure already contains an element of this name.

Argument:

name of the element

1560 instruction is not repeatable

Type:

error

Reason:

This   machine  instruction  cannot  be   repeated  via  a  RPT

construct.

Argument:

none

1600 unexpected end of file

Type:

error

Reason:

It  was tried to  read past the  end of a  file with a BINCLUDE

statement.

Argument:

none

1700 ROM-offset must be in range 0..63

Type:

error

Reason:

The ROM table of the 680x0 coprocessor has only 64 entries.

Argument:

none

1710 invalid function code

Type:

error

Reason:

The  only function code arguments allowed  are SFC, DFC, a data

register,  or a  constant in  the interval  of 0..15  (only for

680x0 MMU).

Argument:

none

1720 invalid function code mask

Type:

error

Reason:

Only  a number  in the  interval 0..15  can be used as function

code mask (only for 680x0 MMU)

Argument:

none

1730 invalid MMU register

Type:

error

Reason:

The  MMU  does  not  have  a  register with this name (only for

680x0 MMU).

Argument:

none

1740 level must be in range 0..7

Type:

error

Reason:

The  level  for  PTESTW  and  PTESTR  must be a constant in the

range of 0...7 (only for 680x0 MMU).

Argument:

none

1750 invalid bit mask

Type:

error

Reason:

The  bit mask used for  a bit field command  has a wrong format

(only for 680x0).

Argument:

none

1760 invalid register pair

Type:

error

Reason:

The  register here defined  cannot be used  in this context, or

there is a syntactic error (only for 680x0).

Argument:

none

1800 open macro definition

Type:

error

Reason:

An  incomplete  macro  definition  was  found. Probably an ENDM

statement is missing.

Argument:

none

1801 IRP without ENDM

Type:

error

Reason:

An  incomplete IRP block was  found. Probably an ENDM statement

is missing.

Argument:

none

1802 IRPC without ENDM

Type:

error

Reason:

An   incomplete  IRPC   block  was   found.  Probably  an  ENDM

statement is missing.

Argument:

none

1803 REPT without ENDM

Type:

error

Reason:

An   incomplete  REPT   block  was   found.  Probably  an  ENDM

statement is missing.

Argument:

none

1804 WHILE without ENDM

Type:

error

Reason:

An   incomplete  WHILE  block  was   found.  Probably  an  ENDM

statement is missing.

Argument:

none

1805 EXITM not called from within macro

Type:

error

Reason:

EXITM   is  designed  to  terminate  a  macro  expansion.  This

instruction  only makes sense within  macros and an attempt was

made to call it in the absence of macros.

Argument:

none

1810 more than 10 macro parameters

Type:

error

Reason:

A macro cannot have more than 10 parameters

Argument:

none

1811 keyword argument not defined in macro

Type:

error

Reason:

a  keyword argument  referred to  a parameter  the called macro

does not provide.

Argument:

used keyword resp. macro parameter

1812 positional argument no longer allowed after keyword argument

Type:

Fehler

Reason:

position  and  keyword  arguments  may  be  mixed  in one macro

call,  however  only  keyword  arguments  are allowed after the

first keyword argument.

Argument:

none

1815 macro double defined

Type:

error

Reason:

A macro was defined more than once in a program section.

Argument:

the multiply defined macro name.

1820 expression must be evaluatable in first pass

Type:

error

Reason:

The  command  used  has  an  influence  on  the  length  of the

emitted  code, so  that forward  references cannot  be resolved

here.

Argument:

none

1830 too many nested IFs

Type:

error

Reason:

(no more implemented)

Argument:

none

1840 ELSEIF/ENDIF without IF

Type:

error

Reason:

A  ELSEIF- or  ENDIF- command  was found,  that is not preceded

by an IF- command.

Argument:

none

1850 nested / recursive macro call

Type:

error

Reason:

(no more implemented)

Argument:

none

1860 unknown function

Type:

error

Reason:

The function invoked was not defined before.

Argument:

The name of the unknown function

1870 function argument out of definition range

Type:

error

Reason:

The  argument  does  not  belong  to the allowed argument range

associated to the referenced function.

Argument:

none

1880 floating point overflow

Type:

error

Reason:

Although  the  argument  is  within  the  range  allowed to the

function arguments, the result is not valid

Argument:

none

1890 invalid value pair

Type:

error

Reason:

The  base-exponent  pair  used  in  the  expression  cannot  be

computed

Argument:

none

1900 instruction must not start on this address

Type:

error

Reason:

No  jumps  can  be  performed  by  the  selected  CPU from this

address.

Argument:

none

1905 invalid jump target

Type:

error

Reason:

No  jumps  can  be  performed  by  the  selected  CPU  to  this

address.

Argument:

none

1910 jump target not on same page

Type:

error

Reason:

Jump command and destination must be in the same memory page.

Argument:

none

1911 jump target not in same section

Type:

error

Reason:

Jump  command and destination must be  in the same (64K) memory

section.

Argument:

none

1920 code overflow

Type:

error

Reason:

An  attempt was  made to  generate more  than 1024 code or data

bytes in a single memory page.

Argument:

none

1925 address overflow

Type:

error

Reason:

The  address  space  for  the  processor type actually used was

filled beyond the maximum allowed limit.

Argument:

none

1930 constants and placeholders cannot be mixed

Type:

error

Reason:

Instructions   that  reserve  memory,   and  instructions  that

define   constants  cannot   be  mixed   in  a   single  pseudo

instruction.

Argument:

none

1940 code must not be generated in structure definition

Type:

error

Reason:

a  STRUCT  construct  is  only  designed  to  describe  a  data

structure  and not  to create  one; therefore,  no instructions

are allowed that generate code.

Argument:

none

1950 parallel construct not possible here

Type:

error

Reason:

Either  these instructions  cannot be  executed in parallel, or

they   are  not  close  enough   each  other,  to  do  parallel

execution.

Argument:

none

1960 invalid segment

Type:

error

Reason:

The referenced segment cannot be used here.

Argument:

The name of the segment used.

1961 unknown segment

Type:

error

Reason:

The  segment referenced with  a SEGMENT command  does not exist

for the CPU used.

Argument:

The name of the segment used

1962 unknown segment register

Type:

error

Reason:

The segment referenced here does not exist (8086 only)

Argument:

none

1970 invalid string

Type:

error

Reason:

The string has an invalid format.

Argument:

none

1980 invalid register name

Type:

error

Reason:

The  referenced register does  not exist, or  it cannot be used

here.

Argument:

none

1985 invalid argument

Type:

error

Reason:

The command used cannot be performed with the REP-prefix.

Argument:

none

1990 indirect mode not allowed

Type:

error

Reason:

Indirect addressing cannot be used in this way

Argument:

none

1995 not allowed in current segment

Type:

error

Reason:

(no more implemented)

Argument:

none

1996 not allowed in maximum mode

Type:

error

Reason:

This register can be used only in minimum mode

Argument:

none

1997 not allowed in minimum mode

Type:

error

Reason:

This register can be used only in maximum mode

Argument:

none

2000 execution packet crosses address boundary

Type:

error

Reason:

An execution packet must not cross a 32-byte address boundary

Argument:

none

2001 multiple use of same execution unit

Type:

error

Reason:

One  of the  CPU's execution  units was  used more than once in

an execution packet

Argument:

the name of the execution unit

2002 multiple long read operations

Type:

error

Reason:

An   execution  packet   contains  more   than  one  long  read

operation, which is not allowed

Argument:

one of the functional units executing a long read

2003 multiple long write operations

Type:

error

Reason:

An   execution  packet  contains  more   than  one  long  write

operation, which is not allowed

Argument:

one of the functional units executing a long write

2004 long read with write operation

Type:

error

Reason:

An  execution  packet  contains  both  a  long read and a write

operation, which is not allowed.

Argument:

one   of   the   execution   units  executing  the  conflicting

operations

2005 too many reads of one register

Type:

error

Reason:

The  same register was  referenced more than  four times in the

same execution packet.

Argument:

the name of the register referenced too often

2006 overlapping destinations

Type:

error

Reason:

The  same register was  written more than  one time in the same

instruction packet, which is not allowed.

Argument:

the name of the register in question

2008 too many absolute branches in one execution packet

Type:

error

Reason:

An  execution  packet  contains  more  than  one direct branch,

which is not allowed.

Argument:

none

2009 instruction cannot be executed on this unit

Type:

error

Reason:

This instruction cannot be executed on this functional unit.

Argument:

none

2010 invalid escape sequence

Type:

error

Reason:

The  special character  defined using  a backslash  sequence is

not defined

Argument:

none

2020 invalid combination of prefixes

Type:

error

Reason:

The  prefix  combination  here  defined  is  not allowed, or it

cannot be translated into binary code

Argument:

none

2030 constants cannot be redefined as variables

Type:

error

Reason:

A  symbol  that  has  once  been  declared as constant with EQU

must not be modified afterwards with SET.

Argument:

the name of the symbol in question

2035 variables cannot be redefined as constants

Type:

error

Reason:

A  symbol  that  has  once  been  declared as variable with SET

must not be redeclared afterwards as constant (e.g. with EQU.

Argument:

the name of the symbol in question

2040 structure name missing

Type:

error

Reason:

A  structure's  definition  lacks  the  identifier name for the

new structure

Argument:

none

2050 empty argument

Type:

error

Reason:

Empty  strings must not  be used in  the argument list for this

statement

Argument:

none

2060 unimplemented instruction

Type:

error

Reason:

The  used  machinen  instruction  is  principally  known to the

assembler,  however,  it  is  currently  not implemented, du to

lack of documentation from the processor manufacturer.

Argument:

the instruction that was used

2070 unnamed structure is not part of another structure

Type:

error

Reason:

An  unnamed structure or  union always must  be part of another

structure or union.

Argument:

none

2080 STRUCT ended by ENDUNION

Type:

error

Reason:

ENDUNION  may  only  be  used  to  finalize the definition of a

union and not of a structure.

Argument:

name of the structure (if available)

2090 Memory address mot on active memory page

Type:

error

Reason:

The  target address  is not  within the  page that is currently

addressable via the page register.

Argument:

none

2100 unknown macro expansion argument

Type:

error

Reason:

An argument to MACEXP could not be interpreted.

Argument:

the unknown argument

2105 too many macro expansion arguments

Type:

error

Reason:

The  number  macro  expansion  arguments  exceeds  the  allowed

limit.

Argument:

the argument that busted the limit

2110 contradicting macro expansion specifications

Type:

error

Reason:

A   specification  about   macro  expansion   and  its  precise

opposite may not be used in the same MACEXP instruction.

Argument:

none

2130 erwarteter Fehler nicht eingetreten

Type:

error

Reason:

An  error or warning announced via  EXPECT did not occur in the

instruction block terminated via ENDEXPECT.

Argument:

The error that was expected

2140 nesting of EXPECT/ENDEXPECT not allowed

Type:

error

Reason:

Code  blocks  framed  via  EXPECT/ENDEXPECT  must  not  contain

nested EXPECT/ENDEXPECT blocks.

Argument:

none

2150 missing ENDEXPECT

Type:

error

Reason:

An  instruction  block  opened  via  EXPECT  was not closed via

ENDEXPECT.

Argument:

none

2160 ENDEXPECT without EXPECT

Type:

error

Reason:

There is no matching previous EXPECT to an ENDEXPECT.

Argument:

none

2170 no default checkpoint register defined

Type:

error

Reason:

No   checkpoint   register   was   specified   for  a  type  12

instruction  and no default  checkpoint register had previously

been defined via the CKPT statement.

Argument:

none

2180 invalid bit field

Type:

error

Reason:

The bit field is not in the required syntax (start,count).

Argument:

the argument in question

2190 argument value missing

Type:

error

Reason:

Arguments must have the form 'variable=value'.

Argument:

the argument in question

2200 unknown argument

Type:

error

Reason:

This   variable  is  not  supported   by  the  selected  target

platform.

Argument:

the argument in question

2210 index register must be 16 bit

Type:

error

Reason:

Z8000 index registers must have a size of 16 bits (Rn).

Argument:

the argument in question

2211 I/O address register must be 16 bit

Type:

error

Reason:

Z8000  registers  used  to  address  I/O  addresses must have a

size of 16 bits (Rn).

Argument:

the argument in question

2212 address register in segmented mode must be 32 bit

Type:

error

Reason:

Z8000  registers to address memory  in segmented mode must have

a size of 32 bits (RRn).

Argument:

the argument in question

2213 address register in non-segmented mode must be 16 bit

Type:

error

Reason:

Z8000  registers to  address memory  in non-segmented mode must

have a size of 16 bits (Rn).

Argument:

the argument in question

2220 invalid structure argument

Type:

error

Reason:

The  argument does not  match any pattern  of allowed arguments

when expanding a structure.

Argument:

the argument in question

2221 too many array dimensions

Type:

error

Reason:

Arrays of structures are limited to being three-dimensional.

Argument:

the dimension argument that was 'too much'

2230 unknown integer notation

Type:

error

Reason:

The  given  integer  notation  does  not  exist, or the leading

plus resp. minus sign is missing.

Argument:

the argument in question

2231 invalid list of integer notations

Type:

error

Reason:

The  requested changes to the  list of usable integer notations

cannot   be   applied,   because   they   would   result  in  a

contradiction.  Currently,  the  only  such  case  are 0hex und

0oct which cannot be used at the same time.

Argument:

none

2240 invalid scale

Type:

error

Reason:

The given argument cannot be used as scaling factor.

Argument:

the argument in question

2250 conflicting string options

Type:

error

Reason:

The  string option  is in  contradiction to  a previously given

option.

Argument:

the option in question

2251 unknown string option

Type:

error

Reason:

The string option does not exist.

Argument:

the option in question

2252 invalid cache invalidate mode

Type:

error

Reason:

Only data, instruction, or both caches may be invalidated.

Argument:

the argument in question

2253 invalid config list

Type:

error

Reason:

The  configuration  list  is  either syntactically incorrect or

contains invalid elements.

Argument:

The list in question or one of its elements

2254 conflicting config options

Type:

error

Reason:

The  option is  in contradiction  to a  previously given option

or repeats a previous one.

Argument:

the option in question

2255 unknown config option

Type:

error

Reason:

The option does not exist.

Argument:

the option in question

2260 invalid CBAR value

Type:

error

Reason:

This  value for  CBAR is  not allowed  (CA must  be larger than

BA).

Argument:

none

2270 page not accessible

Type:

error

Reason:

The  target  address  is  located  in  a  memory  page  that is

currently inaccessible.

Argument:

none

2280 field not accessible

Type:

error

Reason:

The  target  address  is  located  in  a  memory  field that is

currently inaccessible.

Argument:

none

2281 target not in same field

Type:

error

Reason:

Instruction  and  target  address  must  be located in the same

memory field.

Argument:

none

2290 invalid instruction combination

Type:

error

Reason:

These instructions may not be combined with each other.

Argument:

none

2300 unmapped character

Type:

error

Reason:

The  character  string  contains  a  charater  that  cannot  be

mapped.

Argument:

The string in question

2310 invalid length of multi character constant

Type:

error

Reason:

multi   character  constants  must  be  between  one  and  four

characters long.

Argument:

none

2320 no target set (use 'CPU ...' or '-cpu ...' to set one)

Type:

fatal

Reason:

No  target has  been set  so far.  The assembler therefore does

not know which target to generate code for.

Argument:

none

2330 invalid displacement length

Type:

error

Reason:

This  displacement length must  not be used  if this addressing

mode is used.

Argument:

none

10001 error in opening file

Type:

fatal

Reason:

An error was detected while trying to open a file for input.

Argument:

description of the I/O error

10002 error in writing listing

Type:

fatal

Reason:

An error happened while AS was writing the listing file.

Argument:

description of the I/O error

10003 file read error

Type:

fatal

Reason:

An error was detected while reading a source file.

Argument:

description of the I/O error

10004 file write error

Type:

fatal

Reason:

While AS was writing a code or share file, an error happened.

Argument:

description of the I/O error

10006 heap overflow

Type:

fatal

Reason:

The  memory  available  is  not  enough  to  store all the data

needed by AS. Try using the DPMI or OS/2 version of AS.

Argument:

none

10007 stack overflow

Type:

fatal

Reason:

The  program stack crashed, because  too complex formulas, or a

bad  disposition  of  symbols  and/or  macros  were  used.  Try

again, using AS with the option -A.

Argument:

none

10008 INCLUDE nested too deeply

Type:

fatal

Reason:

The  include nesting  depth has  exceeded the  given limit (200

by  default).  The  limit  may  be  raised via the -maxinclevel

command   line  argument,  a  wrong  (recursive)  inclusion  is

however the more probable cause.

Argument:

the INCLUDE statement that exceeded the limit

10010 invalid place holder in listing per-line prefix format

Type:

fatal

Reason:

Only %i, %n, or %a are allowed as place holder.

Argument:

the invalid format

10011 place holder used too often in listing per-line prefix format

Type:

fatal

Reason:

The  place holders %i  and %n each  must not be  used more than

three  times in  the format  string. You're  not satisfied with

that?

Argument:

the format used more than once

B. I/O Error Messages

=====================

The  following error  messages are  generated not  only by AS, but also

by  the auxiliary  programs, like  PLIST, BIND,  P2HEX, and P2BIN. Only

the  most probable error  messages are here  explained. Should you meet

an  undocumented error  message, then  you probably  met a program bug!

Please inform us immediately about this!!

2 file not found

The  file  requested  does  not  exist,  or it is stored on another

drive.

3 path not found

The path of a file does not exist, or it is on another drive.

4 too much open files

There  are no  more file  handles available  to DOS. Increase their

number  changing  the  value  associated  to  FILES=  in  the  file

CONFIG.SYS.

5 file access not allowed

Either  the network access rights do  not allow the file access, or

an attempt was done to rewrite or rename a protected file.

6 invalid file handler

12 invalid access mode

15 invalid drive letter

The required drive does not exist.

16 The file cannot be deleted

17 RENAME cannot be done on this drive

100 Unexpected end of file

A  file  access  tried  to  go  beyond  the  end  of file, although

according  to its  structure this  should not  happen. The  file is

probably corrupted.

101 disk full

This is self explaining! Please, clean up !

102 ASSIGN failed

103 file not open

104 file not open for reading

105 file not open for writing

106 invalid numerical format

150 the disk is write-protected

When  you don't use a hard disk  as work medium storage, you should

sometimes remove the protecting tab from your diskette!

151 unknown device

you  tried  to  access  a  peripheral  unit that is unknown to DOS.

This   should  not  usually  happen,   since  the  name  should  be

automatically interpreted as a filename.

152 drive not ready

close the disk drive door.

153 unknown DOS function

154 invalid disk checksum

A  bad  read  error  on  the  disk.  Try again; if nothing changes,

reformat  the floppy  disk resp.  begin to  take care  of your hard

disk!

155 invalid FCB

156 position error

the  diskette/hard disk controller has not  found a disk track. See

nr. 154 !

157 format unknown

DOS cannot read the diskette format

158 sector not found

As  nr. 156,  but the  controller this  time could  not find a disk

sector in the track.

159 end of paper

You  probably redirected the  output of AS  to a printer. Assembler

printout can be veeery long...

160 device read error

The operating system detected an unclassificable read error

161 device write error

The operating system detected an unclassificable write error

162 general failure error

The  operating system  has absolutely  no idea  of what happened to

the device.

C. Programming Examples

=======================

I  often get  questions about  how to  realize certain  things. Some of

those  are  asked  frequently,  and  it  might be worth documenting the

solutions  in  a  'Tips  and  Tricks'  corner. This chapter is meant to

collect and document them:

C.1. 16 Bit Instructions via Macros

-----------------------------------

Many  8 bit  processors can  only process  eight bits  at once  (as the

name  already implies...). They however  often contain enough registers

to  concatenate two of  them to a  virtual '16 bit  accumulator'. If we

define  macros  to  operate  on  this virtual accumulator, they ideally

should  provide the  same addressing  modes as  the 8  bit instructions

implemented  by the hardware.  To achieve this,  macros somehow have to

'parse' their arguments. How can this be accomplished?

As  an example, the Motorola 6800  contains two accumulators named A an

B.  It is straightforward to  treat them also as  a 16 bit accumulator.

Addressing   modes  should  be  the  same   as  for  8  bit  arithmetic

instructions, namely:

- immediate

- direct (address within first 256 bytes)

- extended (arbitrary address)

- indexed

Therefore,  a macro  implementing a  virtual 16  bit instruction has to

analyze the one or two arguments passsed to it:

1 Indexed addressing is the only mode using two arguments.

2 Immediate addressing is recognized by the leading hash character.

3 Checking  whether an address  is within the  first 256 bytes or not

may be left to the assembler.

The  (single) argument has  to be transformed  into a string to perform

step  2. This string  can then also  be used to  strip the leading hash

character,  to evaluate the actual  immediate value. The complete macro

looks like this:

subd    macro   ARG1,ARG2

if      "ARG2" != ""            ; indexed?

suba    (ARG1)+1,ARG2

sbcb    ARG1,ARG2

elseif                          ; not indexed?

_SARG1   set     "ARG1"           ; convert to string

if      substr(_SARG1,0,1)='#' ; immediate?

_SARG1    set     substr(_SARG1,1,strlen(_SARG1)-1) ; y->del #

suba    #lo(VAL(_SARG1))      ; ...and subtract lo/hi bytes

sbcb    #hi(VAL(_SARG1))

elseif                         ; no immediate->ext. or direct

suba    (ARG1)+1              ; and subtract lo/hi bytes

sbcb    ARG1

endif

endif

endm

Macro  arguments have deliberately been  written in all-uppercase. This

way,  the  macro  works  both  in case-sensitive and non-case-sensitive

mode. The usage of the macro looks like this:

subd    $0007                   ; direct

subd    $1234                   ; absolute

subd    #$55aa                  ; immediate

subd    $12,x                   ; indexed

Of   course,  we  want  to  have  more  16  bit  operations  than  just

subtraction.  One  could  write  a  similar  macro  for  every  type of

operation.  However,  there  is  a  more  elegant  method.  A macro may

itself  contain a macro  definition. So we  can define a  sort of 'meta

macro' which gets the instruction names as arguments:

def16   macro   NEWINST,LOINST,HIINST

NEWINST macro   ARG1,ARG2

if      "ARG2" != ""            ; indexed?

LOINST  (ARG1)+1,ARG2

HIINST  ARG1,ARG2

elseif                          ; not indexed?

_SARG1   set     "ARG1"                 ; convert to string

if      substr(_SARG1,0,1)='#' ; immediate?

_SARG1    set     substr(_SARG1,1,strlen(_SARG1)-1) ; y->del #

LOINST  #lo(VAL(_SARG1))      ; ...and subtract lo/hi bytes

HIINST  #hi(VAL(_SARG1))

elseif                         ; no immediate->ext. or direct

LOINST  (ARG1)+1              ; ...and subtract lo/hi bytes

HIINST  ARG1

endif

endif

endm

endm

The remaining definitions now become one-liners:

def16   addd,adda,adcb

def16   subd,suba,sbcb

def16   andd,anda,andb

def16   ord,ora,orb

def16   eord,eora,eorb

D. Frequently Asked Questions

=============================

In  this chapter,  I tried  to collect  some questions  that arise very

often  together with their  answers. Answers to  the problems presented

in  this chapter might  also be found  at other places  in this manual,

but one maybe does not find them immediately...

Q: I  am  fed  up  with  DOS.  Are  there  versions  of  AS  for other

operating systems ?

A: Apart  from  the  protected  mode  version  that offers more memory

when  working  under  DOS,  ports  exist  for OS/2 and Unix systems

like   Linux  (currently   in  test   phase).  Versions  that  help

operating  system manufacturers  located in  Redmont to become even

richer  are currently not  planned. I will  gladly make the sources

of  AS available  for someone  else who  wants to  become active in

this  direction. The C variant is probably  the best way to start a

port  into  this  direction.  He  should however not expect support

from me that goes beyond the sources themselves...

Q: Is a support of the XYZ processor planned for AS?

A: New  processors are appearing all the time  and I am trying to keep

pace  by  extending  AS.  The  stack  on my desk labeled ''undone''

however  never  goes  below  the  4 inch watermark... Wishes coming

from  users of course play an  important role in the decision which

candidates  will be done first. The  internet and the rising amount

of   documentation   published   in   electronic   form   make  the

acquisition  of  data  books  easier  than  it  used  to be, but it

always  becomes difficult  when more  exotic or older architectures

are  wanted.  If  the  processor  family  in question is not in the

list  of families that  are planned (see  chapter 1), adding a data

book   to  a  request  will   have  a  highly  positive  influence.

Borrowing books is also fine.

Q: Having  a free  assembler is  really fine,  but I  now also had use

for  a  disassembler...and  a  debugger...a  simulator  would  also

really be cool!

A: AS  is a project I work on in  leisure time, the time I have when I

do  not have to care  of how to make  my living. AS already takes a

significant  portion of that time, and  sometimes I make a time-out

to  use my  soldering iron,  enjoy a  Tangerine Dream CD, watch TV,

or  simply to fulfill  some basic human  needs... I once started to

write  the concept  of a  disassembler that  was designed to create

source   code  that   can  be   assembled  and  that  automatically

separates  code  and  data  areas.  I  quickly stopped this project

again  when  I  realized  that  the  remaining  time simply did not

suffice.  I prefer  to work  on one  good program  than to struggle

for  half a dozen  of mediocre apps.  Regarded that way, the answer

to the question is unfortunately ''no''...

Q: The  screen  output  of  AS  is  messed up with strange characters,

e.g. arrows and brackets. Why?

A: AS  will  by  default  use  some  ANSI control sequences for screen

control.  These sequences will appear  unfiltered on your screen if

you  did not install an ANSI  driver. Either install an ANSI driver

or use the DOS command SET USEANSI=N to turn the sequences off.

Q: AS   suddenly  terminates   with  a   stack  overflow  error  while

assembling my program. Did my program become to large?

A: Yes   and  No.  Your  program's  symbol   table  has  grown  a  bit

unsymmetrically   what   lead   to   high  recursion  depths  while

accessing  the table. Errors of this  type especially happen in the

16-bit-OS/2  version of  AS which  has a  very limited  stack area.

Restart  AS  with  the  -A  command  line  switch. If this does not

help,  too complex formula expression are  also a possible cause of

stack  overflows. In  such a  case, try  to split  the formula into

intermediate steps.

Q: It  seems that AS  does not assemble  my program up  to the end. It

worked however with an older version of AS (1.39).

A: Newer  versions  of  AS  no  longer  ignore the END statement; they

actually   terminate   assembly   when   an   END  is  encountered.

Especially  older  include  files  made  by  some  users  tended to

contain   an  END  statement  at   their  end.  Simply  remove  the

superfluous END statements.

Q: I  made an assembly listing  of my program because  I had some more

complicated   assembly   errors   in   my   program.   Upon  closer

investigation  of the  listing, I  found that  some branches do not

point to the desired target but instead to themselves!

A: This  effect happens  in case  of forward  jumps in the first pass.

The  formula parser  does not  yet have  the target  address in its

symbol  table, and  as it  is a  completely independent  module, it

has  to think of a value that  even does not hurt relative branches

with  short  displacement  lengths.  This  is  the  current program

counter  itself...in  the  second  pass,  the  correct values would

have  appeared, but  the second  pass did  not happen due to errors

in  the first one. Correct  the other errors first  so that AS gets

into  the second pass, and the  listing should look more meaningful

again.

Q: Assembly  of my  program works  perfectly, however  I get  an empty

file when I try to convert it with P2HEX or P2BIN.

A: You   probably  did  not  set  the  address  filter  correctly.  By

default,  the filter  is disabled,  i.e. all  data is copied to the

HEX  or  binary  file.  It  is  however possible to create an empty

file  if a manually  set range does  not fit to  the addresses used

by your program.

Q: I  cannot  enter  the  dollar  character  when using P2BIN or P2HEX

under  Unix.  The  automatic  address  range setting does not work,

instead I get strange error messages.

A: Unix  shells  use  the  dollar  character  for  expansion  of shell

variables.   If  you  want  to  pass   a  dollar  character  to  an

application,  prefix it with  a backslash (\).  In the special case

of  the address  range specification  for P2HEX  and P2BIN, you may

also  use 0x  instead of  the dollar  character, which removes this

prblen completely.

Q: I  use  AS  on  a  Linux  system,  the loader program for my target

system  however  runs  on  a  Windows  machine. To simplify things,

both  systems  access  the  same  network drive. Unfortunately, the

Windows  side refuses  to read  the hex  files created by the Linux

side :-(

A: Windows  and Linux systems use  slightly different formats for text

files  (hex files  are a  sort of  text files).  Windows terminates

every  line  with  the  characters  CR  (carriage  return)  and  LF

(linefeed),  however Linux  only uses  the linefeed.  It depends on

the  Windows  program's  'goodwill'  whether  it  will  accept text

files  in  the  Linux  format  or  not.  If  not, it is possible to

transfer  the  files  via  FTP  in  ASCII mode instead of a network

drive.  Alternatively,  the  hex  files  can  be  converted  to the

Windows  format. For example,  the program unix2dos  can be used to

do this, or a small script under Linux:

awk '{print $0"\r"}' test.hex >test_cr.hex

Q: I  have a 16 bit  address in my program  and have to load its upper

and  lower half into  separate CUP registers.  How do I extract the

byte   halves?   Other   assemblers   have  built-in  functions  to

accomodate this.

A: This  can be done  ''by hand'' with  the built-in logical and shift

operators.   However,  there  is  also  a  file  bitfuncs.inc  that

defines the functions lo() respectively hi().

E. Pseudo-Instructions and Integer Syntax

=========================================

This  appendix is designed as  a quick reference to  look up all pseudo

instructions  provided by  AS. The  list is  ordered in  two parts: The

first  part lists the instructions that  are always available, and this

list   is   followed   by   lists   that   enumerate  the  instructions

additionally available for a certain processor family.

Instructions that are always available

......................................

=             :=           ALIGN        BINCLUDE     CASE

CHARSET       CPU          DEPHASE      DOTTEDSTRUCTS ELSE

ELSECASE      ELSEIF       END          ENDCASE      ENDIF

ENDM          ENDS         ENDSECTION   ENDSTRUCT    ENUM

ENUMCONF      ERROR        EQU          .EQU         EVAL

EXITM         FATAL        FORWARD      FUNCTION     GLOBAL

IF            IFB          IFDEF        IFEXIST      IFNB

IFNDEF        IFNEXIST     IFNUSED      IFUSED       INCLUDE

INTSYNTAX     IRP          LABEL        LISTING      MACEXP

MACECP_DFT    MACEXP_OVR   MACRO        MESSAGE      NEWPAGE

NEXTENUM      ORG          .PAGE        PHASE        POPV

PUSHV         PRTEXIT      PRTINIT      PUBLIC       READ

RELAXED       REPT         .RESTORE     RESTOREENV   RORG

.SAVE         SAVEENV      SECTION      SEGMENT      .SET

SHARED        .SHIFT       STRUC        STRUCT       .SWITCH

TITLE         UNION        WARNING      WHILE

Additionally, there are:

- SET  as  an  alias  to  EVAL,  unless  SET  is  already  a  machine

instruction.

- SHIFT resp. SHFT, in case SHIFT is already a machine instruction.

- RESTORE  as an  alias to  RESTOREENV, unless  RESTORE is  already a

machine instruction.

- SAVE  as  an  alias  to  SAVEENV,  unless SAVE is already a machine

instruction.

- PAGE   resp.  PAGESIZE,   in  case   PAGE  is   already  a  machine

instruction.

- SWITCH   resp.  SELECT,  in  case   SWITCH  is  already  a  machine

instruction.

Motorola 680x0/MCF5xxx

......................

Default Integer Syntax: Motorola

DC[.<size>]   DS[.<size>]  FULLPMMU     FPU          PADDING

PMMU          REG          SUPMODE

Motorola 56xxx

..............

Default Integer Syntax: Motorola

DC            DS           PACKING      XSFR         YSFR

PowerPC

.......

Default Integer Syntax: C

BIGENDIAN     DB           DD           DN           DO

DQ            DS           DT           DW           REG

SUPMODE

IBM PALM

........

Default Integer Syntax: IBM

DB            DD           DN           DO           DQ

DS            DT           DW           PORT         REG

Motorola M-Core

...............

Default Integer Syntax: Motorola

DC[.<size>]   DS[.<size>]  REG          SUPMODE

Motorola XGATE

..............

Default Integer Syntax: Motorola

ADR           BYT          DC[.<size>]  DFS          DS[.<size>]

FCB           FCC          FDB          PADDING      REG

RMB

Motorola 68xx/Hitachi 63xx

..........................

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>]  DFS

DS[.<size>]   DW           FCB          FCC          FDB

PADDING       PRWINS(68HC11K4) RMB

Motorola/Freescale 6805/68HC(S)08

.................................

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>]  DFS

DS[.<size>]   DW           FCB          FCC          FDB

PADDING       RMB

Motorola 6809/Hitachi 6309

..........................

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DC[.<size>]

DFS           DS[.<size>]  DW           FCB          FCC

FDB           PADDING      PLAINBASE    RMB

Konami 052001

.............

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>] DFS

DS[.<size>]   DW           FCB          FCC          FDB

PADDING       RMB

Motorola 68HC12

...............

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>]  DFS

DS[.<size>] DW FCB         FCC          FDB

PADDING       RMB

NXP S12Z

........

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>]  DEFBIT

DEFBITFIELD   DFS          DS[.<size>]  DW           FCB

FCC           FDB          PADDING      RMB

Motorola 68HC16

...............

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DC[.<size>]

DFS           DS[.<size>]  DW           FCB          FCC

FDB           PADDING      RMB

Freescale 68RS08

................

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DC[.<size>]

DFS           DS[.<size>]  DW           FCB          FCC

FDB           PADDING

Hitachi H8/300(L/H)

...................

Default Integer Syntax: Motorola

BIT           DC[.<size>]  DS[.<size>]  MAXMODE      PADDING

REG

Hitachi H8/500

..............

Default Integer Syntax: Motorola

ASSUME        BIT          COMPMODE     DATA         DC[.<size>]

DS[.<size>]   MAXMODE      PADDING      REG

Hitachi SH7x00

..............

Default Integer Syntax: Motorola

COMPLITERALS  DC[.<size>]  DS[.<size>]  LTORG        PADDING

REG           SUPMODE

Hitachi HMCS400

...............

Default Integer Syntax: Motorola

DATA          RES          SFR

Hitachi H16

...........

Default Integer Syntax: Motorola

BIT           DC[.<size>]  DS[.<size>]  REG          SUPMODE

65xx/MELPS-740

..............

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DCB

DDB           DFS          DS           DW           FCB

FCC           FDB          RMB

65816/MELPS-7700

................

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DD

DDB           DN           DO           DQ           DS

DT            DW           DFS          FCB          FCC

FDB           RMB

Mitsubishi MELPS-4500

.....................

Default Integer Syntax: Motorola

DATA          RES          SFR

Rockwell PPS-4

..............

Default Integer Syntax: Intel

DATA          DC           DS           RES

Mitsubishi M16

..............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           REG

Mitsubishi M16C

...............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           REG

DEC PDP-11

..........

Default Integer Syntax: C

BYTE          CIS          EIS          FIS          FLT2

FLT4          FP11         REG          SUPMODE      WORD

WD16

....

Default Integer Syntax: C

ASCII         ASCIZ       BYTE          FLT3         REG

WORD

DEC VAX

.......

ACCMODE       ASCIC        ASCII        ASCIZ        BLKB

BLKD          BLKF         BLKG         BLKH         BLKL

BLKO          BLKQ         BLKW         BYTE         D_FLOATING

DOUBLE        F_FLOATING   FLOAT        G_FLOATING   H_FLOATING

LONG          OCTA         QUAD         WORD

Olympia CP-3F/GI LP8000/SGS M380

................................

Default Integer Syntax: Intel

DC            DS           PORT

Intel 4004/4040

...............

Default Integer Syntax: Intel

DATA          DS           REG

Intel 8008

..........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           PORT         Z80SYNTAX

Intel MCS-48

............

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           REG

Intel MCS-(2)51

...............

Default Integer Syntax: Intel

BIGENDIAN     BIT          DB           DD           DN

DO            DQ           DS           DT           DW

PORT          REG          SFR          SFRB         SRCMODE

Intel MCS-96

............

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW

Intel 8080/8085

...............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           PORT

Intel 8086/80186/NEC V20...V5x

..............................

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           PORT

REG

Intel i960

..........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           FPU          REG

SPACE         SUPMODE      WORD

Signetics 8X30x

...............

Default Integer Syntax: Motorola

LIV           RIV

Signetics 2650

..............

Default Integer Syntax: Motorola

ACON          BIGENDIAN    DB           DD           DN

DO            DQ           DS           DT           DW

RES

Philips XA

..........

Default Integer Syntax: Intel

ASSUME        BIT          DB           DC[.<size>]  DD

DN            DO           DQ           DS[.<size>]  DT

DW            PADDING      PORT         REG          SUPMODE

Atmel AVR

.........

Default Integer Syntax: C

BIT           DATA         DB           DD           DN

DO            DQ           DS           DT           DW

PACKING       PORT         REG          RES          SFR

AMD 29K

.......

Default Integer Syntax: C

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           EMULATED

ERG           SUPMODE

Siemens 80C166/167

..................

Default Integer Syntax: Intel

ASSUME        BIT          DB           DD           DN

DO            DQ           DS           DT           DW

REG

Zilog Zx80

..........

Default Integer Syntax: Intel

DB            DD           DEFB         DEFW         DN

DO            DQ           DS           DT           DW

EXTMODE       LWORDMODE    PRWINS(Z180) REG          WARNRELATIVE

Zilog Z8

........

Default Integer Syntax: Intel

DB            DEFBIT       DD           DN           DO

DQ            DS           DT           DW           REG

SFR

Zilog Z8000

...........

Default Integer Syntax: Intel

DB            DD           DEFBIT       DEFBITB      DN

DO            DQ           DS           DT           DW

PORT          REG

Xilinx KCPSM

............

Default Integer Syntax: Intel

CONSTANT      NAMEREG      REG

Xilinx KCPSM3

.............

Default Integer Syntax: Intel

CONSTANT      DB           DD           DN           DO

DQ            DS           DT           DW           NAMEREG

PORT          REG

LatticeMico8

............

Default Integer Syntax: C

DB            DD           DN           DO           DQ

DS            DT           DW           PORT         REG

Toshiba TLCS-900

................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           MAXIMUM      SUPMODE

Toshiba TLCS-90

...............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Toshiba TLCS-870(/C)

....................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Toshiba TLCS-47(0(A))

.....................

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           PORT

Toshiba TLCS-9000

.................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           REG

Toshiba TC9331

..............

Default Integer Syntax: Intel

Microchip PIC16C5x

..................

Default Integer Syntax: Motorola

DATA          RES          SFR          ZERO

Microchip PIC16C8x

..................

Default Integer Syntax: Motorola

DATA          RES          SFR          ZERO

Microchip PIC17C42

..................

Default Integer Syntax: Motorola

DATA          RES          SFR          ZERO

Parallax SX20

.............

Default Integer Syntax: Motorola

BIT           DATA         SFR          ZERO

SGS-Thomson ST6

...............

Default Integer Syntax: Intel

ASCII         ASCIZ        ASSUME       BIT          BYTE

BLOCK         SFR          WORD

SGS-Thomson ST7/STM8

....................

Default Integer Syntax: Intel

DC[.<size>]   DS[.<size>]  PADDING

SGS-Thomson ST9

...............

Default Integer Syntax: Intel

ASSUME        BIT          DB           DD           DN

DO            DQ           DS           DT           DW

REG

6804

....

Default Integer Syntax: Motorola

ADR           BYT          DB           DFS          DS

DW            FCB          FCC          FDB          RMB

SFR

Texas Instruments TMS3201x

..........................

Default Integer Syntax: Intel

DATA          PORT         RES

Texas Instruments TMS32C02x

...........................

Default Integer Syntax: Intel

BFLOAT        BSS          BYTE         DATA         DOUBLE

EFLOAT        TFLOAT       LONG         LQxx         PORT

Qxx           RES          RSTRING      STRING       WORD

Texas Instruments TMS320C3x/C4x

...............................

Default Integer Syntax: Intel

ASSUME        BSS          DATA         EXTENDED     PACKING

SINGLE        WORD

Texas Instruments TM32C020x/TM32C05x/TM32C054x

..............................................

Default Integer Syntax: Intel

BFLOAT        BSS          BYTE         DATA         DOUBLE

EFLOAT        TFLOAT       LONG         LQxx         PORT

Qxx           RES          RSTRING      STRING       WORD

Texas Instruments TMS320C6x

...........................

Default Integer Syntax: Intel

BSS           DATA         DOUBLE       PACKING      SINGLE

WORD

Texas Instruments TMS99xx

.........................

Default Integer Syntax: Intel

BSS           BYTE         DOUBLE       PADDING      SINGLE

WORD

Texas Instruments Instruments TMS1000

.....................................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Texas Instruments TMS70Cxx

..........................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Texas Instruments TMS370

........................

Default Integer Syntax: Intel

DB            DBIT         DD           DN           DO

DQ            DS           DT           DW

Texas Instruments MSP430

........................

Default Integer Syntax: Intel

BSS           BYTE         PADDING      REG          WORD

National IMP-16

...............

Default Integer Syntax: IBM

ASCII         LTORG        PORT         WORD

National IPC-16/INS8900

.......................

Default Integer Syntax: IBM

ASCII         ASSUME       LTORG        WORD

National SC/MP

..............

Default Integer Syntax: C

BIGENDIAN     DB           DD           DN           DO

DQ            DS           DT           DW           REG

National INS807x

................

Default Integer Syntax: C

DB            DD           DN           DO           DQ

DS            DT           DW

National COP4

.............

Default Integer Syntax: C

ADDR          ADDRW        BYTE         DB           DD

DO            DQ           DS           DSB          DSW

DT            DW           FB           FW           SFR

WORD

National COP8

.............

Default Integer Syntax: C

ADDR          ADDRW        BYTE         DB           DD

DQ            DS           DSB          DSW          DT

DW            FB           FW           SFR          WORD

National SC14xxx

................

Default Integer Syntax: C

DC            DC8          DS           DS8          DS16

DW            DW16

National NS32xxx

................

BIGENDIAN     BYTE         CUSTOM       DB           DD

DOUBLE        DO           DQ           DS           DT

DW            FLOAT        FPU          LONG         PMMU

REG           SUPMODE      WORD

Fairchild ACE

.............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Fairchild F8

............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           PORT

NEC �PD7800...�PD7806, �PD78(C)1x

.................................

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           Z80SYNTAX

NEC �PD7807...�PD7809

.....................

Default Integer Syntax: Intel

ASSUME        DB           DEFBIT      DD            DN

DO            DQ           DS           DT           DW

NEC 75xx

........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

NEC �COM-43

...........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

NEC 75K0

........

Default Integer Syntax: Intel

ASSUME        BIT          DB           DD           DN

DO            DQ           DS           DT           DW

SFR

NEC 78K0

........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

NEC 78K2

........

Default Integer Syntax: Intel

BIT           DB           DD           DN           DO

DQ            DS           DT           DW

NEC 78K3

........

Default Integer Syntax: Intel

BIT           DB           DD           DN           DO

DQ            DS           DT           DW

NEC 78K4

........

Default Integer Syntax: Intel

BIT           DB           DD           DN           DO

DQ            DS           DT           DW

NEC �PD772x

...........

Default Integer Syntax: Intel

DATA          PACKING      RES

NEC �PD77230

............

Default Integer Syntax: Intel

DS            DW           PACKING

NEC V60

.......

Default Integer Syntax: Intel

DC[.<size>]   DS[.<size>]  PADDING      REG          SUPMODE

Symbios Logic SYM53C8xx

.......................

Default Integer Syntax: C

DB            DD           DN           DO           DQ

DS            DT           DW

Fujitsu F^2MC8L

...............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Fujitsu F^2MC16L

................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

OKI OLMS-40

...........

Default Integer Syntax: Intel

DATA          RES          SFR

OKI OLMS-50

...........

Default Integer Syntax: Intel

DATA          RES          SFR

Panafacom MN161x

................

Default Integer Syntax: IBM

DC            DS           PACKING

Padauk PMC/PMS/PFSxxx

.....................

Default Integer Syntax: C

BIT           DATA         RES          SFR

Intersil 180x

.............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

XMOS XS1

........

Default Integer Syntax: Motorola

DB            DD           DQ           DN           DO

DS            DT           DW           REG

ATARI Vector

............

Default Integer Syntax: Motorola

MIL STD 1750

............

Default Integer Syntax: Intel

DATA          EXTENDED     FLOAT

KENBAK

......

Default Integer Syntax: Intel

BIT           DB           DD           DN           DO

DQ            DS           DT           DW           REG

CP-1600

.......

Default Integer Syntax: IBM (hex), C (oct)

BYTE          PACKING      RES          TEXT         WORD

ZERO

HP Nano Processor

.................

Default Integer Syntax: C

IM61x0

......

Default Integer Syntax: C

DC            DECIMAL      DS           LTORG        OCTAL

ZERO

Renesas RX

..........

Default Integer Syntax: Intel

BLKB          BLKW         BLKL         BLKD         BYTE

DOUBLE        ENDIAN       FLOAT        LWORD        WORD

Sharp SC61860

.............

Default Integer Syntax: Motorola

ADR           BYT          DB           DC           DFS

DS            DW           FCB          FCC          FDB

RMB

Sharp SC62015

.............

Default Integer Syntax: Intel

DN            DB           DD           DO           DQ

DS            DT           DW

F. Predefined Symbols

=====================

+---------------+-----------+------------+-----------------------------+

| Name          | Data Type | Definition | Meaning                     |

+---------------+-----------+------------+-----------------------------+

+---------------+-----------+------------+-----------------------------+

| ARCHITECTURE  | string    | predef.    | target platform AS was      |

|               |           |            | compiled for, in the style  |

|               |           |            | processor-manufacturer-     |

|               |           |            | operating system            |

| BIGENDIAN     | boolean   | dyn.(0)    | storage of constants MSB    |

|               |           |            | first?                      |

| CASESENSITIVE | boolean   | normal     | case sensitivity in symbol  |

|               |           |            | names?                      |

| CONSTPI       | float     | normal     | constant Pi (3.1415.....)   |

| DATE          | string    | predef.    | date of begin of assembly   |

| FALSE         | boolean   | predef.    | 0 = logically ''false''     |

| HASFPU        | boolean   | dyn.(0)    | coprocessor instructions    |

|               |           |            | enabled?                    |

| HASPMMU       | boolean   | dyn.(0)    | MMU instructions            |

|               |           |            | enabled?                    |

| INEXTMODE     | boolean   | dyn.(0)    | XM flag set for 4 Gbyte     |

|               |           |            | address space?              |

| INLWORDMODE   | boolean   | dyn.(0)    | LW flag set for 32 bit      |

|               |           |            | instructions?               |

| INMAXMODE     | boolean   | dyn.(0)    | processor in maximum        |

|               |           |            | mode?                       |

| INSUPMODE     | boolean   | dyn.(0)    | processor in supervisor     |

|               |           |            | mode?                       |

| INSRCMODE     | boolean   | dyn.(0)    | processor in source mode?   |

| FULLPMMU      | boolean   | dyn.(0/1)  | full PMMU instruction set   |

|               |           |            | allowed?                    |

| LISTON        | boolean   | dyn.(1)    | listing enabled?            |

| MACEXP        | boolean   | dyn.(1)    | expansion of macro con-     |

|               |           |            | structs in listing enabled? |

| MOMCPU        | integer   | dyn.       | number of target CPU        |

|               |           | (68008)    | currently set               |

| MOMCPUNAME    | string    | dyn.       | name of target CPU          |

|               |           | (68008)    | currently set               |

| MOMFILE       | string    | special    | current source file         |

|               |           |            | (including include files)   |

| MOMLINE       | integer   | special    | current line number in      |

|               |           |            | source file                 |

| MOMPASS       | integer   | special    | number of current pass      |

| MOMSECTION    | string    | special    | name of current section or  |

|               |           |            | empty string if out of any  |

|               |           |            | section                     |

| MOMSEGMENT    | string    | special    | name of address space       |

|               |           |            | currently selected          |

|               |           |            | with SEGMENT                |

| NESTMAX       | Integer   | dyn.(256)  | maximum nesting level       |

|               |           |            | of macro expansions         |

| PADDING       | Boolean   | dyn.(1)    | pad byte field to even      |

|               |           |            | count?                      |

| RELAXED       | Boolean   | dyn.(0)    | any syntax allowed integer  |

|               |           |            | constants?                  |

| PC            | Integer   | special    | current program counter     |

|               |           |            | (Thomson)                   |

| TIME          | String    | predef.    | time of begin of assembly   |

|               |           |            | (1st pass)                  |

| TRUE          | Integer   | predef.    | 1 = logically ''true''      |

| VERSION       | Integer   | predef.    | version of AS in BCD        |

|               |           |            | coding, e.g. 1331 hex for   |

|               |           |            | version 1.33p1              |

| WRAPMODE      | Integer   | predef.    | shortened program           |

|               |           |            | counter assumed?            |

|  *            | Integer   | special    | current program counter     |

|               |           |            | (Motorola, Rockwell,        |

|               |           |            | Microchip, Hitachi)         |

| .             | Integer   | special    | curr. program counter       |

|               |           |            | (IM61x0)                    |

| $             | Integer   | special    | current program counter     |

|               |           |            | Intel, Zilog, Texas,        |

|               |           |            | Toshiba, NEC, Siemens,      |

|               |           |            | AMD)                        |

+---------------+-----------+------------+-----------------------------+

To  be exact,  boolean symbols  are just  ordinary integer symbols with

the  difference that AS  will assign only  two different values to them

(0  or 1, corresponding  to False or  True). AS does  not store special

symbols  in  the  symbol  table.  For  performance  reasons,  they  are

realized  with  hardcoded  comparisons  directly  in  the  parser. They

therefore  do  not  show  up  in  the  assembly listing's symbol table.

Predefined  symbols are only set  once at the beginning  of a pass. The

values  of dynamic  symbols may  in contrast  change during assembly as

they  reflect  settings  made  with  related  pseudo  instructions. The

values  added in  parentheses give  the value  present at the beginning

of a pass.

The  names given in this table also  reflect the valid way to reference

these symbols in case-sensitive mode.

The  names listed  here should  be avoided  for own symbols; either one

can  define but not access them  (special symbols), or one will receive

an  error message due  to a double-defined  symbol. The ugliest case is

when  the redefinition  of a  symbol made  by AS  at the beginning of a

pass leads to a phase error and an infinite loop...

G. Shipped Include Files

========================

The  distribution of AS contains a  couple of include files. Apart from

include  files  that  only  refer  to  a specific processor family (and

whose  function should be  immediately clear to  someone who works with

this  family),  there  are  a  few  processor-independent  files  which

include  useful functions. The  functions defined in  these files shall

be explained briefly in the following sections:

G.1. BITFUNCS.INC

-----------------

This  file defines  a couple  of bit-oriented  functions that  might be

hardwired  for other assemblers.  In the case  of AS however, thaey are

implemented with the help of user-defined functions:

-  mask(start,bits)  returns an  integer with  bits bits set starting

at position start;

-  invmask(start,bits) returns one's complement to mask();

-  cutout(x,start,bits)   returns  bits   bits  masked   out  from  x

starting at position start without shifting them to position 0;

-  hi(x) returns the second lowest byte (bits 8..15) of x;

-  lo(x) returns the lowest byte (bits 8..15) of x;

-  hiword(x) returns the second lowest word (bits 16..31) of x;

-  loword(x) returns the lowest word (bits 0..15) of x;

-  odd(x) returns TRUE if x is odd;

-  even(x) returns TRUE if x is even;

-  getbit(x,n) extracts bit n out of x and returns it as 0 or 1;

-  shln(x,size,n)  shifts a word  x of length  size to the  left by n

places;

-  shrn(x,size,n)  shifts a word x  of length size to  the right by n

places;

-  rotln(x,size,n)  rotates the lowest  size bits of  an integer x to

the left by n places;

-  rotrn(x,size,n)  rotates the lowest  size bits of  an integer x to

the right by n places;

G.2. CTYPE.INC

--------------

This  include  file  is  similar  to  the  C include file ctype.h which

offers  functions to classify characters.  All functions deliver either

TRUE or FALSE:

-  isdigit(ch) becomes TRUE if ch is a valid decimal digit (0..9);

-  isxdigit(ch)  becomes  TRUE  if  ch  is  a valid hexadecimal digit

(0..9, A..F, a..f);

-  isupper(ch)  becomes TRUE if ch  is an uppercase letter, excluding

special national characters);

-  islower(ch)  becomes TRUE if  ch is a  lowercase letter, excluding

special national characters);

-  isalpha(ch)  becomes  TRUE  if  ch  is a letter, excluding special

national characters);

-  isalnum(ch)  becomes  TRUE  if  ch  is  either a letter or a valid

decimal digit;

-  isspace(ch)  becomes TRUE  if ch  is an  'empty' character (space,

form feed, line feed, carriage return, tabulator);

-  isprint(ch)  becomes TRUE if ch is  a printable character, i.e. no

control character up to code 31;

-  iscntrl(ch) is the opposite to isprint();

-  isgraph(ch)  becomes  TRUE  if  ch  is  a  printable  and  visible

character;

-  ispunct(ch)  becomes TRUE if  ch is a  printable special character

(i.e. neither space nor letter nor number);

H. Acknowledgments

==================

''If I have seen farther than other men,

it is because I stood on the shoulders of giants.''

--Sir Isaac Newton

''If I haven't seen farther than other men,

it is because I stood in the footsteps of giants.''

--unknown

There  is a  commaon saying  that programs  you write are like children

you  put into the world. It is now  more than 30 years that I have been

working  on  this  assembler,  and  I  have come to the conclusion that

such  a project  is rather  a journey  for its  author. The  people you

meet  and learn to  know on this  journey are at  least as important as

the  perceived target itself. Your learn a  lot on this trip and in the

best  case, one also  understands that things  can also be  seen from a

completely  different perspective. If the  discussions are fruitful, it

is a win for both sides.

While  making this  journey, some  people have  kept a special place in

my  memories, because of  the way they  contributed to this project and

to  the state it  has now achieved.  The following enumeration of these

people  is naturally incomplete, since my  memories are not any more as

good  as they  used to  be. So  the first  'thank you'  goes to all the

people  I (accidentally) omit in  the following paragraphs. The journey

goes on, and maybe our ways will be crossing again!

The  concept of  AS as  a universal  cross assembler came from Bernhard

(C.)  Zschocke  who  needed  a  ''student  friendly'',  i.e. free cross

assembler  for his microprocessor  course and talked  me into extending

an  already  existing  68000  assembler.  The  rest  is  history... The

microprocessor  course  held  at  RWTH  Aachen also always provided the

most   engaged  users  (and  bug-searchers)  of  new  AS  features  and

therefore contributed a lot to today's quality of AS.

The  Internet and  FTP have  proved to  be a  big help for spreading AS

and  reporting  of  bugs.  My  thanks  therefore  go  to the FTP admins

(Bernd  Casimir  in  Stuttgart,  Norbert  Breidor in Aachen, and J�rgen

Mei�burger  in J�lich).  Especially the  last one  personally engaged a

lot to establish a practicable way in J�lich.

As  we are just talking  about the ZAM: Though  Wolfgang E. Nagel never

involved  personally  into  AS,  he  however  was my tutor and boss. He

always  had  at  least  four  eyes  on  what I was doing. Regarding AS,

there seemed to be at least one that smiled...

A  program like  AS cannot  be done  without appropriate data books and

documentation.  I  received  information  from  an  enormous  amount of

people,  ranging from  tips up  to complete  data books. An enumeration

follows (as stated before, this is very probably incomplete!):

Ernst  Ahlers,  Charles  Altmann,  Marco  Awater,  Len  Bayles, Andreas

Bolsch,  Rolf  Buchholz,  Bernd  Casimir,  Nils  Eilers, Gunther Ewald,

Michael    Haardt,   Stephan   Hruschka,   Fred   van   Kempen,   Peter

Kliegelh�fer,  Ulf Meinke,  Udo M�ller,  Matthias Paul,  Norbert Rosch,

Curt  J.  Sampson,  Steffen  Schmid,  Leonhard Schneider, Ernst Schwab,

Michael   Schwingen,   Oliver   Sellke,   Christian   Stelter,   Patrik

Str�mdahl,  Tadashi G.  Takaoka, Oliver  Thamm, Thorsten Thiele, Leszek

Ulman, Rob Warmelink, Andreas Wassatsch, John Weinrich.

...and  a  somewhat  ironic  ''thank  you''  to  Rolf-Dieter-Klein  and

Tobias  Thiel,  who  gave  me  the  initial  impulse with their ASM68K.

Several  things  did  not  work  the  way  I  wanted them to have, so I

thought I could do better or at least different.

And  of course, I did not entirely write  AS on my own. The DOS version

of  AS contained  the OverXMS  routines from  Wilbert van  Leijen which

can  move the overlay  modules into the  extended memory. A really nice

library, easy to use without problems!

The  TMS320C2x/5x code generators  and the file  STDDEF2x.INC come from

Thomas  Sailer,  ETH  Zurich.  It's  surprising,  he  only  needed  one

weekend  to  understand  my  coding  and  to  implement  the  new  code

generator.  Either that  was a  long nightshift  or I am slowly getting

old...the  same praise  goes to  Haruo Asano  for providing the MN1610/

MN1613,   IM6100,  CP1600,   Renesas  RX   and  HP  NanoProcessor  code

generators.

I. Changes since Version 1.3

============================

- version 1.31:

- additional  MCS-51 processor  type 80515.  The number  is again

only  stored  by  the  assembler.  The  file  STDDEF51.INC  was

extended  by  the  necessary  SFRs.  CAUTION! Some of the 80515

SFRs have moved to other addresses!

- additional support for the Z80 processor;

- faster 680x0 code generator.

- version 1.32:

- syntax  for  zero  page  addresses  for  the  65xx  family  was

changed from addr.z to <addr (similar to 68xx);

- additional   support  for  the  6800,   6805,  6301,  and  6811

processors;

- the  8051  part  now  also  understands  DJNZ,  PUSH,  and  POP

(sorry);

- the  assembly listing  now not  also list  the symbols but also

the macros that have been defined;

- additional  instructions IFDEF/IFNDEF  for conditional assembly

based on the existence of a symbol;

- additional  instructions  PHASE/DEPHASE  to  support  code that

shall be moved at runtime to a different address;

- additional  instructions  WARNING,  ERROR,  and  FATAL to print

user-defined error messages;

- the  file  STDDEF51.INC  additionally  contains the macro USING

to simplify working with the MCS-51's register banks;

- command line option u to print segment usage;

- version 1.33:

- additionally supports the 6809 processor;

- added string variables;

- The  instructions TITLE, PRTINIT,  PRTEXIT, ERROR, WARNING, and

FATAL  now expect a string  expression. Constants therefore now

have  to be  enclosed in  '' instead  of '  characters. This is

also true for DB, DC.B, and BYT;

- additional  instruction ALIGN to align  the program counter for

Intel processors;

- additional  instruction LISTING  to turn  the generation  of an

assembly listing on or off;

- additional   instruction  CHARSET  for  user-defined  character

sets.

- version 1.34:

- the  second pass  is now  omitted if  there were  errors in the

first pass;

- additional   predefined  symbol   VERSION  that   contains  the

version number of AS;

- additional   instruction   MESSAGE   to   generate   additional

messages under program control;

- formula parser is now accessible via string constants;

- if  an error  in a  macro occurs,  additionally the line number

in the macro itself is shown;

- additional  function UPSTRING to convert a string to all upper-

case.

- version 1.35:

- additional  function TOUPPER  to convert  a single character to

upper case;

- additional instruction FUNCTION for user-defined functions;

- additional  command  line  option  D  to  define  symbols  from

outside;

- the  environment variable ASCMD for  commonly used command line

options was introduced;

- the  program will additionally  be checked for  double usage of

memory areas if the u option is enabled;

- additional   command  line   option  C   to  generate  a  cross

reference list.

- version 1.36:

- additionally  supports  the  PIC16C5x  and  PIC17C4x  processor

families;

- the  assembly listing  additionally shows  the nesting depth of

include files;

- the  cross  reference  list  additionally  shows the definition

point of a symbol;

- additional  command  line  option  A  to  force  a more compact

layout of the symbol table.

- version 1.37:

- additionally  supports  the  processors  8086, 80186, V30, V35,

8087, and Z180;

- additional   instructions  SAVE  and   RESTORE  for  an  easier

switching of some flags;

- additional operators for logical shifts and bit mirroring;

- command line options may now be negated with a plus sign;

- additional  filter AS2MSG for  a more comfortable  work with AS

under Turbo-Pascal 7.0;

- ELSEIF  now may have  an argument for  construction of IF-THEN-

ELSE ladders;

- additional  CASE construct  for a  more comfortable conditional

assembly;

- user-defined functions now may have more than one argument;

- P2HEX  can  now  additionally  generate  hex  files in a format

suitable for 65xx processors;

- BIND,  P2HEX, and  P2BIN now  have the  same scheme for command

line processing like AS;

- additional   switch  i  for  P2HEX  to  select  one  out  three

possibilities for the termination record;

- additional functions ABS and SGN;

- additional predefined symbols MOMFILE and MOMLINE;

- additional option to print extended error messages;

- additional  instruction IFUSED  and IFNUSED  to check whether a

symbol has been used so far;

- The  environment variables  ASCMD, BINDCMD  etc. now optionally

may  contain the  name of  a file  that provides more space for

options;

- P2HEX  can now generate the  hex formats specified by Microchip

(p4);

- a  page  length  specification  of  0  now  allows  to suppress

automatic formfeeds in the assembly listing completely (p4);

- symbols  defined in  the command  line now  may be  assigned an

arbitrary value (p5).

- version 1.38:

- changed  operation  to  multipass  mode.  This  enables  AS  to

generate optimal code even in case of forward references;

- the  8051  part  now  also  knows  the  generic  JMP  and  CALL

instructions;

- additionally supports the Toshiba TLCS-900 series (p1);

- additional  instruction  ASSUME  to  inform the assembler about

the 8086's segment register contents (p2);

- additionally supports the ST6 series from SGS-Thomson (p2);

- ..and  the 3201x signal processors from Texas Instruments (p2);

- additional  option  F  for  P2HEX  to  override  the  automatic

format selection (p2);

- P2BIN  now can automatically  set the start  resp. stop address

of the address window by specifying dollar signs (p2);

- the   8048   code   generator   now   also  knows  the  8041/42

instruction extensions (p2);

- additionally supports the Z8 microcontrollers (p3).

- version 1.39:

- additional opportunity to define sections and local symbols;

- additional  command line switch h  to force hexadecimal numbers

to use lowercase;

- additional  predefined  symbol  MOMPASS  to  read the number of

the currently running pass;

- additional  command line  switch t  to disable individual parts

of the assembly listing;

- additionally  knows the  L variant  of the  TLCS-900 series and

the MELPS-7700 series from Mitsubishi (p1);

- P2HEX  now  also  accepts  dollar  signs  as  start  resp. stop

address (p2);

- additionally supports the TLCS-90 family from Toshiba (p2);

- P2HEX  now also can  output data in  Tektronix and 16 bit Intel

Hex format (p2);

- P2HEX now prints warnings for address overflows (p2);

- additional  include file STDDEF96.INC  with address definitions

for the TLCS-900 series (p3);

- additional  instruction  READ  to  allow  interactive  input of

values during assembly (p3);

- error  messages are  written to  the STDERR  channel instead of

standard output (p3);

- the  STOP  instruction  missing  for  the 6811 is now available

(scusi, p3);

- additionally supports the �PD78(C)1x family from NEC (p3);

- additionally supports the PIC16C84 from NEC (p3);

- additional  command line  switch E  to redirect  error messages

to a file (p3);

- The MELPS-7700's 'idol' 65816 is now also available (p4);

- the   ST6  pseudo  instruction  ROMWIN  has  been  removed  was

integrated into the ASSUME instruction (p4);

- additionally supports the 6804 from SGS-Thomson (p4);

- via  the  NOEXPORT  option  in  a  macro  definition, it is now

possible  to  define  individually  for  every macro whether it

shall appear in the MAC file or not (p4);

- the  meaning of  MACEXP regarding  the expansion  of macros has

changed  slightly due to the  additional NOEXPAND option in the

macro definition (p4);

- The  additional  GLOBAL  option  in  the  macro  definition now

additionally   allows  to  define   macros  that  are  uniquely

identified by their section name (p4).

- version 1.40:

- additionally supports the DSP56000 from Motorola;

- P2BIN  can now also extract the lower resp. upper half of a 32-

bit word;

- additionally  supports the  TLCS-870 and  TLCS-47 families from

Toshiba (p1);

- a  prefixed ! now  allows to reach  machine instructions hidden

by a macro (p1);

- the  GLOBAL  instruction  now  allows  to  export  symbols in a

qualified style (p1);

- the  additional r  command line  switch now  allows to  print a

list of constructs that forced additional passes (p1);

- it  is now possible to  omit an argument to  the E command line

option; AS will then choose a fitting default (p1);

- the   t  command  line  option  now  allows  to  suppress  line

numbering in the assembly listing (p1);

- escape  sequences may now  also be used  in ASCII style integer

constants (p1);

- the   additional  pseudo  instruction  PADDING  now  allows  to

enable  or  disable  the  insertion  of  padding bytes in 680x0

mode (p2);

- ALIGN is now a valid instruction for all targets (p2);

- additionally knows the PIC16C64's SFRs (p2);

- additionally supports the 8096 from Intel (p2);

- DC additionally allows to specify a repetition factor (r3);

- additionally   supports   the   TMS320C2x   family  from  Texas

Instruments   (implementation  done   by  Thomas   Sailer,  ETH

Zurich, r3); P2HEX has been extended appropriately;

- an equation sign may be used instead of EQU (r3);

- additional ENUM instruction to define enumerations (r3);

- END now has a real effect (r3);

- additional  command  line  switch  n  to get the internal error

numbers in addition to the error messages (r3);

- additionally supports the TLCS-9000 series from Toshiba (r4);

- additionally   supports   the   TMS370xxx   series  from  Texas

Instruments, including a new DBIT pseudo instruction (r5);

- additionally knows the DS80C320's SFR's (r5);

- the  macro processor  is now  also able  to include  files from

within  macros.  This  required  to  modify the format of error

messages  slightly. If you use AS2MSG,  replace it with the new

version! (r5)

- additionally supports the 80C166 from Siemens (r5);

- additional VAL function to evaluate string expressions (r5);

- it  is now possible to construct  symbol names with the help of

string expressions enclosed in braces (r5);

- additionally knows the 80C167's peculiarities (r6);

- the  MELPS740's special  page addressing  mode is now supported

(r6);

- it  is now  possible to  explicitly reference  a symbol  from a

certain  section by  appending its  name enclosed  in brackets.

The construction with an @ sign has been removed! (r6)

- additionally  supports  the  MELPS-4500  series from Mitsubishi

(r7);

- additionally  supports H8/300  and H8/300H  series from Hitachi

(r7);

- settings  made with LISTING  resp. MACEXP may  now be read back

from predefined symbols with the same names (r7);

- additionally   supports   the   TMS320C3x   series  from  Texas

Instruments (r8);

- additionally supports the SH7000 from Hitachi (r8);

- the Z80 part has been extended to also support the Z380 (r9);

- the  68K part has been extended  to know the differences of the

683xx micro controllers (r9);

- a  label not any more  has to be placed  in the first row if it

is marked with a double dot (r9);

- additionally supports the 75K0 series from NEC (r9);

- the  additional command  line option  o allows  to set  a user-

defined name for the code file (r9);

- the  ~~ operator has been moved  to a bit more senseful ranking

(r9);

- ASSUME   now  also  knows  the  6809's  DPR  register  and  its

implications (pardon, r9);

- the  6809  part  now  also  knows  the 6309's secret extensions

(r9);

- binary  constants now also may be  written in a C-like notation

(r9);

- version 1.41:

- the  new  predefined  symbol  MOMSEGMENT  allows to inquire the

currently active segment;

- := is now allowed as a short form for SET/EVAL;

- the  new command  line switch  q allows  to force  a ''silent''

assembly;

- the  key word PARENT  to reference the  parent section has been

extended by PARENT0..PARENT9;

- the  PowerPC  part  has  been  extended  by the microcontroller

versions MPC505 and PPC403;

- symbols  defined  with  SET  or  EQU  may  now be assigned to a

certain segment (r1);

- the  SH7000 part  now also  knows the  SH7600's extensions (and

should compute correct displacements...) (r1);

- the  65XX part now differentiates  between the 65C02 and 65SC02

(r1);

- additionally  to the symbol MOMCPU, there  is now also a string

symbol  MOMCPUNAME  that  contains  the  processor's  full name

(r1);

- P2HEX  now  also  knows  the  32-bit  variant  of the Intel hex

format (r1);

- additionally knows the 87C750's limitations (r2);

- the  internal numbers for  fatal errors have  been moved to the

area  starting  at  10000,  making  more space for normal error

messages (r2);

- unused  symbols are now marked with  a star in the symbol table

(r2);

- additionally supports the 29K family from AMD (r2);

- additionally supports the M16 family from Mitsubishi (r2);

- additionally supports the H8/500 family from Hitachi (r3);

- the  number of data bytes printed per  line by P2HEX can now be

modified (r3);

- the  number of the pass that  starts to output warnings created

by the r command line switch is now variable (r3);

- the  macro processor  now knows  a WHILE  statement that allows

to repeat a piece of code a variable number of times (r3);

- the  PAGE instruction now  also allows to  set the line with of

the assembly listing (r3);

- CPU  aliases may now be defined  to define new pseudo processor

devices (r3);

- additionally supports the MCS/251 family from Intel (r3);

- if  the cross  reference list  has been  enabled, the  place of

the  first  definition  is  given  for  double  definitions  of

symbols (r3);

- additionally   supports   the   TMS320C5x   family  from  Texas

Instruments   (implementation  done   by  Thomas   Sailer,  ETH

Zurich, r3);

- the  OS/2  version  should  now  also  correctly work with long

file  names.  If  one  doesn't  check  every s**t personally...

(r3);

- the  new pseudo instruction  BIGENDIAN now allows  to select in

MCS-51/251  mode  whether  constants  should  be  stored in big

endian or little endian format (r3);

- the   680x0  part  now  differentiates  between  the  full  and

reduced  MMU instruction set;  a manual toggle  can be done via

the FULLPMMU instruction (r3);

- the  new command line  option I allows  to print a  list of all

include files paired with their nesting level (r3);

- additionally supports the 68HC16 family from Motorola (r3);

- the  END statement now optionally  accepts an argument as entry

point for the program (r3);

- P2BIN  and P2HEX now allow to move  the contents of a code file

to a different address (r4);

- comments  appended to  a SHARED  instruction are  now copied to

the share file (r4);

- additionally supports the 68HC12 family from Motorola (r4);

- additionally supports the XA family from Philips (r4);

- additionally supports the 68HC08 family from Motorola (r4);

- additionally supports the AVR family from Atmel (r4);

- to  achieve  better  compatibility  to  the AS11 from Motorola,

the  pseudo  instructions  FCB,  FDB,  FCC,  and RMB were added

(r5);

- additionally supports the M16C from Mitsubishi (r5);

- additionally  supports  the  COP8  from  National Semiconductor

(r5);

- additional  instructions IFB and  IFNB for conditional assembly

(r5);

- the  new  EXITM  instruction  now  allows  to terminate a macro

expansion (r5);

- additionally supports the MSP430 from Texas Instruments (r5);

- LISTING   now  knows  the  additional  variants  NOSKIPPED  and

PURECODE  to  remove  code  that  was  not  assembled  from the

listing (r5);

- additionally supports the 78K0 family from NEC (r5);

- BIGENDIAN is now also available in PowerPC mode (r5);

- additional BINCLUDE instruction to include binary files (r5);

- additional   TOLOWER   and   LOWSTRING   functions  to  convert

characters to lower case (r5);

- it  is now possible to store  data in other segments than CODE.

The file format has been extended appropriately (r5);

- the  DS  instruction  to  reserve  memory  areas  is  now  also

available in Intel mode (r5);

- the  U command line switch now allows  to switch AS into a case

sensitive  mode  that  differentiates  between  upper and lower

case  in the names of  symbols, user-defined functions, macros,

macro parameters, and sections (r5);

- SFRB  now also  knows the  mapping rules  for bit  addresses in

the  RAM areas; warnings  are generated for  addresses that are

not bit addressable (r5);

- additional  instructions PUSHV  and POPV  to save symbol values

temporarily (r5);

- additional  functions  BITCNT,  FIRSTBIT,  LASTBIT,  and BITPOS

for bit processing (r5);

- the  68360  is  now  also  known  as  a  member  of  the  CPU32

processors (r5);

- additionally supports the ST9 family from SGS-Thomson (r6);

- additionally  supports  the  SC/MP  from National Semiconductor

(r6);

- additionally   supports   the   TMS70Cxx   family   from  Texas

Instruments (r6);

- additionally   supports   the   TMS9900   family   from   Texas

Instruments (r6);

- additionally  knows  the  80296's  instruction  set  extensions

(r6);

- the  supported number of Z8 derivatives has been extended (r6);

- additionally knows the 80C504's mask defects (r6);

- additional   register   definition   file   for  Siemens'  C50x

processors (r6);

- additionally supports the ST7 family from SGS-Thomson (r6);

- the  Tntel pseudo instructions  for data disposal  are now also

valid for the 65816/MELPS-7700 (r6);

- for  the 65816/MELPS-7700,  the address  length may  now be set

explicitly via prefixes (r6);

- additionally supports the 8X30x family from Signetics (r6);

- from  now on, PADDING is enabled  by default only for the 680x0

family (r7);

- the  new  predefined  symbol  ARCHITECTURE  can  now be used to

query the platform AS was compiled for (r7);

- additional  statements  STRUCT  and  ENDSTRUCT  to  define data

structures (r7);

- hex  and object  files for  the AVR  tools may now be generated

directly (r7);

- MOVEC now also knows the 68040's control registers (r7);

- additional  STRLEN  function  to  calculate  the  length  of  a

string (r7);

- additional  ability  to  define  register symbols (r7 currently

only Atmel AVR);

- additionally knows the 6502's undocumented instructions (r7);

- P2HEX  and  P2BIN  now  optionally  can  erase  the input files

automatically (r7);

- P2BIN  can  additionally  prepend  the  entry  address  to  the

resulting image (r7);

- additionally  supports the  ColdFire family  from Motorola as a

variation of the 680x0 core (r7);

- BYT/FCB,  ADR/FDB,  and  FCC  now  also  allow  the  repetition

factor known from DC (r7);

- additionally supports Motorola's M*Core (r7);

- the SH7000 part now also knows the SH7700's extensions (r7);

- the   680x0  part   now  also   knows  the  68040's  additional

instructions (r7);

- the  56K part now also knows  the instruction set extensions up

to the 56300 (r7).

- the   new  CODEPAGE  statement  now   allows  to  keep  several

character sets in parallel (r8);

- The argument variations for CHARSET have been extended (r8);

- New string functions SUBSTR and STRSTR (r8);

- additional IRPC statement in the macro processor (r8);

- additional   RADIX  statement  to  set  the  default  numbering

system for integer constants (r8);

- instead of ELSEIF, it is now valid to simply write ELSE (r8);

- == may be used as equality operator instead of = (r8);

- BRANCHEXT  for  the  Philips  XA  now  allows  to automatically

extend the reach of short branches (r8);

- debug output is now also possible in NoICE format (r8);

- additionally supports the i960 family from Intel (r8);

- additionally  supports the �PD7720/7725  signal processors from

NEC (r8);

- additionally  supports the  �PD77230 signal  processor from NEC

(r8);

- additionally   supports  the  SYM53C8xx  SCSI  processors  from

Symbios Logic (r8);

- additionally supports the 4004 from Intel (r8);

- additionally supports the SC14xxx series of National (r8);

- additionally  supports  the  instruction  extensions of the PPC

403GC (r8);

- additional  command line option  cpu to set  the default target

processor (r8);

- key  files now  also may  be referenced  from the  command line

(r8);

- additional  command  line  option  shareout  to  set the output

file for SHARED definitions (r8);

- new  statement  WRAPMODE  to  support  AVR  processors  with  a

shortened program counter (r8);

- additionally  supports the  C20x instruction  subset in the C5x

part (r8);

- hexadecimal  address specifications for the  tools now may also

be made in C notation (r8);

- the   numbering   system   for   integer   results   in  \{...}

expressions is now configurable via OUTRADIX (r8);

- the   register  syntax   for  4004   register  pairs  has  been

corrected (r8);

- additionally supports the F^2MC8L family from Fujitsu (r8);

- P2HEX  now  allows  to  set  the  minimum  address length for S

record addresses (r8);

- additionally supports the ACE family from Fairchild (r8);

-  REG is now also allowed for PowerPCs (r8);

- additional switch in P2HEX to relocate all addresses (r8);

- The  switch  x  now  additionally  allows  a  second  level  of

detailness to print the source line in question (r8).

- version 1.42:

- the default integer syntax for Atmel AVR is now the C Syntax;

- additional  command line  option olist  to set  the list file's

name and location;

- additionally supports the F^2MC16L family from Fujitsu;

- additional instruction PACKING for the AVR family;

- additional implicit macro parameters ALLARGS and ARGCOUNT;

- additional   instruction  SHIFT   to  process   variable  macro

argument lists;

- support for temporary symbols;

- additional  instruction  MAXNEST  to  set  the  maximum nesting

depth of macro expansions;

- additional  command  line  argument  noicemask  to  control the

amount of segments listed in a NoICE debug info file;

- additionally supports the 180x family from Intersil;

- additionally supports the 68HC11K4 address windowing;

- P2HEX  now allows to  vary the address  field length of AVR HEX

files;

- the  new command line option  -gnuerrors allows to output error

messages in a GNU C-style format;

- additionally   supports  the   TMS320C54x  family   from  Texas

Instruments;

- new macro option INTLABEL;

- added    Atmel   MegaAVR   8/16   instructions   and   register

definitions;

-  ENDIF/ENDCASE  show  the  line  number  of  the  corresponding

opening statement in the listing;

- the  8051 part now also supports  the extended address space of

the Dallas DS80C390;

- added nameless temporary smbols;

- additionally supports the undocumented 8085 instructions;

- improved structure handling;

- added EXPRTYPE() function;

- allow line continuation;

- integrated  support  for  KCPSM/PicoBlaze  provided  by Andreas

Wassatsch;

- additionally   supports   the   807x   family   from   National

Semiconductor;

- additionally supports the Intel 4040;

- additionally supports the Zilog eZ8;

- additionally supports the 78K2 family from NEC;

- additionally supports the KCPSM3 variant from Xilinx;

- additionally supports the LatticeMico8;

- additionally  supports the  12X instruction  extensions and the

XGATE core of the 68HC12 family;

- additionally supports the Signetics 2650;

- additionally   supports   the   COP4   family   from   National

Semiconductor;

- additionally supports the HCS08 extensions by Freesacle;

- additionally supports the RS08 family by Freescale;

- additionally supports the Intel 8008;

- add another optional syntax for integer constants;

- added function CHARFROMSTR;

- additionally allow Q for octal constants in Intel mode;

- add another variant for temporary symbols;

- the   PowerPC   part   has   been   extended   by   the  MPC821

(contribution by Marcin Cieslak);

- implicit macro parameters are always case-insensitive;

- add REG statement to MSP430;

- additionally supports the XMOS XS1;

- additional  parameters  GLOBALSYMBOLS  and  NOGLOBALSYMBOLS  to

control whether labels in macros are local or not;

- additionally supports the NEC 75xx series;

- additionally supports the TMS1000 controllers from TI;

- additionally supports the 78K2 family from NEC;

- all   newer  changes  are  only   documented  in  the  separate

changelog file.

J. Hints for the AS Source Code

===============================

As  I already mentioned in the  introduction, I release the source code

of  AS  on  request.  The  following  shall  give  a few hints to their

usage.

J.1. Language Preliminaries

---------------------------

In  the beginning, AS  was a program  written in Turbo-Pascal. This was

roughly  at  the  end  of  the  eighties,  and  there  were a couple of

reasons  for this  choice: First,  I was  much more  used to it than to

any  C compiler,  and compared  to Turbo  Pascal's IDE, all DOS-based C

compilers  were just crawling along. In  the beginning of 1997 however,

it  became clear that  things had changed:  One factor was that Borland

had  decided  to  let  its  confident  DOS developers down (once again,

explicitly  no 'thank you',  you boneheads from  Borland!) and replaced

version  7.0 of  Borland Pascal  with something  called 'Delphi', which

is  probably  a  wonderful  tool  to  develop  Windows  programs  which

consist  of  90%  user  interface  and  accidentaly  a  little  bit  of

content,  however completely  useless for  command-line driven programs

like  AS. Furthermore, my  focus of operating  systems had made a clear

move  towards Unix, and  I probably could  have waited arbitrarily long

for  a  Borland  Pascal  for  Linux  (to  all  those remarking now that

Borland  would be working  on something like  that: this is Vapourware,

don"t  believe them anything until you can  go into a shop and actually

buy it!). It was therefore clear that C was the way to go.

After  this eperience  what results  the usage  of 'island systems' may

have,   I  put  a   big  emphasize  on   portability  while  doing  the

translation  to  C;  however,  since  AS  for example deals with binary

data   in  an   exactly  format   and  uses  operating  systen-specific

functions  at some places  which may need  adaptions when one compliles

AS the first time for a new platform.

AS  is tailored for a C compiler  that conforms to the ANSI C standard;

C++  is  explicitly  not  required.  If  you are still using a compiler

conforming  to  the  outdated  Kernighan&Ritchie  standard,  you should

consider  getting a newer compiler: The  ANSI C standard has been fixed

in  1989 and there should be an  ANSI C compiler for every contemporary

platform,  maybe  by  using  the  old  compiler  to build GNU-C. Though

there  are some switches in the source  code to bring it nearer to K&R,

this   is  not  an  officially  supported  feature  which  I  only  use

internally  to support  a quite  antique Unix.  Everything left  to say

about K&R is located in the file README.KR.

The  inclusion of  some additional  features not  present in the Pascal

version   (e.g.  dynamically   loadable  message   files,  test  suite,

automatic  generation of the documentation  from one source format) has

made  the source  tree substantially  more complicated.  I will attempt

to unwire everything step by step:

J.2. Capsuling System dependencies

----------------------------------

As  I  already  mentioned,  As  has  been  tailored  to provide maximum

platform  independence  and  portability  (at  least  I believe so...).

This  means  packing  all  platform  dependencies  into as few files as

possible.  I will  describe these  files now,  and this  section is the

first one because it is probably one of the most important:

The  Build of all components of AS  takes place via a central Makefile.

To  make it work,  it has to  be accompanied by  a fitting Makefile.def

that  gives the  platform dependent  settings like  compiler flags. The

subdirectory  Makefile.def-samples contains  a couple  of includes that

work  for widespread platforms  (but which need  not be optimal...). In

case   your  platform  is  not  among  them,  you  may  take  the  file

Makefile.def.tmpl as a starting point (and send me the result!).

A   further  component  to  capure  system  dependencies  is  the  file

sysdefs.h.   Practically   all   compilers   predefine   a   couple  of

preprocessor  symbols that describe  the target processor  and the used

operating  system. For example,  on a Sun  Sparc under Solaris equipped

with  the  GNU  compiler,  the  symbols  __sparc  and __SVR4. sysdefs.h

exploits  these symbols  to provide  a homogeneous  environment for the

remaining,  system-independent files.  Especially, this  covers integer

datatypes   of  a  specific  length,  but   it  may  also  include  the

(re)definition  of C functions  which are not  present or non-standard-

like  on a specific platform. It's best  to read this files yourself if

you  like  to  know  which  things  may  occur... Generally, the #ifdef

statement  are  ordered  in  two  levels:  First,  a specific processor

platform  is  selected,  the  the  operating  systems are sorted out in

such a section.

If  you  port  AS  to  a  new  platform,  you  have to find two symbols

typical  for  this  platform  and  extend  sysdefs.h  accordingly. Once

again, I'm interested in the result...

J.3. System-Independent Files

-----------------------------

...represent   the  largest   part  of   all  modules.  Describing  all

functions  in detail  is beyond  the scope  of this  description (those

who  want  to  know  more  probably  start  studying  the  sources,  my

programming  style isn't that  horrible either...), which  is why I can

only give a short list at this place with all modules their function:

J.3.1. Modules Used by AS

- - - - - - - - - - - - -

as.c

....

This  file is  AS's root:  it contains  the main()  function of AS, the

processing  of all  command line  options, the  overall control  of all

passes and parts of the macro processor.

asmallg.c

.........

This   module  processes  all  statements  defined  for  all  processor

targets,  e.g.  EQU  and  ORG.  The  CPU pseudo-op used to switch among

different processor targets is also located here.

asmcode.c

.........

This  module contains the  bookkeping needed for  the code output file.

It  exports an interface that allows to  open and close a code file and

offers  functions to write code to (or  take it back from) the file. An

important  job of  this module  is to  buffer the  write process, which

speeds up execution by writing the code in larger blocks.

asmdebug.c

..........

AS  can  optionally  generate  debug  information  for other tools like

simulators  or debuggers, allowing  a backward reference  to the source

code.  They  get  collected  in  this  module  and  can be output after

assembly in one of several formats.

asmdef.c

........

This   modules  only   contains  declarations   of  constants  used  in

different places and global variables.

asmfnums.c

..........

AS  assigns  internally  assigns  incrementing  numbers  for  each used

source  file. These numbers are  used for quick referencing. Assignment

of  numbers and  the conversion  between names  and numbers takes place

here.

asmif.c

.......

Here  ara  ll  routines  located  controlling conditional assembly. The

most   important  exported  variable  is  a  flag  called  IfAsm  which

controls whether code generation is currently turned on or off.

asminclist.c

............

This  module holds the  definition of the  list stucture that allows AS

to print the nesting of include files to the assembly list file.

asmitree.c

..........

When  searching  for  the  mnemonic  used  in  a line of code, a simple

linear  comparison with  all available  machine instructions  (as it is

still  done  in  most  code  generators,  for reasons of simplicity and

laziness)  is  not  necessary  the  most  effective method. This module

defines  two  improved  structures  (binary  tree and hash table) which

provide  a  more  efficient  search  and  are  destined  to replace the

simple   linear   search   on   a  step-by-step  basis...priorities  as

needed...

asmmac.c

........

Routines  to store  and execute  macro constructs  are located  in this

module. The real macro processor is (as already mentioned) in as.c.

asmpars.c

.........

Here  we  really  go  into  the  innards: This module stores the symbol

tables  (global and local) in two  binary trees. Further more, there is

a  quite large procedure EvalExpression  which analyzes and evaluates a

(formula)  expression.  The  procedure  returns  the  result  (integer,

floating  point, or string)  in a varaint  record. However, to evaluate

expressions   during  code  generation,  one   should  better  use  the

functions       EvalIntExpression,       EvalFloatExpression,       and

EvalStringExpression.   Modifications  for  tha  esake  of  adding  new

target  processors are unnecessary  in this modules  and should be done

with  extreme  care,  since  you  are  touching  something  like  'AS's

roots'.

asmsub.c

........

This  module  collects  a  couple  of  commonly  used subroutines which

primarily deal with error handling and 'advanced' string processing.

bpemu.c

.......

As  already mentioned  at the  beginning, AS  originally was  a program

written  in  Borland  Pascal.  For  some  intrinsic  functions  of  the

compiler,  it was simpler to emulate those  than to touch all places in

the source code where they are used. Well...

chunks.c

........

This  module  defines  a  data  type  to  deal  with  a list of address

ranges.  This  functionality  is  needed  by  AS  for allocation lists;

furthermore, P2BIN and P2HEX use such lists to warn about overlaps.

cmdarg.c

........

This   module  implements   the  overall   mechanism  of  command  line

arguments.  It needs a  specification of allowed  arguments, splits the

command  line and  triggers the  appropriate callbacks.  In detail, the

mechanism includes the following:

- Processing  of arguments  located in  an environment  variable or a

corresponding file;

- Return  of a set  describing which command  line arguments have not

been processed;

- A  backdoor  for  situations  when  an  overlaying IDE converts the

passed command line completely into upper or lower case.

codepseudo.c

............

You  will find  at this  place pseudo  instructions that  are used by a

subset  of code generators.  On the one  hand, this is  the Intel group

of  DB..DO, and on the other hand  their counterparts for 8/16 bit CPUs

from  Motorola  or  Rockwell.  Someone  who  wants  to  extend  AS by a

processor  fitting into  one of  these groups  can get the biggest part

of the necessary pseudo instructions with one call to this module.

codevars.c

..........

For  reasons  of  memory  efficiency,  some  variables commonly used by

diverse code generators.

endian.c

........

Yet  another bit of machine dependence, however  one you do not have to

spend  attention  on:  This  module  automatically  checks  at  startup

whether  a host  machine is  little or  big endian. Furthermore, checks

are  made  if  the  type  definitions  made  for  integer  variables in

sysdefs.h really result in the correct lengths.

headids.c

.........

At  this place,  all processor  families supported  by AS are collected

with  their header IDs  (see chapter 5.1)  and the output  format to be

used  by default by  P2HEX. The target  of this table  is to centralize

the  addition of a new processor as  most as possible, i.e. in contrast

to  earlier versions  of AS,  no further  modifications of tool sources

are necessary.

ioerrs.c

........

The  conversion from  error numbers  to clear  text messages is located

here.  I hope I'll never hit a  system that does not define the numbers

as macros, because I would have to rewrite this module completely...

nlmessages.c

............

The  C version  of AS  reads all  messages from  files at runtime after

the  language to be used  is clear. The format  of message files is not

a  simple  one,  but  instead  a  special compact and preindexed format

that  is generated  at runtime  by a  program called 'rescomp' (we will

talk  about it later).  This module is  the counterpart to rescomp that

reads  the  correct  language  part  into  a character field and offers

functions to access the messages.

nls.c

.....

This  module  checks  which  country-dependent  settings (date and time

format,  country code) are  present at runtime.  Unfortunately, this is

a  highly  operating  system-dependend  task,  and currently, there are

only  three methods  defines: The  MS-DOS method,  the OS/2  method and

the  typical Unix method  via locale functions.  For all other systems,

there is unfortunately currently only NO_NLS available...

stdhandl.c

..........

On  the one hand, here  is a special open  function located knowing the

special  strings !0...!2 as  file names and  creating duplicates of the

standard  file handles  stdin, stdout,  and stderr.  On the other hand,

investiagations   are  done  whether  the   standard  output  has  been

redirected   to  a  device   or  a  file.   On  no-Unix  systems,  this

unfortunately also incorporates some special operations.

stringlists.c

.............

This  is  just  a  little  'hack'  that  defines  routines to deal with

linear  lists of strings, which are  needed e.g. in the macro processor

of AS.

strutil.c

.........

Some commonly needed string operations have found their home here.

version.c

.........

The  currently valid  version is  centrally stored  here for AS and all

other tools.

code????.c

..........

These  modules form the main part of  AS: each module contains the code

generator for a specific processor family.

J.3.2. Additional Modules for the Tools

- - - - - - - - - - - - - - - - - - - -

hex.c

.....

A  small  module  to  convert  integer  numbers to hexadecimal strings.

It's  not absolutely  needed in  C any  more (except for the conversion

of   long  long  variables,  which  unfortunately  not  all  printf()'s

support), but it somehow survived the porting from Pascal to C.

p2bin.c

.......

The sources of P2BIN.

p2hex.c

.......

The sources of P2HEX.

pbind.c

.......

The sources of BIND.

plist.c

.......

The sources of PLIST.

toolutils.c

...........

All  subroutines needed by  several tools are  collected here, e.g. for

reading of code files.

J.4. Modules Needed During the Build of AS

------------------------------------------

a2k.c

.....

This  is a minimal filter converting  ANSI C source files to Kernighan-

Ritchie  style. To  be exact:  only function  heads are converted, even

this  only when they  are roughly formatted  like my programming style.

Noone should therefore think this were a universal C parser!

addcr.c

.......

A  small  filter  needed  during  installation on DOS- or OS/2-systems.

Since  DOS  and  OS/2  use  a  CR/LF  for a newline, inc ontrast to the

single  LF of  Unix systems,  all assembly  include files provided with

AS are sent through this filter during assembly.

bincmp.c

........

For  DOS and OS/2, this module takes  the task of the cmp command, i.e.

the  binary comparison of  files during the  test run. While this would

principally  be possible  with the  comp command  provided with the OS,

bincmp  does not  have any  nasty interactive  questions (which seem to

be an adventure to get rid of...)

findhyphen.c

............

This  is the submodule  in tex2doc providing  hyphenation of words. The

algorithm used for this is shamelessly stolen from TeX.

grhyph.c

........

The definition of hyphenation rules for the german language.

rescomp.c

.........

This  is  AS's  'resource  compiler',  i.e.  the  tool  that converts a

readable file with string resources into a fast, indexed format.

tex2doc.c

.........

A  tool  that  converts  the  LaTeX  documentation  of AS into an ASCII

format.

tex2html.c

..........

A  tool  that  converts  the  LaTeX  documentation  of  AS into an HTML

document.

umlaut.c and unumlaut.c

.......................

These  tiny programs convert national  special characters between their

coding  in ISO8859-1 (all  AS files use  this format upon delivery) and

their  system-specific  coding.  Apart  from  a  plain  ASCII7 variant,

there are currently the IBM character sets 437 and 850.

ushyph.c

........

The definition of hyphenation rules for the english language.

J.5. Generation of Message Files

--------------------------------

As  already mentioned,  the C  source tree  of AS  uses a  dynamic load

principle  for all (error) messages. In  contrast to the Pasacl sources

where  all messages were  bundled in an  include file and compiled into

the  programs,  this  method  eliminates  the  need  to  provide  AS in

multiple  language  variants;  there  is  only one version which checks

for  the langugage to be used  upon runtime and loads the corresponding

component  from the message files. Just  to remind: Under DOS and OS/2,

the  COUNTRY  setting  is  queried,  while  under Unix, the environment

variables LC_MESSAGES, LC_ALL, and LANG are checked.

J.5.1. Format of the Source Files

- - - - - - - - - - - - - - - - -

A  source file for the message  compiler rescomp usually has the suffix

.res. The message compiler generates one or two files from a source:

- a binary file which is read at runtime by AS resp. its tools

- optionally  one further C header file  assigning an index number to

all  messages.  These  index  numbers  in combination with an index

table  in  the  binary  file  allow  a fast access to to individual

messages at runtime.

The  source file for the message compiler  is a pure ASCII file and can

therefore  be modified  with any  editor. It  consists of a sequence of

control  commands  with  parameters.  Empty  lines  and lines beginning

with  a semicolon  are ignored.  Inclusion of  other files  is possible

via the Include statement:

Include <Datei>

The  first two statements  in every source  file must be two statements

describing  the languages defined in  the following. The more important

one is Langs, e.g.:

Langs DE(049) EN(001,061)

describes  that two languages will be defined  in the rest of the file.

The  first one shall be used under  Unix when the language has been set

to  DE via environment variable. Similarly,  It shall be used under DOS

and  OS/2 when the country  code was set to  049. Similarly, the second

set  shall  be  used  for  the  settings  DE  resp.  061  or 001. While

multiple  'telephone  numbers'  may  point  to  a  single language, the

assignment  to  a  Unix  language  code is a one-to-one correspondence.

This  is  no  problem  in  practice  since the LANG variables Unix uses

describe subversions via appendices, e.g.:

de.de

de.ch

en.us

AS  only  compares  the  beginning  of  the strings and therefore still

comes  to  the  right  decision.  The  Default  statement  defines  the

language  that shall be used if either  no language has been set at all

or  a language is used  that is not mentioned  in the asrgument list of

Langs. This is typically the english language:

Default EN

These  definitions  are  followed  by  an  arbitrary  number of Message

statements, i.e. definitions of messages:

Message ErrName

": Fehler "

": error "

In  case  n  languages  were  announced  via  the  Langs statement, the

message  compiler takes  exactly the  following n  as the strings to be

stored.  It is therefore impossible to  leave out certain languages for

individual  messages, and  an empty  line following  the strings should

in  no way  be misunderstood  as an  end marker  for the list; inserted

lines  between statements  only serve  purposes of  better readability.

It  is  however  allowed  to  split individual messages across multiple

lines  in the source  file; all lines  except for the  last one have to

be ended with a backslash as continuation character:

Message TestMessage2

"Dies ist eine" \

"zweizeilige Nachricht"

"This is a" \

"two-line message"

As  already  mentioned,  source  files  are  pure ASCII files; national

special  characters may  be placed  in message  texts (and the compiler

will  correctly pass  them to  the resulting  file), a big disadvantage

however  is that such  a file is  not fully portable  any more: in case

it  is ported to  another system using  a different coding for national

special  characters, the  user will  probably be  confronted with funny

characters  at runtime...special characters  should therefore always be

written  via  special  sequences  borrowed  from  HTML  resp. SGML (see

table  J.1). Linefeeds can be  inserted into a line  via \n, similar to

C.

+--------------------------------------+------------------------+

| Sequence...                          | results in...          |

+--------------------------------------+------------------------+

+--------------------------------------+------------------------+

|  &auml; &ouml; &uuml;                | "a "o "u (Umlauts)     |

|  &Auml; &Ouml; &Uuml;                | "A "O "U               |

|  &szlig;                             | "s (sharp s)           |

|  &agrave; &egrave; &igrave; &ograve; | � � � �                |

|  &ugrave;                            | �                      |

|  &Agrave; &Egrave; &Igrave; &Ograve; | 'A � 'I 'O             |

|  &Ugrave;                            | 'U (Accent grave)      |

|  &aacute; &eacute; &iacute; &oacute; | � � � �                |

|  &uacute;                            | �                      |

|  &Aacute; &Eacute; &Iacute; &Oacute; | `A `E `I `O            |

|  &Uacute;                            | `U (Accent agiu)       |

|  &acirc; &ecirc; &icirc; &ocirc;     | � � � �                |

|  &ucirc;                             | �                      |

|  &Acirc; &Ecirc; &Icirc; &Ocirc;     | ^A ^E ^I ^O            |

|  &Ucirc;                             | ^U (Accent circonflex) |

|  &ccedil; &Ccedil;                   | � �(Cedilla)           |

|  &ntilde; &Ntilde;                   | � �                    |

|  &aring; &Aring;                     | � �                    |

|  &aelig; &Aelig;                     | � �                    |

|  &iquest; &iexcl;                    | inverted ! or ?        |

+--------------------------------------+------------------------+

Table J.1: Syntax for special character in rescomp

J.6. Creation of Documentation

------------------------------

A  source  distribution  of  AS  contains  this  documentation in LaTeX

format  only.  Other  formats  are  created from this one automatically

via  tools provided with  AS. One reason  is to reduce  the size of the

source   distribution,   another   reason   is   that  changes  in  the

documentation only have to be made once, avoiding inconsistencies.

LaTex  was  chosen  as  the  master  format because...because...because

it's  been  there  all  the  time  before.  Additionally, TeX is almost

arbitrarily  portable  and  fits  quite  well  to  the demands of AS. A

standard  distribution therefore  allows a  nice printout  on about any

printer;  for a  conversion to  an ASCII  file that  used to be part of

earlier    distributions,   the   converter    tex2doc   is   included,

additionally  the converter  tex2html allowing  to put  the manual into

the Web.

Generation of the documentation is started via a simple

make docs

The  two converters mentioned  are be built  first, then applied to the

TeX  documentation and  finally, LaTeX  itself is  called. All  this of

course for all languages...

J.7. Test Suite

---------------

Since  AS deals with  binary data of  a precisely defined structure, it

is  naturally sensitive for system  and compiler dependencies. To reach

at  least a  minimum amount  of secureness  that everything  went right

during   compilation,  a  set  of  test  sources  is  provided  in  the

subdirectory  tests that  allows to  test the  freshly built assembler.

These   programs  are   primarily  trimmed   to  find   faults  in  the

translation  of  the  machine  instruction  set,  which are commonplace

when  integer lenghts vary. Target-independent  features like the macro

processors  or conditional assembly  are only casually  tested, since I

assume that they work everywhere when they work for me...

The  test run is started  via a simple make  test. Each test program is

assembled,  converted to  a binary  file, and  compared to  a reference

image.  A test is considered to be  passed if and only if the reference

image  and  the  newly  generated  one  are  identical  on a bit-by-bit

basis.  At the  end of  the test,  the assembly  time for every test is

printed  (those  who  want  may  extend  the  file  BENCHES  with these

results),  accompanied with  a success  or failure  message. Track down

every  error that occurs, even  if it occurs in  a processor target you

are  never going to use!  It is always possible  that this points to an

error  that may also come up for  other targets, but by coincidence not

in the test cases.

J.8. Adding a New Target Processor

----------------------------------

The  probably most common reason to modify  the source code of AS is to

add  a new target  processor. Apart from  adding the new  module to the

Makefile,   there  are  few  places  in   other  modules  that  need  a

modification.  The new  module will  do the  rest by registering itself

in  the list of code generators. I  will describe the needed steps in a

cookbook style in the following sections:

Choosing the Processor's Name

.............................

The name chosen for the new processor has to fulfill two criterias:

1 The  name must not be  already in use by  another processor. If one

starts  AS without any  parameters, a list  of the names already in

use will be printed.

2 If  the name shall  appear completely in  the symbol MOMCPU, it may

not   contain  other  letters  than   A..F  (except  right  at  the

beginning).  The  variable  MOMCPUNAME  however  will always report

the  full name  during assembly.  Special characters  are generally

disallowed,   lowercase  letters  will  be  converted  by  the  CPU

command  to uppercase  letters and  are therefore  senseless in the

processor name.

The  first  step  for  registration  is  making  an  entry  for the new

processor  (family) in the  file headids.c. As  already mentioned, this

file  is also used by  the tools and specifies  the code ID assigned to

a  processor  family,  along  with  the  default  hex file format to be

used.  I  would  like  to  have  some  coordination before choosing the

ID...

Definition of the Code Generator Module

.......................................

The  unit's  name  that  shall  be  responsible  for  the new processor

should  bear at  least some  similarity to  the processor's  name (just

for  the  sake  of  uniformity)  and  should  be  named in the style of

code.....  The head with include statements  is best taken from another

existing code generator.

Except  for an  initialization function  that has  to be  called at the

begginning  of  the  main()  function  in  module  as.c, the new module

neither   has  to  export  variables  nor  functions  as  the  complete

communication  is done at  runtime via indirect  calls. They are simply

done  by a  call to  the function  AddCPU for  each processor type that

shall be treated by this unit:

CPUxxxx:=AddCPU('XXXX',SwitchTo_xxxx);

'XXXX'  is the name chosen  for the processor which  later must be used

in   assembler  programs  to  switch   AS  to  this  target  processor.

SwitchTo_xxxx  (abbreviated as the ''switcher''  in the following) is a

procedure  without parameters that  is called by  AS when the switch to

the  new  processor  actually  takes  place. AddCPU delivers an integer

value  as  result  that  serves  as  an internal ''handle'' for the new

processor.  The global  variable MomCPU  always contains  the handle of

the  target  processor  that  is  currently  set. The value returned by

AddCPU  should be stored  in a private  variable of type CPUVar (called

CPUxxxx  in  the  example  above).  In  case  a  code  generator module

implements  more  than  one  processor  (e.g.  several  processors of a

family),   the  module  can  find   out  which  instruction  subset  is

currently allowed by comparing MomCPU against the stored handles.

The  switcher's  task  is  to  ''reorganize''  AS  for  the  new target

processor.  This  is  done  by  changing  the  values of several global

variables:

- ValidSegs:  Not all processors  have all address  spaces defined by

AS.  This  set  defines  which  subset the SEGMENT instruction will

enable  for  the  currently  active  target processor. At least the

CODE  segment  has  to  be  enabled.  The  complete  set of allowed

segments   can  be   looked  up   the  file  fileformat.h  (Seg....

constants).

- SegInits:  This  array  stores  the  initial program counter values

for  the individual segments (i.e.  the values the program counters

will  initially take  when there  is no  ORG statement).  There are

only  a  few  exceptions  (like  logically separated address spaces

that  physically overlap)  which justify  other initial values than

0.

- Grans:  This array specifies  the size of  the smallest addressable

element  in bytes  for each  segment, i.e.  the size  of an element

that  increases an  address by  1. Most  processors need a value of

1,  even if  they are  16- or  32-bit processors,  but the PICs and

signal processors are cases where higher values are required.

- ListGrans:  This  array  specifies  the  size  of  byte groups that

shall  be shown in  the assembly listing.  For example, instruction

words  of  the  68000  are  always  2  bytes  long  though the code

segment's  granularity is  1. The  ListGran entry  therefore has to

be set to 2.

- SegLimits:  This  array  stores  the  highest  possible address for

each  segment, e.g.  65535 for  a 16-bit  address space. This array

need  not  be  filled  in  case  the  code generator takes over the

ChkPC method.

- ConstMode:  This  variable  may  take  the  values  ConstModeIntel,

ConstModeMoto,  or  ConstModeC  and  rules  which  syntax has to be

used to specify the base of integer constants.

- PCSymbol:  This variable  contains the  string an assembler program

may  use to to get the current  value of the program counter. Intel

processors for example usually use a dollar sign.

- TurnWords:  If the target processor  uses big-endian addressing and

one  of the fields  in ListGran is  larger than one,  set this flag

to true to get the correct byte order in the code output file.

- SetIsOccupied:  Some processors have a  SET machine instruction. If

this  callback is set  to a non-NULL  value, the code generator may

report  back  whether  SET  shall  not  be  interpreted  as  pseudo

instruction.  The  return  value  may  be  constant True or or e.g.

depend   on  the  number  of   argument  if  a  differentiation  is

possible.

- HeaderID:  This variable contains  the ID that  is used to mark the

current  processor  family  in  the  the  code output file (see the

description  of  the  code  format  described  by  AS).  I  urge to

contact  me  before  selecting  the  value  to  avoid  ambiguities.

Values  outside the  range of  $01..$7f should  be avoided  as they

are  reserved  for  special  purposes  (like  a future extension to

allow  linkable code). Even  though this value  is still hard-coded

in  most  code  generators,  the  preferred  method is now to fetch

this value from headids.h via FindFamilyByName.

- NOPCode:  There are  some situations  where AS  has to  fill unused

code   areas  with  NOP  statements.  This  variable  contains  the

machine code of the NOP statement.

- DivideChars:  This string  contains the  characters that  are valid

separation  characters  for  instruction  parameters.  Only extreme

exotics  like the DSP56 require something  else than a single comma

in this string.

- HasAttrs:  Some processors  like the  68k series additionally split

an  instruction into mnemonic  and attribute. If  the new processor

also  does something like that,  set this flag to  true and AS will

deliver  the instructions'  components readily  split in the string

variables  OpPart  and  AttrPart.  If  this  flag is however set to

false,  no splitting  will take  place and  the instruction will be

delivered  as a  single piece  in OpPart.  AttrPart will stay empty

in  this case.  One really  should set  this flag  to false  if the

target  processor does not have  attributes as one otherwise looses

the  opportunity to  use macros  with a  name containing dots (e.g.

to emulate other assemblers).

- AttrChars:  In case  HasAttrs is  true, this  string has to contain

all  characters that can  separate mnemonic and  attribute. In most

cases, this string only contains a single dot.

Do  not assume that any of these  variables has a predefined value; set

them all!!

Apart  from  these  variables,  some  function  pointers have to be set

that  form  the  link  form  AS  to  the  ''active''  parts of the code

generator:

- MakeCode:  This  routine  is  called  after  a source line has been

split  into mnemonic  and parameters.  The mnemonic  is stored into

the  variable OpPart,  and the  parameters can  be looked up in the

array  ArgStr. The  number of  arguments may  be read  from ArgCnt.

The  binary  code  has  to  be  stored into the array BAsmCode, its

length  into CodeLen. In  case the processor  is word oriented like

the   68000  (i.e.  the  ListGran   element  corresponding  to  the

currently  active  segment  is  2),  the  field  may  be  addressed

wordwise   via  WAsmCode.  There  is   also  DAsmCode  for  extreme

cases...  The code  length has  to be  given in units corresponding

to the current segment's granularity.

- SwitchFrom:   This  parameter-less   procedure  enables   the  code

generator  module to  do ''cleanups''  when AS  switches to another

target  processor. This  hook allows  e.g. to  free memory that has

been  allocated in the generator and that  is not needed as long as

the  generator is  not active.  It may  point to an empty procedure

in  the simplest case.  One example for  the usage of  this hook is

the  module CODE370 that builds  its instruction tables dynamically

and frees them again after usage.

- IsDef:  Some processors know additional  instructions that impose a

special  meaning on  a label  in the  first row  like EQU does. One

example  is the BIT instruction found  in an 8051 environment. This

function  has  to  return  TRUE  if  such  a special instruction is

present.  In the simplest case  (no such instructions), the routine

may return a constant FALSE.

Optionally,  the  code  generator  may  additionally  set the following

function pointers:

- ChkPC  :  Though  AS  internally  treats  all  program  counters as

either  32 or  64 bits,  most processors  use an address space that

is  much  smaller.  This  function  informs  AS whether the current

program  counter has exceeded  its allowed range.  This routine may

of  course be  much more  complicated in  case the target processor

has  more  than  one  address  space.  One  example  is  in  module

code16c8x.c.  In  case  everything  is  fine,  the  function has to

return  TRUE,  otherwise  FALSE.  The  code  generator  only has to

implement  this function if it did  not set up the array SegLimits.

This   may  e.g.  become  necessary   when  the  allowed  range  of

addresses in a segment is non-continuous.

- InternSymbol  : Some  processorcs, e.g.  such with  a register bank

in  their internal RAM, predefine  such 'registers' as symbols, and

it  wouldn't make much  sense to define  them in a separate include

file  with 256 or maybe  more EQUs. This hook  allows access to the

code  generator of AS: It obtains  an expression as an ASCII string

and  sets up  the passed  structure of  type TempResult accordingly

when  one of these 'built-in' symbols  is detected. The element Typ

has  to  be  set  to  TempNone  in  case  the  check failed. Errors

messages  from  this  routine  should  be  avoided  as unidentified

names  could signify ordinary  symbols (the parser  will check this

afterwards).  Be  extreme  careful  with  this routine as it allows

you to intervene into the parser's heart!

- DissectBit  :  In  case  the  target platform supports bit objects,

i.e.  objects that  pack both  a register  or memory  address and a

bit  position  into  one  integer  number,  this is the callback to

dissect  such a packed representation and  transform it back into a

source-code   like,  human-readable  form.   This  provides  better

readability of the listing.

- DissectReg   :  In  case  the  target  platform  supports  register

symbols,  this is the callback  that translates register number and

size  back to a source-code  like, human-readable form. Again, this

function is used for the listing.

- QualifyQuote  : This optional  callback allows to  define on a per-

platform  base situations  when a  single quotation  character does

not  lead in a character  string. An example for  this is the Z80's

alternate   register  bank,  which  is   written  as  AF',  or  the

hexadecimal   constant   syntax   H'...   used   on   some  Hitachi

processors.

By  the way:  People who  want to  become immortal  may add a copyright

string.  This is  done by  adding a  call to the procedure AddCopyright

in the module's initialization part (right next to the AddCPU calls):

AddCopyright(

"Intel 80986 code generator (C) 2010 Jim Bonehead");

The  string passed to  AddCopyright will be  printed upon program start

in addition to the standard message.

If  needed, the unit may also use  its initialization part to hook into

a  list of procedures that  are called prior to  each pass of assembly.

Such  a  need  for  example  arises  when  the module's code generation

depends   on   certain   flags   that   can   be  modified  via  pseudo

instructions.  An example  is a  processor that  can operate  in either

user   or  supervisor  mode.  In   user  mode,  some  instructions  are

disabled.  The flag that  tells AS whether  the following code executes

in  user  or  supervisor  mode  might  be  set  via  a  special  pseudo

instruction.  But  there  must  also  be an initialization that assures

that  all  passes  start  with  the  same  state.  The hook offered via

AddInitPassProc  offers  a  chance  to  do  such  initializations.  The

callback  function  passed  to  it  is  called  before  a  new  pass is

started.

The  function  chain  built  up  via  calls  to AddCleanUpProc operates

similar  to AddInitPassProc:  It enables  code generators  to do clean-

ups  after assembly (e.g. freeing of  literal tables). This makes sense

when   multiple  files  are  assembled  with   a  single  call  of  AS.

Otherwise,  one would risk  to have 'junk'  in tables from the previous

run. No module currently uses this feature.

Writing the Code Generator itself

.................................

Now  we finally reached the point  where your creativity is challenged:

It  is up to  you how you  manage to translate  mnemonic and parameters

into  a  sequence  of  machine  code.  The  symbol tables are of course

accessible  (via  the  formula  parser)  just  like everything exported

from  ASMSUB.  Some  general  rules  (take  them  as advises and not as

laws...):

- Try  to split the instruction set  into groups of instructions that

have  the same operand  syntax and that  differ only in  a few bits

of  their machine  code. For  example, one  can do all instructions

without parameters in a single table this way.

- Most  processors have a  fixed spectrum of  addressing modes. Place

the  parsing of an address expression  in a separate routine so you

an reuse the code.

- The  subroutine WrError defines  a lot of  possible error codes and

can  be easily extended. Use this! It  is no good to simply issue a

''syntax error'' on all error conditions!

Studying  other  existing  code  generators  should  also  prove  to be

helpful.

Modifications of Tools

......................

A  microscopic change to the tolls'  sources is still necessary, namely

to  the  routine  Granularity()  in  toolutils.c:  in  case  one of the

processor's  address  spaces  has  a  granularity  different  to 1, the

swich   statement  in  this  place   has  to  be  adapted  accordingly,

otherwise PLIST, P2BIN, and P2HEX start counting wrong...

J.9. Localization to a New Language

-----------------------------------

You  are interested in this topic? Wonderful!  This is an issue that is

often  neglected by other  programmers, especially when  they come from

the country on the other side of the big lake...

The  localization to a  new language can  be split into  two parts: the

adaption  of program  messages and  the translation  of the manual. The

latter  one  is  definitely  a  work  of  gigantic  size,  however, the

adaption  of program messages should  be a work doable  on two or three

weekends,  given that  one knows  both the  new and  one of the already

present  messages. Unfortunately, this translation  cannot be done on a

step-by-step  basis  because  the  resource  compiler  currently cannot

deal  with a  variable amount  of languages  for different messages, so

the slogan is 'all or nothing'.

The  first oeration is to add the  new language to header.res. The two-

letter-abbreviation  used for  this language  is best  fetched from the

nearest  Unix system  (in case  you don't  work on  one anyway...), the

international telephone prefix from a DOS manual.

When  this  is  complete,  one  can  rebuild all necessary parts with a

simple  make and obtains an assembler  that supports one more language.

Do  not forget to forward  the results to me.  This way, all users will

benefit from this with the next release :-)

Bibliography

============

[ 1] Steve Williams:

68030 Assembly Language Reference.

Addison-Wesley, Reading, Massachusetts, 1989

[ 2] Advanced Micro Devices:

AM29240, AM29245, and AM29243 RISC Microcontrollers.

1993

[ 3] Apple Corporation::

6502 Floating Point Routines.

Apple II Reference Manual (Red Book), January 1978, pages 94-95

http://www.6502.org/source/floats/wozfp3.txt

[ 4] Atmel Corp.:

AVR Enhanced RISC Microcontroller Data Book.

May 1996

[ 5] Atmel Corp.:

8-Bit   AVR   Assembler   and   Simulator   Object   File  Formats

(Preliminary).

(part of the AVR tools documentation)

[ 6] Commodore Semiconductor Group:

65CE02 Microprocessor Preliminary Data Sheet.

[ 7] CMD Microcircuits:

G65SC802/G65SC816 CMOS 8/16-Bit Microprocessor.

Family Data Sheet.

[ 8] Freescale Semiconductor:

Digital  Signal Processing  Libraries Using  the ColdFire eMAC and

MAC User's Manual. DSPLIBUM, Rev. 1.2, 03/2006

[ 9] National Semiconductor:

COP410L/COP411L/COP310L/COP311L        Single-Chip       N-Channel

Microcontrollers. RRD-B30M105, March 1992

[10] National Semiconductor:

COPS Family User's Guide.

[11] General Instrument Microelectronics:

Series 1600 Microprocessor System Documentation..

S16DOC-CP-1600-04, May 1975

http://www.bitsavers.org/components/gi/CP1600/

CP-1600_Microprocessor_Users_Manual_May75.pdf

[12] Digital Research:

CP/M 68K Operating System User's Guide.

1983

[13] Cyrix Corp.:

FasMath 83D87 User's Manual.

1990

[14] Dallas Semiconductor:

DS80C320 High-Speed Micro User's Guide.

Version 1.30, 1/94

[15] Fairchild Semiconductor:

ACE1101 Data Sheet.

Preliminary, May 1999

[16] Fairchild Semiconductor:

ACE1202 Data Sheet.

Preliminary, May 1999

[17] Fairchild Semiconductor:

ACEx  Guide  to  Developer  Tools.  AN-8004, Version 1.3 September

1998

[18] Fairchild Micro Systems:

F8 User's Guide. 67095665, 02-13-1976

[19] Fairchild Micro Systems:

F8 Guide to Programming 67095664, 1976

[20] Freescale Semiconductor:

S12XCPUV1 Reference Manual. S12XCPUV1, v01.01, 03/2005

[21] Freescale Semiconductor:

RS08 Core Reference Manual. RS08RM, Rev. 1.0, 04/2006

[22] Freescale Semiconductor:

MC9S12XDP512 Data Sheet. MC9S12XDP512, Rev. 2.11, 5/2005

[23] Fujitsu Limited:

June 1998 Semiconductor Data Book.

CD00-00981-1E

[24] Fujitsu Semiconductor:

F�MC16LX   16-Bit   Microcontroller   MB90500  Series  Programming

Manual.

CM44-00201-1E, 1998

[25]  CPU Comparison with Z80.

https://gbdev.io/pandocs/CPU_Comparison_with_Z80.html

[26] Hitachi Ltd.:

8-/16-Bit Microprocessor Data Book.

1986

[27] Trevor J.Terrel & Robert J. Simpson:

Understanding HD6301X/03X CMOS Microprocessor Systems.

published by Hitachi

[28] Hitachi Microcomputer:

H8/300H Series Programming Manual.

(21-032, no year of release given)

[29] Hitachi America, Ltd.:

HD641016 User's Manual.

ADE-602-003A, September 1989

[30]  HuC6280 CMOS 8-bit Microprocessor Software Manual.

[31] Rockwell:

R65C19 Microcomputer Data Sheet.

Document Number 29400N10, January 1992

[32] Hitachi Semiconductor Design & Development Center:

SH Microcomputer Hardware Manual (Preliminary).

[33] Hitachi Semiconductor and IC Div.:

SH7700 Series Programming Manual.

1st Edition, September 1995

[34] Hitachi  America Ltd.:  HMCS400 Series  Handbook: Users Manual AD-

E00078, March 1988

[35] Hitachi Semiconductor and IC Div.:

H8/500 Series Programming Manual.

(21-20, 1st Edition Feb. 1989)

[36] Hitachi Ltd.:

H8/532 Hardware Manual.

(21-30, no year of release given)

[37] Hitachi Ltd.:

H8/534,H8/536 Hardware Manual.

(21-19A, no year of release given)

[38] Hewlett Packard:

Nano Processor's Users Guide.

Drawing Number A-5955-0331-1

https://www.hp9845.net/9845/downloads/manuals/Nanoprocessor.pdf

[39] IBM Corp.:

PPC403GA Embedded Controller User's Manual.

First Edition, September 1994

[40] IBM Corp.:

IBM5100 ISA Opcodes Micro Instruction Set.

https://github.com/voidstar78/IBM_5100_DOCS/blob/main/

PDFs_5100/IBM5100_ISA_opcodes_micro-instruction_set.pdf

[41] IBM Corp.:

IBM5110 BASIC Reference Manual.

SA21-9306-0, First Edition, January 1978

http://www.bitsavers.org/pdf/ibm/5110/

SA21-9308-0_IBM_5110_BASIC_Reference_Manual_Jan1978.pdf

[42] IBM Corp.:

IBM5100 Maintenance Information Manual.

SY31-0405-3, October 1979

[43] Dokumentation zum IBM5110:

http://computermuseum.informatik.uni-stuttgart.de/dev/

ibm_5110/technik/5110.html

[44] Intel Corp.:

Embedded Controller Handbook.

1987

[45] Intel Corp.:

Microprocessor and Peripheral Handbook, Volume I Microprocessor.

1988

[46] Intel Corp. :

MCS-48 Family of Single Chip Microcomputers User's Manual.

September 1980

[47] Intel Corp. :

80960SA/SB Reference Manual.

1991

[48] Intel Corp.:

8XC196NT Microcontroller User's Manual.

June 1995

[49] Intel Corp.:

8XC251SB High Performance CHMOS Single-Chip Microcontroller.

Sept. 1995, Order Number 272616-003

[50] Intel Corp.:

80296SA Microcontroller User's Manual.

Sept. 1996

[51] Intel Corp.:

4040: Single-Chip 4-Bit P-Channel Microprocessor.

(no year of release given)

[52] Intersil:

CDP1802A, CDP1802AC, CDP1802BC CMOS 8-Bit Microprocessors.

March 1997

[53] : RCA Inc.: CDP1804, CDP1804C Types Objective Data.

(no year of release given)

[54] Intersil:

CDP1805AC,  CDP1806AC CMOS  8-Bit Microprocessor  with On-Chip RAM

and Counter/Timer.

March 1997

[55] Intersil:

IM6100 CMOS 12 Bit Microprocessor.

(no year of release given)

[56] Hirotsugu Kakugawa:

A memo on the secret features of 6309.

(available via World Wide Web:

http://www.cs.umd.edu/users/fms/comp/CPUs/6309.txt)

[57] KENBAK:

Programming Reference Manual KENBAK-1 Computer.

4/1/1971

[58] Lattice Semiconductor Corporation:

LatticeMico8 Microcontroller Users Guide.

Reference Design RD1026, February 2008

[59] Microchip Technology Inc.:

Microchip Data Book.

1993 Edition

[60] US Department Of Defense:

Military    Standard   Sixteen-Bit    Computer   Instruction   Set

Architecture.

MIL-STD-1750A (USAF), 2 July 1980

[61] Mitsubishi Electric:

Single-Chip 8-Bit Microcomputers.

Vol.2, 1987

[62] Mitsubishi Electric:

Single-Chip 16-Bit Microcomputers.

Enlarged edition, 1991

[63] Mitsubishi Electric:

Single-Chip 8 Bit Microcomputers.

Vol.2, 1992

[64] Mitsubishi Electric:

M34550Mx-XXXFP Users's Manual.

Jan. 1994

[65] Mitsubishi Electric:

7751 Series Software Manual.

Rev. 1.01, 980731

[66] Mitsubishi Electric:

M16 Family Software Manual.

First Edition, Sept. 1994

[67] Mitsubishi Electric:

M16C Software Manual.

First Edition, Rev. C, 1996

[68] Mitsubishi Electric:

M30600-XXXFP Data Sheet.

First Edition, April 1996

[69] documentation  about  the  M16/M32-developer's  package from Green

Hills Software

[70] Mostek Corporation:

Circuits and Systems Product Guide.

1980, STD No 01009

[71] Mostek Corporation:

3870/F8 Microcomputer Data Book.

1981, Publication Number MK79602

[72] Motorola Inc.:

Microprocessor, Microcontroller and Peripheral Data.

Vol. I+II, 1988

[73] Motorola Inc.:

MC68881/882 Floating Point Coprocessor User's Manual.

Second Edition, Prentice-Hall, Englewood Cliffs 1989

[74] Motorola Inc.:

MC68851 Paged Memory Management Unit User's Manual.

Second Edition, Prentice-Hall, Englewood Cliffs 1989,1988

[75] Motorola Inc.:

CPU32 Reference Manual.

Rev. 1, 1990

[76] Motorola Inc.:

DSP56000/DSP56001 Digital Signal Processor User's Manual.

Rev. 2, 1990

[77] Motorola Inc.:

MC68340 Technical Summary.

Rev. 2, 1991

[78] Motorola Inc.:

CPU16 Reference Manual.

Rev. 1, 1991

[79] Motorola Inc.:

Motorola M68000 Family Programmer's Reference Manual.

1992

[80] Motorola Inc.:

MC68332 Technical Summary.

Rev. 2, 1993

[81] Motorola Inc.:

PowerPC 601 RISC Microprocessor User's Manual.

1993

[82] Motorola Inc.:

PowerPC(tm) MPC505 RISC Microcontroller Technical Summary.

1994

[83] Motorola Inc.:

PowerPC(tm) MPC821 Portable Microprocessor User's Manual.

1996

[84] Motorola Inc.:

CPU12 Reference Manual.

1st edition, 1996

[85] Motorola Inc.:

CPU08 Reference Manual.

Rev. 1 (no year of release given im PDF-File)

[86] Motorola Inc.:

MC68360 User's Manual.

[87] Motorola Inc.:

MCF 5200 ColdFire Family Programmer's Reference Manual.

1995

[88] Motorola Inc.:

M*Core Programmer's Reference Manual.

1997

[89] Motorola Inc.:

DSP56300 24-Bit Digital Signal Processor Family Manual.

Rev. 0 (no year of release given im PDF-File)

[90] Motorola Inc.:

MC68HC11K4 Technical Data. 1992

[91] National Semiconductor:

SC/MP Programmier- und Assembler-Handbuch.

Publication Number 4200094A, Aug. 1976

[92] National Semiconductor:

COP800 Assembler/Linker/Librarian User's Manual.

Customer Order Number COP8-ASMLNK-MAN

NSC Publication Number 424421632-001B

August 1993

[93] National Semiconductor:

COP87L84BC       microCMOS       One-Time-Programmable       (OTP)

Microcontroller.

Preliminary, March 1996

[94] National Semiconductor:

IMP-16 Programming and Assembler Manual.

Order No. IMP-16S/102YB, Pub. No 4200002B, November 1973

[95] National Semiconductor:

IMP-16L Users Manual.

Order No. IMP-16L/928X, Pub. No 4200028X, April 1974

[96] National Semiconductor:

IPC-16A/520D MOS/LSI Single Chip Microprocessor (PACE).

[97] National Semiconductor:

PACE Microprocessor Assembly Language Programming Manual.

Order No. IPC-16S/969Y, Pub. No 4200130A, January 1977

[98] National Semiconductor:

INS8900 Single-Chip 16-Bit N-Channel Microprocessor.

1977

[99] National Semiconductor:

SC14xxx DIP commands Reference guide.

Application Note AN-D-031, Version 0.4, 12-28-1998

[100] National Semiconductor:

INS8070-Series Microprocessor Family. October 1980

[101] National Semiconductor:

Series 32000 Databook.

1986

[102] National Semiconductor:

NS32CG16-10/NS32CG16-15     High    Performance    Printer/Display

Processor.

Preliminary, November 1995

[103] National Semiconductor and Udo M�ller:

Series 32000 Instruction Set Manual.

6. September 2017, available via www.cpu.ns32k.net

[104] NEC Corp.:

�pD70108/�pD70116/�pD70208/�pD70216/�pD72091 Data Book.

(no year of release given)

[105] NEC Electronics Europe GmbH:

User's Manual �COM-87 AD Family.

(no year of release given)

[106] NEC Corp.:

�COM-75x Family 4-bit CMOS Microcomputer User's Manual.

Vol. I+II (no year of release given)

[107] NEC Corp.:

78K/II  Series 8-Bit  Single-Chip Microcontroller  User's Manual -

Instructions.

Document No. U10228EJ6V0UM00 (6th edition), December 1995

[108] NEC Corp.:

uPD78310/312CW/G 8 Bit CMOS Microcomputer Product Description.

[109] NEC Corp.:

Digital Signal Processor Product Description.

PDDSP.....067V20 (no year of release given)

[110] NEC Corp.:

�PD78070A,   78070AY  8-Bit   Single-Chip  Microcontroller  User's

Manual.

Document No. U10200EJ1V0UM00 (1st edition), August 1995

[111] NEC Corp.:

Data Sheet �PD78014.

[112] NEC Corp.:

�PD70616 Programmer's Reference Manual.

Preliminary Information, November 1986

[113] NEC Corp.:

�COM-43 Single Chip Microcomputer Users' Manual.

January 1978

[114] NXP/Freescale:

CPU S12Z Reference Manual.

CPUS12ZRM, Rev. 1.01, 01/2013

[115] NXP:

MC9S12ZVC-Family Reference Manual and Datasheet.

MC9S12ZVCRMV1, Rev. 1.9, 29-January-2018

[116] Parallax Inc.

SX20AC/SX28AC Data Sheet. Revision 1.7, 4/23/2008

[117] Digital Equipment Corporation:

PDP-11 Architecture Handbook. 1983 Edition, EB-23657-18

[118] Digital Equipment Corporation:

PDP-11  MACRO-11 Language Reference  Manual. March 1983, AA-V027A-

TC

[119] OKI Semiconductor:

Microcontroller Data Book. Second Edition, December 1986

[120] Philips Semiconductor:

MAB84X1, MAF84X1, MAF84AX1 Family Datasheet.

August 1990

[121] Philips Semiconductor:

16-bit 80C51XA Microcontrollers (eXtended Architecture).

Data Handbook IC25, 1996

[122] Rockwell International Corporation:

Parallel  Processing  System  (PPS-4)  Microcomputer  Data  Sheet.

Document No. 29003 N11, 1973

[123] RX Family Product Page,

https://www.renesas.com/us/en/products/

microcontrollers-microprocessors/

rx-32-bit-performance-efficiency-mcus

[124] RX Family Compiler and Assembler Manual:

https://www.renesas.com/us/en/document/mat/

high-performance-embedded-workshop-rx-family-

cc-compiler-package-v101-users-manual-includes-v102

[125] Sharp:

Pocket Computer PC-1350 Machine Language Reference Manual.

https://pockemul.com/wp-content/uploads/2022/12/

PC1350_ML_EN.pdf

[126] Sharp Corporation:

ESR-L Instruction Manual.

https://web.archive.org/web/20060521205349if_/http://

www.sarnau.info:80/files/ESR-LInstructionManual.pdf

[127] Signetics Corporation:

Signetics 2650 Microprocessor.

1975

[128] Signetics Corporation:

2650 User's Manual.

[129] SGS-Ates:

M3870 Microcomputer Family Databook.

1st edition, issued January 1983

[130] SGS-Thomson Microelectronics:

8 Bit MCU Families EF6801/04/05 Databook.

1st edition, 1989

[131] SGS-Thomson Microelectronics:

ST6210/ST6215/ST6220/ST6225 Databook.

1st edition, 1991

[132] SGS-Ates: M380 uP System and Instruction Set.

6.4.1976

[133] SGS-Thomson Microelectronics:

ST7 Family Programming Manual.

June 1995

[134] SGS-Thomson Microelectronics:

ST9 Programming Manual.

3rd edition, 1993

[135] Siemens AG:

SAB80C166/83C166 User's Manual.

Edition 6.90

[136] Siemens AG:

SAB C167 Preliminary User's Manual.

Revision 1.0, July 1992

[137] Siemens AG:

Telephone Controller (Single-Chip 8-Bit

CMOS Microcontroller) SAB80C382/SAB80C482.

May 1987

[138] Siemens AG:

SAB-C502 8-Bit Single-Chip Microcontroller User's Manual.

Edition 8.94

[139] Siemens AG:

SAB-C501 8-Bit Single-Chip Microcontroller User's Manual.

Edition 2.96

[140] Siemens AG:

C504 8-Bit CMOS Microcontroller User's Manual.

Edition 5.96

[141] ST Microelectronics:

STM8 CPU Programming Manual.

PM0044, Doc ID 13590 Rev 3, September 2011

[142] ST Microelectronics:

STM8S  Series and  STM8AF Series  8-bit Microcontrollers Reference

Manual.

RM0016, DocID14587 Rev 14, October 2017

[143] ST Microelectronics:

STM8S003F3 STM8S003K3 Data Sheet.

DS7147 Rev 10, August 2018

[144] C.Vieillefond:

Programmierung des 68000.

Sybex-Verlag D�sseldorf, 1985

[145] Symbios Logic Inc:

Symbios Logic PCI-SCSI-I/O Processors Programming Guide.

Version 2.0, 1995/96

[146] Texas Instruments:

Model   990  Computer/TMS9900   Microprocessor  Assembly  Language

Programmer's Guide.

1977, Manual No. 943441-9701

[147] Texas Instruments:

TMS9995 16-Bit Microcomputer.

Preliminary Data Manual 1981

[148] Texas Instruments:

TMS99105 and TMS99110A 16-Bit Microprocessors.

Preliminary Data Manual 1982

[149] Texas Instruments:

First-Generation TMS320 User's Guide.

1988, ISBN 2-86886-024-9

[150] Texas Instruments:

TMS7000 Family Data Manual.

1991, DB103

[151] Texas Instruments:

TMS320C3x User's Guide.

Revision E, 1991

[152] Texas Instruments:

TMS320C2x User's Guide.

Revision C, Jan. 1993

[153] Texas Instruments:

TMS320C4x User's Guide.

SPRU063C, May 1999

[154] Texas Instruments:

TMS370 Family Data Manual.

1994, SPNS014B

[155] Texas Instruments:

MSP430 Family Software User's Guide.

1994, SLAUE11

[156] Texas Instruments:

MSP430 Metering Application.

1996, SLAAE10A

[157] Texas Instruments:

MSP430 Family Architecture User's Guide.

1995, SLAUE10A

[158] Texas Instruments:

MSP430 MSP430x5xx and MSP430x6xx Family User's Guide.

October 2016, SLAU208

[159] Texas Instruments:

TMS320C62xx CPU and Instruction Set Reference Manual.

Jan. 1997, SPRU189A

[160] Texas Instruments:

TMS320C20x User's Guide.

April 1999, SPRU127C

[161] Texas Instruments:

TMS320C54x DSP Reference Set; Volume 1: CPU and Peripherals.

March 2001, SPRU172C

[162] Texas Instruments:

TMS320C54x DSP; Volume 2: Mnemonic Instruction Set.

March 2001, SPRU172C

[163] Texas Instruments:

TMS  1000  Series  MOS/LSI  One-Chip  Microcomputers  Programmer's

Reference Manual.

CM122-1 1275, 1975

[164] Toshiba Corp.:

8-Bit Microcontroller TLCS-90 Development System Manual.

1990

[165] Toshiba Corp.:

8-Bit Microcontroller TLCS-870 Series Data Book.

1992

[166] Toshiba Corp.:

16-Bit Microcontroller TLCS-900 Series Users Manual.

1992

[167] Toshiba Corp.:

16-Bit Microcontroller TLCS-900 Series Data Book:

TMP93CM40F/ TMP93CM41F.

1993

[168] Toshiba Corp.:

4-Bit   Microcontroller  TLCS-47E/47/470/470A  Development  System

Manual.

1993

[169] Toshiba Corp.:

TLCS-9000/16 Instruction Set Manual Version 2.2.

10. Feb 1994

[170] Toshiba Corp.:

TC9331 Digital Audio Signal Processor Application Information.

[171] Valvo GmbH:

Bipolare Mikroprozessoren und bipolare LSI-Schaltungen.

Datenbuch, 1985, ISBN 3-87095-186-9

[172] Digital Equipment Corporation:

VAX Architecture Reference Manual.

Order Number EY-3459E-DP, 1987

[173] Western Digital Corporation:

WD16 Microcomputer Programmer's Reference Manual.

4 October 1976

[174] Ken Chapman (Xilinx Inc.):

PicoBlaze  8-Bit Microcontroller  for Virtex-E  and Spartan-II/IIE

Devices.

Application Note XAPP213, Version 2.1, February 2003

[175] Xilinx Inc.:

PicoBlaze  8-bit Embedded Microcontroller  User Guide for Spartan-

3, Virtex-II, and Virtex-II Pro FPGAs.

UG129 (v1.1) June 10, 2004

[176] David May::

The XMOS XS1 Architecture.

Publication Date: 2009/10/19, Copyright 2009 XMOS Ltd.

[177] data sheets from Zilog about the Z80 family

[178] Zilog Inc.:

Z8 Microcontrollers Databook.

1992

[179] Zilog Inc.:

Discrete Z8 Microcontrollers Databook.

(no year of release given)

[180] Zilog Inc.:

Z380 CPU Central Processing Unit User's Manual.

(no year of release given)

[181] Zilog Inc.:

eZ8 CPU User Manual.

UM01285-0503

[182] Zilog Inc.:

Z88C00 CMOS Super8 ROMless MCU Product Specification

2003, PS014602-0103

[183] Zilog Inc.:

Z8 Encore! F0830 Series Product Specification

2012, PS025113-1212

[184] Zilog Inc.:

Z8000 Technical Manual

January 1983

[185] Zilog Inc.:

eZ80 CPU User Manual

2015, UM007715-0415

[186] Zilog Inc.:

eZ80190 Product Specification

2008, PS006614-1208

[187] Zilog Inc.:

eZ80F91 MCU Product Specification

2022, PS019217-1222

[188] Zilog Inc.:

eZ80F92/eZ80F93 Product Specification

2020, PS015317-0120

[189] Zilog Inc.:

eZ80L92 MCU Product Specification

2016, PS013015-0316