Alfred Arnold, Stefan Hilse, Stephan Kanthak, Oliver Sellke, Vittorio

De Tomasi

Macro Assembler AS V1.42

--------------------------------------------

User's Manual

Edition April 2025

IBM,  PPC403Gx, OS/2, and PowerPC are registered trademarks of IBM Cor-

poration.

Intel,  MCS-48, MCS-51, MCS-251, MCS-96, MCS-196 und MCS-296 are regis-

tered trademarks of Intel Corp. .

Motorola and ColdFire are registered trademarks of Motorola Inc. .

MagniV is a registered trademark of Freescale Semiconductor.

PicoBlaze is a registered trademark of Xilinx Inc.

eZ80 and Z80 are registered trademarks of Zilog Inc.

UNIX is a registered trademark of the The Open Group.

Linux is a registered trademark of Linus Thorvalds.

Microsoft,  Windows, and MS-DOS are  registered trademarks of Microsoft

Corporation.

All  other  trademarks  not  explicitly  mentioned  in this section and

used in this manual are properties of their respective owners.

This  document has  been processed  with the  LaTeX typesetting system,

using the Linux operating system.

Contents

1. Introduction

1.1. License Agreement

1.2. General Capabilities of the Assembler

1.3. Supported Platforms

2. Assembler Usage

2.1. Hardware Requirements

2.2. Delivery

2.3. Installation

2.4. Start-Up Command, Parameters

2.5. Format of the Input Files

2.6. Format of the Listing

2.7. Symbol Conventions

2.8. Temporary Symbols

2.9. Named Temporary Symbols

2.9.1. Nameless Temporary Symbols

2.9.2. Composed Temporary Symbols

2.10. Formula Expressions

2.10.1. Integer Constants

2.10.2. Floating Point Constants

2.10.3. String Constants

2.10.4. String to Integer Conversion and Character Constants

2.10.5. Evaluation

2.10.6. Operators

2.10.7. Functions

2.11. Forward References and Other Disasters

2.12. Register Symbols

2.13. Share File

2.14. Processor Aliases

3. Pseudo Instructions

3.1. Definitions

3.1.1. SET, EQU, and CONSTANT

3.1.2. SFR and SFRB

3.1.3. XSFR and YSFR

3.1.4. LABEL

3.1.5. BIT

3.1.6. DBIT

3.1.7. DEFBIT and DEFBITB

3.1.8. DEFBITFIELD

3.1.9. PORT

3.1.10. REG and NAMEREG

3.1.11. LIV and RIV

3.1.12. CHARSET

3.1.13. CODEPAGE

3.1.14. ENUM, NEXTENUM, and ENUMCONF

3.1.15. PUSHV and POPV

3.2. Code Modification

3.2.1. ORG

3.2.2. RORG

3.2.3. CPU

3.2.4. SUPMODE, FPU, PMMU, CUSTOM

3.2.5. ACCMODE

3.2.6. CIS, EIS, FIS and FP11

3.2.7. FULLPMMU

3.2.8. PADDING

3.2.9. PACKING

3.2.10. WARNRELATIVE

3.2.11. MAXMODE

3.2.12. EXTMODE and LWORDMODE

3.2.13. SRCMODE

3.2.14. PLAINBASE

3.2.15. BIGENDIAN

3.2.16. WRAPMODE

3.2.17. PANEL

3.2.18. SEGMENT

3.2.19. PHASE and DEPHASE

3.2.20. SAVE and RESTORE

3.2.21. ASSUME

3.2.22. CKPT

3.2.23. EMULATED

3.2.24. BRANCHEXT

3.2.25. Z80SYNTAX

3.2.26. EXPECT and ENDEXPECT

3.3. Data Definitions

3.3.1. DC[.Size]

3.3.2. DS[.Size]

3.3.3. BLKB, BLKW, BLKL, BLKD

3.3.4. DN,DB,DW,DD,DQ,DT, and DO

3.3.5. FLT2, FLT3, FLT4

3.3.6. x_FLOATING

3.3.7. DS, DS8

3.3.8. BLKx

3.3.9. BYT or FCB

3.3.10. BYTE

3.3.11. DC8

3.3.12. ADR or FDB

3.3.13. DDB

3.3.14. DCM

3.3.15. WORD

3.3.16. DW16

3.3.17. ACON

3.3.18. LONG

3.3.19. SINGLE, DOUBLE, and EXTENDED

3.3.20. FLOAT and DOUBLE

3.3.21. SINGLE and DOUBLE

3.3.22. EFLOAT, BFLOAT, and TFLOAT

3.3.23. Qxx and LQxx

3.3.24. DATA

3.3.25. ZERO, CP-1600

3.3.26. FB and FW

3.3.27. ASCII, ASCIC, and ASCIZ

3.3.28. STRING and RSTRING

3.3.29. PACKED

3.3.30. RADIX50

3.3.31. FCC

3.3.32. TEXT

3.3.33. DFS or RMB

3.3.34. BLOCK

3.3.35. SPACE

3.3.36. RES

3.3.37. BSS

3.3.38. DSB and DSW

3.3.39. DS16

3.3.40. ALIGN

3.3.41. LTORG

3.4. Macro Instructions

3.4.1. MACRO

3.4.2. IRP

3.4.3. IRPC

3.4.4. REPT

3.4.5. WHILE

3.4.6. EXITM

3.4.7. SHIFT

3.4.8. MAXNEST

3.4.9. FUNCTION

3.5. Structures

3.5.1. Definition

3.5.2. Usage

3.5.3. Nested Structures

3.5.4. Unions

3.5.5. Nameless Structures

3.5.6. Structures and Sections

3.5.7. Structures and Macros

3.6. Conditional Assembly

3.6.1. IF / ELSEIF / ENDIF

3.6.2. SWITCH / CASE / ELSECASE / ENDCASE

3.7. Listing Control

3.7.1. PAGE, PAGESIZE

3.7.2. NEWPAGE

3.7.3. MACEXP_DFT and MACEXP_OVR

3.7.4. LISTING

3.7.5. PRTINIT and PRTEXIT

3.7.6. TITLE

3.7.7. RADIX

3.7.8. OUTRADIX

3.8. Local Symbols

3.8.1. Basic Definition (SECTION/ENDSECTION)

3.8.2. Nesting and Scope Rules

3.8.3. PUBLIC and GLOBAL

3.8.4. FORWARD

3.8.5. Performance Aspects

3.9. Miscellaneous

3.9.1. SHARED

3.9.2. INCLUDE

3.9.3. BINCLUDE

3.9.4. MESSAGE, WARNING, ERROR, and FATAL

3.9.5. READ

3.9.6. INTSYNTAX

3.9.7. RELAXED

3.9.8. COMPMODE

3.9.9. END

4. Processor-specific Hints

4.1. 6811

4.2. PowerPC

4.3. IBM PALM

4.4. DSP56xxx

4.5. H8/300

4.6. H8/500

4.7. SH7000/7600/7700

4.8. HMCS400

4.9. H16

4.10. OLMS-40

4.11. OLMS-50

4.12. MELPS-4500

4.13. 6502UNDOC

4.14. MELPS-740

4.15. MELPS-7700/65816

4.16. M16

4.17. CP-3F

4.18. 4004/4040

4.19. MCS-48

4.20. MCS-51

4.21. MCS-251

4.22. 8080/8085

4.23. 8085UNDOC

4.24. 8086..V35

4.25. 8X30x

4.26. XA

4.27. AVR

4.28. Z80, Z380

4.29. Z80UNDOC

4.30. GB_Z80 resp. LR35902

4.31. Z380

4.32. Z8, Super8, and eZ8

4.33. Z8000

4.33.1. Conditions

4.33.2. Flags

4.33.3. Indirect Addressing

4.33.4. Direct versus Immediate Addressing

4.34. TLCS-900(L)

4.35. TLCS-90

4.36. TLCS-870

4.37. TLCS-47

4.38. TLCS-9000

4.39. TC9331

4.40. 29xxx

4.41. 80C16x

4.42. PIC16C5x/16C8x

4.43. PIC 17C4x

4.44. SX20/28

4.45. ST6

4.46. ST7

4.47. ST9

4.48. 6804

4.49. TMS3201x

4.50. TMS320C2x

4.51. TMS320C3x/C4x

4.52. TMS9900

4.53. TMS70Cxx

4.54. TMS370xxx

4.55. MSP430(X)

4.56. TMS1000

4.57. COP8

4.58. SC/MP

4.59. SC144xxx

4.60. NS32xxx

4.61. uPD78(C)1x

4.62. 75K0

4.63. 78K0

4.64. 78K2/78K3/78K4

4.65. uPD772x

4.66. uCOM-43

4.67. F2MC16L

4.68. MN161x

4.69. CDP180x

4.70. KENBAK

4.71. HP Nanoprocessor

4.72. IM61x0

5. File Formats

5.1. Code Files

5.2. Debug Files

6. Utility Programs

6.1. PLIST

6.2. BIND

6.3. P2HEX

6.4. P2BIN

6.5. AS2MSG

A. Error Messages of AS

B. I/O Error Messages

C. Programming Examples

C.1. 16 Bit Instructions via Macros

D. Frequently Asked Questions

E. Pseudo-Instructions and Integer Syntax

F. Predefined Symbols

G. Shipped Include Files

G.1. BITFUNCS.INC

G.2. CTYPE.INC

H. Acknowledgments

I. Changes since Version 1.3

J. Hints for the AS Source Code

J.1. Language Preliminaries

J.2. Capsuling System dependencies

J.3. System-Independent Files

J.3.1. Modules Used by AS

J.3.2. Additional Modules for the Tools

J.4. Modules Needed During the Build of AS

J.5. Generation of Message Files

J.5.1. Format of the Source Files

J.6. Creation of Documentation

J.7. Test Suite

J.8. Adding a New Target Processor

J.9. Localization to a New Language

1. Introduction

===============

This  instruction is meant for programmers  who are already very famil-

iar  with Assembler  and who  like to  know how  to work with AS. It is

rather  a reference than a user's manual and so it neither tries to ex-

plain  the  ''language  assembler''  nor  the processors. I have listed

further  literature in  the bibliography  which was  substantial in the

implementation  of the  different code  generators. There  is no book I

know  where  you  can  learn  Assembler  from the start, so I generally

learned this by ''trial and error''.

1.1. License Agreement

----------------------

Before  we can go ''in  medias res'', first of  all the inevitable pro-

logue:

As  in the  present version  is licensed  according to  the Gnu General

Public  License (GPL); the details  of the GPL may  be read in the file

COPYING  bundled with  this distribution.  If you  did not  get it with

AS, complain to the one you got AS from!

Shortly said, the GPL covers the following points:

- Programs  based upon AS must also be licensed according to the GPL;

- distribution is explicitly allowed;

- explicit  disclaiming of all warranties  for damages resulting from

usage of this program.

...however,  I really  urge you  to read  the file  COPYING for the de-

tails!

To  accelerate the error  diagnose and correction,  please add the fol-

lowing details to the bug report:

- Operating system (DOS, Windows, Linux) and its version

- Version of AS used, resp. dates of the EXE-files

- If  you compiled the assembler yourself,  the compiler used and its

version

- If possible, the source file that triggered the bug

You can contact me as follows:

- by Surface Mail:

Alfred Arnold

Hirschgraben 29

D-52062 Aachen

Germany

- by E-Mail: alfred@ccac.rwth-aachen.de

If  someone  likes  to  meet  me  personally to ask questions and lives

near  Aachen (=  Aix-la-Chapelle), you  will be  able to meet me there.

You  can do  this most  probably on  thursdays from  8pm to  9pm at the

RWTH  Aachen Computer Club (Elisabethstrasse  16, first floor, corridor

on the right).

Please  don't call me by phone.  First, complex relations are extremely

hard  to discuss  at phone.  Secondly, the  telephone companies are al-

ready rich enough...

The  latest version of AS  (DPMI, Win32, C) is  available from the fol-

lowing Server:

http://john.ccac.rwth-aachen.de:8000/as

or shortly

http://www.alfsembler.de

Whoever  has no access to  an FTP-Server can ask  me to send the assem-

bler  by  mail.  Only  requests  containing  a  blank  CD-R and a self-

addressed,  (correctly) stamped  envelope will  be answered. Don't send

any money!

Now,  after this inevitable introduction we can turn to the actual doc-

umentation:

1.2. General Capabilities of the Assembler

------------------------------------------

In  contrast to ordinary assemblers, AS  offers the possibility to gen-

erate  code for totally  different processors. At  the moment, the fol-

lowing processor families have been implemented:

- Motorola  68000..68040,, 683xx, and  Coldfire incl. coprocessor and

MMU

- Motorola ColdFire

- Motorola DSP5600x,DSP56300

- Motorola M-Core

- Motorola/IBM MPC601/MPC505/PPC403/MPC821

- IBM PALM

- Motorola 6800, 6801, 68(HC)11(K4) and Hitachi 6301

- Motorola/Freescale 6805, 68HC(S)08

- Motorola 6809 / Hitachi 6309

- Motorola/Freescale 68HC12(X) including XGATE

- Freescale/NXP S12Z (''MagniV'')

- Motorola 68HC16

- Freescale 68RS08

- Konami 052001

- Hitachi H8/300(H)

- Hitachi H8/500

- Hitachi SH7000/7600/7700

- Hitachi HMCS400

- Hitachi H16

- Rockwell  6502, 65(S)C02,  Commodore 65CE02,  WDC W65C02S, Rockwell

65C19, and Hudson HuC6280

- Rockwell PPS-4

- CMD 65816

- Mitsubishi MELPS-740

- Mitsubishi MELPS-7700

- Mitsubishi MELPS-4500

- Mitsubishi M16

- Mitsubishi M16C

- DEC PDP-11

- DEC VAX

- Western Digital WD16

- Intel 4004/4040

- Intel  MCS-48/41, including  Siemens SAB80C382,  and the  OKI vari-

ants

- Intel MCS-51/251, Dallas DS80C390

- Intel MCS-96/196(Nx)/296

- Intel 8080/8085

- Intel i960

- Signetics 8X30x

- Signetics 2650

- Philips XA

- Atmel (Mega-)AVR

- AMD 29K

- Siemens 80C166/167

- Zilog Z80 (including undocumented instructions), Z180, Z380, eZ80

- Sharp LR35902 (,,Gameboy Z80'')

- Sharp SC61860

- Sharp SC62015

- Zilog Z8, Super8, Z8 Encore

- Zilog Z8000

- Xilinx KCPSM/KCPSM3 ('PicoBlaze')

- LatticeMico8

- Toshiba TLCS-900(L)

- Toshiba TLCS-90

- Toshiba TLCS-870(/C)

- Toshiba TLCS-47

- Toshiba TLCS-42

- Toshiba TLCS-9000

- Toshiba TC9331

- Microchip PIC16C54..16C57

- Microchip PIC16C84/PIC16C64

- Microchip PIC17C42

- Parallax SX20/28

- SGS M380/GI LP8000

- SGS-Thomson ST6

- SGS-Thomson ST7/STM8

- SGS-Thomson ST9

- SGS-Thomson 6804

- Texas Instruments TMS32010/32015

- Texas Instruments TMS3202x

- Texas Instruments TMS320C3x/TMS320C4x

- Texas Instruments TMS320C20x/TMS320C5x

- Texas Instruments TMS320C54x

- Texas Instruments TMS320C6x

- Texas Instruments TMS99xx/99xxx

- Texas Instruments TMS7000

- Texas Instruments TMS1000

- Texas Instruments TMS370xxx

- Texas Instruments MSP430(X)

- National Semiconductor IMP-16

- National Semiconductor IPC-16 ('PACE'), INS8900

- National Semiconductor SC/MP

- National Semiconductor INS807x

- National Semiconductor COP4

- National Semiconductor COP8

- National Semiconductor SC144xx

- National Semiconductor NS32xxx

- Olympia CP-3F (resp. SGS M380, GI LP8000)

- Fairchild ACE

- Fairchild F8

- NEC �PD78(C)0x/�PD 78(C)1x

- NEC �COM-43/44/45

- NEC �PD75xx

- NEC �PD 75xxx (alias 75K0)

- NEC 78K0

- NEC 78K2

- NEC 78K3

- NEC 78K4

- NEC �PD7720/7725

- NEC �PD77230

- NEC V60

- Fujitsu F�MC8L

- Fujitsu F�MC16L

- OKI OLMS-40

- OKI OLMS-50

- Panafacom MN1610/MN1613

- Renesas RX

- Padauk PMS/PMC/PFSxxx

- Symbios Logic SYM53C8xx (yes, they are programmable!)

- Intersil CDP1802/1804/1805(A)

- Intersil IM6100/6120

- XMOS XS1

- MIL STD 1750

- KENBAK-1

- GI CP-1600

- HP Nano Processor

under work / planned / in consideration :

- ARM

- Analog Devices ADSP21xx

- DEC VAX

- SGS-Thomson ST20

- Texas Instruments TMS320C8x

- Zilog eZ80

Unloved, but now, however, present :

- Intel 80x86, 80186, Nec V20..V55 incl. coprocessor 8087

The  switch to  a different  code generator  is allowed even within one

file, and as often as one wants!

The  reason for this  flexibility is that  AS has a  history, which may

also  be recognized  by looking  at the  version number. AS was created

as  an extension of a macro assembler  for the 68000 family. On special

request,  I  extended  the  original  assembler  so that it was able to

translate  8051 mnemonics. On  this way (decline  ?!) from the 68000 to

8051,  some other  processors were  created as  by-products. All others

were  added over time due to user requests. So At least for the proces-

sor-independent  core of AS, one may  assume that it is well-tested and

free  of obvious bugs. However, I often  do not have the chance to test

a  new code  generator in  practice (due  to lack  of appropriate hard-

ware),  so  surprises  are  not  impossible  when working with new fea-

tures. You see, the things stated in section 1.1 have a reason...

This  flexibility implies  a somewhat  exotic code  format, therefore I

added  some tools to  work with it.  Their description can  be found in

chapter 6.

AS  is a macro assembler, which means  that the programmer has the pos-

sibility  to define new  ''commands'' by means  of macros. Additionally

it  masters conditional assembling. Labels  inside macros are automati-

cally processed as being local.

For  the assembler, symbols  may have either  integer, string or float-

ing  point values. These will be stored - like interim values in formu-

las  - with a  width of 32  bits for integer  values, 80 or 64 bits for

floating  point values,  and 255  characters for  strings. For a couple

of  micro controllers, there is the  possibility to classify symbols by

segmentation.  So the assembler  has a (limited)  possibility to recog-

nize accesses to wrong address spaces.

The  assembler does  not know  explicit limits  in the nesting depth of

include  files or macros;  a limit is  only given by  the program stack

restricting  the recursion depth.  Nor is there  a limit for the symbol

length, which is only restricted by the maximum line length.

From  version 1.38 on,  AS is a  multipass-assembler. This pompous term

means  no more  than the  fact that  the number  of passes  through the

source  code need not be exactly two.  If the source code does not con-

tain  any forward references, AS needs only one pass. In case AS recog-

nizes  in the second pass that it must use a shorter or longer instruc-

tion  coding, it needs  a third (fourth,  fifth...) pass to process all

symbol  references  correctly.  There  is  nothing more behind the term

''multipass'',  so it will not be  used further more in this documenta-

tion.

After  so much praise a bitter  pill: AS cannot generate linkable code.

An  extension  with  a  linker  needs  considerable  effort  and is not

planned at the moment.

Those  who want to take a look at  the sources of AS can simply get the

Unix  version  of  AS,  which  comes  as source for self-compiling. The

sources  are definitely not in a format that is targeted at easy under-

standing  - the original Pascal version still raises its head at a cou-

ple  of places, and  I do not  share a couple  of common opinions about

'good' C coding.

1.3. Supported Platforms

------------------------

Though  AS started as  a pure DOS  program, there are  a couple of ver-   DOS

sions  available that  are able  to exploit  a bit  more than  the Real

Mode  of an  Intel CPU.  Their usage  is kept  as compatible to the DOS

version  as possible,  but there  are of  course differences concerning

installation  and embedding into the operating system in question. Sec-

tions  in this manual that are only  valid for a specific version of AS

are  marked with  a corresponding  sidemark (at  this paragraph for the

DOS  version) aheaded to  the paragraph. In  detail, the following fur-

ther versions exist (distributed as separate packages):

In  case you runinto memory problems  when assembling large and complex   DPMI

programs  under  DOS,  there  is  a  DOS version that runs in protected

mode  via a DOS  extender and can  therefore make use  of the whole ex-

tended  memory of an  AT. The assembly  becomes significantly slower by

the extender, but at least it works...

There  is a native OS/2 version of AS for friends of IBM's OS/2 operat-   OS/2

ing  system. Since version 1.41r8, this  is a full 32-bit OS/2 applica-

tion,  which of course  means that OS/2  2.x and at  least an 80386 CPU

are mandatory.

You  can leave the area  of PCs-only with the  C version of AS that was   UNIX

designed  to be compilable on a large  number of UNIX systems (this in-

cludes  OS/2 with  the emx  compiler) without  too much of tweaking. In

contrast  to the previously mentioned versions, the C version is deliv-

ered  in source code,  i.e. one has  to create the  binaries by oneself

using  a C compiler. This is by far  the simpler way (for me) than pro-

viding  a dozen of  precompiled binaries for  machines I sometimes only

have limited access to...

2. Assembler Usage

==================

Scotty: Captain, we din' can reference it!

Kirk: Analysis, Mr. Spock?

Spock: Captain, it doesn't appear in the symbol table.

Kirk: Then it's of external origin?

Spock: Affirmative.

Kirk: Mr. Sulu, go to pass two.

Sulu: Aye aye, sir, going to pass two.

2.1. Hardware Requirements

--------------------------

The  hardware  requirements  of  AS  vary substantially from version to

version:

The  DOS version will  principally run on  any IBM-compatible PC, rang-   DOS

ing  from a PC/XT  with 4-dot-little megahertz  up to a Pentium. Howev-

er,  similar to other  programs, the fun  using AS increases the better

your  hardware is. An XT  user without a hard  drive will probably have

significant  trouble placing  the overlay  file on  a floppy because it

is  larger than  500 Kbytes...the  PC should  therefore have at least a

hard  drive,  allowing  acceptable  loading  times.  AS is not very ad-

vanced  in its  main memory  needs: the  program itself  allocates less

than  300  Kbytes  main  memory,  AS  should therefore work on machines

with at least 512 Kbytes of memory.

The  version  of  AS  compiled  for  the  DOS  Protected Mode Interface   DPMI

(DPMI)  requires at least 1 Mbyte of free extended memory. A total mem-

ory  capacity of  at least  2 Mbytes  is therefore the absolute minimum

given  one does not have other tools  in the XMS (like disk caches, RAM

disks,  or a  hi-loaded DOS);  the needs  will rise then appropriately.

If  one uses the DPMI version  in a DOS box of  OS/2, one has to assure

that  DPMI has been  enabled via the  box's DOS settings  (set to on or

auto)  and that a sufficient amount of  XMS memory has been assigned to

the  box.  The  virtual  memory  management  of OS/2 will free you from

thinking about the amount of free real memory.

The  C version  of AS  is delivered  as source  code and  therefore re-   UNIX

quires  a UNIX or OS/2 system equipped  with a C compiler. The compiler

has   to  fulfill  the  ANSI  standard  (GNU-C  for  example  is  ANSI-

compliant).  You can look up in the  README file whether your UNIX sys-

tem  has already  been tested  so that  the necessary  definitions have

been  made. You should reserve about 15  Mbytes of free hard disk space

for  compilation; this value  (and the amount  needed after compilation

to  store the compiled  programs) strongly differs  from system to sys-

tem, so you should take this value only as a rough approximation.

2.2. Delivery

-------------

Principally,  you can obtain AS  in one of two  forms: as a binary or a

source  distribution. In  case of  a binary  distribution, one gets AS,

the  accomanying  tools  and  auxiliary  files readily compiled, so you

can  immediately start to use it after unpacking the archive to the de-

sired  destination  on  your  hard  drive. Binary distibutions are made

for  widespread platforms, where either the  majority of users does not

have  a  compiler  or  the  compilation  is tricky (currently, this in-

cludes  DOS and OS/2).  A source distribution  in contrast contains the

complete  set of C sources to generate  AS; it is ultimately a snapshot

of  the source tree I  use for development on  AS. The generation of AS

from  the sources and their structure  is described in detail in appen-

dix  J, which is why at this  place, only the contents and installation

of a binary distribution will be described:

The  contents of the archive  is separated into several subdirectories,

therefore  you  get  a  directory  subtree  immediately after unpacking

without  having to  sort out  things manually.  The individual directo-

ries contain the following groups of files:

-  BIN: executable programs, text resources;

-  INCLUDE:  include files for assembler programs, e.g. register def-

initions or standard macros;

-  MAN:  quick references for  the individual programs  in Unix 'man'

format.

A  list of the files found in every binary distribution is given in ta-

ble  2.0. In case a file listed in  one of these (or the following) ta-

bles is missing, someone took a nap during copying (probably me)...

Table 2.1: Standard Contents of a Binary Distribution

+-------------------+---------------------------------------------+

| File              | Function                                    |

+-------------------+---------------------------------------------+

+-------------------+---------------------------------------------+

| Directory BIN     |                                             |

+-------------------+---------------------------------------------+

| AS.EXE            | executable of assembler                     |

| PLIST.EXE         | lists contents of code files                |

| PBIND.EXE         | merges code files                           |

| P2HEX.EXE         | converts code files to hex files            |

| P2BIN.EXE         | converts code files to binary files         |

| AS.MSG            | text resources for AS (DOS only)            |

| PLIST.MSG         | text resources for PLIST *)                 |

| PBIND.MSG         | text resources for PBIND *)                 |

| P2HEX.MSG         | text resources for P2HEX *)                 |

| P2BIN.MSG         | text resources for P2BIN *)                 |

| TOOLS.MSG         | common text resources for all tools *)      |

| CMDARG.MSG        | common text resources for all programs *)   |

| IOERRS.MSG        |                                             |

+-------------------+---------------------------------------------+

| *) DOS only                                                     |

+-------------------+---------------------------------------------+

| Directory DOC     |                                             |

+-------------------+---------------------------------------------+

| AS_DE.DOC         | german documentation, ASCII format          |

| AS_DE.HTML        | german documentation, HTML format           |

| AS_DE.TEX         | german documentation, LaTeX format          |

| AS_EN.DOC         | english documentation, ASCII format         |

| AS_EN.HTML        | english documentation, HTML format          |

| AS_EN.TEX         | english documentation, LaTeX format         |

+-------------------+---------------------------------------------+

| Directory INCLUDE |                                             |

+-------------------+---------------------------------------------+

| BCDIC.INC         | definition of BCDIC/code page 359           |

| BITFUNCS.INC      | functions for bit manipulation              |

| CTYPE.INC         | functions for classification of             |

|                   | characters                                  |

| EBCDIC.INC        | includes all EBCDIC variants                |

| CP037.INC         | definition EBCDIC (code page 037)           |

| CP5100.INC        | definition character set IBM 5100           |

| CP5110.INC        | definition EBCDIC (IBM 5110)                |

| 80C50X.INC        | register addresses SAB C50x                 |

| 80C552.INC        | register addresses 80C552                   |

| H8_3048.INC       | register addresses H8/3048                  |

| KENBAK.INC        | register addressed Kenbak-1                 |

| RADIX50.INC       | definition of RADIX 50 character set        |

| REG166.INC        | addresses and instruction macros 80C166/167 |

| REG251.INC        | addresses and bits 80C251                   |

| REG29K.INC        | peripheral addresses AMD 2924x              |

| REG53X.INC        | register addresses H8/53x                   |

| REG6303.INC       | register addresses 6303                     |

| REG683XX.INC      | register addresses 68332/68340/68360        |

| REG7000.INC       | register addresses TMS70Cxx                 |

| REG78310.INC      | register addresses & vectors 78K3           |

| REG78K0.INC       | register addresses 78K0                     |

| REG96.INC         | register addresses MCS-96                   |

| REGACE.INC        | register addresses ACE                      |

| REGEZ80.INC       | register addresses eZ80                     |

| REGF8.INC         | register and memory addresses F8            |

| REGAVROLD.INC     | register and bit addresses AVR family (old) |

| REGAVR.INC        | register and bit addresses AVR family       |

| REGCOLD.INC       | register and bit addresses Coldfire family  |

| REGCOP8.INC       | register addresses COP8                     |

| REGGP32.INC       | register addresses 68HC908GP32              |

| REGH16.INC        | register addresses H16                      |

| REGHC12.INC       | register addresses 68HC12                   |

| REGM16C.INC       | register addresses Mitsubishi M16C          |

| REGMSP.INC        | register addresses TI MSP430                |

| REGPDK.INC        | register and bit addresses PMC/PMS/PFSxxx   |

| REGS12Z.INC       | register and bit addresses S12Z family      |

| REGST6.INC        | register and macro definitions ST6          |

| REGST7.INC        | register and macro definitions ST7          |

| REGSTM8.INC       | register and macro definitions STM8         |

| REGST9.INC        | register and macro definitions ST9          |

| REGV60.INC        | register addresses NEC V60                  |

| REGZ380.INC       | register addresses Z380                     |

| STDDEF04.INC      | register addresses 6804                     |

| STDDEF16.INC      | instruction macros and register addresses   |

|                   | PIC16C5x                                    |

| STDDEF17.INC      | register addresses PIC17C4x                 |

| STDDEF18.INC      | register addresses PIC16C8x                 |

| STDDEF2X.INC      | register addresses TMS3202x                 |

| STDDEF37.INC      | register and bit addresses TMS370xxx        |

| STDDEF3X.INC      | peripheral addresses TMS320C3x              |

| STDDEF4X.INC      | peripheral addresses TMS320C4x              |

| STDDEF47.INC      | instruction macros TLCS-47                  |

| STDDEF51.INC      | definition of SFRs and bits for             |

|                   | 8051/8052/80515                             |

| STDDEF56K.INC     | register addresses DSP56000                 |

| STDDEF5X.INC      | peripheral addresses TMS320C5x              |

| STDDEF60.INC      | instruction macros and register addresses   |

|                   | PowerPC                                     |

| REGSX20.INC       | register and bit addresses Parallax SX20/28 |

| AVR/*.INC         | register and bit addresses AVR family       |

|                   | (do not include directly, use REGAVR.INC)   |

| COLDFIRE/*.INC    | register and bit addresses ColdFire family  |

|                   | (do not include directly, use REGCOLD.INC)  |

| EZ80/*.INC        | register and bit addresses eZ80 family      |

|                   | (do not include directly, use REGCOLD.INC)  |

| PDK/*.INC         | register and bit addresses PMC/PMS/PFSxxx   |

|                   | (do not include directly, use REGPDK.INC)   |

| S12Z/*.INC        | register and bit addresses S12Z family      |

|                   | (do not include directly, use REGS12Z.INC)  |

| ST6/*.INC         | register and bit addresses ST6 family       |

|                   | (do not include directly, use REGST6.INC)   |

| ST7/*.INC         | register and bit addresses ST7 family       |

|                   | (do not include directly, use REGST7.INC)   |

| STM8/*.INC        | register and bit addresses STM8 family      |

|                   | (do not include directly, use REGSTM8.INC)  |

| STDDEF62.INC      | register addresses and macros ST6 (old)     |

| STDDEF75.INC      | register addresses 75K0                     |

| STDDEF87.INC      | register and memory addresses TLCS-870      |

| STDDEF90.INC      | register and memory addresses TLCS-90       |

| STDDEF96.INC      | register and memory addresses TLCS-900      |

| STDDEFXA.INC      | SFR and bit addresses Philips XA            |

| STDDEFZ8.INC      | register addresses Z8 family (old)          |

| REGZ8.INC         | register addresses Z8 family (new)          |

| Z8/*.INC          | register and bit addresses Z8 family        |

|                   | (do not include directly, use REGZ8.INC)    |

+-------------------+---------------------------------------------+

| Directory LIB     |                                             |

+-------------------+---------------------------------------------+

| Directory MAN     |                                             |

+-------------------+---------------------------------------------+

| ASL.1             | Short Reference for AS                      |

| PLIST.1           | Short Reference for PLIST                   |

| PBIND.1           | Short Reference for PBIND                   |

| P2HEX.1           | Short Reference for P2HEX                   |

| P2BIN.1           | Short Reference for P2BIN                   |

+-------------------+---------------------------------------------+

Depending  on the platform,  a binary distribution  however may contain

more  files to  allow operation,  like files  necessary for  DOS exten-

ders.  In case of the DOS DPMI  version, the extensions listed in table   DPMI

2.1  result. Just to  mention it: it  is perfectly O.K.  to replace the

tools  with their counterparts  from a DOS  binary distribution; on the

on  hand,  they  execute  significantly  faster  without the extender's

overhead,  and on the other hand, they  do not need the extended memory

provided by the extender.

+---------------+--------------------------------+

| File          | Function                       |

+---------------+--------------------------------+

+---------------+--------------------------------+

| Directory MAN |                                |

+---------------+--------------------------------+

| ASL.1         | quick reference for AS         |

| PLIST.1       | quick reference for PLIST      |

| PBIND.1       | quick reference for PBIND      |

| P2HEX.1       | quick reference for P2HEX      |

| P2BIN.1       | quick reference for P2BIN      |

+---------------+--------------------------------+

+---------------+--------------------------------+

| Directory BIN |                                |

+---------------+--------------------------------+

| DPMI16BI.OVL  | DPMI server for the assembler  |

| RTM.EXE       | runtime module of the extender |

+---------------+--------------------------------+

Table 2.1: Additional Files in a DPMI Binary Distribution

An  OS/2 binary distribution  contains in addition  to the base files a   OS/2

set  of DLLs belonging  to the runtime  environment of the emx compiler

used  to build AS (table 2.2). In  case you already have these DLLs (or

newer  versions of them), you may delete  these and use your ones inst-

ed.

+---------------+------------------------------+

| File          | function                     |

+---------------+------------------------------+

+---------------+------------------------------+

| Directory BIN |                              |

+---------------+------------------------------+

| EMX.DLL       | runtime libraries for AS and |

| EMXIO.DLL     | its tools                    |

| EMXLIBC.DLL   |                              |

| EMXWRAP.DLL   |                              |

+---------------+------------------------------+

Table 2.2: Additional Files in an OS/2 binary distribution

2.3. Installation

-----------------

There  is no need for  a special installation prior  to usage of AS. It   DOS

is  sufficient to unpack  the archive in  a fitting place  and to add a

few  minor settings. For  example, this is  an installation a user used

to UNIX-like operating systems might choose:

Create  a directory c:\as an  (I will assume in  the following that you

are  going to install AS on drive  C), change to this directory and un-

pack  the archiv,  keeping the  path names  stored in the archive (when

using  PKUNZIP, the command line option  -d is necessary for that). You

now should have the following directory tree:

c:\as

c:\as\bin

c:\as\include

c:\as\lib

c:\as\man

c:\as\doc

c:\as\demos

Now,  append the directory c:\as\bin to  the PATH statement in your AU-

TOEXEC.BAT,  which allows  the system  to find  AS and  its tools. With

your  favourite text editor, create  a file named AS.RC  in the lib di-

rectory with the following contents:

-i c:\as\include

This  so-called  key  file  tells  AS  where  to search for its include

files.  The following statement  must be added  to your AUTOEXEC.BAT to

tell AS to read this file:

set ASCMD=@c:\as\lib\as.rc

There  are many more things  you can preset via  the key file; they are

listed in the following section.

The  installation of the DPMI version  should principally take the same   DPMI

course  as for the pure  DOS version; as soon  as the PATH contains the

bin  directory, the  DOS extender's  files will  be found automatically

and  you should not  notice anything of  this mechanism (except for the

longer  startup time...). When  working on an  80286-based computer, it

is  theoretically possible  tha you  get confronted  with the following

message upon the first start:

machine not in database (run DPMIINST)

Since  the DPMIINST  tool ins  not any  more included in newer versions

of  Borland's DOS  extender, I  suppose that  this is  not an  item any

more...in case you run into this, contact me!

The  installation of the  OS/2 version can  generally be done just like   OS/2

for  the DOS version, with  the addition that the  DLLs have to be made

visible  for the operating  system. In case  you do not  want to extend

the  LIBPATH entry in  your CONFIG.SYS, it  is of course  also valid to

move the DLLs into a directory already listed in LIBPATH.

As  already mentioned,  the installation  instructions in  this section

limit  themselves to binary distributions.  Since an installation under

Unix  is currently alway  a source-based installation,  the only hint I   UNIX

can give here is a reference to appendix J.

2.4. Start-Up Command, Parameters

---------------------------------

AS  is a command line driven program,  i.e. all parameters and file op-

tions are to be given in the command line.

A  couple of message files belongs  to AS (recognizable by their suffix

MSG)  AS accesses to dynamically load  the messages appropriate for the

national  language.  AS  searches  the  following directories for these

files:

- the current directory;

- the EXE-file's directory;

- the  directory named in the AS_MSGPATH environment variable, or al-

ternitavely  the directories  listed in  the PATH environment vari-

able;

- the directory compiled into AS via the LIBDIR macro.

These  files are  indispensable for  a proper  operation of AS, i.e. AS

will terminate immediately if these files are not found.

The  language selection  (currently only  German and  English) is based

on  the COUNTRY setting under DOS and OS/2 respectively on the LANG en-

vironment variable under Unix.

In  order to  fulfill AS's  memory requirements  under DOS, the various   DOS

code  generator modules of  the DOS version  were moved into an overlay

which  is part  of the  EXE file.  A separate  OVR file like in earlier

versions  of  AS  therefore  dose  not  exist any more, AS will however

still  attempt  to  reduce  the  overlaying  delays by using eventually

available  EMS or XMS memory. In case  this results in trouble, you may

suppress  usage  of  EMS  or  XMS  by  setting the environment variable

USEXMS  or USEEMS to n.  E.g., it is possible  to suppress the using of

XMS by the command:

SET USEXMS=n

Since  AS performs  all in-  and output  via the  operating system (and

therefore  it  should  run  also  on  not 100% compatible DOS-PC's) and

needs  some basic display control, it emits ANSI control sequences dur-

ing  the assembly.  In case  you should  see strange  characters in the   DOS/

messages  displayed by AS, your CONFIG.SYS  is obviously lacking a line

like this:

device=ansi.sys

but  the further functions of AS  will not be influenced hereby. Alter-   DPMI

natively  you are  able to  suppress the  output of ANSI sequences com-

pletely by setting the environment variable USEANSI to n.

The  DOS extender of the  DPMI version can be  influenced in its memory   DPMI

allocation  strategies  by  a  couple  of environment variables; if you

need  to know their settings, you may  look up them in the file DPMIUS-

ER.DOC.  It is  additionally able  to extend  the available memory by a

swap  file. To do this,  set up an environment  variable ASXSWAP in the

following way:

SET ASXSWAP=<size>[,file name]

The  size  specification  has  to  be  done  in megabytes and has to be

done.  The file  name in  contrast is  optional; if  it is missing, the

file  is  named  ASX.TMP  and  placed  in the current directory. In any

case, the swap file is deleted after program end.

The  command line  parameters can  roughly be  divided into three cate-

gories:  switches, key file references  (see below) and file specifica-

tions.  Parameters of these two categories  may be arbitrarily mixed in

the  command line. The assembler evaluates  at first all parameters and

then assembles the specified files. From this follow two things:

- the  specified switches affect all  specified source files. If sev-

eral  source  files  shall  be  assembled  with different switches,

this has to be done in separate runs.

- it  is possible to assemble  more than one file  in one shot and to

bring  it to  the top,  it is  allowed that  the file specs contain

wildcards.

Parameter  switches are recognized  by AS by  starting with a slash (/)

or  hyphen (-).  There are  switches that  are only  one character long

and  additionally switches composed  of a whole  word. Whenever AS can-

not  interpret a switch as  a whole word, it  tries to interprete every

letter as an individual switch. For example, if you write

-queit

instead of

-quiet

AS  will take  the letters  q, u,  e, i,  and t as individual switches.

Multiple-letter  switches additionally  have the  difference to single-

letter  switches that AS will accept  an arbitrary mixture of upper and

lower  casing,  whereas  single-letter  switches  may  have a different

meaning depending on whether upper or lower case is used.

At the moment, the following switches are defined:

- l:  sends assembler  listing to  console terminal  (mostly screen).

In  case several passes have to be  done, the listing of all passes

will be send to the console (in opposite to the next option).

- L:  writes assembler  listing into  a file.  The list file will get

the  same name as  the source file,  only the extension is replaced

by LST. Except one uses...

- OLIST:  with a fiel  name as argument  allows to redirect the list-

ing  to a different  file or a  different path. This  option may be

used  multiple times in case multiple  files are assembled with one

execution.

- listline-prefix:  This  option  allows  to  modify the data that is

printed  in front of  every source line  in the listing. The struc-

ture of the format string is described in 2.6.

-  list-unknown-values  resp. no-list-unknown-values: enables or dis-

ables  displaying question  marks for  values unknown  in the first

pass.

- RADIX:  This option  changes the  default number  system to a value

between  2 and 36, thereby overriding  the default of 10. The value

given  by this  switch can  itself be  overridden in the program by

the statement odf same name.

- LISTRADIX:   By  default,   all  numeric   output  in  the  listing

(addresses,  generated code, symbol values) is written in hexadeci-

mal  notation.  This  switch  requests  usage of a different number

system  in the range of  2 to 36. For  instance, '-listradix 8' re-

quests  octal output. If the radix  value is written with a leading

zero  (e.g. 08 instead  of 8), the  program counter's current value

is prited with leading zeros in the listing.

- SPLITBYTE  [character]:  Display  numbers  in  the  listing in byte

groups,  separated by the given character. A period is used as sep-

arator  if no explicit  character is given.  This option is usually

used  in conjunction with the  LISTRADIX option. For instance, list

radix  8  with  a  period  as  character  results  in the so-called

'split octal' notation.

- o:  Sets the new name of the code file generated by AS. If this op-

tion  is used multiple  times, the names  will be assigned, one af-

ter  the other, to the  source files which have  to be assembled. A

negation  (see  below)  of  this  option  in connection with a name

erases  this name from  the list. A  negation without a name erases

the whole list.

- SHAREOUT:ditto for a SHARE file eventually to be created.

- c:  SHARED-variables will be  written in a  format which permits an

easy  integration into a  C-source file. The  extension of the file

is H.

- p:  SHARED-variables  will  be  written  in  a format which permits

easy  integration into the CONST-block of a Pascal program. The ex-

tension of the file is INC.

- a:  SHARED-variables  will  be  written  in  a format which permits

easy  integration into an  assembler source file.  The extension of

the file is INC.

Concerning  effect and function of  the SHARED-symbols please see chap-

ters 2.13 resp. 3.9.1.

- g  [format]: This switch instructs AS  to create an additional file

that  contains debug  information for  the program. Allowed formats

are  the AS-specific  MAP format  ( format=MAP), a NoICE-compatible

command  file (  format=NOICE), and  the Atmel  format used  by the

AVR  tools ( format=ATMEL). The information  stored in the MAP for-

mat  is comprised of a symbol table  and a table describing the as-

signment  of source lines to machine addresses. A more detailed de-

scription  of  the  MAP  format  can  be  found  in section 5.2 The

file's  extension is MAP,  NOI, resp. OBJ,  depending on the chosen

format.  If no explicit format specification  is done, the MAP for-

mat is chosen.

- noicemask  [value]:  By  default,  AS  lists  only symbols from the

CODE  segment in  NoICE debug  info files.  With this option and an

integer  value interpreted  as a  bit mask,  symbols fom other seg-

ments  may be  added. The  assignment of  segments to bit positions

may be taken from table 5.1.

- w: suppress issue of warnings;

- E  [file]: error messages and warnings produced by AS will be redi-

rected  to  a  file.  Instead  of  a  file,  the 5 standard handles

(STDIN..STDPRN)  can also  be specified  as !0  to !4  . Default is

!2,  meaning STDERR. If  the file option  is left out,  the name of

the  error file is the same as of the source file, but with the ex-

tension LOG.

- q:  This switch suppresses  all messages of  AS, the exceptions are

error  messages and  outputs which  are are  forced from the source

file.  The time needed for assembly  is slightly reduced hereby and

if  you call AS from a shell  there is no redirection required. The

disadvantage  is  that  you  may  ''stay  in the dark'' for several

minutes ... It is valid to write quiet instead of q.

- v:  This is verbose, i.e. the  opposite of quiet operation. The on-

ly  additional information  that is  currently printed  is the ver-

sion info.

- version: Prints version information and exits.

- h:  write hexadecimal numbers in  lowercase instead of capital let-

ters. This option is primarily a question of personal taste.

- i  <path list>:  issues a  list of  directories where the assembler

shall  automatically search  for include  files, in  case it didn't

find  a file  in the  current directory.  The different directories

have to be separated by semicolons.

- u:  calculate a list  of areas which  are occupied in the segments.

This  option is effective only in  case a listing is produced. This

option  requires considerable additional  memory and computing per-

formance. In normal operation it should be switched off.

- C:  generates a list  of cross references.  It lists which (global)

symbols  are used in files and lines. This list will also be gener-

ated  only in  case a  listing is  produced. This  option occupies,

too, additional memory capacity during assembly.

- s:  issues a  list of  all sections  (see chapter 3.8). The nesting

is indicated by indentations (Pascal like).

- t:  by means of this switch it  is possible to separate single com-

ponents  of the  standard issued  assembler-listing. The assignment

of  bits to parts can  be found in the  next section, where the ex-

act format of the assembly listing is explained.

- D:  defines symbols.  The symbols  which are  specified behind this

option  and separated  by commas  are written  to the global symbol

table  before starting the  assembly. As default  these symbols are

written  as integer numbers with the value TRUE, by means of an ap-

pended  equal sign, however,  you can select  other values. The ex-

pression  following the equals sign may include operators or inter-

nal  functions, but not  any further symbols,  even if these should

have  been defined before  in the list!  Together with the commands

for  conditional  assembly  (see  there)  you may produce different

program  versions out  of one  source file  by command line inputs.

CAUTION!  If the case-sensitive mode is used, this has to be speci-

fied  in the command line  before any symbol definitions, otherwise

symbol names will be converted to upper case at this place!

- A:  stores  the  list  of  global  symbols in another, more compact

form.  Use this option if the  assembler crashes with a stack over-

flow  because of too long symbol  tables. Sometimes this option can

increase  the processing speed  of the assembler,  but this depends

on the sources.

- x:  Sets the level of  detail for error messages.  The level is in-

creased  resp. decreased  by one  each time  this option  is given.

While  on level 0 (default) only the error message itself is print-

ed,  an extended message is added  beginning at level 1 that should

simplify  the  identification  of  the  error's  cause.  Appendix A

lists  which error messages carry  which extended messages. At lev-

el  2 (maximum), the source line  containing the error is addition-

ally printed.

- n:  If this option is set, the  error messages will be issued addi-

tionally  with their error number (see appendix A). This is primar-

ily  intended for use with shells  or IDE's to make the identifica-

tion of errors easier by those numbers.

- U:  This option switches AS to  the case-sensitive mode, i.e. upper

and  lower case in the names  of symbols, sections, macros, charac-

ter  sets, and  user-defined functions  will be distinguished. This

is not the case by default.

- P:  Instructs  AS  to  write  the  source  text  processed by macro

processor  and conditional  assembly into  a file. Additional blank

and  pure comment lines are missing  in this file. The extension of

this file is I.

- M:  If this switch is given, AS generates a file, that contains de-

finitions  of macros  defined in  the source  file that did not use

the  NOEXPORT  option.  This  new  file  has  the  same name as the

source file, only the extension is modified into MAC.

- G:  this switch defines  whether AS should  produce code or not. If

switched  off,  the  processing  will  be  stopped  after the macro

processor.  This switch is activated  by default (logically, other-

wise  you would not  get a code  file). This switch  can be used in

conjunction  with the P  switch, if only  the macro processor of AS

shall be used.

- r  [n]: issue  warnings if  situations occur  that force  a further

pass.  This information can  be used to  reduce the number of pass-

es.  You may optionally specify the  number of the first pass where

issuing  of such messages shall start. Without this argument, warn-

ings  will come  starting with  the first  pass. Be  prepared for a

bunch of messages!!

- bigendian:  This switch sets  big endian mode  for values placed in

memory  right from the program's beginning, given the target archi-

tecture   supports  the  pseudo  instruction   of  same  name  (see

3.2.15).

- plainbase:  This switch  enables omission  of an  empty index argu-

ment right from the program's beginning (see 3.2.14).

- underscrore-macroargs:  This  switch  enables  usage  of underscore

characters in macro argument names (see 3.4.1).

- relaxed:  this switch enables  the RELAXED mode  right from the be-

ginning  of the program,  which otherwise has  to be enabled by the

pseudo instruction of sane name(see section 3.9.7).

- intsyntax:  this switch allows to augment or reduce the list of al-

lowed  integer constant  syntaxes right  from the  beginning of the

program. See section 3.9.6 for a list of allowed arguments.

- supmode:  this switch enables right from  the beginning of the pro-

gram  usage of  machine instructions  that may  only be used in the

processor's supervisor mode (see section 3.2.4).

- Y:  This switch instructs AS to to suppress all messages about out-

of-branch  conditions, once the necessity  for another pass is giv-

en.  See section  2.11 for  the (rare)  situations that  might make

use of this switch necessary.

- cpu  <name>:  this  switch  allows  to  set the target processor AS

shall  generate code for, in case  the source file does not contain

a  CPU instruction. If  the selected target  supports CPU arguments

(see  section  3.2.3),  they  may  be  used  on the command line as

well.  Using this switch with  ? or list as  argument lists all im-

plemented targets.

- alias  <new>=<old>:  defines  the  processor  type  <new>  to be an

alias  for  the  type  <old>.  See  section  2.14  for the sense of

processor aliases.

-  gnuerrors:  display messages  about errors  resp. warnings  not in

the  AS standard  format, but  instead in  a format  similar to the

GNU  C compiler. This  simplifies the integration  of AS into envi-

ronments  tuned for this  format, however also  suppresses the dis-

play of precise error positions in macro bodies!

-  maxerrors  [n]: instructs the assembler  to terminate assembly af-

ter the given number of errors.

-  maxerrors  [n]: instructs  the assembler  to terminate assembly if

the  include  nesting  level  exceeds  the  given limit (default is

200).

-  Werror: instructs the assembler to treat warnings as errors.

-  wrelative  resp.  wno-relative:  instructs  the assembler to issue

warnings  if a  relative jump  instead of  an absolute  is possible

(only for Z80 target).

-  wsign-extension  resp.  wno-sign-extension:  instructs  the assem-

bler  to issue warnings  about implicit sign  extensions (onlx 68K,

MOVEQ).

- compmode:  This switch  instructs the  assembler to  operate by de-

fault  in compatibility mode.  See section 3.9.8  for more informa-

tion about this mode.

- packing:  This switch  overrides the  architecture specific default

of the PACKING option (see section 3.2.9).

As  long as switches require no  arguments and their concatenation does

not  result in a multi-letter switch,  it is possible to specify sever-

al switches at one time, as in the following example :

as test*.asm firstprog -cl /i c:\as\8051\include

All  files TEST*.ASM as  well as the  file FIRSTPROG.ASM will be assem-

bled,  whereby listings of all files  are displayed on the console ter-

minal.  Additional sharefiles will  be generated in  the C- format. The

assembler  should search for additional  include files in the directory

C:\AS\8051\INCLUDE.

This  example shows that  the assembler assumes  ASM as the default ex-

tension for source files.

A  bit of caution should  be applied when using  switches that have op-

tional  arguments: if a  file specification immediately  follows such a

switch  without the  optional argument,  AS will  try to interprete the

file specification as argument - what of course fails:

as -g test.asm

The  solution in this  case would either  be to move  the -g option the

end or to specify an explicit MAP argument.

Beside  from specifying options in  the command line, permanently need-

ed  options may be placed in  the environment variable ASCMD. For exam-

ple,  if  someone  always  wants  to  have  assembly listings and has a

fixed  directory for include  files, he can  save a lot  of typing with

the following command:

set ascmd=-L -i c:\as\8051\include

The  environment options are processed before  the command line, so op-

tions  in the command line can override contradicting ones in the envi-

ronment variable.

In  the case of very  long path names, space  in the ASCMD variable may

become  a problem. For  such cases a  key file may  be the alternative,

in  which the options can be written in  the same way as in the command

line  or the  ASCMD-variable. But  this file  may contain several lines

each  with a maximum length of 255 characters.  In a key file it is im-

portant,  that for options which require  an argument, switches and ar-

gument  have to be  written in the  same line. AS  gets informed of the

name of the key file by a @ aheaded in the ASCMD variable, e.g.

set ASCMD=@c:\as\as.key

In  order to neutralize  options in the  ASCMD variable (or  in the key

file),  prefix the option with a plus  sign. For example, if you do not

want  to generate  an assembly  listing in  an individual case, the op-

tion can be retracted in this way:

as +L <file>

Naturally  it is not consequently  logical to deny an  option by a plus

sign.... UNIX soit qui mal y pense.

References  to key  files may  not only  come from  the ASCMD variable,

but  also directly from the command  line. Similarly to the ASCMD vari-

able, prepend the file's name with a @ character:

as @<file> ....

The  options read from  a key file  in this situation  are processed as

if  they had been written out in the  command line in place of the ref-

erence,  not like  the key  file referenced  by the ASCMD variable that

is processed prior to the command line options.

Referencing  a key file from a key  file itself is not allowed and will

be answered wit an error message by AS.

In  case that you like to start AS  from another program or a shell and

this  shell hands over  only lower-case or  capital letters in the com-

mand  line, the following workaround  exists: if a tilde  (~) is put in

front  of an option letter, the  following letter is always interpreted

as  a lower-case letter. Similarly a  # demands the interpretation as a

capital  letter.  For  example,  the  following  transformations result

for:

/~I ---> /i

-#u ---> -U

In  dependence of the  assembly's outcome, the  assembler ends with the

following return codes:

0 error free run, at maximum warnings occurred

1 The  assembler displayed only its  command-line parameters and ter-

minated immediately afterwards.

2 Errors occurred during assembly, no code file has been produced.

3 A  fatal error  occurred what  led to  immediate termination of the

run.

4 An  error occurred already  while starting the  assembler. This may

be a parameter error or a faulty overlay file.

255 An  internal error  occurred during  initialization that should not

occur  in any case...reboot, try again, and contact me if the prob-

lem is reproducible!

Similar  to UNIX, OS/2 extends an  application's data segment on demand   OS/2

when  the  application  really  needs  the memory. Therefore, an output

like

511 KByte available memory

does  not indicate a shortly  to come system crash  due to memory lack,

it  simply shows the distance  to the limit when  OS/2 will push up the

data segment's size again...

As  there is no  compatible way in  C under different operating systens   UNIX

to  find out  the amount  of available  memory resp.  stack, both lines

are missing completely from the statistics the C version prints.

2.5. Format of the Input Files

------------------------------

Like  most  assemblers,  AS  expects  exactly  one instruction per line

(blank  lines are  naturally allowed  as well).  The lines  must not be

longer than 255 characters, additional characters are discarded.

A single line has following format:

[label[:]] <mnemonic>[.attr] [param[,param..]] [;comment]

A  line may also be  split over several lines  in the source file, con-

tinuation  characters chain these parts together  to a single line. One

must  however consider that, due to  the internal buffer structure, the

total  line must not be longer  than 256 characters. Line references in

error  messages  always  relate  to  the  last  line of such a composed

source line.

The  colon for the label  is optional, in case  the label starts in the

first  column (the consequence is that  a machine or pseudo instruction

must  not start in column 1). It is  necessary to set the colon in case

the  label does  not start  in the  first column  so that AS is able to

distinguish  it from a mnemonic.  In the latter case,  there must be at

least  one space  between colon  and mnemonic  if the processor belongs

to  a family  that supports  an attribute  that denotes  an instruction

format  and is  separated from  the mnemonic  by a colon. This restric-

tion  is  necessary  to  avoid  ambiguities:  a  distinction  between a

mnemonic  with format and a label  with mnemonic would otherwise be im-

possible.

Some  signal processor  families from  Texas Instruments optionally use

a  double line (||) in place of  the label to signify the parallel exe-

cution  with the  previous instruction(s).  If these  two assembler in-

structions  become  a  single  instruction  word at machine level (C3x/

C4x),  an  additional  label  in  front  of  the  second instruction of

course  does not make sense  and is not allowed.  The situation is dif-

ferent  for the  C6x with  its instruction  packets of variable length:

If  someone  wants  to  jump  into  the middle of an instruction packet

(bad  style, if you ask me...), he has to place the necessary label be-

fore  into a  separate line.  The same  is valid  for conditions, which

however may be combined with the double line in a single source line.

The  attribute is used by a  couple of processors to specify variations

or  different codings of a certain  instruction. The most prominent us-

age  of the attibute is  is the specification of  the operand size, for

example in the case of the 680x0 family (table 2.3).

+-----------+-------------------------------------+---------------------+

| attribute | arithmetic-logic instruction        | jump instruction    |

+-----------+-------------------------------------+---------------------+

+-----------+-------------------------------------+---------------------+

| B         | byte (8 bits)                       | 8-bit-displacement  |

| W         | word (16 bits)                      | 16-bit-displacement |

| L         | long word (32 bits)                 | 16-bit-displacement |

| Q         | quad word (64 bits)                 | ---------           |

| C         | half precision (16 bits)            | ---------           |

| S         | single precision (32 bits)          | 8-bit-displacement  |

| D         | double precision (64 bits)          | ---------           |

| X         | extended precision (80/96 bits)     | 32-bit-displacement |

| P         | decimal floating point (80/96 bits) | ---------           |

+-----------+-------------------------------------+---------------------+

Table 2.3: Allowed Attributes (Example 680x0)

Since  this manual is not also meant as a user's manual for the proces-

sor  families supported by  AS, this is  unfortunately not the place to

enumerate  all possible attributes for  all families. It should however

be  mentioned that in general, not all instructions of a given instruc-

tion  set allow  all attributes  and that  the omission of an attribute

generally  leads  to  the  usage  of  the ''natural'' operand size of a

processor  family. For more thorough studies, consult a reasonable pro-

grammer's manual, e.g. [1] for the 68K's.

In  the case  of TLCS-9000,  H8/500, and  M16(C), the  attribute serves

both  as  an  operand  size  specifier  (if  it is not obvious from the

operands)  and as a  description of the  instruction format to be used.

A  colon has to be  used to separate the  format from the operand size,

e.g. like this:

add.w:g   rw10,rw8

This  example does  not show  that there  may be a format specification

without  an operand size. In contrast, if an operand size is used with-

out  a  format  specification,  AS  will automatically use the shortest

possible  format. The  allowed formats  and operand  sizes again depend

on  the machine instruction and  may be looked up  e.g. in [166], [35],

[66], resp. [67].

The  number of  instruction parameters  depends on  the mnemonic and is

principally  located between  0 and  20. The  separation of the parame-

ters  from each  other is  to be  performed only  by commas (exception:

DSP56xxx,  its parallel data transfers are separated with blanks). Com-

mas  that are included in brackets or  quotes, of course, are not taken

into consideration.

Instead  of a comment  at the end,  the whole line  can consist of com-

ment if it starts in the first column with a semicolon.

To  separate the individual components you  may also use tabulators in-

stead of spaces.

2.6. Format of the Listing

--------------------------

The  listing produced by  AS using the  command line options  i or I is

roughly divisible into the following parts :

1 augmented reproduction of the source code;

2 symbol list;

3 usage list;

4 cross reference list.

The  two last ones are only generated if they have been demanded by ad-

ditional command line options.

In  the first part, AS lists the  complete contents of all source files

including  the produced code. A line  of this listing has the following

form:

[<n>] <line>/<address> <code> <source>

In  the field n, AS  displays the include nesting  level. The main file

(the  file where  assembly was  started) has  the depth  0, an included

file  from  there  has  depth  1  etc..  Depth  0 is not displayed: for

source  lines in the main file, this  field is replaced by an appropri-

ate  amount of spaces, or is  omitted entirely if no include statements

have  been used  so far.  The 'memory'  whether there have been include

statements  and up to which level, spans  more than one pass. This way,

the  assembler 'learns'  the maximum  include depth  in the  first pass

and  is able to  print this field  with consistent width throughout the

whole listing.

In  the field line,  the source line  number of the  referenced file is

issued.  The first  line of  a file  has the  number 1.  The address to

which  the code generated  from this line  is written follows after the

slash  in the field address. The number  system used for the address is

set  via the listradix command line  option (2.4), also whether the ad-

dress  is printed with leading zeros  or not. The currently used target

defines  the width of this field by  the size of the address space: for

a  processor with a 64K address  space, four hex digits are sufficient,

while  eight  digits  are  needed  if  the  address  space's  size is 4

GBytes.

The  code produced  is written  behind address  in the field code, also

in  the  number  system  defined  by  the  list radix. Depending on the

processor  target and current  address space, the  values are formatted

with  different length. In  the simplemost case,  the code may be shown

as  a sequence  of 8  bit bytes.  Words of  16 or  32 bits  may also be

used,  in case the  address space is  not byte-addressable, or the tar-

get  processor's instruction  words are  longer than  one byte. For in-

stance,  the  address  space  of  the  68000  or  the  PDP-11  is byte-

addressable,  their machine  instructions however  have a  length of 16

bits.  There are even a few cases when  the word length is not a multi-

ple of 8 bit, like 12 bits for the PDP-8.

If  forward references are used, the final  values for parts of the ma-

chine  code cannot be  listed in the  first pass. AS  will then make an

assumption  about  the  value  that  permits  further  assembly.  For a

(growing)  number of targets, these fields will be displayed with ques-

tion marks. The following targets currently support this feature:

- 65xx/65XXX

- 8080/8085

- (e)Z80/Z180/Z380

- 8051/80251

In  case these question  marks conflict with  further processing of the

listing,  this feature may be  disabled via the -no-list-unknown-values

command line switch.

If  more code  is generated  than the  field can take, additional lines

will be generated, in which case only this field is used.

Finally,  in the field  source, the line  of the source  file is issued

in its original form.

Internally,  the structure  of the  data in  front of every source line

is  controlled by a format  string, which may be  modified via the com-

mand  line switch  -listline-prefix. It  supports the  following place-

holders:

-  %[c]i:  The current include nesting  depth, with an optional field

width.  Without an  explicit field  width, this  may be expanded to

nothing or an equivalent number of spaces if the depth is zero.

-  %[c]n:  The  current  source  line  number, with an optional field

width.  The number is printed  right-aligned within this field. The

default for the field width is five characters.

-  %[c]a:  Similarly,  the  current  memory  address,  with a target-

specific default field width, as described above.

In  all cases, a field width with  a leading zero results in filling up

the  unused space  with zeros  instead of  blanks. The  default of this

format  string is %i%n/%a. To achieve an output similar to the one gen-

erated  by AS  versions previous  to 1.42  Build 249, %1i%5n/%8a may be

used as format string.

The  symbol table was designed in a way  that it can be displayed on an

80-column   display   whenever   possible.   For  symbols  of  ''normal

length'',  a  double  column  output  is  used. If symbols exceed (with

their  name and value) the limit  of 40 columns (characters), they will

be  issued in a separate  line. The output is  done in alphabetical or-

der.  Symbols that  have been  defined but  were never  used are marked

with a star (*) as prefix.

The  parts  mentioned  so  far  as  well  as  the  list  of all macros/

functions  defined  can  be  selectively  masked  out from the listing.

This  can  be  done  by  the  already mentioned command line switch -t.

There  is an internal  byte inside AS  whose bits represent which parts

are  to be written. The  assignment of bits to  parts of the listing is

listed in table 2.4.

+-----+--------------------------------+

| bit | part                           |

+-----+--------------------------------+

+-----+--------------------------------+

| 0   | source file(s) + produced code |

| 1   | symbol table                   |

| 2   | macro list                     |

| 3   | function list                  |

| 4   | line numbering                 |

| 5   | register symbol list           |

| 7   | character set table            |

+-----+--------------------------------+

Table 2.4: Assignment of Bits to Listing Components

All bits are set to 1 by default, when using the switch

-t <mask>

Bits  set in <mask>  are cleared, so  that the respective listing parts

are  suppressed. Accordingly it  is possible to  switch on single parts

again  with a  plus sign,  in case  you had  switched off too much with

the  ASCMD variable... If someone wants  to have, for example, only the

symbol table, it is enough to write:

-t 2

The  usage list issues the occupied  areas hexadecimally for every sin-

gle  segment. If the area  has only one address,  only this is written,

otherwise the first and last address.

The  cross reference list issues any defined symbol in alphabetical or-

der and has the following form:

symbol <symbol name> (=<value>,<file>/<line>):

file <file 1>:

<n1>[(m1)]  ..... <nk>[(mk)]

.

.

file <file l>:

<n1>[(m1)]  ..... <nk>[(mk)]

The  cross reference  list lists  for every  symbol in  which files and

lines  it has  been used.  If a  symbol was  used several  times in the

same  line, this would be indicated by  a number in brackets behind the

line  number. If a  symbol was never  used, it would  not appear in the

list;  The same  is true  for a  file that  does not contain any refer-

ences for the symbol in question.

CAUTION!  AS can only print the  listing correctly if it was previously

informed  about the output  media's page length  and width! This has to

be  done with the  PAGE instruction (see  3.7.1). The preset default is

a length of 60 lines and an unlimited line width.

2.7. Symbol Conventions

-----------------------

Symbols  are allowed  to be  up to  255 characters  long (as hinted al-

ready  in the  introduction) and  are being  distinguished on the whole

length, but the symbol names have to meet some conventions:

Symbol  names are  allowed to  consist of  a random combination of let-

ters,  digits, underlines  and dots,  whereby the  first character must

not  be a digit.  The dot is  only allowed to  meet the MCS-51 notation

of  register bits and should - as far  as possible - not be used in own

symbol  names. To separate  symbol names in  any case the underline (_)

and not the dot (.) should be used .

AS  is by default  not case-sensitive, i.e.  it does not matter whether

one  uses upper  or lower  case characters.  The command  line switch U

however  allows to  switch AS  into a  mode where  upper and lower case

makes  a  difference.  The  predefined  symbol  CASESENSITIVE signifies

whether   AS  has  been  switched  to   this  mode:  TRUE  means  case-

sensitiveness, and FALSE its absence.

Table  2.5 shows  the most  important symbols  which are  predefined by

AS.

+-------------------+---------------------------------------------+

| name              | meaning                                     |

+-------------------+---------------------------------------------+

+-------------------+---------------------------------------------+

| TRUE              | logically ''true''                          |

| FALSE             | logically ''false''                         |

| CONSTPI           | Pi (3.1415.....)                            |

| FLOATMAX          | largest representable floating point number |

| VERSION           | version of AS in BCD-coding,                |

|                   | e.g. 1331 hex for version 1.33p1            |

| ARCHITECTURE      | target platform AS was compiled for, in     |

|                   | the style processor-manufacturer-operating  |

|                   | system                                      |

| DATE              | date and                                    |

| TIME              | time of the assembly (start)                |

| MOMCPU            | current target CPU                          |

|                   | (see the CPU instruction)                   |

| MOMFILE           | current source file                         |

| MOMLINE           | line number in source file                  |

| MOMPASS           | number of the currently running pass        |

| MOMSECTION        | name of the current section                 |

|                   | or an empty string                          |

|  *, ,. $ resp. PC | current value of program counter            |

+-------------------+---------------------------------------------+

Table 2.5: Predefined Symbols

CAUTION!  While it does not matter  in case-sensitive mode which combi-

nation  of upper  and lower  case to  use to  reference predefined sym-

bols,  one  has  to  use  exactly  the  version given above (only upper

case) when AS is in case-sensitive mode!

Additionally  some pseudo instructions define  symbols that reflect the

value  that has  been set  with these  instructions. Their descriptions

are explained at the individual commands belonging to them.

On  most platforms,  the name  INF is  reserved as infinity in floating

point format. It therefore may not be used for user-defined symbols.

A  hidden feature (that has to be  used with care) is that symbol names

may  be  assembled  from  the  contents  of string symbols. This can be

achieved  by framing the string symbol's name with curly braces and in-

serting  it into the new symbol's name.  This allows for example to de-

fine a symbol's name based on the value of another symbol:

cnt             set     cnt+1

temp            equ     "\{CNT}"

jnz     skip{temp}

.

.

skip{temp}:     nop

CAUTION:  The programmer  has to  assure that  only valid  symbol names

are generated!

A  complete list of all symbols predefined by AS can be found in appen-

dix F.

Apart  from its value, every symbol  also owns a marker which signifies

to  which segment it  belongs. Such a  distinction is mainly needed for

processors  that have more  than one address  space. The additional in-

formation  allows AS  to issue  a warning  when a  wrong instruction is

used  to access a  symbol from a  certain address space.  A segment at-

tribute  is automatically added to a symbol  when is gets defined via a

label  or  a  special  instruction  like  BIT; a symbol defined via the

''allround  instructions'' SET resp. EQU  is however ''typeless'', i.e.

its  usage will  never trigger  warnings. A  symbol's segment attribute

may be queried via the buit-in function SYMTYPE, e.g.:

Label:

.

.

Attr    equ     symtype(Label)  ; results in 1

The  individual segment types  have the assigned  numbers listed in ta-

ble  2.6. Register symbols  which do not  really fit into  the order of

normal  symbols are explained in section 2.12. The SYMTYPE function de-

livers -1 as result when called with an undefined symbol as argument.

The  functions DEFINED resp.  SYMEXIST allow to  check whether a symbol

is  defined  or  not.  The  difference  between  them: SYMEXIST returns

'false'  for a  forward declaration  in pass  one, and 'true' in subse-

quent  passes. In  contrast, DEFINED  consistently returns  'false' for

forward  declarations in  all passes  - the  meaning is  'defined up to

this point in the source'.

DEFINED  not only accepts simple symbol names, but also complete formu-

la  expressions. In this case,  the return value is  'true' only if the

expression  does  not  contain  any  symbols  that are undefined by the

rule given in the previous paragraph.

+-------------------+--------------+

| segment           | return value |

+-------------------+--------------+

| <none>            |       0      |

| CODE              |       1      |

| DATA              |       2      |

| IDATA             |       3      |

| XDATA             |       4      |

| YDATA             |       5      |

| BITDATA           |       6      |

| IO                |       7      |

| REG               |       8      |

| ROMDATA           |       9      |

| EEDATA            |      10      |

| <Register Symbol> |      128     |

+-------------------+--------------+

Table 2.6: return values of the SYMTYPE function

2.8. Temporary Symbols

----------------------

Especially  when dealing with programs  that contain sequences of loops

of  if-like statements, one  is continuously faced  with the problem of

inventing  new  names  for  labels  -  labels of which you know exactly

that  you will  never need  to reference  them again afterwards and you

really  would like to get  'rid' of them somehow.  A simple solution if

you  don't  want  to  swing  the  large hammer of sections (see chapter

3.8)  are temporary symbols which  remain valid as long  as a new, non-

temporary  symbol gets defined. Other assemblers offer a similar mecha-

nism  which is commonly  referred as 'local  symbols'; however, for the

sake  of a better distinction, I want  to stay with the term 'temporary

symbols'.  AS knows three different types  of temporary symbols, in the

hope  to offer everyone 'switching' to AS a solution that makes conver-

sion  as  easy  as  possible.  However, practically every assembler has

its  own interpretation of this feature, so there will be only few cas-

es where a 1:1 solution for existing code:

2.9. Named Temporary Symbols

----------------------------

A  symbol whose  name starts  with two  dollar signs (something that is

neither  allowed  for  non-temporary  symbols  nor  for constants) is a

named  temporary symbol.  AS keeps  an internal  counter which is reset

to  0 before assembly begins and which gets incremented upon every def-

inition  of a non-temporary symbol. When  a temporary symbol is defined

or  referenced,  both  leading  dollar  signs  are  discarded  and  the

counter's  current value  is appended.  This way,  one regains the used

symbol  names with  every definition  of a  non-temporary symbol  - but

you  also cannot reach the previously  symbols any more! Temporary sym-

bols  are therefore  especially suited  for usage  in small instruction

blocks,  typically  a  dozen  of  machine  instructions, definitely not

more than one screen. Otherwise, one easily gets confused...

Here is a small example:

$$loop: nop

dbra    d0,$$loop

split:

$$loop: nop

dbra    d0,$$loop

Without  the non-temporary label between the  loops, of course an error

message about a double-defined symbol would be the result.

2.9.1. Nameless Temporary Symbols

- - - - - - - - - - - - - - - - -

For  all those who regard named  temporary symbols still as too compli-

cated,  there is an even  simpler variant: If one  places a single puls

or  minus  sign  as  a  label,  this  is  converted  to symbol names of

__forwnn  respectively __backmm,  with nn  respectively mm  being coun-

ters  that start  counting at  zero. Those  symbols are  referenced via

the  special names - --  --- respectively + ++  +++, which refer to the

three  last 'minus symbols'  and the next  three 'plus symbols'. There-

fore,  the selection between these two  variants depends on whether one

wants to forward- or backward-reference a symbol.

Apart  from plus  and minus,  defining nameless  temporary symbols also

exists  in a third variant, namely a  slash (/). A temporary symbol de-

fined  in this  way may  be referenced  both backward and forward, i.e.

it  is treated either as a plus or  a minus, depending on the way it is

being referenced.

Nameless  temporary symbols are usually used  in constructs that fit on

one  screen page,  like skipping  a few  machine instructions  or tight

loops  - things  would becone  to puzzling  otherwise (this only a good

advice,  however...). An  example for  this is  the following  piece of

code, this time as 65xx code:

cpu     6502

-       ldx     #00

-       dex

bne     -           ; branch to 'dex'

lda     RealSymbol

beq     +           ; branch to 'bne --'

jsr     SomeRtn

iny

+       bne     --          ; branch to 'ldx #00'

SomeRtn:

rts

RealSymbol:

dfs     1

inc	ptr

bne 	+      	    ; branch to 'tax'

inc 	ptr+1

+ 	tax

bpl 	++     	    ; branch to 'dex'

beq 	+      	    ; branch forward to 'rts'

lda 	#0

/  	rts            	    ; slash used as wildcard.

+ 	dex

beq 	-           ; branch backward to 'rts'

ptr:	dfs	2

2.9.2. Composed Temporary Symbols

- - - - - - - - - - - - - - - - -

This  is maybe  the type  of temporary  symbols that  is nearest to the

concept  of local  symbols and  sections. Whenever  a symbol's name be-

gins  with a dot (.), the symbol  is not directly stored with this name

in  the symbol  table. Instead,  the name  of the most recently-defined

symbol  not  beginning  with  a  dot  is prepended to the symbols name.

This  way,  'non-dotted'  symbols  take  the role of section separators

and  'dotted' symbol  names may  be reused  after a 'non-dotted' symbol

has been defined. Take a look at the following little example:

proc1:				; non-temporary symbol 'proc1'

.loop	moveq	#20,d0		; actually defines 'proc1.loop'

dbra	d0,.loop

rts

proc2:				; non-temporary symbol 'proc2'

.loop	moveq	#10,d1		; actually defines 'proc2.loop'

jsr	proc1

dbra	d1,.loop

rts

Note  that it is  still possible to  access all temporary symbols, even

without  being in  the same  'area', by  simply using the composed name

(like 'proc2.loop' in the previous example).

It  is principally possible to  combine composed temporary symbols with

sections,  which makes  them also  to local  symbols. Take however into

account  that the most  recent non-temporary symbol  is not stored per-

section,  but simply globally. This may change however in a future ver-

sion, so one shouldn't rely on the current behaviour.

2.10. Formula Expressions

-------------------------

In  most places where the assembler  expects numeric inputs, it is pos-

sible  to specify not  only simple symbols  or constants, but also com-

plete  formula  expressions.  The  components  of these formula expres-

sions  can be either single symbols and constants. Constants may be ei-

ther integer, floating point, or string constants.

2.10.1. Integer Constants

- - - - - - - - - - - - -

Integer  constants describe non-fractional numbers.  They are witten as

a  sequence of digits. This may  be done in different numbering systems

(see table 2.7).

+---------+---------------+---------------+-----------+----------------+

|         |   Intel Mode  | Motorola Mode |   C Mode  |    IBM Mode    |

+---------+---------------+---------------+-----------+----------------+

+---------+---------------+---------------+-----------+----------------+

| Decimal |     Direct    |     Direct    |   Direct  |     Direct     |

| Hex     |    Suffix H   |    Prefix $   | Prefix 0x | X'..' or H'..' |

| Ident   |      hexh     |      $hex     |   0xhex   |     x'hex'     |

|         |               |               |           |     h'hex'     |

| Binary  |    Suffix B   |    Prefix %   | Prefix 0b |      O'..'     |

| Ident   |      binb     |      %bin     |   0bbin   |     b'bin'     |

| Octal   | Suffix O or Q |    Prefix @   |  Prefix 0 |      B'..'     |

| Ident   |      octo     |      @oct     |    0oct   |     o'oct'     |

|         |      octq     |               |           |                |

| ASCII   |               |               |           |      A'..'     |

| Ident   |               |               |           |     a'asc'     |

+---------+---------------+---------------+-----------+----------------+

Table 2.7: Defined Numbering Systems and Notations

In  case the numbering system has  not been explicitly stated by adding

the  special control characters listed in  the table, the number system

is derived as follows:

- If  a RADIX  statement was  given, use  the number  system given by

it, otherwise:

- If  a -radix command  line switch was  given, use the number system

given by it, otherwise:

- Use decimal (base 10).

Both  the RADIX statement  and the -radix  command line switch also al-

low  to set up 'unusual' numbering systems,  i.e. others than 2, 8, 10,

or 16.

Valid  digits are numbers  from 0 to  9 and letters  from A to Z (value

10  to 35) up  to the numbering  system's base minus  one. An exception

from  this is the ASCII represenation:  For this variant, a character's

ASCII  value (or its code  in the currently active  code page, see sec-

tion  3.1.12)  describes  a  whole  byte.  Therefore, integer constants

written  this  way  are  identical  to multi character constants. These

two expressions:

'ABCD'

A'ABCD'

are  identical, the 'A'  prefix is redundant.  One may enable this syn-

tax  for existing  code, because  there are  a few  original assemblers

(e.g. for the Signetics 2650) that support this syntax.

Independent  of the target, AS  implements multi character constants in

big  endian order, which means that  'ABCD' results in an integer value

of  0x41424344.  Why  this?  Well,  AS's  first target was the Motorola

60008,  and no one ever objected...the  only exception from this is the

PDP-11  (and the WD16  which uses an  LSI-11): For better compatibility

to  DEC's MACRO-11,  multi character  are little  endian if this target

is used. For instance, 'AB' results in ain integer value of 0x4241.

The  usage of  letters in  integer constants  however brings along some

ambiguities  since symbol names are also  sequences of numbers and let-

ters:  a symbol name however must not  start with a character from 0 to

9.  This means that an  integer constant which is  not clearly marked a

such  with a  special prefix  character must  not begin  with a letter.

One  has to add  an additional, otherwise  superfluous zero in front in

such  cases. The most prominent case is the writing of hexadecimal con-

stants  in Intel mode:  If the leftmost  digit is between  A and F, the

trailing  H doesn't  help to  clarify, an  additional 0  has to be pre-

fixed  (e.g. 0F0H  instead of  F0H). The  Motorola and C syntaxes which

both  mark the numbering system at the  front of a constant do not have

this issue.

Quite  tricky is furthermore that the higher the default numbering sys-

tem  set via RADIX  becomes, the more  letters used to denote numbering

systems  in Intel and C syntax  become 'eaten'. For example, you cannot

write  binary  constants  anymore  after  a  RADIX  16, and starting at

RADIX  18, the  Intel syntax  even doesn't  allow to  write hexadecimal

constants any more. Therefore CAUTION!

Appendix  E lists which syntax is used  by which target by default. In-

dependent  of  this  default,  there  is  always  the  option to add or

delete  individual syntax  variants via  the INTSYNTAX instruction (see

section  3.9.6). The names listed as Ident, prefixed with a plus or mi-

nus sign, serve as arguments to this instruction.

The  RELAXED instruction (see  section 3.9.7) serves  as a sort 'global

enable  switch': in relaxed  mode, all notations  may be used, indepen-

dent  of the  selected target  processor. The  result is  that an arbi-

trary  syntax may be  used (possibly loosing  compatibility to standard

assemblers).

Both  INTSYNTAX and RELAXED specifically enable  usage of the 'IBM syn-

tax' for all targets, which is sometimes found on other assemblers:

This  notation puts the actual value  into apostrophes and prepends the

numbering  system ('x'  or 'h'  for hexadecimal,  'o' for octal and 'b'

for  binary). So, the integer constant  305419896 can be written in the

following ways:

x'12345678'

h'12345678'

o'2215053170'

b'00010010001101000101011001111000'

Another  variant of  this notation  for some  targets is  to leave away

the  closing apostrophe, to allow simpler  porting of existing code. It

is not recommended for new programs.

2.10.2.  Floating Point Constants

- - - - - - - - - - - - - - - - -

Floating  point constants are to be written in the usual scientific no-

tation, which is known in the most general form:

[-]<integer digits>[.post decimal positions][E[-]exponent]

CAUTION!  The assembler first tries to  interprete a constant as an in-

teger  constant and makes a floating-point  format try only in case the

first  one  failed.  If  someone  wants  to enforce the evaluation as a

floating  point number,  this can  be done  by dummy post decimal posi-

tions, e.g. 2.0 instead of 2.

2.10.3.  String Constants

- - - - - - - - - - - - -

String  constants have  to be  enclosed in  single or  double quotation

marks.  In order to make it possible to include quotation marks or spe-

cial  characters in string constants,  an ''escape mechanism'' has been

implemented, which should sound familiar for C programmers:

The  assembler  understands  a  backslash  (\) with a following decimal

number  of three digits maximum  in the string as  a character with the

according  decimal ASCII  value. The  numerical value may alternitavely

be  written in hexadecimal or octal notation  if it is prefixed with an

x  resp. a 0.  In case of  hexadecimal notation, the  maximum number of

digits  is limited to 2. For example,  it is possible to include an ETC

character  by  writing\3.  But  be  careful  with the definition of NUL

characters!  The C  version currently  uses C  strings to store strings   UNIX

internally.  As C strings use a  NUL character for termination, the us-

age of NUL characters in strings is currently not portable!

Some  frequently used control  characters can also  be reached with the

following abbreviations:

\b : Backspace           \a : Bell         \e : Escape

\t : Tabulator           \n : Linefeed     \r : Carriage Return

\\ : Backslash           \' or \H : Apostrophe

\" or \I : Quotation marks

Both  upper and lower  case characters may  be used for the identifica-

tion letters.

By  means of this  escape character, you  can even work formula expres-

sions into a string, if they are enclosed by curly braces: e.g.

message "root of 81 : \{sqrt(81)}"

results in

root of 81 : 9

AS  chooses with the help  of the formula result  type the correct out-

put  format, further  string constants,  however, are  to be avoided in

the  expression.  Otherwise  the  assembler  will  get  mixed up at the

transformation  of capitals  into lower  case letters.  Integer results

will  by  default  be  written  in  hexadecimal  notation, which may be

changed via the OUTRADIX instruction.

Except  for  the  insertion  of  formula  expressions, you can use this

''escape-mechanism''  as well in ASCII  defined integer constants, like

this:

move.b   #'\n',d0

However,  everything  has  its  limits,  because the parser with higher

priority,  which disassembles a line  into op-code and parameters, does

not know what it is actually working with, e.g. here:

move.l   #'\'abc',d0

After  the third apostrophe, it  will not find the  comma any more, be-

cause  it presumes  that it  is the  start of  a further character con-

stant.  An error message about a  wrong parameter number is the result.

A workaround would be to write e.g., \i instead of \'.

2.10.4.  String to Integer Conversion and Character Constants

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Earlier   versions  of  AS  strictly  distinguished  between  character

strings  and  so-called  ''character  constants'':  At  first glance, a

character  constant looks like a string, the characters are however en-

closed  in single  instead of  double quotation  marks. Such  an object

had  the data  type 'Integer',  i.e. it  represented a  number with the

value  given by  the (ASCII)  code of  the character,  and it was some-

thing completely different:

move.b   #65,d0

move.b   #'A',d0      ; equal to first instruction

move.b   #"A",d0      ; not allowed in older versions!

This  strict  differentiation  no  longer  exists,  so it is irrelevant

whether  single or double quotes  are used. If an  integer value is ex-

pected  as argument, and a string is used, the conversion via the char-

acter's  (ASCII)  value  is  done  ''on  the  fly'' at this place. This

means  that in the  example given, all  three lines result  in the same

machine code.

Such  an  implicit  conversion  to  integer  values also take place for

strings  consisting of  multiple constancs,  which are sometimes called

''multi character constants'':

'A'    ==$41

'AB'   ==$4142

'ABCD' ==$41424344

Multi  character constants are the only case where using single or dou-

ble  quotes still  makes a  difference. Many  targets define pseudo in-

structions  to dispose constants in  memory, and which accept different

data  types.  In  such  a  case,  it  is  still necessary to use double

quotes if a character string shall be placed in memory:

dc.w    "ab"  ; disposes two words (0x0041,0x0042)

dc.w    'ab'  ; disposes one word (0x4142)

Important:  using the correct quotation is not necessary if the charac-

ter  string is longer than the used  operand size, which is two charac-

ters or 16 bits in this example.

2.10.5.  Evaluation

- - - - - - - - - -

The  calculation of intermediary results  within formula expressions is

always  done with the highest resolution  available on the host system.

For  integer numbers, this  is 32 or  64 bits. For  floating point num-

bers,  the  range  is  approximately  +/-1.8*10^308 (IEEE Double Preci-

sion)  or  +/-1.1*10^4932  (IEEE  Extended  Precision). A possible test

for value range overflows is done only on the final result.

2.10.6. Operators

- - - - - - - - -

The  assembler provides the  operands listed in  table 2.8 for combina-

tion.

+---------+------------------+-------+-------+-------+--------+-----+------+

| Operand | Function         | #Args |  Int  | Float | String | Reg | Rank |

+---------+------------------+-------+-------+-------+--------+-----+------+

+---------+------------------+-------+-------+-------+--------+-----+------+

|    <>   | inequality       |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    !=   | alias for <>     |       |       |       |        |     |      |

|    >=   | greater or equal |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    <=   | less or equal    |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    <    | truly smaller    |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    >    | truly greater    |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    =    | equality         |   2   |  yes  |  yes  |   yes  | yes |  14  |

|    ==   | alias for =      |       |       |       |        |     |      |

|         |                  |       |       |       |        |     |      |

|    !!   | log. XOR         |   2   |  yes  |   no  |   no   |  no |  13  |

|    ||   | log. OR          |   2   |  yes  |   no  |   no   |  no |  12  |

|    &&   | log. AND         |   2   |  yes  |   no  |   no   |  no |  11  |

|    ~~   | log. NOT         |   1   |  yes  |   no  |   no   |  no |   2  |

|         |                  |       |       |       |        |     |      |

|    -    | difference       |   2   |  yes  |  yes  |   no   |  no |  10  |

|    +    | sum              |   2   |  yes  |  yes  |   yes  |  no |  10  |

|    #    | modulo division  |   2   |  yes  |   no  |   no   |  no |   9  |

|    /    | quotient         |   2   | yes*) |  yes  |   no   |  no |   9  |

|     *   | product          |   2   |  yes  |  yes  |   no   |  no |   9  |

|     ^   | power            |   2   |  yes  |  yes  |   no   |  no |   8  |

|         |                  |       |       |       |        |     |      |

|    !    | binary XOR       |   2   |  yes  |   no  |   no   |  no |   7  |

|    |    | binary OR        |   2   |  yes  |   no  |   no   |  no |   6  |

|    &    | binary AND       |   2   |  yes  |   no  |   no   |  no |   5  |

|    ><   | mirror of bits   |   2   |  yes  |   no  |   no   |  no |   4  |

|    >>   | log. shift right |   2   |  yes  |   no  |   no   |  no |   3  |

|    <<   | log. shift left  |   2   |  yes  |   no  |   no   |  no |   3  |

|     ~   | binary NOT       |   1   |  yes  |   no  |   no   |  no |   1  |

+---------+------------------+-------+-------+-------+--------+-----+------+

|                      *) remainder will be discarded                      |

+---------+------------------+-------+-------+-------+--------+-----+------+

Table 2.8: Operators Predefined by AS

''Rank''  is the priority  of an operator  at the separation of expres-

sions  into subexpressions. The operator with  the highest rank will be

evaluated  at the very end.  The order of evaluation  can be defined by

new bracketing.

The  comparison operators deliver TRUE in  case the condition fits, and

FALSE  in case it  doesn't. For the  logical operators an expression is

TRUE in case it is not 0, otherwise it is FALSE.

For  operators with two  arguments, the order  of operand evaluation is

undefined.  The only exception  from this is  the logical AND and logi-

cal  OR: If  the result  is unambiguously  defined by the left operand,

the right operand is not evaluated at all.

Two  details have to  be kept im  mind when comparing register symbols.

First,  two register symbols are equal if they refer to the same regis-

ter.  Some processors have alias names  for registers, and these alias-

es  are regarded as equal. Fr instance,  the A7 register of a 68000 may

also  be referred to as  SP, and those two  register symbols are equal.

On  the other hand,  some processors have  more than one  set of regis-

ters.  The  68040,  fo  rinstance,  has 'normal' (integer) and floating

point  registers. There is no greater  or smaller relation between reg-

isters  from different  groups, the  corresponding operators always re-

turn FALSE. Only a test for equality or inequality makes sense.

The  mirroring of bits probably needs  a little bit of explanation: the

operator  mirrors the lowest  bits in the  first operand and leaves the

higher  priority bits unchanged. The number of bits which is to be mir-

rored is given by the right operand and may be between 1 and 32 .

A  small pitfall  is hidden  in the  binary complement: As the computa-

tion  is always done with 32 resp.  64 bits, its application on e.g. 8-

bit  masks usually results  in values taht  do not fit  into 8-bit num-

bers  any more  due to  the leading  ones. A  binary AND with a fitting

mask is therefore unavoidable!

2.10.7. Functions

- - - - - - - - -

In  addition to  the operators,  the assembler  defines another line of

primarily   transcendental  functions  with  floating  point  arguments

which are listed in tables 2.9 and 2.10.

+----------+----------------------+------------------------+----------------+

| name     | meaning              | argument               | result         |

+----------+----------------------+------------------------+----------------+

+----------+----------------------+------------------------+----------------+

| SQRT     | square root          | arg >= 0               | floating point |

|          |                      |                        |                |

| SIN      | sine                 | arg in R               | floating point |

| COS      | cosine               | arg in R               | floating point |

| TAN      | tangent              | arg != (2n+1)*(Pi)/(2) | floating point |

| COT      | cotangent            | arg != n*Pi            | floating point |

|          |                      |                        |                |

| ASIN     | inverse sine         | | arg | <= 1           | floating point |

| ACOS     | inverse cosine       | | arg | <= 1           | floating point |

| ATAN     | inverse tangent      | arg in R               | floating point |

| ACOT     | inverse cotangent    | arg in R               | floating point |

|          |                      |                        |                |

| EXP      | exponential function | arg in R               | floating point |

| ALOG     | 10 power of argument | arg in R               | floating point |

| ALD      | 2 power of argument  | arg in R               | floating point |

| SINH     | hyp. sine            | arg in R               | floating point |

| COSH     | hyp. cosine          | arg in R               | floating point |

| TANH     | hyp. tangent         | arg in R               | floating point |

| COTH     | hyp. cotangent       | arg != 0               | floating point |

|          |                      |                        |                |

| LN       | nat. logarithm       | arg > 0                | floating point |

| LOG      | dec. logarithm       | arg > 0                | floating point |

| LD       | bin. logarithm       | arg > 0                | floating point |

| ASINH    | inv. hyp. Sine       | arg in R               | floating point |

| ACOSH    | inv. hyp. Cosine     | arg >= 1               | floating point |

| ATANH    | inv. hyp. Tangent    | arg < 1                | floating point |

| ACOTH    | inv. hyp. Cotangent  | arg > 1                | floating point |

|          |                      |                        |                |

| INT      | integer part         | arg in R               | floating point |

+----------+----------------------+------------------------+----------------+

| BITCNT   | number of one's      | integer                | integer        |

| FIRSTBIT | lowest 1-bit         | integer                | integer        |

+----------+----------------------+------------------------+----------------+

Table  2.9: Functions Predefined  by AS -  Part 1 (Integer and Floating

Point Functions

+-------------+----------------------+----------------+----------------+

| name        | meaning              | argument       | result         |

+-------------+----------------------+----------------+----------------+

+-------------+----------------------+----------------+----------------+

| LASTBIT     | highest 1-bit        | integer        | integer        |

| BITPOS      | unique 1-bit         | integer        | integer        |

|             |                      |                |                |

| SGN         | sign (0/1/-1)        | floating point | integer        |

|             |                      | or integer     |                |

| ABS         | absolute value       | integer or     | integer or     |

|             |                      | floating point | floating point |

| TOUPPER     | matching capital     | integer        | integer        |

| TOLOWER     | matching lower case  | integer        | integer        |

|             |                      |                |                |

| UPSTRING    | changes all          | string         | string         |

|             | characters           |                |                |

|             | into capitals        |                |                |

|             |                      |                |                |

| LOWSTRING   | changes all          | string         | string         |

|             | characters           |                |                |

|             | into to lower case   |                |                |

|             |                      |                |                |

| STRLEN      | returns the length   | string         | integer        |

|             | of a string          |                |                |

|             |                      |                |                |

| SUBSTR      | extracts parts of a  | string,        | string         |

|             | string               | integer,       |                |

|             |                      | integer        |                |

| CHARFROMSTR | extracts a character | string,        | integer        |

|             | from a string        | integer        |                |

| STRSTR      | searches a substring | string,        | integer        |

|             | in a string          | string         |                |

| VAL         | evaluates contents   | string         | depends on     |

|             | as expression        |                | argument       |

| EXPRTYPE    | delivers type of     | integer,       | 0              |

|             | argument             | float,         | 1              |

|             |                      | string         | 2              |

+-------------+----------------------+----------------+----------------+

Table  2.10: Functions  Predefined by  AS -  Part 2 (Integer and String

Functions

The  functions FIRSTBIT, LASTBIT, and BITPOS  return -1 as result if no

resp.  not exactly one bit is  set. BITPOS additionally issues an error

message in such a case.

The  string function SUBSTR expects the  source string as first parame-

ter,  the start position as  second and the number  of characters to be

extracted  as third parameter  (a 0 means  to extract all characters up

to  the  end).  Similarly,  CHARFROMSTR  expects  the  source string as

first  argument and the character position  as second argument. In case

the  position  argument  is  larger  or  equal  to  the source string's

length,  SUBSTR returns an  empty string while  CHARFROMSTR returns -1.

A  position argument  smaller than  zero is  treated as zero by SUBSTR,

while CHARFROMSTR will return -1 also in this case.

Here  is an  example how  to use  these both  functions. The task is to

put  a string  into memory,  with the  string end  being signified by a

set MSB in the last character:

dbstr   macro   arg

if      strlen(arg) > 1

db     substr(arg, 0, strlen(arg) - 1)

endif

if      strlen(arg) > 0

db     charfromstr(arg, strlen(arg) - 1) | 80h

endif

endm

STRSTR  returns the  first occurence  of the  second string  within the

first  one resp. -1 if  the search pattern was  not found. Similarly to

SUBSTR and CHARFROMSTR, the first character has the position 0.

If  a function expects floating point  arguments, this does not mean it

is impossible to write e.g.

sqr2 equ sqrt(2)

In  such cases an automatic type  conversion is engaged. In the reverse

case  the INT-function  has to  be applied  to convert a floating point

number  to an integer. When using this function, you have to pay atten-

tion  that the  result produced  always is  a signed integer and there-

fore has a value range of approximately +/-2.0E9.

When  AS is switched  to case-sensitive mode,  predefined functions may

be  accessed with an arbitrary combination  of upper and lower case (in

contrast  to predefined symbols). However,  in the case of user-defined

functions  (see section 3.4.9),  a distinction between  upper and lower

case  is made. This has e.g. the  result that if one defines a function

Sin,  one can  afterwards access  this function  via Sin, but all other

combinations  of upper and lower case will lead to the predefined func-

tion.

For  a correct conversion of lower  case letters into capital letters a   DOS/DPMI

DOS version >= 3.30 is required.

2.11. Forward References and Other Disasters

--------------------------------------------

This  section is  the result  of a  significant amount  of hate  on the

(legal)  way some people program. This way  can lead to trouble in con-

junction  with AS in  some cases. The  section will deal with so-called

'forward  references'. What makes a  forward reference different from a

usual  reference? To understand the difference, take a look at the fol-

lowing  programming example (please excuse my  bias for the 68000 fami-

ly that is also present in the rest of this manual):

move.l  #10,d0

loop:   move.l  (a1),d1

beq     skip

neg.l   d1

skip:   move.l  d1,(a1+)

dbra    d0,loop

If  one overlooks  the loop  body with  its branch statement, a program

remains  that is  extremely simple  to assemble:  the only reference is

the  branch back to the body's  beginning, and as an assembler process-

es  a program from the beginning to  the end, the symbol's value is al-

ready  known before it is  needed the first time.  If one has a program

that  only contains such  backward references, one  has the nice situa-

tion  that only one pass through the  source code is needed to generate

a  correct and  optimal machine  code. Some  high level  languages like

Pascal  with their  strict rule  that everything  has to be defined be-

fore  it is used exploit exactly this property to speed up the compila-

tion.

Unfortunately,  things are  not that  simple in  the case of assembler,

because  one sometimes  has to  jump forward  in the  code or there are

reasons  why one has to move  variable definitions behind the code. For

our  example, this is the case for  the conditional branch that is used

to  skip over another  instruction. When the  assembler hits the branch

instruction  in the first pass, it  is confronted with the situation of

either  leaving blank all instruction fields  related to the target ad-

dress  or offering a value that  ''hurts noone'' via the formula parser

(which  has to evaluate the address  argument). In case of a ''simple''

assembler  that supports only one  target architecture with a relative-

ly  small number of  instructions to treat,  one will surely prefer the

first  solution, but the effort for AS with its dozens of target archi-

tectures  would have  become extremely  high. Only  the second  way was

possible:  If an unknown symbol is detected in the first pass, the for-

mula  parser delivers  the program  counter's current  value as result!

This  is the only  value suitable to  offer an address  to a branch in-

struction  with unknown distance  length that will  not lead to errors.

This  answers also a  frequently asked question  why a first-pass list-

ing  (it will not be  erased e.g. when AS  does not start a second pass

due  to additional errors) partially shows  wrong addresses in the gen-

erated  binary code - they are  the result of unresolved forward refer-

ences.

The  example listed above however  uncovers an additional difficulty of

forward  references: Depending  on the  distance of  branch instruction

and  target  in  the  source  code,  the  branch  may be either long or

short.  The  decision  however  about  the  code length - and therefore

about  the addresses of following labels -  cannot be made in the first

pass  due to  missing knowledge  about the  target address. In case the

programmer  did  not  explicitly  mark  whether  a long or short branch

shall  be used, genuine  2-pass assemblers like  older versions of MASM

from  Microsoft  ''solve''  the  problem  by  reserving  space  for the

longest  version  in  the  first  pass  (all label addresses have to be

fixed  after the first pass) and  filling the remaining space with NOPs

in  the  second  pass.  AS  versions  up  to 1.37 did the same before I

switched  to the  multipass principle  that removes  the strict separa-

tion  into two  passes and  allows an  arbitrary number of passes. Said

in  detail, the  optimal code  for the  assumed values  is generated in

the  first pass. In case  AS detects that values  of symbols changed in

the  second pass due  to changes in  code lengths, simply  a third pass

is  done,  and  as  the  second  pass'es  new symbol values might again

shorten  or  lengthen  the  code,  a  further pass is not impossible. I

have  seen 8086 programs  that needed 12  passes to get everything cor-

rect  and  optimal.  Unfortunately,  this  mechanism  does not allow to

specify  a maximum number passes; I can  only advise that the number of

passes  goes down when one makes more use of explicit length specifica-

tions.

Especially  for large programs, another  situation might arise: the po-

sition  of a forward  directed branch has  moved so much  in the second

pass  relative to the first  pass that the old  label value still valid

is  out of  the allowed  branch distance.  AS knows  of such situations

and  suppresses all error  messages about too  long branches when it is

clear  that another pass  is needed. This  works for 99%  of all cases,

but  there are also constructs where the first critical instruction ap-

pears  so early that AS  had no chance up  to now to recognize that an-

other  pass is needed.  The following example  constructs such a situa-

tion  with the  help of  a forward  reference (and  was the  reason for

this section's heading...):

cpu   6811

org     $8000

beq     skip

rept    60

ldd    Var

endm

skip:   nop

Var     equ     $10

Due  to the  address position,  AS assumes  long addresses in the first

pass  for the LDD  instructions, what results  in a code  length of 180

bytes  and an out  of branch error  message in the  second pass (at the

point  of the BEQ  instruction, the old  value of skip  is still valid,

i.e.  AS does not  know at this  point that the  code is only 120 bytes

long  in reality) is the result. The error can be avoided in three dif-

ferent ways:

1 Explicitly  tell AS  to use  short addressing  for the LDD instruc-

tions (ldd <Var)

2 Remove  this  damned,  rotten  forward  reference and place the EQU

statement  at the beginning where it has  to be (all right, I'm al-

ready calming down...)

3 For  real die-hards:  use the  -Y command  line option. This option

tells  AS to forget  the error message  when the address change has

been detected. Not pretty, but...

Another  tip regarding  the EQU  instruction: AS  cannot know  in which

context  a symbol defined with  EQU will be used,  so an EQU containing

forward  references will not  be done at  all in the  first pass. Thus,

if  the symbol defined  with EQU gets  forward-referenced in the second

pass:

move.l  #sym2,d0

sym2    equ     sym1+5

sym1    equ     0

one  gets an  error message  due to  an undefined  symbol in the second

pass...but why on earth do people do such things?

Admittedly,  this was quite  a lengthy excursion,  but I thought it was

necessary. Which is the essence you should learn from this section?

1 AS  always tries to  generate the shortest  code possible. A finite

number  of passes is  needed for this.  If you do  not tweak AS ex-

tremely, AS will know no mercy...

2 Whenever  sensible and possible, explicitly  specify branch and ad-

dress  lengths.  There  is  a  chance of significantly reducing the

number of passes by this.

3 Limit  forward references  to what  is absolutely  needed. You make

your and AS's live much easier this way!

2.12. Register Symbols

----------------------

valid  for:  PowerPC,  M-Core,  XGate,  4004/4040,  MCS-48/(2)51, 8086,

80C16x,  AVR, XS1,  Z8, KCPSM,  Mico8, MSP430(X),  ST9, M16,  M16C, H8/

300, H8/500, SH7x00, H16, i960, XA, 29K, TLCS-9000, KENBAK, SC/MP

Sometimes  it is desirable not only  to assign symbolic names to memory

addresses  or constants, but also to a register, to emphasize its func-

tion  in a certain  program section. This  is no problem for processors

that  treat registers simply  as another address  space, as this allows

to  use numeric expressions and one can  use simple EQUs to define such

symbols.  (e.g. for the MCS-96 or  TMS70000). However, for most proces-

sors,  register  identifiers  are  fixed  literals which are seperately

treated   by  AS  for  speed   reasons.  Therefore,  registers  symbols

(sometime  also called 'register aliases') are  also a separate type of

symbols  in the symbol table. Just like  other symbols, they may be de-

fined  or re-defined with  EQU or SET,  and there is  a specialized REG

instruction which accepts only symbols and expressions of this type.

On  the other  hand, register  symbols are  subject of  a couple of re-

strictions:  the number of  literals is limited  and depends on the se-

lected  target  processor,  and  arithmetic  operations are not possibl

eon registers A construct like tihs:

myreg   reg     r17         ; definition of register symbol

addi    myreg+1,3   ; does not work!

is not valid. Simple assignments are however possible:

myreg   reg     r17         ; definition of register symbol

myreg2  reg     myreg       ; myreg2 -> r17

Furthermore,  forward references are even  more critical than for other

types  of symbols. If a  symbol is not (yet)  defined, AS does not know

which  type it is  going to have,a  nd will decide  for a plain integer

number.  For most target processors, a  number is the equivalent of ab-

solute  memory  addressing,  and  on  most  processors, usage of memory

operands  is more  limited than  of registers.  Depending on situation,

one  will get  an error  message about  a non-allowed  addressing mode,

and no second pass will be started...

Analogous  to ordinary symbols, register  symbols are local to sections

and  it is possible  to access a  register symbol from  a specific sec-

tion by appending the section's name enclosed in brackets.

2.13. Share File

----------------

This  function is a by-product from  the old pure-68000 predecessors of

AS,  I have kept them in case  someone really needs it. The basic prob-

lem  is  to  access  certain  symbols produced during assembly, because

possibly  someone would like to access  the memory of the target system

via  this address  information. The  assembler allows  to export symbol

values  by means of  SHARED pseudo commands  (see there). For this pur-

pose,  the assembler  produces a  text file  with the  required symbols

and  its values in  the second pass.  This file may  be included into a

higher-level  language or another assembler  program. The format of the

text  file (C,  Pascal or  Assembler) can  be set  by the  command line

switches p, c or, a.

CAUTION!  If none of the  switches is given, no  file will be generated

and  it makes no  difference if SHARED-commands  are in the source text

or not!

When  creating a Sharefile, AS  does not check if  a file with the same

name  already exists,  such a  file will  be simply  overwritten. In my

opinion  a request does  not make sense,  because AS would  ask at each

run  if it should overwrite the old  version of the Sharefile, and that

would be really annoying...

2.14. Processor Aliases

-----------------------

Common  microcontroller families are like  rabbits: They become more at

a  higher speed than  you can provide  support for them. Especially the

development  of processor cores as building blocks for ASICs and of mi-

crocontroller  families with  user-definable peripherals  has led  to a

steeply  rising number  of controllers  that only  deviate from a well-

known  type by a slightly modified  peripheral set. But the distinction

among  them is  still important,  e.g. for  the design of include files

that  only define the appropriate subset  of peripherals. I have strug-

gled  up to now  to integrate the  most important reperesentatives of a

processor  family into AS (and  I will continue to  do this), but some-

times  I just cannot keep pace  with the development...there was an ur-

gent  need for a mechanism to extend  the list of processors by the us-

er.

The  result are  processor aliases:  the alias  command line option al-

lows  to define  a new  processor type,  whose instruction set is equal

to  another processor built into AS.  After switching to this processor

via  the CPU instruction, AS behaves exactly as if the original proces-

sor  had been used, with a  single difference: the variables MOMCPU re-

sp.  MOMCPUNAME are set to the alias  name, which allows to use the new

name for differentiation, e.g. in include files.

There  were two  reasons to  realize the  definition of  aliases by the

command  line and  not by  pseudo instructions:  first, it would anyway

be  difficult to put the alias definitions together with register defi-

nitions  into a  single include  file, because  a program that wants to

use  such a file would have to include  it before and after the CPU in-

struction  - an imagination  that lies somewhere  between inelegant and

impossible.  Second, the definition  in the command  line allows to put

the  definitions in a key file that is executed automatically at start-

up  via the ASCMD variable, without a  need for the program to take any

further care about this.

3. Pseudo Instructions

======================

Not  all pseudo  instructions are  defined for  all processors.  A note

that  shows the range of validity is therefore prepended to every indi-

vidual description.

3.1. Definitions

----------------

3.1.1. SET, EQU, and CONSTANT

- - - - - - - - - - - - - - -

valid for: all processors, CONSTANT only for KCPSM(3)

SET  and  EQU  allow  the  definition  of typeless constants, i.e. they

will  not be assigned  to a segment  and their usage  will not generate

warnings  because of  segment mixing.  EQU defines  constants which can

not  be  modified  (by  EQU)  again,  but SET permits the definition of

variables,  which can be  modified during the  assembly. This is useful

e.g.  for the allocation of resources  like interrupt vectors, as shown

in the following example:

VecCnt  set     0       ; somewhere at the beginning

.

.

.

DefVec  macro   Name    ; allocate a new vector

Name    equ     VecCnt

VecCnt  set     VecCnt+4

endm

.

.

.

DefVec  Vec1    ; results in Vec1=0

DefVec  Vec2    ; results in Vec2=4

constants  and variables are internally stored in the same way, the on-

ly  difference is that  they are marked  as unchangeable if defined via

EQU.  Trying to change a constant with SET will result in an error mes-

sage.

EQU/SET allow to define constants of all possible types, e.g.

IntTwo   equ    2

FloatTwo equ    2.0

Some  processors  unfortunately  have  already  a  SET instruction. For

these  targets, EVAL must be used  instead of SET if no differentiation

via  the argument  count is  possible. As  an alternative, it is always

possible  to explicitly invoke  the pseudo instruction  by prepending a

period (.SET instead of SET).

A  single equation sign or .EQU may  be used instead of EQU. Similarly,

one  may simply write := instead  of SET resp. EVAL. Furthermore, there

is  an 'alternate'  syntax that  does not  take the  symbol's name from

the  label  field,  but  instead  from  the  first argument. So for in-

stance, it is valid to write:

EQU   IntTwo,2

EQU   FloatTwo,2.0

For  compatibility reasons to the  original assembler, the KCPSM target

also  knows the CONSTANT  statement, which -  in contrast to  EQU - al-

ways expects name and value as arguments. For example:

CONSTANT  const1, 2

CONSTANT is however limited to integer constants.

Symbols  defined with SET or EQU are typeless by default, but optional-

ly  a segment  name (CODE,  DATA, IDATA,  XDATA, YDATA, BITDATA, IO, or

REG)  or MOMSEGMENT for the currently active  segment may be given as a

second  or third parameter, allowing to  assign the symbol to a specif-

ic  address space. AS does not check  at this point if the used address

space exists on the currently active target processor!

A  little hidden  extra feature  allows to  set the program counter via

SET  or EQU, something  one would ordinarily  do via ORG. To accomplish

this,  use the special  value as symbol  name that may  also be used to

query  the current program  counter's value. Depending  on the selected

target  architecture, this is either an  asterisk, a dollar sign, a pe-

riod, or PC.

In  case the target architecture supports instruction attributes to de-

fine  the operand size (e.g. on 680x0),  those are also allowed for SET

and  EQU. The operand size will be  stored along with the symbol's val-

ue in the symbol table. Its use is architecture-dependant.

3.1.2. SFR and SFRB

- - - - - - - - - -

valid for: various, SFRB only MCS-51

These  instructions act like EQU, but symbols defined with them are as-

signed  to the directly  addressable data resp.  I/O segment, i.e. they

are  preferrably used  for the  definition of  (as the name lets guess)

hardware  registers mapped  into the  data res.  I/O area.  The allowed

range  of values is equal to the range allowed for ORG in the data seg-

ment  (see section 3.2.1). The difference  between SFR and SFRB is that

SFRB  marks the register as bit  addressable, which is why AS generates

8  additional symbols  which will  be assigned  to the  bit segment and

carry the names xx.0 to xx.7, e.g.

PSW     sfr     0d0h    ; results in PSW = D0H (data segment)

PSW     sfrb    0d0h    ; results in extra PSW.0 = D0H (bit)

;               to PSW.7 = D7H (bit)

The  SFRB instruction is not any more  defined for the 80C251 as it al-

lows  direct bit access  to all SFRs  without special bit symbols; bits

like PSW.0 to PSW.7 are automatically present.

Whenever  a bit-addressable register is defined  via SFRB, AS checks if

the  memory address is bit addressable  (range 20h..3fh resp. 80h, 88h,

90h,  98h...0f8h). If  it is  not bit-addressable,  a warning is issued

and the generated bit symbols are undefined.

3.1.3.  XSFR and YSFR

- - - - - - - - - - -

valid for: DSP56xxx

Also  the DSP56000 has a few  peripheral registers memory-mapped to the

RAM,  but the affair becomes complicated because there are two data ar-

eas,  the X-  and Y-area.  This architecture  allows on  the one hand a

higher  parallelism, but forces on the  other hand to divide the normal

SFR  instruction into  the two  above mentioned  variations. They works

identically  to SFR, just that XSFR defines a symbol in the X- address-

ing  space and YSFR a corresponding  one in the Y-addressing space. The

allowed value range is 0..$ffff.

3.1.4.  LABEL

- - - - - - -

valid for: all processors

The  function of  the LABEL  instruction is  identical to  EQU, but the

symbol  does not become  typeless, it gets  the attribute ''code''. LA-

BEL  is needed  exactly for  one purpose:  Labels are normally local in

macros,  that means  they are  not accessible  outside of a macro. With

an EQU instruction you could get out of it nicely, but the phrasing

<name>  label   $

generates a symbol with correct attributes.

3.1.5.  BIT

- - - - - -

valid  for: MCS/(2)51, XA, 80C166, 75K0,  ST9, AVR, S12Z, SX20/28, H16,

H8/300, H8/500, KENBAK, Padauk

BIT  serves to  equate a  single bit  of a  memory cell with a symbolic

name.  This instruction varies from  target platform to target platform

due  to the different ways in  which processors handle bit manipulation

and addressing:

The  MCS/51 family has an own address space for bit operands. The func-

tion  of BIT is therefore  quite similar to SFR,  i.e. a simple integer

symbol  with the specified value is  generated and assigned to the BDA-

TA  segment. For all other processors, bit addressing is done in a two-

dimensional  fashion with address and bit  position. In these cases, AS

packs  both parts into an  integer symbol in a  way that depends on the

currently  active target processor and  separates both parts again when

the  symbol is used. The latter is  is also valid for the 80C251: While

an instruction like

My_Carry bit    PSW.7

would  assign  the  value  0d7h  to  My_Carry  on  an  8051, a value of

070000d0h  would be generated on an  80C251, i.e. the address is locat-

ed  in bits 0..7  and the bit  position in bits  24..26. This procedure

is  equal to the  way the DBIT  instruction handles things  on a TMS370

and  is also used on the 80C166,  with the only difference that bit po-

sitions may range from 0..15:

MSB     BIT     r5.15

On  a Philips XA, the  bit's address is located  in bits 0..9 just with

the  same coding as used  in machine instructions, and  the 64K bank of

bits in RAM memory is placed in bits 16..23.

The  BIT instruction of the  75K0 family even goes  further: As bit ex-

pressions  may not only  use absolute base  addresses, even expressions

like

bit1    BIT     @h+5.2

are allowed.

The  ST9 in  turn allows  to invert  bits, what  is also allowed in the

BIT instruction:

invbit  BIT     r6.!3

More  about the  ST9's BIT  instruction can  be found  in the processor

specific hints.

In  case of H16, note  that the address and  bit position arguments are

swapped.  This was done to  make the syntax of  BIT consistent with the

machine instructions that maipulate individual bits.

3.1.6. DBIT

- - - - - -

valid for: TMS 370xxx

Though  the TMS370 series  does not have  an explicit bit segment, sin-

gle  bit symbols may be simulated  with this instruction. DBIT requires

two  operands, the  address of  the memory  cell that  contains the bit

and the exact position of the bit in the byte. For example,

INT3        EQU  P019

INT3_ENABLE DBIT 0,INT3

defines  the bit  that enables  interrupts via  the INT3  pin. Bits de-

fined  this way may  be used in  the instructions SBIT0, SBIT1, CMPBIT,

JBIT0, and JBIT.

3.1.7. DEFBIT and DEFBITB

- - - - - - - - - - - - -

S12Z

....

The  S12Z family's  processor core  provides instructions to manipulate

individual  bits in registers or  memory cells. To conveniently address

bits  in the CPU's  I/O area (first  4 Kbytes of  the address space), a

bit  may be given a symbolic name. The bit is defined by its memory ad-

dress and the bit position:

<name>         defbit[.size]   <address>,<position>

The  address  must  be  located  within  the  first  4  Kbytes, and the

operand  size may be 8, 16,  or 32 bits (size=b/w/l). Consequently, the

position  may at  most be  7, 15  or 31.  If no  operand size is given,

byte  size (.b) is assumed. A bit defined this way may be used as argu-

ment for the instructions BCLR, BSET, BTGL, BRSET, and BRCLR:

mybit   defbit.b  $200,4

bclr.b    $200,#4

bclr      mybit

Both  uses of  bclr in  this example  generate identical  code. Since a

bit  defined this  way ''knows''  its size,  the size  attribute may be

omitted when using it.

It  is also possible  to define bits  that are located  within a struc-

ture's element:

mystruct struct    dots

reg      ds.w      1

flag     defbit    reg,4

ends

org       $100

data     mystruct

bset      data.flag  ; same as bset.w $100,#4

Super8

......

Opposed  to the 'classic' Z8, the  Super8 core supports instructions to

operate  on bits  in working  or general  registers. ONe however has to

to  regard that some of them can only  operate on bits in one of the 16

working  registers. The DEFBIT instruction allows to define bits of ei-

ther type:

workbit defbit  r3,#4

slow    defbit  emt,#6

Bits  that have been defined this way  may be used just like a argument

duple of register and bit position:

ldb     r3,emt,#6

ldb     r3,slo          ; same result

bitc    r3,#4

bitc    workbit         ; same result

Z8000

.....

The  Z8000 features  instructions to  set and  clear bits, however they

cannot  access addresses  in I/O  space. For  this reason,  both DEFBIT

and  DEFBITB only allow to define bit objects in memory space. The dif-

ferentiation  in operand size  is important because  the Z8000 is a big

endian  processor: bit n of  a 16 bit word  at address m corresponds to

bit n of an 8-bit byte at address m+1.

�PD7807...�PD7809

.................

The  lowest 16 bytes of the working  area and special registers with an

address less than 16 are bit addressable.

3.1.8.  DEFBITFIELD

- - - - - - - - - -

valid for: S12Z

The  S12Z family's CPU core not only  deals with individual bits, it is

also  able to  extract a  field of  consecutive bits from an 8/16/24/32

value  or to insert a  bit field into such  a value. Similar to DEFBIT,

a bit field may be defined symbolically:

<Name>     defbitfield[.size] <address>,<width>:<position>

Opposed  to individual bits,  an operand size  of 24 bits  (.p) is also

alloweed.  The range of position  and width is accordingly  0 to 23 re-

sp.  1 to  24. It  is also  allowed to  define bit  fields as  parts of

structures:

mystruct struct      dots

reg      ds.w        1

clksel   defbitfield reg,4:8

ends

org       $100

data     mystruct

bfext     d2,data.clksel ; fetch $100.w bits 4..11

; to D2 bits 0..7

bfins     data.clksel,d2 ; insert D2 bits 0..7 into

; $100.w bits 4..11

The  internal representation of  bits defined via  DEFBIT is equivalent

to  bit fields with  a width of  one. Therefore, a symbolically defined

bit may also be used as argument for BFINS and BFEXT.

3.1.9. PORT

- - - - - -

valid  for: PALM, 8008/8080/8085/8086, XA, Z80, Z8000, 320C2x/5x, TLCS-

42/47, AVR, F8, IMP-16

PORT  works similar to EQU, just the  symbol becomes assigned to the I/

O-address  range.  Allowed  values  are  0..7  for  the 3201x and 8008,

0..15  for  the  320C2x  and  PALM,  0..65535  for the 8086, Z8000, and

320C5x, 0..63 for the AVR, and 0..255 for the rest.

Example : an 8255 PIO is located at address 20H:

PIO_port_A port 20h

PIO_port_B port PIO_port_A+1

PIO_port_C port PIO_port_A+2

PIO_ctrl   port PIO_port_A+3

3.1.10. REG and NAMEREG

- - - - - - - - - - - -

valid for: 680x0, AVR, M*Core, ST9, 80C16x, Z8000, KCPSM,

PDP-11, WD16

( NAMEREG valid only for KCPSM(3)), LatticeMico8, MSP430(X)

Though  it always has the same  syntax, this instruction has a slightly

different  meaning from processor  to processor: If  the processor uses

a  separate addressing space for registers,  REG has the same effect as

a  simple EQU for  this address space  (e.g. for the  ST9). REG defines

register  symbols for all other  processors whose function is described

in section 2.12.

NAMEREG  exists for compatibility reasons  to the original KCPSM assem-

bler.  It has an identical function,  however both register and symbol-

ic name are given as arguments, for example:

NAMEREG  s08, treg

On  the PDP-11, REG may additionally be  used without a name in the la-

bel  field. It then expects a single  ON or OFF as argument and enables

or  disables the  built-in register  aliases (Rn  = %n,  SP =  R6, PC =

R7).  They are  available by  default, and  should only  be disabled if

they  conflict with  own synmbol  names in  a program. The current set-

ting may be read from the symbol DEFAULT_REGSYMS.

3.1.11. LIV and RIV

- - - - - - - - - -

valid for: 8X30x

LIV  and RIV  allow to  define so-called  ''IV bus objects''. These are

groups  of bits located in a peripheral  memory cell with a length of 1

up  to 8 bits, which can afterwards be referenced symbolically. The re-

sult  is that one  does not anymore  have to specify address, position,

and  length separately  for instructions  that can  refer to peripheral

bit  groups.  As  the  8X30x  processors feature two peripheral address

spaces  (a  ''left''  and  a  ''right''  one),  there  are two separate

pseudo  instructions. The parameters of  these instructions are however

equal:  three parameters have  to be given  that specify address, start

position  and length.  Further hints  for the  usage of bus objects can

be found in section 4.25 .

3.1.12. CHARSET

- - - - - - - -

valid for: all processors

Single  board  systems,  especially  when  driving LCDs, frequently use

character  sets different to ASCII. So it is probably purely coinciden-

tal  that the umlaut  coding corresponds with  the one used  by the PC.

And  there are of course also  (historical) systems that use some vari-

ant  of  EBCDIC...to  avoid  error-prone  manual encoding in the source

code,  the assembler contains a  translation table for characters which

assigns  a target  character to  each (ASCII)  character in  the source

code.  Use the CHARSET instruction to  modify this table, which initial

translates  one-to-one. CHARSET  may be  used with  a variety  of argu-

ments:

A simple

CHARSET

without any argument resets the table to the one-to-one default.

If  only a single argument  is given, it has  to be a string expression

which is interpreted as a file name by AS:

CHARSET  "mapping.bin"

AS  reads the first 256 bytes from  this table and copies them into the

translation  table. This allows to  activate complex, externally gener-

ated tables with a single statement.

All  other variants modify a  single entry or a  sequence of entries in

the table. Use two (integer) arguments to change a single entry:

CHARSET '�',128

means  that the target system codes the  '�' into the number 128. It is

als  possible to define that a  certain character is unavailable on the

target system. Leave the second argument empty to define this:

CHARSET '[',

If  the 'deleted' character shall be  disposed in memory, this reported

as an error.

Use  three arguments  to remap  a whole  range of characters. The first

and  second argument define the character  range, and the third one de-

fines  the mapping of the first  character. For instance, if the target

system does not support lower case characters,

CHARSET 'a','z','A'

translates  all lower-case  characters automatically  into the matching

capital  letters. Similar  to a  single character,  it is also possible

to 'unmap' a range of characters:

CHARSET 'a','z',

forbids usage of lower case letters.

The  last variant  (again only  with two  arguments), a  string defines

the  mapping of  a sequence  of characters.  Mapping of  lower to upper

case may therefore also be written like this: be written as

CHARSET 'a',"ABCDEFGHIJKLMNOPQRSTUVWXYZ"

CAUTION!  CHARSET not only  affects string constants  stored in memory,

but  also multi character constants,  i.e. integer constants written as

''ASCII''.  This means that  an already modified  translation table can

lead to different results in the examples mentioned above!

The  built-in function CODEPAGE_VAL allows  to query the translation of

a  single character in the current code page. It will return -1 for un-

mapped characters.

3.1.13.  CODEPAGE

- - - - - - - - -

valid for: all processors

Though  the CHARSET statement gives  unlimited freedom in the character

assignment  between host  and target  platform, switching among differ-

ent  character sets can become quite  tedious if several character sets

have  to be supported on the  target platform. The CODEPAGE instruction

however  allows  to  define  and  keep  different character sets and to

switch  with a  single statement  among them.  CODEPAGE expects  one or

two  arguments: the name of the set  to be used hereafter and optional-

ly  the name  of another  table that  defines its initial contents (the

second  parameter therefore only has a  meaning for the first switch to

the  table when AS  automatically creates it).  If the second parameter

is  missing, the initial contents of the  new table are copied from the

previously  active set.  All subsequent  CHARSET statements only modify

the new set.

At  the beginning of a pass,  AS automatically creates a single charac-

ter  set with  the name  STANDARD with  a one-to-one translation. If no

CODEPAGE  instructions are  used, all  settings made  via CHARSET refer

to this table.

3.1.14.  ENUM, NEXTENUM, and ENUMCONF

- - - - - - - - - - - - - - - - - - -

valid for: all processors

Similar  to the  same-named instruction  known from  C, ENUM is used to

define  enumeration types,  i.e. a  sequence of  integer constants that

are  assigned sequential values  starting at 0.  The parameters are the

names of the symbols, like in the following example:

ENUM    SymA,SymB,SymC

This  instruction will  assign the  values 0,  1, and  2 to the symbols

SymA, SymB, and SymC.

If  you want to split an enumeration  over more than one line, use NEX-

TENUM  instead of ENUM for the second  and all following lines. The in-

ternal  counter  that  assigns  sequential  values to alls symbols will

then not be reset to zero, like in the following case:

ENUM     January=1,February,March,April,May,June

NEXTENUM July,August,September,October

NEXTENUM November,December

This  example also demonstrates that it  is possible to assign explicit

values  to individual symbols. The internal counter will be updated ac-

cordingly if this feature is used.

A  definition of a symbol with ENUM  is equal to a definition with EQU,

i.e.  it is  not possible  to assign  a new  value to a symbol that al-

ready exists.

The  ENUMCONF  statement  allows  to  influence  the behaviour of ENUM.

ENUMCONF  accepts one  or two  arguments. The  first argument is always

the  value the internal  counter is incremented  for every symbol in an

enumeration. For instance, the statement

ENUMCONF 2

has  the  effect  that  symbols  get  the  values 0,2,4,6... instead of

0,1,2,3...

The  second (optional) argument  of ENUMCONF rules  which address space

the  defined symbols  are assigned  to. By  default, symbols defined by

ENUM are typeless. For instance, the statement

ENUMCONF 1,CODE

defines  that  they  should  be  assigned  to  the  instruction address

space.  The names of  the address spaces  are the same  as for the SEG-

MENT  instruction (3.2.18),  with the  addition of  NOTHING to generate

typeless symbols again.

3.1.15.  PUSHV and POPV

- - - - - - - - - - - -

valid for: all processors

PUSHV  and POPV allow to  temporarily save the value  of a symbol (that

is  not macro-local) and  to restore it  at a later  point of time. The

storage  is done  on stacks,  i.e. Last-In-First-Out memory structures.

A  stack has a  name that has  to fulfill the  general rules for symbol

names  and it  exists as  long as  it contains  at least one element: a

stack  that did not  exist before is  automatically created upon PUSHV,

and  a stack becoming  empty upon a  POPV is deleted automatically. The

name  of the  stack that  shall be  used to  save or restore symbols is

the  first parameter of PUSH resp. POPV,  followed by a list of symbols

as  further  parameters.  All  symbols  referenced  in the list already

have  to exist, it is therefore  not possible to implicitly define sym-

bols with a POPV instruction.

Stacks  are a global resource,  i.e. their names are  not local to sec-

tions.

It  is important  to note  that symbol  lists are always processed from

left  to  right.  Someone  who  wants  to  pop several variables from a

stack  with a POPV  therefore has to  use the exact  reverse order used

in the corresponding PUSHV!

The name of the stack may be left blank, like this:

pushv   ,var1,var2,var3

.

.

popv    ,var3,var2,var1

AS will then use a predefined internal default stack.

AS  checks at the end of a pass  if there are stacks that are not empty

and  issues their names together with their ''filling level''. This al-

lows  to find out if  there are any unpaired  PUSHVs or POPVs. However,

it  is in no case possible to save  values in a stack beyond the end of

a pass: all stacks are cleared at the beginning of a pass!

3.2. Code Modification

----------------------

3.2.1.  ORG

- - - - - -

valid for: all processors

ORG  allows to  load the  internal address  counter (of  the assembler)

with  a new  value. The  value range  depends on the currently selected

segment  and on the processor type (table  3.1). The lower bound is al-

ways zero, and the upper bound is the given value minus 1.

CAUTION:  If the  PHASE instruction  is also  used, one  has to keep in

mind  that the argument of ORG always  is the load address of the code.

Expressions  using the $  or * symbol  to refer to  the current program

counter  however deliver the  execution address of  the code and do not

yield  the  desired  result  when  used  as  argument for ORG. The RORG

statement (3.2.2) should be used in such cases.

Table 3.1: Address Ranges for ORG

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Target     |  CODE  |  DATA |   I-  |   X-  |  Y-  | BIT- |   IO  | REG | ROM- |  EE-  |

|            |        |       |  DATA |  DATA | DATA | DATA |       |     | DATA |  DATA |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68xxx/     |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| MCF        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| DSP56000   |  64K/  |  ---  |  ---  |  64K/ | 64K/ |  --- |  ---  | --- |  --- |  ---  |

| DSP56300   |   16M  |       |       |  16M  |  16M |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PowerPC    |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PALM       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| M*Core     |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6800,6301, |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 6811       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6805/      |   8K/  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| HC08       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6809,      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 6309,      |        |       |       |       |      |      |       |     |      |       |

| 052001     |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68HC12,    |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 68HC12X,   |        |       |       |       |      |      |       |     |      |       |

| XGATE      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| S12Z       |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68HC16     |   1M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68RS08     |   16K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| H8/300     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| H8/300H    |   16M  |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| H8/500     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| (Min)      |        |       |       |       |      |      |       |     |      |       |

| H8/500     |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| (Max)      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SH7000/    |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 7600/7700  |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| HD614023   |   2K   |  160  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

| HD614043   |   4K   |  256  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

| HD614081   |   8K   |  512  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| HD641016   |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6502,      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| MELPS-     |        |       |       |       |      |      |       |     |      |       |

| 740        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| HUC6280    |   2M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 65816,     |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| MELPS-     |        |       |       |       |      |      |       |     |      |       |

| 7700       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PPS-4      |   4K   |   4K  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MELPS-     |   8K   |  416  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 4500       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| M16        |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| M16C       |   1M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PDP-11     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |  256K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |  4M^10 |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| VAX        |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| WD16       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 4004       |   4K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 8008       |   16K  |   8   |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MCS-48,    | 1/2/4/ |  ---  |  256  | 256^8 |  --- |  --- |  ---  | --- |  --- |  ---  |

| MCS-41     | 6/8K^6 |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MCS-51     |   64K  |  256  | 256^1 |  64K  |  --- |  256 |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 80C390     |   16M  |  256  | 256^1 |  16M  |  --- |  256 |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MCS-251    |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  512  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MCS-96     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 196(N)/    |   16M  |       |       |       |      |      |       |     |      |       |

| 296        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 8080,      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

| 8085       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 80x86,     |   64K  |  64K  |  ---  |  64K  |  --- |  --- |  64K  | --- |  --- |  ---  |

| V20..55    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 68xx0      |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 8X30x      |   8K   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 2650       |   32K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| XA         |   16M  |  16M  |  ---  |  ---  |  --- |  --- |  2K^3 | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| AVR        | 128K^6 | 32K^6 |  ---  |  ---  |  --- |  --- |   64  | --- |  --- |  8K^7 |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 29XXX      |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 80C166,    |  256K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 80C167     |   16M  |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| GBZ80      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---- | --- |  --- |  ---  |

| Z80,       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

| Z180,      | 512K^2 |       |       |       |      |      |  256  |     |      |  ---  |

| eZ80,      |   16M  |       |       |       |      |      |  64K  |     |      |  ---  |

| Z380       |   4G   |       |       |       |      |      |   4G  |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Z8         |   64K  |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| eZ8        |   64K  |  256  |  ---  |  64K  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Z8001,     |   8M   |  ---  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

| Z8003      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Z8002,     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

| Z8004      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| KCPSM      |   256  |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| KCPSM3     |   256  |   64  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| Mico8      |  4096  |  256  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-      |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 900(L)     |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-90    |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 870(/C)    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-42    |  512,  |   32  |  ---  |  ---  |  --- |  --- |   3,  | --- |  --- |  ---  |

| TLCS-42    |  1024  |       |       |       |      |      |   6   | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-47    |   64K  |   1K  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TLCS-      |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 9000       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TC9331     |   320  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PIC        |   2K   |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 16C5x      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PIC        |   2K   |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 16C5x      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PIC        |        |       |       |       |      |      |       |     |      |       |

| 16C64,     |   8K   |  512  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- | 256^6 |

| 16C86      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PIC        |   64K  |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 17C42      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SX20       |   2K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ST6        |   4K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ST7        |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| STM8       |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ST9        |   64K  |  64K  |  ---  |  ---  |  --- |  --- |  ---  | 256 |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 6804       |   4K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 32010      |   4K   |  144  |  ---  |  ---  |  --- |  --- |   8   | --- |  --- |  ---  |

| 32015      |   4K   |  256  |       |       |      |      |   8   |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C2x     |   64K  |  64K  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C3x     |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C40     |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C44     |   32M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 320C5x/    |   64K  |  64K  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

| 320C20x/   |        |       |       |       |      |      |       |     |      |       |

| 320C54x    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TMS        |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 9900       |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TMS        |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 70Cxx      |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 370xxx     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSP430     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TMS1000    |   1K   |   64  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| TMS1200    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| TMS1100    |   2K   |  128  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| TMS1300    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| IMP-16     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  128  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| IPC-16     |   16K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SC/MP      |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 807x       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| COP4       |   512  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| COP8       |   8K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SC144xx    |   256  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| NS16008/   |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| NS32008/   |        |       |       |       |      |      |       |     |      |       |

| NS08032/   |        |       |       |       |      |      |       |     |      |       |

| NS16032/   |        |       |       |       |      |      |       |     |      |       |

| NS32016/   |        |       |       |       |      |      |       |     |      |       |

| NS32032/   |        |       |       |       |      |      |       |     |      |       |

| NS32CG16   |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| NS32332/   |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| NS32532    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ACE        |  4K^4  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| CP-3F/     |   16K  |   48  |  ---  |  ---  |  --- |  --- |   8   |     |      |       |

| M380/      |        |       |       |       |      |      |       |     |      |       |

| LP8000     |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| F3850      |   64K  |   64  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

| F8         |   4K   |   64  |  ---  |  ---  |  --- |  --- |  256  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 78(C)xx    |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |   640  |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 550        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |  1000  |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 554, 652   |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |  1000  |   64  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 547, 552   |        |       |       |       |      |      |       |     |      |       |

| 651        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| �PD        |  2000  |   96  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| 546, 553   |        |       |       |       |      |      |       |     |      |       |

| 556, 557   |        |       |       |       |      |      |       |     |      |       |

| 650        |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 7566       |   1K   |   64  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 7508       |   4K   |  256  |  ---  |  ---  |  --- |  --- |   16  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 75K0       |   16K  |   4K  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 78K0       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 78K2       |   1M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 78K3       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 78K4       |  16M^5 |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 7720       |   512  |  128  |  ---  |  ---  |  --- |  --- |  ---  | --- |  512 |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 7725       |   2K   |  256  |  ---  |  ---  |  --- |  --- |  ---  | --- | 1024 |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 77230      |   8K   |  ---  |  ---  |  512  |  512 |  --- |  ---  | --- |  1K  |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 70616      |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  16M  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 53C8XX     |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| F^2MC8L    |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| F^2MC16L   |   16M  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5840    |   2K   |  128  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5842    |   768  |   32  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM58421   |  1.5K  |   40  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

| MSM58422   |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5847    |  1.5K  |   96  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5054    |   1K   |   62  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5055    |  1.75K |   96  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM5056    |  1.75K |   90  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MSM6051    |  2.5K  |  119  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MN1610     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| MN1613     |  256K  |  ---  |  ---  |  ---  |  --- |  --- |  64K  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| PMCxxx/    |   1..  |  64.. |  ---  |  ---  |  --- |  --- |  32.. | --- |  --- |  ---  |

| PMSxxx/    |  4K^9  | 256^9 |       |       |      |      | 128^9 |     |      |       |

| PFSxxx     |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 180x       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |   8   | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| XS1        |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| 1750       |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| KENBAK     |   256  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| CP1600     |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| NANO       |   2K   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| IM6100     |   4K   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| IM6120     |   32K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| RX...      |   4G   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SC61860    |   64K  |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| SC62015    |   1M   |  ---  |  ---  |  ---  |  --- |  --- |  ---  | --- |  --- |  ---  |

|            |        |       |       |       |      |      |       |     |      |       |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^1 Initial value 80h.                                                                  |

| As the 8051 does not have any RAM beyond 80h, this value has to be                     |

| adapted with ORG for the 8051 as target processor!                                     |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^2 As the Z180 still can address only 64K logically, the whole                         |

| address space can only be reached via PHASE instructions!                              |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^3 initial value 400h.                                                                 |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^4 initial value 800h resp. 0C00h                                                      |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^5 area for program code is limited to 1 MByte                                         |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^6 size depends on target processor                                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^7 size and availibility depend on target processor                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^8 only on variants supporting the MOVX instruction                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^9 device dependent                                                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

| ^10 model dependent                                                                    |

+------------+--------+-------+-------+-------+------+------+-------+-----+------+-------+

In  case that different  variations in a  processor family have address

spaces of different size, the maximum range is listed for each.

ORG  is mostly  needed to  give the  code a  new starting address or to

put  different,  non-continuous  code  parts  into  one source file. In

case  there is  no explicit  other value  listet in  a table entry, the

initial  address for this segment (i.e.  the start address used without

ORG) is 0.

3.2.2. RORG

- - - - - -

valid for: all processors

RORG  modifies the program  counter just like  ORG, however it does not

expect  an absolute  address as  argument. Instead,  it expects a rela-

tive  value (positive  or negative)  that is  added to the current pro-

gram  counter. A possible application of this statement is the reserva-

tion  of a certain  amount of address  space, or the  use in code parts

that  are included  multiple times  (e.g. via  macros or  includes) and

that  shall be position-independent. Another  application is the use in

code  that has  an execution  address different  from the  load address

(i.e.  the PHASE  statement is  used). There  is no  symbol to refer to

the  current load  address, but  it can  be referred  to indirectly via

the RORG statement.

3.2.3.  CPU

- - - - - -

valid for: all processors

This  command rules for which processor  the further code shall be gen-

erated.  Instructions of  other processor  families are  not accessible

afterwards and will produce error messages!

The  processors can  roughly be  distinguished in  families, inside the

families  different types  additionally serve  for a  detailed distinc-

tion:

a) 68008 -> 68000 -> 68010 -> 68012 ->

MCF5202 -> MCF5204 -> MCF5206 -> MCF5208->

MCF52274 -> MCF52277 -> MCF5307 -> MCF5329 ->

MCF5373 -> MCF5407 -> MCF5470 -> MCF5471 ->

MCF5472 -> MCF5473 -> MCF5474 -> MCF5475 ->

MCF51QM ->

68332 -> 68340 -> 68360 ->

68020 -> 68030 -> 68040

The  differences  in  this  family  are additional instructions and ad-

dressing  modes (starting  from the  68020). A  small exception  is the

step  to the  68030 that  misses two  instructions: CALLM  and RTM. The

three  representatives  of  the  683xx  family  have the same processor

core  (a slightly reduced 68020  CPU), however completely different pe-

ripherals.  MCF5xxx represents various ColdFire variants from Motorola/

Freescale/NXP,  RISC  processors  downwardly  binary  compatible to the

680x0.  For  the  68040,  additional  control  registers (reachable via

MOVEC)  and  instructions  for  control  of  the on-chip MMU and caches

were added.

b) 56000 --> 56002 --> 56300

While  the 56002 only adds instructions for incrementing and decrement-

ing  the accumulators,  the 56300  core is  almost a new processor: all

address  spaces are enlarged  from 64K words  to 16M and  the number of

instructions almost has been doubled.

c) PPC403 -> MPPC403 -> MPC505 -> MPC601

-> MPC821 -> RS6000

The  PPC403 is a reduced  version of the PowerPC  line without a float-

ing  point unit, which is why  all floating point instructions are dis-

abled  for  him;  in  turn,  some microcontroller-specific instructions

have  been added  which are  unique in  this family.  The GC variant of

the  PPC403 incorporates an additional MMU and has therefore some addi-

tional  instructions  for  its  control.  The MPC505 (a microcontroller

variant  without a  FPU) only  differ in  its peripheral registers from

the  601 as long as I  do not know it better  - [82] is a bit reluctant

in  this  respect...  The  RS6000  line  knows  a few instructions more

(that  are emulated on  many 601-based systems),  IBM additionally uses

different  mnemonics for their pure  workstation processors, as a remi-

niscence of 370 mainframes...

d) IBM5100, IBM5110, IBM5120

These  three types currently  all reference to  the same (PALM) prozes-

sor core.

e) MCORE

f) XGATE

g) 6800 -> 6801 -> 6301 -> 6811

While  the  6801  only  offers  a  few additional instructions (and the

6301  even a few more),  the 6811 provides a  second index register and

much more instructions.

h) 6809/6309 and 6805/68HC(S)08

These  processors  are  partially  source-code  compatible to the other

68xx  processors, but  they have  a different  binary code format and a

significantly  reduced  (6805)  resp.  enhanced (6809) instruction set.

The  6309 is a CMOS  version of the 6809  which is officially only com-

patible  to the 6809, but inofficially  offers more registers and a lot

of new instructions (see [56]).

i) 68HC12 --> 68HC12X

The  12X core  offers a  couple of  new instructions,  and existing in-

structions were were enriched with new addressing modes.

j) S912ZVC19F0MKH, S912ZVC19F0MLF,

S912ZVCA19F0MKH, S912ZVCA19F0MLF,

S912ZVCA19F0WKH, S912ZVH128F2CLQ,

S912ZVH128F2CLL, S912ZVH64F2CLQ,

S912ZVHY64F1CLQ, S912ZVHY32F1CLQ,

S912ZVHY64F1CLL, S912ZVHY32F1CLL,

S912ZVHL64F1CLQ, S912ZVHL32F1CLQ,

S912ZVHL64F1CLL, S912ZVHL32F1CLL,

S912ZVFP64F1CLQ, S912ZVFP64F1CLL,

S912ZVH128F2VLQ, S912ZVH128F2VLL,

S912ZVH64F2VLQ, S912ZVHY64F1VLQ,

S912ZVHY32F1VLQ, S912ZVHY64F1VL,

S912ZVHY32F1VLL, S912ZVHL64F1VLQ

All  variants contain the same processor  core and the same instruction

set,  only the  on-chip peripherals  and the  amount of built-in memory

(RAM, Flash-ROM, EEPROM) vary from device to device.

k) 68HC16

l) 052001

This  chip is  an own  creation of  Konami and  similar to the Motorola

6809  in architecture  an instruction  set. However,  it is  not binary

compatible  and does not provide  all instructions and addressing modes

of its 'role model'.

m) HD6413308 -> HD6413309

These  both names represent the 300 and 300H variants of the H8 family;

the  H version  owns a  larger address  space (16  Mbytes instead of 64

Kbytes),  double-width registers  (32 bits),  and knows  a few more in-

structions  and addressing  modes. It  is still  binary upward compati-

ble.

n) HD6475328 -> HD6475348 -> HD6475368 -> HD6475388

These  processors all share the same  CPU core; the different types are

only  needed to  include the  correct subset  of registers  in the file

REG53X.INC.

o) SH7000 -> SH7600 --> SH7700

The  processor core  of the  7600 offers  a few  more instructions that

close  gaps in the 7000's instruction set (delayed conditional and rel-

ative  and  indirect  jumps,  multiplications  with 32-bit operands and

multiply/add  instructions). The 7700  series (also known  as SH3) fur-

thermore  offers  a  second  register  bank, better shift instructions,

and instructions to control the cache.

p) HD614023 --> HD614043 --> HD614081

These  three variants of the  HMCS400 series differ by  the size of the

internal ROM and RAM.

q) HD641016

This is currently the only target with H16 core.

r) 6502 -> 65(S)C02

-> 65CE02 / W65C02S / 65C19 /

MELPS740 / HUC6280 / 6502UNDOC

The  CMOS version  defines some  additional instructions,  as well as a

number  of  some  instruction/addressing  mode  combinations were added

which  were not possible on  the 6502. The W65C02S  adds two opcodes to

the  65C02 instruction set  to give more  fine-grained control over how

to  stop the CPU for low power  modes. The 65SC02 lacks the bit manipu-

lation  instructions of the 65C02.  The 65CE02 adds branch instructions

with  16-bit displacement, a Z register, a 16 bit stack pointer, a pro-

grammable base page, and a couple of new instructions.

The  65C19 is not  binary upward compatible  to the original 6502! Some

addressing  modes  have  been  replaced  by  others.  Furthermore, this

processor  contains instruction set  extensions that facilitate digital

signal processing.

The  Mitsubishi micro controllers in  opposite expand the 6502 instruc-

tion  set primarily to bit operations and multiplication / division in-

structions.  Except for the unconditional  jump and instructions to in-

crement/decrement  the  accumulator,  the  instruction  extensions have

nothing in common.

For  the HuC 6280, the  feature that sticks out  most is the larger ad-

dress  space of 2 MByte  instead of 64 KBytes.  This is achieved with a

buil-tin  banking mechanism. Furthermore, it  features some special in-

structions  to communicate with  a video processor  (this chip was used

in video games) and to copy memory areas.

The  6502UNDOC  processor  type  enables  access  to the "undocumented"

6502  instructions, i.e. the  operations that result  from the usage of

bit  combinations in the  opcode that are  not defined as instructions.

The  variants supported  by AS  are listed  in the  appendix containing

processor-specific hints.

s) MELPS7700, 65816

Apart  from  a  '16-bit-version'  of  the 6502's instruction set, these

processors  both offer some instruction  set extensions. These are how-

ever  orthogonal as  they are  oriented along  their 8-bit predecessors

(65C02  resp. MELPS-740).  Partially, different  mnemonics are used for

the same operations.

t) PPS-4

u) MELPS4500

v) M16

w) M16C

x) PDP-11/03, PDP-11/04, PDP-11/05, PDP-11/10,

PDP-11/15, PDP-11/20, PDP-11/23, PDP-11/24,

PDP-11/34, PDP-11/35, PDP-11/40, PDP-11/44,

PDP-11/45, PDP-11/50, MicroPDP-11/53,

PDP-11/55, PDP-11/60, PDP-11/70, MicroPDP-11/73,

MicroPDP-11/83, PDP-11/84, MicroPDP-11/93,

PDP-11/94, T-11

The  various  models  of  the  PDP-11  series differ in instruction set

(both  in  built-in  instructions  as  well as in available extensions)

and the supported address space (64, 256, or 4096 KBytes).

y) WD16

The  WD16 uses the  same processor as  the LSI-11, however with differ-

ent  microcode.  As  a  consequence,  register set and addressing modes

are  the same as for a PDP-11,  however the instruction set is slightly

different  and instructions also  available on the  PDP-11 have differ-

ent machine codes.

z) MICROVAX-I, MICROVAX-II,

VAX-11/725, VAX-11/730, VAX-11/750,

VAX-11/780, VAX-11/782, VAX-11/785,

VAX-8200, VAX-8300, VAX-8500, VAX-8600

VAX-8650, VAX-8800

All  implementations of  the VAX  architecture share  the same core in-

struction  set.  However,  certain  extensions,  like  instructions  to

process  strings, packed decimal numbers or certain floating point for-

mats, are not available in hardware.

aa) CP-3F, LP8000, M380

The  chipset's processor element was sold  by AEG/Olympia, GI, and SGS-

Ates  under the respective  names. There are  no differences in the in-

struction set and address spaces.

ab) 4004 -> 4040

In  comparison to its predecessor, the  4040 features about a dozen ad-

ditional machine instructions.

ac) 8008 -> 8008NEW

Intel  redefined the 8008's  mnemonics around 1975,  the second variant

reflects  this  new  instruction  set.  A  simultaneous support of both

sets was not possible due to mnemonic conflicts.

ad) 8021, 8022,

8401, 8411, 8421, 8461,

8039, (MSM)80C39, 8048, (MSM)80C48, 8041, 8042, 80C382

For  the ROM-less versions  8039 and 80C39,  the commands which are us-

ing  the BUS (port 0) are forbidden. The 8021 and 8022 are special ver-

sions  with a  strongly shrinked  instruction set,  for which  the 8022

has  two A/D-  converters and  the necessary  control-commands. The in-

struction  set of  the MAB8401  to 8461  (designed by Philips) is some-

where  in between  the 8021/8022  and a  ''complete'' MC-48 instruction

set.  On the other hand,  they provide serial ports  and up to 8 KBytes

of program memory.

It  is possible to  transfer the CMOS-versions  with the IDL resp. HALT

command  into a stop mode with  lower current consumption. The 8041 and

8042  have some additional instructions  for controlling the bus inter-

face,  but in turn a few other  commands were omitted. The code address

space  of 8041,  8042, 84x1,  8021, and  8022 is not externally extend-

able,  and so  AS limits  the code  segment of  these processors to the

size  of the internal ROM. The  (SAB)80C382 is a variant especially de-

signed  by Siemens for  usage in telephones.  It also knows  a HALT in-

struction,  plus ist supports indirect addressing  for DJNZ and DEC. In

turn,  several instructions  of the  'generic' 8048  were left out. The

OKI  variants (MSM...)  also feature  indirect addressing  for DJNZ and

DEC,  plus enhanced  control of  power-down modes,  plus the full basic

MCS-48 instrucion set.

ae) 87C750 -> 8051, 8052, 80C320, 80C501, 80C502,

80C504, 80515, and 80517

-> 80C390

-> 80C251

The  87C750 can only access a maximum  of 2 Kbytes program memory which

is  why it lacks the LCALL and  LJMP instructions. AS does not make any

distinction  among  the  processors  in  the  middle,  instead  it only

stores  the different names  in the MOMCPU  variable (see below), which

allows  to query the setting with  IF instructions. An exception is the

80C504  that has a mask  flaw in its current  versions. This flaw shows

up  when an  AJMP or  ACALL instruction  starts at  the second last ad-

dress  of a 2K page. AS will automatically use long instructions or is-

sues  an error message in such  situations. The 80C251 in contrast rep-

resents  a drastic progress in the the direction 16/32 bits, larger ad-

dress  spaces, and  a more  orthogonal instruction  set. One might call

the  80C390  the  'small  solution':  Dallas Semiconductor modified in-

struction  set and  architecture only  as far  as it  was necessary for

the 16 Mbytes large address spaces.

af) 8096 -> 80196 -> 80196N -> 80296

Apart  from a different  set of SFRs  (which however strongly vary from

version  to version), the 80196 knows several new instructions and sup-

ports  a 'windowing' mechanism  to access the  larger internal RAM. The

80196N  family extends the address space  to 16 Mbytes and introduces a

set  of instructions to access addresses beyond 64Kbytes. The 80296 ex-

tends  the CPU core by instructions  for signal processing and a second

windowing  register, however removes  the Peripheral Transaction Server

(PTS) and therefore looses again two machine instructions.

ag) 8080 -> V30EMU -> 8085 -> 8085UNDOC

The  8085 knows  the additional  commands RIM  and SIM  for controlling

the  interrupt mask  and the  two I/O-pins.  The type 8085UNDOC enables

additional  instructions that  are not  documented by  Intel. These in-

structions are documented in section 4.23.

V30EMU  as target behaves  like an 8080,  with the addition  of the in-

structions  RETEM and CALLN.  These allow to  end or interrupt the 8080

emulation on a V20/V30/V40/V50.

ah) 8088,8086

-> 80188,80186

-> V20,V30,V40,V50

-> V33,V53

-> V25,V35

-> V55

-> V55SC

-> V55PI

Processors  listed in the  same line feature  an identical CPU core and

therefore  identical  instruction  set.  Going  down the lines, new in-

structions  are added,  with the  NEC CPUs  going different 'branches',

coming from the 'V20 basic instruction set'.

ai) 80960

aj) 8X300 -> 8X305

The  8X305 features a  couple of additional  registers that miss on the

8X300.  Additionally,  it  can  do  new operations with these registers

(like direct writing of 8 bit values to peripheral addresses).

ak) XAG1, XAG2, XAG3

These  processors only differ  in the size  of their internal ROM which

is defined in STDDEFXA.INC.

al) AT90S1200, AT90S2313, AT90S2323, AT90S233,

AT90S2343, AT90S4414, AT90S4433, AT90S4434,

AT90S8515, AT90C8534, AT90S8535,

ATTINY4, ATTINY5, ATTINY9,

ATTINY10, ATTINY11, ATTINY12, ATTINY13, ATTINY13A,

ATTINY15, ATTINY20, ATTINY24(A), ATTINY25,

ATTINY26, ATTINY28, ATTINY40, ATTINY44(A),

ATTINY45, ATTINY48, ATTINY84(A), ATTINY85,

ATTINY87, ATTINY88, ATTINY102, ATTINY104,

ATTINY167, ATTINY261, ATTINY261A, ATTINY43U,

ATTINY441, ATTINY461, ATTINY461A, ATTINY828,

ATTINY841, ATTINY861, ATTINY861A, ATTINY1634,

ATTINY2313, ATTINY2313A, ATTINY4313, ATMEGA48,

ATMEGA8, ATMEGA8515, ATMEGA8535, ATMEGA88,

ATMEGA8U2, ATMEGA16U2, ATMEGA32U2,

ATMEGA16U4, ATMEGA32U4, ATMEGA32U6, AT90USB646,

AT90USB647, AT90USB1286, AT90USB1287, AT43USB355,

ATMEGA16, ATMEGA161, ATMEGA162, ATMEGA163,

ATMEGA164, ATMEGA165, ATMEGA168, ATMEGA169,

ATMEGA32, ATMEGA323, ATMEGA324, ATMEGA325,

ATMEGA3250, ATMEGA328, ATMEGA329, ATMEGA3290,

ATMEGA406, ATMEGA64, ATMEGA640, ATMEGA644,

ATMEGA644RFR2, ATMEGA645, ATMEGA6450,

ATMEGA649, ATMEGA6490, ATMEGA103, ATMEGA128,

ATMEGA1280, ATMEGA1281, ATMEGA1284,

ATMEGA1284RFR2, ATMEGA2560, ATMEGA2561

The  various AVR chip  variants mainly differ  in the amount of on-chip

memory  (flash,  SRAM,  EEPROM)  an  the  set  of  built-in peripherals

(GPIO,  timers, UART, A/D converter...). Compared to the AT90... prede-

cessors,  the ATmega  chip also  provide additional instructions, while

the ATtinys do not support the multiplication instructions.

am) AM29245 -> AM29243 -> AM29240 -> AM29000

The  further one  moves to  the right  in this  list, the fewer the in-

structions  become that  have to  be emulated  in software.  While e.g.

the  29245 not  even owns  a hardware  multiplier, the two representors

in  the middle  only lack  the floating  point instructions.  The 29000

serves  as a 'generic' type that  understands all instructions in hard-

ware.

an) 80C166 -> 80C167,80C165,80C163

80C167  and 80C165/163  have an  address space  of 16 Mbytes instead of

256  Kbytes,  and  furthermore  they  know some additional instructions

for  extended addressing  modes and  atomic instruction sequences. They

are  'second generation' processors and differ  from each other only in

the amount of on-chip peripherals.

ao) LR35902/GBZ80 -> Z80 -> Z80UNDOC

-> Z180

-> eZ80190, eZ80L92, eZ80F91,

eZ80F92, eZ80F93,

-> Z380

While  there are only  a few additional  instructions for the Z180, the

Z380  owns 32-bit registers, a linear address space of 4 Gbytes, a cou-

ple  of instruction  set extensions  that make  the overall instruction

set  considerably more orthogonal, and  new addressing modes (referring

to  index register halves, stack  relative). These extensions partially

already  exist on the Z80 as undocumented extensions and may be switch-

ed  on via  the Z80UNDOC  variant. A  list with the additional instruc-

tions can be found in the chapter with processor specific hints.

The  processor built  into the  Gameboy (official  designation LR35092,

commonly  referred to as ''Gameboy  Z80'') is a mixture  of an 8080 and

Z80.  It lacks the  IX/IY registers, the  I/O address space, the second

register bank and a couple of 16 bit instructions.

The  Zilog eZ80  variants extend  the Z80  architecture with a 16 MByte

address  space, 24 bit registers and moderate additions to the instruc-

tion  set. Some  variants feature  an I  register that  is only  8 bits

wide,  and the  eZ80190 additionally  lacks a  few string  I/O instruc-

tios.  Otherwise, they only differ in  the amonut of on-chip memory and

peripherals.

ap) Z8601, Z8603, z86C03, z86E03, Z86C06, Z86E06,

Z86C08, Z86C21, Z86E21, Z86C30, Z86C31, Z86C32 Z86C40

-> Z88C00, Z88C01

-> eZ8, Z8F0113, Z8F011A, Z8F0123, Z8F012A,

Z8F0130, Z8F0131, Z8F0213, Z8F021A, Z8F0223, Z8F022A,

Z8F0230, Z8F0231, Z8F0411, Z8F0412, Z8F0413, Z8F041A,

Z8F0421, Z8F0422, Z8F0423, Z8F042A, Z8F0430, Z8F0431,

Z8F0811, Z8F0812, Z8F0813, Z8F081A, Z8F0821, Z8F0822,

Z8F0823, Z8F082A, Z8F0830, Z8F0831, Z8F0880, Z8F1232,

Z8F1233, Z8F1621, Z8F1622, Z8F1680, Z8F1681, Z8F1682,

Z8F2421, Z8F2422, Z8F2480, Z8F3221, Z8F3222, Z8F3281,

Z8F3282, Z8F4821, Z8F4822, Z8F4823, Z8F6081, Z8F6082,

Z8F6421, Z8F6422, Z8F6423, Z8F6481, Z8F6482

The  variants with Z8 core only differ  in internal memory size and on-

chip  peripherals, i.e. the choice does not  have an effect on the sup-

ported  instruction set.  Super8 and  eZ8 are  substantially different,

each  with an instruction set that  was vastly extended (into different

directions),  and they are  not fully upward-compatible  on source code

level as well.

aq) Z8001, Z8002, Z8003, Z8004

The  operation mode (segmented  for Z8001 and  Z8003, non-segmented for

Z8002  and Z8004) is selected via the processor type. There is current-

ly no further differentiation between Z8001/8002 and Z8003/8004.

ar) KCPSM, KCPSM3

Both  processor cores are not  available as standalone components, they

are  provided as  logic cores  for gate  arrays made  by Xilinx  The -3

variant  offers  a  larger  address  space and some additional instruc-

tions. Note that it is not binary upward-compatible!

as) MICO8_05, MICO8_V3, MICO8_V31

Lattice  unfortunately  changed  the  machine  instructions  more  than

once,  so different targets became  necessary to provide continued sup-

port  for older  projects. The  first variant  is the  one described in

the 2005 manual, the two other ones represent versions 3.0 resp. 3.1.

at) 96C141, 93C141

These  two  processors  represent  the  two variations of the processor

family:  TLCS-900 and  TLCS-900L. The  differences of  these two varia-

tions will be discussed in detail in section 4.34.

au) 90C141

av) 87C00, 87C20, 87C40, 87C70

The  processors of the TLCS-870 series  have an identical CPU core, but

different  peripherals depending  on the  type. In  part registers with

the  same  name  are  located  at  different  addresses.  The file STD-

DEF87.INC  uses, similar  to the  MCS-51-family, the distinction possi-

ble  by different types to provide  the correct symbol set automatical-

ly.

aw) TLCS-870/C

Currently,  only the processor core of  the TLCS-870/C family is imple-

mented.

ax) 47C00 -> 470C00 -> 470AC00

These  three variations of the TLCS-47-family  have on-chip RAM and ROM

of  different size, which leads  to several bank switching instructions

being added or suppressed.

ay) 4240P, 4250N, 4260P, 4270N

42C00Y, 42C40P, 42C50N, 42C60P, 42C70N

The  CMOS variants support two  additional instructions. Otherwise, the

variants  differ in  the number  of I/O  ports and  the size of program

memory.

az) 97C241

ba) TC9331

bb) 16C54 -> 16C55 -> 16C56 -> 16C57

These  processors differ  by the  available code  area, i.e. by the ad-

dress limit after which AS reports overruns.

bc) 16C84, 16C64

Analog  to the MCS-51 family, no distinction is made in the code gener-

ator,  the different numbers only serve  to include the correct SFRs in

STDDEF18.INC.

bd) 17C42

be) SX20, SX28

The SX20 uses a smaller housing and lacks port C.

bf) ST6200, ST6201, ST6203, ST6208, ST6209,

ST6210, ST6215, ST6218, ST6220, ST6225,

ST6228, ST6230, ST6232, ST6235, ST6240,

ST6242, ST6245, ST6246, ST6252, ST6253,

ST6255, ST6260, ST6262, ST6263, ST6265,

ST6280, ST6285

The  various ST6 derivates differ in  the amount of on-chip peripherals

and built-in memory.

bg) ST7

ST72251G1, ST72251G2, ST72311J2, ST72311J4,

ST72321BR6, ST72321BR7, ST72321BR9, ST72325S4,

ST72325S6, ST72325J7, ST72325R9, ST72324J6,

ST72324K6, ST72324J4, ST72324K4, ST72324J2,

ST72324JK21, ST72325S4, ST72325J7, ST72325R9,

ST72521BR6, ST72521BM9, ST7232AK1, ST7232AK2,

ST7232AJ1, ST7232AJ2, ST72361AR4, ST72361AR6,

ST72361AR7, ST72361AR9, ST7FOXK1, ST7FOXK2,

ST7LITES2Y0, ST7LITES5Y0, ST7LITE02Y0,

ST7LITE05Y0, ST7LITE09Y0

ST7LITE10F1, ST7LITE15F1, ST7LITE19F1,

ST7LITE10F0, ST7LITE15F0, ST7LITE15F1,

ST7LITE19F0, ST7LITE19F1,

ST7LITE20F2, ST7LITE25F2, ST7LITE29F2,

ST7LITE30F2, ST7LITE35F2, ST7LITE39F2,

ST7LITE49K2,

ST7MC1K2, ST7MC1K4, ST7MC2N6, ST7MC2S4,

ST7MC2S6, ST7MC2S7, ST7MC2S9, ST7MC2R6,

ST7MC2R7, ST7MC2R9, ST7MC2M9,

STM8

STM8S001J3, STM8S003F3, STM8S003K3, STM8S005C6,

STM8S005K6, STM8S007C8, STM8S103F2, STM8S103F3,

STM8S103K3, STM8S105C4, STM8S105C6, STM8S105K4,

STM8S105K6, STM8S105S4, STM8S105S6, STM8S207MB,

STM8S207M8, STM8S207RB, STM8S207R8, STM8S207R6,

STM8S207CB, STM8S207C8, STM8S207C6, STM8S207SB,

STM8S207S8, STM8S207S6, STM8S207K8, STM8S207K6,

STM8S208MB, STM8S208RB, STM8S208R8, STM8S208R6,

STM8S208CB, STM8S208C8, STM8S208C6, STM8S208SB,

STM8S208S8, STM8S208S6, STM8S903K3, STM8S903F3,

STM8L050J3, STM8L051F3, STM8L052C6, STM8L052R8,

STM8L001J3, STM8L101F1, STM8L101F2, STM8L101G2,

STM8L101F3, STM8L101G3, STM8L101K3, STM8L151C2,

STM8L151K2, STM8L151G2, STM8L151F2, STM8L151C3,

STM8L151K3, STM8L151G3, STM8L151F3, STM8L151C4,

STM8L151C6, STM8L151K4, STM8L151K6, STM8L151G4,

STM8L151G6, STM8L152C4, STM8L152C6, STM8L152K4,

STM8L152K6, STM8L151R6, STM8L151C8, STM8L151M8,

STM8L151R8, STM8L152R6, STM8L152C8, STM8L152K8,

STM8L152M8, STM8L152R8, STM8L162M8, STM8L162R8,

STM8AF6366, STM8AF6388, STM8AF6213, STM8AF6223,

STM8AF6226, STM8AF6246, STM8AF6248, STM8AF6266,

STM8AF6268, STM8AF6269, STM8AF6286, STM8AF6288,

STM8AF6289, STM8AF628A, STM8AF62A6, STM8AF62A8,

STM8AF62A9, STM8AF62AA, STM8AF5268, STM8AF5269,

STM8AF5286, STM8AF5288, STM8AF5289, STM8AF528A,

STM8AF52A6, STM8AF52A8, STM8AF52A9, STM8AF52AA,

STM8AL3136, STM8AL3138, STM8AL3146, STM8AL3148,

STM8AL3166, STM8AL3168, STM8AL3L46, STM8AL3L48,

STM8AL3L66, STM8AL3L68, STM8AL3188, STM8AL3189,

STM8AL318A, STM8AL3L88, STM8AL3L89, STM8AL3L8A,

STM8TL52F4, STM8TL52G4, STM8TL53C4, STM8TL53F4,

STM8TL53G4

The  STM8 core extends the address space  to 16 Mbytes and introduces a

couple  of new instructions. Though many instructions have the same ma-

chine code as for ST7, it is not binary upward compatible.

bh) ST9020, ST9030, ST9040, ST9050

These  4 names represent  the four ''sub-families''  of the ST9 family,

which  only differ in their  on-chip peripherals. Their processor cores

are  identical, which  is why  this distinction  is again  only used in

the include file containing the peripheral addresses.

bi) 6804

bj) 32010->32015

The  TMS32010 owns just 144 bytes of internal RAM, and so AS limits ad-

dresses  in the data  segment just up  to this amount. This restriction

does not apply for the 32015, the full range from 0..255 can be used.

bk) 320C25 -> 320C26 -> 320C28

These  processors  only  differ  slightly  in their on-chip peripherals

and in their configuration instructions.

bl) 320C30, 320C31 -> 320C40, 320C44

The  320C31 is a reduced version  with the same instruction set, howev-

er  fewer peripherals.  The distinction  is exploited  in STDDEF3X.INC.

The  C4x variants are  sourcecode upward compatible,  the machine codes

of  some instructions are  however slightly different.  Once again, the

C44  is a stripped-down  version of the  C40, with less peripherals and

a smaller address space.

bm) 320C203 -> 320C50, 320C51, 320C53

The  first one  represents the  C20x family  of signal processors which

implement  a subset of  the C5x instruction  set. The distinction among

the C5x processors is currently not used by AS.

bn) 320C541

This one at the moment represents the TMS320C54x family...

bo) TI990/4, TI990/10, TI990/12

TMS9900, TMS9940, TMS9995, TMS99105, TMS99110

The  TMS99xx/99xxx  processors  are  basically  single chip implementa-

tions  of the TI990 minicomputers. Some  TI990 models are even based on

such  a processor instead of a discrete CPU. The individual models dif-

fer  in their  instruction set  (the TI990/12  has the largest one) and

the presence of a privileged mode.

bp) TMS70C00, TMS70C20, TMS70C40,

TMS70CT20, TMS70CT40,

TMS70C02, TMS70C42, TMS70C82,

TMS70C08, TMS70C48

All  members of this family share the  same CPU core, they therefore do

not  differ in their instruction set.  The differences manifest only in

the  file  REG7000.INC  where  address  ranges and peripheral addresses

are  defined. Types listed in the same  row have the same amount of in-

ternal  RAM and the  same on-chip peripherals,  they differ only in the

amount of integrated ROM.

bq) 370C010, 370C020, 370C030, 370C040 and 370C050

Similar  to the  MCS-51 family,  the different  types are  only used to

differentiate  the peripheral  equipment in  STDDEF37.INC; the instruc-

tion set is always the same.

br) MSP430 -> MSP430X

The  X  variant  of  the  CPU  core  extends  the address space from 64

KiBytes  to 1  MiByte and  augments the  instruction set,  e.g. by pre-

fixed to repeat instructions.

bs) TMS1000, TMS1100, TMS1200, TMS1300

TMS1000  and TMS1200  each provide  1 KByte  of ROM  and 64  nibbles of

RAM,  while TMS1100  and TMS1300  provide twice  the amount  of RAM and

ROM.  Furthermore, TI has defined a significantly different default in-

struction  set fot  TMS1100 and  TMS1300(AS only  knows the default in-

struction sets!)

bt) IMP-16C/200, IMP-16C/300, IMP-16P/200, IMP-16P/300, IMP-16L

The  IMP-16L  defines  a  few  additional  bits in its status register,

plus  more branch conditions. It  supports the extended instruction set

just like the /300 variants.

bu) IPC-16, INS8900

The  INS8900 is just a re-implementation of  PACE in a more modern NMOS

manufacturing process; there are no differences in instruction set.

bv) SC/MP

bw) 8070

This  processor  represents  the  whole  807x family (which consists at

least  of the  8070, 8072,  and 8073),  which however  shares identical

CPU cores.

bx) COP87L84

This  is the only  member of National  Semiconductor's COP8 family that

is  currently supported. I know that  the family is substantially larg-

er  and that there are  representors with differently large instruction

sets  which will be  added when a  need occurs. It  is a beginning, and

National's documentation is quite extensive...

by) COP410 -> COP420 -> COP440 -> COP444

The  COP42x derivates  offer some  additional instructions,  plus other

instructions have an extended operand range.

bz) SC14400, SC14401, SC14402, SC14404, SC14405,

SC14420, SC14421, SC14422, SC14424

This  series of  DECT controllers  differentiates itself  by the amount

of  instructions, since  each of  them supports  different B field for-

mats and their architecture has been optimized over time.

ca) NS16008, NS32008, NS08032, NS16032, NS32016, NS32032,

NS32332, NS32CG16, NS32532

National  renamed the first-generation CPUs  several times in the early

years,  NS16008/NS32008/NS08032  resp.  NS16032/NS32016  are  the  same

chips.  NS32332 and  NS32532 support  an address  space of 4 GBytes in-

stead  of 16 MBytes, and the NS32CG16 is an embedded variant with addi-

tional instructions for bit block transfers.

cb) ACE1101, ACE1202

cc) F3850, MK3850,

MK3870, MK3870/10, MK3870/12, "MK3870/20, MK3870/22,

MK3870/30, MK3870/32, MK3870/40, MK3870/42,

MK3872, MK3873, MK3873/10, MK3873/12, MK3873/20,

MK3873/22, MK3874, MK3875, MK3875/22, MK3875/42,

MK3876, MK38P70/02, MK38C70, MK38C70/10,

MK38C70/20, MK97400, MK97410, MK97500, MK97501,

MK97503

This  huge  amount  of  variants  partially  results from the fact that

Mostek  renamed some variants  in the early  80s. The new naming scheme

allows  to  deduce  the  amount  of  internal  ROM  (0  to 4 for 0 to 4

Kbytes)  and executable RAM  (0 or 2  for 0 or  64 bytes) from the suf-

fix.  3850 and  MK975xx support  a 64K  address space,  which is only 4

Kbytes  for all  other variants.  P variants  have an  EEPROM piggyback

socket  for prototyping, C  variants are fabricated  in CMOS technology

and  feature two new  machine instructions (HET  and HAL). The MK3873's

feature  is a  built-in serial  port, while  the MK3875 offers a second

supply voltage pin to buffer the internal memory in standby mode.

cd) 7800, 7801, 7802

78C05, 78C06

7807, 7808, 7809

7810->78C10, 78C11, 78C12, 78C14, 78C17, 78C18

�PD7800  to �PD7802  represent the  ''first generation''  of the uCOM87

family  from NEC. �PD78C05  and �PD78C06 are  reduced variants that im-

plement  only a subset  of the instruction  set. 7807 to 7809 represent

the  uCOM87  series,  whose  instruction  set  was vastly extended. All

�PD781x  variants belong to the uCOM87AD series, which adds an A/D con-

verter  - however, instructions for  bit processing were again removed.

NOTE:  The instruction set  is in general  only partially binary upward

compatible!  The NMOS version �PD7810  has no stop-mode; the respective

command  and the ZCM register are  omitted. CAUTION! NMOS and CMOS ver-

sion partially differ in the reset values of some registers!

ce) uPD550, uPD554, uPD652,

uPD547, uPD552, uPD651,

uPD546, uPD553, uPD556, uPD557, uPD650

These  three groups of controllers belong  to the �COM-45, �COM-44, amd

�COM-43  family. The first two families implement a subset of the �COM-

43  instruction set. Otherwise,  the chips differ  by the amount of on-

chip ROM and RAM.

cf) 7500 <-> 7508

There  are two different types of CPU  cores in the �PD75xx family: the

7566  represents the the  'instruction set B',  which provides less in-

structions,  less registers and smaller address spaces. The 7508 repre-

sents  the 'full'  instruction set  A. CAUTION!  These instruction sets

are not 100% binary compatible!

cg) 75402,

75004, 75006, 75008,

75268,

75304, 75306, 75308, 75312, 75316,

75328,

75104, 75106, 75108, 75112, 75116,

75206, 75208, 75212, 75216,

75512, 75516

This  'cornucopia' of  processors differs  only by  the RAM size in one

group;  the groups themselves  again differ by  their on-chip peripher-

als  on the one hand and by  their instruction set's power on the other

hand.

ch) 78070

This  is currently the only  member of NEC's 78K0  family I am familiar

with. Similar remarks like for the COP8 family apply!

ci) 78214

This is currently the representor of NEC's 78K2 family.

cj) 78310

This is currently the representor of NEC's 78K3 family.

ck) 784026

This is currently the representor of NEC's 78K4 family.

cl) 7720 -> 7725

The  �PD7725  offers  larger  address  spaces and som more instructions

compared  to his  predecessor. CAUTION!  The processors  are not binary

compatible to each other!

cm) 77230

cn) 70616

This is currently the representor of NEC's V60 family.

co) SYM53C810, SYM53C860, SYM53C815, SYM53C825,

SYM53C875, SYM53C895

The  simpler members  of this  family of  SCSI processors lack some in-

struction  variants, furthermore they are different in their set of in-

ternal registers.

cp) MB89190

This processor type represents Fujitsu's F^2MC8L series...

cq) MB9500

...just  like this one  does it currently  for the 16-bit variants from

Fujitsu!

cr) MSM5840, MSM5842, MSM58421, MSM58422, MSM5847

These  variants of the  OLMS-40 family differ  in their instruction set

and in the amount of internal program and data memory.

cs) MSM5054, MSM5055, MSM5056, MSM6051, MSM6052

The  as for the OLMS-40 family:  differences in instruction set and the

amount of internal program and data memory.

ct) MN1610[ALT] -> MN1613[ALT]

In  addition to its predecessor's features,  the MN1613 offers a larger

address  space, a floating point  unit and a couple  of new machine in-

structions.

cu) RXV1, RX110, RX111, RX113, RX130, RX210,

RX21A, RX220, RX610, RX621, RX62N, RX630,

RX631 -->

RXV2, RX140, RX230, RX231, RX64M,

RX651 -->

RXV3, RX660, RX671, RX72M, RX72N

Controllers  of the  RX series  can coarsely  be classified  into three

groups  or  generations.  From  generation  to  generation (RXv1, RXv2,

RXv3), new machine instructions were added.

cv) PMC150, PMS150, PFS154, PMC131, PMS130, PMS131

PMS132, PMS132B, PMS152, PMS154B, PMS154C, PFS173

PMS133, PMS134, DF69, MCS11, PMC232, PMC234, PMC251

PMC271,PMC884, PMS232, PMS234, PMS271

The  Padauk controllers  differ in  the size  of the internal (ROM/RAM)

memory,  the type of  internal ROM (erasable  or OTP), the built-in pe-

ripherals, and their instruction set (both extent and binary coding).

cw) 1802 -> 1804, 1805, 1806 -> 1804A, 1805A, 1806A

1804,  1805, and 1806  feature an instruction  set that is slightly en-

hanced,  compared to the 'original' 1802, plus on-chip RAM and an inte-

grated  timer.  The  A  variants  extend  the  instruction set by DSAV,

DBNZ, and instructions for addition and subtraction in BCD format.

cx) XS1

This type represents the XCore-"family".

cy) 1750

MIL  STD 1750  is a  standard, therefore  there is  only one (standard)

variant...

cz) KENBAK

Since there has never been a KENBAK-2, the target is simply KENBAK...

da) CP-1600

db) HPNANO

dc) 6100 -> 6120

The  IM6120 supports a larger address space (32K instead of 4K) and ad-

ditional machine instructions.

dd) SC61860

This  is the processor used in  the Sharp PC-12xx...PC-15xx pocket com-

puters.

de) SC62015

This is the processor used in the Sharp PC-E500.

NONE  is a special target  that has not been  mentioned so far. This is

the  default target if no  target has been defined  on the command line

via  -cpu, and if no CPU statement  has been encountered so far. target

independent  pseudo instructions are still  possible in this situation,

however  it is not possible to create any code, neither via machine in-

structions,  nor via placing  data in memory.  In principle, it is also

possible  to explicitly select this target via -cpu or CPU. The practi-

cal use of this is however limited.

The  CPU instruction  needs the  processor type  as a simple literal, a

calculation like:

CPU     68010+10

is not allowed. Valid calls are e.g.

CPU     8051

or

CPU     6800

Regardless  of the processor  type currently set,  the integer variable

MOMCPU  contains the current status as  a hexadecimal number. For exam-

ple,  MOMCPU=$68010 for  the 68010  or MOMCPU=80C48H  for the 80C48. As

one  cannot express  all letters  as hexadecimal  digits (only A..F are

possible),  all other letters must must be omitted in the hex notation;

for example, MOMCPU=80H for the Z80.

You  can take advantage of this  feature to generate different code de-

pending  on the processor type. For example,  the 68000 does not have a

machine  instruction  for  a  subroutine  return with stack correction.

With  the variable MOMCPU you can define  a macro that uses the machine

instruction or emulates it depending on the processor type:

myrtd   macro   disp

if      MOMCPU<$68010 ; emulate for 68008 & 68000

move.l (sp),disp(sp)

lea    disp(sp),sp

rts

elseif

rtd    #disp         ; direct use on >=68010

endif

endm

cpu     68010

myrtd   12            ; results in RTD #12

cpu     68000

myrtd   12            ; results in MOVE../LEA../RTS

As  not all processor names  are built only out  of numbers and letters

from  A..F, the  full name  is additionally  stored in the string vari-

able named MOMCPUNAME.

The  assembler implicitly switches back to  the CODE segment when a CPU

instruction  is executed.  This is  done because  CODE is the only seg-

ment all processors support.

Note  that 68008 is  no longer the  default target. If  no -cpu command

line  argument has been given, the target  is set to the reserved value

NONE  up to the first CPU statement. Target-independent pseudo instruc-

tions  are still allowed in this  situation, like defining constants or

macros.  It is  however not  possible to  generate any code, neither by

machine instructions nor by disposing data in memory.

Some  targets define  options or  variants that  are so fundamental for

operation,  that they  have to  be selected  with the  CPU instruction.

Such  options  are  appended  to  the  argument,  separated  by  double

colons:

CPU <CPU Name>:<var1>=<val1>:<var2>=<val2>:...

See  the  respective  section  with  processor-specific  hints to check

whether a certain target supports such options.

3.2.4. SUPMODE, FPU, PMMU, CUSTOM

- - - - - - - - - - - - - - - - -

- SUPMODE   valid  for:  680x0,   NS32xxx,  PDP-11,  i960,  TLCS-900,

SH7000, i960, 29K, XA, PowerPC, M*Core, V60, and TMS9900

- FPU valid for: 680x0, NS32xxx, 80x86

- PMMU valid for: 680x0, NS32xxx

- CUSTOM valid for: NS32xxx

These  three switches  allow to  define which  parts of the instruction

set  shall  be  disabled  because  the  necessary preconditions are not

valid  for the  following piece  of code.  The parameter  for these in-

structions  may be  either ON  or OFF,  the current  status can be read

out of a variable which is either TRUE or FALSE.

The commands have the following meanings in detail:

- SUPMODE:  allows  or  prohibits  commands,  for whose execution the

processor  has to be  within the supervisor  mode. The status vari-

able is called INSUPMODE.

- FPU:  allows or prohibits  the commands of  the numerical coproces-

sors  8087, NS32081/32381  resp. 68881  or 68882.  The status vari-

able  is called FPUAVAIL. For NS32xxx as target, specifying the ex-

plicit  FPU type  (NS32081, NS32181,  NS32381, or  NS32580) is also

possible,  to enable  or disable  the additional  registers and in-

structions.

- PMMU:  allows or  prohibits the  commands of  the memory management

unit  68851 resp.  of the  built-in MMU  of the 68030. CAUTION! The

68030-MMU  supports only a relatively small subset of the 68851 in-

structions.  This  is  controlled  via  the FULLPMMU statement. The

status  variable is called PMMUAVAIL. For NS32xxx as target, speci-

fying  the  explicit  MMU  type  as  target  (NS32082,  NS32381, or

NS32352)  is  also  possible,  to  enable  access  to the MMU-type-

specific register set.

- CUSTOM:  allows  or  prohibits  the  commands  reserved  for custom

slave processors.

The  usage of of  instructions prohibited in  this manner will generate

a warning at SUPMODE, at PMMU and FPU a real error message.

3.2.5.  ACCMODE

- - - - - - - -

- valid for: VAX

The  VAX not only  knows a user  and supervisor mode,  it supports four

privilege  levels of this  type. Going from  more to less rights, these

are  named  Kernel,  Executive,  Supervisor,  and User. The ACCMODE in-

struction  informs the  assembler about  the access  mode the following

code  is executed within.  Not all machine  instructions are allowed in

all  modes. Valid  arguments are  either the  mode names  mentioned be-

fore,  or a number between zero (kernel  mode) to three (user mode). As

default,  user mode  is assumed,  and the  current setting  (as numeric

value) may be read from a symbol of same name.

3.2.6. CIS, EIS, FIS and FP11

- - - - - - - - - - - - - - -

- valid for: PDP-11

These  statements enable or disable  the availibility of certain PDP-11

instruction  set extensions. For  one of these  statements to be avail-

able,  the respective  instructions must  not be  part of the machine's

basic  instruction set, and  there must have  been an upgrade option to

add them. In detail:

-  CIS:  ,,Commercial Instruction  Set'', i.e.  instructions to oper-

ate  on  packed  and  non-packed  BCD numbers with variable length.

They were available as an option on the LSI-11 and the PDP-11/44.

-  EIS:  The  instructions  MUL,  DIV,  ASH  und ASHC, which were not

part  of the base  instruction set on  older or smaller PDP-11 sys-

tems.  They were  available as  an option  on the  LSI-11 resp. the

PDP-11/35 and PDP-11/40.

-  FIS:  Stack oriented instructions implementing the basic mathemat-

ic  operations on  floating point  numbers in  F format  (32 bits).

They  were available as  an option on  the LSI-11 resp. the PDP-11/

35 and PDP-11/40.

-  FP11:  Full floating point support  with separate FPU registers in

F and D format (32/64 bits).

3.2.7. FULLPMMU

- - - - - - - -

valid for: 680x0

Motorola  integrated  the  MMU  into  the  processor  starting with the

68030,  but the built-in  FPU is equipped  only with a relatively small

subset  of the 68851 instruction set. AS will therefore disable all ex-

tended  MMU instructions when the target  processor is 68030 or higher.

It  is however possible  that the internal  MMU has been  disabled in a

68030-based  system and the processor  operates with an external 68851.

One  can the  use a  FULLPMMU ON  to tell  AS that the complete MMU in-

struction  set is allowed.  Vice versa, one  may use a  FULLPMMU OFF to

disable  all additional  instruction in  spite of  a 68020 target plat-

form  to assure that portable code  is written. The switch between full

and  reduced instruction set  may be done  as often as  needed, and the

current  setting may  be read  from a  symbol with  the same name. CAU-

TION!  The CPU instruction  implicitly sets or  resets this switch when

its  argument is a 68xxx processor!  FULLPMMU therefore has to be writ-

ten after the CPU instruction!

3.2.8.  PADDING

- - - - - - - -

valid for: 680x0, 68xx, M*Core, XA, H8, SH7000, MSP430(X), TMS9900,

ST7/STM8, AVR (only if code segment granularity is 8 bits)

Various  processor  families  have  a  requirement that objects of more

than  one byte length must  be located on a  n even address. Aside from

data  objects, this may  also include instruction  words. For instance,

word  accesses to  an odd  address result  in an  exception on a 68000,

while  other processors like the H8 force the lowest address bit to ze-

ro.

The  PADDING instruction allows  to activate a  mechanism that tries to

avoid  such misalignments. If the  situation arises that an instruction

word,  or a data object of 16 bits  or more (created e.g. via DC) would

be  stored on an odd address,  a padding byte is automatically inserted

before.  Such a padding byte is displayed  in the listing in a separate

line that contains the remark

<padding>

If  the source line also  contained a label, the  label still points to

the  address of  the code  or data  object, i.e.  right behind  the pad

byte.  The same is  true for a  label in a  source line immediately be-

fore,  as long as this line only  holds the label and no other instruc-

tion. So, in the follwing example:

padding  on

org      $1000

dc.b     1

adr1:  nop

dc.b     1

adr2:

nop

dc.b     1

adr3:  equ      *

nop

the  labels adr1 and adr2 hold the  addresses of the respective NOP in-

structions,  which were made even by inserting a pad byte. adr3 in con-

trast holds the address of the pad byte preceding the third NOP.

Similar  to the previous  instructions, the argument  to PADDING may be

either  ON or OFF,  and the current  setting may be  read from a symbol

with  the same name. PADDING  is by default only  enabled for the 680x0

family, it has to be turned on explicitly for all other families.

3.2.9.  PACKING

- - - - - - - -

valid  for: 56000, AVR, TMS3203x/4x, TMS3206x, MN1610, CP1600, �PD7720/

7725, �PD77230

In  some way, PACKING is similar to PADDING, it just has a somewhat op-

posite  effect: While  PADDING extends  the disposed  data to  get full

words  and keep a  possible alignment, PACKING  squeezes several values

into  a single word. This makes sense  for the AVR's code segment since

the  CPU has a special instruction (  LPM) to access single bytes with-

in  a 16-bit word. In case this  option is turned on (argument ON), two

byte  values are packed into a single word by DATA, similar to the sin-

gle  characters of string arguments. The  value range of course reduces

to  -128...+255. If this option is  turned off (argument OFF), each in-

teger  argument  obtains  its  own  word  and  may  take  values from -

32768...+65535.

This  distinction is only  made for integer  arguments of DATA, strings

will  always be  packed.. Keep  further in  mind that packing of values

only  works within the arguments of a DATA statement; if one has subse-

quent  DATA statements, there will still  be half-filled words when the

argument count is odd!

3.2.10.  WARNRELATIVE

- - - - - - - - - - -

valid for: Zx80

This  switch instructs the  assembler whether to  issue warnings when a

relative  jump instead  of an  absolute one  would be possible. The de-

fault  is OFF respectively what was  defined via the command line argu-

ments -wrelative and -wno-relative.

The current setting may be read from a symbol of same name.

3.2.11. MAXMODE

- - - - - - - -

valid for: TLCS-900, H8

The  processors of  the TLCS-900-family  are able  to work  in 2 modes,

the  minimum and maximum mode. Depending on the actual mode, the execu-

tion  environment and the  assembler are a  little bit different. Along

with  this  instruction  and  the  parameter  ON or OFF, AS is informed

that  the following  code will  run in  maximum resp. minimum mode. The

actual  setting can be read from  the variable INMAXMODE. Presetting is

OFF, i.e. minimum mode.

Similarly,  one uses  this instruction  to tell  AS in  H8 mode whether

the  address space is 64K or 16  Mbytes. This setting is always OFF for

the 'small' 300 version and cannot be changed.

3.2.12. EXTMODE and LWORDMODE

- - - - - - - - - - - - - - -

valid for: Z380

The  Z380 may operate  in altogether 4  modes, which are  the result of

setting  two flags: The XM flag rules whether the processor shall oper-

ate  wit an address space of  64 Kbytes or 4 Gbytes  and it may only be

set  to 1  (after a  reset, it  is set  to 0 for compatibility with the

Z80).  The LW  flag in  turn rules  whether word  operations shall work

with  a word size of 16 or 32  bits. The setting of these two flags in-

fluences  range checks  of constants  and addresses,  which is  why one

has  to tell AS the setting of  these two flags via these instructions.

The  default assumption is  that both flags  are 0, the current setting

(ON  or OFF)  may be  read from  the predefined symbols INEXTMODE resp.

INLWORDMODE.

3.2.13. SRCMODE

- - - - - - - -

valid for: MCS-251

Intel   substantially  extended  the  8051  instruction  set  with  the

80C251,  but unfortunately there was only  a single free opcode for all

these  new instructions.  To avoid  a processor  that will be eternally

crippled  by a prefix,  Intel provided two  operating modes: the binary

and  the source mode.  The new processor  is fully binary compatible to

the  8051 in binary mode, all  new instructions require the free opcode

as  prefix. In source mode, the  new instructions exchange their places

in  the code tables with the  corresponding 8051 instructions, which in

turn  then need a  prefix. One has  to inform AS  whether the processor

operates  in source mode (ON) or binary  mode (OFF) to enable AS to add

prefixes  when required. The current setting may be read from the vari-

able INSRCMODE. The default is OFF.

3.2.14. PLAINBASE

- - - - - - - - -

valid for: 6809

Historically,  AS allows to omit an empty first argument on indexed ad-

dress expressions. An

lda  x

for instance was equivalent to

lda  ,x

Though  meant as a feature, this wa  occasionally rather seen as a bug.

Current  versios therefore no longer allow to omit an empty index argu-

ment  and will  instead emit  a wrong  argument count error message. If

this  feature is still desired  or needed for existing  code, it may be

enabled via a

plainbase on

statement.  The  current  setting  may  be  read  from a symbol of same

name.

3.2.15. BIGENDIAN

- - - - - - - - -

valid for: MCS-51/251, PowerPC, SC/MP, 2650, NS32000

Intel  broke  with  its  own  principles  when  the 8051 series was de-

signed:  in contrast to  all traditions, the  processor uses big-endian

ordering  for all multi-byte values! While this  was not a big deal for

MCS-51  processors  (the  processor  could  access memory only in 8-bit

portions,  so everyone  was free  to use  whichever endianess one want-

ed),  it may  be a  problem for  the 251  as it  can fetch whole (long-

)words  from memory  and expects  the MSB  to be  first. As this is not

the  way of constant disposal earlier versions  of AS used, one can use

this  instruction to toggle between big  and little endian mode for the

instructions  DB, DW, DD,  DQ, DT, and  DO. BIGENDIAN OFF (the default)

puts  the LSB first  into memory as  it used to  be on earlier versions

of  AS, BIGENDIAN ON engages the big-endian mode compatible to the MCS-

251.  One may of course change this  setting as often as one wants; the

current setting can be read from the symbol with the same name.

The  Renesas RX  as target  also supports  a selectable  endianess. For

compatibility  to the original assembler,  the statement is named ENDI-

AN and accepts LITTLE or BIG as argument.

3.2.16.  WRAPMODE

- - - - - - - - -

valid for: Atmel AVR

After  this switch has been set to  ON, AS will assume that the proces-

sor's  program counter does not  have the full length  of 16 bits given

by  the architecture, but  instead a length  that is exactly sufficient

to  address the  internal ROM.  For example,  in case of the AT90S8515,

this  means 12 bits,  corresponding to 4  Kwords or 8  Kbytes. This as-

sumption  allows relative branches from the  ROM's beginning to the end

and  vice versa which  would result in  an out-of-branch error when us-

ing  strict arithmetics. Here, they work because the carry bits result-

ing  from  the  target  address  computation are discarded. Assure that

the  target processor  you are  using works  in the outlined way before

you  enable this option! In case  of the abovementioned AT90S8515, this

option  is even necessary because  it is the only  way to perform a di-

rect jump through the complete address space...

This  switch is set to  OFF by default, and  its current setting may be

read from a symbol with same name.

3.2.17. PANEL

- - - - - - -

valid for: IM61x0

This  switch  is  used  to  inform  the assembler whether the following

code  is executet with a set or cleared Control Panel Flip-Flop. A cou-

ple  of IOT instructions are  only allowed if the  flip-flop has a cer-

tain  state. Usage of these instructions in the other state will be re-

ported as an error by the assembler.

The current setting may be read from the symbol INPANEL.

3.2.18. SEGMENT

- - - - - - - -

valid for: all processors

Some  microcontrollers  and  signal  processors  know  various  address

ranges,  which do not overlap with  each other and require also differ-

ent  instructions  and  addressing  modes  for  access. To manage these

ones  also, the  assembler provides  various program  counters, you can

switch  among them to and  from by the use  of the SEGMENT instruction.

For  subroutines included with INCLUDE, this  e.g. allows to define da-

ta  used by the main program or  subroutines near to the place they are

used.  In detail, the  following segments with  the following names are

supported:

- CODE: program code;

- DATA: directly addressable data (including SFRs);

- XDATA:  data in externally  connected RAM or  X-addressing space of

the DSP56xxx or ROM data for the �PD772x;

- YDATA: Y-addressing space of the DSP56xxx;

- IDATA: indirectly addressable (internal) data;

- BITDATA:  the part  of the  8051-internal RAM  that is  bitwise ad-

dressable;

- IO: I/O-address range;

- REG: register bank of the ST9;

- ROMDATA: constant ROM of the NEC signal processors;

- EEDATA: built-in EEPROM.

See  also section  3.2.1 (ORG)  for detailed  information about address

ranges  and initial values of the  segments. Depending on the processor

family, not all segment types will be permitted.

The  bit segment is managed as if it  would be a byte segment, i.e. the

addresses will be incremented by 1 per bit.

Labels  get the same type  as attribute as the  segment that was active

when  the label was defined. So the  assembler has a limited ability to

check  whether you access  symbols of a  certain segment with wrong in-

structions. In such cases the assembler issues a warning.

Example:

CPU     8051    ; MCS-51-code

segment code    ; test code

setb    flag    ; no warning

setb    var     ; warning : wrong segment

segment data

var     db      ?

segment bitdata

flag    db      ?

3.2.19. PHASE and DEPHASE

- - - - - - - - - - - - -

valid for: all processors

For  some applications  (especially on  Z80 systems),  the code must be

moved  to  another  address  range  before  execution. If the assembler

didn't  know about this, it would align  all labels to the load address

(not  the start address). The programmer  is then forced to write jumps

within  this area either independent of location or has to add the off-

set  at each symbol  manually. The first  one is not  possible for some

processors,  the last one  is extremely error-prone.  With the commands

PHASE  and DEPHASE, it is possible to inform the assembler at which ad-

dress the code will really be executed on the target system:

phase   <address>

informs  the assembler  that the  following code  shall be  executed at

the  specified address. The assembler  calculates thereupon the differ-

ence  to the real program counter and adds this difference for the fol-

lowing operations:

- address values in the listing

- filing of label values

- program  counter references  in relative  jumps and address expres-

sions

- readout of the program counter via the symbols * or $

By using the instruction

DEPHASE

,  this ''shifting'' is reverted to the  value previous to the most re-

cent  PHASE instruction. PHASE und DEPHASE may be used in a nested man-

ner.

The  assembler keeps  phase values  for all  defined segments, although

this instruction pair only makes real sense in the code segment.

3.2.20.  SAVE and RESTORE

- - - - - - - - - - - - -

valid for: all processors

The  command SAVE forces the assembler  to push the contents of follow-

ing variables onto an internal stack:

- currently selected processor type (set by CPU);

- currently active memory area (set by SEGMENT);

- the flag whether listing is switched on or off (set by LISTING);

- the  flags  that  define  which  part  of  expanded macros shall be

printed in the assembly listing (set by /MACEXP_DFT/MACEXP_OVR).

- currently active character translation table (set by CODEPAGE).

The  counterpart RESTORE  pops the  values saved  last from this stack.

These  two  commands  were  primarily  designed  for  include files, to

change  the  above  mentioned  variables  in  any  way  inside of these

files,  without  loosing  their  original  content. This may be helpful

e.g.  in include files with own,  fully debugged subroutines, to switch

the listing generation off:

SAVE            ; save old status

LISTING OFF     ; save paper

.               ; the actual code

.

RESTORE         ; restore

In  opposite to  a simple  LISTING OFF  .. ON-pair,  the correct status

will  be restored, in case the  listing generation was switched off al-

ready before.

The  assembler checks if the number of SAVE-and RESTORE-commands corre-

sponds and issues error messages in the following cases:

- RESTORE, but the internal stack is empty;

- the stack not empty at the end of a pass.

In  case the currently used target  has machine instructions named SAVE

or  RESTORE, this  functionality may  be reached  via SAVEENV resp. RE-

STOREENV.  As an alternative,  it is always  possible to explicitly in-

voke  the  pseudo  instructions  by  prepending  a  period (.SAVE resp.

.RESTORE).

3.2.21.  ASSUME

- - - - - - - -

valid for: various

This  instruction allows to tell AS the current setting of certain reg-

isters  whose contents  cannot be  described with  a simple  ON or OFF.

These  are  typically  registers  that  influence  addressing modes and

whose  contents are important to know for  AS in order to generate cor-

rect  addressing. It is  important to note  that ASSUME only informs AS

about  these, no  machine code  is generated  that actually loads these

values into the appropriate registers!

A  value defined with ASSUME can  be queried or integrated into expres-

sions  via the built-in  function ASSUMEDVAL. This  is the case for all

architectures  listed  in  the  following  sub-sections  except for the

8086.

65CE02

......

The  65CE02 features  a a  register named  'B' that  is used to set the

'base  page'. In comparison to the  original 6502, this allows the pro-

grammer  to place  the memory  page addressable  with short (8 bit) ad-

dresses  anywhere in the 64K address space. This register is set to ze-

ro  after a reset, so  the 65CE02 behaves like  its predecessor. A base

page  at zero is also  the default assumption of  the assembler. It may

be  informed about its actual contents via a ASSUME B:xx statement. Ad-

dresses  located in this page will  then automatically be addressed via

short addressing modes.

6809

....

In  contrast to  its 'predecessors'  like the  6800 and 6502, the posi-

tion  of the direct page,  i.e. the page of  memory that can be reached

with  single-byte addresses,  can be  set freely.  This is done via the

'direct  page register' that sets the page  number. One has to assign a

corresponding  value to  this register  via ASSUME  is the contents are

different  from the  default of  0, otherwise  wrong addresses  will be

generated!

68HC11K4

........

Also  for the HC11,  the designers finally  weren't able to avoid bank-

ing,  to address more  than 64 Kbytes  with only 16  address lines. The

registers  MMSIZ, MMWBR, MM1CR,  and MM2CR control  whether and how the

additional  512K  address  ranges  are  mapped  into  the CPU's address

space.  AS initially assumes  the reset state  of these registers, i.e.

all are set to $00 and windowing is disabled.

Furthermore,  the  settings  of  the  registers CONFIG, INIT, and INIT2

may  be specified. This enables the  assembler to deduce the mapping of

I/O  registers, internal RAM  and EEPROM into  CPU address space, which

has priority over mapping of memory via windowing.

68HC12X

.......

Similar  to its cousin  without the appended  'X', the HC12X supports a

short  direct addressing mode. In this case  however, it can be used to

address  more than just the  first 256 bytes of  the address space. The

DIRECT  register specifices  which 256  byte page  of the address space

is  addressed by this  addressing mode. ASSUME  is used to  tell AS the

current  value of this register, so  it is able to automatically select

the  most efficient address ing mode  when absolute addresses are used.

The default is 0, which corresponds to the reset state.

68HC16

......

The  68HC16 employs  a set  of bank  registers to  address a space of 1

Mbyte  with its registers  that are only  16 bits wide. These registers

supply  the upper 4 bits. Of these,  the EK register is responsible for

absolute  data accesses (not  jumps!). AS checks  for each absolute ad-

dress  whether the upper 4  bits of the address  are equal to the value

of  EK specified via  ASSUME. AS issues  a warning if  they differ. The

default for EK is 0.

H8/500

......

In  maximum mode, the extended address space of these processors is ad-

dressed  via  a  couple  of  bank  registers.  They  carry the names DP

(registers  from 0..3, absolute addresses), EP  (register 4 and 5), and

TP  (stack). AS needs the current value  of DP to check if absolute ad-

dresses  are within the currently addressable  bank; the other two reg-

isters  are only used for indirect  addressing and can therefore not be

monitored;  it is  a question  of personal  taste whether one specifies

their  values or not. The BR  register is in contrast important because

it  rules which 256-byte page may  be accessed with short addresses. It

is  common for all registers that AS  does not assume any default value

for  them as they are  undefined after a CPU  reset. Everyone who wants

to  use absolute addresses must therefore  assign values to at least DR

and DP!

MELPS740

........

Microcontrollers  of  this  series  know  a ''special page'' addressing

mode  for the  JSR instruction  that allows  a shorter coding for jumps

into  the last  page of  on-chip ROM.  The size  of this ROM depends of

course  on the  exact processor  type, and  there are  more derivatives

than  it  would  be  meaningful  to  offer via the CPU instruction...we

therefore  have to rely on  ASSUME to define the  address of this page,

e.g.

ASSUME  SP:$1f

in case the internal ROM is 8K.

MELPS7700/65816

...............

These  processors contain a  lot of registers  whose contents AS has to

know  in order to  generate correct machine  code. These are the regis-

ters in question:

+--------+----------------------+-------------+---------+

| name   | function             | value range | default |

+--------+----------------------+-------------+---------+

+--------+----------------------+-------------+---------+

| DT/DBR | data bank            | 0-$ff       | 0       |

| PG/PBR | code Bank            | 0-$ff       | 0       |

| DPR    | directly addr. page  | 0-$ffff     | 0       |

| X      | index register width | 0 or 1      | 0       |

| M      | accumulator width    | 0 or 1      | 0       |

+--------+----------------------+-------------+---------+

To  avoid endless  repetitions, see  section 4.15  for instructions how

to  use  these  registers.  The  handling  is  otherwise similar to the

8086,  i.e.  multiple  values  may  be  set with one instruction and no

code  is generated  that actually  loads the  registers with  the given

values. This is again up to the programmer!

MCS-196/296

...........

Starting  with the  80196, all  processors of  the MCS-96 family have a

register  'WSR' that allows  to map memory  areas from the extended in-

ternal  RAM or the SFR range into  areas of the register file which may

then  be accessed  with short  addresses. If  one informs  AS about the

value  of the  WSR register,  it can  automatically find out whether an

absolute  address can be addressed with  a single-byte address via win-

dowing;  consequently, long  addresses will  be automatically generated

for  registers covered by  windowing. The 80296  contains an additional

register  WSR1 to allow  simultaneous mapping of  two memory areas into

the  register file.  In case  it is  possible to  address a memory cell

via both areas, AS will always choose the way via WSR!

For  indirect addressing, displacements may be  either short (8 bits, -

128  to +127) or  long (16 bits).  The assembler will automatically use

the  shortest possible encoding for a  given displacement. It is howev-

er  possible to enforce  a 16-bit coding  by prefixing the displacement

argument  with  a  bigger  sign  ((>). Similarly, absolute addresses in

the  area from 0ff80h to 0ffffh may be reached via a short offset rela-

tive to the "null register".

8086

....

The  8086 is  able to  address data  from all  segments in all instruc-

tions,  but it however needs  so-called ''segment prefixes'' if another

segment  register than  DS shall  be used.  In addition  it is possible

that  the DS register  is adjusted to  another segment, e.g. to address

data  in the code segment  for longer parts of  the program. As AS can-

not  analyze the code's  meaning, it has  to informed via this instruc-

tion  to  what  segments  the  segment  registers  point at the moment,

e.g.:

ASSUME  CS:CODE, DS:DATA    .

It  is possible to assign assumptions  to all four segment registers in

this  way. This  instruction produces  no code,  so the  program itself

has to do the actual load of the registers with the values.

The  usage of this instruction  has on the one  hand the result that AS

is  able to automatically put ahead  prefixes at sporadic accesses into

the  code segment, or on the other hand, one can inform AS that the DS-

register was modified and you can save explicit CS:-instructions.

Valid  arguments behind the colon are  CODE, DATA and NOTHING. The lat-

ter  value informs AS that a  segment register contains no usable value

(for AS). The following values are preinitialized:

CS:CODE, DS:DATA, ES:NOTHING, SS:NOTHING

Z180

....

The   Z180  contains  a  built-in  MMU   whicht  maps  the  CPU  core's

''logical''  address space of 64 KBytes  to a physical address space of

512  KBytes. The precise mappig is  defined via the the registers CBAR,

CBR  and BBR. Opposed  to the 68HC11K4,  the assembler will not perform

any  automatic translations of  physical to logical  addresses; only an

internal  mapping table is kept. It  is however possible to access this

table via the phys2cpu() function.

eZ80

....

The eZ80 mode can operate in two modes:

- In  Z80 mode, the BC, DE, HL, IX,  IY, and SP registers are 16 bits

wide,  and only the 64K  page defined by the  MBASE register is ad-

dressable.  For instructions with  an absolute address  or a non-8-

bit immediate value as argument, two bytes are fetched.

- In  ADL mode,  the mentioned  registers are  24 bits  wide, and the

whole  16 MByte  address space  can be  addressed. For instructions

with  an  absolute  address  or  a non-8-bit immediate value, three

bytes are fetched.

Since  instruction encoding and  range checking depends  on the operat-

ing  mode, the assembler needs to know which mode the CPU currently us-

es.  By using  ASSUME to  set ADL  either to  0 or  1, the assembler is

told  about the  default operating  mode, i.e.  the mode if no suffixes

are  given. Furthermore,  the assembler  can be  told about the current

value  of MBASE  (0 to  0ff hex)  as well.  The default  assumption for

both  values is zero, same as to the  values set in the CPU after a re-

set.

XA

..

The  XA family has a data address  space of 16 Mbytes, a process howev-

er  can always address within  a 64K segment only  that is given by the

DS  register. One has to inform AS about the current value of this reg-

ister in order to enable it to check accesses to absolute addresses.

29K

...

The  processors of  the 29K  family feature  a register RBP that allows

to  protect banks  of 16  registers against  access from user mode. The

corresponding  bit has to be set  to achieve the protection. ASSUME al-

lows  to tell AS which  value RBP currently contains.  AS can warn this

way  in case  a try  to access  protected registers  from user  mode is

made.

80C166/167

..........

Though  none  of  the  80C166/167's  registers  is  longer than sixteen

bits,  this processor  has 18/24  address lines  and can  therefore ad-

dress  up to 256Kbytes/16Mbytes. To resolve this contradiction, it nei-

ther  uses the well-known (and  ill-famed) Intel method of segmentation

nor  does it  have inflexible  bank registers...no,  it uses paging! To

accomplish  this, the logical address space  of 64 Kbytes is split into

4  pages  of  16  Kbytes,  and  for  each page there is a page register

(named  DPP0..DPP3)  that  rules  which  of  the 16/1024 physical pages

shall  be mapped to this  logical page. AS always  tries to present the

address  space with  a size  of 256Kbytes/16MBytes  in the sight of the

programmer,  i.e. the physical page is  taken for absolute accesses and

the  setting of  bits 14/15  of the  logical address  is deduced. If no

page  register fits,  a warning  is issued.  AS assumes by default that

the  four registers linearly map the first  64 Kbytes of memory, in the

following style:

ASSUME  DPP0:0,DPP1:1,DPP2:2,DPP3:3

The  80C167 knows  some additional  instructions that  can override the

page  registers'  function.  The  chapter with processor-specific hints

describes how these instructions influence the address generation.

Some  machine instructions  have a  shortened form  that can be used if

the argument is within a certain range:

- MOV Rn,#<0..15>

- ADD/ADDC/SUB/SUBC/CMP/XOR/AND/OR Rn, #<0..7>

- LOOP Rn,#<0..15>

The  assembler automatically  uses to  the shorter  coding if possible.

If  one wants to  enforce the longer  coding, one may  place a 'bigger'

character  right before the expression (behind the double cross charac-

ter!).  Vice versa,  a 'smaller'  character can  be used  to assure the

shorter  coding  is  used.  In  case  the  operand does not fulfill the

range  restrictions  for  the  shorter  coding,  an error is generated.

This  syntax may also be  used for branches and  calls which may either

have a short displacement or a long absolute argument.

TLCS-47

.......

The  direct data address space of these processors (it makes no differ-

ence  whether you address directly  or via the HL  register) has a size

of  only 256 nibbles. Because the  ''better'' family members have up to

1024  nibbles of RAM on  chip, Toshiba was forced  to introduce a bank-

ing  mechanism via the DMB  register. AS manages the  data segment as a

continuous  addressing  space  and  checks  at any direct addressing if

the  address is in the currently active bank. The bank AS currently ex-

pects can be set by means of

ASSUME  DMB:<0..3>

The default value is 0.

IPC-16/INS8900

..............

The  processor provides an input pin  named BPS to select which address

range  shall be  directly addressable:  either the  first 256 words, or

both  the lowest and  topmost 128 words.  The first variant  is the de-

fault, an ASSUME BPS:1 switches to the second variant.

ST6

...

The  microcontrollers of  the ST62  family are  able to  map a part (64

bytes)  of the  code area  into the  data area,  e.g. to load constants

from  the ROM. This means also that at  one moment only one part of the

ROM  can be addressed.  A special register  rules which part  it is. AS

cannot  check the contents of this register directly, but it can be in-

formed  by this instruction that  a new value has  been assigned to the

register.  AS then  can test  and warn  if necessary, in case addresses

of  the  code  segment  are  accessed,  which  are  not  located in the

''announced''  window. If, for example, the  variable VARI has the val-

ue 456h, so

ASSUME  ROMBASE:VARI>>6

sets  the AS-internal variable to 11h,  and an access to VARI generates

an access to address 56h in the data segment.

It  is possible to assign a simple  NOTHING instead of a value, e.g. if

the  bank register is used temporarily as  a memory cell. This value is

also the default.

The  program counter of these  controller only has a  width of 12 bits.

This  means that some sort of banking  scheme had to be introduced if a

device  includes  more  than  4  KBytes  of program memory. The banking

scheme  splits  both  proram  space  and  program  memory in pages of 2

KBytes.  Page one of the program space always accesses page one of pro-

gram  memory. The PRPR  register present on  such devices selects which

page  of program memory is accessed via  addresses 000h to 7ffh of pro-

gram  space. As an  initial approcimation, AS  regards program space to

be  linear and of  the size of  program memory. If  a jump or call from

page  one is made to code in one  of the other pages, it checks whether

the  assumed contents  of the  PRPR register  match the destination ad-

dress.  If a jump or call is done from one of the other pages to an ad-

dress  outside of page  one, it checks  whether the destination address

is  within the same page. IMPORTANT:  The program counter itself is on-

ly  12 bits wide.  It is therefore  not possible to  jump from one page

to  another one, without  an intermediate step  of jumping back to page

one.  Changing the  PRPR register  while operating  outside of page one

would result in ''pulling out'' the code from under one's feet.

ST9

...

The  ST9 family uses exactly the  same instructions to address code and

data  area. It depends  on the setting  of the flag  register's DP flag

which  address space is referenced. To enable  AS to check if one works

with  symbols  from  the  correct  address  space  (this of course only

works  with absolute  accesses!), one  has to  inform AS whether the DP

flag  is currently 0 (code) or 1  (data). The initial value of this as-

sumption is 0.

78K2

....

78K2  is an 8/16 bit  architecture, which has later  been extended to a

one-megabyte  addres space  via banking.  Banking is  realized with the

registers  PM6 (normal  case) resp.  P6 (alternate  case with & as pre-

fix)  that supply the missing upper four address bits. At least for ab-

solute  addresses, AS can check whether  the current, linear 20-bit ad-

dress is within the given 64K window.

78K3

....

Processors  witrh a  78K3 core  have register  banks that consist of 16

registers.  These registers may  be used via  their numbers (R0 to R15)

or  their  symbolic  names  (X=R0,  A=R1,  C=R2,  B=R3, VPL=R8, VPH=R9,

UPL=R10,  UPH=R11, E=R12, D=R13, L=R14,  H=R15). The processor core has

a  register select bit (RSS) to switch  the mapping of A/X and B/C from

R0..R3  to R4..R7. This  is mainly important  for instructions that im-

plicitly  use one of these registers  (i.e. instruction that do not en-

code  the register  number in  the machine  code). However,  it is also

possible to inform the assembler about the changed mapping via a

assume rss:1

The  assmebler will then insert the alternate register numbers into ma-

chine  instructions that  explicitly encode  the register numbers. Vice

versa, R5 will be treated like A instead of R1 in the source code.

78K4

....

78K4  was designed as  an 'upgrade path'  from 78K3, which  is why this

processor  core contains  the same  RSS bit  to control  the mapping of

registers AX and BC (though NEC discourages use of it in new code).

Aside  from many new instructins and addressing modes, the most signif-

icant  extension is the larger address space of 16 MBytes, of which on-

ly  the first MByte may be used  for program code. The CPU-internal RAM

and  all special  function registers  may be  positioned either  at the

top  of the first MByte  or the top of  the first 64 KByte page. Choice

is  made via the LOCATION machine instruction  that either takes a 0 or

15  as argument.  Together with  remapping RAM  and SFRs, the processor

also  switches the  address ranges  that may  be reached  with short (8

bit)  addresses. Parallel to using LOCATION,  one has to inform the as-

sembler  about  this  setting  via  a  ASSUME LOCATION:.. statement. It

will  then use  short addressing  for the  proper ranges. The assembler

will assume a default of 0 for LOCATION.

320C3x/C4x

..........

As  all instruction  words of  this processor  family are  only 32 bits

long  (of which  only 16  bits were  reserved for  absolute addresses),

the  missing upper 8/16 bits have to  be added from the DP register. It

is  however still possible to specify a full 24/32-bit address when ad-

dressing,  AS will  check then  whether the  upper 8  bits are equal to

the  DP register's assumed  values. ASSUME is  different to the LDP in-

struction  in the  sense that  one cannot  specify an arbitrary address

out  of the  bank in  question, one  has to  extract the  upper bits by

hand:

ldp     @addr

assume  dp:addr>>16

.

.

ldi     @addr,r2

uPD78(C)10

..........

These  processors have  a register  (V) that  allows to move the ''zero

page'',  i.e.  page  of  memory  that  is addressable by just one byte,

freely  in the  address space,  within page  limits. By reasons of com-

forts you don't want to work with expressions such as

inrw    Lo(counter)

so  AS takes over this  job, but only under  the premise that it is in-

formed  via the  ASSUME-command about  the contents  of the V register.

If  an instruction  with short  addressing is  used, it will be checked

if  the upper half of the address expression corresponds to the expect-

ed content. A warning will be issued if both do not match.

75K0

....

As  the whole address space  of 12 bits could  not be addressed even by

the  help  of  register  pairs  (8  bits), NEC had to introduce banking

(like  many others too...):  the upper 4  address bits are fetched from

the  MBS register (which can be assigned values from 0 to 15 by the AS-

SUME  instruction),  which  however  will  only  be regarded if the MBE

flag  has been set to  1. If it is  0 (default), the lowest and highest

128  nibbles of the  address space can  be reached without banking. The

ASSUME  instruction is undefined  for the 75402  as it contains neither

a  MBE flag nor an  MBS register; the initial  values cannot be changed

therefore.

F�MC16L

.......

Similar  to many other  families of microcontrollers,  this family suf-

fers  somewhat  from  its  designers  miserliness: registers of only 16

bits  width are  faced with  an address  space of  24 bits. Once again,

bank  registers had to fill  the gap. In detail,  these are PCB for the

progam  code, DTB for all data  accesses, ADB for indirect accesses via

RW2/RW6,  and SSB/USB for the  stacks. They may all  take values from 0

to  255 and are by default assumed to  be 0, with the exception of 0ffh

for PCB.

Furthermore,  a DPR  register exists  that specifies  which memory page

within  the 64K bank given by DTB  may be reached with 8 bit addresses.

The  default for DPR is 1, resulting  in a default page of 0001xxh when

one takes DTB's default into account.

MN1613

......

The  MN1613 is an  extension of an  architecture with 16 bit addresses.

The  address extension is  done by a  set of "segment registers" (CSBR,

SSBR,  TSR0, and TSR1), each  of which is four  bits wide. The contents

of  a segment  register, left-shifted  by 14  bits, is  added to the 16

bit  addresses. This way,  a process may  access a memory  window of 64

KWords  within the address space of 256 KWords. The assembler uses seg-

ment  register values reported  via ASSUME to  warn whether an absolute

address  is outside  the window  defined by  the used segment register.

If  the address is within  the window, it will  compute the correc t16-

bit  offset. Naturally,  this cannot  be done  when indirect addressing

is used.

IM61x0

......

These  micro processors  implement the  instruction set  of a PDP/8 and

therefore  fundamentally support an address  space of 4 KWords. Banking

allows  to extend this address to eight ''fields' of 4 KWords. Address-

ing  of data  and jumps  are principally  only possible within the same

field,  with one  exception: The  IB register  allows to perform a jump

to  another 4K field.  It provides the  upper three bits  of the 15 bit

target  address, if IB has  been set to a  value unequal to NOTHING via

ASSUME.

3.2.22.  CKPT

- - - - - - -

valid for: TI990/12

Type  12  instructions  require  a  checkpoint  register for execution.

This  register may either  be specified explicitly  as fourth argument,

or  a default  for all  following code  may be  given via this instruc-

tion.  If neither a CKPT instruction  nor an explicit checkpoint regis-

ter  was used, an error  is reported. The default  of no default regis-

ter may be restored by using NOTHING as argument to CKPT.

3.2.23.  EMULATED

- - - - - - - - -

valid for: 29K

AMD  defined the  29000's series  exception handling  for undefined in-

structions  in  a  way  that  there  is a separate exception vector for

each  instruction.  This  allows  to  extend  the  instruction set of a

smaller  member of this  family by a  software emulation. To avoid that

AS  quarrels about these instructions  as being undefined, the EMULATED

instruction  allows to  tell AS  that certain  instructions are allowed

in  this case. The check if the  currently set processors knows the in-

struction  is then  skipped. For  example, if  one has written a module

that  supports 32-bit  IEEE numbers  and the  processor does not have a

FPU, one writes

EMULATED FADD,FSUB,FMUL,FDIV

EMULATED FEQ,FGE,FGT,SQRT,CLASS

3.2.24. BRANCHEXT

- - - - - - - - -

valid for: XA

BRANCHEXT  with either  ON or  OFF as  argument tells  AS whether short

branches  that are only available with  an 8-bit displacement shall au-

tomatically  be 'extended', for example  by replacing a single instruc-

tion like

bne     target

with  a longer sequence  of same functionality,  in case the branc tar-

get  is out of  reach for the  instruction's displacement. For example,

the replacement sequence for bne would be

beq     skip

jmp     target

skip:

In  case there  is no  fitting 'opposite'  for an  instruction, the se-

quence may become even longer, e.g. for jbc:

jbc     dobr

bra     skip

dobr:   jmp     target

skip:

This  feature however has the side  effect that there is no unambigious

assignment  between machine  and assembly  code any  more. Furthermore,

additional  passes may  be the  result if  there are  forward branches.

One should therefore use this feature with caution!

3.2.25. Z80SYNTAX

- - - - - - - - -

G"ultigkeit: 8008, 8080/8085, �PD78xx

With  ON as  argument, one  can optionally  write machine assembler in-

structions  in the form  Zilog defined them  for the Z80. For instance,

you  simply use LD  with self-explaining operands  instead of MVI, LXI,

MOV, STA, LDA, SHLD, LHLD, LDAX, STAX or SPHL.

Since  some mnemonics  have a  different meaning  in 8008/8080  and Z80

syntax,  it is not possible to program in 'Z80 style' all the time, un-

less  the '8080  syntax' is  turned off  entirely by using EXCLUSIVE as

argument.  The details of this operation mode  can be looked up in sec-

tion 4.22.

A  built-in symbol  of same  name allows  to query  the operation mode.

The mapping is 0=OFF, 1=ON, and 2=EXCLUSIVE.

3.2.26.  EXPECT and ENDEXPECT

- - - - - - - - - - - - - - -

This  pair of instructions  may be used  to frame a  piece of code that

is  expected to trigger one  or more error or  warning messages. If the

errors  or warnings (identified by their numbers, see chapter A) do oc-

cur,  they  are  suppressed  and  assembly  continues without any error

(naturally,  without creating  code at  the erroneous places). However,

if  warnings or errors that were  expected do not occur, ENDEXPECT will

emit  errors about them. The main  usage scenario of these instructions

are  the self tests  in the tests/  subdirectory. For instance, one may

check this way if range checking of operands works as expected:

cpu      68000

expect   1320     ; immediate shift only for 1..8

lsl.l    #10,d0

endexpect

3.3. Data Definitions

---------------------

The  instructions described in this  section partially overlap in their

functionality,  but each processor  family defines other  names for the

same  function. To stay  compatible with the  standard assemblers, this

way of implementation was chosen.

If  not explicitly mentioned otherwise, all instructions for data depo-

sition  (not those for reservation of  memory!) allow an arbitrary num-

ber of parameters which are being processed from left to right.

3.3.1.  DC[.Size]

- - - - - - - - -

valid  for: 680x0,  M*Core, 68xx,  H8, SH7x00,  DSP56xxx, XA, ST7/STM8,

MN161x, IM61x0, CP-3F, SC61860

This  instruction places  one or  several constants  of the type speci-

fied  by the  attribute into  memory. The  attributes are the same ones

as  defined in section  2.5, and there  is additionally the possibility

for byte constants to place string constants in memory, like

String  dc.B "Hello world!\0"

The  parameter count may be  between 1 and 20.  A repeat count enclosed

in  brackets may additionally be prefixed  to each parameter; for exam-

ple,  one can for  example fill the  area up to  the next page boundary

with zeroes with a statement like

dc.b    [(*+255)&$ffffff00-*]0

CAUTION!  This function easily allows to reach  the limit of 1 Kbyte of

generated code per line!

The  assembler can automatically  add another byte  of data in case the

byte  sum should  become odd,  to keep  the word alignment. This behav-

iour may be turned on and off via the PADDING instruction.

Decimal  floating point numbers stored  with this instruction (DC.P...)

can  cover the  whole range  of extended  precision, one however has to

pay  attention to the detail  that the coprocessors currently available

from  Motorola (68881/68882)  ignore the  thousands digit  of the expo-

nent at the read of such constants!

The default attribute is W, that means 16-bit-integer numbers.

For  the DSP56xxx, the  data type is  fixed to integer  numbers (an at-

tribute  is therefore neither  necessary nor allowed),  which may be in

the  range  of  -8M  up  to  16M-1.  String constants are also allowed,

whereby three characters are packed into each word.

Opposed  to the  standard Motorola  assembler, it  is also valid to re-

serve  memory space  with this  statement, by  using a question mark as

operand.  This is an extension added  by some third-party suppliers for

68K  assemblers, similar to what  Intel assemblers provide. However, it

should  be clear  that usage  of this  feature may  lead to portability

problems.  Furthermore, question  marks as  operands must  not be mixed

with 'normal' constants in a single statement.

3.3.2.  DS[.Size]

- - - - - - - - -

valid  for: 680x0,  M*Core, 68xx,  H8, SH7x00,  DSP56xxx, XA, ST7/STM8,

MN161x, IM61x0, CP-3F, PPS-4, SC61860

On  the one hand, this instruction  enables to reserve memory space for

the  specified count  of numbers  of the  type given  by the attribute.

Therefore,

DS.B    20

for example reserves 20 bytes of memory, but

DS.X    20

reserves 240 bytes!

The  other purpose  is the  alignment of  the program  counter which is

achieved by a count specification of 0. In this way, with a

DS.W    0  ,

the  program counter will be rounded up  to the next even address, with

a

DS.D 0

in  contrast to  the next  double word  boundary. Memory cells possibly

staying  unused thereby are  neither zeroed nor  filled with NOPs, they

simply stay undefined.

The default for the operand length is - as usual - W, i.e. 16 bits.

For  the 56xxx, the operand  length is fixed to  words (of 24 bit), at-

tributes therefore do not exist just as in the case of DC.

3.3.3. BLKB, BLKW, BLKL, BLKD

- - - - - - - - - - - - - - -

valid for: Renesas RX

These  statements are used  to reserve memory  on Renesas RX. The total

amount  of  memory  reserved  is  the  instruction's argument times the

operand  size given by  the instruction (1  byte for BLKB,  2 bytes for

BLKW, 4 bytes for BLKL, and 8 bytes for BLKD).

3.3.4.  DN,DB,DW,DD,DQ,DT, and DO

- - - - - - - - - - - - - - - - -

valid for: Intel (except for 4004/4040), Zilog, Toshiba,

NEC, TMS370, Siemens, AMD, MELPS7700/65816,

M16(C), National, ST9, Atmel, TMS70Cxx, TMS1000,

Signetics, �PD77230, Fairchild, Intersil,

XS1, SC62015

These  commands are - one  could say - the  Intel counterpart to DS and

DC,  and as  expected, their  logic is  a little  bit different: First,

the specification of the operand length is moved into the mnemonic:

- DN: 4-bit integer

- DB: 8-bit byte or ASCII string similar to DC.B

- DW: 16-bit integer or half precision floating point

- DD: 32-bit integer or single precision floating point

- DQ: 64-bit integer or double precision floating point

- DT:  packed BCD  integer or  extended precision  floating point (80

bits)

- DO: quad precision floating point (128 bits)

Second,  the distinction between constant  definition and memory reser-

vation  is done by the operand. A  reservation of memory is marked by a

? :

db      ?       ; reserves a byte

dw      ?,?     ; reserves memory for 2 words (=4 byte)

dd      -1      ; places the constant -1 (FFFFFFFFH) !

Reserved  memory and constant definitions must  not be mixed within one

instruction:

db      "hello",?       ; --> error message

Additionally,  the DUP  Operator permits  the repeated  placing of con-

stant sequences or the reservation of whole memory blocks:

db      3 dup (1,2)     ; --> 1 2 1 2 1 2

dw      20 dup (?)      ; reserves 40 bytes of memory

As  you  can  see,  the  DUP-argument  must be enclosed in parentheses,

which  is also why it may consist of several components, that may them-

selves be DUPs...the stuff therefore works recursively.

DUP  is however also  a place where  one can get  in touch with another

limit  of the assembler:  a maximum of  1024 bytes of  code or data may

be  generated in  one line.  This is  not valid  for the reservation of

memory, only for the definition of constant arrays!

The  DUP operator only gets recognized if  it is itself not enclosed in

parentheses,  and if  there is  a non-empty  argument to its left. This

way, it is possible to use a symbol of same name as argument.

DB  and DW on 65xx and  68xx targets additionally support the 'Motorola

variant'  of the  DUP operator,  namely a  prefix that holds the number

of repetitions in square brackets.

Several  platforms define pseudo  instructions with same functionality,

but different names:

- In  order to be  compatible to the  M80, DEFB/DEFW may  be used in-

stead of DB/DW in Z80-mode.

- BYTE/ADDR  resp. WORD/ADDRW in COP4/8 mode  are an alias for DB re-

sp.  DW, with the  pairs differing in  byte order: instructions de-

fined  by National for  address storage use  big endian, BYTE resp.

WORD in contrast use little endian.

- BYTE  resp. WORD  on PDP-11,  VAX and  WD16 work  like DB resp. DW,

however only accept integer arguments.

- LWORD  resp. QUAD on VAXwork like  DD resp. DQ, however only accept

integer arguments.

- The Renesas RX target provides:

- BYTE (like DB)

- WORD (like DW, only integer arguments)

- LWORD (like DD, only integer arguments)

- FLOAT (like DD, only floating point arguments)

- DOUBLE (like DQ, only floating point arguments)

If  DB is used in  an address space that  is not byte addressable (like

the  Atmel AVR's CODE segment),  bytes are packed in  pairs into 16 bit

words,  according to the endianess given  by the architecture: for lit-

tle  endian, the LSB is  filled first. If the  total number of bytes is

odd,  one half of the last word  remains unused, just like the argument

list  had been padded. It  will also not be  used if another DB immedi-

ately  follows in source code. The analogous  is true for DN, just with

the  difference that two or  four nibbles are packet  into a byte or 16

bit word.

The  NEC 77230 is special  with its DW instruction:  It more works like

the  DATA statement of its smaller  brothers, but apart from string and

integer  arguments, it also  accepts floating point  values (and stores

them  in the  processor's proprietary  32-bit format).  There is no DUP

operator!

When  floating point  constants are  stored, they  cannot have a larger

precision  or range than the floating point  type used on the host sys-

tem.  For instance,  if the  common 64  bit IEEE  754 format is used on

the  host side,  the value  range is  limited to  roughly +/-1.8*10^308

(see  the variable FLOATMAX),  and the lowest  two resp. eight bytes of

DT resp. DO are filled with zeros.

3.3.5. FLT2, FLT3, FLT4

- - - - - - - - - - - -

valid for: PDP-11 (FLT2, FLT4),

WD16 (FLT3)

FLT2  and FLT4 are similar in function to DD bzw. DQ. However, they on-

ly  store floating point numbers (in DEC's own F and D formats) in mem-

ory.  The WD16 uses an own, 48  bits (three machine words) long format.

Floating  point numbers  in this  format can  be stored  in memory with

the FLT3 instruction.

3.3.6. x_FLOATING

- - - - - - - - -

valid for: VAX

These  instructions  store  floating  point  constants in DEC format in

memory.  x represents one  of the four  supported formats (F,  D, G and

H).  FLOAT  is  an  alias  for  F_FLOATING,  and DOUBLE is an alias for

D_FLOATING. Otherwise, these instructions work like DD and DQ.

3.3.7.  DS, DS8

- - - - - - - -

valid for: Intel, Zilog, Toshiba, NEC, TMS370, Siemens, AMD,

M16(C), National, ST9, TMS7000, TMS1000, Intersil,

6502, 68xx

With this instruction, you can reserve a memory area:

DS      <count>

It is an abbreviation of

DB      <count> DUP (?)

Although  this could easily  be made by  a macro, some  people grown up

with  Motorola CPUs (Hi Michael!) suggest  DS to be a built-in instruc-

tion...I hope they are satisfied now ;-)

DS8  is defined  as an  alias for  DS on  the National  SC14xxx. Beware

that  the code memory of  these processors is organized  in words of 16

bits,  it is therefore impossible to  reserve individual bytes. In case

the  argument of DS is odd, it will be rounded up to the next even num-

ber.

3.3.8. BLKx

- - - - - -

valid for: VAX

These  instructions reserve storage space for  the given number of ele-

ments,  with the element size coded into the instruction's last charac-

ter.  Therefore, the  size of  the reserved  area in  bytes is equal to

the  element count  for BLKB,  double the  count for  BLKW, and sixteen

times the count for BLKO and BLKH.

3.3.9. BYT or FCB

- - - - - - - - -

valid for: 6502, 68xx, SC61860

By  this instruction,  byte constants  or ASCII  strings are  placed in

65xx/68xx-mode,  it therefore  corresponds to  DC.B on  the 68000 or DB

on  Intel (which ias also allowed).  Similarly to DC, a repetition fac-

tor  enclosed in brackets ([..]) may be prepended to every single para-

meter.

3.3.10.  BYTE

- - - - - - -

valid for: ST6, 320C2(0)x, 320C5x, MSP, TMS9900, CP-1600

Ditto.  Note  that  when  in  320C2(0)x/5x  mode, the assembler assumes

that  a label on  the left side  of this instruction  has no type, i.e.

it  belongs to  no address  space. This  behaviour is  explained in the

processor-specific hints.

The  PADDING  instruction  allows  to  set  whether odd counts of bytes

shall be padded with a zero byte in MSP/TMS9900 mode.

The  operation of BYTE on CP-1600 is somewhat different: the 16-bit in-

teger  arguments are stored byte-wise in two consecutive words of memo-

ry  (LSB first).  If individual  8-bit values  shall be stored in memor

(optionally packed), use the TEXT instruction!

3.3.11. DC8

- - - - - -

valid for: SC144xx

This  statement is an  alias for DB,  i.e. it may  be used to dump byte

constants or strings to memory.

3.3.12.  ADR or FDB

- - - - - - - - - -

valid for: 6502, 68xx, SC61860

ADR  resp.  FDB  stores  word  constants  when in 65xx/68xx mode. It is

therefore  the equivalent  to DC.W  on the  68000 or  DW on Intel plat-

forms  (which is  also allowed).  Similarly to  DC, a repetition factor

enclosed  in brackets ([..])  may be prepended  to every single parame-

ter.

3.3.13. DDB

- - - - - -

valid for: 6502, MELPS-7700

This  instructions works similar to ADR,  just with the difference that

the 16 bit values are stored in big endian order.

3.3.14. DCM

- - - - - -

valid for: 6502

This  instructions allows to dispose  floating point constants in memo-

ry,  in the  format that  described in  [3]: An  exponent of eight bits

and  a mantissa of  24 bits in  two's complement representation, stored

in big-endian order.

3.3.15.  WORD

- - - - - - -

valid for: ST6, i960, 320C2(0)x, 320C3x/C4x/C5x, MSP, CP-1600,

IMP-16, IPC-16

If  assembling for the  320C3x/C4x or i960,  this command stores 32-bit

words,  16-bit  words  for  the  other  families.  Note  that  when  in

320C2(0)x/5x  mode,  the  assembler  assumes  that  a label on the left

side  of this instruction  has no type,  i.e. it belongs  to no address

space.  This  behaviour  is  explained  at the discussion on processor-

specific hints.

3.3.16.  DW16

- - - - - - -

valid for: SC144xx

This  instruction is for  SC144xx targets a  way to dump  word (16 bit)

constants to memory. CAUTION!! It is therefore an alias for DW.

3.3.17.  ACON

- - - - - - -

valid for: 2650

ACON  works the same way  as DW, the 16  bit numbers are however stored

in big endian format.

3.3.18.  LONG

- - - - - - -

valid for: 320C2(0)x, 320C5x

LONG  stores a 32-bit  integer to memory  with the order LoWord-HiWord.

Note  that when in 320C2(0)x/5x mode,  the assembler assumes that a la-

bel  on the left side of this  instruction has no type, i.e. it belongs

to  no address  space. This  behaviour is  explained in  the processor-

specific hints.

3.3.19.  SINGLE, DOUBLE, and EXTENDED

- - - - - - - - - - - - - - - - - - -

valid for: 320C3x/C4x (not DOUBLE), 320C6x (not EXTENDED)

Both  commands  store  floating-point  constants  to memory. In case of

the  320C3x/C4x,  they  are  not  stored  in  IEEE-format.  Instead the

processor-specific  formats with 32 and 40 bit are used. In case of EX-

TENDED  the resulting constant occupies two memory words. The most sig-

nificant  8 bits  (the exponent)  are written  to the  first word while

the other ones (the mantissa) are copied into the second word.

3.3.20.  FLOAT and DOUBLE

- - - - - - - - - - - - -

valid for: 320C2(0)x, 320C5x

These  two commands store floating-point  constants in memory using the

standard  IEEE 32-bit  and 64-bit  IEEE formats.  The least significant

byte  is copied to the first  allocated memory location. Note that when

in  320C2(0)x/5x  mode  the  assembler  assumes  that all labels on the

left  side of an instruction  have no type, i.e.  they belong to no ad-

dress  space.  This  behaviour  is  explained in the processor-specific

hints.

3.3.21. SINGLE and DOUBLE

- - - - - - - - - - - - -

valid for: TMS99xxx

These  two commands store floating-point  constants in memory using the

processor's  floating  point  format,  which  is  equal  to the IBM/360

floating point format.

3.3.22.  EFLOAT, BFLOAT, and TFLOAT

- - - - - - - - - - - - - - - - - -

valid for: 320C2(0)x, 320C5x

Another  three floating  point commands.  All of  them support non-IEEE

formats, which should be easily applicable on signal processors:

- EFLOAT: mantissa with 16 bits, exponent with 16 bits

- BFLOAT: mantissa with 32 bits, exponent with 16 bits

- TFLOAT: mantissa with 64 bits, exponent with 32 bits

The  three commands share  a common storage  strategy. In all cases the

mantissa  precedes the exponent in memory,  both are stored as 2's com-

plement  with  the  least  significant  byte  first.  Note that when in

320C2(0)x/5x  mode the  assembler assumes  that all  labels on the left

side  of an instruction  have no type,  i.e. they belong  to no address

space. This behaviour is explained in the processor-specific hints.

3.3.23.  Qxx and LQxx

- - - - - - - - - - -

valid for: 320C2(0)x, 320C5x

Qxx  and LQxx can be  used to generate constants  in a fixed point for-

mat.  xx denotes a  2-digit number. The  operand is first multiplied by

2  ^xx before converting it  to binary notation. Thus  xx can be viewed

as  the  number  of  bits  which  should be reserved for the fractional

part  of the constant in  fixed point format. Qxx  stores only one word

(16 bit) while LQxx stores two words (low word first):

q05     2.5     ; --> 0050h

lq20    ConstPI ; --> 43F7h 0032h

Please  do not  flame me  in case  I calculated  something wrong  on my

HP28...

3.3.24.  DATA

- - - - - - -

valid  for: PIC,  320xx, AVR,  MELPS-4500, H8/500,  HMCS400, 4004/4040,

�PD772x, OLMS-40/50, Padauk

This  command stores data  in the current  segment. Both integer values

as  well as character  strings are supported.  On 16C5x/16C8x, 17C4x in

data  segment,  and  on  the  4500,  4004, and HMCS400 in code segment,

characters  occupy one word. On AVR,  17C4x in code segment, �PD772x in

the  data segments, and  on 3201x/3202x, in  general two characters fit

into  one  word  (LSB  first).  The  �PD77C25 can hold three bytees per

word  in the code segment. When  in 320C3x/C4x, mode the assembler puts

four  characters into one word (MSB first). In contrast to this charac-

ters  occupy two memory locations in the data segment of the 4500, sim-

ilar  in the 4004 and HMCS400.  The range of integer values corresponds

to  the word width of each processor  in a specific segment. This means

that  DATA has the same  result than WORD on  a 320C3x/C4x (and that of

SINGLE if AS recognizes the operand as a floating-point constant).

3.3.25. ZERO, CP-1600

- - - - - - - - - - -

valid for: PIC

Generates  a continuous string of zero  words in memory (which equals a

NOP on PIC).

3.3.26. FB and FW

- - - - - - - - -

valid for: COP4/8

These  instruction allow to fill memory blocks with a byte or word con-

stant.  The first operand specifies the  size of the memory block while

the second one sets the filling constant itself.

3.3.27. ASCII, ASCIC, and ASCIZ

- - - - - - - - - - - - - - - -

valid for: ST6, PDP-11, VAX, IMP-16, IPC-16 ( ASCII)

ST6, PDP-11, VAX ( ASCIZ)

PDP-11, VAX ( ASCIC)

These  commands store  string constants  to memory.  While ASCII writes

the  character information only,  ASCIZ additionally appends  a zero to

the end of the string, and ASCIC prepends a length byte.

3.3.28.  STRING and RSTRING

- - - - - - - - - - - - - -

valid for: 320C2(0)x, 320C5x

These  commands are functionally  equivalent to DATA,  but integer val-

ues  are limited to the range of  byte values. This enables two charac-

ters  or numbers to be packed together into one word. Both commands on-

ly  differ in the order they use  to write bytes: STRING stores the up-

per  one first then the  lower one, RSTRING does  this vice versa. Note

that  when in 320C2(0)x/5x  mode the assembler  assumes that a label on

the  left side of this  instruction has no type,  i.e. it belongs to no

address  space. This  behaviour is  explained in the processor-specific

hints.

3.3.29.  PACKED

- - - - - - - -

valid for: PDP-11, VAX

This  instruction may  be use  to put  numbers in packed decimal format

into  mmemory. Ist argument  is either a  (signed) integer number, or a

string.  Of course, the latter may only  contain characters from 0 to 9

and  an optional plus or minus sign  at the beginning. The maximum num-

ber  of digits is 31, and the sign  is appended as last digit. The val-

ues  12 resp. 13  represent a plus  resp. minus sign.  If the number of

digits plus sign is odd, a zero digit is inserted at the beginning.

Optionally,  a symbol's name may be  given as second argument. The num-

ber  of  digits,  excluding  the  sign  and  the  optional null pad, is

stored in this symbol.

3.3.30. RADIX50

- - - - - - - -

valid for: diverse

This  instruction may be  used to put  strings in packed RADIX50 format

into  memory. This  coding was  common for  file names  in DEC environ-

ments  and packs three characters into one  16 bit word. RADIX50 is not

a  built-in  instruction.  Instead,  it  is  defined  as a macro in the

radix50.inc include file.

3.3.31. FCC

- - - - - -

valid for: 6502, 68xx

When  in 65xx/68xx mode, string constants  are generated using this in-

struction.  In contrast  to the  original assembler  AS11 from Motorola

(this  is the  main reason  why AS  understands this command, the func-

tionality  is contained  within the  BYT instruction)  you must enclose

the  string argument by double quotation marks instead of single quota-

tion  marks or slashes.  Similarly to DC,  a repetition factor enclosed

in brackets ([..]) may be prepended to every single parameter.

3.3.32.  TEXT

- - - - - - -

In  CP-1600 mode,  This instruction  is used  to store string constants

in packed format, i.e. two characters per word.

3.3.33.  DFS or RMB

- - - - - - - - - -

valid for: 6502, 68xx

Reserves  a memory  block when  in 6502/68xx  mode. It is therefore the

equivalent to DS.B on the 68000 or DB ? on Intel platforms.

3.3.34. BLOCK

- - - - - - -

valid for: ST6

Ditto.

3.3.35. SPACE

- - - - - - -

valid for: i960

Ditto.

3.3.36. RES

- - - - - -

valid  for: PIC,  MELPS-4500, HMCS400,  3201x, 320C2(0)x,  320C5x, AVR,

�PD772x, OLMS-40/50, Padauk, CP-1600, PPS-4, 2650

This  command allocates  memory. When  used in  code segments the argu-

ment  counts words (10/12/14/16 bit). In  data segments it counts bytes

for  PICs, nibbles  for 4500,  PPS-4, and  OLMS-40/50 and words for the

TI devices.

3.3.37. BSS

- - - - - -

valid for: 320C2(0)x, 320C3x/C4x/C5x/C6x, MSP

BSS  works like RES,  but when in  320C2(0)x/5x mode, the assembler as-

sumes  that a label on  the left side of  this instruction has no type,

i.e  it belongs  to no  address space.  This behaviour  is explained in

the processor-specific hints.

3.3.38. DSB and DSW

- - - - - - - - - -

valid for: COP4/8

Both  instructions allocate  memory and  ensure compatibility to ASMCOP

from  National. While  DSB takes  the argument  as byte count, DSW uses

it as word count (thus it allocates twice as much memory than DSB).

3.3.39.  DS16

- - - - - - -

valid for: SC144xx

This  instruction  reserves  memory  in  steps  of  full words, i.e. 16

bits. It is an alias for DW.

3.3.40. ALIGN

- - - - - - -

valid for: all processors

Takes  the argument to  align the program  counter to a certain address

boundary.  AS increments  the program  counter to  the next multiple of

the  argument.  So,  ALIGN  corresponds  to  DS.x on 68000, but is much

more flexible at the same time.

Example:

align     2

aligns  to an even  address (PC mod  2 = 0).  If Align is  used in this

form  with only one argument, the  contents of the skipped memory space

is  not defined. An optinal  second argument may be  used to define the

(byte) value used to fill the area.

3.3.41. LTORG

- - - - - - -

valid for: SH7x00, IM61x0, IMP-16, IPC-16

Although  the SH7000 processor  can do an  immediate register load with

8  bit only, AS  shows up with  no such restriction.  This behaviour is

instead  simulated  through  constants  in  memory. Storing them in the

code  segment (not far  away from the  register load instruction) would

require  an  additional  jump.  AS  Therefore  gathers the constants an

stores  them at  an address  specified by  LTORG. Details are explained

in the processor-specific section somewhat later.

3.4. Macro Instructions

-----------------------

valid for: all processors

Now  we finally reach the things  that make a macro assembler different

from  an ordinary assembler:  the ability to  define macros (guessed it

!?).

When  speaking about 'macros', I generally  mean a sequence of (machine

or  pseudo) instructions which are united  to a block by special state-

ments  and can  then be  treated in  certain ways.  The assembler knows

the following statements to work with such blocks:

3.4.1.  MACRO

- - - - - - -

is  probably the most important  instruction for macro programming. The

instruction sequence

<name>  MACRO   [parameter list]

<instructions>

ENDM

defines  the  macro  <name>  to  be  the enclosed instruction sequence.

This  definition by  itself does  not generate  any code! In turn, from

now  on the instruction sequence can simply  be called by the name, the

whole  construct therefore shortens and  simplifies programs. A parame-

ter  list may  be added  to the  macro definition  to make  things even

more useful.

While  a macro's name only  has to conform to  the usual rules for sym-

bol  names (see section 2.7), there  is the additional restriction that

parameter  names  may  not  contain  underscore characters. This may be

changed  via  the  -underscore-macroargs  command  line  argument,  but

sinve  it has additional  side effects, it  should only be  used if ab-

solutely necessary.

A  switch to case-sensitive mode influences  both macro names and para-

meters.

Similar  to symbols, macros  are local, i.e.  they are only  known in a

section  and its subsections when the  definition is done from within a

section.  This behaviour however  can be controlled  in wide limits via

the options PUBLIC and GLOBAL described below.

A  default value  may be  provided for  each macro  parameter (appended

via  an equal  sign). This  value is  used if  there is no argument for

this  parameter at  macro call  or if  the positional argument (see be-

low) for this parameter is empty.

Apart  from  the  macro  parameters  themselves, the parameter list may

contain  control  parameters  which  influence  the  processing  of the

macro.  These parameters  are distinguished  from normal  parameters by

being  enclosed in curly  braces. The following  control parameters are

defined:

- EXPAND/NOEXPAND:  rule whether  the enclosed  code shall be written

to  the listing when the macro is expanded. The default is the val-

ue set by the pseudo instruction MACEXP_DFT.

- EXPIF/NOEXPIF:  rule whether instructions  for conditional assembly

and  code excluded by it  shall be written to  the listing when the

macro  is expanded. The default is the  value set by the pseudo in-

struction MACEXP_DFT.

- EXPMACRO/NOEXPMACRO:  rule  whether  macros  defined in the macro's

body  shall be written  to the listing  when the macro is expanded.

The  default  is  the  value  set  by  the  pseudo instruction MAC-

EXP_DFT.

- EXPREST/NOEXPREST  : rule whether a  macro body's lines not fitting

into  the  first  two  categories  shall  be written to the listing

when  the macro is  expanded. The default  is the value  set by the

pseudo instruction MACEXP_DFT.

- PUBLIC[:section  name]: assigns the  macro to a  parent section in-

stead  of the current  section. A section  can make macros accessi-

ble  for the outer  code this way.  If the section specification is

missing,  the macro becomes completely global,  i.e. it may be ref-

erenced from everywhere.

- GLOBAL[:section  name]:  rules  that  in  addition to the macro it-

self,  another macro shall be generated  that has the same contents

but  is assigned to the specified  section. Its name is constructed

by  concatenating  the  current  section's  name to the macro name.

The  section specified must be a parent section of the current sec-

tion;  if the  specification is  missing, the  additional macro be-

comes  globally visible. For example, if a  macro A is defined in a

section  B  that  is  a  child  section of section C, an additional

global  macro named  C_B_A would  be generated.  In contrast,  if C

had  been specified  as target  section, the  macro would  be named

B_A  and be assigned to section C. This option is turned off by de-

fault  and it only has an effect when it is used from within a sec-

tion. The macro defined locally is not influenced by this option.

- EXPORT/NOEXPORT:  rules whether the definition  of this macro shall

be  written to a separate  file in case the  -M command line option

was  given.  This  way,  definitions  of  'private'  macros  may be

mapped  out selectively. The default  is FALSE, i.e. the definition

will  not be written  to the file.  The macro will  be written with

the  concatenated  name  if  the  GLOBAL  option  was  additionally

present.

- INTLABEL/NOINTLABEL  :  rules  whether  a  label  defined in a line

that  calls this macro  may be used  as an additional parameter in-

side the label or not, instead of simply 'labeling' the line.

- GLOBALSYMBOLS/NOGLOBALSYMBOLS  :  rules  whether  labels defined in

the  macro's body shall  be local to  this macro or  also be avail-

able  outside the macro.  The default is  to keep them local, since

using a macro multiple time would be difficult otherwise.

The  control parameters described above  are removed from the parameter

list  by AS, i.e.  they do not  have a further  influence on processing

and usage.

When  a macro is called, the parameters given for the call are textual-

ly  inserted  into  the  instruction  block and the resulting assembler

code  is assembled  as usual.  Zero length  parameters are  inserted in

case  too few  parameters are  specified. It  is important to note that

string  constants are not protected from  macro expansions. The old IBM

rule:

It's not a bug, it's a feature!

applies  for this detail. The gap was left to allow checking of parame-

ters  via string comparisons. For example,  one can analyze a macro pa-

rameter in the following way:

mul     MACRO   para,parb

IF      UpString("PARA")<>"A"

MOV    a,para

ENDIF

IF      UpString("PARB")<>"B"

MOV    b,parb

ENDIF

!mul     ab

ENDM

It  is important for the example  above that the assembler converts all

parameter  names  to  upper  case  when  operating  in case-insensitive

mode,  but  this  conversion  never  takes  place inside of string con-

stants.  Macro parameter  names therefore  have to  be written in upper

case when they appear in string constants.

Macro  parameter  expansion  furthermore  depends  on whether parameter

names  may contain  underscores or  not. If  not (the  default), under-

scores  in the macro body also have  a function of 'limiters' to detect

parameter names. So in the following example:

setled  macro led,value

out   led,value

ld    led_shadow,value

endm

the  parameter 'led'  would be  replaced in  both source lines, whereas

only  in the first one if  the command line switch underscore-macroargs

is  used. Several of the include files  shipped with AS rely on the de-

fault  behaviour.  This  switch  should  therefore  only be used if ab-

solutely necessary.

Macro  arguments may  be given  in either  of two  forms: positional or

keyword arguments.

For  positional arguments, the assignment of arguments to macro parame-

ters  simply results from the position of arguments, i.e. the first ar-

gument  is assigned to the first  parameter, the second argument to the

second  parameter  and  so  on.  If  the number of arguments is smaller

than  the number  of parameters,  eventually defined  default values or

simply  an empty string are inserted. The same is valid for empty argu-

ments in the argument list.

Keyword  arguments on the other  hand explicitly define which parameter

they relate to, by being prefixed with the parameter's name:

mul  para=r0,parb=r1

Again,  non-assigned parameters will use  an eventually defined default

or an empty string.

As  a difference  to positional  arguments, keyword  arguments allow to

assign an empty string to a parameter with a non-empty default.

Mixing  of positional and keyword arguments in one macro call is possi-

ble,  however it is  not allowed to  use positional arguments after the

first keyword argument.

The  same naming rules as for usual  symbols also apply for macro para-

meters,  with the exception that only  letters and numbers are allowed,

i.e.  dots and underscores are forbidden.  This constraint has its rea-

son  in a  hidden feature:  the underscore  allows to concatenate macro

parameter names to a symbol, like in the following example:

concat  macro   part1,part2

call    part1_part2

endm

The call

concat  module,function

will therefore result in

call    module_function

Apart  from the parameters  explicitly declared for  a macro, four more

'implicitly'   declared  parameters   exist.  Since   they  are  always

present, they cannot not be redeclared as explicit parameters:

-  ATTRIBUTE  refers to the attribute appended  to the macro call, in

case  the currently active architecture supports attributes for ma-

chine instructions. See below for an example!

-  ALLARGS  refers to a comma-separated  list of all arguments passed

to a macro, usable e.g. to pass them on to a IRP statement.

-  ARGCOUNT  refers to  the actual  count of  parameters passed  to a

macro.  Note however that this number  is never lower than the for-

mal  parameter count  of the  macro, since  AS will fill up missing

arguments with empty strings!

-  __LABEL__  refers  to  a  label  present  in a line that calls the

macro.  This replacement  only takes  place if  the INTLABEL option

was set for this macro!

IMPORTANT:  the  names  of  these  implicit  parameters  are also case-

insensitive if AS was told to operate case-sensitive!

The  purpose of being  able to 'internally'  use a label  in a macro is

surely  not immediately obvious. There might  be cases where moving the

macro's  entry point  into its  body may  be useful. The most important

application  however are  TI signal  processors that  use a double pipe

symbol in the label's column to mark parallelism, like this:

instr1

||  instr2

(since  both instructions  merge into  a single  word of  machine code,

you  cannot branch  to the  second instruction  - so  occupying the la-

bel's  position  doesn't  hurt).  The  problem  is  however  that  some

'convenience  instructions' are  realized as  macros. A parallelization

symbol  written in front of a macro  call normally would be assigned to

the  macro itself, not to the  macro body's first instruction. However,

things work with this trick:

myinstr    macro {INTLABEL}

__LABEL__  instr2

endm

instr1

||         myinstr

The  result after expanding myinstr is  identical to the previous exam-

ple without macro.

Recursion  of macros,  i.e. the  repeated call  of a  macro from within

its  own body is completely legal. However,  like for any other sort of

recursion,  one has to  assure that there  is an end  at someplace. For

cases  where one  forgot this,  AS keeps  an internal counter for every

macro  that is  incremented when  an expansion  of this  macro is begun

and  decremented again when the expansion  is completed. In case of re-

cursive  calls, this counter  reaches higher and  higher values, and at

a  limit settable  via NESTMAX,  AS will  refuse to  expand. Be careful

when  you turn off this emergency  brake: the memory consumption on the

heap may go beyond all limits and even shut down a Unix system...

A small example to remove all clarities ;-)

A  programmer  braindamaged  by  years  of programming Intel processors

wants  to have the instructions PUSH/POP  also for the 68000. He solves

the 'problem' in the following way:

push    macro   op

move.ATTRIBUTE op,-(sp)

endm

pop     macro   op

move.ATTRIBUTE (sp)+,op

endm

If one writes

push    d0

pop.l   a2    ,

this results in

move.   d0,-(sp)

move.l  (sp)+,a2

A macro definition must not cross include file boundaries.

Labels  defined in  macros always  are regarded  as being local, unless

the  GLOBALSYMBOLS was used in the  macro's definition. If a single la-

bel  shall be made public in a  macro that uses local labels otherwise,

it  may be defined  with a LABEL  statement which always creates global

symbols (similar to BIT, SFR...):

<Name>  label   $

When  parsing a line, the assembler  first checks the macro list after-

wards  looks for processor  instructions, which is  why macros allow to

redefine  processor  instructions.  However,  the definition should ap-

pear  previously to  the first  invocation of  the instruction to avoid

phase errors like in the following example:

bsr     target

bsr     macro   targ

jsr     targ

endm

bsr     target

In  the first pass, the macro is  not known when the first BSR instruc-

tion  is assembled; an  instruction with 4  bytes of length is generat-

ed.  In the  second pass  however, the  macro definition is immediately

available  (from the first pass), a JSR  of 6 bytes length is therefore

generated.  As a  result, all  labels following  are too  low by  2 and

phase  errors occur  for them.  An additional  pass is necessary to re-

solve this.

Because  a machine  or pseudo  instruction becomes  hidden when a macro

of  same name  is defined,  there is  a backdoor  to reach the original

meaning:  the search for  macros is suppressed  if the name is prefixed

with  an exclamation mark (!).  This may come in  handy if one wants to

extend  existing instructions  in their  functionality, e.g.  the TLCS-

90's shift instructions:

srl     macro   op,n            ; shift by n places

rept    n               ; n simple instructions

!srl   op

endm

endm

From now on, the SRL instruction has an additional parameter...

Macro Expansion in the Listing

..............................

If  a macro is being called, the macro's body is included in the assem-

bly  listing, after arguemnts have been expanded. This can significant-

ly  increase the listing's size and make  it hard to read. It is there-

fore  possible to suppress  this expansion totally  or in parts. Funda-

mentally,  AS divides the source lines  contained in a macro's body in-

to three classes:

- Macro  definitions,  i.e.  the  macro  is  used  to  define another

macro, or it contains REPT/IRP/IRPC/WHILE blocks.

- Instructions  for conditional  assembly plus  any source lines that

have  not been assembled due  to conditional assembly. Since condi-

tional  assembly may depend on macro arguments, this subset may al-

so vary.

- All  remaining sourc  elines that  do not  fall under the first two

categories.

Which  parts  occur  in  the  listing  may  be defined individually for

every  macro. When defining a macro, the  default is the set defined by

the  most recent MACEXP_DFT instruction (3.7.3).  If one of the EXPAND/

NOEXPAND,  EXPIF/NOEXPIF EXPMACRO/NOEXPMACRO,  or EXPREST/NOEXPREST di-

rectives  is  used  in  the  macro's definition, they act additionally,

but  with higher preference.  For instance, if  expansion had been dis-

abled  completely (MACEXP_DFT  OFF), adding  the directive  EXPREST has

the  effect that when using  this macro, only lines  are written to the

listing  that remain after conditional assembly and are no macro defin-

itions themselves.

In  consequence,  changing  the  set  via  MACEXP_DFT  has no effect on

macros  that  have  been  defined  before this statement. The listing's

section  shows for defined macros the effective set of expansion direc-

tives.  The  list  given  in  curly  braces  is shorted so that it only

conatins  the last (and therefore valid)  directive for a certain class

of  source lines. A  NOIF given via  MACEXP_DFT will therefore not show

up if the directive EXPIF had been given specifically for this macro.

There  might  be  cases  where  it  is useful to override the expansion

rules  for a certain macro, regardless  whether they were given by MAC-

EXP_DFT  or individual directives. The statement MACEXP_OVR (3.7.3) ex-

ists  for such cases. It only has an effects on macros subsequently be-

ing  expanded. Once again, directives given by this instruction are re-

garded  in addition to a macro's rules, and they do with higher priori-

ty. A MACEXP_OVR without any arguments disables such an override.

3.4.2.  IRP

- - - - - -

is  a simplified macro definition for  the case that an instruction se-

quence  shall be applied  to a couple  of operands and  the the code is

not  needed any more afterwards. IRP needs  a symbol for the operand as

its  first parameter,  and an  (almost) arbitrary  number of parameters

that  are sequentially  inserted into  the block  of code. For example,

one can write

irp     op, acc,b,dpl,dph

push    op

endm

to push a couple of registers to the stack, what results in

push    acc

push    b

push    dpl

push    dph

Analog  to a macro  definition, the argument  list may contain the fol-

lowing  control parameters (marked  as such by  being enclosed in curly

braces):

- GLOBALSYMBOLS  resp.  NOGLOBALSYMBOLS  control  whether defined la-

bels are local for every individual pass or not.

- EXPAND resp. NOEXPAND

- EXPIF resp. NOEXPIF

- EXPMACRO resp. NOEXPMACRO

- EXPREST resp. NOEXPREST

3.4.3. IRPC

- - - - - -

IRPC  is a variant of IRP where  the first argument's occurences in the

lines  up  to  ENDM  are  successively  replaced by the characters of a

string  instead of further parameters.  For example, an especially com-

plicated way of placing a string into memory would be:

irpc    char,"Hello World"

db      'CHAR'

endm

CAUTION!  As  the  example  already  shows,  IRPC only inserts the pure

character;  it is the  programmer's task to  assure that valid code re-

sults  (in this example by inserting  quotes, including the detail that

no automatic conversion to uppercase characters is done).

3.4.4. REPT

- - - - - -

is  the simplest way to employ  macro constructs. The code between REPT

and  ENDM is assembled as often as  the integer argument of REPT speci-

fies.  This statement is commonly used in small loops to replace a pro-

grammed loop to save the loop overhead.

An example for the sake of completeness:

rept    3

rr      a

endm

rotates the accumulator to the right by three digits.

The allowed control directives are the same as for IRP.

In  case REPT's argument  is equal to  or smaller than  0, no expansion

at  all is done. This  is different to older  versions of AS which used

to  be a bit 'sloppy'  in this respect and  always made a single expan-

sion.

3.4.5.  WHILE

- - - - - - -

WHILE  operates similarly to REPT, but  the fixed number of repetitions

given  as an  argument is  replaced by  a boolean  expression. The code

framed  by WHILE  and ENDM  is assembled  until the  expression becomes

logically  false. This may  mean in the  extreme case that the enclosed

code  is not assembled at all in  case the expression was already false

when  the construct was  found. On the  other hand, it  may happen that

the  expression  stays  true  forever  and AS will run infinitely...one

should  apply  therefore  a  bit  of  accuracy  when one uses this con-

struct,  i.e. the  code must  contain a  statement that  influences the

condition, e.g. like this:

cnt     set     1

sq      set     cnt*cnt

while   sq<=1000

dc.l    sq

cnt      set     cnt+1

sq       set     cnt*cnt

endm

This example stores all square numbers up to 1000 to memory.

The allowed control directives are the same as for IRP and REPT.

Currently  there exists a  little ugly detail  for WHILE: an additional

empty  line that was not present in  the code itself is added after the

last  expansion. This  is a  'side effect'  based on  a weakness of the

macro  processor and it is  unfortunately not that easy  to fix. I hope

noone minds...

3.4.6.  EXITM

- - - - - - -

EXITM  offers a way  to terminate a  macro expansion or  one of the in-

structions  REPT, IRP, or  WHILE prematurely. Such  an option helps for

example  to replace  encapsulations with  IF-ENDIF-ladders in macros by

something  more readable. Of  course, an EXITM  itself always has to be

conditional,  what leads  us to  an important  detail: When an EXITM is

executed,  the stack of open  IF and SWITCH constructs  is reset to the

state  it had just before the  macro expansion started. This is impera-

tive  for conditional  EXITM's as  the ENDIF  resp. ENDCASE that frames

the  EXITM statement will  not be reached  any more; AS  would print an

error  message without this trick. Please  keep also in mind that EXITM

always  only  terminates  the  innermost  construct if macro constructs

are  nested!  If  one  want  to  completely  break out of a nested con-

struct, one has to use additional EXITM's on the higher levels!

3.4.7.  SHIFT

- - - - - - -

SHIFT  is a tool  to construct macros  with variable argument lists: it

discards  the first parameter, with the  result that the second parame-

ter  takes its place and  so on. This way  one could process a variable

argument  list...if you do  it the right  way. For example, the follow-

ing does not work...

pushlist  macro reg

rept  ARGCOUNT

push  reg

shift

endm

endm

...because  the macro  gets expanded  once, its  output is  captured by

REPT  and  then  executed  n  times.  Therefore,  the first argument is

saved n times...the following approach works better:

pushlist  macro reg

if      "REG"<>""

push    reg

shift

pushlist ALLARGS

endif

endm

Effectively,  this is a recursion that  shortens the argument list once

per  step. The important trick is that  a new macro expansion is start-

ed in each step...

In  case SHIFT ist already a  machine instruction for a certain target,

SHFT  may be used instead, or  the pseudo instruction is referenced ex-

plicitly by prepending a period (.SHIFT instead of SHIFT).

3.4.8.  MAXNEST

- - - - - - - -

MAXNEST  allows to  adjust how  often a  mcro may be called recursively

before  AS terminates with an error message. The argument may be an ar-

bitrary  positive integer value,  with the special  value 0 turning the

this  security brake completely off (be  careful with that...). The de-

fault  value for  the maximum  nesting level  is 256; its current value

may be read from the integer symbol of same name.

3.4.9. FUNCTION

- - - - - - - -

Though  FUNCTION is not  a macro statement  in the inner  sense, I will

describe  this instruction at this place  because it uses similar prin-

ciples like macro replacements.

This  instruction  is  used  to  define  new functions that may then be

used  in formula expressions like  predefined functions. The definition

must have the following form:

<name>  FUNCTION <arg>,..,<arg>,<expression>

The  arguments are the values that are 'fed into' the function. The de-

finition  uses symbolic  names for  the arguments.  The assembler knows

by  this that where  to insert the  actual values when  the function is

called. This can be seen from the following example:

isdigit FUNCTION ch,(ch>='0')&&(ch<='9')

This  function checks  whether the  argument (interpreted  as a charac-

ter)  is a number  in the currently  valid character set (the character

set can be modified via CHARSET, therefore the careful wording).

The  arguments' names  (CH in  this case)  must conform to the stricter

rules  for macro parameter  names, i.e. the  special characters . and _

are not allowed.

User-defined  functions can  be used  in the  same way as builtin func-

tions,  i.e. with a  list of parameters,  separated by commas, enclosed

in parentheses:

IF isdigit(char)

message "\{char} is a number"

ELSEIF

message "\{char} is not a number"

ENDIF

When  the function  is called,  all parameters  are calculated once and

are  then inserted into the function's  formula. This is done to reduce

calculation  overhead and to  avoid side effects.  The individual argu-

ments  have to  be separated  by commas  when a  function has more than

one parameter.

CAUTION!  Similar  to  macros,  one  can  use user-defined functions to

override  builtin functions.  This is  a possible  source for phase er-

rors.  Such  definitions  therefore  should  be  done  before the first

call!

The  result's type may depend  on the type of  the input arguments. For

example, the function

double  function x,x+x

may  have an integer,  a float, or  even a string  as result, depending

on the argument's type!

When  AS operates in case-sensitive mode,  the case matters when defin-

ing  or  referencing  user-defined  functions,  in  contrast to builtin

functions!

3.5. Structures

---------------

valid for: all processors

Even  in assembly language  programs, there is  sometimes the necessity

to  define composed  data structures,  similar to high-level languages.

AS  supports both the definition and  usage of structures with a couple

of  statements. These  statements shall  be explained  in the following

section.

3.5.1. Definition

- - - - - - - - -

The  definiton of  a structure  is begun  with the statement STRUCT and

ends  with ENDSTRUCT (lazy  people may also  write STRUC resp. ENDSTRUC

or  ENDS  instead).  A  optional  label preceding these instructions is

taken  as the name  of the structure  to be defined;  it is optional at

the  end of the definition and may  be used to redefine the length sym-

bol's  name (see  below). The  remaining procedure  is simple: Together

with  STRUCT, the cuurent  program counter is  saved and reset to zero.

All  labels defined between STRUCT and ENDSTRUCT therefore are the off-

sets  of the structure's  data fields. Reserving  space is done via the

same  instructions that  are also  otherwise used  for reserving space,

like  e.g. DS.x for Motorola  CPUs or DB &  co. for Intel-style proces-

sors.  The rules for  rounding up lengths  to assure certain alignments

also  apply here - if  one wants to define  'packed' structures, a pre-

ceding  PADDING OFF  may be  necessary. Vice  versa, alignments  may be

forced with ALIGN or similar instructions.

Since  such a  definition only  represents a  sort of 'prototype', only

instructions  that  reserve  memory  may  be used, no instructions that

dispose constants or generate code.

Labels  defined inside  structures (i.e.  the elements'  names) are not

stored  as-is. Instead, the structure's name is prepended to them, sep-

arated  with  a  special  character.  By  default, this is the underbar

(_).  This behaviour however may be  modified with two arguments passed

to the STRUCT statement:

- NOEXTNAMES  suppressed the  prepending of  the structure's name. In

this  case, it  is the  programmer's responsibility  to assure that

field names are not used more than once.

- DOTS  instructs AS to  use the dot  as connecting character instead

of  the underbar. It should however  be pointed out that on certain

target  architectures, the  dot has  a special  meaning for bit ad-

dressing, which may lead to problems!

It  is futhermore possible to turn the usage  of a dot on resp. off for

all following structures:

dottedstructs <on|off>

Aside  from the  element names,  AS also  defines a further symbol with

the  structure's overall length when  the definition has been finished.

This  symbol has the name LEN, which  is being extended with the struc-

ture's  name via the same rules -  or alternitavely with the label name

given with the ENDSTRUCT statement.

In practice, this may things may look like in this example:

Rec     STRUCT

Ident   db      ?

Pad     db      ?

Pointer dd      ?

Rec     ENDSTRUCT

In this example, the symbol REC_LEN would be assigned the value 6.

3.5.2.  Usage

- - - - - - -

Once  a structure  has been  assigned, usage  is as  simple as possible

and similar to a macro: a simple

thisrec Rec

reserves  as much memory  as needed to  hold an instance  of the struc-

ture,  and  additionally  defines  a  symbol  for  every element of the

structure  with its  address, in  this case THISREC_IDENT, THISREC_PAD,

and  THISREC_POINTER. A label naturally must  not be omitted when call-

ing a structure; if it is missing, an error will be emitted.

Additional  arguments  allow  to  reserve  memory  for a whole array of

structures.  The dimensions (up to three)  are defined via arguments in

square brackets:

thisarray Rec [10],[2]

In  this example,  space for  2*10=20 structures  is reserved. For each

individual  structure in the  array, proper symbols  are generated that

have the array indices in their name.

3.5.3.  Nested Structures

- - - - - - - - - - - - -

Is  is perfectly valid to call  an already defined structure within the

definition  of another  structure. The  procedure that  is taking place

then  is a combination  of the definition  and calling described in the

previous  two  sections:  elements  of  the  substructure are being de-

fined,  the name of  the instance is  being prepended, and  the name of

the  super-structure is once again  geing prepended to this concatenat-

ed name. This may look like the following:

TreeRec struct

left    dd         ?

right   dd         ?

data    Rec

TreeRec endstruct

It  is also  allowed to  define one  structure inside of another struc-

ture:

TreeRec struct

left    dd         ?

right   dd         ?

TreeData struct

name      db         32 dup(?)

id        dw         ?

TreeData endstruct

TreeRec endstruct

3.5.4. Unions

- - - - - - -

A  union is a special  form of a structure  whose elements are not laid

out  sequentially in memory. Instead all elements occupy the same memo-

ry  and are located at offset 0 in the structure. Naturally, such a de-

finition  basically does nothing more than  to assign the value of zero

to  a couple of symbols. It may  however be useful to clarify the over-

lap  in a program  and therefore to  make it more  'readable'. The size

of a union is the maximum of all elements' lengths.

3.5.5.  Nameless Structures

- - - - - - - - - - - - - -

The  name of a structure or union is  optional if it is part of another

(named)  structure or union.  Elements of this  structure will then be-

come part of of the 'next higher' named structure. For example,

TreeRec struct

left    dd         ?

right   dd         ?

struct

name      db         32 dup(?)

id        dw         ?

endstruct

TreeRec endstruct

generates the symbols TREEREC_NAME and TREEREC_ID.

Futhermore,  no symbol holding  its length is  generated for an unnamed

structure or union.

3.5.6.  Structures and Sections

- - - - - - - - - - - - - - - -

Symbols  that are created in the course  of defining or usage of struc-

tures  are treated  just like  normal symbols,  i.e. when used within a

section,  these symbols are  local to the  section. The same is however

also  true  for  the  structures  themselves,  i.e. a structure defined

within a section cannot be used outside of the section.

3.5.7.  Structures and Macros

- - - - - - - - - - - - - - -

If  one wants to instantiate structures via  macros, one has to use the

GLOBALSYMBOLS  options when defining the macro to make the defined sym-

bols  visible outside  the macro.  For instance,  a list  of structures

can be defined in the following way:

irp     name,{GLOBALSYMBOLS},rec1,rec2,rec3

name    Rec

endm

3.6. Conditional Assembly

-------------------------

valid for: all processors

The  assembler supports  conditional assembly  with the  help of state-

ments  like IF... resp.  SWITCH... . These  statements work at assembly

time  allowing or  disallowing the  assembly of  program parts based on

conditions.  They are therefore  not to be  compared with IF statements

of  high-level languages (though it would  be tempting to extend assem-

bly  language  with  structurization  statements  of  higher level lan-

guages...).

The  following  constructs  may  be  nested arbitrarily (until a memory

overflow occurs).

3.6.1.  IF / ELSEIF / ENDIF

- - - - - - - - - - - - - -

IF  is the most common and  most versatile construct. The general style

of an IF statement is as follows:

IF      <expression 1>

.

.

<block 1>

.

.

ELSEIF  <expression 2>

.

.

<block 2>

.

.

(possibly more ELSEIFs)

.

.

ELSEIF

.

.

<block n>

.

.

ENDIF

IF  serves as an  entry, evaluates the  first expression, and assembles

block  1 if the  expression is true  (i.e. not 0).  All further ELSEIF-

blocks  will  then  be  skipped.  However,  if the expression is false,

block  1 will be skipped and expression 2 is evaluated. If this expres-

sion  turns out to be true, block  2 is assembled. The number of ELSEIF

parts  is variable  and results  in an  IF-THEN-ELSE ladder of an arbi-

trary  length. The  block assigned  to the  last ELSEIF  (without argu-

ment)  only  gets  assembled  if  all previous expressions evaluated to

false;  it therefore forms a 'default'  branch. It is important to note

that  only one of the blocks will be assembled: the first one whose IF/

ELSEIF had a true expression as argument.

The  ELSEIF parts are optional, i.e. IF  may directly be followed by an

ENDIF. An ELSEIF without parameters must be the last branch.

ELSEIF  always  refers  to  the  innermost,  unfinished IF construct in

case IF's are nested.

In  addition to  IF, the  following further  conditional statements are

defined:

- IFDEF  <expressionl>: true if  the expression does  not contain any

symbols  that  are  undefined  by  the  source assembled up to this

point.   CAUTION!  Querying  forward  declarations  will  return  a

'false', also in the second and subsequent passes.

- IFNDEF <expression>: counterpart to IFDEF.

- IFSYMEXIST  <symbol> :  true if  the queried  symbol exists  in the

symbol  table.  CAUTION!  Querying  a  forward  declaration returns

false in the first pass, but true in all subsequent passes.

- IFNSYMEXIST <symbol>: counterpart to IFSYMEXIST.

- IFUSED  <symbol>: true if  if the given  symbol has been referenced

at least once up to now.

- IFNUSED <symbol>: counterpart to IFUSED.

- IFEXIST  <name>: true if the given  file exists. The same rules for

search  paths and syntax apply as  for the INCLUDE instruction (see

section 3.9.2).

- IFNEXIST <name>: counterpart to IFEXIST.

- IFB  <arg-list>: true  if all  arguments of  the parameter list are

empty strings.

- IFNB <arg-list>: counterpart to IFB.

It  is valid to write  ELSE instead of ELSEIF  since everybody seems to

be used to it...

For  everyIF...  statement,  there  has  to  be  a corresponding ENDIF.

'Open'  constructs will lead to  an error message at  the end of an as-

sembly  path. The way AS has 'paired'  ENDIF statements with IFs may be

deduced  from the assembly  listing: for ENDIF,  the line number of the

corresponding IF... will be shown.

3.6.2. SWITCH / CASE / ELSECASE / ENDCASE

- - - - - - - - - - - - - - - - - - - - -

CASE  is a special  case of IF  and is designed  for situations when an

expression  has to be compared  with a couple of  values. This could of

course also be done with a series of ELSEIFs, but the following form

SWITCH  <expression>

.

.

CASE    <value 1>

.

<block 1>

.

CASE    <value 2>

.

<block 2>

.

(further CASE blocks)

.

CASE    <value n-1>

.

<block n-1>

.

ELSECASE

.

<block n>

.

ENDCASE

has  the advantage  that the  expression is  only written once and also

only  gets evaluated once. It is therefore less error-prone and slight-

ly faster than an IF chain, but obviously not as flexible.

It  is possible  to specify  multiple values  separated by  commas to a

CASE  statement in  order to  assemble the  following block in multiple

cases.  The ELSECASE branch again serves as  a 'trap' for the case that

none  of the CASE conditions  was met. AS will  issue a warning in case

it is missing and all comparisons fail.

Even  when value lists of CASE branches overlap, only one branch is ex-

ecuted, which is the first one in case of ambiguities.

SWITCH  only serves  to open  the whole  construct; an arbitrary number

of  statements  may  be  between  SWITCH  and the first CASE (but don't

leave  other  IFs  open!),  for  the  sake  of  better readability this

should however not be done.

In  case that SWITCH  is already a  machine instruction on the selected

processor  target, the  construct may  be openend  via SELECT,  or by a

leading  period to  explicitly invoke  the pseudo  instruction (.SWITCH

instead of SWITCH).

Similarly  to  IF  constructs,  there  must  be exactly one ENDCASE for

every  SWITCH. Analogous to  ENDIF, for ENDCASE  the line number of the

corresponding SWITCH is shown in the listing.

3.7. Listing Control

--------------------

valid for: all processors

3.7.1. PAGE, PAGESIZE

- - - - - - - - - - -

PAGE  is used to  tell AS the  dimensions of the  paper that is used to

print  the assembly listing. The first  parameter is thereby the number

of  lines after  which AS  shall automatically  output a form feed. One

should  however  take  into  account  that  this value does not include

heading  lines  including  an  eventual  line specified with TITLE. The

minimum  number of lines is 5, and the maximum value is 255. A specifi-

cation  of 0 has the  result that AS will  not do any form feeds except

those  triggered by a  NEWPAGE instruction or  those implicitly engaged

at the end of the assembly listing (e.g. prior to the symbol table).

The  specification of the listing's length  in characters is an option-

al  second parameter and serves two purposes:  on the one hand, the in-

ternal  line  counter  of  AS  will  continue  to  run correctly when a

source  line has  to be  split into  several listing  lines, and on the

other  hand there are  printers (like some  laser printers) that do not

automatically  wrap into a new line at line end but instead simply dis-

card  the rest. For  this reason, AS  does line breaks  by itself, i.e.

lines  that are too long are split  into chunks whose lengths are equal

to  or smaller than the  specified width. This may  lead to double line

feeds  on printers that  can do line  wraps on their  own if one speci-

fies  the exact line  width as listing  width. The solution  for such a

case  is to  reduce the  assembly listing's  width by  1. The specified

line  width may  lie between  5 and  255 characters;  a line width of 0

means  similarly to the page length that  AS shall not do any splitting

of  listing lines; lines  that are too  long of course  cannot be taken

into account of the form feed then any more.

The  default setting for the  page length is 60  lines, the default for

the  line width  is 0;  the latter  value is  also assumed when PAGE is

called with only one parameter.

In  case PAGE is already a  machine instruction on the selected proces-

sor  target, use instead PAGESIZE  to define the paper  size. As an al-

ternative,  it is always  possible to explicitly  invoke the pseudo in-

struction by prepending a period (.PAGE instead of PAGE).

CAUTION!  There is no way  for AS to check  whether the specified list-

ing length and width correspond to the reality!

3.7.2.  NEWPAGE

- - - - - - - -

NEWPAGE  can be used  to force a  line feed though  the current line is

not  full up to now. This might  be useful to separate program parts in

the  listing that  are logically  different. The  internal line counter

is  reset and the page counter is  incremented by one. The optional pa-

rameter  is in conjunction  with a hierarchical  page numbering AS sup-

ports  up  to  a  chapter  depth  of  4.  0 always refers to the lowest

depth,  and the  maximum value  may vary  during the assembly run. This

may look a bit puzzling, as the following example shows:

page 1,     instruction NEWPAGE 0 -> page 2

page 2,     instruction NEWPAGE 1 -> page 2.1

page 2.1,   instruction NEWPAGE 1 -> page 3.1

page 3.1,   instruction NEWPAGE 0 -> page 3.2

page 3.2,   instruction NEWPAGE 2 -> page 4.1.1

NEWPAGE  <number> may therefore result  in changes in different digits,

depending  on the current chapter depth.  An automatic form feed due to

a  line counter  overflow or  a NEWPAGE  without parameter  is equal to

NEWPAGE  0. Previous  to the  output of  the symbol  table, an implicit

NEWPAGE <maximum up to now> is done to start a new 'main chapter'.

3.7.3.  MACEXP_DFT and MACEXP_OVR

- - - - - - - - - - - - - - - - -

Once  a macro  is tested  and 'done',  one might  not want to see it in

the  listing when it is used. As  described in the section about defin-

ing  and using macros (3.4.1), additional arguments to the MACRO state-

ment  allow to control whether a macro's  body is expanded upon its us-

age  and if yes, which parts of it. In case that several macros are de-

fined  in  a  row,  it  is  not  necessary to give these directives for

every  single macro. The pseudo  instruction MACEXP_DFT defines for all

following  macros which parts shall be  expanded upon invocation of the

macro:

- ON resp. OFF enable or disable expansion completely.

- The  arguments IF  resp. NOIF  enable or  disable expansion  of in-

structions  for conditional  assembly, plus  the expansion  of code

parts the were excluded because of conditional assembly.

- Macro  definitions (which includes  REPT, WHILE and  IRP(C)) may be

excluded  from or included in the  expanded parts via the arguments

MACRO resp. NOMACRO.

- All  other lines not  fitting into the  first two categories may be

excluded  from or included in the  expanded parts via the arguments

REST resp. NOREST.

The  default is  ON, i.e.  defined macros  will be expanded completely,

of  course unless specific expansion  arguments were given to individu-

al  macros. Furthermore,  arguments given  to MACEXP_DFT  work relative

to  the current setting:  for instance, if  expansion is turned on com-

pletely initially, the statement

MACEXP_DFT  noif,nomacro

has  the result that for macros  defined in succession, only code parts

that  are no macro definition and  that are not excluded via condition-

al assembly will be listed.

This  instruction  plus  the  per-macro directives provide fine-grained

per-macro  over the parts  being expanded. However,  there may be cases

in  practice where  one wants  to see  the expanded  code of a macro at

one  place and not at  the other. This is  possible by using the state-

ment  MACEXP_OVR: it accepts the  same arguemnts like MACEXP_DFT, these

however  act as overrides for all  macros being expanded in the follow-

ing  code. This  is in  contrast to  MACEXP_DFT which influences macros

being  defined in the following code.  For instance, if one defined for

a  macro that neither macro  definitions nor conditional assembly shall

be expanded in the listing, a

MACEXP_OVR  MACRO

re-enables  expansion of  macro definitions  for its  following usages,

while a

MACEXP_OVR  ON

forces  expansion  of  the  complete  macro  body  in the listing. MAC-

EXP_OVR  without arguments  again disables  all overrides,  macros will

again behave as individually specified upon definition.

Both  statements  also  have  an  effect on other macro-like constructs

(REPT,  IRP, IRPC  WHILE). However,  since these  are expanded only one

and  ,,in-place'', the  functional difference  of these  two statements

becomes  minimal. In  case of  differences, the  override set  via MAC-

EXP_OVR has a higher priority.

The  Setting currently made via MACEXP_DFT  may be read from the prede-

fined  symbol MACEXP. For backward  compatibility reasons, it is possi-

ble  to use  the statement  MACEXP instead  of MACEXP_DFT. However, one

should not make use of this in new programs.

3.7.4.  LISTING

- - - - - - - -

works  like MACEXP  and accepts  the same  parameters, but is much more

radical: After a

listing off   ,

nothing  at all  will be  written to  the listing. This directive makes

sense  for tested code parts or include files to avoid a paper consump-

tion  going beyond  all bounds.  CAUTION! If  one forgets  to issue the

counterpart  somewhere later, even  the symbol table  will not be writ-

ten  any  more!  In  addition  to  ON  and  OFF,  LISTING  also accepts

NOSKIPPED  and PURECODE as  arguments. Program parts  that were not as-

sembled  due to conditional  assembly will not  be written to the list-

ing  when NOSKIPPED is  set, while PURECODE  - as the  name indicates -

even  suppresses the IF directives themselves in the listing. These op-

tions  are useful if one uses  macros that act differently depending on

parameters and one only wants to see the used parts in the listing.

The  current setting may be read  from the symbol LISTING (0=OFF, 1=ON,

2=NOSKIPPED, 3=PURECODE).

3.7.5.  PRTINIT and PRTEXIT

- - - - - - - - - - - - - -

Quite  often it  makes sense  to switch  to another printing mode (like

compressed  printing) when the listing is sent  to a printer and to de-

activate  this mode again at the end  of the listing. The output of the

needed  control sequences can  be automated with  these instructions if

one  specifies the  sequence that  shall be  sent to  the output device

prior  to the  listing with  PRTINIT <string>  and similarly the deini-

tialization  string with PRTEXIT <string>. <string>  has to be a string

expression  in both cases. The syntax  rules for string constants allow

to  insert control characters  into the string  without too much tweak-

ing.

When  writing the listing,  the assembler does  not differentiate where

the  listing actually  goes, i.e.  printer control  characters are sent

to the screen without mercy!

Example:

For  Epson printers, it makes sense to switch them to compressed print-

ing because listings are so wide. The lines

prtinit "\15"

prtexit "\18"

assure  that the compressed mode  is turned on at  the beginning of the

listing and turned off afterwards.

3.7.6.  TITLE

- - - - - - -

The  assembler normally adds a header line  to each page of the listing

that  contains the source  file's name, date,  and time. This statement

allows  to extend the page header  by an arbitrary additional line. The

string that has to be specified is an arbitrary string expression.

Example:

For  the Epson printer  already mentioned above,  a title line shall be

written  in wide mode,  which makes it  necessary to turn  off the com-

pressed mode before:

title   "\18\14Wide Title\15"

(Epson  printers automatically turn off  the wide mode at  the end of a

line.)

3.7.7.  RADIX

- - - - - - -

RADIX  with a numerical argument between 2 and 36 sets the default num-

bering  system for  integer constants,  i.e. the  numbering system used

if  nothing else  has been  stated explicitly.  The default  is 10, and

there  are some possible  pitfalls to keep  in mind which are described

in section 2.10.1.

Independent  of the  current setting,  the argument  of RADIX is always

decimal;  furthermore, no symbolic  or formula expressions  may be used

as argument. Only use simple constant numbers!

A  RADIX statement overrides  a setting given  by a -radix command line

switch.

If  the IM61x0 is the current  target, the instructions DECIMAL and OC-

TAL are available as shortforms for RADIX 10 respectively RADIX 8.

3.7.8. OUTRADIX

- - - - - - - -

OUTRADIX  can in a  certain way be  regarded as the  opposite to RADIX:

This  statement allows to  configure which numbering  system to use for

integer  results when  \{...} constructs  are used  in string constants

(see  section 2.10.3). Valid arguments range  again from 2 to 36, while

the default is 16.

3.8. Local Symbols

------------------

valid for: all processors

local  symbols and the section concept  introduced with them are a com-

pletely  new function that was introduced  with version 1.39. One could

say  that this part  is version ''1.0''  and therefore probably not the

optimum.  Ideas and  (constructive) criticism  are therefore especially

wanted.  I admittedly  described the  usage of  sections how I imagined

it.  It is  therefore possible  that the  reality is not entirely equal

to  the model  in my  head. I  promise that  in case  of discrepancies,

changes  will occur that the reality  gets adapted to the documentation

and  not vice versa (I  was told that the  latter sometimes takes place

in larger companies...).

AS  does not generate linkable code  (and this will probably not change

in  the near  future :-().  This fact  forces one  to always assemble a

program  in a whole.  In contrast to  this technique, a separation into

linkable modules would have several advantages:

- shorter  assembly times as only the modified modules have to be re-

assembled;

- the  option to set  up defined interfaces  among modules by defini-

tion of private and public symbols;

- the  smaller length  of the  individual modules  reduces the number

of  symbols per module  and therefore allows  to use shorter symbol

names that are still unique.

Especially  the last  item was  something that  always nagged  me: once

there  was a label's name defined at the beginning of a 2000-lines pro-

gram,  there was no  way to reuse  it somehow -  even not at the file's

other  end  where  routines  with  a  completely different context were

placed. I was forced to use concatenated names in the style of

<subprogram name>_<symbol name>

that  had lengths ranging  from 15 to  25 characters and  made the pro-

gram  difficult to  overlook. The  concept of  section described in de-

tail  in the following  text was designed  to cure at  least the second

and  third item of  the list above.  It is completely  optional: if you

do  not want to use  sections, simply forget them  and continue to work

like you did with previous versions of AS.

3.8.1.  Basic Definition (SECTION/ENDSECTION)

- - - - - - - - - - - - - - - - - - - - - - -

A  section represents a part of  the assembler program enclosed by spe-

cial statements and has a unique name chosen by the programmer:

.

.

<other code>

.

.

SECTION <section's name>

.

.

<code inside of the section>

.

.

ENDSECTION [section's name]

.

.

<other code>

.

.

The  name of  a section  must conform  to the  conventions for s symbol

name;  AS stores section  and symbol names  in separate tables which is

the  reason why a  name may be  used for a  symbol and a section at the

same  time. Section  names must  be unique  in a  sense that there must

not  be more than one  section on the same  level with the same name (I

will  explain in the  next part what  ''levels'' mean). The argument of

ENDSECTION  is optional, it may  also be omitted; if  it is omitted, AS

will  show the  section's name  that has  been closed with this ENDSEC-

TION.  Code inside a section  will be processed by  AS exactly as if it

were outside, except for three decisive differences:

- Symbols  defined within  a section  additionally get  an internally

generated  number that  corresponds to  the section.  These symbols

are  not  accessible  by  code  outside  the  section  (this can be

changed by pseudo instructions, later more about this).

- The  additional  attribute  allows  to  define  symbols of the same

name  inside and outside the section; the attribute makes it possi-

ble  to use a symbol name multiple times without getting error mes-

sages from AS.

- If  a symbol of a certain name  has been defined inside and outside

of  a section, the ''local'' one  will be preferred inside the sec-

tion,  i.e. AS first searches the symbol  table for a symbol of the

referenced  name that  also was  assigned to  the section. A search

for  a global  symbol of  this name  only takes  place if the first

search fails.

This  mechanism  e.g.  allows  to  split  the  code into modules as one

might  have done  it with  linkable code.  A more fine-grained approach

would  be to pack  every routine into  a separate section. Depending on

the  individual routines' lengths, the symbols for internal use may ob-

tain very short names.

AS  will by default  not differentiate between  upper and lower case in

section  names;  if  one  however  switches to case-sensitive mode, the

case will be regarded just like for symbols.

The  organization described  up to  now roughly  corresponds to what is

possible  in the C language that places  all functions on the same lev-

el.  However, as my ''high-level''  ideal was Pascal and  not C, I went

one step further:

3.8.2.  Nesting and Scope Rules

- - - - - - - - - - - - - - - -

It  is valid to define further sections  within a section. This is ana-

log  to the option given in Pascal to define procedures inside a proce-

dure or function. The following example shows this:

sym     EQU        0

SECTION    ModuleA

SECTION    ProcA1

sym       EQU        5

ENDSECTION ProcA1

SECTION    ProcA2

sym       EQU        10

ENDSECTION ProcA2

ENDSECTION ModuleA

SECTION    ModuleB

sym      EQU        15

SECTION    ProcB

ENDSECTION ProcB

ENDSECTION ModuleB

When  looking up a symbol,  AS first searches for  a symbol assigned to

the  current section, and afterwards traverses  the list of parent sec-

tions  until the global symbols are  reached. In our example, the indi-

vidual sections see the values given in table 3.1 for the symbol sym:

+---------+-------+-----------------+

| section | value | from section... |

+---------+-------+-----------------+

+---------+-------+-----------------+

| Global  | 0     | Global          |

+---------+-------+-----------------+

| ModuleA | 0     | Global          |

+---------+-------+-----------------+

| ProcA1  | 5     | ProcA1          |

+---------+-------+-----------------+

| ProcA2  | 10    | ProcA2          |

+---------+-------+-----------------+

| ModuleB | 15    | ModuleB         |

+---------+-------+-----------------+

| ProcB   | 15    | ModuleB         |

+---------+-------+-----------------+

Table 3.1: Valid values for the Individual Sections

This  rule can be  overridden by explicitly  appending a section's name

to  the symbol's name. The section's name  has to be enclosed in brack-

ets:

move.l  #sym[ModulB],d0

Only  sections that are in the parent  section path of the current sec-

tion  may be used.  The special values  PARENT0..PARENT9 are allowed to

reference  the  n-th  ''parent''  of  the  current  section; PARENT0 is

therefore  equivalent to  the current  section itself,  PARENT1 the di-

rect  parent and  so on.  PARENT1 may  be abbreviated  as PARENT. If no

name is given between the brackets, like in this example:

move.l  #sym[],d0 ,

one  reaches the global  symbol. CAUTION! If  one explicitly references

a  symbol from a  certain section, AS  will only seek  for symbols from

this  section, i.e. the traversal of  the parent sections path is omit-

ted!

Similar  to Pascal, it is allowed  that different sections have subsec-

tions  of the same name; the  principle of locality avoids irritations.

One  should IMHO still use this  feature as seldom as possible: Symbols

listed  in the symbol  resp. cross reference  list are only marked with

the  section they are  assigned to, not  with the ''section hierarchy''

lying  above them (this really would  have busted the available space);

a differentiation is made very difficult this way.

As  a SECTION instruction does  not define a label  by itself, the sec-

tion  concept has an important difference  to Pascal's concept of nest-

ed  procedures: a pascal  procedure can automatically  ''see'' its sub-

procedures(functions),  AS requires an explicit  definition of an entry

point. This can be done e.g. with the following macro pair:

proc    MACRO   name

SECTION name

name    LABEL   $

ENDM

endp    MACRO   name

ENDSECTION name

ENDM

This  example also shows  that the locality  of labels inside macros is

not  influenced by sections. It makes the trick with the LABEL instruc-

tion necessary.

This  does of  course not  solve the  problem completely.  The label is

still  local and  not referencable  from the  outside. Those  who think

that  it  would  suffice  to  place  the  label in front of the SECTION

statement  should be quiet  because they would  spoil the bridge to the

next theme:

3.8.3.  PUBLIC and GLOBAL

- - - - - - - - - - - - -

The  PUBLIC statement allows to change the  assignment of a symbol to a

certain  section. It  is possible  to treat  multiple symbols  with one

statement,  but I will use an example  with only one symbol in the fol-

lowing  (not hurting  the generality  of this  discussion). In the sim-

plest  case, one declares a symbol to  be global, i.e. it can be refer-

enced from anywhere in the program:

PUBLIC  <name>

As  a symbol cannot be moved in the symbol table once it has been sort-

ed  in, this statement  has to appear  before the symbol  itself is de-

fined.  AS stores all PUBLICs in a  list and removes an entry from this

list  when the  corresponding symbol  is defined.  AS prints  errors at

the end of a section in case that not all PUBLICs have been resolved.

Regarding  the hierarchical section  concept, the method  of defining a

symbol  as purely global looks extremely  brute. There is fortunately a

way  to do this in  a bit more differentiated  way: by appending a sec-

tion name:

PUBLIC  <name>:<section>

The  symbol will  be assigned  to the  referenced section and therefore

also  becomes accessible for all its  subsections (except they define a

symbol  of the  same name  that hides  the ''more  global'' symbol). AS

will  naturally protest if  several subsections try  to export a symbol

of  same name to  the same level.  The special PARENTn values mentioned

in  the previous section are also valid  for <section> to export a sym-

bol  exactly n levels up in  the section hierarchy. Otherwise only sec-

tions  that are  parent sections  of the  current section are valid for

<section>.  Sections that are  in another part  of the section tree are

not  allowed. If  several sections  in the  parent section  path should

have  the same name (this  is possible), the lowest  level will be tak-

en.

This tool lets the abovementioned macro become useful:

proc    MACRO   name

SECTION name

PUBLIC  name:PARENT

name    LABEL   $

ENDM

This  setting is equal  to the Pascal  model that also  only allows the

''father'' to see its children, but not the ''grandpa''.

AS  will quarrel about double-defined symbols  if more than one section

attempts  to export a symbol  of a certain name  to the same upper sec-

tion.  This  is  by  itself  a  correct  reaction,  and  one  needs  to

''qualify''  symbols somehow to make  them distinguishable if these ex-

ports  were deliberate. A  GLOBAL statement does  just this. The syntax

of  GLOBAL is identical  to PUBLIC, but  the symbol stays local instead

of  being assigned to  a higher section.  Instead, an additional symbol

of  the same value but with the  subsection's name appended to the sym-

bol's  name is created,  and only this  symbol is made public according

to  the  section  specification.  If  for  example two sections A and B

both  define a symbol named  SYM and export it  with a GLOBAL statement

to  their parent  section, the  symbols are  sorted in  under the names

A_SYM resp. B_SYM .

In  case that source and target section  are separated by more than one

level, the complete name path is prepended to the symbol name.

3.8.4.  FORWARD

- - - - - - - -

The  model described so far  may look beautiful, but  there is an addi-

tional  detail not present in Pascal  that may spoil the happiness: As-

sembler  allows forward references. Forward references may lead to sit-

uations  where AS accesses a symbol from  a higher section in the first

pass.  This is not a  disaster by itself as  long as the correct symbol

is  used in the  second pass, but  accidents of the  following type may

happen:

loop:   .

<code>

.

.

SECTION sub

.               ; ***

.

bra.s   loop

.

.

loop:   .

.

ENDSECTION

.

.

jmp     loop    ; main loop

AS  will take the global label loop  in the first pass and will quarrel

about  an  out-of-branch  situation  if  the  program part at <code> is

long  enough. The second  pass will not  be started at  all. One way to

avoid  the ambiguity would  be to explicitly  specify the symbol's sec-

tion:

bra.s   loop[sub]

If  a local  symbol is  referenced several  times, the  brackets can be

saved  by using a  FORWARD statement. The  symbol is thereby explicitly

announced  to be local, and  AS will only look  in the local symbol ta-

ble  part when this  symbol is referenced.  For our example, the state-

ment

FORWARD loop

should be placed at the position marked with ***.

FORWARD  must not  only be  stated prior  to a symbol's definition, but

also  prior to its first usage in a  section to make sense. It does not

make  sense to define a symbol private and public; this will be regard-

ed as an error by AS.

3.8.5.  Performance Aspects

- - - - - - - - - - - - - -

The  multi-stage lookup in  the symbol table  and the decision to which

section  a symbol  shall be  assigned of  course cost  a bit of time to

compute.  An 8086  program of  1800 lines  length for example took 34.5

instead  of 33 seconds after a  modification to use sections (80386 SX,

16MHz,  3 passes).  The overhead  is therefore  limited. As  it has al-

ready  been stated  at the  beginning, is  is up  to the  programmer if

(s)he wants to accept it. One can still use AS without sections.

3.9. Miscellaneous

------------------

3.9.1. SHARED

- - - - - - -

valid for: all processors

This  statement instructs AS to write  the symbols given in the parame-

ter  list (regardless if they are integer, float or string symbols) to-

gether  with their values into the share file. It depends upon the com-

mand  line parameters described  in section 2.4  whether such a file is

generated  at all  and in  which format  it is  written. If  AS detects

this  instruction and no share file is  generated, a warning is the re-

sult.

CAUTION!  A comment possibly  appended to the  statement itself will be

copied  to the first line outputted to  the share file (if SHARED's ar-

gument  list is  empty, only  the comment  will be  written). In case a

share  file is written  in C or  Pascal format, one  has to assure that

the  comment  itself  does  not  contain character sequences that close

the comment (''*/'' resp. ''*)''). AS does not check for this!

3.9.2.  INCLUDE

- - - - - - - -

valid for: all processors

This  instruction inserts the  file given as  a parameter into the just

as  if it would  have been inserted  with an editor  (the file name may

optionally  be enclosed with  '' characters). This  instruction is use-

ful  to split source files  that would otherwise not  fit into the edi-

tor or to create ''tool boxes''.

In  case that the  file name does  not have an  extension, an extension

of  INC is assumed in a firstg step.  Only if no sch file exists, or if

the  specified name contains  a period (and  therefore an extension), a

file of exactly this name is searched for.

The  assmebler primarily tries  to open the  file in the directory con-

taining  the source file with the  INCLUDE statenemt. This means that a

path  contained in  the file  specification is  relative to this file's

directory,  not to  the directory  the assembler  was called  from. Via

the  -i <path list> option, one can  specify a list of directories that

will  automatically  be  searched  for  the  file.  If  the file is not

found, a fatal error occurs, i.e. assembly terminates immediately.

For  compatibility reasons, it is valid to  enclose the file name in ''

characters, i.e.

include stddef51

and

include "stddef51.inc"

are  equivalent. CAUTION! This freedom of  choice is the reason why on-

ly a string constant but no string expression is allowed!

3.9.3. BINCLUDE

- - - - - - - -

valid for: all processors

BINCLUDE  can be used to embed  binary data generated by other programs

into  the code generated  by AS (this  might theoretically even be code

created by AS itself...). BINCLUDE has three forms:

BINCLUDE <file>

This way, the file is completely included.

BINCLUDE <file>,<offset>

This  way, the file's contents are  included starting at <offset> up to

the file's end.

BINCLUDE <file>,<offset>,<length>

This way, <length> bytes are included starting at <offset>.

The  same rules  regarding search  paths and  assumed suffixes apply as

for INCLUDE.

3.9.4. MESSAGE, WARNING, ERROR, and FATAL

- - - - - - - - - - - - - - - - - - - - -

valid for: all processors

Though  the assembler checks source files as strict as possible and de-

livers  differentiated error messages, it  might be necessary from time

to  time to  issue additional  error messages  that allow  an automatic

check  for logical error. The  assembler distinguishes among three dif-

ferent  types of error  messages that are  accessible to the programmer

via the following three instructions:

- WARNING:  Errors that hint  at possibly wrong  or inefficient code.

Assembly continues and a code file is generated.

- ERROR:  True errors in  a program. Assembly  continues to allow de-

tection  of possible further  errors in the  same pass. A code file

is not generated.

- FATAL:  Serious errors that  force an immediate  termination of as-

sembly. A code file may be generated but will be incomplete.

All  three  instructions  have  the  same  format  for the message that

shall  be issued: an  arbitrary string expression,  which may be a sim-

ple  string constant, but  as well a  complex expression that evaluates

to  a string. It  also includes the  feature to embed  symbol values in

strings, which is described in 2.7:

message "Start Address is \{start_address}"

Instructions  generating warnings  or errors  typically only make sense

in  conjunction wit conditional assembly. For  example, if there is on-

ly  a limited address  space for a  program, one can  test for overflow

in the following way:

ROMSize equ     8000h   ; 27256 EPROM

ProgStart:

.

.

<the program itself>

.

.

ProgEnd:

if      ProgEnd-ProgStart>ROMSize

error  "\athe program is too long!"

endif

Apart  from the  instructions generating  errors, there  is also an in-

struction  MESSAGE that simply  prints a message  to the assembly list-

ing  and th ecosole  (the latter only  if the quiet  mode is not used).

Its usage is equal to the other three instructions.

3.9.5. READ

- - - - - -

valid for: all processors

One  could say  that READ  is the  counterpart to the previous instruc-

tion  group: it allows  to read values  from the keyboard during assem-

bly.  You might ask what this is good for. I will break with the previ-

ous  principles and put an example before the exact description to out-

line the usefulness of this instruction:

A  program needs for data  transfers a buffer of  a size that should be

set  at assembly time.  One could store  this size in  a symbol defined

with EQU, but it can also be done interactively with READ:

IF      MomPass=1

READ    "buffer size",BufferSize

ENDIF

Programs  can this  way configure  themselves dynamically during assem-

bly  and one could hand over the  source to someone who can assemble it

without  having to dive into the  source code. The IF conditional shown

in  the example should always be used  to avoid bothering the user mul-

tiple times with questions.

READ  is quite  similar to  SET with  the difference  that the value is

read  from the  keyboard instead  of the  instruction's arguments. This

for  example also implies  that AS will  automatically set the symbol's

type  (integer, float or string)  or that it is  valid to enter formula

expressions instead of a simple constant.

READ  may either have one or  two parameters because the prompting mes-

sage  is optional.  AS will  print a  message constructed from the sym-

bol's name if it is omitted.

3.9.6.  INTSYNTAX

- - - - - - - - -

valid for: all processors

This  instruction allows  to modify  the set  of notations  for integer

constants  in various number  systems. - After  selection of a CPU tar-

get,  a certain default set is installed  (see section E). This set may

be  augmented with  other notations,  or notations  may be removed from

it.  INTSYNTAX takes an arbitrary list  of arguments which either begin

with  a plus or  minus character, followed  by the notation's identifi-

er. For instance, the following statement

INTSYNTAX    -0oct,+0hex

has  the result that a  leading zero marks a  hexadecimal instead of an

octal  constant, a common  usage on some  assemblers for the SC/MP. The

identifiers  for all notations can  be found in table  2.7. There is no

limit  on combining notations, except  when they contradict each other.

For  instance, it would not  be allowed to enable  0oct and 0hex at the

same time.

3.9.7.  RELAXED

- - - - - - - -

valid for: all processors

By  default, AS  assigns a  distinct syntax  for integer constants to a

processor  family  (which  is  in  general  equal to the manufacturer's

specifications,  as long as  the syntax is  not too bizarre...). Every-

one  however has  his own  preferences for  another syntax and may well

live  with the  fact that  his programs  cannot be  translated any more

with the standard assembler. If one places the instruction

RELAXED ON

right  at the  program's beginning,  one may  furtherly use  any syntax

for  integer constants, even mixed in a  program. AS tries to guess au-

tomatically  for every  expression the  syntax that  was used. This au-

tomatism  does not  always deliver  the result  one might have in mind,

and  this is also the reason why this option has to be enable explicit-

ly:  if there are no prefixes  or postfixes that unambiguously identify

either  Intel or Motorola syntax, the C  mode will be used. Leading ze-

roes that are superfluous in other modes have a meaning in this mode:

move.b  #08,d0

This  constant will be understood as  an octal constant and will result

in  an error message  as octal numbers  may only contain  digits from 0

to  7. One might  call this a  lucky case; a  number like 077 would re-

sult  in trouble without getting a  message about this. Without the re-

laxed  mode, both expressions unambiguously  would have been identified

as decimal constants.

The current setting may be read from a symbol with the same name.

3.9.8. COMPMODE

- - - - - - - -

valid for: various processors

Though  the assember strives to  behave like the correspondig "original

assemblers",  there are  cases when  emulating the original assembler's

behaviour  would forbid code  optimizations which are  valid and useful

in my opinion. Use the statement

compmode on

to  switch to a 'compatibility mode' which prioritizes 'original behav-

iour'  to most efficient code. See  the respective section with proces-

sor-specific  hints whether there  are any situations  for the specific

target.

Compatibility  mode is disabled by default,  unless it was activated by

the  command line switch of same name.  The current setting may be read

from a symbol with the same name.

3.9.9.  END

- - - - - -

valid for: all processors

END  marks the end of an  assembler program. Lines that eventually fol-

low  in the source file  will be ignored. IMPORTANT:  END may be called

from  within a macro, but the  IF-stack for conditional assembly is not

cleared  automatically.  The  following  construct therefore results in

an error:

IF      DontWantAnymore

END

ELSEIF

END  may optionally have  an integer expression  as argument that marks

the  program's entry point. AS stores this in the code file with a spe-

cial record and it may be post-processed e.g. with P2HEX.

END  has always been  a valid instruction  for AS, but  the only reason

for  this in earlier releases  of AS was compatibility;  END had no ef-

fect.

4. Processor-specific Hints

===========================

When  writing the individual  code generators, I  strived for a maximum

amount  of compatibility  to the  original assemblers.  However, I only

did  this as long as it did  not mean an unacceptable additional amount

of  work. I listed  important differences, details  and pitfalls in the

following chapter.

4.1. 6811

---------

''Where  can I buy such  a beast, a HC11  in NMOS?'', some of you might

ask.  Well, of course it does not  exist, but an H cannot be represent-

ed  in a hexadecimal  number (older versions  of AS would  not have ac-

cepted  such a name because of this), and  so I decided to omit all the

letters...

''Someone  stating that something is  impossible should be at least

as cooperative as not to hinder the one who currently does it.''

From  time to time, one  is forced to revise  one's opinions. Some ver-

sions  earlier, I stated at  his place that I  couldn't use AS's parser

in  a way  that it  is also  possible to  to separate  the arguments of

BSET/BCLR  resp.  BRSET/BRCLR  with  spaces.  However, it seems that it

can  do more than I wanted to  believe...after the n+1th request, I sat

down  once again  to work  on it  and things  seem to work now. You may

use  either spaces or commas, but not in all variants, to avoid ambigu-

ities:  for every variant of an instruction,  it is possible to use on-

ly  commas or a mixture of spaces  and commas as Motorola seems to have

defined  it (their  data books  do not  always have  the quality of the

corresponding hardware...):

Bxxx  abs8 #mask         is equal to Bxxx  abs8,#mask

Bxxx  disp8,X #mask      is equal to Bxxx  disp8,X,#mask

BRxxx abs8 #mask addr    is equal to BRxxx abs8,#mask,addr

BRxxx disp8,X #mask addr is equal to BRxxx disp8,X,#mask,addr

In  this list,  xxx is  a synonym  either for  SET or CLR; #mask is the

bit  mask to be applied  (the # sign is  optional). Of course, the same

statements are also valid for Y-indexed expression (not listed here).

With  the K4  version of  the HC11,  Motorola has  introduced a banking

scheme,  which one one hand  easily allows to once  again extend an ar-

chitecture  that has become 'too small', but  on the other hand not re-

ally  makes programmers' and tool  developers' lifes simpler...how does

one sensibly map something like this on a model for a programmer?

The  K4 architecture  extends the  HC11 address  space by 2x512 Kbytes,

which  means that  we now  have a  total address  space of 64+1024=1088

Kbytes.  AS acts  like this  were one  large unified addres space, with

the following layout:

- $000000...$00ffff: the old HC11 address space

- $010000...$08ffff: Window 1

- $090000...$10ffff: Window 2

Via  the ASSUME statement,  one tells AS  how the banking registers are

set  up, which  in turn  describes which  extended areas  are mapped to

which  physical addresses. For absolute  addresses modes with addresses

beyond  $10000, AS automatically computes  the address within the first

64K  that is to be used. Of  course this only works for direct address-

ing  modes, it is the programmer's  responsibility to keep the overview

for indirect or indexed addressing modes!

In  case one is  not really sure  if the current  mapping is really the

desired  one, the pseudo  instruction PRWINS may  be used, which prints

the  assumes MMxxx register contents  plus the current mapping(s), like

this:

MMSIZ $e1 MMWBR $84 MM1CR $00 MM2CR $80

Window 1: 10000...12000 --> 4000...6000

Window 1: 90000...94000 --> 8000...c000

An instruction

jmp     *+3

located  at  $10000  would  effectively  result  in  a  jump to address

$4003.

4.2. PowerPC

------------

Of  course, it is a bit  crazy idea to add support  in AS for a proces-

sor  that  was  mostly  designed  for  usage in work stations. Remember

that  AS mainly is  targeted at programmers  of single board computers.

But  things that  today represent  the absolute  high end  in computing

will  be average tomorrow and  maybe obsolete the next  day, and in the

meantime,  the Z80 as the  8088 have been retired  as CPUs for personal

computers  and  been  moved  to  the embedded market; modified versions

are  marketed as  microcontrollers. With  the appearance  of the MPC505

and  PPC403, my suspicion has  proven to be true  that IBM and Motorola

try to promote this architecture in as many fields as possible.

However,  the current support is a bit incomplete: Temporarily, the In-

tel-style  mnemonics are used to allow storage of data and the more un-

common  RS/6000  machine  instructions  mentioned  in  [81] are missing

(hopefully  noone misses them!). I will finish this as soon as informa-

tion about them is available!

4.3. IBM PALM

-------------

IBM's  PALM processor has  been ''Terra Incognita''  for long time, be-

cause  it  never  has  been  used  outside of IBM. Furthermore, the IBM

5100  to 5120  that were  equipped with  it were  exotic and expensive,

and  were quickly forgotten over the success of the IBM PC. Only Chris-

tian  Corti's extensive reverse engineering  made it possible to imple-

ment this target [43].

When  Christian began  to reverse  engineer the  PALM processor, he did

not  know the assembler mnemonics  defined by IBM, so  he had to choose

his  own  ones.  He  of  course  did this with the background knowledge

about  decades  of  other  processor  architectures that were developed

from 1973 (when PALM was constructed) until today.

If  you compare his mnemonics with the  ones from IBM (a document about

them  was finally published  in [40]), I  see parallels to the assembly

language  of the Intel 8080/8085 on the  one side, and the Zilog Z80 on

the  other side. ''Intel Mnemonics'' pack  the addressing mode into the

mnemonic's  name (like  MVI for  ''MoVe Immediate''  or LDHD for ''LoaD

Halfword  Direct''). This is  significantly easier to  parse and trans-

form into machine language for an assembler.

The  other mnemonics group all machine instructions doing a certain op-

eration  under the same  mnemonic, like LD  for ''LoaD'' or  MOVE for a

(16  bit) data  move. This  makes usage  for a programmer much simpler,

parsing  the different  addressing modes  however results  in some more

work for an assembler.

So,  both sets of mnemonics have their justification: The IBM ones sim-

ply  because they are ''the  original'' ones and are  use in all vendor

documentation,  and the  new ones  because they  are simply more under-

standable  and easier to use. I  therefore decided to support both sets

in  my assembler,  and this  was fortunately  possible without creating

any  conflicts.  Support  includes  the  ''macro  instructions''  CALL,

RCALL,  JMP, BRA, LWI and  RET. And there are  a few things I added my-

self:

-  AND  and OR also  accept an immediate  operand as second argument.

This  is mapped  to the  SET resp.  CLR instructions, and of course

the value gets inverted for AND.

-  MOVE  also accepts an  immediate argument as  source and generates

the same machine code as LWI. The same is true for MOVB and LBI.

Macro  instructions  consisting  of  more  than one (half) word however

create  a new problem: The only  form of conditional execution support-

ed  by the PALM  processor is a  conditional skip of  the following in-

struction  word. If such a skip is  followed by a macro instruction, it

would  only partially be skipped. I  have therefore added a small state

machine  that attempts to detect such  sequences and will issue a warn-

ing.

The  IBM 5110 and 5120 do not  use the ASCII character set, but instead

EBCDIC  as known from IBM mainframes.  The include subdirectory holds a

file  that  may  be  used  to  convert from ASCII to EBCDIC. IMPORTANT:

This  file defines  EBCDIC as  an extra  code page,  so the translation

has to be activated with the statement

codepage        cp037

One  more word about  integer constant syntax:  Christian Corti had de-

cided  to use the ''Motorola  Syntax'', i.e. hexadecimal constants must

be  prefixed with a dollar sign. As the PALM is an IBM design, I decid-

ed  to use the ''IBM Syntax'' by default, which means that numeric con-

stants  are enclosed in apostrophes and prefixed with an X for hexadec-

imal  values.  To  assemble  the  code  examples from Christian's pages

without  modifying them, add  the following statement  at the program's

beginning:

intsyntax       +$hex,-x'hex'

4.4. DSP56xxx

-------------

Motorola,  which devil rode  you! Which person  in your company had the

''brilliant''  idea  to  separate  the  parallel  data  transfers  with

spaces!  In result,  everyone who  wants to  make his  code a  bit more

readable, e.g. like this:

move    x:var9 ,r0

move    y:var10,r3   ,

is  p****ed because the space gets recognized as a separator for paral-

lel data transfers!

Well...Motorola  defined it  that way,  and I  cannot change  it. Using

tabs  instead of spaces to separate the parallel operations is also al-

lowed,  and the individual  operations' parts are  again separated with

commas, as one would expect it.

[76]  states that instead of using MOVEC, MOVEM, ANDI or ORI, it is al-

so  valid  to  use  the  more  general  Mnemonics  MODE,  AND or OR. AS

(currently) does not support this.

4.5. H8/300

-----------

Regarding  the assembler syntax of  these processors, Hitachi generous-

ly  copied from Motorola (that wasn't  by far the worst choice...), un-

fortunately  the  company  wanted  to  introduce  its  own  format  for

hexadecimal  numbers. To make it  even worse, it is  a format that uses

unbalanced single quotes, just like Microchip does:

mov.w #h'ff,r0

This  format is  not supported  by default.  Instead, one  has to write

hexadecimal  numbers in the well-known  Motorola syntax: with a leading

dollar  sign. If you  really need the  'Hitachi Syntax', e.g. to assem-

ble  existing code,  enable the  RELAXED mode.  Bear in  mind that this

syntax  has received few testing so  far. I can therefore not guarantee

that it will work in all cases!

4.6. H8/500

-----------

The  H8/500's MOV instruction features  an interesting and uncommon op-

timization:  If the target operand  has a size of  16 bits, it is still

possible  to use an 8-bit (immediate)  source operand. For example, for

an instruction like this:

mov.w #$ffff,@$1234

it  is possible to encode  the immediate source code  operand just as a

single  $ff and to save one byte  in code size. The processor automati-

cally  performs a sign extension, which turns $ff into the desired val-

ue  $ffff. AS is aware of this  optimization and will use it, unless it

was explicitly forbidden via a :16 suffix at the immediate operand.

Feedback  from  users  trying  to  assemble  existing code has revealed

that  the original Hitachi assembler  implements this optimization in a

different  way: it  assumes a  zero instead  of a  sign extension. This

means  that immediate  values from  0 to  255 ($0000  to $00ff) and not

from  -128 to  +127 ($ff80  to $007f)  are encoded  as one  byte. Tests

with  physical hardware brought the  result that the Programmers Manual

is  correct: The  processor performs  a sign  extension. AS will there-

fore  by default only use the shorter  encoding if a value ranging from

-128  to +127 respectively $ff80  to $007f is used.  If you have exist-

ing  code that assumes values from $80  to $ff are encoded as one byte,

you may activate a 'compatibility mode', either by the statement

compmode on

in the source code or by the command line switch of same name.

Aside  from this, the same  remarks regarding hexadecimal number syntax

apply as for H8/500.

4.7. SH7000/7600/7700

---------------------

Unfortunately,  Hitachi once again used  their own format for hexadeci-

mal  numbers, and  once again  I was  not able  to reproduce  this with

AS...please use Motorola syntax!

When  using literals  and the  LTORG instruction,  a few things have to

be  kept in  mind if  you do  not want  to suddenly get confronted with

strange error messages:

Literals  exist due to  the fact that  the processor is  unable to load

constants  out of a range of -128  to 127 with immediate addressing. AS

(and  the  Hitachi  assembler)  hide  this  inability  by the automatic

placement  of constants  in memory  which are  then referenced  via PC-

relative  addressing. The question  that now arises  is where to locate

these  constants in memory. AS does  not automatically place a constant

in  memory  when  it  is  needed;  instead, they are collected until an

LTORG  instruction occurs. The  collected constants are  then dumped en

bloc,  and their addresses are stored  in ordinary labels which are al-

so visible in the symbol table. Such a label's name is of the form

LITERAL_s_xxxx_n  .

In  this name, s  represents the literal's  type. Possible values are W

for  16-bit constants, L for 32-bit  constants and F for forward refer-

ences  where AS cannot decide in  anticipation which size is needed. In

case  of s=W or L,  xxxx denotes the constant's  value in a hexadecimal

notation,  whereas xxxx is  a simple running  number for forward refer-

ences  (in a forward reference,  one does not know  the value of a con-

stant  when it is  referenced, so one  obviously cannot incorporate its

value  into the name). n is a counter that signifies how often a liter-

al  of this value previously occurred  in the current section. Literals

follow  the standard rules  for localization by  sections. It is there-

fore  absolutely necessary to  place literals that  were generated in a

certain section before the section is terminated!

The  numbering with n  is necessary because  a literal may occur multi-

ple  times in  a section.  One reason  for this  situation is  that PC-

relative  addressing only  allows positive  offsets; Literals that have

once  been placed with an LTORG can  therefore not be referenced in the

code  that follows. The other reason is that the displacement is gener-

ally limited in length (512 resp. 1024 bytes).

An  automatic LTORG at the end of  a program or previously to switching

to  a different target CPU does not  occur; if AS detects unplaced lit-

erals in such a situation, an error message is printed.

As  the PC-relative addressing mode uses the address of the current in-

struction  plus  4,  it  is  not  possible  to access a literal that is

stored directly after the instruction, like in the following example:

mov     #$1234,r6

ltorg

This  is a  minor item  since the  CPU anyway  would try to execute the

following  data as code.  Such a situation  should not occur  in a real

program...another  pitfall is far more  real: if PC-relative addressing

occurs  just behind  a delayed  branch, the  program counter is already

set  to the destination address, and the displacement is computed rela-

tive  to the branch target  plus 2. Following is  an example where this

detail leads to a literal that cannot be addressed:

bra     Target

mov     #$12345678,r4        ; is executed

.

.

ltorg                        ; here is the literal

.

.

Target: mov     r4,r7                ; execution continues here

As  Target+2 is on an address  behind the literal, a negative displace-

ment  would  result.  Things  become  especially  hairy when one of the

branch  instructions JMP, JSR, BRAF, or BSRF is used: as AS cannot cal-

culate  the target  address (it  is generated  at runtime from a regis-

ter's  contents), a PC  value is assumed  that should never fit, effec-

tively disabling any PC-relative addressing at this point.

It  is not possible to deduce the  memory usage from the count and size

of  literals. AS might  need to insert  a padding word  to align a long

word  to an address that  is evenly divisible by  4; on the other hand,

AS  might reuse  parts of  a 32-bit  literal for other 16-bit literals.

Of  course multiple use of  a literal with a  certain value will create

only  one entry. However, such  optimizations are completely suppressed

for  forward references as  AS does not  know anything about their val-

ue.

As  literals use the  PC-relative addressing which  is only allowed for

the  MOV instruction, the usage of literals  is also limited to MOV in-

structions.  The way AS uses the operand size is a bit tricky: A speci-

fication  of a byte or word move  means to generate the shortest possi-

ble  instruction that results in the desired value placed in the regis-

ter's  lowest 8 resp. 16  bits. The upper 24  resp. 16 bits are treated

as  ''don't care''. However, if one  specifies a longword move or omits

the  size specification  completely, this  means that  the complete 32-

bit  register should  contain the  desired value.  For example,  in the

following sequence

mov.b   #$c0,r0

mov.w   #$c0,r0

mov.l   #$c0,r0   ,

the  first instruction  will result  in true  immediate addressing, the

second  and third instruction will use a  word literal: As bit 7 in the

number  is set, the byte instruction  will effectively create the value

$FFFFFFC0  in the register. According  to the convention, this wouldn't

be  the desired value in the second  and third example. However, a word

literal  is also  sufficient for  the third  case because the processor

will copy a cleared bit 15 of the operand to bits 16..31.

As  one can see, the  whole literal stuff is  rather complex; I'm sorry

but  there was no chance of  making things simpler. It is unfortunately

a  part of its nature that one sometimes gets error messages about lit-

erals  that were  not found,  which logically  should not occur because

AS  does the literal processing completely on his own. However, if oth-

er  errors occur in the second pass, all following labels will move be-

cause  AS does not generate any code  any more for statements that have

been  identified  as  erroneous.  As  literal names are partially built

from  other symbols' values, other  errors might follow because literal

names  searched in the second pass differ  from the names stored in the

first  pass and  AS quarrels  about undefined  symbols...if such errors

should  occur, please correct  all other errors  first before you start

cursing on me and literals...

People  who come out of the Motorola  scene and want to use PC-relative

addressing  explicitly  (e.g.  to  address  variables  in  a  position-

independent  way) should know  that if this  addressing mode is written

like in the programmer's manual:

mov.l   @(Var,PC),r8

no  implicit conversion  of the  address to  a displacement will occur,

i.e.  the operand  is inserted  as-is into  the machine code (this will

probably  generate  a  value  range  error...).  If you want to use PC-

relative  addressing on the SH7x00,  simply use ''absolute'' addressing

(which does not exist on machine level):

mov.l   Var,r8

In  this  example,  the  displacement  will be calculated correctly (of

course,  the same limitations apply for  the displacement as it was the

case for literals).

4.8. HMCS400

------------

The  instruction set of  these 4 bit  processors spontaneously reminded

me  of the  8080/8085 -  many mnemonics,  the addressing mode (e.g. di-

rect  or indirect) is coded into  the instruction, and the instructions

are  sometimes hard to  memorize. AS or  course supports this syntax as

Hitachi  defined it. I however implemented another variant for most in-

structions  that is  - in  my opinion  - more  beautiful and  better to

read.  The approach  is similar  to what  Zilog did  back then  for the

Z80.  For  instance,  all  machine  instructions  that transfer data in

some  form, may the operands be  constants, registers, or memory cells,

may  be used via the LD  instruction. Similar 'meta instructions' exist

for  arithmetic and logical  instructions. A complete  list of all meta

instructions  and their  operands can  be found  in the  tables 4.8 and

4.8, their practical use can be seen in the file t_hmcs4x.asm.

+--------------------+------------------------------------+

| Meta Instruction   | Replaces                           |

+--------------------+------------------------------------+

| LD src, dest       | LAI, LBI, LMID, LMIIY,             |

|                    | LAB, LBA, LAY, LASPX, LASPY, LAMR, |

|                    | LWI, LXI, LYI, LXA, LYA, LAM, LAMD |

|                    | LBM, LMA, LMAD, LMAIY, LMADY       |

| XCH src, dest      | XMRA, XSPX, XSPY, XMA, XMAD, XMB   |

| ADD src, dest      | AYY, AI, AM, AMD                   |

| ADC src, dest      | AMC, AMCD                          |

| SUB src, dest      | SYY                                |

| SBC src, dest      | SMC, SMCD                          |

| OR src, dest       | OR, ORM, ORMD                      |

| AND src, dest      | ANM, ANMD                          |

| EOR src, dest      | EORM, EORMD                        |

| CP cond, src, dest | INEM, INEMD, ANEM, ANEMD, BNEM,    |

|                    | YNEI, ILEM, ILEMD, ALEM, ALEMD,    |

|                    | BLEM, ALEI                         |

| BSET bit           | SEC, SEM, SEMD                     |

| BCLR bit           | REC, REM, REMD                     |

| BTST bit           | TC, TM, TMD                        |

+--------------------+------------------------------------+

Table 4.1: Meta Instructions HMCS400

+-----------+------------------------------------+

| Operand   | Types                              |

+-----------+------------------------------------+

| src, dest | A, B, X, Y, W, SPX, SPY (register) |

|           | M (memory addressed by X/Y/W)      |

|           | M+ (ditto, with auto increment)    |

|           | M- (ditto, with auto decrement)    |

|           | #val (2/4 bits immediate)          |

|           | addr10 (memory direct)             |

|           | MRn (memory register 0..15)        |

| cond      | NE (unequal)                       |

|           | LE (less or equal)                 |

| bit       | CA (carry)                         |

|           | bitpos,M                           |

|           | bitpos,addr10                      |

| bitpos    | 0..3                               |

+-----------+------------------------------------+

Table 4.2: Operand Types for HMCS400 Meta Instructions

4.9. H16

--------

The  instruction  set  of  the  H16's  core  well  deserves  the  label

''CISC'':  complex  addressing  modes,  instructions of extremely vari-

able  length, and there are many  shortforms for instructions with com-

mon   operands.  For  instance,  several  instructions  know  different

''formats'',  depending on the type  of source and destination operand.

The  general rule is that AS will always use the shortest possible for-

mat, unless it was specified explicitly: angegeben:

mov.l     r4,r7     ; uses R format

mov.l     #4,r7     ; uses RQ format

mov.l     #4,@r7    ; uses Q format

mov.l     @r4,@r7   ; uses G format

mov:q.l   #4,r7     ; forces Q instead of RQ format

mov:g.l   #4,r7     ; forces G instead of RQ format

For  immediate arguments, the ''natural'  argument length is used, e.g.

2  bytes for 16 bits.  Shorter or longer arguments  may be forced by an

appended  operand size (.b, .w, .l or  :8, :16, :32). However, the rule

for  displacements  and  absolute  addresses  is that the shortest form

will  be used  if no  explicit size  is given. This includes exploiting

that  the processor does not output the  uppermost eight bits of an ad-

dress.  Therefore, an  absolute address  of $ffff80  can be  coded as a

single byte ($80).

Furthermore,   AS  knows  the  ''accumulator  bit'',  i.e.  the  second

operand  of a two-operand  instruction my be  left away if the destina-

tion is register zero. There is currently no override this behaviour.

Additionally, the following optimizations are performed:

- MOV  R0,<ea>  gets  optimized  to  MOVF  <ea>, unless <ea> is a PC-

relative   expression  and  the  size  of  the  displacement  would

change.  This optimization may be disabled by specifying an explic-

it format.

- SUB  does not support the  Q format, however it  may be replaced by

ADD:Q  with a negated immediate argument,  given the argument is in

the  range -127...+128. This  optimization may as  well be disabled

by specifying an explicit format.

4.10. OLMS-40

-------------

Similar  to  the  HMCS400,  addressing  modes  are  largely encoded (or

rather  encrypted..) into into  the mnemonics, and  also here I decided

to  provide an  alternate notation  that is  more modern  and better to

read.  A complete list of all  meta instructions and their operands can

be  found in the tables 4.10 and  4.10, their practical use can be seen

in the file t_olms4.asm.

+------------------+----------------------------------------+

| Meta Instruction | Replaces                               |

+------------------+----------------------------------------+

| LD dest, src     | LAI, LLI, LHI, L,                      |

|                  | LAL, LLA, LAW, LAX, LAY, LAZ,          |

|                  | LWA, LXA, LYA, LPA, LTI, RTH, RTL      |

| DEC dest         | DCA, DCL, DCM, DCW, DCX, DCY, DCZ, DCH |

| INC dest         | INA, INL, INM, INW, INX, INY, INZ      |

| BSET bit         | SPB, SMB, SC                           |

| BCLR bit         | RPB, RMB, RC                           |

| BTST bit         | TAB, TMB, Tc                           |

+------------------+----------------------------------------+

Table 4.3: Meta Instructions OLMS-40

+-----------+---------------------------------------------+

| Operand   | Types                                       |

+-----------+---------------------------------------------+

| src, dest | A, W, X, Y, Z, DPL, DPH (Register)          |

|           | T, TL, TH (Timer, obere/untere H"alfte)     |

|           | (DP), M (Speicher adressiert durch DPH/DPL) |

|           | #val (4/8 bit immediate)                    |

|           | PP (Port-Pointer)                           |

| bit       | C (Carry)                                   |

|           | (PP), bitpos                                |

|           | (DP), bitpos                                |

|           | (A), bitpos                                 |

| bitpos    | 0..3                                        |

+-----------+---------------------------------------------+

Table 4.4: Operand Types for OLMS-40 Meta Instructions

4.11. OLMS-50

-------------

The  data memory of these 4 bit  controllers consists of up to 128 nib-

bles.  However, only  a very  small subset  of the machine instructions

have  enough space to accomodate seven address bits, which menas that -

once  again - banking must help  out. The majority of instructions that

address  memory only  contain the  lower four  bits of the RAM address,

and  unless the  lowest 16  nibbles of  the memory  shall be addressed,

the  P register delivers  the necessary upper  address bits. The assem-

bler is told about its current value via an

assume  p:<value>

statement, e.g. directly after a PAGE instruction.

Speaking  of PAGE:  both PAGE  and SWITCH  are machine  instructions on

these  controllers, i.e. the do not  have the function known from other

targets.  The pseudo  instruction to  start a  SWITCH/CASE construct is

SELECT  in OLMS-50 mode, and  the listing's page size  is set via PAGE-

SIZE.

4.12. MELPS-4500

----------------

The  program memory of these microcontrollers  is organized in pages of

128  words. Honestly said, this  organization only exists because there

are  on the one  hand branch instructions  with a target  that must lie

within  the same  page, and  on the  other hand  ''long'' branches that

can  reach the whole address space. The standard syntax defined by Mit-

subishi  demands that page number and offset  have to be written as two

distinct  arguments for the  latter instructions. As  this is quite in-

convenient  (except for indirect jumps, a  programmer has no other rea-

son  to deal with  pages), AS also  allows to write  the target address

in a ''linear'' style, for example

bl      $1234

instead of

bl      $24,$34 .

4.13. 6502UNDOC

---------------

Since  the 6502's undocumented instructions  naturally aren't listed in

any  data book, they shall be listed  shortly at this place. Of course,

you  are using them  on your own  risk. There is  no guarantee that all

mask  revisions will support all variants!  They anyhow do not work for

the  CMOS successors of the 6502,  since they allocated the correspond-

ing bit combinations with "official" instructions...

The following symbols are used:

&         binary AND

|         binary OR

^        binary XOR

<<        logical shift left

>>        logical shift right

<<<       rotate left

>>>       rotate right

<-        assignment

(..)      contents of ..

..        bits ..

A         accumulator

X,Y       index registers X,Y

S         stack pointer

An        accumulator bit n

M         operand

C         carry

PCH       upper half of program counter

Instruction      : JAM or KIL or CRS

Function         : none, prozessor is halted

Addressing Modes : implicit

Instruction      : SLO

Function         : M<-((M)<<1)|(A)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : ANC

Function         : A<-(A)&(M), C<- A7

Addressing Modes : immediate

Instruction      : RLA

Function         : M<-((M)<<1)&(A)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : SRE

Function         : M<-((M)>>1)^(A)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : ASR

Function         : A<-((A)&(M))>>1

Addressing Modes : immediate

Instruction      : RRA

Function         : M<-((M)>>>1)+(A)+(C)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : ARR

Function         : A<-((A)&(M))>>>1

Addressing Modes : immediate

Instruction      : SAX

Function         : M<-(A)&(X)

Addressing Modes : absolute long/short, Y-indexed short,

Y-indirect

Instruction      : ANE

Function         : M<-((A)&$ee)|((X)&(M))

Addressing Modes : immediate

Instruction      : SHA

Function         : M<-(A)&(X)&(PCH+1)

Addressing Modes : X/Y-indexed long

Instruction      : SHS

Function         : X<-(A)&(X), S<-(X), M<-(X)&(PCH+1)

Addressing Modes : Y-indexed long

Instruction      : SHY

Function         : M<-(Y)&(PCH+1)

Addressing Modes : Y-indexed long

Instruction      : SHX

Function         : M<-(X)&(PCH+1)

Addressing Modes : X-indexed long

Instruction      : LAX

Function         : A,X<-(M)

Addressing Modes : absolute long/short, Y-indexed long/short,

X/Y-indirect

Instruction      : LXA

Function         : X04<-(X)04&(M)04,

A04<-(A)04&(M)04

Addressing Modes : immediate

Instruction      : LAE

Function         : X,S,A<-((S)&(M))

Addressing Modes : Y-indexed long

Instruction      : DCP

Function         : M<-(M)-1, Flags<-((A)-(M))

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

Instruction      : SBX

Function         : X<-((X)&(A))-(M)

Addressing Modes : immediate

Instruction      : ISB

Function         : M<-(M)+1, A<-(A)-(M)-(C)

Addressing Modes : absolute long/short, X-indexed long/short,

Y-indexed long, X/Y-indirect

4.14. MELPS-740

---------------

Microcontrollers  of  this  family  have  a  quite  nice, however well-

hidden  feature: If one sets bit 5  of the status register with the SET

instruction,  the accumulator will be replaced with the memory cell ad-

dressed  by the X  register for all  load/store and arithmetic instruc-

tions.  An attempt to integrate this  feature cleanly into the assembly

syntax  has not been made so far, so the only way to use it is current-

ly  the  ''hard''  way  (SET...instructions  with  accumulator address-

ing...CLT).

Not  all  MELPS-740  processors  implement  all instructions. This is a

place  where the programmer  has to watch  out for himself  that no in-

structions  are used that  are unavailable for  the targeted processor;

AS  does not differentiate among the individual processors of this fam-

ily.  For a description of the  details regarding special page address-

ing, see the discussion of the ASSUME instruction.

4.15. MELPS-7700/65816

----------------------

As  it seems,  these two  processor families  took disjunct development

paths,  starting from  the 6502  via their  8 bit predecessors. Shortly

listed, the following differences are present:

- The 65816 does not have a B accumulator.

- The 65816 does not have instructions to multiply or divide.

- The  65816 misses  the instructions  SEB, CLB,  BBC, BBS, CLM, SEM,

PSH,  PUL and  LDM. Instead,  the instructions  TSB, TRB, BIT, CLD,

SED, XBA, XCE and STZ take their places in the opcode table.

The  following  instructions  have  identical  function,  yet different

names:

+-------+------------++-------+------------+

| 65816 | MELPS-7700 || 65816 | MELPS-7700 |

+-------+------------++-------+------------+

+-------+------------++-------+------------+

|  REP  |     CLP    ||  PHK  |     PHG    |

|  TCS  |     TAS    ||  TSC  |     TSA    |

|  TCD  |     TAD    ||  TDC  |     TDA    |

|  PHB  |     PHT    ||  PLB  |     PLT    |

|  WAI  |     WIT    ||       |            |

+-------+------------++-------+------------+

Especially  tricky are  the instructions  PHB, PLB  and TSB:  these in-

structions  have  a  totally  different  encoding  and  meaning on both

processors!

Unfortunately,  these processors address their memory  in a way that is

IMHO  even one level higher on  the open-ended chart of perversity than

the  Intel-like segmentation: They  do banking! Well,  this seems to be

the  price for the 6502 upward-compatibility;  before one can use AS to

write  code for these processors,  one has to inform  AS about the con-

tents of several registers (using the ASSUME instruction):

The  M flag rules whether the accumulators  A and B should be used with

8  bits (1) or 16  bits (0) width. Analogously,  the X flag decides the

width  of the X  and Y index  registers. AS needs  this information for

the  decision  about  the  argument's  width  when immediate addressing

(#<constant>) occurs.

The  memory is organized  in 256 banks  of 64 KBytes.  As all registers

in  the CPU  core have  a maximum  width of  16 bits,  the upper 8 bits

have  to be fetched from 2 special  bank registers: DT delivers the up-

per  8  bits  for  data  accesses,  and  PG  extends the 16-bit program

counter  to 24 bits. A 16 bits wide register DPR allows to move the ze-

ro  page known  from the  6502 to  an arbitrary  location in  the first

bank.  If AS  encounters an  address (it  is irrelevant if this address

is  part of an absolute, indexed,  or indirect expression), the follow-

ing addressing modes will be tested:

1 Is  the address  in the  range of  DPR..DPR+$ff? If yes, use direct

addressing with an 8-bit address.

2 Is  the address contained in the  page addressable via DT (resp. PG

for  branch instructions)? If  yes, use absolute  addressing with a

16-bit address.

3 If nothing else helps, use long addressing with a 24-bit address.

As  one can see from this  enumeration, the knowledge about the current

values  of DT, PG and  DPR is essential for  a correct operation of AS;

if  the  specifications  are  incorrect,  the  program will probably do

wrong  addressing at  runtime. This  enumeration also  implied that all

three  address lengths are available; if this  is not the case, the de-

cision chain will become shorter.

The  automatic determination of the  address length described above may

be  overridden by  the usage  of prefixes.  If one prefixes the address

by  a <, >, or >> without a  separating space, an address with 1, 2, or

3  bytes of  length will  be used,  regardless if  this is  the optimal

length.  If one uses an  address length that is  either not allowed for

the  current instruction  or too  short for  the address, an error mes-

sage is the result.

To  simplify porting of 6502 programs,  AS uses the Motorola syntax for

hexadecimal  constants  instead  of  the  Intel/IEEE syntax that is the

format  preferred by Mitsubishi for their  740xxx series. I still think

that  this is the  better format, and  it looks as  if the designers of

the  65816 were of the same  opinion (as the RELAXED instruction allows

the  alternative use of  Intel notation, this  decision should not hurt

anything).  Another  important  detail  for  the porting of programs is

that  it is valid to  omit the accumulator A  as target for operations.

For  example, it  is possible  to simply  write LDA  #0 instead  of LDA

A,#0.

A  real goodie  in the  instruction set  are the instructions MVN resp.

MVP  to do block transfers.  However, their address specification rules

are  a bit strange: bits 0--15 are stored in index registers, bits 16--

23  are part of  the instruction. When  one uses AS,  one simply speci-

fies  the full destination and source addresses. AS will then automati-

cally  grab the correct  bits. This is  a fine yet important difference

Mitsubishi's  assembler where you  have to extract  the upper 8 bits on

your  own. Things become  really convenient when  a macro like the fol-

lowing is used:

mvpos   macro   src,dest,len

if      MomCPU=$7700

lda    #len

elseif

lda    #(len-1)

endif

ldx     #(src&$ffff)

ldy     #(dest&$ffff)

mvp     dest,src

endm

Caution,  possible pitfall:  if the  accumulator contains  the value n,

the  Mitsubishi chip will  transfer n bytes,  but the 65816 will trans-

fer n+1 bytes!

The  PSH and  PUL instructions  are also  very handy because they allow

to  save a user-defined  set to be  saved to the  stack resp. to be re-

stored  from the  stack. According  to the  Mitsubishi data  book [62],

the  bit mask has to be specified  as an immediate operand, so the pro-

grammer  either has to  keep all bit<->register  assignments in mind or

he  has to define  some appropriate symbols.  To make things simpler, I

decided  to extend the syntax at this point:  It is valid to use a list

as  argument which may contain an  arbitrary sequence of register names

or immediate expressions. Therefore, the following instructions

psh     #$0f

psh     a,b,#$0c

psh     a,b,x,y

are  equivalent. As immediate expressions are still valid, AS stays up-

ward compatible to the Mitsubishi assemblers.

One  thing I did not fully understand while studying the Mitsubishi as-

sembler  is the treatment of the  PER instruction: this instruction al-

lows  to push a 16-bit variable onto  the stack whose address is speci-

fied  relative to the program counter. Therefore, it is an absolute ad-

dressing  mode from the  programmer's point of  view. Nevertheless, the

Mitsubishi  assembler requests  immediate addressing,  and the instruc-

tions  argument is placed into  the code just as-is.  One has to calcu-

late  the address  in his  own, which  is something symbolic assemblers

were  designed for to  avoid...as I wanted  to stay compatible, AS con-

tains  a compromise: If one chooses  immediate addressing (with a lead-

ing  # sign), AS will behave like  the original from Mitsubishi. But if

the  # sign  is omitted,  as will  calculate the difference between the

argument's  value and the current program  counter and insert this dif-

ference instead.

A  similar situation  exists for  the PEI  instruction that  pushes the

contents  of a  16-bit variable  located in  the zero  page: Though the

operand  represents an address, once  again immediate addressing is re-

quired.  In this  case, AS  will simply  allow both variants (i.e. with

or without a # sign).

4.16. M16

---------

The  M16 family is a  family of highly complex  CISC processors with an

equally  complicated  instruction  set.  One  of  the instruction set's

properties  is the  detail that  in an  instruction with  two operands,

both  operands may be  of different sizes.  The method of appending the

operand  size as an  attribute of the  instruction (known from Motorola

and  adopted  from  Mitsubishi)  therefore  had  to  be extended: it is

valid  to append  attributes to  the operands  themselves. For example,

the following instruction

mov     r0.b,r6.w

reads  the lowest 8  bits of register  0, sign-extends them  to 32 bits

and  stores the result into  register 6. However, as  one does not need

this  feature in  9 out  of 10  cases, it  is still valid to append the

operand size to the instruction itself, e.g.

mov.w   r0,r6

Both  variants may be mixed;  in such a case,  an operand size appended

to  an  operand  overrules  the  ''default''. An exception are instruc-

tions  with two operands.  For these instructions,  the default for the

source  operand is the destination operand's  size. For example, in the

following example

mov.h   r0,r6.w

register  0 is accessed  with 32 bits,  the size specification appended

to  the instruction is not used at all. If an instruction does not con-

tain  any size  specifications, word  size (w)  will be used. Remember:

in  contrast to  the 68000  family, this  means 32  bits instead  of 16

bits!

The  chained addressing modes  are also rather  complex; the ability of

AS  to automatically  assign address  components to  parts of the chain

keeps  things at least halfway manageable.  The only way of influencing

AS  allows (the  original assembler  from Mitsubishi/Green Hills allows

a  bit more  in this  respect) is  the explicit setting of displacement

lengths by appending :4, :16 and :32.

4.17. CP-3F

-----------

The  CP-3F was  developed in  the early  1970, a  time when development

systems  were also much less powerful  than today. So when the assembly

language  for a new processor  was designed, it was  also a design tar-

get  that it  should be  easily translatable  into machine language. So

the  CP-3F's original assembler  mnemonics group into  few classes that

can  be treated the same by  an assembler: instructions with an immedi-

ate  argument  of  3,  4,  or  8  bits,  instructions  with  a register

operand,  branches and  instructions with  no argument  at all. This is

simple  to translate into  machine code, the  readability of the source

code  however leaves  to be  desired by  today's standards - comparable

to  Intel's assembly language for the 8080. I therefore decided to pro-

vide  an alternate syntax which more clearly expresses what an instruc-

tion does:

Table 4.5: Alternate Instruction Syntax for CP-3F

+----------+---------------+----------------------------+

| Original | Alternate     | Function                   |

+----------+---------------+----------------------------+

+----------+---------------+----------------------------+

| las imm4 | ld a,#<imm4   | A <-- 0:imm4               |

+----------+---------------+----------------------------+

| lal imm8 | ld a,#>imm8   | A <-- imm8                 |

+----------+---------------+----------------------------+

| lss imm3 | ld s,#imm3    | S <-- imm3                 |

+----------+---------------+----------------------------+

| lts imm3 | ld t,#imm3    | T <-- imm3                 |

+----------+---------------+----------------------------+

| anl imm8 | and [a,]#imm8 | A <-- A & imm8             |

+----------+---------------+----------------------------+

| eol imm8 | xor [a,]#imm8 | A <-- A ^ imm8             |

+----------+---------------+----------------------------+

| orl imm8 | or [a,]#imm8  | A <-- A | imm8             |

+----------+---------------+----------------------------+

| adl imm8 | add [a,]#imm8 | A <-- A + imm8             |

+----------+---------------+----------------------------+

| cml imm8 | cp [a,]#imm8  | Flags <-- A - imm8         |

+----------+---------------+----------------------------+

| lav      | ld a,v        | A <-- V                    |

+----------+---------------+----------------------------+

| law      | ld a,w        | A <-- W                    |

+----------+---------------+----------------------------+

| lax      | ld a,x        | A <-- X                    |

+----------+---------------+----------------------------+

| lay      | ld a,y        | A <-- Y                    |

+----------+---------------+----------------------------+

| sav      | ld v,a        | V <-- A                    |

+----------+---------------+----------------------------+

| saw      | ld w,a        | W <-- A                    |

+----------+---------------+----------------------------+

| sax      | ld x,a        | X <-- A                    |

+----------+---------------+----------------------------+

| say      | ld y,a        | Y <-- A                    |

+----------+---------------+----------------------------+

| sat      | ld t,a        | T <-- A                    |

+----------+---------------+----------------------------+

| sst      | ld st,a       | S|T <-- A                  |

+----------+---------------+----------------------------+

| als      | sla [a]       | A(7..1) <-- A(6..0),       |

|          | sla [a],1     | A(0) <-- 0                 |

+----------+---------------+----------------------------+

| ars      | srl [a]       | A(6..0) <-- A(7..1),       |

|          | srl [a],1     | A(7) <-- 0                 |

+----------+---------------+----------------------------+

| alf      | sla [a],4     | A(7..4) <-- A(3..0),       |

|          |               | A(3..0) <-- 0              |

+----------+---------------+----------------------------+

| arf      | srl [a],4     | A(3..0) <-- A(7..4),       |

|          |               | A(7..4) <-- 0              |

+----------+---------------+----------------------------+

| lar n    | ld a,n        | A <-- R(n), n=0..11        |

+----------+---------------+----------------------------+

| lar 12   | ld a,(st)     | A <-- R(S,T)               |

+----------+---------------+----------------------------+

| lar 13   | ld a,(st)-    | A <-- R(S,T),              |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| lar 14   | ld a,(st)+    | A <-- R(S,T),              |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| sar n    | ld n,a        | R(n) <-- A, n=0..11        |

+----------+---------------+----------------------------+

| sar 12   | ld (st),a     | R(S,T) <-- A               |

+----------+---------------+----------------------------+

| sar 13   | ld (st)-,a    | R(S,T) <-- A,              |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| sar 14   | ld (st)+,a    | R(S,T) <-- A,              |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| adr n    | add a,n       | A <-- A + R(n), n=0..11    |

+----------+---------------+----------------------------+

| adr 12   | add a,(st)    | A <-- A + R(S,T)           |

+----------+---------------+----------------------------+

| adr 13   | add a,(st)-   | A <-- A + R(S,T),          |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| adr 14   | add a,(st)+   | A <-- A + R(S,T),          |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| anr n    | and a,n       | A <-- A & R(n), n=0..11    |

+----------+---------------+----------------------------+

| anr 12   | and a,(st)    | A <-- A & R(S,T)           |

+----------+---------------+----------------------------+

| anr 13   | and a,(st)-   | A <-- A & R(S,T),          |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| anr 14   | and a,(st)+   | A <-- A & R(S,T),          |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| eor n    | xor a,n       | A <-- A ^ R(n), n=0..11    |

+----------+---------------+----------------------------+

| eor 12   | xor a,(st)    | A <-- A ^ R(S,T)           |

+----------+---------------+----------------------------+

| eor 13   | xor a,(st)-   | A <-- A ^ R(S,T),          |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| eor 14   | xor a,(st)+   | A <-- A ^ R(S,T),          |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| dec n    | dec a,n       | R(n) <-- R(n) - 1, n=0..11 |

+----------+---------------+----------------------------+

| dec 12   | dec a,(st)    | R(S,T) <-- R(S,T) - 1      |

+----------+---------------+----------------------------+

| dec 13   | dec a,(st)-   | R(S,T) <-- R(S,T) - 1,     |

|          |               | S <-- S-1                  |

+----------+---------------+----------------------------+

| dec 14   | dec a,(st)+   | R(S,T) <-- R(S,T) - 1,     |

|          |               | S <-- S+1                  |

+----------+---------------+----------------------------+

| six      | ld (z(x)),a   | (Z(module X)) <-- A        |

+----------+---------------+----------------------------+

| lix      | ld a,(z(x))   | A <-- (Z(module X))        |

+----------+---------------+----------------------------+

| liy      | ld a,(z(y))   | A <-- (Z(module Y))        |

+----------+---------------+----------------------------+

| sqx      | ld q(x),a     | Q(module X) <-- X|A        |

+----------+---------------+----------------------------+

| sqy      | ld q(y),a     | Q(module Y) <-- Y|A        |

+----------+---------------+----------------------------+

| szx      | ld z(x),a     | Z(module X) <-- X|A        |

+----------+---------------+----------------------------+

| szy      | ld z(y),a     | Z(module Y) <-- Y|A        |

+----------+---------------+----------------------------+

4.18. 4004/4040

---------------

Thanks  to John Weinrich, I now have the official Intel data sheets de-

scribing  these 'grandfathers'  of all  microprocessors, and  the ques-

tions  about the syntax  of register pairs  (for 8-bit operations) have

been  weeded out for the  moment: It is RnRm  with n resp. m being even

integers  in the range from 0  to E resp. 1 to  F. The equation m = n +

1 must be fulfilled.

4.19. MCS-48

------------

The  maximum address  space of  these processors  is 4 Kbytes, resp. up

to  8 Kbytes on some Philips varaints.  This address space is not orga-

nized  in a  linear way  (how could  this be  on an  Intel CPU...). In-

stead,  it is split  into 2 banks  of 2 Kbytes.  The only way to change

the  program counter  from one  bank to  the other are the instructions

CALL  and JMP, by setting the most  significant bit of the address with

the instructions SEL MB0 to SEL MB3.

The  assembler may be informed about  the bank currently being selected

for jumps and calls, via an ASSUME statement:

\asname{}SUME MB:<0..3>

If  one tries to  jump to an  address in a  different bank, a warnig is

issued.

If  the  special  value  NOTHING  is  used  (this is by the way the de-

fault),  an automatism uilt into JMP and CALL is activated. It will in-

sert  a SEL MBx instruction if the current program counter and the tar-

get  address are located in different  banks. Explicit usage of SEL MBx

instructions  is no longer necessary  (though it remains possible), and

it  might interfere  with this  mechanism, like  in the following exam-

ple:

000:  SEL      MB1

JMP      200h

AS  assumes that the MB  flag is 0 and  therefore does not insert a SEL

MB0 instruction, with the result that the CPU jumps to address A00h.

Furthermore,  one should keep in mind that a jump instruction might be-

come longer (3 instead of 2 bytes).

4.20. MCS-51

------------

The   assembler  is   accompanied  by   the  files  STDDEF51.INC  resp.

80C50X.INC  that  define  all  bits  and  SFRs  of the processors 8051,

8052,  and 80515  resp. 80C501,  502, and  504. Depending on the target

processor  setting (made  with the  CPU statement),  the correct subset

will  be included.  Therefore, the  correct order  for the instructions

at the beginning of a program is

CPU     <processor type>

INCLUDE stddef51.inc   .

Otherwise,  the  MCS-51  pseudo  instructions  will  lead to error mes-

sages.

As  the 8051 does not  have instructions to to  push the registers 0..7

onto  the  stack,  one  has  to  work with absolute addresses. However,

these  addresses depend on which register  bank is currently active. To

make  this situation a little bit  better, the include files define the

macro  USING that  accepts the  symbols Bank0...Bank3  as arguments. In

response,  the macro  will assign  the registers'  correct absolute ad-

dresses  to  the  symbols  AR0..AR7.  This  macro  should be used after

every  change of the  register banks. The  macro itself does not gener-

ate any code to switch to the bank!

The  macro also  makes bookkeeping  about which  banks have  been used.

The  result is  stored in  the integer  variable RegUsage: bit 0 corre-

sponds  to bank 0,  bit 1 corresponds  to bank 1.  and so on. To output

its  contents after the  source has been  assembled, use something like

the following piece of code:

irp       BANK,Bank0,Bank1,Bank2,Bank3

if        (RegUsage&(2^BANK))<>0

message   "bank \{BANK} has been used"

endif

endm

The  multipass feature introduced  with version 1.38  allowed to intro-

duce  the additional instructions  JMP and CALL.  If branches are coded

using  these instructions, AS  will automatically use  the variant that

is  optimal for the  given target address.  The options are SJMP, AJMP,

or  LJMP for JMP resp.  ACALL or LCALL for  CALL. Of course it is still

possible  to use these variants directly, in  case one wants to force a

certain coding.

4.21. MCS-251

-------------

When  designing the 80C251, Intel really tried  to make the move to the

new  family as smooth  as possible for  programmers. This culminated in

the  fact that  old applications  can run  on the new processor without

having  to recompile  them. However,  as soon  as one  wants to use the

new  features, some  details have  to be  regarded which  may turn into

hidden pitfalls.

The  most important  thing is  the absence  of a distinct address space

for  bits on the 80C251. All SFRs can now be addressed bitwise, regard-

less  of their address. Furthermore, the  first 128 bytes of the inter-

nal  RAM are  also bit  addressable. This  has become  possible because

bits  are not any more  handled by a separate  address space that over-

laps  other address spaces. Instead,  similar to other processors, bits

are  addressed with a two-dimensional address that consists of the mem-

ory  location containing  the bit  and the  bit's location in the byte.

One  result is that in an expression like PSW.7, AS will do the separa-

tion  of address  and bit  position itself.  Unlike to  the 8051, it is

not  any more necessary to explicitly  generate 8 bit symbols. This has

the  other result  that the  SFRB instruction  does not exist any more.

If  it is used  in a program  that shall be  ported, it may be replaced

with a simple SFR instruction.

Furthermore,  Intel  cleaned  up  the  cornucopia  of different address

spaces  on the  8051: the  internal RAM  (DATA resp.  IDATA), the XDATA

space  and the former  CODE space were  unified to a  single CODE space

that  is now  16 Mbytes  large. The  internal RAM  starts at address 0,

the  internal ROM starts at address  ff0000h, which is the address code

has  to be relocated  to. In contrast,  the SFRs were  moved to a sepa-

rate  address space  (which AS  refers to  as the IO segment). However,

they  have the same  addresses in this  new address space  as they used

to  have on  the 8051.  The SFR  instructions knows  of this difference

and  automatically assigns  symbols to  either the  DATA or IO segment,

depending  on  the  target  processor.  As  there is no BIT segment any

more,  the BIT instruction operates  completely different: Instead of a

linear  address  ranging  from  0..255,  a  bit symbol now contains the

byte's  address in bit 0..7,  and the bit position  in bits 24..26. Un-

fortunately,  creating arrays of  flags with a  symbolic address is not

that simple any more: On an 8051, one simply wrote:

segment bitdata

bit1    db      ?

bit2    db      ?

or

defbit  macro   name

name    bit     cnt

cnt     set     cnt+1

endm

On a 251, only the second way still works, like this:

adr     set     20h     ; start address of flags

bpos    set     0       ; in the internal RAM

defbit  macro   name

name    bit     adr.bpos

bpos    set     bpos+1

if      bpos=8

bpos     set     0

adr      set     adr+1

endif

endm

Another  small detail: Intel now prefers CY  instead of C as a symbolic

name  for the carry,  so you might  have to rename  an already existing

variable  of the same  name in your  program. However, AS will continue

to  understand also  the old  variant when  using the instructions CLR,

CPL,  SETB, MOV, ANL, or ORL. The same is conceptually true for the ad-

ditional  registers  R8..R15,  WR0..WR30,  DR0..DR28,  DR56, DR60, DPX,

and SPX.

Intel  would like everyone  to write absolute  addresses in a syntax of

XX:YYYY,  where XX is a  64K bank in the  address space resp. signifies

addresses  in the  I/O space  with an  S. As  one might guess, I am not

amused  about this, which is why it  is legal to alternitavely use lin-

ear  addresses in all places. Only the S for I/O addresses is incircum-

ventable, like in this case:

Carry   bit     s:0d0h.7

Without  the prefix,  AS would  assume an  address in the CODE segment,

and only the first 128 bits in this space are bit-addressable...

Like  for the 8051, the generic  branch instructions CALL and JMP exist

that  automatically choose the  shortest machine code  depending on the

address  layout. However, while JMP also may use the variant with a 24-

bit  address, CALL will not  do this for a  good reason: In contrast to

ACALL  and LCALL,  ECALL places  an additional  byte onto  the stack. A

CALL  instruction would  result where  you would  not know what it will

do. This problem does not exist for the JMP instructions.

There  is one  thing I  did not  understand: The  80251 is also able to

push  immediate operands onto the stack,  and it may push either single

bytes  or  complete  words.  However,  the  same mnemonic (PUSH) is as-

signed  to both variants - how on  earth should an assembler know if an

instruction like

push    #10

shall  push a byte  or a word  containing the value  10? So the current

rule  is that PUSH always  pushes a byte; if  one wants to push a word,

simply use PUSHW instead of PUSH.

Another  well-meant advise:  If you  use the  extended instruction set,

be  sure to  operate the  processor in  source mode; otherwise, all in-

structions  will  become  one  byte  longer!  The old 8051 instructions

that  will in  turn become  one byte  longer are  not a  big matter: AS

will  either replace them automatically with new, more general instruc-

tions  or they deal  with obsolete addressing  modes (indirect address-

ing via 8 bit registers).

4.22. 8080/8085

---------------

As mentioned before, the statement

Z80SYNTAX <ON|OFF|EXCLUSIVE>

makes  it possible  to write  the vast  majority of  8080/8085 instruc-

tions  in 'Z80 style', i.e. with  less mnemonics but with operands that

are  easier to  understand. In  non-exclusive mode,  the Z80  syntax is

not  allowed  for  the  following  instructions,  because they conflict

with existing 8080 mnemonics:

- CP  in 'Intel  syntax' means  'Call on  Positive', in  Zilog syntax

however  it means  'Compare'. If  you use  CP with a numeric value,

it  is not possible  for the assembler  to recognize whether a jump

to  an absolute  address or  a compare  with an  immediate value is

meant.  The assembler will generate a  jump in this case, since the

Intel  syntax has precedence  in case of  ambiguities. If one wants

the  comparison, one may  explicitly write down  the accumulator as

destination operand, e.g. CP A,12h instead of CP 12h.

- JP  in Intel syntax means 'Jump  on Positive', in Zilog syntax how-

ever,  this is the  jump instruction in  general. Conditional jumps

in  Zilog syntax (JP cond,addr) are  unambigious because of the two

arguments.  With only one argument,  the assembler will however al-

ways  generate the conditional  jump. If you  want an unconditional

jump  to an absolute address, you still  have to use the Intel syn-

tax ((JMP addr).

The  8085 supports the  instructions RIM and  SIM that are  not part of

the  Z80 instruction  set. They  may be  written in  'Z80 style'  as LD

A,IM resp. LD IM,A.

The  'generic jump' J  known from the  Z80 target is  also available if

Z80  syntax is  activated. However,  since the  8080/8085 does not sup-

port relative jumps, J is always translated to JP.

4.23. 8085UNDOC

---------------

Similarly  to the Z80 or 6502, Intel did not further specify the undoc-

umented  8085 instructions.  This however  means that  other assemblers

might  use  different  mnemonics  for  the  same function. Therefore, I

will  list  the  instructions  in  the  following. Once again, usage of

these  instructions is at one's own risk  - even the Z80 which is prin-

cipally  upward compatible  to the  8085 uses  the opcodes for entirely

different functions...

Instruction : DSUB [reg]

Z80 Syntax : SUB HL,reg

Function  : HL <- HL - reg

Flags     : CY, S, X5, AC, Z, V, P

Arguments : reg = B for BC (optional for non-Z80 syntax)

Instruction : ARHL

Z80 Syntax : SRA HL

Function  : HL,CY <- HL >> 1 (arithmetisch)

Flags     : CY

Arguments : none resp. fixed for Z80 syntax

Instruction : RDEL

Z80 Syntax : RLC DE

Function  : CY,DE <- DE << 1

Flags     : CY, V

Arguments : none resp. fixed for Z80 syntax

Instruction : LDHI d8

Z80 Syntax : ADD DE,HL,d8

Function  : DE <- HL + d8

Flags     : none

Arguments : d8 = 8-bit constant, registers fixed for Z80 syntax

Instruction : LDSI d8

Z80 Syntax : ADD DE,SP,d8

Function  : DE <- SP + d8

Flags     : none

Arguments : d8 = 8-bit constant, registers fixed for Z80 syntax

Instruction : RSTflag

Z80 Syntax : RST flag

Function  : restart to 40h if flag=1

Flags     : none

Arguments : flag = V for overflow bit

Instruction : SHLX [reg]

Z80 Syntax : LD (reg),HL

Function  : [reg] <- HL

Flags     : none

Arguments : reg = D/DE for DE (optional for non-Z80 syntax)

Instruction : LHLX [reg]

Z80 Syntax : LD HL,(reg)

Function  : HL <- [reg]

Flags     : none

Arguments : reg = D/DE for DE (optional for non-Z80 syntax)

Instruction : JNX5 addr

Z80 Syntax : JP NX5, addr

Function  : jump to addr if X5=0

Flags     : none

Arguments : addr = absolute 16-bit address

Instruction : JX5 addr

Z80 Syntax : JP X5,addr

Function  : jump to addr if X5=1

Flags     : none

Arguments : addr = absolute 16-bit address

X5  refers to the otherwise  unused bit 5 in  the processor status word

(PSW).

4.24. 8086..V35

---------------

Actually,  I had  sworn myself  to keep  the segment disease of Intel's

8086  out of the assembler. However, as there was a request and as stu-

dents  are more flexible than the developers of this processor obvious-

ly  were, there  is now  a rudimentary  support of  these processors in

AS.  When saying, 'rudimentary', it does  not mean that the instruction

set  is not fully  covered. It means  that the whole pseudo instruction

stuff  that is  available when  using MASM,  TASM, or something equiva-

lent  does not exist.  To put it  in clear words,  AS was not primarily

designed  to write assembler programs for PC's (heaven forbid, this re-

ally  would have meant  reinventing the wheel!);  instead, the develop-

ment  of programs for  single-board computers was  the main goal (which

may also be equipped with an 8086 CPU).

For  die-hards who still want to write  DOS programs with AS, here is a

small list of things to keep in mind:

- Only COM files may be created.

- Only  use the  CODE segment,  and place  also all variables in this

segment.

- DOS  initializes all segment registers to  the code segment. An AS-

SUME  DS:DATA, SS:DATA right  at the program's  beginning is there-

fore necessary.

- DOS  loads the code to a start address  of 100h. An ORG to this ad-

dress is absolutely necessary.

- The  conversion to a binary  file is done with  P2BIN (see later in

this document), with an address filter of $-$.

For  these  processors,  AS  only  supports  a small programming model,

i.e.  there is one code segment with a  maximum of 64 Kbytes and a data

segment  of equal size for data (which  cannot be set to initial values

for  COM  files).  The  SEGMENT  instruction  allows  to switch between

these  two segments. From  this facts results  that branches are always

intrasegment  branches if  they refer  to targets  in this  single code

segment.  In case that  far jumps should  be necessary, they are possi-

ble  via CALLF or JMPF with a  memory address or a Segment:Offset value

as argument.

Another  big  problem  of  these  processors is their assembler syntax,

which  is sometimes ambiguous and whose  exact meaning can then only be

deduced  by looking at  the current context.  In the following example,

either  absolute  or  immediate  addressing  may be meant, depending on

the symbol's type:

mov     ax,value

When  using AS,  an expression  without brackets  always is interpreted

as  immediate  addressing.  For  example,  when either a variable's ad-

dress  or its contents  shall be loaded,  the differences listed in ta-

ble 4.5 are present between MASM and AS:

+-----------+--------------------+---------------+

| assembler | address            | contents      |

+-----------+--------------------+---------------+

+-----------+--------------------+---------------+

| MASM      | mov ax,offset vari | mov ax,vari   |

|           | lea ax,vari        | mov ax,[vari] |

|           | lea ax,[vari]      |               |

|           |                    |               |

| AS        | mov ax,vari        | mov ax,[vari] |

|           | lea ax,[vari]      |               |

+-----------+--------------------+---------------+

Table 4.5: Differences AS<->MASM Concerning Addressing Syntax

When  addressing via  a symbol,  the assembler  checks whether they are

assigned  to the data segment and  tries to automatically insert an ap-

propriate  segment prefix. This  happens for example  when symbols from

the  code segment are accessed without  specifying a CS segment prefix.

However,  this mechanism can  only work if  the ASSUME instruction (see

there) has previously been applied correctly.

The  Intel syntax  also requires  to store  whether bytes or words were

stored  at a symbol's address.  AS will do this  only when the DB resp.

DW  instruction is in the same source  line as the label. For any other

case,  the operand  size has  to be  specified explicitly with the BYTE

PTR,  WORD PTR,... operators. As long as a register is the other opera-

tor,  this may be  omitted, as the  operand size is  then clearly given

by the register's name.

In  an 8086-based system,  the coprocessor is  usually synchronized via

via  the  processor's  TEST  input  line  which is connected to toe co-

processor's  BUSY output line. AS supports  this type of handshaking by

automatically  inserting  a  WAIT  instruction  prior to every 8087 in-

struction.  If this is undesired for any reason, an N has to be insert-

ed after the F in the mnemonic; for example,

FINIT

FSTSW   [vari]

becomes

FNINIT

FNSTSW  [vari]

This variant is valid for all coprocessor instructions.

4.25. 8X30x

-----------

The  processors of this family have  been optimized for an easy manipu-

lation  of  bit  groups  at  peripheral addresses. The instructions LIV

and  RIV were introduced to deal with  such objects in a symbolic fash-

ion. They work similar to EQU, however they need three parameters:

1 the  address of  the peripheral  memory cell  that contains the bit

group (0..255);

2 the number of the group's first bit (0..7);

3 the length of the group, expressed in bits (1..8).

CAUTION!  The 8X30x  does not  support bit  groups that  span over more

than  one  memory  address.  Therefore,  the  valid value range for the

length  can be  stricter limited,  depending on  the start position. AS

does  not perform any checks at this  point, you simply get strange re-

sults at runtime!

Regarding  the machine code, length and  position are expressed vis a 3

bit  field in the  instruction word and  a proper register number (LIVx

resp.  RIVx). If one uses a  symbolic object, AS will automatically as-

sign  correct values to this  field, but it is  also allowed to specify

the  length explicitly  as a  third operand  if one  does not work with

symbolic  objects. If AS finds such  a length specification in spite of

a  symbolic operand,  it will  compare both  lengths and issue an error

if  they do not  match (the same  will happen for  the MOVE instruction

if  two symbolic  operands with  different lengths  are used  - the in-

struction simply only has a single length field...).

Apart  from the real machine instructions,  AS defines similarly to its

''idol''  MCCAP  some  pseudo  instructions  that  are  implemented  as

builtin macros:

- NOP is a shortform for MOVE AUX,AUX

- HALT is a shortform for JMP *

- XML ii is a shortform for XMIT ii,R12 (only 8X305)

- XMR ii is a shortform for XMIT ii,R13 (only 8X305)

- SEL  <busobj> is a  shortform for XMIT  <adr>,IVL/IVR, i.e. it per-

forms the necessary preselection to access <busobj>.

The  CALL  and  RTN  instructions  MCCAP  also implements are currently

missing  due to  sufficient documentation.  The same  is true for a set

of  pseudo instructions to  store constants to  memory. Time may change

this...

4.26. XA

--------

Similar   to   its   predecessor   MCS/51,   but  in  contrast  to  its

'competitor'  MCS/251, the Philips XA has  a separate address space for

bits,  i.e. all  bits that  are accessible  via bit instructions have a

certain,  one-dimensional address which is  stored as-is in the machine

code.  However, I could not take  the obvious opportunity to offer this

third  address space (code  and data are  the other two)  as a separate

segment.  The reason is that - in contrast to the MCS/51 - some bit ad-

dresses  are ambiguous: bits with  an address from 256  to 511 refer to

the  bits of  memory cells  20h..3fh in  the current data segment. This

means  that these addresses may  correspond to different physical bits,

depending  on the current state. Defining bits  with the help of DC in-

structions  - something that would be  possible with a separate segment

-  would not  make too  much sense.  However, the BIT instruction still

exists  to define individual bits (regardless  if they are located in a

register,  the RAM or  SFR space) that  can then be referenced symboli-

cally.  If the bit  is located in  RAM, the address  of the 64K-bank is

also  stored. This way, AS can check whether the DS register has previ-

ously be assigned a correct value with an ASSUME instruction.

In  contrast, nothing can  stop AS's efforts  to align potential branch

targets  to even addresses.  Like other XA  assemblers, AS does this by

inserting NOPs right before the instruction in question.

4.27. AVR

---------

In  contrast to the AVR assembler, AS  by default uses the Intel format

to  write hexadecimal  contants instead  of the  C syntax. All right, I

did  not look into the (free) AVR  assembler before, but when I started

with  the AVR  part, there  was hardly  mor einformation  about the AVR

than  a preliminary manual  describing processor types  that were never

sold...this problem can be solved with a simple RELAXED ON.

Optionally,  AS can generate so-called "object  files" for the AVRs (it

also  works  for  other  CPUs,  but  it  does  not  make  any sense for

them...).  These are  files containing  code and  source line info what

e.g.  allows a step-by-step execution on  source level with the WAVRSIM

simulator  delivered by  Atmel. Unfortunately,  the simulator  seems to

have  trouble with  source file  names longer  than approx.  20 charac-

ters:  Names are truncated  and/or extended by  strange special charac-

ters  when the  maximum length  is exceeded.  AS therefore  stores file

name  specifications  in  object  files  without  a path specification.

Therefore,  problems may arise when files  like includes are not in the

current directory.

A  small specialty are machine instructions  that have already been de-

fined  by Atmel  as part  of the  architecture, but  up to  now haven't

been  implemented in any  of the family's  members. The instructions in

question  are MUL, JMP, and CALL.  Considering the latter ones, one may

ask  himself  how  to  reach  the  4  Kwords large address space of the

AT90S8515  when the  'next best'  instructions RJMP  and RCALL can only

branch  up  to  2  Kwords  forward  or  backward.  The  trick  is named

'discarding  the upper address  bits' and described  in detail with the

WRAPMODE statement.

All  AVR targets  support the  optional CPU  argument CODESEGSIZE. Like

in this example,

cpu atmega8:codesegsize=0

it  may be  used to  instruct the  assembler to  treat the code segment

(i.e.  the internal flash  ROM) as being  organized in bytes instead of

16  bit words. This is  the view when the  LPM instruction is used, and

which  some other (non Atmel) assemblers use in general. It has the ad-

vantage  that addresses in  the CODE segment  need not be multiplied by

two  if used for data accesses. On the  other hand, care has to be tak-

en  that instructions do  not start on  an odd address  - this would be

the  equivalent of an  instruction occupying fractions  of flash words.

The  PADDING option is  therefore enabled by  default, while it remains

possible  to define  arrays of  bytes via  multiple uses  of DB or DATA

without  the risk of padding bytes inserted in between. Target address-

es  for  relative  and  absolute  branches automatically get divided by

two  in this ''byte mode''.  The default is the  organizazion in 16 bit

word  as used by  the original Atmel  assembler. This may explicitly be

selected by using the argument codesegsize=1.

4.28. Z80, Z380

---------------

The  Z80 supports two  types of jump  instructions: relative ( JR) sup-

ports  jump distances from  -128 to +127  bytes, while absolute jumps (

JP)  allow to  reach the  complete address  space. AS additionally sup-

ports  a pseudo instruction J  which automatically selects the shortest

possible  variant, depending  on the  target address  and the condition

(relative  jumps do not  support all conditions).  This also applies to

all  targets 'derived' from  Z80, like Z80UNDOC,  Z180, RABBIT2000, and

LR35902.

J  is also  offered for  the Z380,  but since  the Z380 supports larger

jump  distances, absolute jumps will only be used if the largest possi-

ble jump distance (+/- 8 MByte) no longer suffices.

This  instruction should  be used  with thought  and care, since JR and

JP  are not entirely equivalent in  function: code that exclusively us-

es  relative jumps  is position-independent,  while absolute  jumps re-

quire relocation.

4.29. Z80UNDOC

--------------

As  one might guess, Zilog did not  make any syntax definitions for the

undocumented  instructions;  furthermore,  not  everyone might know the

full  set. It  might therefore  make sense  to list all instructions at

this place:

Similar  to  the  Z380  and  eZ80,  it  is  possible to access the byte

halves  of IX and IY separately.  In detail, these are the instructions

that allow this:

INC Rx              LD R,Rx             LD  Rx,n

DEC Rx              LD Rx,R             LD  Rx,Ry

ADD/ADC/SUB/SBC/AND/XOR/OR/CP A,Rx

Rx  and Ry are synonyms for IXL, IXU,  IYL or IYU. Keep however in mind

that  in the case of LD Rx,Ry, both  registers must be part of the same

index register.

The  coding of shift  instructions leaves an  undefined bit combination

which  is now accessible as the  ttySL1, SLI, SLIA, or SLS instruction.

It  works like SLA with  the difference of entering  a 1 into bit posi-

tion  0. CAUTION! Some sources also  name this operation SLL. It decid-

ed  to  not  offer  this,  since  it is misleading: SLL translates into

"shift  left logically", and  the operation performed  by this instruc-

tion  is no logical left  shift. If one should  define SLL at all, then

as  an alias for  SLA. If you  have existing code  that uses SLL in the

meaning of SL1/SLI, define it via a macro.

Like  all other  (docummented) shift  instructions, this  also works in

another undocumented variant:

SLIA    R,(XY+d)

SLIA    (XY+d),R

In  this case, R is an arbitrary 8-bit register (excluding index regis-

ter  halves...), and  (XY+d) is  a normal  indexed address. This opera-

tion  has the additional  effect of copying  the result into the regis-

ter. This also works for the RES and SET instructions:

SET/RES R,n,(XY+d)

SET/RES n,(XY+d),R

Furthermore, two hidden I/O instructions exist:

IN      (C) resp. TSTI

OUT     (C),0

Their  operation  should  be  clear.  CAUTION! Noone can guarantee that

all  mask  revisions  of  the  Z80  execute these instructions, and the

Z80's  successors will react with  traps if they find  one of these in-

structions. Use them on your own risk...

4.30. GB_Z80 resp. LR35902

--------------------------

The  LR35902 SoC used  in the original  Gameboy was developed by Sharp,

and  the  CPU  core  is  (probably)  the  same as in the SM83 microcon-

trollers.  Regarding its instruction set,  it is somewhere ''half way''

between  8080 and Z80,  however with its  own omissions and extensions.

Sharp  of course defined an assembler  syntax for the new instructions.

However,  variations have established itself  in the ''Gameboy scene''.

I tried to regard those as well (as far as I am aware of them):

+---------------+---------------+------------------+

| Sharp         | Alternate     | Function         |

+---------------+---------------+------------------+

+---------------+---------------+------------------+

| LD A,(HLD)    | LD A,(HL-)    | A <-- (HL),      |

|               | LDD A,(HL)    | HL <-- HL-1      |

+---------------+---------------+------------------+

| LD A,(HLI)    | LD A,(HL+)    | A <-- (HL),      |

|               | LDI A,(HL)    | HL <-- HL+1      |

+---------------+---------------+------------------+

| LD (HLD),A    | LD (HL-),A    | (HL) <-- A,      |

|               | LDD (HL),A    | HL <-- HL-1      |

+---------------+---------------+------------------+

| LD (HLI),A    | LD (HL+),A    | (HL) <-- A,      |

|               | LDI (HL),A    | HL <-- HL+1      |

+---------------+---------------+------------------+

| LD A,(C)      | LD A,(FF00+C) | A <-- (0ff00h+C) |

|               | LDH A,(C)     |                  |

+---------------+---------------+------------------+

| LD (C),A      | LD (FF00+C),A | (0ff00h+C) <-- A |

|               | LDH (C),A     |                  |

+---------------+---------------+------------------+

| LD (FF00+n),A | LDH (n),A     | (0ff00h+n) <-- A |

+---------------+---------------+------------------+

| LD A,(FF00+n) | LDH A,(n)     | A <-- (0ff00h+n) |

+---------------+---------------+------------------+

| LDHL SP,d     | LD HL,SP+d    | HL <-- SP + d    |

+---------------+---------------+------------------+

| LDX A,(nn)    | LD A,(nn)     | A <-- (nn) ^1    |

+---------------+---------------+------------------+

| LDX (nn),A    | LD (nn),A     | (nn) <-- A ^1    |

+---------------+---------------+------------------+

| ^1 enforces 16 bit addressing                    |

+---------------+---------------+------------------+

4.31. Z380

----------

As  this processor was designed as a grandchild of the still most popu-

lar  8-bit microprocessor, it was a  sine-qua-non design target to exe-

cute  existing Z80 programs without  modification (of course, they exe-

cute  a bit faster, roughly  by a factor of  10...). Therefore, all ex-

tended  features  can  be  enabled  after  a  reset by setting two bits

which  are named XM (eXtended  Mode, i.e. a 32-bit  instead of a 16-bit

address  space) respectively  LW (long  word mode,  i.e. 32-bit instead

of  16-bit operands). One has to  inform AS about their current setting

with  the instructions EXTMODE  resp. LWORDMODE, to  enable AS to check

addresses  and constants against  the correct upper  limits. The toggle

between  32- and 16-bit instruction  of course only influences instruc-

tions  that are available in a  32-bit variant. Unfortunately, the Z380

currently  offers such variants  only for load  and store instructions;

arithmetic  can only be done  in 16 bits. Zilog  really should do some-

thing  about  this,  otherwise  the  most  positive description for the

Z380 would be ''16-bit processor with 32-bit extensions''...

The  whole thing  becomes complicated  by the  ability to  override the

operand  size  set  by  LW  with  the instruction prefixes DDIR W resp.

DDIR  LW. AS  will note  the occurrence  of such  instructions and will

toggle  setting  for  the  instruction  following directly. By the way,

one  should never explicitly  use other DDIR  variants than W resp. LW,

as  AS will introduce them automatically  when an operand is discovered

that  is too  long. Explicit  usage might  puzzle AS. The automatism is

so powerful that in a case like this:

DDIR    LW

LD      BC,12345678h   ,

the  necessary IW prefix  will automatically be  merged into the previ-

ous instruction, resulting in

DDIR    LW,IW

LD      BC,12345668h   .

The  machine code that was  first created for DDIR  LW is retracted and

replaced, which is signified with an R in the listing.

4.32. Z8, Super8, and eZ8

-------------------------

The  CPU core  contained in  the Z8  microcontrollers does  not contain

any  specific registers.  Instead, a  block of  16 consecutive cells of

the  internal address  space (contains  RAM and  I/O registers)  may be

used  as 'work  registers' and  be addressed  with 4-bit addresses. The

RP  registers define which  memory block is  used as work registers: on

a  classic Z8, bits 4 to  7 of RP define the  'offset' that is added to

a  4-bit work register address to get a complete 8-bit address. The Su-

per8  core features  two register  pointers (RP0  and RP1), which allow

mapping  the  lower  and  upper  half  of  work  registers  to separate

places.

Usually,  one refers  to work  registers as  R0..R15 in assembly state-

ments.  It is however also posssible to regard work registers as an ef-

ficient way to address a block of memory addresses in internal RAM.

The  ASSUME statement is used  to inform AS about  the current value of

RP.  AS is then  capable to automatically  decide whether an address in

internal  RAM may be  reached with a  4-bit or 8-bit  address. This may

be used to assign symbolic names to work registers:

op1     equ     040h

op2     equ     041h

srp     #040h

assume  rp:040h

ld      op1,op2         ; equal to ld r0,r1

Note  that though  the Super8  does not  have an  RP register (only RP0

and  RP1), RP as argument to ASSUME is  still allowed - it will set the

assumed  values of RP0 and  RP1 to value resp.  value+8, as the SRP ma-

chine instruction does on the Super 8 core.

Opposed  to the  original Zilog  assembler, it  is not necessary to ex-

plicitly  specify 'work  register addressing'  with a prefixed exclama-

tion  mark. AS however also understands this syntax - a prefixed excla-

mation  mark enforces 4-bit addressing, even  when the address does not

lie  within the 16-address block  defined by RP (AS  will issue a warn-

ing  in that case).  Vice versa, a  prefixed > character enforces 8-bit

addressing  even  when  the  address  is  within the current 16-address

block.

The  eZ8  takes  this  'game'  to  the next level: the internal address

space  now has 12 instead  of 8 bits. To  assure compatibility with the

old  Z8 core,  Zilog placed  the additional  4 bits  in the  lower four

bits  of RP. For instance,  an RP value of  12h defines an address win-

dow from 210h to 21fh.

At  the same time, the lower four bits of RP define a window of 256 ad-

dresses  that can be  addressed with 8-bit  addresses. The mechanism to

automatically  select  between  8-  and  12-bit addresses is analogous.

'Long'  12-bit addresses  may be  enforced by  prefixing two  > charac-

ters.

4.33. Z8000

-----------

A Z8001/8003 may be operated in one of two modes:

-  Non-Segmented:  The memory address space  is limited to 64 KBytes,

and  all addresses  are 'simple'  linear 16  bit addresses. Address

registers  are single 16 bit  registers (Rn), and absolute address-

es within instructions are one byte long.

-  Segmented:  Memory is structured into up  to 128 segments of up to

64  KBytes size. Addresses consist of a  7 bit segment number and a

16  bit  offset.  Address  registers  are register pairs (RRn). Ab-

solute  addresses in instructions  occupy two 16  bit words, unless

the offset is smaller than 256.

The  operation mode (segmented  or non-segmented) therefore  has an in-

fluence  on the generated  code and is  selected implicitly via the se-

lected  processor type. For instance, if the  target is a Z8001 in non-

segmented mode, use Z8002 as target.

However,  similar to the  8086, there is  no 'real' support  for a seg-

mented  memory model  in AS.  In segmented  mode, the segment number is

simply  interpreted as the  upper seven bits  of a virtually linear ad-

dress  space. Though  this is  not what  Zilog intended,  it is the way

the segment number was used on the Z8001 if the system had no MMU.

AS  in general  implements the  Z8000 machine  instruction syntax as it

is  specified by  Zilog in  its manuals.  However, there are assemblers

that  support extensions or  variations of the  syntax. AS implements a

few of them as well:

4.33.1.  Conditions

- - - - - - - - - -

In  addition to the conditions defined by Zilog, the following alterna-

tive names are defined:

+-----------+-------+---------------------------+

| Alternate | Zilog | Meaning                   |

+-----------+-------+---------------------------+

+-----------+-------+---------------------------+

| ZR        | Z     | Z = 1                     |

| CY        | C     | C = 1                     |

| LLE       | ULE   | (C OR Z) = 1              |

| LGE       | UGE   | C = 0                     |

| LGT       | UGT   | ((C = 0) AND (Z = 0)) = 1 |

| LLT       | ULT   | C = 1                     |

+-----------+-------+---------------------------+

4.33.2. Flags

- - - - - - -

SETFLG,  COMFLG und RESFLG accept the  following alternate names as ar-

guments:

+-----------+-------+------------+

| Alternate | Zilog | Meaning    |

+-----------+-------+------------+

+-----------+-------+------------+

| ZR        | Z     | Zero Flag  |

| CY        | C     | Carry Flag |

+-----------+-------+------------+

4.33.3. Indirect Addressing

- - - - - - - - - - - - - -

It  is valid  to write  Rn^ instead  of @Rn,  if the option AMDSyntax=1

was  given to the CPU  statement. If an I/O  address is addressed indi-

rectly, this option even allows to write just Rn.

4.33.4.  Direct versus Immediate Addressing

- - - - - - - - - - - - - - - - - - - - - -

The  Zilog syntax mandates that immediate  addressing has to be done by

prefixing  the argument with a hash  character. However, if the AMDSyn-

tax=1  option  was  given  to  the  CPU statement, the type of argument

(label  or constant) decides whether  immediate or direct addressing is

to  be used. Immediate addressing may  be forced by prefixing the argu-

ment  with a  circumflex, i.e.  to load  the address  of a label into a

register.

4.34. TLCS-900(L)

-----------------

These  processors may run in  two operating modes: on  the one hand, in

minimum  mode, which  offers almost  complete source code compatibility

to  the Z80 and TLCS-90,  and on the other  hand in maximum mode, which

is  necessary to  make full  use of  the processor's  capabilities. The

main differences between these two modes are:

- width of the registers WA, BC, DE, and HL: 16 or 32 bits;

- number of register banks: 8 or 4;

- code address space: 64 Kbytes or 16 Mbytes;

- length of return addresses: 16 or 32 bits.

To  allow AS  to check  against the  correct limits,  one has to inform

him  about the current execution mode  via the MAXMODE instruction (see

there). The default is the minimum mode.

From  this follows  that, depending  on the  operating mode, the 16-bit

resp.  32-bit versions of  the bank registers  have to be  used for ad-

dressing,  i.e. WA, BC, DE and HL  for the minimum mode resp. XWA, XBC,

XDE  and XHL for the  maximum mode. The registers  XIX..XIZ and XSP are

always  32 bits wide  and therefore always  have to to  be used in this

form  for addressing; in this detail,  existing Z80 code definitely has

to  be adapted (not  including that there  is no I/O  space and all I/O

registers are memory-mapped...).

Absolute   addresses  and  displacements  may  be  coded  in  different

lengths.  Without  an  explicit  specification,  AS will always use the

shortest  possible coding.  This includes  eliminating a zero displace-

ment,  i.e. (XIX+0)  becomes (XIX).  If a  certain length is needed, it

may  be  forced  by  appending  a  suffix  (:8,  :16,  :24) to the dis-

placmenet resp. the address.

The  syntax chosen by  Toshiba is a  bit unfortunate in  the respect of

choosing  an single quote (') to  reference the previous register bank.

The  processor independent  parts of  AS already  use this character to

mark character constants. In an instruction like

ld      wa',wa   ,

AS  will not recognize  the comma for  parameter separation. This prob-

lem  can be circumvented by  usage of an inverse  single quote (`), for

example

ld      wa`,wa

Toshiba  delivers an  own assembler  for the  TLCS-900 series (TAS900),

which is different from AS in the following points:

Symbol Conventions

..................

- TAS900  differentiates symbol  names only  on the  first 32 charac-

ters.  In contrast,  AS always  stores symbol  names with  the full

length  (up to 255  characters) and uses  them all for differentia-

tion.

- TAS900  allows to write integer constants  either in Intel or C no-

tation  (with a 0 prefix  for octal or a  0x prefix for hexadecimal

constants).  By default, AS only  supports the Intel notation. With

the  help of the RELAXED instruction,  one also gets the C notation

(among other).

- AS  does  not  distinguish  between  upper  and lower case. In con-

trast,  TAS900 differentiates between  upper- and lowercase letters

in  symbol names.  One needs  to engage  the -u command line option

to force AS to do this.

Syntax

......

For  many instructions, the  syntax checking of  AS is less strict than

the  checking of TAS900.  In some (rare)  cases, the syntax is slightly

different.  These extensions  and changes  are on  the one hand for the

sake  of a better portability of existing  Z80 codes, on the other hand

they  provide a simplification  and better orthogonality  of the assem-

bly syntax:

- In  the case of  LDA, JP, and  CALL, TAS requires  that address ex-

pressions  like XIX+5 must  not be placed  in parentheses, as it is

usually  the case.  For the  sake of  better orthogonality,  AS re-

quires  parentheses for  LDA. They  are optional  if JP  resp. CALL

are used with a simple, absolute address.

- In  the case of JP, CALL, JR, and  SCC, AS leaves the choice to the

programmer  whether to  explicitly write  out the default condition

T  (= true) as first parameter or  not. TAS900 in contrast only al-

lows  to use the default condition  implicitly (e.g. jp (xix+5) in-

stead of jp t,(xix+5)).

- For  the EX instruction,  AS allows operand  combinations which are

not  listed in [166]  but can be  reduced to a standard combination

by   swapping  the  operands.  Combinations  like  EX  f`,f  or  EX

wa,(xhl)  become  possible.  In  contrast,  TAS900  limits  to  the

'pure' combinations.

- AS  allows to  omit an  increment resp.  decrement of  1 when using

the  instructions INC and  DEC. TAS900 instead  forces the program-

mer to explicit usage of '1'.

- The  similar is true for the  shift instructions: If the operand is

a  register, TAS900 requires  that even a  shift count of  1 has to

be  written  explicitly;  however,  when  the operand is in memory,

the  hardware limits the shift  count to 1 which  must not be writ-

ten  in this case. With  AS, a shift count  of 1 is always optional

and valid for all types of operands.

Macro Processor

...............

The  macro processor  of TAS900  is an  external program  that operates

like  a preprocessor. It  consists of two  components: The first one is

a  C-like preprocessor, and the second  one is a special macro language

(MPL)  that reminds of high level  languages. The macro processor of AS

instead  is oriented towards ''classic''  macro assemblers like MASM or

M80 (both programs from Microsoft). It is a fixed component of AS.

Output Format

.............

TAS900  generates relocatable code that  allows to link separately com-

piled  programs to a single  application. AS instead generates absolute

machine  code that is not linkable. There are currently no plans to ex-

tend AS in this respect.

Pseudo Instructions

...................

Due  to the  missing linker,  AS lacks  a couple of pseudo instructions

needed  for relocatable code TAS900  implements. The following instruc-

tions are available with equal meaning:

EQU, DB, DW, ORG, ALIGN, END, TITLE, SAVE, RESTORE

The  latter  two  have  an  extended  functionality for AS. Some TAS900

pseudo  instructions can  be replaced  with equivalent  AS instructions

(see table 4.6).

+--------------+---------------------+-----------------------------+

| TAS900       | AS                  | meaning/function            |

+--------------+---------------------+-----------------------------+

+--------------+---------------------+-----------------------------+

| DL <Data>    | DD <Data>           | define longword constants   |

+--------------+---------------------+-----------------------------+

| DSB <number> | DB <number> DUP (?) | reserve bytes of memory     |

+--------------+---------------------+-----------------------------+

| DSW <number> | DW <number> DUP (?) | reserve words of memory     |

+--------------+---------------------+-----------------------------+

| DSD <number> | DD <number> DUP (?) | reserve longwords of memory |

+--------------+---------------------+-----------------------------+

| $MIN[IMUM]   | MAXMODE OFF         | following code runs         |

|              |                     | in minimum mode             |

+--------------+---------------------+-----------------------------+

| $MAX[IMUM]   | MAXMODE ON          | following code runs         |

|              |                     | in maximum mode             |

+--------------+---------------------+-----------------------------+

| $SYS[TEM]    | SUPMODE ON          | following code runs         |

|              |                     | in system mode              |

+--------------+---------------------+-----------------------------+

| $NOR[MAL]    | SUPMODE OFF         | following code runs         |

|              |                     | in user mode                |

+--------------+---------------------+-----------------------------+

| $NOLIST      | LISTING OFF         | turn off assembly listing   |

+--------------+---------------------+-----------------------------+

| $LIST        | LISTING ON          | turn on assembly listing    |

+--------------+---------------------+-----------------------------+

| $EJECT       | NEWPAGE             | start new page in listing   |

+--------------+---------------------+-----------------------------+

Table 4.6: equivalent instructions TAS900<->AS

Toshiba  manufactures two  versions of  the processor  core, with the L

version  being an ''economy version''. AS  will make the following dif-

ferences between TLCS-900 and TLCS-900L:

- The  instructions MAX and NORMAL are not allowed for the L version;

the MIN instruction is disabled for the full version.

- The  L version  does not  know the  normal stack  pointer XNSP/NSP,

but instead has the interrupt nesting register INTNEST.

The  instructions  SUPMODE  and  MAXMODE  are  not  influenced, just as

their  initial  setting  OFF.  The  programmer  has to take care of the

fact  that the  L version  starts in  maximum mode  and does not have a

normal  mode. However, AS  shows a bit  of mercy against  the L variant

by suppressing warnings for privileged instructions.

4.35. TLCS-90

-------------

Maybe  some people might ask themselves if  I mixed up the order a lit-

tle  bit, as Toshiba first released the  TLCS-90 as an extended Z80 and

afterwards  the  16-bit  version  TLCS-900.  Well, I discovered the '90

via  the '900 (thank you Oliver!).  The two families are quite similar,

not  only regarding  their syntax  but also  in their architecture. The

hints  for the  '90 are  therefore a  subset of  of the chapter for the

'900:  As the  '90 only  allows shifts,  increments, and  decrements by

one,  the count  need not  and must  not be  written as the first argu-

ment.  Once  again,  Toshiba  wants  to  omit  parentheses  for  memory

operands  of LDA,  JP, and  CALL, and  once again  AS requires them for

the  sake of  orthogonality (the  exact reason  is of  course that this

way,  I saved  an extra  in the  address parser,  but one  does not say

such a thing aloud).

Principally,  the  TLCS-90  series  already  has  an address space of 1

Mbyte  which is  however only  accessible as  data space  via the index

registers.  AS therefore does not regard  the bank registers and limits

the  address space  to 64  Kbytes. This  should not  limit too  much as

this area above is anyway only reachable via indirect addressing.

4.36. TLCS-870

--------------

Once  again Toshiba...a company  quite productive at  the moment! Espe-

cially  this  branch  of  the  family (all Toshiba microcontrollers are

quite  similar in their  binary coding and  programming model) seems to

be  targeted towards the 8051 market:  the method of separating the bit

position  from the address  expression with a  dot had its  root in the

8051.  However, it creates  now exactly the  sort of problems I antici-

pated  when working on the 8051 part: On the one hand, the dot is a le-

gal  part of symbol names, but on the other hand, it is part of the ad-

dress  syntax. This means that AS has to separate address and bit posi-

tion  and  must  process  them  independently. Currently, I solved this

conflict  by seeking  the dot  starting at  the end  of the expression.

This  way, the last dot is regarded  as the separator, and further dots

stay  parts of the  address. I continue  to urge everyone  to omit dots

in symbol names, they will lead to ambiguities:

LD      CF,A.7  ; accumulator bit 7 to carry

LD      C,A.7   ; constant 'A.7' to accumulator

4.37. TLCS-47

-------------

This  family of 4-bit microcontrollers should  mark the low end of what

is  supportable by AS.  Apart from the  ASSUME instruction for the data

bank  register (see there), there is only  one thing that is worth men-

tioning:  In the data and I/O  segment, nibbles are reserved instead of

byte  (it's a  4-bitter...). The  situation is  similar to the bit data

segment  of the 8051, where  a DB reserves a  single bit, with the dif-

ference that we are dealing with nibbles.

Toshiba  defined  an  ''extended  instruction  set'' for this processor

family  to facilitate the  work with their  limited instruction set. In

the  case of AS, it is defined in the include file STDDEF47.INC. Howev-

er,  some  instructions  that  could  not  be  realized  as  macros are

''builtins''  and  are  therefore  also  available  without the include

file:

- the  B instruction  that automatically  chooses the optimal version

of the jump instruction (BSS; BS, or BSL);

- LD in the variant of HL with an immediate operand;

- ROLC and RORC with a shift amplitude higher than one.

4.38. TLCS-9000

---------------

This  was the first  time that I  implemented a processor  for AS which

was  not yet available at that point  of time. And unfortunately, I re-

ceived  back  then  information  that  Toshiba  had decided no to maket

this  processor at all.  This of course  had the result  that the TLCS-

9000 part of the assembler

1 was  a ''paper design'',  i.e. there was  so far no  chance to test

it on real hardware and

2 the  documentation for the '9000 I could get hold of [169] was pre-

liminary and was unclear in a couple of detail issues.

So i effect, this target went into 'dormant mode'...

...cut,  20 years have  passed: all of  a sudden, people are contacting

me  and tell me that Toshiba actually  did sell TLCS-9000 chips to cus-

tomers,  and  they  ask  for  documentation  to do reverse engineering.

Maybe  this will shed  some light on  the remaining unclarities. Never-

theless,  errors in  this code  generator are  quite possible (and will

of  course be fixed!).  At least the  few examples listed  in [169] are

assembled correctly.

Displacements  included in  machine instructions  may only  have a cer-

tain  maximum length (e.g. 9  or 13 bits). In  case the displacement is

longer,  a prefix containing the 'upper  bits' must be prepended to the

instruction.  AS will  automatically insert  such prefixes  when neces-

sary,  however  it  is  also  possible  to  force  usage of a prefix by

adding a leading '>'. An example for this:

ld:g.b  (0h),0       ; no prefix

ld:g.b  (400000h),0  ; prefix added automatically

ld:g.b  (>0h),0      ; forced prefix

4.39. TC9331

------------

Toshiba  supplied a (DOS-based) assembler  for this processor which was

named  ASM31T.  This  assembler  supports  a  number of syntax elements

which  could not  be mapped  on the  capabilities of AS without risking

incompatibilities  for  existing  source  files  for other targets. The

following   issues  might  require  changes  on  programs  written  for

ASM31T:

- ASM31T  supports C-like  comments (/*  ... */)  which may also span

multiple  lines. Such comments are not  supported by AS and have to

be replaced by comments beginning with a semicolon.

- Similar  to ASM31T, AS supports  comments with round parentheses ((

...  )),  however  only  within  a  single command argument. Should

such  a comment  contain a  comma, this  comma will be treated like

an  argument separator  and the  comment will  not be  skipped when

parsing the arguments.

- ASM31T  allows symbol  and label  names containing  a dash. AS does

not  allow this,  because the  dash is  regarded to be the subtrac-

tion  operator. It would be unclear whether an expression like end-

start  represents a  single symbol  or the  difference of  two sym-

bols.

- ASM31T  requires an END statement as the last statement of the pro-

gram; this is optional for AS.

Furthermore,  AS currently  lacks the  capabilities to detect conflict-

ing  uses of functional units in a machine instructions. Toshiba's doc-

umentation is a bit difficult to understand in this respect...

4.40. 29xxx

-----------

As  it was already  described in the  discussion of the ASSUME instruc-

tion,  AS can use the information about  the current setting of the RBP

register  to  detect  accesses  to  privileged  registers in user mode.

This  ability is of course limited to direct accesses (i.e. without us-

ing  the registers IPA...IPC), and there  is one more pitfall: as local

registers  (registers  with  a  number  >127) are addressed relative to

the  stack pointer, but the  bits in RBP always  refer to absolute num-

bers,  the check  is NOT  done for  local registers. An extension would

require  AS to  know always  the absolute  value of  SP, which would at

least fail for recursive subroutines...

4.41. 80C16x

------------

As  it was already  explained in the  discussion of the ASSUME instruc-

tion,  AS tries to hide  the fact that the  processor has more physical

than  logical RAM as far as possible.  Please keep in mind that the DPP

registers  are valid only for data  accesses and only have an influence

on  absolute addressing, neither on  indirect nor on indexed addresses.

AS  cannot  know  which  value  the  computed  address may take at run-

time...  The paging  unit unfortunately  does not  operate for code ac-

cesses  so one has to work with  explicit long or short CALLs, JMPs, or

RETs.  At least  for the  ''universal'' instructions  CALL and  JMP, AS

will  automatically use the shortest variant,  but at least for the RET

one  should know where  the call came  from. JMPS and CALLS principally

require  to write segment and address  separately, but AS is written in

a way that it can split an address on its own, e.g. one can write

jmps    12345h

instead of

jmps    1,2345h

Unfortunately,  not  all  details  of  the  chip's internal instruction

pipeline  are hidden:  if CP  (register bank  address), SP  (stack), or

one  of the  paging registers  are modified,  their value is not avail-

able  for  the  instruction  immediately  following. AS tries to detect

such  situations and  will issue  a warning  in such cases. Once again,

this mechanism only works for direct accesses.

Bits  defined with the  BIT instruction are  internally stored as a 12-

bit  word, containing  the address  in bits  4..11 and the bit position

in  the four LSBs. This  order allows to refer  the next resp. previous

bit  by  incrementing  or  decrementing  the address. This will however

not  work  for  explicit  bit  specifications  when  a word boundary is

crossed.  For example, the following expression  will result in a range

check error:

bclr    r5.15+1

We need a BIT in this situation:

msb     bit     r5.15

.

.

bclr    msb+1

The  SFR area was doubled  for the 80C167/165/163: bit  12 flags that a

bit  lies in  the second  part. Siemens  unfortunately did  not foresee

that  256 SFRs (128 of them bit addressable) would not suffice for suc-

cessors  of the 80C166.  As a result,  it would be  impossible to reach

the  second SFR area from F000H..F1DFH  with short addresses or bit in-

structions if the developers had not included a toggle instruction:

EXTR    #n

This  instruction  has  the  effect  that  for  the next n instructions

(0<n<5),  it is possible to address  the alternate SFR space instead of

the  normal  one.  AS  does  not  only generate the appropriate machine

code  when it  encounters this  instruction. It  also sets  an internal

flag  that will only allow accesses to  the alternate SFR space for the

next  n  instructions.  Of  course,  they  may  not contain jumps... Of

course,  it is always possible  to define bits from  either area at any

place,  and it is always possible  to reach all registers with absolute

addresses.  In contrast,  short and  bit addressing  only works for one

area  at a time, attempts  contradicting to this will  result in an er-

ror message.

The  situation is  similar for  prefix instructions  and absolute resp.

indirect  addressing: as  the prefix  argument and  the address expres-

sion  cannot always be  evaluated at assembly  time, chances for check-

ing  are limited and AS will  limit itself to warnings...in detail, the

situation is as follows:

- fixed  specification of a 64K bank  with EXTS or EXTSR: the address

expression  directly contains the  lower 16 bits  of the target ad-

dress.  If the  prefix and  the following  instruction have  a con-

stant  operand, AS will  check if the  the prefix argument and bits

16..23 of the target address are equal.

- fixed  specification of a 16K page  with EXTP or EXTPR: the address

expression  directly contains the  lower 14 bits  of the target ad-

dress.  Bits 14  and 15  are fixed  to 0,  as the processor ignores

them  in this  mode. If  the prefix  and the  following instruction

have  a constant operand, AS will  check if the the prefix argument

and bits 14..23 of the target address are equal.

An  example to clarify things  a bit (the DPP  registers have their re-

set values):

extp    #7,#1      ; range from 112K..128K

mov     r0,1cdefh  ; results in address 0defh in code

mov     r0,1cdefh  ; -->warning

exts    #1,#1      ; range from 64K..128K

mov     r0,1cdefh  ; results in address 0cdefh in code

mov     r0,1cdefh  ; -->warning

4.42. PIC16C5x/16C8x

--------------------

Similar  to the MCS-48 family, the  PICs split their program memory in-

to  several banks because the opcode does  not offer enough space for a

complete  address.  AS  uses  the  same automatism for the instructions

CALL  and GOTO, i.e. the  PA bits in the  status word are set according

to  the start and  target address. However,  this procedure is far more

problematic compared to the 48's:

1 The  instructions  are  not  any  more  one  word long (up to three

words).  Therefore, it is  not guaranteed that  they can be skipped

with a conditional branch.

2 It  is possible  that the  program counter  crosses a page boundary

while  the program sequence is executed.  The setting of PA bits AS

assumes may be different from reality.

The  instructions that operate on register  W and another register nor-

mally  require  a  second  parameter  that specifies whether the result

shall  be stored in  W or the  register. Under AS,  it is valid to omit

the  second parameter. The assumed target  then depends upon the opera-

tion's  type: For  unary operations,  the result  is by  default stored

back into the register. These instructions are:

COMF, DECF, DECFSZ, INCF, INCFSZ, RLF, RRF, and SWAPF

The other operations by default regard W as an accumulator:

ADDWF, ANDWF, IORWF, MOVF, SUBWF, and XORWF

The  syntax defined  by Microchip  to write  literals is  quite obscure

and  reminds of the syntax used  on IBM 360/370 systems (greetings from

the  stone-age...). To avoid introducing  another branch into the pars-

er,  with  AS  one  has  to  write  constants  in  the  Motorola syntax

(optionally Intel or C in RELAXED mode).

4.43. PIC 17C4x

---------------

With  two exceptions, the same  hints are valid as  for its two smaller

brothers:  the corresponding include file only contains register defin-

itions,  and the problems concerning  jump instructions are much small-

er.  The only exception  is the LCALL  instruction, which allows a jump

with a 16-bit address. It is translated with the following ''macro'':

MOVLW   <addr15..8>

MOWF    3

LCALL   <addr0..7>

4.44. SX20/28

-------------

The  limited length of the instruction  word does not permit specifying

a  complete program memory address (11  bits) or data memory address (8

bits).  The CPU core  augments the truncated  address from the instruc-

tion  word with  the PA  bits from  the STATUS  registers, respectively

with  the upper bits of the FSR  register. It is possible to inform the

assembler  via ASSUME instructions about the contents of these two reg-

isters.  In case that addresses are  used that are inaccessible with th

current values, a warning is issued.

4.45. ST6

---------

These  processors have the ability to  map their code ROM pagewise into

the  data area. I am not keen  on repeating the whole discussion of the

ASSUME  instruction at this place, so I refer to the corresponding sec-

tion  (3.2.21)  for  an  explanation  how  to read constants out of the

code ROM without too much headache.

Some  builtin ''macros'' show up when  one analyzes the instruction set

a  bit more  in detail.  The instructions  I found  are listed in table

4.7 (there are probably even more...):

+-------------+------------+

| instruction | in reality |

+-------------+------------+

+-------------+------------+

| CLR A       | SUB A,A    |

| SLA A       | ADD A,A    |

| CLR addr    | LDI addr,0 |

| NOP         | JRZ PC+1   |

+-------------+------------+

Table 4.7: Hidden Macros in the ST62's Instruction Set

Especially  the last  case is  a bit  astonishing...unfortunately, some

instructions  are really missing. For example, there is an AND instruc-

tion  but no OR...not to speak of  an XOR. For this reason, the include

file  STDDEF62.INC contains  also some  helping macros (additionally to

register definitions).

The  original assembler  AST6 delivered  by SGS-Thomson  partially uses

different  pseudo instructions  than AS.  Apart from  the fact  that AS

does  not mark  pseudo instructions  with a  leading dot, the following

instructions are identical:

ASCII, ASCIZ, BLOCK, BYTE, END, ENDM, EQU, ERROR, MACRO,

ORG, TITLE, WARNING

Table  4.8 shows the instructions which have AS counterparts with simi-

lar function.

+----------+---------------------+------------------------------+

| AST6     | AS                  | meaning/function             |

+----------+---------------------+------------------------------+

+----------+---------------------+------------------------------+

| .DISPLAY | MESSAGE             | output message               |

+----------+---------------------+------------------------------+

| .EJECT   | NEWPAGE             | new page in assembly listing |

+----------+---------------------+------------------------------+

| .ELSE    | ELSEIF              | conditional assembly         |

+----------+---------------------+------------------------------+

| .ENDC    | ENDIF               | conditional assembly         |

+----------+---------------------+------------------------------+

| .IFC     | IF...               | conditional assembly         |

+----------+---------------------+------------------------------+

| .INPUT   | INCLUDE             | insert include file          |

+----------+---------------------+------------------------------+

| .LIST    | LISTING, MACEXP_DFT | settings for listing         |

+----------+---------------------+------------------------------+

| .PL      | PAGE                | page length of listing       |

+----------+---------------------+------------------------------+

| .ROMSIZE | CPU                 | set target processor         |

+----------+---------------------+------------------------------+

| .VERS    | VERSION (symbol)    | query version                |

+----------+---------------------+------------------------------+

| .SET     | EVAL                | redefine variables           |

+----------+---------------------+------------------------------+

Table 4.8: Equivalent Instructions AST6<->AS

4.46. ST7

---------

In  [133], the .w  postfix to signify  16-bit addresses is only defined

for  memory indirect  operands. It  is used  to mark  that a 16-bit ad-

dress  is stored  at a  zero page  address. AS additionally allows this

postfix  for absolute  addresses or  displacements of  indirect address

expressions  to force 16-bit  displacements in spite  of an 8-bit value

(0..255).

4.47. ST9

---------

The  ST9's bit  addressing capabilities  are quite  limited: except for

the  BTSET instruction,  only bits  within the  current set  of working

registers  are accessible. A bit address  is therefore of the following

style:

rn.[!]b   ,

whereby  ! means an optional  complement of a source  operand. If a bit

is  defined symbolically, the  bit's register number  is stored in bits

7..4,  the bit's position is stored in  bits 3..1 and the optional com-

plement  is kept in  bit 0. AS  distinguishes explicit and symbolic bit

addresses  by the  missing dot.  A bit's  symbolic name  therefore must

not  contain a dot, thought it would  be legal in respect to the gener-

al  symbol name conventions. It is  also valid to invert a symbolically

referred bit:

bit2    bit     r5.3

.

.

bld     r0.0,!bit2

This  opportunity also allows to undo an inversion that was done at de-

finition of the symbol.

The  include file REGST9.INC defines the  symbolic names of all on-chip

registers  and their  associated bits.  Keep however  in mind  that the

bit  definitions only work after  previously setting the working regis-

ter bank to the address of these peripheral registers!

In  contrast to the  definition file delivered  with the AST9 assembler

from  SGS-Thomson, the names of peripheral  register names are only de-

fined  as  general  registers  (R...),  not  also  as working registers

(r...).  The  reason  for  this  is  that  AS does not support register

aliases; a tribute to assembly speed.

4.48. 6804

----------

To  be  honest:  I  only  implemented  this  processor in AS to quarrel

about  SGS-Thomson's peculiar behaviour.  When I first  read the 6804's

data  book, the ''incomplete'' instruction  set and the built-in macros

immediately  reminded me  of the  ST62 series  manufactured by the same

company.  A more thorough comparison of the opcodes gave surprising in-

sights:  A 6804 opcode  can be generated  by taking the equivalent ST62

opcode  and mirroring all the  bits! So Thomson obviously  did a bit of

processor  core recycling...which would be all  right if they would not

try  to hide  this: different  peripherals, motorola  instead of Zilog-

style  syntax, and the awful detail  of not mirroring operand fields in

the  opcode (e.g. bit  fields containing displacements).  The last item

is  also the reason  that finally convinced  me to support  the 6804 in

AS.  I personally  can only  guess which  department at Thomson did the

copy...

In  contrast to  its ST62  counterpart, the  include file  for the 6804

does  not contain instruction macros  that help a bit  to deal with the

limited  machine instruction  set. This  is left  as an exercise to the

reader!

4.49. TMS3201x

--------------

It  seems that every semiconductor's ambition is to invent an own nota-

tion  for hexadecimal numbers. Texas  Instrument took an especially ec-

centric  approach for  these processors:  a >  sign as prefix! The sup-

port  of such a format in AS  would have lead to extreme conflicts with

AS's  compare and shift  operators. I therefore  decided to use the In-

tel  notation, which is what TI also  uses for the 340x0 series and the

3201x's successors...

The  instruction word of  these processors unfortunately  does not have

enough  bits to  store all  8 bits  for direct  addressing. This is why

the  data address space is split into  two banks of 128 words. AS prin-

cipally  regards  the  data  address  space  as a linear segment of 256

words  and automatically clears bit 7  on direct accesses (an exception

is  the SST  instruction that  can only  write to  the upper bank). The

programmer  has to take care that the  bank flag always has the correct

value!

Another  hint that is well  hidden in the data  book: The SUBC instruc-

tion  internally needs more than one clock for completion, but the con-

trol  unit already  continues to  execute the  next instruction. An in-

struction  following SUBC therefore may  not access the accumulator. AS

does not check for such conditions!

4.50. TMS320C2x

---------------

As  I did  not write  this code  generator myself  (that does not lower

its  quality by any  standard), I can  only roughly line  out why there

are  some instructions that force a  prefixed label to be untyped, i.e.

not  assigned to any specific address space:  The 2x series of TMS sig-

nal  processors has a code and a  data segment which are both 64 Kbytes

large.  Depending on external circuitry, code  and data space may over-

lap,  e.g. to allow  storage of constants  in the code  area and access

them  as data. Data  storage in the  code segment may  be necessary be-

cause  older versions of AS assume  that the data segment only consists

of  RAM that  cannot have  a defined  power-on state  in a single board

system.  They therefore  reject storage  of contents  in other segments

than  CODE. Without  the feature  of making  symbols untyped,  AS would

punish  every  access  to  a  constant  in  code  space  with a warning

(''symbol  out of wrong segment''). To  say it in detail, the following

instructions make labels untyped:

BSS, STRING, RSTRING, BYTE, WORD , LONG

FLOAT, DOUBLE, EFLOAT, BFLOAT and TFLOAT

If  one needs a typed label in  front of one of these instructions, one

can  work around this by placing the  label in a separate line just be-

fore  the pseudo instruction itself. On  the other hand, it is possible

to  place an  untyped label  in front  of another pseudo instruction by

defining the label with EQU, e.g.

<name>  EQU     $        .

4.51. TMS320C3x/C4x

-------------------

The  syntax detail that  created the biggest  amount of headache for me

while  implementing this processor family  is the splitting of parallel

instructions  into two  separate source  code lines.  Fortunately, both

instructions  of such a  construct are also  valid single instructions.

AS  therefore first  generates the  code for  the first instruction and

replaces  it by the parallel machine  code when a parallel construct is

encountered  in the second  line. This operation  can be noticed in the

assembly  listing by  the machine  code address  that does  not advance

and the double dot replaced with a R.

Compared  to the TI assembler, AS is  not as flexible regarding the po-

sition  of the  double lines  that signify  a parallel  operation (||):

One  either has to place them like  a label (starting in the first col-

umn)  or to prepend them to the  second mnemonic. The line parser of AS

will run into trouble if you do something else...

4.52. TMS9900

-------------

Similar  to most older TI microprocessor  families, TI used an own for-

mat  for hexadecimal and  binary constants. AS  instead favours the In-

tel syntax which is also common for newer processor designs from TI.

The  TI syntax for registers allows to  use a simple integer number be-

tween  0 and 15 instead of a real name (Rx or WRx). This has two conse-

quences:

- R0...R15  resp.  WR0..WR15  are  simple  predefined integer symbols

with  values from 0  to 15, and  the definition of register aliases

is a simple matter of EQU.

- In  contrast to several other processors,  I cannot offer the addi-

tional  AS feature that allows to  omit the character sigifying ab-

solute  addressing (a @ sign in  this case). As a missing character

would  mean register numbers  (from 0 to  15) in this  case, it was

not possible to offer the optional omission.

Furthermore,  TI sometimes uses  Rx to name  registers and WRx at other

places...currently both variants are recognized by AS.

4.53. TMS70Cxx

--------------

This  processor family  belongs to  the older  families developed by TI

and   therefore  TI's  assemblers  use  their  proprietary  syntax  for

hexadecimal  resp. binary constants  (a prefixed <  resp. ? character).

As  this format could not be realized  for AS, the Intel syntax is used

by  default. This is the format TI to which also switched over when in-

troducing  the successors, of this family,  the 370 series of microcon-

trollers.  Upon a closer inspection  of both's machine instruction set,

one  discovers that about 80% of all instruction are binary upward com-

patible,  and that also  the assembly syntax  is almost identical - but

unfortunately  only almost. TI also took  the chance to make the syntax

more  orthogonal  and  simple.  I  tried  to  introduce the majority of

these changes also into the 7000's instruction set:

- It  is valid to use  the more common #  sign for immediate address-

ing instead of the percent sign.

- If  a port  address (P...)  is used  as source  or destination in a

AND,  BTJO, BTJZ, MOV, OR, or  XOR instruction, it is not necessary

to  use the mnemonic variant with an  appended P - the general form

is sufficient.

- The  prefixed @ sign  for absolute or  B-relative addressing may be

omitted.

- Instead of CMPA, CMP with A as target may be written.

- Instead  of LDA resp.  STA, one can  simply use the MOV instruction

with A as source resp. destination.

- One can write MOVW instead of MOVD.

- It is valid to abbreviate RETS resp. RETI as RTS resp. RTI.

- TSTA resp. TSTB may be written as TST A resp. TST B.

- XCHB B is an alias for TSTB.

An  important note: these variants are  only allowed for the TMS70Cxx -

the corresponding 7000 variants are not allowed for the 370 series!

4.54. TMS370xxx

---------------

Though  these processors do  not have specialized  instructions for bit

manipulation,  the assembler  creates (with  the help  of the  DBIT in-

struction  - see  there) the  illusion as  if single bits were address-

able.  To achieve this,  the DBIT instructions  stores an address along

with  a bit position into  an integer symbol which  may then be used as

an  argument to  the pseudo  instructions SBIT0,  SBIT1, CMPBIT, JBIT0,

and  JBIT1. These  are translated  into the  instructions OR, AND, XOR,

BTJZ, and BTJO with an appropriate bit mask.

There  is nothing magic about these  bit symbols, they are simple inte-

ger  values that contain the  address in their lower  and the bit posi-

tion  in their upper half. One  could construct bit symbols without the

DBIT instruction, like this:

defbit  macro   name,bit,addr

name    equ     addr+(bit<<16)

endm

but  this technique would  not lead to  the EQU-style syntax defined by

TI  (the symbol to be defined replaces the label field in a line). CAU-

TION!  Though  DBIT  allows  an  arbitrary address, the pseudo instruc-

tions  can only operate with addresses  either in the range from 0..255

or  1000h..10ffh. The  processor does  not have  an absolute addressing

mode for other memory ranges...

4.55. MSP430(X)

---------------

The  MSP was designed to be a  RISC processor with a minimal power con-

sumption.  The  set  of  machine  instructions was therefore reduced to

the  absolute minimum (RISC  processors do not  have a microcode ROM so

every  additional  instruction  has  to  be implemented with additional

silicon  that increases  power consumption).  A number  of instructions

that  are hardwired  for other  processors are  therefore emulated with

other  instructions. Older  versions of  AS implemented  these instruc-

tions  via macros in the  file REGMSP.INC. If one  did not include this

file,  you got  error messages  for more  than half of the instructions

defined  by TI. This  has been changed  in recent versions:  as part of

adding  the 430X instruction set,  implementation of these instructions

was  moved into the assmebler's core.  REGMSP.INC now only contains ad-

dresses  of I/O registers. If you need  the old macros for some reason,

they have been moved to the file EMULMSP.INC.

Instruction  emulation also  covers some  special cases  not handled by

the original TI assembler. For instance,

rlc  @r6+

is automatically assembled as

addc @r6+,-2(r6)

4.56. TMS1000

-------------

At  last, world's first microcontroller finally  also supported in AS -

it  took long  to fill  this gap,  but now  it is done. This target has

some pitfalls that will be discussed shortly in this section.

First,  the instruction set  of these controllers  is partially defined

via  the ROM mask, i.e. the function  of some opcodes may be freely de-

fined  to some degree. AS only  knows the instructions and codings that

are  described as default codings  in [163]. If you  have a special ap-

plication  with an instruction set deviating  from this, you may define

and modify instructions via macros and the DB instruction.

Furthermore,  keep in mind that branches and subroutine calls only con-

tain  the lower 6 bits of the target  address. The upper 4 resp. 5 bits

are  fetched from page  and chapter registers  tha thave to  be set be-

forehand.  AS cannot check  whether these registers  have been set cor-

rectly  by the programmer! At least for  the cas of staying in the same

chapter,  there are  the assmebler  pseudo instructions  CALLL resp. BL

that  combine  an  LDP  and  CALL/BR instruction. Regarding the limited

amount  of program memory,  this is a  convenient yet inefficient vari-

ant.

4.57. COP8

----------

National  unfortunately also decided to use  the syntax well known from

IBM  mainframes (and much  hated by me..)  to write non-decimal integer

constants.  Just like  with other  processors, this  does not work with

AS's  parser. ASMCOP however fortunately also seems to allow the C syn-

tax,  which is why this  became the default for  the COP series and the

SC/MP...

4.58. SC/MP

-----------

If  indirect addressing with displacement is  used on the SC/MP, and if

the  base or pointer register is not  P0/PC, a displacement of -128 (80

hex)  has a special  meaning: the contents  of the E  register are used

as  displacement instead of this value. If using the 'classic NS assem-

bler',  the programmer has to know  about this, and even explicitly use

it:

ereg   equ -128

ld  ereg(p1)

This  however bears the  risk that -128  may accidentally be the result

of  a computed  displacement, and  you might  have a  hard time finding

out  why the program does not do  what was indented. I therefore decid-

ed to make this special value more explicit:

If  a displacement of -128  is used, a warning  is issued. One may sim-

ply  ignore this warning. If you want to  get rid of it, use the built-

in literal E, which explicitly references the register of same name:

ld e(p1)

Since  the SC/MP target supports register  symbols, it is also possible

to define the 'own symbol' in a proper way:

ereg   reg e

ld  ereg(p1)

This  should reduce  the amount  of necessary  changes in existing code

to a minimum.

4.59. SC144xxx

--------------

Originally,  National offered  a relatively  simple assembler  for this

series  of DECT controllers.  An much more  powerful assembler has been

announced  by IAR, but  it is not  available up to  now. However, since

the  development tools  made by  IAR are  as much target-independent as

possible,  one  can  roughly  estimate  the pseudo instructions it will

support  by looking at  other available target  platforms. With this in

mind,  the (few) SC144xx-specific instructions  DC, DC8, DW16, DS, DS8,

DS16,  DW  were  designed.  Of  course,  I  didn't want to reinvent the

wheel  for pseudo instructions  whose functionality is  already part of

the  AS core. Therefore, here is  a little table with equivalences. The

statements  ALIGN, END, ENDM,  EXITM, MACRO, ORG,  RADIX, SET, and REPT

both  exist for the  IAR assembler and  AS and have same functionality.

Changes are needed for the following instructions:

+---------------------+-----------------+---------------------------+

| IAR                 | AS              | Funktion                  |

+---------------------+-----------------+---------------------------+

+---------------------+-----------------+---------------------------+

| #include            | include         | include file              |

| #define             | SET, EQU        | define symbol             |

| #elif, ELIF, ELSEIF | ELSEIF          | start another             |

|                     |                 | IF branch                 |

| #else, ELSE         | ELSE            | last branch of an IF      |

|                     |                 | construct                 |

| #endif, ENDIF       | ENDIF           | ends an IF construct      |

| #error              | ERROR, FATAL    | create error message      |

| #if, IF             | IF              | start an IF construct     |

| #ifdef              | IFDEF           | symbol defined ?          |

| #ifndef             | IFNDEF          | symbol not defined ?      |

| #message            | MESSAGE         | output message            |

| =, DEFINE, EQU      | =, EQU          | fixed value assignment    |

| EVEN                | ALIGN 2         | force PC to be equal      |

| COL, PAGSIZ         | PAGE            | set page size for listing |

| ENDR                | ENDM            | end REPT construct        |

| LSTCND, LSTOUT      | LISTING         | control amount of listing |

| LSTEXP, LSTREP      | MACEXP          | list expanded macros?     |

| LSTXRF              |  <command line> | generate cross reference  |

| PAGE                | NEWPAGE         | new page in listing       |

| REPTC               | IRPC            | repetition with character |

|                     |                 | replacement               |

+---------------------+-----------------+---------------------------+

There  is  no  direct  equivalent  for  CASEON, CASEOFF, LOCAL, LSTPAG,

#undef, and REPTI.

A  100% equivalent is  of course impossible  as long as  there is no C-

like  preprocessor in AS. C-like comments unfortunately are also impos-

sible  at the moment. Caution: When modifying  IAR codes for AS, do not

forget  to move  converted preprocessor  statements out  of column 1 as

AS reserves this column exclusively for labels!

4.60. NS32xxx

-------------

As  one might  expect from  a CISC  processor, the  NS32xxx series pro-

vides  sophisticated and complex addressing  modes. National defied the

assembly  syntax for each of them in  its manuals, and this is also the

syntax  AS implements. However, as for every architecture that was sup-

ported  by third-party tools, there  are deviations and extensions, and

I added a few of them to AS:

The syntax to use PC-relative addressing, as defined by National, is:

movb r0,*+disp

This  of course quite  clearly expresses what  is happening at runtime,

one  however has  to compute  the distance  himself if a certain memory

location is to be addressed:

movb r0,*+(addr-*)

The  first simplification is that under  certain conditions, it is suf-

ficient to just write:

movb r0,addr

since  absolute addressierung is marked by  a @ prefix. This is allowed

under the following conditions:

- Immediate  addressierung is  not allowed,  e.g. because the operand

is the destination and there is no risk os ambiguities.

- An  index extension  is used  (appended in  square brackets), which

must not be combined with immediate addressing.

As  an  alterntative,  AS  also  supports  the following way to use PC-

relative addressing:

movb r0,addr(pc)

Analog to the 68000, the distance is computed automatically.

The external mode, whis written this way in National syntax:

movb r0,ext(disp1)+disp2

there is another supported syntax variant:

movb r0,disp2(disp1(ext))

which used to be common in UNIX environments.

4.61. uPD78(C)1x

----------------

For  relative, unconditional instructions, there  is the JR instruction

branch  distance -32...+31, one byte),  and the JRE instruction (branch

distance  -256...+255, two  bytes). AS  furthermore knows  the J pseudo

instruction,  which automatically  selects the  shortest possible vari-

ant.

Architecture  and instructon set  of these processors  are coarsely re-

lated  to the  Intel 8080/8085  - thi  is also  true for the mnemonics.

The  adressing mode  (direct, indirect,  immediate) is  packed into the

mnemonic,  and 16 bit registers (BC, DE,  HL) are written with just one

letter.  However, since  NEC itself  also uses  at some places written-

out  register names and  parentheses to signify  indirect addressing, I

decided  to support some  alternative notations next  to the 'official'

ones.  Some non-NEC  tools like  disassemblers seem  to use these nota-

tions either:

- It is allowed to use BC, (B), or (BC) instead of B.

- It is allowed to use DE, (D), or (DE) instead of D.

- It is allowed to use HL, (H), or (HL) instead of H.

- It is allowed to use DE+, (D+), (DE+), or (DE)+ instead of D+.

- It is allowed to use HL+, (H+), (HL+), or (HL)+ instead of H+.

- It is allowed to use DE-, (D-), (DE-), or (DE)- instead of D-.

- It is allowed to use HL-, (H-), (HL-), or (HL)- instead of H-.

- It  is allowed  to use  DE++, (D++),  (DE++), or  (DE)++ instead of

D++.

- It  is allowed  to use  HL++, (H++),  (HL++), or  (HL)++ instead of

H++.

- It  is allowed to use DE--, (D--), (DE--), or (DE)-- instead of D--

.

- It  is allowed to use HL--, (H--), (HL--), or (HL)-- instead of H--

.

- It  is allowed  to use  HL+A, A+H,  A+HL, (H+A),  (HL+A), (A+H), or

(A+HL) instead of H+A.

- It  is allowed  to use  HL+B, B+H,  B+HL, (H+B),  (HL+B), (B+H), or

(B+HL) instead of H+B.

- It  is allowed to use HL+EA,  EA+H, EA+HL, (H+EA), (HL+EA), (EA+H),

or (EA+HL) instead of H+EA.

Since  architecture and  instruction set  are so  ''8080-like'', it was

straightforward  to support  the Z80SYNTAX  statement, which  allows to

write  many machine instructions  in a more  intuitive and better-known

way.  However, since both the uCON87 family's architecture and instruc-

tion  set differ from the 8080 in a couple of details, it is not possi-

ble  to provide  a complete  one-to-one mapping.  Not all  original in-

structions  have  a  ''Z80  equivalent'',  and  some instructions known

from  8080 and Z80  do not exist  on the uCOM87.  It therefore does not

make sense to support Z80SYNTAX EXCLUSIVE.

The  following table lists  all instructions defined  in Z80SYNTAX mode

and their equivalents in original syntax:

Table 4.10: Instruction Variants in Z80SYNTAX Mode

+-----------------+----------------+-----------------------+-----------+

| Z80SYNTAX       | Original       | Operation             | CPUs      |

+-----------------+----------------+-----------------------+-----------+

+-----------------+----------------+-----------------------+-----------+

| LD r1,A         | MOV r1,A       | r1<-A                 | 1,2,3,4   |

|                 |                |                       | (r1!=EAx) |

|                 |                |                       | 3,4       |

|                 |                |                       | (r1=EAx)  |

| LD A,r1         | MOV A,r1       | A<-r1                 | 1,2,3,4   |

|                 |                |                       | (r1!=EAx) |

|                 |                |                       | 3,4       |

|                 |                |                       | (r1=EAx)  |

| LD sr,A         | MOV sr,A       | sr<-A                 | 1,2,3,4   |

| LD A,sr1        | MOV A,sr1      | A<-sr1                | 1,2,3,4   |

| LD r,(word)     | MOV r,word     | r<-(word)             | 1,2,3,4   |

|                 |                |                       | (r1!=V)   |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r1=V)    |

| LD (word),r     | MOV word,r     | (word)<-r             | 1,2,3,4   |

|                 |                |                       | (r1!=V)   |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r1=V)    |

| LD r,byte       | MVI r,byte     | r<-byte               | 1,2,3,4   |

|                 |                |                       | (r1!=V)   |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r1=V)    |

| LD sr2,byte     | MVI sr2,byte   | sr2<-byte             | 2,3,4     |

| LD (wa),byte    | MVIW wa,byte   | (wa)<-byte            | 2,3,4     |

| LD (rpa1),byte  | MVIX rpa1,byte | (rpa1)<-byte          | 2,3,4     |

| LD (wa),a       | STAW wa        | (wa)<-A               | 1,2,3,4   |

| LD A,(wa)       | LDAW wa        | A<-(wa)               | 1,2,3,4   |

| LD (rpa),a      | STAX rpa       | (rpa)<-(wa)           | 1,2       |

| LD (rpa2),a     | STAX rpa2      | (rpa2)<-(wa)          | 3,4       |

| LD A,(rpa)      | LDAX rpa       | (wa)<-(rpa)           | 1,2       |

| LD A,(rpa2)     | LDAX rpa2      | (wa)<-(rpa2)          | 3,4       |

| LD rp3,EA       | DMOV rp3,EA    | rp3<-EA               | 3,4       |

| LD sr3,EA       | DMOV sr3,EA    | sr3<-EA               | 3,4       |

| LD EA,sr4       | DMOV EA,sr4    | EA<-sr4               | 3,4       |

| LD EA,rp3       | DMOV EA,rp3    | EA<-rp3               | 3,4       |

| LD (word),BC    | SBCD word      | (word)<-BC            | 1,2,3,4   |

| LD (word),DE    | SDED word      | (word)<-DE            | 1,2,3,4   |

| LD (word),HL    | SHLD word      | (word)<-HL            | 1,2,3,4   |

| LD (word),SP    | SSPD word      | (word)<-SP            | 1,2,3,4   |

| LD (rpa3),EA    | STEAX rpa3     | (rpa3)<-EA            | 3,4       |

| LD BC,(word)    | LBCD word      | BC<-(word)            | 1,2,3,4   |

| LD DE,(word)    | LDED word      | DE<-(word)            | 1,2,3,4   |

| LD HL,(word)    | LHLD word      | HL<-(word)            | 1,2,3,4   |

| LD SP,(word)    | LSPD word      | SP<-(word)            | 1,2,3,4   |

| LD EA,(rpa3)    | LDEAX rpa3     | EA<-(word)            | 3,4       |

| LD rp,word      | LXI rp,word    | rp<-word              | 1,2,3,4   |

| LD EA,word      | LXI EA,word    | EA<-word              | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| ADD A,(rpa)     | ADDX rpa       | A<-A+(rpa)            | 1,2,3,4   |

| ADD A,byte      | ADI A,byte     | A<-A+byte             | 1,2,3,4   |

| ADD r,byte      | ADI r,byte     | r<-r+byte             | 2,3,4     |

| ADD sr2,byte    | ADI sr2,byte   | sr2<-A+sr2            | 2,3,4     |

| ADD A,(wa)      | ADDW wa        | A<-A+(wa)             | 2,3,4     |

| ADD EA,r2       | EADD EA,r2     | EA<-EA+r2             | 3,4       |

| ADD EA,rp3      | DADD EA,rp3    | EA<-EA+rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| ADC A,(rpa)     | ADCX rpa       | A<-A+(rpa)+CY         | 1,2,3,4   |

| ADC A,byte      | ACI A,byte     | A<-A+byte+CY          | 1,2,3,4   |

| ADC r,byte      | ACI r,byte     | r<-r+A+CY             | 2,3,4     |

| ADC sr2,byte    | ACI sr2,byte   | sr2<-A+sr2+CY         | 2,3,4     |

| ADC A,(wa)      | ADCW wa        | A<-A+(wa)+CY          | 2,3,4     |

| ADC EA,rp3      | DADC EA,rp3    | EA<-EA+rp3+CY         | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| ADDNC A,(rpa)   | ADDNCX rpa     | A<-A+(rpa)            | 1,2,3,4   |

|                 |                | skip if !CY           |           |

| ADDNC A,byte    | ADINC A,byte   | A<-A+byte             | 1,2,3,4   |

|                 |                | skip if !CY           |           |

| ADDNC r,byte    | ADINC r,byte   | r<-r+A                | 2,3,4     |

|                 |                | skip if !CY           |           |

| ADDNC sr2,byte  | ADINC sr2,byte | sr2<-A+sr2            | 2,3,4     |

|                 |                | skip if !CY           |           |

| ADDNC A,(wa)    | ADDNCW wa      | A<-A+(wa)             | 2,3,4     |

|                 |                | skip if !CY           |           |

| ADDNC EA,rp3    | DADDNC EA,rp3  | EA<-EA+rp3            | 3, 4      |

|                 |                | skip if !CY           |           |

+-----------------+----------------+-----------------------+-----------+

| SUB A,(rpa)     | SUBX rpa       | A<-A-(rpa)            | 1,2,3,4   |

| SUB A,byte      | SUI A,byte     | A<-A-byte             | 1,2,3,4   |

| SUB r,byte      | SUI r,byte     | r<-r-A                | 2,3,4     |

| SUB sr2,byte    | SUI sr2,byte   | sr2<-A-sr2            | 2,3,4     |

| SUB A,(wa)      | SUBW wa        | A<-A-(wa)             | 2,3,4     |

| SUB EA,r2       | ESUB EA,r2     | EA<-EA-r2             | 3,4       |

| SUB EA,rp3      | DSUB EA,rp3    | EA<-EA-rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SBB A,(rpa)     | SBBX rpa       | A<-A-(rpa)-CY         | 1,2,3,4   |

| SBB A,byte      | SBI A,byte     | A<-A-byte-CY          | 1,2,3,4   |

| SBB r,byte      | SBI r,byte     | r<-r-A-CY             | 2,3,4     |

| SBB sr2,byte    | SBI sr2,byte   | sr2<-A-sr2-CY         | 2,3,4     |

| SBB A,(wa)      | SBBW wa        | A<-A-(wa)-CY          | 2,3,4     |

| SBB EA,rp3      | DSBB EA,rp3    | EA<-EA-rp3-CY         | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SUBNB A,(rpa)   | SUBNBX rpa     | A<-A-(rpa)            | 1,2,3,4   |

|                 |                | skip if !CY           |           |

| SUBNB A,byte    | SUINB A,byte   | A<-A-byte             | 1,2,3,4   |

|                 |                | skip if !CY           |           |

| SUBNB r,byte    | SUINB r,byte   | r<-r-A                | 2,3,4     |

|                 |                | skip if !CY           |           |

| SUBNB sr2,byte  | SUINB sr2,byte | sr2<-A-sr2            | 2,3,4     |

|                 |                | skip if !CY           |           |

| SUBNB A,(wa)    | SUBNBW wa      | A<-A-(wa)             | 2,3,4     |

|                 |                | skip if !CY           |           |

| SUBNB EA,rp3    | DSUBNB EA,rp3  | EA<-EA-rp3            | 3,4       |

|                 |                | skip if !CY           |           |

+-----------------+----------------+-----------------------+-----------+

| AND A,r         | ANA A,r        | A<-A&r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| AND r,A         | ANA r,A        | r<-A&r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| AND A,(rpa)     | ANAX rpa       | A<-A&(rpa)            | 1,2,3,4   |

| AND A,byte      | ANI A,byte     | A<-A&byte             | 1,2,3,4   |

| AND r,byte      | ANI r,byte     | r<-r&byte             | 2,3,4     |

| AND sr2,byte    | ANI sr2,byte   | sr2<-sr2&byte         | 1,2,3,4   |

| AND A,(wa)      | ANAW wa        | A<-A&(wa)             | 2,3,4     |

| AND (wa),byte   | ANIW wa,byte   | (wa)<-(wa)&byte       | 1,2,3,4   |

| AND EA,rp3      | DAN EA,rp3     | EA<-EA&rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| OR A,r          | ORA A,r        | A<-A|r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| OR r,A          | ORA r,A        | r<-A|r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| OR A,(rpa)      | ORAX rpa       | A<-A|(rpa)            | 1,2,3,4   |

| OR A,byte       | ORI A,byte     | A<-A|byte             | 1,2,3,4   |

| OR r,byte       | ORI r,byte     | r<-r|byte             | 2,3,4     |

| OR sr2,byte     | ORI sr2,byte   | sr2<-sr2|byte         | 1,2,3,4   |

| OR A,(wa)       | ORAW wa        | A<-A|(wa)             | 2,3,4     |

| OR (wa),byte    | ORIW wa,byte   | (wa)<-(wa)|byte       | 1,2,3,4   |

| OR EA,rp3       | DOR EA,rp3     | EA<-EA|rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| XOR A,r         | XRA A,r        | A<-A^r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| XOR r,A         | XRA r,A        | r<-A^r                | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| XOR A,(rpa)     | XRAX rpa       | A<-A^(rpa)            | 1,2,3,4   |

| XOR A,byte      | XRI A,byte     | A<-A^byte             | 2,3,4     |

| XOR r,byte      | XRI r,byte     | r<-r^byte             | 2,3,4     |

| XOR sr2,byte    | XRI sr2,byte   | sr2<-sr2^byte         | 2,3,4     |

| XOR A,(wa)      | XRAW wa        | A<-A^(wa)             | 2,3,4     |

| XOR EA,rp3      | DXR EA,rp3     | EA<-EA^rp3            | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKGT A,r        | GTA A,r        | skip if A>r           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKGT r,A        | GTA r,A        | skip if r>A           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKGT A,(rpa)    | GTAX rpa       | skip if A>(rpa)       | 1,2,3,4   |

| SKGT A,byte     | GTI A,byte     | skip if A>byte        | 1,2,3,4   |

| SKGT r,byte     | GTI r,byte     | skip if r>byte        | 2,3,4     |

| SKGT sr2,byte   | GTI sr2,byte   | skip if sr2>byte      | 2,3,4     |

| SKGT A,(wa)     | GTAW wa        | skip if A>(wa)        | 2,3,4     |

| SKGT (wa),byte  | GTIW wa,byte   | skip if (wa)>byte     | 1,2,3,4   |

| SKGT EA,rp3     | DGT EA,rp3     | skip if EA>rp3        | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKLT A,r        | LTA A,r        | skip if A<r           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKLT r,A        | LTA r,A        | skip if r<A           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKLT A,(rpa)    | LTAX rpa       | skip if A<(rpa)       | 1,2,3,4   |

| SKLT A,byte     | LTI A,byte     | skip if A<byte        | 1,2,3,4   |

| SKLT r,byte     | LTI r,byte     | skip if r<byte        | 2,3,4     |

| SKLT sr2,byte   | LTI sr2,byte   | skip if sr2<byte      | 2,3,4     |

| SKLT A,(wa)     | LTAW wa        | skip if A<(wa)        | 2,3,4     |

| SKLT (wa),byte  | LTIW wa,byte   | skip if (wa)<byte     | 1,2,3,4   |

| SKLT EA,rp3     | DLT EA,rp3     | skip if EA<rp3        | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKNE A,r        | NEA A,r        | skip if A!=r          | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKNE r,A        | NEA r,A        | skip if r!=A          | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKNE A,(rpa)    | NEAX rpa       | skip if A!=(rpa)      | 1,2,3,4   |

| SKNE A,byte     | NEI A,byte     | skip if A!=byte       | 1,2,3,4   |

| SKNE r,byte     | NEI r,byte     | skip if r!=byte       | 2,3,4     |

| SKNE sr2,byte   | NEI sr2,byte   | skip if sr2!=byte     | 2,3,4     |

| SKNE A,(wa)     | NEAW wa        | skip if A!=(wa)       | 2,3,4     |

| SKNE (wa),byte  | NEIW wa,byte   | skip if (wa)!=byte    | 1,2,3,4   |

| SKNE EA,rp3     | DNE EA,rp3     | skip if EA!=rp3       | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKEQ A,r        | EQA A,r        | skip if A=r           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKEQ r,A        | EQA r,A        | skip if r=A           | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKEQ A,(rpa)    | EQAX rpa       | skip if A=(rpa)       | 1,2,3,4   |

| SKEQ A,byte     | EQI A,byte     | skip if A=byte        | 1,2,3,4   |

| SKEQ r,byte     | EQI r,byte     | skip if r=byte        | 2,3,4     |

| SKEQ sr2,byte   | EQI sr2,byte   | skip if sr2=byte      | 2,3,4     |

| SKEQ A,(wa)     | EQAW wa        | skip if A=(wa)        | 2,3,4     |

| SKEQ (wa),byte  | EQIW wa,byte   | skip if (wa)=byte     | 1,2,3,4   |

| SKEQ EA,rp3     | DEQ EA,rp3     | skip if AEA=rp3       | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKON A,r        | ONA A,r        | skip if (A&r)!=0      | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKON r,A        | ONA r,A        | skip if (r&A)!=0      | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKON A,(rpa)    | ONAX rpa       | skip if (A&(rpa))!=0  | 1,2,3,4   |

| SKON A,byte     | ONI A,byte     | skip if (A&byte)!=0   | 1,2,3,4   |

| SKON r,byte     | ONI r,byte     | skip if (r&byte)!=0   | 2,3,4     |

| SKON sr2,byte   | ONI sr2,byte   | skip if (sr2&byte)!=0 | 2,3,4     |

| SKON A,(wa)     | ONAW wa        | skip if (A&(wa))!=0   | 2,3,4     |

| SKON (wa),byte  | ONIW wa,byte   | skip if (wa)&byte!=0  | 1,2,3,4   |

| SKON EA,rp3     | DON EA,rp3     | skip if (EA&rp3)!=0   | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| SKOFF A,r       | OFFA A,r       | skip if (A&r)=0       | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKOFF r,A       | OFFA r,A       | skip if (r&A)=0       | 1,2,3,4   |

|                 |                |                       | (r!=V)    |

|                 |                |                       | 2,3,4     |

|                 |                |                       | (r=V)     |

| SKOFF A,(rpa)   | OFFAX rpa      | skip if (A&(rpa))=0   | 1,2,3,4   |

| SKOFF A,byte    | OFFI A,byte    | skip if (A&byte)=0    | 1,2,3,4   |

| SKOFF r,byte    | OFFI r,byte    | skip if (r&byte)=0    | 2,3,4     |

| SKOFF sr2,byte  | OFFI sr2,byte  | skip if (sr2&byte)=0  | 2,3,4     |

| SKOFF A,(wa)    | OFFAW wa       | skip if (A&(wa))=0    | 2,3,4     |

| SKOFF (wa),byte | OFFIW wa,byte  | skip if (wa)&byte=0   | 1,2,3,4   |

| SKOFF EA,rp3    | DOFF EA,rp3    | skip if (EA&rp3)=0    | 3,4       |

+-----------------+----------------+-----------------------+-----------+

| INC r2          | INR r2         | r2<-r2+1              | 1,2,3,4   |

| INC (wa)        | INRW wa        | (wa)<-(wa)+1          | 1,2,3,4   |

| INC rp          | INX rp         | rp<-rp+1              | 1,2,3,4   |

+-----------------+----------------+-----------------------+-----------+

| DEC r2          | DCR r2         | r2<-r2-1              | 1,2,3,4   |

| DEC (wa)        | DCRW wa        | (wa)<-(wa)-1          | 1,2,3,4   |

| DEC rp          | DCX rp         | rp<-rp-1              | 1,2,3,4   |

+-----------------+----------------+-----------------------+-----------+

| CPU Group 1: 78C05, 78C06                                            |

| CPU Group 2: 7800, 7801, 7802                                        |

| CPU Group 3: 7807, 7808, 7809, 7810                                  |

| CPU Group 4: 7810, 78C1x                                             |

+-----------------+----------------+-----------------------+-----------+

4.62. 75K0

----------

Similar  to other  processors, the  assembly language  of the 75 series

also  knows pseudo bit operands, i.e. it is possible to assign a combi-

nation  of address and bit number to a  symbol that can then be used as

an  argument for bit  oriented instructions just  like explicit expres-

sions.  The following three instructions  for example generate the same

code:

ADM     sfr     0fd8h

SOC     bit     ADM.3

skt     0fd8h.3

skt     ADM.3

skt     SOC

AS  distinguishes direct and  symbolic bit accesses  by the missing dot

in  symbolic names;  it is  therefore forbidden  to use  dots in symbol

names to avoid misunderstandings in the parser.

The  storage format of bit symbols  mostly accepts the binary coding in

the  machine instructions themselves: 16 bits  are used, and there is a

''long''  and a ''short''  format. The short  format can store the fol-

lowing variants:

- direct accesses to the address range from 0FBxH to 0FFxH

- indirect accesses in the style of Addr.@L (0FC0H <= Addr <=0FFFH)

- indirect accesses in the style of @H+d4.bit

The  upper byte is  set to 0,  the lower byte  contains the bit expres-

sion  coded according to [106]. The  long format in contrast only knows

direct  addressing, but it  can cover the  whole address space (given a

correct  setting of  MBS and  MBE). A  long expression stores bits 0..7

of  the address in  the lower byte,  the bit position  in bits 8 and 9,

and  a constant value of  01 in bits 10  and 11. The highest bits allow

to  distinguish easily between long and  short addresses via a check if

the  upper byte is  0. Bits 12..15  contain bits 8..11  of the address;

they  are not needed to  generate the code, but  they have to be stored

somewhere  as the  check for  correct banking  can only take place when

the symbol is actually used.

4.63. 78K0

----------

NEC  uses  different  ways  to  mark  absolute  addressing  in its data

books:

- absolute short: no prefix

- absolute long: prefix of !

- PC relative: prefix of $

Under  AS, these prefixes  are only necessary  if one wants  to force a

certain  addressing  mode  and  the  instruction allows different vari-

ants.  Without  a  prefix,  AS  will  automatically select the shortest

variant.  It should  therefore rarely  be necessary  to use a prefix in

practice.

4.64. 78K2/78K3/78K4

--------------------

Analogous  to the  78K0, NEC  here also  uses dollar signs and exclama-

tion  marks to  specify different  lengths of  address expressions. The

selection  between  long  and  short  addresses  is  done automatically

(both  in RAM and  SFR areas), only  relative addressing has  to be se-

lected  explicitly,  if  an  instruction  supports  both variants (like

BR).

An  additional  remark  (which  is  also  true for the 78K0): Those who

want  to use Motorola  syntax via RELAXED,  might have to put hexadeci-

mal  constants in parentheses,  since the leading  dollar sign might be

misunderstood as relative addressing...

4.65. uPD772x

-------------

Both  the 7720  and 7725  are provided  by the  same code generator and

are  extremely  similar  in  their  instruction set. One should however

not  beleive  that  they  are  binary  compatible: To get space for the

longer  address fields  and additional  instructions, the bit positions

of  some fields in the instruction  word have changed, and the instruc-

tion  length has changed from 23 to  24 bits. The code format therefore

uses different header ids for both CPUs.

They  both have in  common that in  addition to the  code and data seg-

ment,  there is  also a  ROM for  storage of  constants. In the case of

AS, it is mapped onto the ROMDATA segment!

4.66. uCOM-43

-------------

The  uCOM-43 instruction set contains  an instruction CZP that performs

a  single byte subroutine call to the  addresses 0, 4, 8, 12...60. How-

ever,  the available  documentation is  unclear about the instruction's

argument  in source code: should it be the target address (i.e., a mul-

tiple  of four) or  th vector contained  in the opcode  (i.e., a number

from 0 to 15). To decide, the assembler performs some guessing:

- If  the argument is larger  than 15, it must  be an address, other-

wise:

- If  the argument is  not a multiple  of four, it  must be a vector,

otherwise:

- If the argument is zero, no distinction is possible, otherwise:

- If  the argument  is a  symbol from  CODE space  (e.g. a label), it

must be an address, otherwise:

- The argument is a vector.

The  final two rules only  apply to the values  4, 8, and 12, and since

the  decision is  not intuitively  clear, the  decision for a vector is

accompanied  with a warning.  In case an  address was meant, define the

argument as symbol from CODe space:

dest   label  4

.

.

.

czp dest

In  case a  vector was  meant, and  the warning  bothers you, it may be

suppressed this way:

vector equ    4

.

.

.

expect 480

czp    vector

endexpect

I  admit that  this situation  is not  100% satisfactory. If anyone has

information  about the behaviour of the  original NEC assembler, I'd be

grateful for contacting me.

4.67. F2MC16L

-------------

Along  with  the  discussion  of  the  ASSUME statement, it has already

been  mentioned that  it is  important to  inform AS  about the correct

current  values of all bank registers -  if your program uses more than

64K  RAM or 64K  ROM. With these  assumptions in mind,  AS checks every

direct  memory access for attempts to  access a memory location that is

currently  not in  reach. Of  course, standard  situations only require

knowledge  of DTB  and DPR  for this  purpose, since  ADB resp. SSB/USB

are  only  used  for  indirect  accesses  via RW2/RW6 resp. RW3/RW7 and

this  mechanism  anyway  doesn't  work  for indirect accesses. However,

similar  to the 8086, it  is possible to place  a prefix in front of an

instruction  to  replace  DTB  by  a  different  register. AS therefore

keeps  track  of  used  segment  prefixes and toggles appropriately for

the  next machine instruction. A  pseudo instruction placed between the

prefix  and the machine instruction does  not reset the toggle. This is

also  true for pseudo  instructions that store  data or modify the pro-

gram counter. Which doesn't make much sense anyway...

4.68. MN161x

------------

This  target is  special because  there are  two different code genera-

tors  one may choose from.  The first one was  kindly provided by Haruo

Asano  and that  may be  reached via  the CPU names MN1610 resp.MN1613.

The  other one  was written  by me  and is  activated via the CPU names

MN1610ALT  resp. MN1613ALT.  If you  want to  use the MN1613's extended

address  space of  256 KWords,  or if  you want  to experiment with the

MN1613's floating point formant, you have to use the ALT target.

4.69. CDP180x

-------------

This  family of  processors supports  both long  and short  branches: a

short  branch is  only possible  within the  same 256 byte memory page,

and  a long branch is possible to  any target in the 64K address space.

The  assembly  syntax  provides  different  mnemonics for both variants

(the  long variant with  a leading 'L'),  but there is  no variant that

would  let the assembler  decide itself between  long or short. AS sup-

ports such 'pseudo instructions' as an extension:

- JMP becomes BR oder LBR.

- JZ becomes BZ oder LBZ.

- JNZ becomes BNZ oder LBNZ.

- JDF becomes BDF oder LBDF.

- JPZ becomes BPZ oder LBPZ.

- JGE becomes BGE oder LBGE.

- JNF becomes BNF oder LBNF.

- JM becomes BM oder LBM.

- JL becomes BL oder LBL.

- JQ becomes BQ oder LBQ.

- JNQ becomes BNQ oder LBNQ.

4.70. KENBAK

------------

The  KENBAK-1 was developed  in 1970, at  a time when  the first micro-

processor  was still three years away. One  may assume that for the few

hobbyists  that could  afford the  kit back  then, this was their first

and  only computer. As  a consequence, they  had nothing they could run

an  assembler on, the KENBAK-1 itself with  its 256 bytes of memory was

way  too small for  such a task.  The preferred method  was to use pre-

printed  tables, which had  fields to fill  in instructions and machine

codes.  Once this ''programming job'' was done, one would enter the ma-

chine code manually via the computer's switch row.

The  effect of  this is  that though  the KENBAK's assembly language is

described  in the  manual, there  is no  real formal  definition of it.

When  Grant Stockly released new KENBAK kits  a few years ago, he did a

first  implementation  of  the  KENBAK  on my assembler. Unfortunately,

this  never went upstream. I tried to take up his ideas in my implemen-

tation,  but on  the other  hand I  also tried  to offer  a syntax that

should  be familiar to programmers of  6502, Z80 or similar processors.

The following table lists the syntax differences:

Table 4.10: KENBAK-Befehlssyntax

+------------------------------------+------------------------+------------------------+

| Stockly                            | Alternativ             | Bemerkung              |

+------------------------------------+------------------------+------------------------+

+------------------------------------+------------------------+------------------------+

| Arithmetic/Logic (ADD/SUB/LOAD/STORE/AND/OR/LNEG)                                    |

+------------------------------------+------------------------+------------------------+

| instr Constant, Reg, Wert,         | instr Reg, #Wert       | immediate              |

| instr Memory, Reg, Addr,           | instr Reg, Addr        | direct                 |

| instr Indirect, Reg, Addr,         | instr Reg, (Addr)      | direct                 |

| instr Indexed, Reg, Addr,          | instr Reg, Addr,X      | indexed                |

| instr Indirect-Indexed, Reg, Addr, | instr Reg, (Addr),X    | indirect-indexed       |

+------------------------------------+------------------------+------------------------+

| Jumps                                                                                |

+------------------------------------+------------------------+------------------------+

| JPD Reg, Cond, Addr                | JP Reg, Cond, Addr     | conditional-direct     |

| JPI Reg, Cond, Addr                | JP Reg, Cond, (Addr)   | conditional-indirect   |

| JMD Reg, Cond, Addr                | JM Reg, Cond, Addr     | conditional-direct     |

| JMI Reg, Cond, Addr                | JM Reg, Cond, (Addr)   | conditional-indirect   |

| JPD Unconditional, Cond, Addr      | JP Addr                | unconditional-direct   |

| JPI Unconditional, Cond, Addr      | JP (Addr)              | unconditional-indirect |

| JMD Unconditional, Cond, Addr      | JM Addr                | unconditional-direct   |

| JMI Unconditional, Cond, Addr      | JM (Addr)              | unconditional-indirect |

+------------------------------------+------------------------+------------------------+

| Jump Conditions                                                                      |

+------------------------------------+------------------------+------------------------+

| Non-zero                           | NZ                     | != 0                   |

| Zero                               | Z                      | = 0                    |

| Negative                           | N                      | < 0                    |

| Positive                           | P                      | >= 0                   |

| Positve-Non-zero                   | PNZ                    | > 0                    |

+------------------------------------+------------------------+------------------------+

| Skips                                                                                |

+------------------------------------+------------------------+------------------------+

| SKP 0, bit, Addr                   | SKP0 bit, Addr [,Dest] |                        |

| SKP 1, bit, Addr                   | SKP1 bit, Addr [,Dest] |                        |

+------------------------------------+------------------------+------------------------+

| Bit Manipulation                                                                     |

+------------------------------------+------------------------+------------------------+

| SET 0, bit, Addr                   | SET0 bit, Addr         |                        |

| SET 1, bit, Addr                   | SET1 bit, Addr         |                        |

+------------------------------------+------------------------+------------------------+

| Shifts/Rotates                                                                       |

+------------------------------------+------------------------+------------------------+

| SHIFT LEFT, cnt, Reg               | SFTL [cnt,] Reg        |                        |

| SHIFT RIGHT, cnt, Reg              | SFTR [cnt,] Reg        | arithm. Shift          |

| ROTATE LEFT, cnt, Reg              | ROTL [cnt,] Reg        |                        |

| ROTATE RIGHT, cnt, Reg             | ROTR [cnt,] Reg        |                        |

+------------------------------------+------------------------+------------------------+

There  is no  pseudo instruction  to switch  between these syntax vari-

ants. They may both be used anytime and in an arbitrary mix.

The  target address  [Dest] that  may optionally  be added  to skip in-

structions  will not become part of the machine code. The assembler on-

ly  checks whether  the processor  wil actually  skip to  the given ad-

dress.  This allows  for instance  to check  whether one actually tries

to  skip a one-byte  instruction. If the  shift count argument [cnt] is

omitted, a one-bit shift/rotate is coded.

4.71. HP Nanoprocessor

----------------------

The  HP Nanoprocessor  does not  provide any  instructions to read data

from  the ROM  address space.  The respective  instructions LDR and STR

rather  represent  what  is  called  ''immediate  addressing'' on other

processors.  For this reson,  there are pseudo  instructions that would

allow placing data constants im ROM memory or to reserve space in it.

4.72. IM61x0

------------

This  microprocessor  is  effectively  a  single chip implementation of

the  PDP8/E, which  is why  Digital Equipment's  PAL-II is  usually the

''reference  assembler'' for  source code  samples. The  AS implementa-

tion  deviates from the PAL-III syntax in a couple of areas, among oth-

er  reasons also because  several things only  could have been provided

with  huge efforts.  Here are  some hints  about how  to adapt existing

code:

- PAL-III  marks labels by an appended  comma. AS instead uses an ap-

pended  double colon, or  no special character  at all if the label

begins in the first column of a line.

- Placing  constants in memory  is done with  PAL-III simply by writ-

ing  the numeric constant instead of a mnemonic. AS uses the DC in-

struction  to  place  data,  which  will  also accept more than one

word  as  argument.  However,  the  LTORG  mechanism may be used in

some  cases to  get around  explicitly placing  constants in memory

at all.

- The program counter is set by ORG address instead of*address.

5. File Formats

===============

In  this chapter, the formats of  files AS generates shall be explained

whose formats are not self-explanatory.

5.1. Code Files

---------------

The  format for code files  generated by the assembler  must be able to

separate  code parts that  were generated for  different target proces-

sors;  therefore,  it  is  a  bit  different  from  most other formats.

Though  the assembler package  contains tools to  deal with code files,

I think is a question of good style to describe the format in short:

If  a code  file contains  multibyte values,  they are stored in little

endian  order. This  rule is  already valid  for the  16-bit magic word

$1489, i.e. every code file starts with the byte sequence $89/$14.

This  magic word is  followed by an  arbitrary number of ''records''. A

record  may either  contain a  continuous piece  of the code or certain

additional  information. Even without  switching to different processor

types,  a  file  may  contain  several code-containing records, in case

that  code or  constant data  areas are  interrupted by reserved memory

areas  that should  not be  initialized. This  way, the assembler tries

to keep the file as short as possible.

Common  to  all  records  is  a  header byte which defines the record's

type  and its contents.  Written in a  PASCALish way, the record struc-

ture can be described in the following way:

FileRecord = RECORD CASE Header:Byte OF

$00:(Creator:ARRAY[] OF Char);

$01..

$7f:(StartAdr : LongInt;

Length   : Word;

Data     : ARRAY[0..Length-1] OF Byte);

$80:(EntryPoint:LongInt);

$81:(Header   : Byte;

Segment  : Byte;

Gran     : Byte;

StartAdr : LongInt;

Length   : Word;

Data     : ARRAY[0..Length-1] OF Byte);

END

This  description  does  not  express  fully  that  the  length of data

fields is variable and depends on the value of the Length entries.

A  record with a header  byte of $81 is  a record that may contain code

or  data  from  arbitrary  segments.  The first byte (Header) describes

the  processor family the  following code resp.  data was generated for

(see table 5.1).

Table 5.1: Header Bytes for the Different Processor Families

+--------+--------------++--------------------------+--------+

| Header | Family       ||          Header          | Family |

+--------+--------------++--------------------------+--------+

+--------+--------------++--------------------------+--------+

|   $01  | 680x0, 6833x ||                          |        |

|        | $02          ||       ATARI_VECTOR       |        |

|        | $03          ||          M*Core          |        |

|        | $04          ||           XGATE          |        |

|        | $05          ||          PowerPC         |        |

|        | $06          ||           XCore          |        |

|        | $07          ||          TMS1000         |        |

|        | $08          ||          NS32xxx         |        |

|        | $09          ||         DSP56xxx         |        |

|        | $0a          ||          CP-1600         |        |

|        | $0b          ||     HP Nano Processor    |        |

|        | $0c          ||        IM6100/6120       |        |

|        | $0d          ||          NEC V60         |        |

|        | $0e          ||         IBM PALM         |        |

|        | $0f          ||           CP-3F          |        |

|        | $10          ||      Rockwell PPS-4      |        |

|        | $11          ||      65xx/MELPS-740      |        |

|        | $12          ||        MELPS-4500        |        |

|        | $13          ||            M16           |        |

|        | $14          ||           M16C           |        |

|        | $15          ||          F^2MC8L         |        |

|        | $16          ||         F^2MC16L         |        |

|        | $17          ||          IMP-16          |        |

|        | $18          ||      IPC-16/INS8900      |        |

|        | $19          ||     65816/MELPS-7700     |        |

|        | $1a          ||           PDK13          |        |

|        | $1b          ||           PDK14          |        |

|        | $1c          ||           PDK15          |        |

|        | $1d          ||           PDK16          |        |

|        | $1e          ||        Renesas RX        |        |

|        | $1f          ||          SC61860         |        |

|        | $20          ||          SC62015         |        |

|        | $21          ||          MCS-48          |        |

|        | $22          ||       Konami 052001      |        |

|        | $23          ||          PDP-11          |        |

|        | $24          ||           WD16           |        |

|        | $25          ||         SYM53C8xx        |        |

|        | $26          ||            VAX           |        |

|        | $27          ||          KENBAK          |        |

|        | $28          ||          �COM-43         |        |

|        | $29          ||           29xxx          |        |

|        | $2a          ||           i960           |        |

|        | $2b          ||          TLCS-42         |        |

|        | $31          ||          MCS-51          |        |

|        | $32          ||            ST9           |        |

|        | $33          ||            ST7           |        |

|        | $35          ||           Z8000          |        |

|        | $35          ||          Super8          |        |

|        | $36          ||          MN161x          |        |

|        | $37          ||           2650           |        |

|        | $38          ||         1802/1805        |        |

|        | $39          ||      MCS-96/196/296      |        |

|        | $3a          ||           8X30x          |        |

|        | $3b          ||            AVR           |        |

|        | $3c          ||            XA            |        |

|        | $3d          || AVR (8-Bit Code-Segment) |        |

|        | $3e          ||           8008           |        |

|        | $3f          ||         4004/4040        |        |

|        | $40          ||            H16           |        |

|        | $41          ||         8080/8085        |        |

|        | $42          ||         8086..V35        |        |

|        | $43          ||           SX20           |        |

|        | $44          ||            F8            |        |

|        | $45          ||           S12Z           |        |

|        | $46          ||           78K4           |        |

|        | $47          ||         TMS320C6x        |        |

|        | $48          ||          TMS9900         |        |

|        | $49          ||         TMS370xxx        |        |

|        | $4a          ||          MSP430          |        |

|        | $4b          ||        TMS320C54x        |        |

|        | $4c          ||        80C166/167        |        |

|        | $4d          ||          OLMS-50         |        |

|        | $4e          ||          OLMS-40         |        |

|        | $4f          ||       MIL STD 1750       |        |

|        | $50          ||         HMCS-400         |        |

|        | $51          ||     Z80/180/380/eZ80     |        |

|        | $52          ||         TLCS-900         |        |

|        | $53          ||          TLCS-90         |        |

|        | $54          ||         TLCS-870         |        |

|        | $55          ||          TLCS-47         |        |

|        | $56          ||         TLCS-9000        |        |

|        | $57          ||        TLCS-870/C        |        |

|        | $58          ||         NEC 78K3         |        |

|        | $59          ||            eZ8           |        |

|        | $5a          ||          TC9331          |        |

|        | $5b          ||          KCPSM3          |        |

|        | $5c          ||       LatticeMico8       |        |

|        | $5d          ||         NEC 75xx         |        |

|        | $5e          ||          68RS08          |        |

|        | $5f          ||           COP4           |        |

|        | $60          ||           78K2           |        |

|        | $61          ||     6800, 6301, 6811     |        |

|        | $62          ||         6805/HC08        |        |

|        | $63          ||           6809           |        |

|        | $64          ||           6804           |        |

|        | $65          ||          68HC16          |        |

|        | $66          ||          68HC12          |        |

|        | $67          ||            ACE           |        |

|        | $68          ||         H8/300(H)        |        |

|        | $69          ||          H8/500          |        |

|        | $6a          ||           807x           |        |

|        | $6b          ||           KCPSM          |        |

|        | $6c          ||          SH7000          |        |

|        | $6d          ||          SC14xxx         |        |

|        | $6e          ||           SC/MP          |        |

|        | $6f          ||           COP8           |        |

|        | $70          ||         PIC16C8x         |        |

|        | $71          ||         PIC16C5x         |        |

|        | $72          ||         PIC17C4x         |        |

|        | $73          ||         TMS-7000         |        |

|        | $74          ||         TMS3201x         |        |

|        | $75          ||         TMS320C2x        |        |

|        | $76          ||       TMS320C3x/C4x      |        |

|        | $77          ||      TMS320C20x/C5x      |        |

|        | $78          ||            ST6           |        |

|        | $79          ||            Z8            |        |

|        | $7a          ||        �PD78(C)10        |        |

|        | $7b          ||           75K0           |        |

|        | $7c          ||           78K0           |        |

|        | $7d          ||          �PD7720         |        |

|        | $7e          ||          �PD7725         |        |

|        | $7f          ||         �PD77230         |        |

|        |              ||                          |        |

+--------+--------------++--------------------------+--------+

The  Segment field signifies  the address space  the following code be-

longs to. The assignment defined in table 5.1 applies.

+--------+-------------++--------+---------+

| number | segment     || number | segment |

+--------+-------------++--------+---------+

+--------+-------------++--------+---------+

|   $00  | <undefined> ||   $01  | CODE    |

|   $02  | DATA        ||   $03  | IDATA   |

|   $04  | XDATA       ||   $05  | YDATA   |

|   $06  | BDATA       ||   $07  | IO      |

|   $08  | REG         ||   $09  | ROMDATA |

+--------+-------------++--------+---------+

Table 5.1: Codings of the Segment Field

The  Gran field describes the code's  ''granularity'', i.e. the size of

the  smallest addressable unit in the  following set of data. This val-

ue  is a function of processor type and segment and is an important pa-

rameter  for the  interpretation of  the following  two fields that de-

scribe  the block's start  address and its  length: While the start ad-

dress  refers to the granularity, the  Length value is always expressed

in  bytes! For example, if the start  address is $300 and the length is

12,  the resulting end  address would be  $30b for a  granularity of 1,

however  $303 for a granularity of  4! Granularities that differ from 1

are  rare and  mostly appear  in DSP  CPU's that  are not  designed for

byte  processing. For  example, a  DSP56K's address  space is organized

in  64  Kwords  of  16  bits.  The  resulting  storage  capacity is 128

Kbytes,  however  it  is  organized  as  2 ^16 words that are addressed

with addresses 0,1,2,...65535!

The  start  address  is  always  32  bits  in  size, independent of the

processor  family. In  contrast, the  length specification  has only 16

bits,  i.e.  a  record  may  have  a  maximum length of 4+4+2+(64K-1) =

65545 bytes.

Data  records with a Header ranging from  $01 to $7f present a shortcut

and  preserve  backward  compatibility  to  earlier  definitions of the

file  format: in their case, the  Header directly defines the processor

type,  the target segment is  fixed to CODE and  the granularity is im-

plicitly  given by the processor type, rounded  up to the next power of

two.  AS prefers to use  these records whenever data  or code should go

into the CODE segment.

A  record with  a Header  of $80  defines an  entry point, i.e. the ad-

dress  where execution  of the  program should  start. Such a record is

the  result of an  END statement with  a corresponding address as argu-

ment.

The  last record in a  file bears the Header  $00 and has only a string

as  data field. This string does not have an explicit length specifica-

tion;  its end  is equal  to the  file's end.  The string contains only

the  name of the program that created the file and has no further mean-

ing.

5.2. Debug Files

----------------

Debug  files may optionally be generated  by AS. They deliver important

information  for tools used  after assembly, like  disassemblers or de-

buggers.  AS can generate debug  files in one of  three formats: On the

one  hand, the object format  used by the AVR  tools from Atmel respec-

tively  a NoICE-compatible command  file, and on  the other hand an own

format.  The first two are  described in detail in  [5] resp. the NoICE

documentations,  which is  why the  following description limits itself

to the AS-specific MAP format:

The information in a MAP file is split into three groups:

- symbol table

- memory usage per section

- machine addresses of source lines

The  second item is  listed first in  the file. A  single entry in this

list consists of two numbers that are separated by a : character:

<line number>:<address>

Such  an entry  states that  the machine  code generated for the source

statement  in  a  certain  line  is  stored  at  the  mentioned address

(written  in hexadecimal notation). With  such an information, a debug-

ger  can display the corresponding  source lines while stepping through

a  program. As a program may consist  of several include files, and due

to  the fact that a lot of  processors have more than one address space

(though  admittedly  only  one  of  them  is  used  to store executable

code),  the entries  described above  have to  be sorted.  AS does this

sorting  in two levels: The primary sorting criteria is the target seg-

ment,  and the  entries in  one of  these sections are sorted according

to  files.  The  sections  resp.  subsections  are separated by special

lines in the style of

Segment <segment name>

resp.

File <file name>   .

The  source line info is  followed by the symbol  table. Similar to the

source  line info,  the symbol  table is  primarily sorted  by the seg-

ments  individual symbols  are assigned  to. In  contrast to the source

line  info,  an  additional  section  NOTHING exists which contains the

symbols  that are  not assigned  to any  specific segment (e.g. symbols

that  have been defined with a simple  EQU statement). A section in the

symbol table is started with a line of the following type:

Symbols in Segment <segment name>

The  symbols in a section are  sorted according to the alphabetical or-

der  of  their  names,  and  one  symbol  entry consists of exactly one

line.  Such a  line consists  of six  fields witch  are separated by at

least a single space:

The  first field is  the symbol's name,  possibly extended by a section

number  enclosed in  brackets. Such  a section  number limits the range

of  validity for  a symbol.  The second  field designates  the symbol's

type:  Int stands  for integer  values, Float  for floating  point num-

bers,  and String  for character  arrays. The  third field finally con-

tains  the symbol's value. If the symbol  contains a string, it is nec-

essary  to use  a special  encoding for  control characters and spaces.

Without  such a coding,  spaces in a  string could be misinterpreted as

delimiters  to the  next field.  AS uses  the same  syntax that is also

valid  for assembly source  files: Instead of  the character, its ASCII

value  with  a  leading  backslash  (\)  is  inserted. For example, the

string

This is a test

becomes

This\032is\032\a\032test   .

The  numerical value always has three  digits and has to be interpreted

as  a decimal  value. Naturally,  the backslash  itself also  has to be

coded this way.

The  fourth  field  specifies  -  if  available  - the size of the data

structure  placed at  the address  given by  the symbol. A debugger may

use  this information to automatically  display variables in their cor-

rect  length when they  are referred symbolically.  In case AS does not

have  any information  about the  symbol size,  this field  simply con-

tains the value -1.

The  fifth field states  via the values  0 or 1  if the symbol has been

used  during assembly.  A program  that reads  the symbol table can use

this  field to skip  unused symbols as  they are probably unused during

the following debugging/disassembly session.

Finally,  the sixth field  states via the  values 0 or  1 if the symbol

is  a constant (0) or variable(1).  Constant symbols are set once, e.g.

via  the  EQU  statement  or  a  label,  while variables are allowed to

change  their value during  the course of  assembly. The MAP file lists

the final value.

The  third section in a debug  file describes the program's sections in

detail.  The need for such a  detailed description arises from the sec-

tions'  ability to limit the validity  range of symbols. A symbolic de-

bugger  for example cannot  use certain symbols  for a reverse transla-

tion,  depending on the  current PC value.  It may also  have to regard

priorities  for symbol usage  when a value  is represented by more than

one  symbol. The definition of a section starts with a line of the fol-

lowing form:

Info for Section nn ssss pp

nn  specifies the  section's number  (the number  that is  also used in

the  symbol table as a  postfix for symbol names),  ssss gives its name

and  pp the number of its parent section. The last information is need-

ed  by a retranslator to  step upward through a  tree of sections until

a  fitting symbol is found. This first  line is followed by a number of

further  lines  that  describe  the  code  areas  used by this section.

Every  single entry  (exactly one  entry per  line) either  describes a

single  address  or  an  address  range  given  by a lower and an upper

bound  (separation of  lower and  upper bound  by a  minus sign). These

bounds  are ''inclusive'',  i.e. the  bounds themselves  also belong to

the  area. Is is important to note  that an area belonging to a section

is  not additionally listed  for the section's  parent sections (an ex-

ception  is of course  a deliberate multiple  allocation of address ar-

eas,  but you would not do this, would you?). On the one hand, this al-

lows  an optimized storage of memory areas during assembly. On the oth-

er  hand, this should not be  an obstacle for symbol backtranslation as

the  single entry already gives an unambiguous entry point for the sym-

bol  search path.  The description  of a  section is  ended by an empty

line or the end of the debug file.

Program  parts that lie  out of any  section are not listed separately.

This  implicit ''root section'' carries the  number -1 and is also used

as  parent section  for sections  that do  not have  a real parent sec-

tion.

It  is possible  that the  file contains  empty lines or comments (semi

colon  at line start).  A program reading  the file has  to ignore such

lines.

6. Utility Programs

===================

To  simplify the work with  the assembler's code format  a bit, I added

some  tools to  aid processing  of code  files. These  programs are re-

leased under the same license terms as stated in section 1.1!

Common  to all programs are the  possible return codes they may deliver

upon completion (see table 6.1).

+-------------+----------------------------------+

| return code | error condition                  |

+-------------+----------------------------------+

+-------------+----------------------------------+

|      0      | no errors                        |

|      1      | error in command line parameters |

|      2      | I/O error                        |

|      3      | file format error                |

+-------------+----------------------------------+

Table 6.1: Return Codes of the Utility Programs

Just  like AS, all programs take their  input from STDIN and write mes-

sages  to  STDOUT  (resp.  error  messages to STDERR). Therefore, input

and output redirections should not be a problem.

In  case that  numeric or  address specifications  have to  be given in

the  command line,  they may  also be  written in hexadecimal notation,

either  ba allending a h or prepending  a dollar character or a 0x like

in C. (e.g. 10h, $10, or 0x10 instead of 16).

Unix  shells  however  assign  a  special  meaning  to the dollar sign,   UNIX

which  makes it  necessary to  escape a  dollar sign  with a backslash.

The 0x variant is definitely more comfortable in this case.

Otherwise,  calling conventions and variations  are equivalent to those

of  AS (except for PLIST and AS2MSG); i.e. it is possible to store fre-

quently  used parameters in an environment variable (whose name is con-

structed  by  appending  CMD  to  the  program's name, i.e. BINDCMD for

BIND),  to negate options, and to use all upper- resp. lower-case writ-

ing (for details on this, see section 2.4).

Address  specifications always relate to the granularity of the proces-

sor  currently in question;  for example, on  a PIC, an address differ-

ence of 1 means a word and not a byte.

6.1. PLIST

----------

PLIST  is the simplest  one of the  five programs supplied: its purpose

is  simply to list all  records that are stored  in a code file. As the

program does not do very much, calling is quite simple:

PLIST <file name>

The  file name will  automatically be extended  with the extension P if

it doesn't already have one.

CAUTION!  At this place, no wildcards are allowed! If there is a neces-

sity  to list several files with  one command, use the following ''mini

batch'':

for %n in (*.p) do plist %n

PLIST  prints the code file's contents in a table style, whereby exact-

ly  one line will be  printed per record. The  individual rows have the

following meanings:

- code type: the processor family the code has been generated for.

- start  address:  absolute  memory  address  that expresses the load

destination for the code.

- length: length of this code chunk in bytes.

- end  address: last address of this code chunk. This address is cal-

culated as start address+length-1.

All outputs are in hexadecimal notation.

Finally,  PLIST will print a  copyright remark (if there  is one in the

file), together with a summaric code length.

Simply  said, PLIST is a sort of DIR  for code files. One can use it to

examine a file's contents before one continues to process it.

6.2. BIND

---------

BIND  is a  program that  allows to  concatenate the records of several

code  files into  a single  file. A  filter function  is available that

can  be used to copy  only records of certain  types. Used in this way,

BIND can also be used to split a code file into several files.

The general syntax of BIND is

BIND <source file(s)> <target file> [options]

Just  like  AS,  BIND  regards  all  command line arguments that do not

start  with a +, -  or / as file  specifications, of which the last one

must  designate  the  destination  file.  All other file specifications

name sources, which may again contain wildcards.

Currently, BIND defines only one command line option:

- f  <Header[,Header]>: sets a list of  record headers that should be

copied.  Records with other header IDs  will not be copied. Without

such  an option, all  records will be  copied. The headers given in

the  list correspond to the HeaderID  field of the record structure

described  in section 5.1. Individual headers in this list are sep-

arated with commas.

For  example, to filter  all MCS-51 code  out of a  code file, use BIND

in the following way:

BIND <source name> <target name> -f $31

If  a file name misses an extension,  the extension P will be added au-

tomatically.

6.3. P2HEX

----------

P2HEX  is an  extension of  BIND. It  has all  command line  options of

BIND  and  uses  the  same  conventions  for file names. In contrary to

BIND,  the target file is written as a  Hex file, i.e. as a sequence of

lines which represent the code as ASCII hex numbers.

P2HEX  knows nine different  target formats, which  can be selected via

the command line parameter F:

- Motorola S-Records (-F Moto))

- MOS Hex (-F MOS)

- Intel Hex (Intellec-8, -F Intel)

- 16-Bit Intel Hex (MCS-86, -F Intel16)

- 32-Bit Intel Hex (-F Intel32))

- Tektronix Hex (-F Tek)

- Texas Instruments DSK (-F DSK)

- Atmel AVR Generic (-F Atmel, see [5])

- Lattice Mico8 prom_init (-F Mico8)

- C arrays, for inclusion into C(++) source files (-F C)

If  no target format is  explicitly specified, P2HEX will automatically

choose  one  depending  in  the  processor type: S-Records for Motorola

CPUs,  Hitachi, and  TLCS-900, MOS  for 65xx/MELPS,  DSK for the 16 bit

signal  processors from  Texas, Atmel  Generic for  the AVRs, and Intel

Hex  for  the  rest.  Depending  on  the  start addresses width, the S-

Record  format will use  Records of type  1, 2, or  3, however, records

in  one group will always  be of the same  type. This automatism can be

partially suppressed via the command line option

-M <1|2|3>

A  value of 2 resp.  3 assures that that  S records with a minimum type

of  2 resp. 3 will be used, while  a value of 1 corresponds to the full

automatism.

Normally,  the AVR  format always  uses an  address length  of 3 bytes.

Some  programs  however  do  not  like  that...which  is why there is a

switch

-avrlen <2|3>

that  allows to reduce the address length to two bytes in case of emer-

gency.

The  Mico8 format is different from all  the other formats in having no

address  fields - it is plain list  of all instruction words in program

memory.  When using it,  be sure that  the used address  range (as dis-

pleyed e.g. by PLIS) starts at zero and is continuous.

The  Intel, MOS and Tektronix formats  are limited to 16 bit addresses,

the  16-bit Intel format reaches 4  bits further. Addresses that are to

long  for a given format will be  reported by P2HEX with a warning; af-

terwards, they will be truncated (!).

For the PIC microcontrollers, the switch

-m <0..3>

allows  to generate the three different  variants of the Intel Hex for-

mat.  Format 0 is INHX8M which contains all bytes in a Lo-Hi-Order. Ad-

dresses  become double as  large because the  PICs have a word-oriented

address  space that  increments addresses  only by  one per  word. This

format  is also the default. With  Format 1 (INHX16M), bytes are stored

in  their natural order. This is the  format Microchip uses for its own

programming  devices. Format  2 (INHX8L)  resp. 3  (INHX8H) split words

into  their lower resp.  upper bytes. With  these formats, P2HEX has to

be  called twice to  get the complete  information, like in the follow-

ing example:

p2hex test -m 2

rename test.hex test.obl

p2hex test -m 3

rename test.hex test.obh

For  the Motorola  format, P2HEX  additionally uses  the S5 record type

mentioned  in [12].  This record  contains the  number of  data records

(S1/S2/S3)  to  follow.  As  some  programs  might not know how to deal

with this record, one can suppress it with the option

+5  .

The  C format is  different in the  sense that it  always has to be se-

lected  explicitly. The output file is  basically a complete piece of C

or  C++ code that contains the data  as a list of C arrays. Additional-

ly  to  the  data  itself,  a  list  of descriptors is written that de-

scribes  the start,  length, and  end address  of each  data block. The

contents of these descriptors may be configured via the option

-cformat <format>

Each letter in format defines an element of the descriptor:

- A  d or D  defines a pointer  to the data  itself. Usage of a lower

or  upper case letter  defines whether lowercase  or uppercase let-

ters are used for hexadecimal constants.

- An  s or  S defines  the start  address of  the data, either as un-

signed or unsigned long.

- An  l or L  defines the length  of the data,  either as unsigned or

unsigned long.

- An  e or E  defines the end  address of the  data, specifically the

last  address  used  by  the  data,  either as unsigned or unsigned

long.

In  case a source  file contains code  record for different processors,

the  different hex formats  will also show  up in the  target file - it

is therefore strongly advisable to use the filter function.

Apart  form this filter  function, P2HEX also  supports an address fil-

ter,  which is useful to split the  code into several parts (e.g. for a

set of EPROMs):

-r <start address>-<end address>

The  start address is the first address  in the window, and the end ad-

dress  is the last  address in the  window, not the  first address that

is  out of  the window.  For example,  to split  an 8051 program into 4

2764 EPROMs, use the following commands:

p2hex <source file> eprom1 -f $31 -r $0000-$1fff

p2hex <source file> eprom2 -f $31 -r $2000-$3fff

p2hex <source file> eprom3 -f $31 -r $4000-$5fff

p2hex <source file> eprom4 -f $31 -r $6000-$7fff

It  is allowed to specifiy  a single dollar character  or '0x' as start

or  stop  address.  This  means  that  the lowest resp. highest address

found  in the source file  shall be taken as  start resp. stop address.

The  default range is  '0x-0x', i.e. all  data from the  source file is

transferred.

CAUTION!  This type of splitting does  not change the absolute address-

es  that will be written into the  files! If the addresses in the indi-

vidual  hex files  should rather  start at  0, one  can force this with

the additional switch

-a     .

On  the other hand, to move the  addresses to a different location, one

may use the switch

-R <value> .

The  value given is an offset, i.e.  it is added to the addresses given

in the code file.

By  using an offset, it is possible to move a file's contents to an ar-

bitrary  position. This  offset is  simply appended  to a  file's name,

surrounded  with  parentheses.  For  example,  if  the  code  in a file

starts  at address  0 and  you want  to move  it to address 1000 hex in

the hex file, append ($1000) to the file's name (without spaces!).

In  case the  source file(s)  not only  contain data  for the code seg-

ment, the switch

-segment <name>

allows  to select the  segment data is  extracted from and converted to

HEX  format. The segment names  are the same as  for the SEGMENT pseudo

instruction  (3.2.18). The TI  DSK is a  special case since  it has the

ability  to distinguish between  data and code  in one file.  If TI DSK

is  the output format, P2HEX will  automatically extract data from both

segments if no segment was specified explicitly.

Similar to the -r option, the argument

-d <start>-<end>

allows  to designate the  address range that  should be written as data

instead of code.

The option

-e <address>

is  valid for the DSK,  Intel, and Motorola formats.  Its purpose is to

set  the entry  address that  will be  inserted into  the hex  file. If

such  a command line  parameter is missing,  P2HEX will search a corre-

sponding  entry in the code file. If  even this fails, no entry address

will  be written to the hex file  (DSK/Intel) or the field reserved for

the entry address will be set to 0 (Motorola).

Unfortunately,  one finds different  statements about the  last line of

an  Intel-Hex file in literature.  Therefore, P2HEX knows three differ-

ent  variants that  may be  selected via  the command-line  parameter i

and an additional number:

0  :00000001FF

1  :00000001

2  :0000000000

By default, variant 0 is used which seems to be the most common one.

If  the target file  name does not  have an extension,  an extension of

HEX is supposed.

By  default, P2HEX  will print  a maximum  of 16  data bytes  per line,

just  as most other tools that output  Hex files. If you want to change

this, you may use the switch

-l <count>   .

The  allowed range of values goes from  2 to 254 data bytes; odd values

will implicitly be rounded down to an even count.

In  most cases,  the temporary  code files  generated by  AS are not of

any further need after P2HEX has been run. The command line option

-k

allows  to  instruct  P2HEX  to  erase them automatically after conver-

sion.

In  contrast to BIND,  P2HEX will not  produce an empty  target file if

only  one file  name (i.e.  the target  name) has  been given. Instead,

P2HEX  will use the corresponding code  file. Therefore, a minimal call

in the style of

P2HEX <name>

is possible, to generate <name>.hex out of <name>.p.

6.4. P2BIN

----------

P2BIN  works similar to  P2HEX and offers  the same options (except for

the  a and i options  that do not make  sense for binary files), howev-

er,  the result  is stored  as a  simple binary  file instead  of a hex

file. Such a file is for example suitable for programming an EPROM.

P2BIN  knows three additional options  to influence the resulting bina-

ry file:

- l  <8 bit number>: sets  the value that should  be used to fill un-

used  memory areas. By  default, the value  $ff is used. This value

assures  that  every  half-way  intelligent  EPROM burner will skip

these  areas. This option allows to set different values, for exam-

ple  if you  want to  generate an  image for  the EPROM versions of

MCS-48  microcontrollers (empty cells of  their EPROM array contain

zeroes, so $00 would be the correct value in this case).

- s:  commands  the  program  to  calculate  a checksum of the binary

file.  This sum is printed as a 32-bit value, and the two's comple-

ment  of the  least significant  bit will  be stored  in the file's

last  byte. This way, the modulus-  256-sum of the file will become

zero.

- m:  is designed for the  case that a CPU  with a 16- or 32-bit data

bus  is used and the  file has to be  split for several EPROMs. The

argument may have the following values:

- ALL: copy everything

- ODD: copy all bytes with an odd address

- EVEN: copy all bytes with an even address

- BYTE0..BYTE3: copy only bytes with an address of 4n+0 .. 4n+3

- WORD0,  WORD1: copy only the lower  resp. upper 16- bit word of

a 32-bit word

To  avoid  confusions:  If  you  use  this option, the resulting binary

file  will become  smaller because  only a  part of  the source will be

copied.  Therefore, the resulting  file will be  smaller by a factor of

2 or 4 compared to ALL. This is just natural...

In  case the code file  does not contain an  entry address, one may set

it  via the -e command line option  just like with P2HEX. Upon request,

P2BIN  prepends  the  resulting  image  with  this address. The command

line option

-S

activates  this function.  It expects  a numeric  specification ranging

from  1 to  4 as  parameter which  specifies the  length of the address

field  in bytes. This number  may optionally be prepended  wit a L or B

letter  to set the endian order of the address. For example, the speci-

fication  B4 generates a  4 byte address  in big endian  order, while a

specification  of L2 or simply 2 creates a 2 byte address in little en-

dian order.

6.5. AS2MSG

-----------

AS2MSG  is not a  tool in the  real sense, it  is a filter that was de-

signed  to simplify the  work with the  assembler for (fortunate) users

of  Borland Pascal 7.0.  The DOS IDEs  feature a 'tools'  menu that can

be  extended with own  programs like AS.  The filter allows to directly

display  the error messages paired  with a line specification delivered

by  AS in the editor window.  A new entry has to  be added to the tools

menu to achieve this (Options/Tools/New). Enter the following values:

- Title: ~m~acro assembler

- Program path: AS

- Command line:

-E !1 $EDNAME $CAP MSG(AS2MSG) $NOSWAP $SAVE ALL

- assign a hotkey if wanted (e.g. Shift-F7)

The  -E option  assures that  Turbo Pascal  will not  become puzzled by

STDIN and STDERR.

I  assume that AS and  AS2MSG are located in  a directory listed in the

PATH  variable. After pressing the  appropriate hotkey (or selecting AS

from  the tools  menu), as  will be  called with  the name  of the file

loaded  in the  active editor  window as  parameter. The error messages

generated  during assembly are redirected to  a special window that al-

lows  to browse  through the  errors. Ctrl-Enter  jumps to an erroneous

line.  The  window  additionally  contains  the statistics AS prints at

the end of an assembly. These lines obtain the dummy line number 1.

TURBO.EXE  (Real  Mode)  and  BP.EXE  (Protected  Mode) may be used for

this  way of working with  AS. I recommend however  BP, as this version

does  not have  to 'swap'  half of  the DOS  memory before before AS is

called.

A. Error Messages of AS

=======================

Here  is a list  of all error  messages emitted by  AS. Each error mes-

sage is described by:

- the  internal error number  (it is displayed  only if AS is started

with the -n option)

- the text of the error message

- error type:

- Warning:  informs the user that a  possible error was found, or

that  some inefficient binary code  could be generated. The as-

sembly process is not stopped.

- Error:  an error was detected.  The assembly process continues,

but no binary code is emitted.

- Fatal:  unrecoverable error. The  assembly process is terminat-

ed.

- reason of the error: the situation originating the error.

- argument: a further explanation of the error message.

5 useless displacement

Type:

warning

Reason:

680x0,  6809 and  COP8 CPUs:  an address  displacement of 0 was

given.  An address expression  without displacement is generat-

ed,  and a convenient number of NOPs are emitted to avoid phas-

ing errors.

Argument:

none

10 short addressing possible

Type:

warning

Reason:

680x0-,  6502 and  68xx CPUs:  a given  memory location  can be

reached  using  short  addressing.  A short addressing instruc-

tion  is emitted, together with the  required number of NOPs to

avoid phasing errors.

Argument:

none

20 short jump possible

Type:

warning

Reason:

680x0-  and 8086 CPUs  can execute jumps  using a short or long

displacement.  If a shorter jump  was not explicitly requested,

in  the first  pass room  for the  long jump  is reserved. Then

the  code for  the shorter  jump is  emitted, and the remaining

space is filled with NOPs t

Argument:

none

25 relative jump possible

Type:

warning

Reason:

Z80  jumps  may  be  either  relative  or absolute. An absolute

jump  was requested in this  case, however a (shorter) relative

jump would be possible as well.

Argument:

the jump instruction's argument

30 no sharefile created, SHARED ignored

Type:

warning

Reason:

A  SHARED directive was  found, but on  the command line no op-

tions were specified, to generate a shared file.

Argument:

none

40 FPU possibly cannot read this value (>=1E1000)

Type:

warning

Reason:

The  BCD-floating  point  format  used  by the 680x0-FPU allows

such  a large exponent, but  according to the latest databooks,

this  cannot be  fully interpreted.  The corresponding  word is

assembled,  but the associated function is not expected to pro-

duce the correct result.

Argument:

none

50 privileged instruction

Type:

warning

Reason:

A  Supervisor-mode directive  was used,  that was  not preceded

by an explicit SUPMODE ON directive

Argument:

none

60 distance of 0 not allowed for short jump (NOP created instead)

Type:

warning

Reason:

A  short jump with  a jump distance  equal to 0  is not allowed

by  680x0  resp.  COP8  processors,  since  the associated code

word  is used to  identify long jump  instruction. Instead of a

jump instruction, AS emits a NOP

Argument:

none

70 symbol out of wrong segment

Type:

warning

Reason:

The  symbol  used  as  an  operand  comes from an address space

that cannot be addressed together with the given instruction

Argument:

none

75 segment not accessible

Type:

warning

Reason:

The  symbol  used  as  an  operand  belongs to an address space

that  cannot be accessed  with any of  the segment registers of

the 8086

Argument:

The name of the inaccessible segment

80 change of symbol values forces additional pass

Type:

warning

Reason:

A  symbol changed  value, with  respect to  previous pass. This

warning is emitted only if the -r option is used.

Argument:

name of the symbol that changed value.

90 overlapping memory usage

Type:

warning

Reason:

The  analysis of the usage list  shows that part of the program

memory  was used  more than  once. The  reason can be an exces-

sive usage of ORG directives.

Argument:

none

95 overlapping register usage

Type:

warning

Reason:

The  instruction  uses  whole  registers  or parts thereof in a

non-allowed way.

Argument:

The offending argument

100 none of the CASE conditions was true

Type:

warning

Reason:

A  SWITCH...CASE directive without  ELSECASE clause was execut-

ed, and none of the CASE conditions was found to be true.

Argument:

none

110 page might not be addressable

Type:

warning

Reason:

The  symbol  used  as  an  operand  was not found in the memory

page defined by an ASSUME directive (ST6, 78(C)10).

Argument:

none

120 register number must be even

Type:

warning

Reason:

The  CPU  allows  to  concatenate  only  register  pairs, whose

start address is even (RR0, RR2, ..., only for Z8).

Argument:

none

130 obsolete instruction, usage discouraged

Type:

warning

Reason:

The  instruction used, although supported,  was superseded by a

new  instruction. Future versions of the  CPU could no more im-

plement the old instruction.

Argument:

none

140 unpredictable execution of this instruction

Type:

warning

Reason:

The  addressing mode used for this instruction is allowed, how-

ever  a register is used  in such a way  that its contents can-

not be predicted after the execution of the instruction.

Argument:

none

150 localization operator senseless out of a section

Type:

warning

Reason:

An  aheaded @ must  be used, so  that it is explicitly referred

to  the local  symbols used  in the  section. When the operator

is  used out of a section,  there are no local symbols, because

this operator is useless in this context.

Argument:

none

160 senseless instruction

Type:

warning

Reason:

The  instruction used has no meaning,  or it can be substituted

by an other instruction, shorter and more rapidly executed.

Argument:

none

170 unknown symbol value forces additional pass

Type:

warning

Reason:

AS  expects a  forward definition  of a  symbol, i.e.  a symbol

was  used before it was defined. A further pass must be execut-

ed. This warning is emitted only if the -r option was used.

Argument:

none

180 address is not properly aligned

Type:

warning

Reason:

An  address  was  used  that  is  not  an exact multiple of the

operand  size. Although the CPU  databook forbids this, the ad-

dress  could be  stored in  the instruction  word, so AS simply

emits a warning.

Argument:

none.

190 I/O-address must not be used here

Type:

warning

Reason:

The  addressing mode or  the address used  are correct, but the

address  refers to the  peripheral registers, and  it cannot be

used in this circumstance.

Argument:

none.

200 possible pipelining effects

Type:

warning

Reason:

A  register is used in a series  of instructions, so that a se-

quence  of instructions probably does  not generate the desired

result.  This usually  happens when  a register  is used before

its new content was effectively loaded in it.

Argument:

the register probably causing the problem.

210 multiple use of address register in one instruction

Type:

warning

Reason:

A  register used  for the  addressing is  used once more in the

same  instruction, in a  way that results  in a modification of

the  register  value.  The  resulting  address  does not have a

well defined value.

Argument:

the register used more than once.

220 memory location is not bit addressable

Type:

warning

Reason:

Via  a SFRB  statement, it  was tried  to declare a memory cell

as  bit addressable  which is  not bit  addressable due  to the

8051's architectural limits.

Argument:

none

230 stack is not empty

Type:

warning

Reason:

At  the end of  a pass, a  stack defined by  the program is not

empty.

Argument:

the name of the stack and its remaining depth

240 NUL character in string, result is undefined

Type:

warning

Reason:

A  string constant contains a  NUL character. Though this works

with  the Pascal version, it is a  problem for the C version of

AS  since  C  itself  terminates  strings with a NUL character.

i.e.  the  string  would  have  its  end  for  C  just  at this

point...

Argument:

none

250 instruction crosses page boundary

Type:

warning

Reason:

The  parts of a  machine statement partiallly  lie on different

pages.  As the  CPU's instruction  counter does  not get incre-

mented  across page  boundaries, the  processor would  fetch at

runtime  the first byte of the old page instead of the instruc-

tion's following byte; the program

Argument:

none

255 range underflow

Type:

warning

Reason:

A  numeric value  was below  the allowed  range. AS brought the

value  back into  the allowed  range by  truncating upper bits,

but  it is not  guaranteed that meaningful  and correct code is

generated by this.

Argument:

none

260 range overflow

Type:

warning

Reason:

A  numeric value  was above  the allowed  range. AS brought the

value  back into  the allowed  range by  truncating upper bits,

but  it is not  guaranteed that meaningful  and correct code is

generated by this.

Argument:

none

270 negative argument for DUP

Type:

warning

Reason:

The  repetition argument  of a  DUP directive  was smaller than

0. Analogous to a count of exactly 0, no data is stored.

Argument:

none

280 single  X operand interpreted as  indexed and not implicit address-

ing

Type:

warning

Reason:

A  single X operand may be  interpreted either as register X or

x-indexed  addressing  with  zero  displacement, since Motorola

does  not specify  this variant.  AS chooses  the latter, which

may not be the desired one.

Argument:

none

300 bit number will be truncated

Type:

warning

Reason:

This   instruction  only   operates  on   byte  resp.  longword

operands.  bit numbers beyond 7 resp.  31 will be treated modu-

lo-8 resp. modulo-32 by the CPU.

Argument:

none

310 invalid register pointer value

Type:

warning

Reason:

Valid  values for the  RP register range  from 0x00 to 0x70 re-

sp. 0xf0, because all other areas are unused on the Z8.

Argument:

none

320 macro argument redefined

Type:

warning

Reason:

A  macro parameter was  assigned two or  more different values.

This  may happen by usage of  keyword arguments. The last argu-

ment is actually used.

Argument:

name of the macro parameter

330 deprecated instruction

Type:

warning

Reason:

This  instruction  is  deprecated  and  should  not be used any

more in new programs.

Argument:

the instruction that should be used instead.

340 source operand is longer or same size as destination operand

Type:

warning

Reason:

The  source  operand's  size  is  larger  than  the destination

operand's  size,  expressed  in  bits.  Sign  or zero extension

does  not make sense with these arguments. See the CPU's refer-

ence manual for its behaviour in this situation.

Argument:

none

350 TRAP number represents valid instruction

Type:

warning

Reason:

A  TRAP with this  number uses the  same machine code  as a ma-

chine instruction supported by the CPU.

Argument:

none

360 Padding added

Type:

warning

Reason:

The  amount of bytes placed  in memory is odd;  one half of the

last 16 bit word remains unused.

Argument:

none

370 register number wraparound

Type:

warning

Reason:

The  start register number plus  the count of registers results

in a last register beyond the end of the register bank.

Argument:

the argument holding the register count

380 using indexed instead of indirect addressing

Type:

warning

Reason:

Indirect  addressing is not allowed at this place. Instead, in-

dexed  addressing  with  a  dummy  displacement of zero will be

used.

Argument:

the argument holding the indirect addressing expression

390 not allowed in normal mode

Type:

warning

Reason:

This  machine instruction  is only  allowed in  panel mode, not

during ''normal operation''.

Argument:

the machine instruction in question

400 not allowed in panel mode

Type:

warning

Reason:

This  machine instruction is only allowed during ''normal oper-

ation'', not in panel mode.

Argument:

the machine instruction in question

410 argument out of range

Type:

warning

Reason:

The  argument or the sum of  two arguments is outside the range

allowed  for this  instruction, though  the instruction princi-

pally provides room for larger values.

Argument:

the argument in question

420 attempt to skip multiword instruction

Type:

warning

Reason:

The  previous instruction was a skip instruction, which can on-

ly  skip  a  single  (half)  word.  The  current instruction is

longer  than one word, so a skip  would jump into the middle of

it.

Argument:

the multi-word instruction in question

430 implicit sign extension

Type:

warning

Reason:

As  part of executing this instruction, the processor will per-

form  a sign extension to the full register width. For the giv-

en  argument,  this  means  the  register's  upper bits will be

filled  with ones  and not  zeros. Depending  on the usage that

follows, this may be irrelevant or not.

Argument:

the value in question

440 numeric  value -128 means usage of E register's content (use liter-

al 'E' to avoid this warning)

Type:

warning

Reason:

On  the SC/MP, a  displacement of -128  means in this case that

the  actual displacement  is not  -128, but  instead taken from

the  E register. The  assembler cannot decide  for sure whether

this  was intended or  the accidental result  of a computation,

and therefore warns. Use the literal val

Argument:

the displacement argument in question

450 I/O address must be accessed via INS/OUTS

Type:

warning

Reason:

I/O  addresses  in  the  range  of  0  to  3 are located in the

processor  module  and  can  only  be  accessed via the INS and

OUTS instructios, not via IN or OUT.

Argument:

the address argument in question

460 CASE limit does not match number of branch addresses

Type:

warning

Reason:

The  CASE instruction expects  a branch table  with as many en-

tries,  as the  limit argument  says, plus  one. The  limit and

the number of branch addresses do not fit together.

Argument:

the limit argument

470 instruction assembled as NOP

Type:

warning

Reason:

The  machinne  code  assigned  to  this instruction is used for

different  purposes.  Since  this  instruction  anyway does not

perform any operation, it is assembled as a NOP.

Argument:

none

480 argument treated as vector

Type:

warning

Reason:

There  is no way  to umambiguously decide  whether the argument

is  an address or a  vector. Since the argument  is a plain nu-

meric  value not assigned  to any segment,  it is assumed to be

a vector.

Argument:

the argument in question

490 interpreting too large integer constant as float

Type:

warning

Reason:

Though  the argument  is a  syntactically correct  integer con-

stant,  its value is  outside of what  can internally be repre-

sented  as integer. The number is  therefore stored as a float-

ing point number.

Argument:

the argument in question

500 code generation outside of CODE segment

Type:

warning

Reason:

machine  code can  only be  generated inside  the CODE segment.

All  other  address  spaces  do  not  have  the  necessary word

width.

Argument:

none

1000 symbol double defined

Type:

error

Reason:

A  new value is assigned  to a symbol, using  a label or a EQU,

PORT,  SFR, LABEL,  SFRB or  BIT instruction:  however this can

be done only using SET/EVAL.

Argument:

the  name of  the offending  symbol, and  the line number where

it  was defined for the first time, according to the symbol ta-

ble.

1010 symbol undefined

Type:

error

Reason:

A  symbol is still not defined  in the symbol table, also after

a second pass.

Argument:

the name of the undefined symbol.

1020 invalid symbol name

Type:

error

Reason:

A  symbol does not  fulfill the requirements  that symbols must

have  to be considered  valid by AS.  Please pay attention that

more  stringent syntax rules exist  for macros and function pa-

rameters.

Argument:

the wrong symbol name

1030 reserved symbol name

Type:

error

Reason:

A  symbol is  valid by  itself, but  this specific  name is re-

served  for other purposes. It therefore cannot be used for us-

er-defined symbols.

Argument:

the symbol name in question

1090 invalid format

Type:

error

Reason:

The  instruction format used  does not exist  for this instruc-

tion.

Argument:

the known formats for this command

1100 useless attribute

Type:

error

Reason:

The  instruction (processor  or pseudo)  cannot be  used with a

point-suffixed attribute.

Argument:

none

1105 attribute may only be one character long

Type:

error

Reason:

The  attribute following a point  after an instruction must not

be longer or shorter than one character.

Argument:

none

1107 undefined attribute

Type:

error

Reason:

This instruction uses an invalid attribute.

Argument:

none

1110 wrong number of operands

Type:

error

Reason:

The  number of arguments issued  for the instruction (processor

or  pseudo)  does  not  conform  with  the  accepted  number of

operands.

Argument:

the expected number of arguments resp. operands

1112 failed splitting argument into parts

Type:

error

Reason:

For  some targets (e.g. DSP56000),  the comma-separated have to

be split into individual operands, which failed.

Argument:

none

1115 wrong number of operations

Type:

error

Reason:

The number of options given with this command is not correct.

Argument:

none

1120 addressing mode must be immediate

Type:

error

Reason:

The  instruction  can  be  used  only  with  immediate operands

(preceded by #).

Argument:

none

1130 invalid operand size

Type:

error

Reason:

Although  the operand  is of  the right  type, it does not have

the correct length (in bits).

Argument:

none

1131 conflicting operand sizes

Type:

error

Reason:

The operands used have different length (in bits)

Argument:

none

1132 undefined operand size

Type:

error

Reason:

It  is not possible  to estimate, from  the opcode and from the

operands,  the size of the operand  (a trouble with 8086 assem-

bly). You must define it with a BYTE or WORD PTR prefix.

Argument:

none

1133 expected integer or string, but got floating point number

Type:

error

Reason:

A  floating point  number cannot  be used  as argument  at this

place.

Argument:

the argument in question

1134 expected integer, but got floating point number

Type:

error

Reason:

A  floating point  number cannot  be used  as argument  at this

place.

Argument:

the argument in question

1136 expected floating point number, but got string

Type:

error

Reason:

A string cannot be used as argument at this place.

Argument:

the argument in question

1137 operand type mismatch

Type:

Error

Reason:

The  two arguments  of an  operator are  not of  same data type

(integer/float/string).

Argument:

keines

1138 expected string, but got integer

Type:

error

Reason:

An integer cannot be used as argument at this place.

Argument:

the argument in question

1139 expected string, but got floating point number

Type:

error

Reason:

An  floating point  number cannot  be used  as argument at this

place.

Argument:

the argument in question

1140 too many arguments

Type:

error

Reason:

No more than 20 arguments can be given to any instruction

Argument:

none

1141 expected integer, but got string

Type:

error

Reason:

A string cannot be used as argument at this place.

Argument:

the argument in question

1142 expected integer or floating point number, but got string

Type:

error

Reason:

A string cannot be used as argument at this place.

Argument:

the argument in question

1143 expected string

Type:

error

Reason:

Only  a string (enclosed in single quotes) may be used as argu-

ment at this place.

Argument:

the argument in question

1144 expected integer

Type:

error

Reason:

Only an integer number may be used as argument at this place.

Argument:

the argument in question

1145 expected  integer, floating point number  or string but got regis-

ter

Type:

error

Reason:

A register symbol may not be used as argument at this place.

Argument:

the argument in question

1146 expected integer or string

Type:

error

Reason:

A  floating point number or register  symbol may not be used as

argument at this place.

Argument:

the argument in question

1147 expected register

Type:

error

Reason:

Only an register may be used as argument at this place.

Argument:

the argument in question

1148 register symbol for different target

Type:

error

Reason:

The  used register  symbol was  defined for  a target different

from the current one and is not compatible.

Argument:

the argument in question

1149 expected floating point argument but got integer

Type:

error

Reason:

Only  a floating point argument may  be used at this place, but

an integer argument was given.

Argument:

the argument in question

1151 expected integer or floating point number but got register

Type:

error

Reason:

Only  an integer or floating point  number argument may be used

at this place, but a register was given.

Argument:

the argument in question

1152 expected integer or string but got register

Type:

error

Reason:

Only  an integer or string argument  may be used at this place,

but a register was given.

Argument:

the argument in question

1153 expected integer but got register

Type:

error

Reason:

Only  an integer argument may be used at this place, but a reg-

ister was given.

Argument:

the argument in question

1154 string too long

Type:

error

Reason:

The  string is too  long to be  represented with leading length

byte.

Argument:

the argument in question

1200 unknown instruction

Type:

error

Reason:

An  instruction  was  used  that  is neither an AS instruction,

nor  a  known  macine  instruction  for  the  current processor

type.

Argument:

none

1300 number of opening/closing brackets does not match

Type:

error

Reason:

The  expression parser  found an  expression enclosed by paren-

theses,  where the  number of  opening and  closing parentheses

does not match.

Argument:

the wrong expression

1310 division by 0

Type:

error

Reason:

An  expression on the right side of a division or modulus oper-

ation was found to be equal to 0.

Argument:

none

1315 range underflow

Type:

error

Reason:

An integer word underflowed the allowed range.

Argument:

the  value of the word and  the allowed minimum (in most cases,

maybe I will complete this one day...)

1320 range overflow

Type:

error

Reason:

An integer word overflowed the allowed range.

Argument:

the  value of the world, and  the allowed maximum (in most cas-

es, maybe I will complete this one day...)

1322 not a power of two

Type:

error

Reason:

only powers of two (1,2,4,8,...) are allowed at this place.

Argument:

The value in question

1323 invalid decimal digit

Type:

error

Reason:

A  string as  argument to  PACKED may  only contain the charac-

ters  from 0 to 9,  and optionally a plus  or minus sign at the

beginning.

Argument:

The argument in question

1324 decimal string too long

Type:

error

Reason:

A packed decimal number must not be longer than 31 digits.

Argument:

The argument in question

1325 address is not properly aligned

Type:

error

Reason:

The  given address does not correspond  with the size needed by

the  data transfer, i.e. it is  not an integral multiple of the

operand size. Not all processor types can use unaligned data.

Argument:

none

1330 distance too big

Type:

error

Reason:

The displacement used for an address is too large.

Argument:

none

1331 target not on same page

Type:

error

Reason:

Instruction  and operand  address must  be located  in the same

memory page.

Argument:

the address argument in question

1340 short addressing not allowed

Type:

error

Reason:

The  address of  the operand  is outside  of the  address space

that can be accessed using short-addressing mode.

Argument:

none

1350 addressing mode not allowed here

Type:

error

Reason:

the  addressing  mode  used,  although usually possible, cannot

be used here.

Argument:

none

1351 address must be even

Type:

error

Reason:

At  this point, only even addresses  are allowed, since the low

order bits are used for other purposes or are reserved.

Argument:

the argument in question

1352 address must be aligned

Type:

error

Reason:

At  this point, only aligned (i.e.  a mulitple of 2,4,8...) ad-

dresses  are allowed,  since the  low order  bits are  used for

other purposes or are reserved.

Argument:

the argument in question

1355 addressing mode not allowed in parallel operation

Type:

error

Reason:

The  addressing  mode(s)  used  are  allowed in sequential, but

not in parallel instructions

Argument:

none

1360 undefined condition

Type:

error

Reason:

The  branch condition used for a  conditional jump does not ex-

ist.

Argument:

none

1365 incompatible conditions

Type:

error

Reason:

The  used combination of  conditions is not  possible in a sin-

gle instruction.

Argument:

the condition where the incompatibility was detected.

1366 unknown flag

Type:

error

Reason:

The given flag does not exist.

Argument:

the argument using the flag in question

1367 duplicate flag

Type:

error

Reason:

The given flag has already been used in the list of flags.

Argument:

the argument duplicating the flag

1368 unknown interrupt

Type:

error

Reason:

The given interrupt does not exist.

Argument:

the argument using the interrupt in question

1369 duplicate interrupt

Type:

error

Reason:

The  given interrupt has  already been used  in the list of in-

terrupt.

Argument:

the argument duplicating the interrupt

1370 jump distance too big

Type:

error

Reason:

the  jump instruction and destination  are too apart to execute

the jump with a single step

Argument:

none

1371 jump distance is zero

Type:

error

Reason:

the  jump  destination  is  right  behind the jump instruction,

and a jump distance of zero cannot be encoded.

Argument:

the target address in source code

1375 jump distance is odd

Type:

error

Reason:

Since  instruction must only be  located at even addresses, the

jump  distance between  two instructions  must always  be even,

and  the LSB of  the jump distance  is used otherwise. This is-

sue  was not verified here. The  reason is usually the presence

of an odd number of data in bytes or a wro

Argument:

none

1376 skip target mismatch

Type:

error

Reason:

The  gien branch target is not  the address the processor would

jump to if the skip instruction were executed.

Argument:

the given (intended) jump target

1380 invalid argument for shifting

Type:

error

Reason:

only  a constant  or a  data register  can be used for defining

the shift size. (only for 680x0)

Argument:

none

1390 operand must be in range 1..8

Type:

error

Reason:

constants  for shift size  or ADDQ argument  can be only within

the 1..8 range (only for 680x0)

Argument:

none

1400 shift amplitude too big

Type:

error

Reason:

(no more used)

Argument:

none

1410 invalid register list

Type:

error

Reason:

The  register list argument of MOVEM or FMOVEM has a wrong for-

mat (only for 680x0)

Argument:

none

1420 invalid addressing mode for CMP

Type:

error

Reason:

The  operand combination used  with the CMP  instruction is not

allowed (only for 680x0)

Argument:

none

1430 invalid CPU type

Type:

error

Reason:

The  processor type  used as  argument for  CPU command  is un-

known to AS.

Argument:

the unknown processor type

1431 invalid FPU type

Type:

error

Reason:

The  co-processor type used as argument  for FPU command is un-

known to AS.

Argument:

the unknown co-processor type

1432 invalid PMMU type

Type:

error

Reason:

The  MMU type used  as argument for  PMMU command is unknown to

AS.

Argument:

the unknown MMU type

1440 invalid control register

Type:

error

Reason:

The  control register  used by  a MOVEC  is not (yet) available

for the processor defined by the CPU command.

Argument:

none

1445 invalid register

Type:

error

Reason:

The  register used, although valid, cannot be used in this con-

text.

Argument:

none

1446 register(s) listed more than once

Type:

error

Reason:

A  register appears more than once  in the list of registers to

be saved or restored.

Argument:

none

1447 register bank mismatch

Type:

error

Reason:

An address expression uses registers from different banks.

Argument:

the register in question

1448 undefined register length

Type:

error

Reason:

Registers  of different  size may  be used  at this  place, and

the register length cannot be deduced from the address alone.

Argument:

the argument in question

1449 invalid operation on register

Type:

error

Reason:

This  operation may not  be applied to  this register, e.g. be-

cause the register is read-only or write-only.

Argument:

the register in question

1450 RESTORE without SAVE

Type:

error

Reason:

A  RESTORE command  was found,  that cannot  be coupled  with a

corresponding SAVE.

Argument:

none

1460 missing RESTORE

Type:

error

Reason:

After the assembling pass, a SAVE command was missing.

Argument:

none.

1465 unknown macro control instruction

Type:

error

Reason:

A macro option parameter is unknown to AS.

Argument:

the dubious option.

1470 missing ENDIF/ENDCASE

Type:

error

Reason:

after  the  assembling,  some  of  the  IF- or CASE- constructs

were found without the closing command

Argument:

none

1480 invalid IF-structure

Type:

error

Reason:

The command structure in a IF- or SWITCH- sequence is wrong.

Argument:

none

1483 section name double defined

Type:

error

Reason:

In  this program module a section  with the same name still ex-

ists.

Argument:

the multiple-defined name

1484 unknown section

Type:

error

Reason:

In the current scope, there are no sections with this name

Argument:

the unknown name

1485 missing ENDSECTION

Type:

error

Reason:

Not all the sections were properly closed.

Argument:

none

1486 wrong ENDSECTION

Type:

error

Reason:

The  given ENDSECTION does not refer  to the most deeply nested

one.

Argument:

none

1487 ENDSECTION without SECTION

Type:

error

Reason:

An  ENDSECTION command  was found,  but the  associated section

was not defined before.

Argument:

none

1488 unresolved forward declaration

Type:

error

Reason:

A  symbol  declared  with  a  FORWARD or PUBLIC statement could

not be resolved.

Argument:

the  name of  the unresolved  symbol, plus  the position of the

forward declaration in the source.

1489 conflicting FORWARD <-> PUBLIC-declaration

Type:

error

Reason:

A symbol was defined both as public and private.

Argument:

the name of the symbol.

1490 wrong numbers of function arguments

Type:

error

Reason:

The  number of arguments  used for referencing  a function does

not  match the number of arguments defined in the function def-

inition.

Argument:

none

1491 duplicate function argument name

Type:

error

Reason:

Two or more arguments of a function have the same name.

Argument:

the argument with duplicate name.

1495 unresolved literals (missing LTORG)

Type:

error

Reason:

At  the end of the program,  or just before switching to anoth-

er processor type, unresolved literals still remain.

Argument:

none

1500 instruction not allowed on

Type:

error

Reason:

Although  the instruction  is correct,  it cannot  be used with

the selected member of the CPU family.

Argument:

The processor variants that would support this instruction.

1501 FPU instructions are not enabled

Type:

error

Reason:

FPU  instruction set extensions must be enabled to use this in-

struction.

Argument:

none

1502 PMMU instructions are not enabled

Type:

error

Reason:

PMMU  instruction set  extensions must  be enabled  to use this

instruction.

Argument:

none

1503 full PMMU instruction set is not enabed

Type:

error

Reason:

This  instrction is  only contained  in the 68851's instruction

set,  not in the reduced instruction  set of the integrated PM-

MU.

Argument:

none

1504 Z80 syntax was not allowed

Type:

error

Reason:

This  instruction is only  allowed if Z80  syntax for 8080/8085

instructions has been enabled.

Argument:

none

1505 addressing mode not allowed on

Type:

error

Reason:

Although  the  addressing  mode  used  is correct, it cannot be

used with the selected member of the CPU family.

Argument:

The  processor  variants  that  would  support  this addressing

mode.

1506 not allowed in exclusive Z80 syntax mode

Type:

error

Reason:

This  instrction is no  longer allowed if  exclusive Z80 syntax

mode for 8080/8085 instructions has been set.

Argument:

none

1507 FPU instruction not supported on ...

Type:

error

Reason:

Although  this  FPU  instruction  exists,  it cannot be used on

the selected type of FPU.

Argument:

The instruction in question

1508 Custom instructions are not enabled

Type:

error

Reason:

Custom  instruction set extensions must  be enabled to use this

instruction.

Argument:

The instruction in question

1509 instruction extension not enabled

Type:

error

Reason:

This  instruction is part  of an extension  whose usage has not

been enabled.

Argument:

The extension's name

1510 invalid bit position

Type:

error

Reason:

Either  the number  of bits  specified is  not allowed,  or the

command is not completely specified.

Argument:

none

1520 only ON/OFF allowed

Type:

error

Reason:

This pseudo command accepts as argument either ON or OFF

Argument:

none

1530 stack is empty or undefined

Type:

error

Reason:

It  was tried  to access  a stack  via a  POPV instruction that

was either never defined or already emptied.

Argument:

the name of the stack in question

1540 not exactly one bit set

Type:

error

Reason:

Not  exactly one  bit was  set in  a mask  passed to the BITPOS

function.

Argument:

none

1550 ENDSTRUCT without STRUCT

Type:

error

Reason:

An  ENDSTRUCT instruction  was found  though there is currently

no structure definition in progress.

Argument:

none

1551 open structure definition

Type:

error

Reason:

After  end of assembly,  not all STRUCT  instructions have been

closed with appropriate ENDSTRUCTs.

Argument:

the innermost, unfinished structure definition

1552 wrong ENDSTRUCT

Type:

error

Reason:

the  name parameter of an ENDSTRUCT instruction does not corre-

spond to the innermost open structure definition.

Argument:

none

1553 phase definition not allowed in structure definition

Type:

error

Reason:

What  should I  say about  that? PHASE  inside a  record simply

does not make sense and only leads to confusion...

Argument:

none

1554 invalid STRUCT directive

Type:

error

Reason:

Only  EXTNAMES, NOEXTNAMES, DOTS, and NODOTS are allowed as di-

rectives of a STRUCT statement.

Argument:

the unknown directive

1555 structure re-defined

Type:

error

Reason:

A structure of this name has already been defined.

Argument:

the name of the structure

1556 unresolvable structure element reference

Type:

error

Reason:

An  element in a structure  references to another element, how-

ever  this referenced element was not  defined or itself has an

unresolvable reference.

Argument:

the name of the element itself and the referenced one

1557 duplicate structure element

Type:

error

Reason:

The structure already contains an element of this name.

Argument:

name of the element

1560 instruction is not repeatable

Type:

error

Reason:

This  machine  instruction  cannot  be  repeated via a RPT con-

struct.

Argument:

none

1600 unexpected end of file

Type:

error

Reason:

It  was tried to  read past the  end of a  file with a BINCLUDE

statement.

Argument:

none

1700 ROM-offset must be in range 0..63

Type:

error

Reason:

The ROM table of the 680x0 coprocessor has only 64 entries.

Argument:

none

1710 invalid function code

Type:

error

Reason:

The  only function code arguments allowed  are SFC, DFC, a data

register,  or a  constant in  the interval  of 0..15  (only for

680x0 MMU).

Argument:

none

1720 invalid function code mask

Type:

error

Reason:

Only  a number  in the  interval 0..15  can be used as function

code mask (only for 680x0 MMU)

Argument:

none

1730 invalid MMU register

Type:

error

Reason:

The  MMU  does  not  have  a  register with this name (only for

680x0 MMU).

Argument:

none

1740 level must be in range 0..7

Type:

error

Reason:

The  level  for  PTESTW  and  PTESTR  must be a constant in the

range of 0...7 (only for 680x0 MMU).

Argument:

none

1750 invalid bit mask

Type:

error

Reason:

The  bit mask used for  a bit field command  has a wrong format

(only for 680x0).

Argument:

none

1760 invalid register pair

Type:

error

Reason:

The  register here defined  cannot be used  in this context, or

there is a syntactic error (only for 680x0).

Argument:

none

1800 open macro definition

Type:

error

Reason:

An  incomplete  macro  definition  was  found. Probably an ENDM

statement is missing.

Argument:

none

1801 IRP without ENDM

Type:

error

Reason:

An  incomplete IRP block was  found. Probably an ENDM statement

is missing.

Argument:

none

1802 IRPC without ENDM

Type:

error

Reason:

An  incomplete IRPC  block was  found. Probably  an ENDM state-

ment is missing.

Argument:

none

1803 REPT without ENDM

Type:

error

Reason:

An  incomplete REPT  block was  found. Probably  an ENDM state-

ment is missing.

Argument:

none

1804 WHILE without ENDM

Type:

error

Reason:

An  incomplete WHILE block  was found. Probably  an ENDM state-

ment is missing.

Argument:

none

1805 EXITM not called from within macro

Type:

error

Reason:

EXITM  is  designed  to  terminate  a macro expansion. This in-

struction  only makes  sense within  macros and  an attempt was

made to call it in the absence of macros.

Argument:

none

1810 more than 10 macro parameters

Type:

error

Reason:

A macro cannot have more than 10 parameters

Argument:

none

1811 keyword argument not defined in macro

Type:

error

Reason:

a  keyword argument  referred to  a parameter  the called macro

does not provide.

Argument:

used keyword resp. macro parameter

1812 positional argument no longer allowed after keyword argument

Type:

Fehler

Reason:

position  and  keyword  arguments  may  be  mixed  in one macro

call,  however  only  keyword  arguments  are allowed after the

first keyword argument.

Argument:

none

1815 macro double defined

Type:

error

Reason:

A macro was defined more than once in a program section.

Argument:

the multiply defined macro name.

1820 expression must be evaluatable in first pass

Type:

error

Reason:

The  command used has  an influence on  the length of the emit-

ted code, so that forward references cannot be resolved here.

Argument:

none

1830 too many nested IFs

Type:

error

Reason:

(no more implemented)

Argument:

none

1840 ELSEIF/ENDIF without IF

Type:

error

Reason:

A  ELSEIF- or  ENDIF- command  was found,  that is not preceded

by an IF- command.

Argument:

none

1850 nested / recursive macro call

Type:

error

Reason:

(no more implemented)

Argument:

none

1860 unknown function

Type:

error

Reason:

The function invoked was not defined before.

Argument:

The name of the unknown function

1870 function argument out of definition range

Type:

error

Reason:

The  argument does not belong to the allowed argument range as-

sociated to the referenced function.

Argument:

none

1880 floating point overflow

Type:

error

Reason:

Although  the argument is within the range allowed to the func-

tion arguments, the result is not valid

Argument:

none

1890 invalid value pair

Type:

error

Reason:

The  base-exponent pair used  in the expression  cannot be com-

puted

Argument:

none

1900 instruction must not start on this address

Type:

error

Reason:

No  jumps can  be performed  by the  selected CPU from this ad-

dress.

Argument:

none

1905 invalid jump target

Type:

error

Reason:

No  jumps  can  be  performed  by  the selected CPU to this ad-

dress.

Argument:

none

1910 jump target not on same page

Type:

error

Reason:

Jump command and destination must be in the same memory page.

Argument:

none

1911 jump target not in same section

Type:

error

Reason:

Jump  command and destination must be  in the same (64K) memory

section.

Argument:

none

1920 code overflow

Type:

error

Reason:

An  attempt was  made to  generate more  than 1024 code or data

bytes in a single memory page.

Argument:

none

1925 address overflow

Type:

error

Reason:

The  address  space  for  the  processor type actually used was

filled beyond the maximum allowed limit.

Argument:

none

1930 constants and placeholders cannot be mixed

Type:

error

Reason:

Instructions  that  reserve  memory,  and instructions that de-

fine  constants  cannot  be  mixed  in a single pseudo instruc-

tion.

Argument:

none

1940 code must not be generated in structure definition

Type:

error

Reason:

a  STRUCT construct is only designed  to describe a data struc-

ture  and not to create one; therefore, no instructions are al-

lowed that generate code.

Argument:

none

1950 parallel construct not possible here

Type:

error

Reason:

Either  these instructions  cannot be  executed in parallel, or

they  are not  close enough  each other,  to do parallel execu-

tion.

Argument:

none

1960 invalid segment

Type:

error

Reason:

The referenced segment cannot be used here.

Argument:

The name of the segment used.

1961 unknown segment

Type:

error

Reason:

The  segment referenced with  a SEGMENT command  does not exist

for the CPU used.

Argument:

The name of the segment used

1962 unknown segment register

Type:

error

Reason:

The segment referenced here does not exist (8086 only)

Argument:

none

1970 invalid string

Type:

error

Reason:

The string has an invalid format.

Argument:

none

1980 invalid register name

Type:

error

Reason:

The  referenced register does  not exist, or  it cannot be used

here.

Argument:

none

1985 invalid argument

Type:

error

Reason:

The command used cannot be performed with the REP-prefix.

Argument:

none

1990 indirect mode not allowed

Type:

error

Reason:

Indirect addressing cannot be used in this way

Argument:

none

1995 not allowed in current segment

Type:

error

Reason:

(no more implemented)

Argument:

none

1996 not allowed in maximum mode

Type:

error

Reason:

This register can be used only in minimum mode

Argument:

none

1997 not allowed in minimum mode

Type:

error

Reason:

This register can be used only in maximum mode

Argument:

none

2000 execution packet crosses address boundary

Type:

error

Reason:

An execution packet must not cross a 32-byte address boundary

Argument:

none

2001 multiple use of same execution unit

Type:

error

Reason:

One  of the  CPU's execution  units was  used more than once in

an execution packet

Argument:

the name of the execution unit

2002 multiple long read operations

Type:

error

Reason:

An  execution packet  contains more  than one  long read opera-

tion, which is not allowed

Argument:

one of the functional units executing a long read

2003 multiple long write operations

Type:

error

Reason:

An  execution packet contains  more than one  long write opera-

tion, which is not allowed

Argument:

one of the functional units executing a long write

2004 long read with write operation

Type:

error

Reason:

An  execution packet contains both a  long read and a write op-

eration, which is not allowed.

Argument:

one  of the  execution units  executing the  conflicting opera-

tions

2005 too many reads of one register

Type:

error

Reason:

The  same register was  referenced more than  four times in the

same execution packet.

Argument:

the name of the register referenced too often

2006 overlapping destinations

Type:

error

Reason:

The  same register was  written more than  one time in the same

instruction packet, which is not allowed.

Argument:

the name of the register in question

2008 too many absolute branches in one execution packet

Type:

error

Reason:

An  execution  packet  contains  more  than  one direct branch,

which is not allowed.

Argument:

none

2009 instruction cannot be executed on this unit

Type:

error

Reason:

This instruction cannot be executed on this functional unit.

Argument:

none

2010 invalid escape sequence

Type:

error

Reason:

The  special character  defined using  a backslash  sequence is

not defined

Argument:

none

2020 invalid combination of prefixes

Type:

error

Reason:

The  prefix combination here defined is not allowed, or it can-

not be translated into binary code

Argument:

none

2030 constants cannot be redefined as variables

Type:

error

Reason:

A  symbol  that  has  once  been  declared as constant with EQU

must not be modified afterwards with SET.

Argument:

the name of the symbol in question

2035 variables cannot be redefined as constants

Type:

error

Reason:

A  symbol  that  has  once  been  declared as variable with SET

must not be redeclared afterwards as constant (e.g. with EQU.

Argument:

the name of the symbol in question

2040 structure name missing

Type:

error

Reason:

A  structure's  definition  lacks  the  identifier name for the

new structure

Argument:

none

2050 empty argument

Type:

error

Reason:

Empty  strings must not  be used in  the argument list for this

statement

Argument:

none

2060 unimplemented instruction

Type:

error

Reason:

The  used machinen instruction is  principally known to the as-

sembler,  however, it is currently  not implemented, du to lack

of documentation from the processor manufacturer.

Argument:

the instruction that was used

2070 unnamed structure is not part of another structure

Type:

error

Reason:

An  unnamed structure or  union always must  be part of another

structure or union.

Argument:

none

2080 STRUCT ended by ENDUNION

Type:

error

Reason:

ENDUNION  may  only  be  used  to  finalize the definition of a

union and not of a structure.

Argument:

name of the structure (if available)

2090 Memory address mot on active memory page

Type:

error

Reason:

The  target address  is not  within the  page that is currently

addressable via the page register.

Argument:

none

2100 unknown macro expansion argument

Type:

error

Reason:

An argument to MACEXP could not be interpreted.

Argument:

the unknown argument

2105 too many macro expansion arguments

Type:

error

Reason:

The  number macro expansion arguments  exceeds the allowed lim-

it.

Argument:

the argument that busted the limit

2110 contradicting macro expansion specifications

Type:

error

Reason:

A  specification about  macro expansion  and its  precise oppo-

site may not be used in the same MACEXP instruction.

Argument:

none

2130 erwarteter Fehler nicht eingetreten

Type:

error

Reason:

An  error or warning announced via  EXPECT did not occur in the

instruction block terminated via ENDEXPECT.

Argument:

The error that was expected

2140 nesting of EXPECT/ENDEXPECT not allowed

Type:

error

Reason:

Code  blocks framed via EXPECT/ENDEXPECT must not contain nest-

ed EXPECT/ENDEXPECT blocks.

Argument:

none

2150 missing ENDEXPECT

Type:

error

Reason:

An  instruction block opened via EXPECT was not closed via END-

EXPECT.

Argument:

none

2160 ENDEXPECT without EXPECT

Type:

error

Reason:

There is no matching previous EXPECT to an ENDEXPECT.

Argument:

none

2170 no default checkpoint register defined

Type:

error

Reason:

No  checkpoint register  was specified  for a  type 12 instruc-

tion  and no  default checkpoint  register had  previously been

defined via the CKPT statement.

Argument:

none

2180 invalid bit field

Type:

error

Reason:

The bit field is not in the required syntax (start,count).

Argument:

the argument in question

2190 argument value missing

Type:

error

Reason:

Arguments must have the form 'variable=value'.

Argument:

the argument in question

2200 unknown argument

Type:

error

Reason:

This  variable is  not supported  by the  selected target plat-

form.

Argument:

the argument in question

2210 index register must be 16 bit

Type:

error

Reason:

Z8000 index registers must have a size of 16 bits (Rn).

Argument:

the argument in question

2211 I/O address register must be 16 bit

Type:

error

Reason:

Z8000  registers  used  to  address  I/O  addresses must have a

size of 16 bits (Rn).

Argument:

the argument in question

2212 address register in segmented mode must be 32 bit

Type:

error

Reason:

Z8000  registers to address memory  in segmented mode must have

a size of 32 bits (RRn).

Argument:

the argument in question

2213 address register in non-segmented mode must be 16 bit

Type:

error

Reason:

Z8000  registers to  address memory  in non-segmented mode must

have a size of 16 bits (Rn).

Argument:

the argument in question

2220 invalid structure argument

Type:

error

Reason:

The  argument does not  match any pattern  of allowed arguments

when expanding a structure.

Argument:

the argument in question

2221 too many array dimensions

Type:

error

Reason:

Arrays of structures are limited to being three-dimensional.

Argument:

the dimension argument that was 'too much'

2230 unknown integer notation

Type:

error

Reason:

The  given  integer  notation  does  not  exist, or the leading

plus resp. minus sign is missing.

Argument:

the argument in question

2231 invalid list of integer notations

Type:

error

Reason:

The  requested changes to the  list of usable integer notations

cannot  be applied, because  they would result  in a contradic-

tion.  Currently, the  only such  case are  0hex und 0oct which

cannot be used at the same time.

Argument:

none

2240 invalid scale

Type:

error

Reason:

The given argument cannot be used as scaling factor.

Argument:

the argument in question

2250 conflicting string options

Type:

error

Reason:

The  string option  is in  contradiction to  a previously given

option.

Argument:

the option in question

2251 unknown string option

Type:

error

Reason:

The string option does not exist.

Argument:

the option in question

2252 invalid cache invalidate mode

Type:

error

Reason:

Only data, instruction, or both caches may be invalidated.

Argument:

the argument in question

2253 invalid config list

Type:

error

Reason:

The  configuration  list  is  either syntactically incorrect or

contains invalid elements.

Argument:

The list in question or one of its elements

2254 conflicting config options

Type:

error

Reason:

The  option is  in contradiction  to a  previously given option

or repeats a previous one.

Argument:

the option in question

2255 unknown config option

Type:

error

Reason:

The option does not exist.

Argument:

the option in question

2260 invalid CBAR value

Type:

error

Reason:

This  value for  CBAR is  not allowed  (CA must  be larger than

BA).

Argument:

none

2270 page not accessible

Type:

error

Reason:

The  target address  is located  in a  memory page that is cur-

rently inaccessible.

Argument:

none

2280 field not accessible

Type:

error

Reason:

The  target address is  located in a  memory field that is cur-

rently inaccessible.

Argument:

none

2281 target not in same field

Type:

error

Reason:

Instruction  and  target  address  must  be located in the same

memory field.

Argument:

none

2290 invalid instruction combination

Type:

error

Reason:

These instructions may not be combined with each other.

Argument:

none

2300 unmapped character

Type:

error

Reason:

The  character  string  contains  a  charater  that  cannot  be

mapped.

Argument:

The string in question

2310 invalid length of multi character constant

Type:

error

Reason:

multi  character constants must be between one and four charac-

ters long.

Argument:

none

2320 no target set (use 'CPU ...' or '-cpu ...' to set one)

Type:

fatal

Reason:

No  target has  been set  so far.  The assembler therefore does

not know which target to generate code for.

Argument:

none

2330 invalid displacement length

Type:

error

Reason:

This  displacement length must  not be used  if this addressing

mode is used.

Argument:

none

10001 error in opening file

Type:

fatal

Reason:

An error was detected while trying to open a file for input.

Argument:

description of the I/O error

10002 error in writing listing

Type:

fatal

Reason:

An error happened while AS was writing the listing file.

Argument:

description of the I/O error

10003 file read error

Type:

fatal

Reason:

An error was detected while reading a source file.

Argument:

description of the I/O error

10004 file write error

Type:

fatal

Reason:

While AS was writing a code or share file, an error happened.

Argument:

description of the I/O error

10006 heap overflow

Type:

fatal

Reason:

The  memory available is not enough to store all the data need-

ed by AS. Try using the DPMI or OS/2 version of AS.

Argument:

none

10007 stack overflow

Type:

fatal

Reason:

The  program stack crashed, because  too complex formulas, or a

bad  disposition  of  symbols  and/or  macros  were  used.  Try

again, using AS with the option -A.

Argument:

none

10008 INCLUDE nested too deeply

Type:

fatal

Reason:

The  include nesting  depth has  exceeded the  given limit (200

by  default). The limit may be raised via the -maxinclevel com-

mand  line argument,  a wrong  (recursive) inclusion is however

the more probable cause.

Argument:

the INCLUDE statement that exceeded the limit

10010 invalid place holder in listing per-line prefix format

Type:

fatal

Reason:

Only %i, %n, or %a are allowed as place holder.

Argument:

the invalid format

10011 place holder used too often in listing per-line prefix format

Type:

fatal

Reason:

The  place holders %i  and %n each  must not be  used more than

three  times in  the format  string. You're  not satisfied with

that?

Argument:

the format used more than once

B. I/O Error Messages

=====================

The  following error  messages are  generated not  only by AS, but also

by  the auxiliary  programs, like  PLIST, BIND,  P2HEX, and P2BIN. Only

the  most probable error  messages are here  explained. Should you meet

an  undocumented error  message, then  you probably  met a program bug!

Please inform us immediately about this!!

2 file not found

The  file  requested  does  not  exist,  or it is stored on another

drive.

3 path not found

The path of a file does not exist, or it is on another drive.

4 too much open files

There  are no  more file  handles available  to DOS. Increase their

number  changing the  value associated  to FILES=  in the file CON-

FIG.SYS.

5 file access not allowed

Either  the network access rights do  not allow the file access, or

an attempt was done to rewrite or rename a protected file.

6 invalid file handler

12 invalid access mode

15 invalid drive letter

The required drive does not exist.

16 The file cannot be deleted

17 RENAME cannot be done on this drive

100 Unexpected end of file

A  file access tried to go beyond the end of file, although accord-

ing  to its structure this should  not happen. The file is probably

corrupted.

101 disk full

This is self explaining! Please, clean up !

102 ASSIGN failed

103 file not open

104 file not open for reading

105 file not open for writing

106 invalid numerical format

150 the disk is write-protected

When  you don't use a hard disk  as work medium storage, you should

sometimes remove the protecting tab from your diskette!

151 unknown device

you  tried  to  access  a  peripheral  unit that is unknown to DOS.

This  should not usually happen, since the name should be automati-

cally interpreted as a filename.

152 drive not ready

close the disk drive door.

153 unknown DOS function

154 invalid disk checksum

A  bad read error on the disk.  Try again; if nothing changes, ref-

ormat the floppy disk resp. begin to take care of your hard disk!

155 invalid FCB

156 position error

the  diskette/hard disk controller has not  found a disk track. See

nr. 154 !

157 format unknown

DOS cannot read the diskette format

158 sector not found

As  nr. 156,  but the  controller this  time could  not find a disk

sector in the track.

159 end of paper

You  probably redirected the  output of AS  to a printer. Assembler

printout can be veeery long...

160 device read error

The operating system detected an unclassificable read error

161 device write error

The operating system detected an unclassificable write error

162 general failure error

The  operating system  has absolutely  no idea  of what happened to

the device.

C. Programming Examples

=======================

I  often get  questions about  how to  realize certain  things. Some of

those  are asked frequently, and it  might be worth documenting the so-

lutions  in a 'Tips and  Tricks' corner. This chapter  is meant to col-

lect and document them:

C.1. 16 Bit Instructions via Macros

-----------------------------------

Many  8 bit  processors can  only process  eight bits  at once  (as the

name  already implies...). They however  often contain enough registers

to  concatenate two of  them to a  virtual '16 bit  accumulator'. If we

define  macros  to  operate  on  this virtual accumulator, they ideally

should  provide the same addressing modes as the 8 bit instructions im-

plemented  by the  hardware. To  achieve this,  macros somehow  have to

'parse' their arguments. How can this be accomplished?

As  an example, the Motorola 6800  contains two accumulators named A an

B.  It is straightforward to  treat them also as  a 16 bit accumulator.

Addressing  modes should be  the same as  for 8 bit arithmetic instruc-

tions, namely:

- immediate

- direct (address within first 256 bytes)

- extended (arbitrary address)

- indexed

Therefore,  a macro  implementing a  virtual 16  bit instruction has to

analyze the one or two arguments passsed to it:

1 Indexed addressing is the only mode using two arguments.

2 Immediate addressing is recognized by the leading hash character.

3 Checking  whether an address  is within the  first 256 bytes or not

may be left to the assembler.

The  (single) argument has  to be transformed  into a string to perform

step  2. This string  can then also  be used to  strip the leading hash

character,  to evaluate the actual  immediate value. The complete macro

looks like this:

subd    macro   ARG1,ARG2

if      "ARG2" != ""            ; indexed?

suba    (ARG1)+1,ARG2

sbcb    ARG1,ARG2

elseif                          ; not indexed?

_SARG1   set     "ARG1"           ; convert to string

if      substr(_SARG1,0,1)='#' ; immediate?

_SARG1    set     substr(_SARG1,1,strlen(_SARG1)-1) ; y->del #

suba    #lo(VAL(_SARG1))      ; ...and subtract lo/hi bytes

sbcb    #hi(VAL(_SARG1))

elseif                         ; no immediate->ext. or direct

suba    (ARG1)+1              ; and subtract lo/hi bytes

sbcb    ARG1

endif

endif

endm

Macro  arguments have deliberately been  written in all-uppercase. This

way,  the  macro  works  both  in case-sensitive and non-case-sensitive

mode. The usage of the macro looks like this:

subd    $0007                   ; direct

subd    $1234                   ; absolute

subd    #$55aa                  ; immediate

subd    $12,x                   ; indexed

Of  course, we want to  have more 16 bit  operations than just subtrac-

tion.  One could  write a  similar macro  for every  type of operation.

However,  there is a more elegant method.  A macro may itself contain a

macro  definition. So we can  define a sort of  'meta macro' which gets

the instruction names as arguments:

def16   macro   NEWINST,LOINST,HIINST

NEWINST macro   ARG1,ARG2

if      "ARG2" != ""            ; indexed?

LOINST  (ARG1)+1,ARG2

HIINST  ARG1,ARG2

elseif                          ; not indexed?

_SARG1   set     "ARG1"                 ; convert to string

if      substr(_SARG1,0,1)='#' ; immediate?

_SARG1    set     substr(_SARG1,1,strlen(_SARG1)-1) ; y->del #

LOINST  #lo(VAL(_SARG1))      ; ...and subtract lo/hi bytes

HIINST  #hi(VAL(_SARG1))

elseif                         ; no immediate->ext. or direct

LOINST  (ARG1)+1              ; ...and subtract lo/hi bytes

HIINST  ARG1

endif

endif

endm

endm

The remaining definitions now become one-liners:

def16   addd,adda,adcb

def16   subd,suba,sbcb

def16   andd,anda,andb

def16   ord,ora,orb

def16   eord,eora,eorb

D. Frequently Asked Questions

=============================

In  this chapter, I tried to collect some questions that arise very of-

ten  together with their answers. Answers  to the problems presented in

this  chapter might also be  found at other places  in this manual, but

one maybe does not find them immediately...

Q: I  am fed up with  DOS. Are there versions  of AS for other operat-

ing systems ?

A: Apart  from  the  protected  mode  version  that offers more memory

when  working  under  DOS,  ports  exist  for OS/2 and Unix systems

like  Linux (currently in  test phase). Versions  that help operat-

ing  system manufacturers located in  Redmont to become even richer

are  currently not  planned. I  will gladly  make the sources of AS

available  for someone else who wants  to become active in this di-

rection.  The C variant  is probably the  best way to  start a port

into  this direction. He should however  not expect support from me

that goes beyond the sources themselves...

Q: Is a support of the XYZ processor planned for AS?

A: New  processors are appearing all the time  and I am trying to keep

pace  by extending AS. The stack on my desk labeled ''undone'' how-

ever  never goes below  the 4 inch  watermark... Wishes coming from

users  of course play an important  role in the decision which can-

didates  will be done first. The  internet and the rising amount of

documentation  published  in  electronic  form make the acquisition

of  data books  easier than  it used  to be,  but it always becomes

difficult  when more exotic  or older architectures  are wanted. If

the  processor family  in question  is not  in the list of families

that  are planned (see chapter 1), adding  a data book to a request

will  have  a  highly  positive  influence. Borrowing books is also

fine.

Q: Having  a free  assembler is  really fine,  but I  now also had use

for  a disassembler...and a debugger...a simulator would also real-

ly be cool!

A: AS  is a project I work on in  leisure time, the time I have when I

do  not have to care  of how to make  my living. AS already takes a

significant  portion of that time, and  sometimes I make a time-out

to  use my  soldering iron,  enjoy a  Tangerine Dream CD, watch TV,

or  simply to fulfill  some basic human  needs... I once started to

write  the concept  of a  disassembler that  was designed to create

source  code  that  can  be  assembled and that automatically sepa-

rates  code and  data areas.  I quickly  stopped this project again

when  I realized that the remaining  time simply did not suffice. I

prefer  to work  on one  good program  than to  struggle for half a

dozen  of mediocre apps. Regarded that way, the answer to the ques-

tion is unfortunately ''no''...

Q: The  screen  output  of  AS  is  messed up with strange characters,

e.g. arrows and brackets. Why?

A: AS  will by default use some ANSI control sequences for screen con-

trol.  These  sequences  will  appear  unfiltered on your screen if

you  did not install an ANSI  driver. Either install an ANSI driver

or use the DOS command SET USEANSI=N to turn the sequences off.

Q: AS  suddenly terminates  with a  stack overflow  error while assem-

bling my program. Did my program become to large?

A: Yes  and No. Your program's symbol  table has grown a bit unsymmet-

rically  what lead to high recursion depths while accessing the ta-

ble.  Errors of this type especially happen in the 16-bit-OS/2 ver-

sion  of AS which  has a very  limited stack area.  Restart AS with

the  -A command  line switch.  If this  does not  help, too complex

formula  expression are also  a possible cause  of stack overflows.

In such a case, try to split the formula into intermediate steps.

Q: It  seems that AS  does not assemble  my program up  to the end. It

worked however with an older version of AS (1.39).

A: Newer  versions of AS no longer  ignore the END statement; they ac-

tually  terminate assembly  when an  END is encountered. Especially

older  include files  made by  some users  tended to contain an END

statement  at their end.  Simply remove the  superfluous END state-

ments.

Q: I  made an assembly listing  of my program because  I had some more

complicated  assembly errors in my  program. Upon closer investiga-

tion  of the listing,  I found that  some branches do  not point to

the desired target but instead to themselves!

A: This  effect happens  in case  of forward  jumps in the first pass.

The  formula parser  does not  yet have  the target  address in its

symbol  table, and  as it  is a  completely independent  module, it

has  to think of a value that  even does not hurt relative branches

with  short  displacement  lengths.  This  is  the  current program

counter  itself...in  the  second  pass,  the  correct values would

have  appeared, but  the second  pass did  not happen due to errors

in  the first one. Correct  the other errors first  so that AS gets

into  the second pass, and the  listing should look more meaningful

again.

Q: Assembly  of my  program works  perfectly, however  I get  an empty

file when I try to convert it with P2HEX or P2BIN.

A: You  probably  did  not  set  the  address filter correctly. By de-

fault,  the filter is disabled, i.e. all  data is copied to the HEX

or  binary file. It is however possible  to create an empty file if

a  manually set range  does not fit  to the addresses  used by your

program.

Q: I  cannot enter the dollar character  when using P2BIN or P2HEX un-

der  Unix. The automatic  address range setting  does not work, in-

stead I get strange error messages.

A: Unix  shells use the dollar character  for expansion of shell vari-

ables.  If you want  to pass a  dollar character to an application,

prefix  it with  a backslash  (\). In  the special  case of the ad-

dress  range specification  for P2HEX  and P2BIN,  you may also use

0x  instead of the dollar character, which removes this prblen com-

pletely.

Q: I  use AS on a Linux system,  the loader program for my target sys-

tem  however runs  on a  Windows machine.  To simplify things, both

systems  access the same network  drive. Unfortunately, the Windows

side refuses to read the hex files created by the Linux side :-(

A: Windows  and Linux systems use  slightly different formats for text

files  (hex files  are a  sort of  text files).  Windows terminates

every  line  with  the  characters  CR  (carriage  return)  and  LF

(linefeed),  however Linux  only uses  the linefeed.  It depends on

the  Windows  program's  'goodwill'  whether  it  will  accept text

files  in the Linux format or not. If not, it is possible to trans-

fer  the files via  FTP in ASCII  mode instead of  a network drive.

Alternatively,  the hex files can be  converted to the Windows for-

mat.  For example, the program unix2dos can  be used to do this, or

a small script under Linux:

awk '{print $0"\r"}' test.hex >test_cr.hex

Q: I  have a 16 bit  address in my program  and have to load its upper

and  lower half into  separate CUP registers.  How do I extract the

byte  halves? Other  assemblers have  built-in functions to accomo-

date this.

A: This  can be done  ''by hand'' with  the built-in logical and shift

operators.  However,  there  is  also  a file bitfuncs.inc that de-

fines the functions lo() respectively hi().

E. Pseudo-Instructions and Integer Syntax

=========================================

This  appendix is designed as  a quick reference to  look up all pseudo

instructions  provided by  AS. The  list is  ordered in  two parts: The

first  part lists the instructions that  are always available, and this

list  is followed by lists  that enumerate the instructions additional-

ly available for a certain processor family.

Instructions that are always available

......................................

=             :=           ALIGN        BINCLUDE     CASE

CHARSET       CPU          DEPHASE      DOTTEDSTRUCTS ELSE

ELSECASE      ELSEIF       END          ENDCASE      ENDIF

ENDM          ENDS         ENDSECTION   ENDSTRUCT    ENUM

ENUMCONF      ERROR        EQU          .EQU         EVAL

EXITM         FATAL        FORWARD      FUNCTION     GLOBAL

IF            IFB          IFDEF        IFEXIST      IFNB

IFNDEF        IFNEXIST     IFNSYMEXIST  IFNUSED      IFSYMEXIST

IFUSED        INCLUDE      INTSYNTAX    IRP          LABEL

LISTING       MACEXP       MACECP_DFT   MACEXP_OVR   MACRO

MESSAGE       NEWPAGE      NEXTENUM     ORG          .PAGE

PHASE         POPV         PUSHV        PRTEXIT      PRTINIT

PUBLIC        READ         RELAXED      REPT         .RESTORE

RESTOREENV    RORG         .SAVE        SAVEENV      SECTION

SEGMENT       .SET         SHARED       .SHIFT       STRUC

STRUCT        .SWITCH      TITLE        UNION        WARNING

WHILE

Additionally, there are:

- SET  as an alias to EVAL, unless  SET is already a machine instruc-

tion.

- SHIFT resp. SHFT, in case SHIFT is already a machine instruction.

- RESTORE  as an alias to RESTOREENV, unless RESTORE is already a ma-

chine instruction.

- SAVE  as an alias to SAVEENV, unless  SAVE is already a machine in-

struction.

- PAGE  resp. PAGESIZE,  in case  PAGE is  already a machine instruc-

tion.

- SWITCH  resp. SELECT, in case SWITCH  is already a machine instruc-

tion.

Motorola 680x0/MCF5xxx

......................

Default Integer Syntax: Motorola

DC[.<size>]   DS[.<size>]  FULLPMMU     FPU          PADDING

PMMU          REG          SUPMODE

Motorola 56xxx

..............

Default Integer Syntax: Motorola

DC            DS           PACKING      XSFR         YSFR

PowerPC

.......

Default Integer Syntax: C

BIGENDIAN     DB           DD           DN           DO

DQ            DS           DT           DW           REG

SUPMODE

IBM PALM

........

Default Integer Syntax: IBM

DB            DD           DN           DO           DQ

DS            DT           DW           PORT         REG

Motorola M-Core

...............

Default Integer Syntax: Motorola

DC[.<size>]   DS[.<size>]  REG          SUPMODE

Motorola XGATE

..............

Default Integer Syntax: Motorola

ADR           BYT          DC[.<size>]  DFS          DS[.<size>]

FCB           FCC          FDB          PADDING      REG

RMB

Motorola 68xx/Hitachi 63xx

..........................

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>]  DFS

DS[.<size>]   DW           FCB          FCC          FDB

PADDING       PRWINS(68HC11K4) RMB

Motorola/Freescale 6805/68HC(S)08

.................................

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>]  DFS

DS[.<size>]   DW           FCB          FCC          FDB

PADDING       RMB

Motorola 6809/Hitachi 6309

..........................

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DC[.<size>]

DFS           DS[.<size>]  DW           FCB          FCC

FDB           PADDING      PLAINBASE    RMB

Konami 052001

.............

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>] DFS

DS[.<size>]   DW           FCB          FCC          FDB

PADDING       RMB

Motorola 68HC12

...............

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>]  DFS

DS[.<size>] DW FCB         FCC          FDB

PADDING       RMB

NXP S12Z

........

Default Integer Syntax: Motorola

ADR           BYT          DB           DC[.<size>]  DEFBIT

DEFBITFIELD   DFS          DS[.<size>]  DW           FCB

FCC           FDB          PADDING      RMB

Motorola 68HC16

...............

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DC[.<size>]

DFS           DS[.<size>]  DW           FCB          FCC

FDB           PADDING      RMB

Freescale 68RS08

................

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DC[.<size>]

DFS           DS[.<size>]  DW           FCB          FCC

FDB           PADDING

Hitachi H8/300(L/H)

...................

Default Integer Syntax: Motorola

BIT           DC[.<size>]  DS[.<size>]  MAXMODE      PADDING

REG

Hitachi H8/500

..............

Default Integer Syntax: Motorola

ASSUME        BIT          COMPMODE     DATA         DC[.<size>]

DS[.<size>]   MAXMODE      PADDING      REG

Hitachi SH7x00

..............

Default Integer Syntax: Motorola

COMPLITERALS  DC[.<size>]  DS[.<size>]  LTORG        PADDING

REG           SUPMODE

Hitachi HMCS400

...............

Default Integer Syntax: Motorola

DATA          RES          SFR

Hitachi H16

...........

Default Integer Syntax: Motorola

BIT           DC[.<size>]  DS[.<size>]  REG          SUPMODE

65xx/MELPS-740

..............

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DCB

DDB           DFS          DS           DW           FCB

FCC           FDB          RMB

65816/MELPS-7700

................

Default Integer Syntax: Motorola

ADR           ASSUME       BYT          DB           DD

DDB           DN           DO           DQ           DS

DT            DW           DFS          FCB          FCC

FDB           RMB

Mitsubishi MELPS-4500

.....................

Default Integer Syntax: Motorola

DATA          RES          SFR

Rockwell PPS-4

..............

Default Integer Syntax: Intel

DATA          DC           DS           RES

Mitsubishi M16

..............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           REG

Mitsubishi M16C

...............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           REG

DEC PDP-11

..........

Default Integer Syntax: C

BYTE          CIS          EIS          FIS          FLT2

FLT4          FP11         REG          SUPMODE      WORD

WD16

....

Default Integer Syntax: C

ASCII         ASCIZ       BYTE          FLT3         REG

WORD

DEC VAX

.......

ACCMODE       ASCIC        ASCII        ASCIZ        BLKB

BLKD          BLKF         BLKG         BLKH         BLKL

BLKO          BLKQ         BLKW         BYTE         D_FLOATING

DOUBLE        F_FLOATING   FLOAT        G_FLOATING   H_FLOATING

LONG          OCTA         QUAD         WORD

Olympia CP-3F/GI LP8000/SGS M380

................................

Default Integer Syntax: Intel

DC            DS           PORT

Intel 4004/4040

...............

Default Integer Syntax: Intel

DATA          DS           REG

Intel 8008

..........

Default Integer Syntax: Intel

DB            DD           DFB          DN           DO

DQ            DS           DT           DW           PORT

Z80SYNTAX

Intel MCS-48

............

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           REG

Intel MCS-(2)51

...............

Default Integer Syntax: Intel

BIGENDIAN     BIT          DB           DD           DN

DO            DQ           DS           DT           DW

PORT          REG          SFR          SFRB         SRCMODE

Intel MCS-96

............

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW

Intel 8080/8085

...............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           PORT

Intel 8086/80186/NEC V20...V5x

..............................

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           PORT

REG

Intel i960

..........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           FPU          REG

SPACE         SUPMODE      WORD

Signetics 8X30x

...............

Default Integer Syntax: Motorola

LIV           RIV

Signetics 2650

..............

Default Integer Syntax: Motorola

ACON          BIGENDIAN    DB           DD           DN

DO            DQ           DS           DT           DW

RES

Philips XA

..........

Default Integer Syntax: Intel

ASSUME        BIT          DB           DC[.<size>]  DD

DN            DO           DQ           DS[.<size>]  DT

DW            PADDING      PORT         REG          SUPMODE

Atmel AVR

.........

Default Integer Syntax: C

BIT           DATA         DB           DD           DN

DO            DQ           DS           DT           DW

PACKING       PORT         REG          RES          SFR

AMD 29K

.......

Default Integer Syntax: C

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           EMULATED

ERG           SUPMODE

Siemens 80C166/167

..................

Default Integer Syntax: Intel

ASSUME        BIT          DB           DD           DN

DO            DQ           DS           DT           DW

REG

Zilog Zx80

..........

Default Integer Syntax: Intel

DB            DD           DEFB         DEFW         DN

DO            DQ           DS           DT           DW

EXTMODE       LWORDMODE    PRWINS(Z180) REG          WARNRELATIVE

Zilog Z8

........

Default Integer Syntax: Intel

DB            DEFBIT       DD           DN           DO

DQ            DS           DT           DW           REG

SFR

Zilog Z8000

...........

Default Integer Syntax: Intel

DB            DD           DEFBIT       DEFBITB      DN

DO            DQ           DS           DT           DW

PORT          REG

Xilinx KCPSM

............

Default Integer Syntax: Intel

CONSTANT      NAMEREG      REG

Xilinx KCPSM3

.............

Default Integer Syntax: Intel

CONSTANT      DB           DD           DN           DO

DQ            DS           DT           DW           NAMEREG

PORT          REG

LatticeMico8

............

Default Integer Syntax: C

DB            DD           DN           DO           DQ

DS            DT           DW           PORT         REG

Toshiba TLCS-900

................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           MAXIMUM      SUPMODE

Toshiba TLCS-90

...............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Toshiba TLCS-870(/C)

....................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Toshiba TLCS-47(0(A))

.....................

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           PORT

Toshiba TLCS-42

...............

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           PORT

Toshiba TLCS-9000

.................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           REG

Toshiba TC9331

..............

Default Integer Syntax: Intel

Microchip PIC16C5x

..................

Default Integer Syntax: Motorola

DATA          RES          SFR          ZERO

Microchip PIC16C8x

..................

Default Integer Syntax: Motorola

DATA          RES          SFR          ZERO

Microchip PIC17C42

..................

Default Integer Syntax: Motorola

DATA          RES          SFR          ZERO

Parallax SX20

.............

Default Integer Syntax: Motorola

BIT           DATA         SFR          ZERO

SGS-Thomson ST6

...............

Default Integer Syntax: Intel

ASCII         ASCIZ        ASSUME       BIT          BYTE

BLOCK         SFR          WORD

SGS-Thomson ST7/STM8

....................

Default Integer Syntax: Intel

DC[.<size>]   DS[.<size>]  PADDING

SGS-Thomson ST9

...............

Default Integer Syntax: Intel

ASSUME        BIT          DB           DD           DN

DO            DQ           DS           DT           DW

REG

6804

....

Default Integer Syntax: Motorola

ADR           BYT          DB           DFS          DS

DW            FCB          FCC          FDB          RMB

SFR

Texas Instruments TMS3201x

..........................

Default Integer Syntax: Intel

DATA          PORT         RES

Texas Instruments TMS32C02x

...........................

Default Integer Syntax: Intel

BFLOAT        BSS          BYTE         DATA         DOUBLE

EFLOAT        TFLOAT       LONG         LQxx         PORT

Qxx           RES          RSTRING      STRING       WORD

Texas Instruments TMS320C3x/C4x

...............................

Default Integer Syntax: Intel

ASSUME        BSS          DATA         EXTENDED     PACKING

SINGLE        WORD

Texas Instruments TM32C020x/TM32C05x/TM32C054x

..............................................

Default Integer Syntax: Intel

BFLOAT        BSS          BYTE         DATA         DOUBLE

EFLOAT        TFLOAT       LONG         LQxx         PORT

Qxx           RES          RSTRING      STRING       WORD

Texas Instruments TMS320C6x

...........................

Default Integer Syntax: Intel

BSS           DATA         DOUBLE       PACKING      SINGLE

WORD

Texas Instruments TMS99xx

.........................

Default Integer Syntax: Intel

BSS           BYTE         DOUBLE       PADDING      SINGLE

WORD

Texas Instruments Instruments TMS1000

.....................................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Texas Instruments TMS70Cxx

..........................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Texas Instruments TMS370

........................

Default Integer Syntax: Intel

DB            DBIT         DD           DN           DO

DQ            DS           DT           DW

Texas Instruments MSP430

........................

Default Integer Syntax: Intel

BSS           BYTE         PADDING      REG          WORD

National IMP-16

...............

Default Integer Syntax: IBM

ASCII         LTORG        PORT         WORD

National IPC-16/INS8900

.......................

Default Integer Syntax: IBM

ASCII         ASSUME       LTORG        WORD

National SC/MP

..............

Default Integer Syntax: C

ASCII         BIGENDIAN    BYTE         DB           DBYTE

DD            DN           DO           DQ           DS

DT            DW           REG

National INS807x

................

Default Integer Syntax: C

DB            DD           DN           DO           DQ

DS            DT           DW

National COP4

.............

Default Integer Syntax: C

ADDR          ADDRW        BYTE         DB           DD

DO            DQ           DS           DSB          DSW

DT            DW           FB           FW           SFR

WORD

National COP8

.............

Default Integer Syntax: C

ADDR          ADDRW        BYTE         DB           DD

DQ            DS           DSB          DSW          DT

DW            FB           FW           SFR          WORD

National SC14xxx

................

Default Integer Syntax: C

DC            DC8          DS           DS8          DS16

DW            DW16

National NS32xxx

................

BIGENDIAN     BYTE         CUSTOM       DB           DD

DOUBLE        DO           DQ           DS           DT

DW            FLOAT        FPU          LONG         PMMU

REG           SUPMODE      WORD

Fairchild ACE

.............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Fairchild F8

............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW           PORT

NEC �PD7800...�PD7806, �PD78(C)1x

.................................

Default Integer Syntax: Intel

ASSUME        DB           DD           DN           DO

DQ            DS           DT           DW           Z80SYNTAX

NEC �PD7807...�PD7809

.....................

Default Integer Syntax: Intel

ASSUME        DB           DEFBIT      DD            DN

DO            DQ           DS           DT           DW

NEC 75xx

........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

NEC �COM-43

...........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

NEC 75K0

........

Default Integer Syntax: Intel

ASSUME        BIT          DB           DD           DN

DO            DQ           DS           DT           DW

SFR

NEC 78K0

........

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

NEC 78K2

........

Default Integer Syntax: Intel

BIT           DB           DD           DN           DO

DQ            DS           DT           DW

NEC 78K3

........

Default Integer Syntax: Intel

BIT           DB           DD           DN           DO

DQ            DS           DT           DW

NEC 78K4

........

Default Integer Syntax: Intel

BIT           DB           DD           DN           DO

DQ            DS           DT           DW

NEC �PD772x

...........

Default Integer Syntax: Intel

DATA          PACKING      RES

NEC �PD77230

............

Default Integer Syntax: Intel

DS            DW           PACKING

NEC V60

.......

Default Integer Syntax: Intel

DC[.<size>]   DS[.<size>]  PADDING      REG          SUPMODE

Symbios Logic SYM53C8xx

.......................

Default Integer Syntax: C

DB            DD           DN           DO           DQ

DS            DT           DW

Fujitsu F^2MC8L

...............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

Fujitsu F^2MC16L

................

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

OKI OLMS-40

...........

Default Integer Syntax: Intel

DATA          RES          SFR

OKI OLMS-50

...........

Default Integer Syntax: Intel

DATA          RES          SFR

Panafacom MN161x

................

Default Integer Syntax: IBM

DC            DS           PACKING

Padauk PMC/PMS/PFSxxx

.....................

Default Integer Syntax: C

BIT           DATA         RES          SFR

Intersil 180x

.............

Default Integer Syntax: Intel

DB            DD           DN           DO           DQ

DS            DT           DW

XMOS XS1

........

Default Integer Syntax: Motorola

DB            DD           DQ           DN           DO

DS            DT           DW           REG

ATARI Vector

............

Default Integer Syntax: Motorola

MIL STD 1750

............

Default Integer Syntax: Intel

DATA          EXTENDED     FLOAT

KENBAK

......

Default Integer Syntax: Intel

BIT           DB           DD           DN           DO

DQ            DS           DT           DW           REG

CP-1600

.......

Default Integer Syntax: IBM (hex), C (oct)

BYTE          PACKING      RES          TEXT         WORD

ZERO

HP Nano Processor

.................

Default Integer Syntax: C

IM61x0

......

Default Integer Syntax: C

DC            DECIMAL      DS           LTORG        OCTAL

ZERO

Renesas RX

..........

Default Integer Syntax: Intel

BLKB          BLKW         BLKL         BLKD         BYTE

DOUBLE        ENDIAN       FLOAT        LWORD        WORD

Sharp SC61860

.............

Default Integer Syntax: Motorola

ADR           BYT          DB           DC           DFS

DS            DW           FCB          FCC          FDB

RMB

Sharp SC62015

.............

Default Integer Syntax: Intel

DN            DB           DD           DO           DQ

DS            DT           DW

F. Predefined Symbols

=====================

+---------------+-----------+------------+-----------------------------+

| Name          | Data Type | Definition | Meaning                     |

+---------------+-----------+------------+-----------------------------+

+---------------+-----------+------------+-----------------------------+

| ARCHITECTURE  | string    | predef.    | target platform AS was      |

|               |           |            | compiled for, in the style  |

|               |           |            | processor-manufacturer-     |

|               |           |            | operating system            |

| BIGENDIAN     | boolean   | dyn.(0)    | storage of constants MSB    |

|               |           |            | first?                      |

| CASESENSITIVE | boolean   | normal     | case sensitivity in symbol  |

|               |           |            | names?                      |

| CONSTPI       | float     | normal     | constant Pi (3.1415.....)   |

| DATE          | string    | predef.    | date of begin of assembly   |

| FALSE         | boolean   | predef.    | 0 = logically ''false''     |

| HASFPU        | boolean   | dyn.(0)    | coprocessor instructions    |

|               |           |            | enabled?                    |

| HASPMMU       | boolean   | dyn.(0)    | MMU instructions            |

|               |           |            | enabled?                    |

| INEXTMODE     | boolean   | dyn.(0)    | XM flag set for 4 Gbyte     |

|               |           |            | address space?              |

| INLWORDMODE   | boolean   | dyn.(0)    | LW flag set for 32 bit      |

|               |           |            | instructions?               |

| INMAXMODE     | boolean   | dyn.(0)    | processor in maximum        |

|               |           |            | mode?                       |

| INSUPMODE     | boolean   | dyn.(0)    | processor in supervisor     |

|               |           |            | mode?                       |

| INSRCMODE     | boolean   | dyn.(0)    | processor in source mode?   |

| FULLPMMU      | boolean   | dyn.(0/1)  | full PMMU instruction set   |

|               |           |            | allowed?                    |

| LISTON        | boolean   | dyn.(1)    | listing enabled?            |

| MACEXP        | boolean   | dyn.(1)    | expansion of macro con-     |

|               |           |            | structs in listing enabled? |

| MOMCPU        | integer   | dyn.       | number of target CPU        |

|               |           | (68008)    | currently set               |

| MOMCPUNAME    | string    | dyn.       | name of target CPU          |

|               |           | (68008)    | currently set               |

| MOMFILE       | string    | special    | current source file         |

|               |           |            | (including include files)   |

| MOMLINE       | integer   | special    | current line number in      |

|               |           |            | source file                 |

| MOMPASS       | integer   | special    | number of current pass      |

| MOMSECTION    | string    | special    | name of current section or  |

|               |           |            | empty string if out of any  |

|               |           |            | section                     |

| MOMSEGMENT    | string    | special    | name of address space       |

|               |           |            | currently selected          |

|               |           |            | with SEGMENT                |

| NESTMAX       | Integer   | dyn.(256)  | maximum nesting level       |

|               |           |            | of macro expansions         |

| PADDING       | Boolean   | dyn.(1)    | pad byte field to even      |

|               |           |            | count?                      |

| RELAXED       | Boolean   | dyn.(0)    | any syntax allowed integer  |

|               |           |            | constants?                  |

| PC            | Integer   | special    | current program counter     |

|               |           |            | (Thomson)                   |

| TIME          | String    | predef.    | time of begin of assembly   |

|               |           |            | (1st pass)                  |

| TRUE          | Integer   | predef.    | 1 = logically ''true''      |

| VERSION       | Integer   | predef.    | version of AS in BCD        |

|               |           |            | coding, e.g. 1331 hex for   |

|               |           |            | version 1.33p1              |

| WRAPMODE      | Integer   | predef.    | shortened program           |

|               |           |            | counter assumed?            |

|  *            | Integer   | special    | current program counter     |

|               |           |            | (Motorola, Rockwell,        |

|               |           |            | Microchip, Hitachi)         |

| .             | Integer   | special    | curr. program counter       |

|               |           |            | (IM61x0)                    |

| $             | Integer   | special    | current program counter     |

|               |           |            | Intel, Zilog, Texas,        |

|               |           |            | Toshiba, NEC, Siemens,      |

|               |           |            | AMD)                        |

+---------------+-----------+------------+-----------------------------+

To  be exact,  boolean symbols  are just  ordinary integer symbols with

the  difference that AS  will assign only  two different values to them

(0  or 1, corresponding  to False or  True). AS does  not store special

symbols  in the symbol  table. For performance  reasons, they are real-

ized  with hardcoded  comparisons directly  in the  parser. They there-

fore  do not  show up  in the  assembly listing's  symbol table. Prede-

fined  symbols are only set  once at the beginning  of a pass. The val-

ues  of dynamic symbols may in  contrast change during assembly as they

reflect  settings  made  with  related  pseudo instructions. The values

added  in  parentheses  give  the  value  present at the beginning of a

pass.

The  names given in this table also  reflect the valid way to reference

these symbols in case-sensitive mode.

The  names listed  here should  be avoided  for own symbols; either one

can  define but not access them  (special symbols), or one will receive

an  error message due  to a double-defined  symbol. The ugliest case is

when  the redefinition  of a  symbol made  by AS  at the beginning of a

pass leads to a phase error and an infinite loop...

G. Shipped Include Files

========================

The  distribution of AS contains a  couple of include files. Apart from

include  files  that  only  refer  to  a specific processor family (and

whose  function should be  immediately clear to  someone who works with

this  family), there  are a  few processor-independent  files which in-

clude  useful functions. The functions defined  in these files shall be

explained briefly in the following sections:

G.1. BITFUNCS.INC

-----------------

This  file defines  a couple  of bit-oriented  functions that  might be

hardwired  for other assemblers.  In the case  of AS however, thaey are

implemented with the help of user-defined functions:

-  mask(start,bits)  returns an  integer with  bits bits set starting

at position start;

-  invmask(start,bits) returns one's complement to mask();

-  cutout(x,start,bits)  returns bits  bits masked  out from x start-

ing at position start without shifting them to position 0;

-  hi(x) returns the second lowest byte (bits 8..15) of x;

-  lo(x) returns the lowest byte (bits 8..15) of x;

-  hiword(x) returns the second lowest word (bits 16..31) of x;

-  loword(x) returns the lowest word (bits 0..15) of x;

-  odd(x) returns TRUE if x is odd;

-  even(x) returns TRUE if x is even;

-  getbit(x,n) extracts bit n out of x and returns it as 0 or 1;

-  shln(x,size,n)  shifts a word  x of length  size to the  left by n

places;

-  shrn(x,size,n)  shifts a word x  of length size to  the right by n

places;

-  rotln(x,size,n)  rotates the lowest  size bits of  an integer x to

the left by n places;

-  rotrn(x,size,n)  rotates the lowest  size bits of  an integer x to

the right by n places;

G.2. CTYPE.INC

--------------

This  include file is similar  to the C include  file ctype.h which of-

fers  functions to  classify characters.  All functions  deliver either

TRUE or FALSE:

-  isdigit(ch) becomes TRUE if ch is a valid decimal digit (0..9);

-  isxdigit(ch)  becomes  TRUE  if  ch  is  a valid hexadecimal digit

(0..9, A..F, a..f);

-  isupper(ch)  becomes TRUE if ch  is an uppercase letter, excluding

special national characters);

-  islower(ch)  becomes TRUE if  ch is a  lowercase letter, excluding

special national characters);

-  isalpha(ch)  becomes TRUE if ch is a letter, excluding special na-

tional characters);

-  isalnum(ch)  becomes TRUE if ch is either a letter or a valid dec-

imal digit;

-  isspace(ch)  becomes TRUE  if ch  is an  'empty' character (space,

form feed, line feed, carriage return, tabulator);

-  isprint(ch)  becomes TRUE if ch is  a printable character, i.e. no

control character up to code 31;

-  iscntrl(ch) is the opposite to isprint();

-  isgraph(ch)  becomes TRUE if ch is a printable and visible charac-

ter;

-  ispunct(ch)  becomes TRUE if  ch is a  printable special character

(i.e. neither space nor letter nor number);

H. Acknowledgments

==================

''If I have seen farther than other men,

it is because I stood on the shoulders of giants.''

--Sir Isaac Newton

''If I haven't seen farther than other men,

it is because I stood in the footsteps of giants.''

--unknown

There  is a  commaon saying  that programs  you write are like children

you  put into the world. It is now  more than 30 years that I have been

working  on  this  assembler,  and  I  have come to the conclusion that

such  a project  is rather  a journey  for its  author. The  people you

meet  and learn to  know on this  journey are at  least as important as

the  perceived target itself. Your learn a  lot on this trip and in the

best  case, one also  understands that things  can also be  seen from a

completely  different perspective. If the  discussions are fruitful, it

is a win for both sides.

While  making this  journey, some  people have  kept a special place in

my  memories, because of  the way they  contributed to this project and

to  the state it  has now achieved.  The following enumeration of these

people  is naturally incomplete, since my  memories are not any more as

good  as they used to be. So the first 'thank you' goes to all the peo-

ple  I  (accidentally)  omit  in  the following paragraphs. The journey

goes on, and maybe our ways will be crossing again!

The  concept of  AS as  a universal  cross assembler came from Bernhard

(C.)  Zschocke who needed  a ''student friendly'',  i.e. free cross as-

sembler  for his microprocessor course and  talked me into extending an

already  existing 68000  assembler. The  rest is  history... The micro-

processor  course held at RWTH Aachen also always provided the most en-

gaged  users (and bug-searchers) of new  AS features and therefore con-

tributed a lot to today's quality of AS.

The  Internet and  FTP have  proved to  be a  big help for spreading AS

and  reporting  of  bugs.  My  thanks  therefore  go  to the FTP admins

(Bernd  Casimir  in  Stuttgart,  Norbert  Breidor in Aachen, and J�rgen

Mei�burger  in J�lich).  Especially the  last one  personally engaged a

lot to establish a practicable way in J�lich.

As  we are just talking  about the ZAM: Though  Wolfgang E. Nagel never

involved  personally into AS, he however was  my tutor and boss. He al-

ways  had at least four  eyes on what I  was doing. Regarding AS, there

seemed to be at least one that smiled...

A  program like  AS cannot  be done  without appropriate data books and

documentation.  I received information from  an enormous amount of peo-

ple,  ranging from tips up to  complete data books. An enumeration fol-

lows (as stated before, this is very probably incomplete!):

Ernst  Ahlers,  Charles  Altmann,  Marco  Awater,  Len  Bayles, Andreas

Bolsch,  Rolf  Buchholz,  Bernd  Casimir,  Nils  Eilers, Gunther Ewald,

Daniel  E. Germann, Michael Haardt,  Stephan Hruschka, Fred van Kempen,

Peter  Kliegelh�fer,  Ulf  Meinke,  Udo  M�ller, Matthias Paul, Norbert

Rosch,  Curt  J.  Sampson,  Steffen  Schmid,  Leonhard Schneider, Ernst

Schwab,  Michael  Schwingen,  Oliver  Sellke, Christian Stelter, Patrik

Str�mdahl,  Tadashi G.  Takaoka, Oliver  Thamm, Thorsten Thiele, Leszek

Ulman, Rob Warmelink, Andreas Wassatsch, John Weinrich.

...and  a somewhat  ironic ''thank  you'' to  Rolf-Dieter-Klein and To-

bias  Thiel, who gave me the  initial impulse with their ASM68K. Sever-

al  things did not work the  way I wanted them to  have, so I thought I

could do better or at least different.

And  of course, I did not entirely write  AS on my own. The DOS version

of  AS contained  the OverXMS  routines from  Wilbert van  Leijen which

can  move the overlay  modules into the  extended memory. A really nice

library, easy to use without problems!

The  TMS320C2x/5x code generators  and the file  STDDEF2x.INC come from

Thomas  Sailer, ETH Zurich.  It's surprising, he  only needed one week-

end  to understand my  coding and to  implement the new code generator.

Either  that was  a long  nightshift or  I am  slowly getting old...the

same  praise  goes  to  Haruo  Asano  for  providing the MN1610/MN1613,

IM6100, CP1600, Renesas RX and HP NanoProcessor code generators.

I. Changes since Version 1.3

============================

- version 1.31:

- additional  MCS-51 processor  type 80515.  The number  is again

only  stored by  the assembler.  The file  STDDEF51.INC was ex-

tended  by the necessary SFRs. CAUTION!  Some of the 80515 SFRs

have moved to other addresses!

- additional support for the Z80 processor;

- faster 680x0 code generator.

- version 1.32:

- syntax  for  zero  page  addresses  for  the  65xx  family  was

changed from addr.z to <addr (similar to 68xx);

- additional  support for the 6800,  6805, 6301, and 6811 proces-

sors;

- the  8051  part  now  also  understands  DJNZ,  PUSH,  and  POP

(sorry);

- the  assembly listing  now not  also list  the symbols but also

the macros that have been defined;

- additional  instructions IFDEF/IFNDEF  for conditional assembly

based on the existence of a symbol;

- additional  instructions  PHASE/DEPHASE  to  support  code that

shall be moved at runtime to a different address;

- additional  instructions WARNING, ERROR, and FATAL to print us-

er-defined error messages;

- the  file  STDDEF51.INC  additionally  contains the macro USING

to simplify working with the MCS-51's register banks;

- command line option u to print segment usage;

- version 1.33:

- additionally supports the 6809 processor;

- added string variables;

- The  instructions TITLE, PRTINIT,  PRTEXIT, ERROR, WARNING, and

FATAL  now expect a string  expression. Constants therefore now

have  to be enclosed in '' instead of ' characters. This is al-

so true for DB, DC.B, and BYT;

- additional  instruction ALIGN to align  the program counter for

Intel processors;

- additional  instruction LISTING  to turn  the generation  of an

assembly listing on or off;

- additional   instruction  CHARSET  for  user-defined  character

sets.

- version 1.34:

- the  second pass  is now  omitted if  there were  errors in the

first pass;

- additional  predefined  symbol  VERSION  that contains the ver-

sion number of AS;

- additional  instruction  MESSAGE  to  generate  additional mes-

sages under program control;

- formula parser is now accessible via string constants;

- if  an error  in a  macro occurs,  additionally the line number

in the macro itself is shown;

- additional  function UPSTRING to convert a string to all upper-

case.

- version 1.35:

- additional  function TOUPPER  to convert  a single character to

upper case;

- additional instruction FUNCTION for user-defined functions;

- additional  command line option  D to define  symbols from out-

side;

- the  environment variable ASCMD for  commonly used command line

options was introduced;

- the  program will additionally  be checked for  double usage of

memory areas if the u option is enabled;

- additional  command line  option C  to generate  a cross refer-

ence list.

- version 1.36:

- additionally  supports the PIC16C5x and PIC17C4x processor fam-

ilies;

- the  assembly listing  additionally shows  the nesting depth of

include files;

- the  cross  reference  list  additionally  shows the definition

point of a symbol;

- additional  command line option A to  force a more compact lay-

out of the symbol table.

- version 1.37:

- additionally  supports  the  processors  8086, 80186, V30, V35,

8087, and Z180;

- additional  instructions SAVE and RESTORE for an easier switch-

ing of some flags;

- additional operators for logical shifts and bit mirroring;

- command line options may now be negated with a plus sign;

- additional  filter AS2MSG for  a more comfortable  work with AS

under Turbo-Pascal 7.0;

- ELSEIF  now may have  an argument for  construction of IF-THEN-

ELSE ladders;

- additional  CASE construct  for a  more comfortable conditional

assembly;

- user-defined functions now may have more than one argument;

- P2HEX  can  now  additionally  generate  hex  files in a format

suitable for 65xx processors;

- BIND,  P2HEX, and  P2BIN now  have the  same scheme for command

line processing like AS;

- additional  switch i for  P2HEX to select  one out three possi-

bilities for the termination record;

- additional functions ABS and SGN;

- additional predefined symbols MOMFILE and MOMLINE;

- additional option to print extended error messages;

- additional  instruction IFUSED  and IFNUSED  to check whether a

symbol has been used so far;

- The  environment variables  ASCMD, BINDCMD  etc. now optionally

may  contain the  name of  a file  that provides more space for

options;

- P2HEX  can now generate the  hex formats specified by Microchip

(p4);

- a  page length specification of 0  now allows to suppress auto-

matic formfeeds in the assembly listing completely (p4);

- symbols  defined in the command line now may be assigned an ar-

bitrary value (p5).

- version 1.38:

- changed  operation to multipass mode. This enables AS to gener-

ate optimal code even in case of forward references;

- the  8051 part now also knows the generic JMP and CALL instruc-

tions;

- additionally supports the Toshiba TLCS-900 series (p1);

- additional  instruction  ASSUME  to  inform the assembler about

the 8086's segment register contents (p2);

- additionally supports the ST6 series from SGS-Thomson (p2);

- ..and  the 3201x signal processors from Texas Instruments (p2);

- additional  option F for  P2HEX to override  the automatic for-

mat selection (p2);

- P2BIN  now can automatically  set the start  resp. stop address

of the address window by specifying dollar signs (p2);

- the  8048 code  generator now  also knows  the 8041/42 instruc-

tion extensions (p2);

- additionally supports the Z8 microcontrollers (p3).

- version 1.39:

- additional opportunity to define sections and local symbols;

- additional  command line switch h  to force hexadecimal numbers

to use lowercase;

- additional  predefined  symbol  MOMPASS  to  read the number of

the currently running pass;

- additional  command line  switch t  to disable individual parts

of the assembly listing;

- additionally  knows the  L variant  of the  TLCS-900 series and

the MELPS-7700 series from Mitsubishi (p1);

- P2HEX  now also  accepts dollar  signs as  start resp. stop ad-

dress (p2);

- additionally supports the TLCS-90 family from Toshiba (p2);

- P2HEX  now also can  output data in  Tektronix and 16 bit Intel

Hex format (p2);

- P2HEX now prints warnings for address overflows (p2);

- additional  include file STDDEF96.INC  with address definitions

for the TLCS-900 series (p3);

- additional  instruction READ to allow interactive input of val-

ues during assembly (p3);

- error  messages are  written to  the STDERR  channel instead of

standard output (p3);

- the  STOP  instruction  missing  for  the 6811 is now available

(scusi, p3);

- additionally supports the �PD78(C)1x family from NEC (p3);

- additionally supports the PIC16C84 from NEC (p3);

- additional  command line  switch E  to redirect  error messages

to a file (p3);

- The MELPS-7700's 'idol' 65816 is now also available (p4);

- the  ST6 pseudo instruction  ROMWIN has been  removed was inte-

grated into the ASSUME instruction (p4);

- additionally supports the 6804 from SGS-Thomson (p4);

- via  the NOEXPORT option in a  macro definition, it is now pos-

sible  to define individually for  every macro whether it shall

appear in the MAC file or not (p4);

- the  meaning of  MACEXP regarding  the expansion  of macros has

changed  slightly due to the  additional NOEXPAND option in the

macro definition (p4);

- The  additional GLOBAL option in the macro definition now addi-

tionally  allows to define macros  that are uniquely identified

by their section name (p4).

- version 1.40:

- additionally supports the DSP56000 from Motorola;

- P2BIN  can now also extract the lower resp. upper half of a 32-

bit word;

- additionally  supports the  TLCS-870 and  TLCS-47 families from

Toshiba (p1);

- a  prefixed ! now  allows to reach  machine instructions hidden

by a macro (p1);

- the  GLOBAL instruction now allows to export symbols in a qual-

ified style (p1);

- the  additional r  command line  switch now  allows to  print a

list of constructs that forced additional passes (p1);

- it  is now possible to  omit an argument to  the E command line

option; AS will then choose a fitting default (p1);

- the  t command line option now  allows to suppress line number-

ing in the assembly listing (p1);

- escape  sequences may now  also be used  in ASCII style integer

constants (p1);

- the  additional pseudo  instruction PADDING  now allows  to en-

able  or disable the  insertion of padding  bytes in 680x0 mode

(p2);

- ALIGN is now a valid instruction for all targets (p2);

- additionally knows the PIC16C64's SFRs (p2);

- additionally supports the 8096 from Intel (p2);

- DC additionally allows to specify a repetition factor (r3);

- additionally  supports the TMS320C2x  family from Texas Instru-

ments  (implementation done by Thomas  Sailer, ETH Zurich, r3);

P2HEX has been extended appropriately;

- an equation sign may be used instead of EQU (r3);

- additional ENUM instruction to define enumerations (r3);

- END now has a real effect (r3);

- additional  command  line  switch  n  to get the internal error

numbers in addition to the error messages (r3);

- additionally supports the TLCS-9000 series from Toshiba (r4);

- additionally  supports the TMS370xxx  series from Texas Instru-

ments, including a new DBIT pseudo instruction (r5);

- additionally knows the DS80C320's SFR's (r5);

- the  macro processor  is now  also able  to include  files from

within  macros.  This  required  to  modify the format of error

messages  slightly. If you use AS2MSG,  replace it with the new

version! (r5)

- additionally supports the 80C166 from Siemens (r5);

- additional VAL function to evaluate string expressions (r5);

- it  is now possible to construct  symbol names with the help of

string expressions enclosed in braces (r5);

- additionally knows the 80C167's peculiarities (r6);

- the  MELPS740's special  page addressing  mode is now supported

(r6);

- it  is now  possible to  explicitly reference  a symbol  from a

certain  section by  appending its  name enclosed  in brackets.

The construction with an @ sign has been removed! (r6)

- additionally  supports  the  MELPS-4500  series from Mitsubishi

(r7);

- additionally  supports H8/300  and H8/300H  series from Hitachi

(r7);

- settings  made with LISTING  resp. MACEXP may  now be read back

from predefined symbols with the same names (r7);

- additionally  supports the TMS320C3x  series from Texas Instru-

ments (r8);

- additionally supports the SH7000 from Hitachi (r8);

- the Z80 part has been extended to also support the Z380 (r9);

- the  68K part has been extended  to know the differences of the

683xx micro controllers (r9);

- a  label not any more  has to be placed  in the first row if it

is marked with a double dot (r9);

- additionally supports the 75K0 series from NEC (r9);

- the  additional command  line option  o allows  to set  a user-

defined name for the code file (r9);

- the  ~~ operator has been moved  to a bit more senseful ranking

(r9);

- ASSUME  now also knows the 6809's DPR register and its implica-

tions (pardon, r9);

- the  6809  part  now  also  knows  the 6309's secret extensions

(r9);

- binary  constants now also may be  written in a C-like notation

(r9);

- version 1.41:

- the  new  predefined  symbol  MOMSEGMENT  allows to inquire the

currently active segment;

- := is now allowed as a short form for SET/EVAL;

- the  new command line switch q allows to force a ''silent'' as-

sembly;

- the  key word PARENT  to reference the  parent section has been

extended by PARENT0..PARENT9;

- the  PowerPC part has been extended by the microcontroller ver-

sions MPC505 and PPC403;

- symbols  defined with SET or EQU may  now be assigned to a cer-

tain segment (r1);

- the  SH7000 part  now also  knows the  SH7600's extensions (and

should compute correct displacements...) (r1);

- the  65XX part now differentiates  between the 65C02 and 65SC02

(r1);

- additionally  to the symbol MOMCPU, there  is now also a string

symbol  MOMCPUNAME  that  contains  the  processor's  full name

(r1);

- P2HEX  now also knows the 32-bit  variant of the Intel hex for-

mat (r1);

- additionally knows the 87C750's limitations (r2);

- the  internal numbers for  fatal errors have  been moved to the

area  starting  at  10000,  making  more space for normal error

messages (r2);

- unused  symbols are now marked with  a star in the symbol table

(r2);

- additionally supports the 29K family from AMD (r2);

- additionally supports the M16 family from Mitsubishi (r2);

- additionally supports the H8/500 family from Hitachi (r3);

- the  number of data bytes printed per  line by P2HEX can now be

modified (r3);

- the  number of the pass that  starts to output warnings created

by the r command line switch is now variable (r3);

- the  macro processor  now knows  a WHILE  statement that allows

to repeat a piece of code a variable number of times (r3);

- the  PAGE instruction now  also allows to  set the line with of

the assembly listing (r3);

- CPU  aliases may now be defined  to define new pseudo processor

devices (r3);

- additionally supports the MCS/251 family from Intel (r3);

- if  the cross  reference list  has been  enabled, the  place of

the  first definition is  given for double  definitions of sym-

bols (r3);

- additionally  supports the TMS320C5x  family from Texas Instru-

ments (implementation done by Thomas Sailer, ETH Zurich, r3);

- the  OS/2  version  should  now  also  correctly work with long

file  names.  If  one  doesn't  check  every s**t personally...

(r3);

- the  new pseudo instruction  BIGENDIAN now allows  to select in

MCS-51/251  mode whether constants should  be stored in big en-

dian or little endian format (r3);

- the  680x0  part  now  differentiates  between the full and re-

duced  MMU instruction  set; a  manual toggle  can be  done via

the FULLPMMU instruction (r3);

- the  new command line  option I allows  to print a  list of all

include files paired with their nesting level (r3);

- additionally supports the 68HC16 family from Motorola (r3);

- the  END statement now optionally  accepts an argument as entry

point for the program (r3);

- P2BIN  and P2HEX now allow to move  the contents of a code file

to a different address (r4);

- comments  appended to  a SHARED  instruction are  now copied to

the share file (r4);

- additionally supports the 68HC12 family from Motorola (r4);

- additionally supports the XA family from Philips (r4);

- additionally supports the 68HC08 family from Motorola (r4);

- additionally supports the AVR family from Atmel (r4);

- to  achieve  better  compatibility  to  the AS11 from Motorola,

the  pseudo  instructions  FCB,  FDB,  FCC,  and RMB were added

(r5);

- additionally supports the M16C from Mitsubishi (r5);

- additionally  supports  the  COP8  from  National Semiconductor

(r5);

- additional  instructions IFB and  IFNB for conditional assembly

(r5);

- the  new EXITM instruction now allows  to terminate a macro ex-

pansion (r5);

- additionally supports the MSP430 from Texas Instruments (r5);

- LISTING  now knows the additional  variants NOSKIPPED and PURE-

CODE  to remove  code that  was not  assembled from the listing

(r5);

- additionally supports the 78K0 family from NEC (r5);

- BIGENDIAN is now also available in PowerPC mode (r5);

- additional BINCLUDE instruction to include binary files (r5);

- additional  TOLOWER and LOWSTRING  functions to convert charac-

ters to lower case (r5);

- it  is now possible to store  data in other segments than CODE.

The file format has been extended appropriately (r5);

- the  DS instruction to reserve memory  areas is now also avail-

able in Intel mode (r5);

- the  U command line switch now allows  to switch AS into a case

sensitive  mode  that  differentiates  between  upper and lower

case  in the names of  symbols, user-defined functions, macros,

macro parameters, and sections (r5);

- SFRB  now also  knows the  mapping rules  for bit  addresses in

the  RAM areas; warnings  are generated for  addresses that are

not bit addressable (r5);

- additional  instructions PUSHV  and POPV  to save symbol values

temporarily (r5);

- additional  functions  BITCNT,  FIRSTBIT,  LASTBIT,  and BITPOS

for bit processing (r5);

- the  68360 is now also  known as a member  of the CPU32 proces-

sors (r5);

- additionally supports the ST9 family from SGS-Thomson (r6);

- additionally  supports  the  SC/MP  from National Semiconductor

(r6);

- additionally  supports the  TMS70Cxx family  from Texas Instru-

ments (r6);

- additionally  supports  the  TMS9900  family from Texas Instru-

ments (r6);

- additionally  knows  the  80296's  instruction  set  extensions

(r6);

- the  supported number of Z8 derivatives has been extended (r6);

- additionally knows the 80C504's mask defects (r6);

- additional  register definition file  for Siemens' C50x proces-

sors (r6);

- additionally supports the ST7 family from SGS-Thomson (r6);

- the  Tntel pseudo instructions  for data disposal  are now also

valid for the 65816/MELPS-7700 (r6);

- for  the 65816/MELPS-7700,  the address  length may  now be set

explicitly via prefixes (r6);

- additionally supports the 8X30x family from Signetics (r6);

- from  now on, PADDING is enabled  by default only for the 680x0

family (r7);

- the  new  predefined  symbol  ARCHITECTURE  can  now be used to

query the platform AS was compiled for (r7);

- additional  statements  STRUCT  and  ENDSTRUCT  to  define data

structures (r7);

- hex  and object  files for  the AVR  tools may now be generated

directly (r7);

- MOVEC now also knows the 68040's control registers (r7);

- additional  STRLEN  function  to  calculate  the  length  of  a

string (r7);

- additional  ability  to  define  register symbols (r7 currently

only Atmel AVR);

- additionally knows the 6502's undocumented instructions (r7);

- P2HEX  and P2BIN now optionally can erase the input files auto-

matically (r7);

- P2BIN  can additionally  prepend the  entry address  to the re-

sulting image (r7);

- additionally  supports the  ColdFire family  from Motorola as a

variation of the 680x0 core (r7);

- BYT/FCB,  ADR/FDB, and FCC  now also allow  the repetition fac-

tor known from DC (r7);

- additionally supports Motorola's M*Core (r7);

- the SH7000 part now also knows the SH7700's extensions (r7);

- the  680x0 part now also  knows the 68040's additional instruc-

tions (r7);

- the  56K part now also knows  the instruction set extensions up

to the 56300 (r7).

- the  new CODEPAGE statement now  allows to keep several charac-

ter sets in parallel (r8);

- The argument variations for CHARSET have been extended (r8);

- New string functions SUBSTR and STRSTR (r8);

- additional IRPC statement in the macro processor (r8);

- additional  RADIX statement  to set  the default numbering sys-

tem for integer constants (r8);

- instead of ELSEIF, it is now valid to simply write ELSE (r8);

- == may be used as equality operator instead of = (r8);

- BRANCHEXT  for the Philips  XA now allows  to automatically ex-

tend the reach of short branches (r8);

- debug output is now also possible in NoICE format (r8);

- additionally supports the i960 family from Intel (r8);

- additionally  supports the �PD7720/7725  signal processors from

NEC (r8);

- additionally  supports the  �PD77230 signal  processor from NEC

(r8);

- additionally  supports the SYM53C8xx  SCSI processors from Sym-

bios Logic (r8);

- additionally supports the 4004 from Intel (r8);

- additionally supports the SC14xxx series of National (r8);

- additionally  supports  the  instruction  extensions of the PPC

403GC (r8);

- additional  command line option  cpu to set  the default target

processor (r8);

- key  files now  also may  be referenced  from the  command line

(r8);

- additional  command  line  option  shareout  to  set the output

file for SHARED definitions (r8);

- new  statement WRAPMODE to support AVR processors with a short-

ened program counter (r8);

- additionally  supports the  C20x instruction  subset in the C5x

part (r8);

- hexadecimal  address specifications for the  tools now may also

be made in C notation (r8);

- the  numbering  system  for  integer  results in \{...} expres-

sions is now configurable via OUTRADIX (r8);

- the  register syntax for 4004  register pairs has been correct-

ed (r8);

- additionally supports the F^2MC8L family from Fujitsu (r8);

- P2HEX  now  allows  to  set  the  minimum  address length for S

record addresses (r8);

- additionally supports the ACE family from Fairchild (r8);

-  REG is now also allowed for PowerPCs (r8);

- additional switch in P2HEX to relocate all addresses (r8);

- The  switch x now additionally allows a second level of detail-

ness to print the source line in question (r8).

- version 1.42:

- the default integer syntax for Atmel AVR is now the C Syntax;

- additional  command line  option olist  to set  the list file's

name and location;

- additionally supports the F^2MC16L family from Fujitsu;

- additional instruction PACKING for the AVR family;

- additional implicit macro parameters ALLARGS and ARGCOUNT;

- additional  instruction SHIFT  to process  variable macro argu-

ment lists;

- support for temporary symbols;

- additional  instruction  MAXNEST  to  set  the  maximum nesting

depth of macro expansions;

- additional  command  line  argument  noicemask  to  control the

amount of segments listed in a NoICE debug info file;

- additionally supports the 180x family from Intersil;

- additionally supports the 68HC11K4 address windowing;

- P2HEX  now allows to  vary the address  field length of AVR HEX

files;

- the  new command line option  -gnuerrors allows to output error

messages in a GNU C-style format;

- additionally  supports the TMS320C54x family from Texas Instru-

ments;

- new macro option INTLABEL;

- added  Atmel  MegaAVR  8/16  instructions  and register defini-

tions;

-  ENDIF/ENDCASE  show the line number of the corresponding open-

ing statement in the listing;

- the  8051 part now also supports  the extended address space of

the Dallas DS80C390;

- added nameless temporary smbols;

- additionally supports the undocumented 8085 instructions;

- improved structure handling;

- added EXPRTYPE() function;

- allow line continuation;

- integrated  support  for  KCPSM/PicoBlaze  provided  by Andreas

Wassatsch;

- additionally  supports the  807x family  from National Semicon-

ductor;

- additionally supports the Intel 4040;

- additionally supports the Zilog eZ8;

- additionally supports the 78K2 family from NEC;

- additionally supports the KCPSM3 variant from Xilinx;

- additionally supports the LatticeMico8;

- additionally  supports the  12X instruction  extensions and the

XGATE core of the 68HC12 family;

- additionally supports the Signetics 2650;

- additionally  supports the  COP4 family  from National Semicon-

ductor;

- additionally supports the HCS08 extensions by Freesacle;

- additionally supports the RS08 family by Freescale;

- additionally supports the Intel 8008;

- add another optional syntax for integer constants;

- added function CHARFROMSTR;

- additionally allow Q for octal constants in Intel mode;

- add another variant for temporary symbols;

- the   PowerPC   part   has   been   extended   by   the  MPC821

(contribution by Marcin Cieslak);

- implicit macro parameters are always case-insensitive;

- add REG statement to MSP430;

- additionally supports the XMOS XS1;

- additional  parameters  GLOBALSYMBOLS  and  NOGLOBALSYMBOLS  to

control whether labels in macros are local or not;

- additionally supports the NEC 75xx series;

- additionally supports the TMS1000 controllers from TI;

- additionally supports the 78K2 family from NEC;

- all  newer changes are only documented in the separate changel-

og file.

J. Hints for the AS Source Code

===============================

As  I already mentioned in the  introduction, I release the source code

of  AS on request.  The following shall  give a few  hints to their us-

age.

J.1. Language Preliminaries

---------------------------

In  the beginning, AS  was a program  written in Turbo-Pascal. This was

roughly  at the end  of the eighties,  and there were  a couple of rea-

sons  for this choice: First, I was much  more used to it than to any C

compiler,  and compared to Turbo Pascal's  IDE, all DOS-based C compil-

ers  were just crawling along. In the beginning of 1997 however, it be-

came  clear that  things had  changed: One  factor was that Borland had

decided  to let its confident DOS  developers down (once again, explic-

itly  no 'thank  you', you  boneheads from  Borland!) and replaced ver-

sion  7.0 of  Borland Pascal  with something  called 'Delphi', which is

probably  a wonderful  tool to  develop Windows  programs which consist

of  90% user interface and accidentaly  a little bit of content, howev-

er  completely useless for  command-line driven programs  like AS. Fur-

thermore,  my focus of operating systems  had made a clear move towards

Unix,  and I probably could have  waited arbitrarily long for a Borland

Pascal  for Linux  (to all  those remarking  now that  Borland would be

working  on  something  like  that:  this  is Vapourware, don"t believe

them  anything until you can  go into a shop  and actually buy it!). It

was therefore clear that C was the way to go.

After  this eperience  what results  the usage  of 'island systems' may

have,  I put  a big  emphasize on  portability while doing the transla-

tion  to C; however, since AS for  example deals with binary data in an

exactly  format and  uses operating  systen-specific functions  at some

places  which may need  adaptions when one  compliles AS the first time

for a new platform.

AS  is tailored for a C compiler  that conforms to the ANSI C standard;

C++  is explicitly not required. If you are still using a compiler con-

forming  to the outdated Kernighan&Ritchie standard, you should consid-

er  getting a  newer compiler:  The ANSI  C standard  has been fixed in

1989  and there  should be  an ANSI  C compiler  for every contemporary

platform,  maybe  by  using  the  old  compiler  to build GNU-C. Though

there  are some switches in the source  code to bring it nearer to K&R,

this  is not an officially supported feature which I only use internal-

ly  to support a quite  antique Unix. Everything left  to say about K&R

is located in the file README.KR.

The  inclusion of  some additional  features not  present in the Pascal

version  (e.g. dynamically loadable message files, test suite, automat-

ic  generation of  the documentation  from one  source format) has made

the  source tree substantially more complicated.  I will attempt to un-

wire everything step by step:

J.2. Capsuling System dependencies

----------------------------------

As  I already mentioned, As has  been tailored to provide maximum plat-

form  independence  and  portability  (at  least I believe so...). This

means  packing all  platform dependencies  into as  few files as possi-

ble.  I will describe  these files now,  and this section  is the first

one because it is probably one of the most important:

The  Build of all components of AS  takes place via a central Makefile.

To  make it work,  it has to  be accompanied by  a fitting Makefile.def

that  gives the  platform dependent  settings like  compiler flags. The

subdirectory  Makefile.def-samples contains  a couple  of includes that

work  for widespread platforms  (but which need  not be optimal...). In

case  your platform  is not  among them,  you may  take the  file Make-

file.def.tmpl as a starting point (and send me the result!).

A  further component  to capure  system dependencies  is the  file sys-

defs.h.  Practically all  compilers predefine  a couple of preprocessor

symbols  that describe the target processor and the used operating sys-

tem.  For example, on a  Sun Sparc under Solaris  equipped with the GNU

compiler,  the  symbols  __sparc  and  __SVR4. sysdefs.h exploits these

symbols  to provide a  homogeneous environment for  the remaining, sys-

tem-independent  files. Especially, this covers  integer datatypes of a

specific  length, but it may also include the (re)definition of C func-

tions  which are not  present or non-standard-like  on a specific plat-

form.  It's best to read this files  yourself if you like to know which

things  may  occur...  Generally,  the  #ifdef statement are ordered in

two  levels: First, a specific processor  platform is selected, the the

operating systems are sorted out in such a section.

If  you port AS to  a new platform, you  have to find two symbols typi-

cal  for this  platform and  extend sysdefs.h  accordingly. Once again,

I'm interested in the result...

J.3. System-Independent Files

-----------------------------

...represent  the  largest  part  of  all modules. Describing all func-

tions  in detail  is beyond  the scope  of this  description (those who

want  to know more probably start  studying the sources, my programming

style  isn't that horrible either...),  which is why I  can only give a

short list at this place with all modules their function:

J.3.1. Modules Used by AS

- - - - - - - - - - - - -

as.c

....

This  file is  AS's root:  it contains  the main()  function of AS, the

processing  of all  command line  options, the  overall control  of all

passes and parts of the macro processor.

asmallg.c

.........

This  module processes  all statements  defined for  all processor tar-

gets,  e.g. EQU and ORG. The CPU pseudo-op used to switch among differ-

ent processor targets is also located here.

asmcode.c

.........

This  module contains the  bookkeping needed for  the code output file.

It  exports an interface that allows to  open and close a code file and

offers  functions to write code to (or  take it back from) the file. An

important  job of  this module  is to  buffer the  write process, which

speeds up execution by writing the code in larger blocks.

asmdebug.c

..........

AS  can optionally generate debug information for other tools like sim-

ulators  or  debuggers,  allowing  a  backward  reference to the source

code.  They get collected in this module and can be output after assem-

bly in one of several formats.

asmdef.c

........

This  modules only contains  declarations of constants  used in differ-

ent places and global variables.

asmfnums.c

..........

AS  assigns  internally  assigns  incrementing  numbers  for  each used

source  file. These numbers are  used for quick referencing. Assignment

of  numbers and  the conversion  between names  and numbers takes place

here.

asmif.c

.......

Here  ara  ll  routines  located  controlling conditional assembly. The

most  important exported  variable is  a flag  called IfAsm  which con-

trols whether code generation is currently turned on or off.

asminclist.c

............

This  module holds the  definition of the  list stucture that allows AS

to print the nesting of include files to the assembly list file.

asmitree.c

..........

When  searching for the mnemonic used in  a line of code, a simple lin-

ear  comparison  with  all  available  machine  instructions  (as it is

still  done  in  most  code  generators,  for reasons of simplicity and

laziness)  is not necessary the most  effective method. This module de-

fines  two improved structures (binary tree  and hash table) which pro-

vide  a more  efficient search  and are  destined to replace the simple

linear search on a step-by-step basis...priorities as needed...

asmmac.c

........

Routines  to store  and execute  macro constructs  are located  in this

module. The real macro processor is (as already mentioned) in as.c.

asmpars.c

.........

Here  we really go into the innards:  This module stores the symbol ta-

bles  (global and local) in two binary  trees. Further more, there is a

quite  large procedure  EvalExpression which  analyzes and  evaluates a

(formula)  expression.  The  procedure  returns  the  result  (integer,

floating  point, or string)  in a varaint  record. However, to evaluate

expressions  during code  generation, one  should better  use the func-

tions  EvalIntExpression,  EvalFloatExpression,  and  EvalStringExpres-

sion.  Modifications for tha esake of  adding new target processors are

unnecessary  in  this  modules  and  should  be done with extreme care,

since you are touching something like 'AS's roots'.

asmsub.c

........

This  module collects a couple of  commonly used subroutines which pri-

marily deal with error handling and 'advanced' string processing.

bpemu.c

.......

As  already mentioned  at the  beginning, AS  originally was  a program

written  in Borland Pascal. For some intrinsic functions of the compil-

er,  it was simpler  to emulate those  than to touch  all places in the

source code where they are used. Well...

chunks.c

........

This  module  defines  a  data  type  to  deal  with  a list of address

ranges.  This functionality is needed by  AS for allocation lists; fur-

thermore, P2BIN and P2HEX use such lists to warn about overlaps.

cmdarg.c

........

This  module  implements  the  overall  mechanism of command line argu-

ments.  It needs a specification of  allowed arguments, splits the com-

mand  line and triggers the appropriate callbacks. In detail, the mech-

anism includes the following:

- Processing  of arguments  located in  an environment  variable or a

corresponding file;

- Return  of a set  describing which command  line arguments have not

been processed;

- A  backdoor  for  situations  when  an  overlaying IDE converts the

passed command line completely into upper or lower case.

codepseudo.c

............

You  will find  at this  place pseudo  instructions that  are used by a

subset  of code generators.  On the one  hand, this is  the Intel group

of  DB..DO, and on the other hand  their counterparts for 8/16 bit CPUs

from  Motorola or Rockwell. Someone who wants to extend AS by a proces-

sor  fitting into one of  these groups can get  the biggest part of the

necessary pseudo instructions with one call to this module.

codevars.c

..........

For  reasons of memory efficiency, some  variables commonly used by di-

verse code generators.

endian.c

........

Yet  another bit of machine dependence, however  one you do not have to

spend  attention  on:  This  module  automatically  checks  at  startup

whether  a host  machine is  little or  big endian. Furthermore, checks

are  made if  the type  definitions made  for integer variables in sys-

defs.h really result in the correct lengths.

headids.c

.........

At  this place,  all processor  families supported  by AS are collected

with  their header IDs  (see chapter 5.1)  and the output  format to be

used  by default by  P2HEX. The target  of this table  is to centralize

the  addition of a new processor as  most as possible, i.e. in contrast

to  earlier versions  of AS,  no further  modifications of tool sources

are necessary.

ioerrs.c

........

The  conversion from  error numbers  to clear  text messages is located

here.  I hope I'll never hit a  system that does not define the numbers

as macros, because I would have to rewrite this module completely...

nlmessages.c

............

The  C version  of AS  reads all  messages from  files at runtime after

the  language to be used  is clear. The format  of message files is not

a  simple  one,  but  instead  a  special compact and preindexed format

that  is generated  at runtime  by a  program called 'rescomp' (we will

talk  about it later).  This module is  the counterpart to rescomp that

reads  the  correct  language  part  into  a character field and offers

functions to access the messages.

nls.c

.....

This  module  checks  which  country-dependent  settings (date and time

format,  country code) are  present at runtime.  Unfortunately, this is

a  highly operating system-dependend task, and currently, there are on-

ly  three methods defines:  The MS-DOS method,  the OS/2 method and the

typical  Unix  method  via  locale  functions.  For  all other systems,

there is unfortunately currently only NO_NLS available...

stdhandl.c

..........

On  the one hand, here  is a special open  function located knowing the

special  strings !0...!2 as  file names and  creating duplicates of the

standard  file handles  stdin, stdout,  and stderr.  On the other hand,

investiagations  are done  whether the  standard output  has been redi-

rected  to a device  or a file.  On no-Unix systems, this unfortunately

also incorporates some special operations.

stringlists.c

.............

This  is just a little  'hack' that defines routines  to deal with lin-

ear  lists of strings, which are needed  e.g. in the macro processor of

AS.

strutil.c

.........

Some commonly needed string operations have found their home here.

version.c

.........

The  currently valid  version is  centrally stored  here for AS and all

other tools.

code????.c

..........

These  modules form the main part of  AS: each module contains the code

generator for a specific processor family.

J.3.2. Additional Modules for the Tools

- - - - - - - - - - - - - - - - - - - -

hex.c

.....

A  small  module  to  convert  integer  numbers to hexadecimal strings.

It's  not absolutely  needed in  C any  more (except for the conversion

of  long long  variables, which  unfortunately not  all printf()'s sup-

port), but it somehow survived the porting from Pascal to C.

p2bin.c

.......

The sources of P2BIN.

p2hex.c

.......

The sources of P2HEX.

pbind.c

.......

The sources of BIND.

plist.c

.......

The sources of PLIST.

toolutils.c

...........

All  subroutines needed by  several tools are  collected here, e.g. for

reading of code files.

J.4. Modules Needed During the Build of AS

------------------------------------------

a2k.c

.....

This  is a minimal filter converting  ANSI C source files to Kernighan-

Ritchie  style. To  be exact:  only function  heads are converted, even

this  only when they  are roughly formatted  like my programming style.

Noone should therefore think this were a universal C parser!

addcr.c

.......

A  small  filter  needed  during  installation on DOS- or OS/2-systems.

Since  DOS and OS/2 use a CR/LF for  a newline, inc ontrast to the sin-

gle  LF of  Unix systems,  all assembly  include files provided with AS

are sent through this filter during assembly.

bincmp.c

........

For  DOS and OS/2, this module takes  the task of the cmp command, i.e.

the  binary comparison of  files during the  test run. While this would

principally  be possible  with the  comp command  provided with the OS,

bincmp  does not  have any  nasty interactive  questions (which seem to

be an adventure to get rid of...)

findhyphen.c

............

This  is the submodule  in tex2doc providing  hyphenation of words. The

algorithm used for this is shamelessly stolen from TeX.

grhyph.c

........

The definition of hyphenation rules for the german language.

rescomp.c

.........

This  is AS's 'resource compiler', i.e.  the tool that converts a read-

able file with string resources into a fast, indexed format.

tex2doc.c

.........

A  tool that converts the LaTeX documentation  of AS into an ASCII for-

mat.

tex2html.c

..........

A  tool that converts the LaTeX documentation  of AS into an HTML docu-

ment.

umlaut.c and unumlaut.c

.......................

These  tiny programs convert national  special characters between their

coding  in ISO8859-1 (all  AS files use  this format upon delivery) and

their  system-specific  coding.  Apart  from  a  plain  ASCII7 variant,

there are currently the IBM character sets 437 and 850.

ushyph.c

........

The definition of hyphenation rules for the english language.

J.5. Generation of Message Files

--------------------------------

As  already mentioned,  the C  source tree  of AS  uses a  dynamic load

principle  for all (error) messages. In  contrast to the Pasacl sources

where  all messages were  bundled in an  include file and compiled into

the  programs, this method eliminates the  need to provide AS in multi-

ple  language variants; there is only  one version which checks for the

langugage  to be used  upon runtime and  loads the corresponding compo-

nent  from the message files.  Just to remind: Under  DOS and OS/2, the

COUNTRY  setting is  queried, while  under Unix,  the environment vari-

ables LC_MESSAGES, LC_ALL, and LANG are checked.

J.5.1. Format of the Source Files

- - - - - - - - - - - - - - - - -

A  source file for the message  compiler rescomp usually has the suffix

.res. The message compiler generates one or two files from a source:

- a binary file which is read at runtime by AS resp. its tools

- optionally  one further C header file  assigning an index number to

all  messages. These index numbers in combination with an index ta-

ble  in the binary file  allow a fast access  to to individual mes-

sages at runtime.

The  source file for the message compiler  is a pure ASCII file and can

therefore  be modified  with any  editor. It  consists of a sequence of

control  commands  with  parameters.  Empty  lines  and lines beginning

with  a semicolon  are ignored.  Inclusion of  other files  is possible

via the Include statement:

Include <Datei>

The  first two statements  in every source  file must be two statements

describing  the languages defined in  the following. The more important

one is Langs, e.g.:

Langs DE(049) EN(001,061)

describes  that two languages will be defined  in the rest of the file.

The  first one shall be used under  Unix when the language has been set

to  DE via environment variable. Similarly,  It shall be used under DOS

and  OS/2 when the country  code was set to  049. Similarly, the second

set  shall be used for  the settings DE resp.  061 or 001. While multi-

ple  'telephone numbers'  may point  to a  single language, the assign-

ment  to a Unix  language code is  a one-to-one correspondence. This is

no  problem in  practice since  the LANG  variables Unix  uses describe

subversions via appendices, e.g.:

de.de

de.ch

en.us

AS  only  compares  the  beginning  of  the strings and therefore still

comes  to the  right decision.  The Default  statement defines the lan-

guage  that shall be used if either no  language has been set at all or

a  language is  used that  is not  mentioned in  the asrgument  list of

Langs. This is typically the english language:

Default EN

These  definitions  are  followed  by  an  arbitrary  number of Message

statements, i.e. definitions of messages:

Message ErrName

": Fehler "

": error "

In  case n languages  were announced via  the Langs statement, the mes-

sage  compiler  takes  exactly  the  following  n  as the strings to be

stored.  It is therefore impossible to  leave out certain languages for

individual  messages, and  an empty  line following  the strings should

in  no way  be misunderstood  as an  end marker  for the list; inserted

lines  between statements  only serve  purposes of  better readability.

It  is  however  allowed  to  split individual messages across multiple

lines  in the source  file; all lines  except for the  last one have to

be ended with a backslash as continuation character:

Message TestMessage2

"Dies ist eine" \

"zweizeilige Nachricht"

"This is a" \

"two-line message"

As  already mentioned, source files are pure ASCII files; national spe-

cial  characters may be placed in  message texts (and the compiler will

correctly  pass them to the resulting  file), a big disadvantage howev-

er  is that such a file  is not fully portable any  more: in case it is

ported  to another  system using  a different  coding for national spe-

cial  characters, the user will probably be confronted with funny char-

acters  at  runtime...special  characters  should  therefore  always be

written  via special sequences  borrowed from HTML  resp. SGML (see ta-

ble J.1). Linefeeds can be inserted into a line via \n, similar to C.

+--------------------------------------+-----------------------+

| Sequence...                          | results in...         |

+--------------------------------------+-----------------------+

+--------------------------------------+-----------------------+

|  &auml; &ouml; &uuml;                | "a "o "u (Umlauts)    |

|  &Auml; &Ouml; &Uuml;                | "A "O "U              |

|  &szlig;                             | "s (sharp s)          |

|  &agrave; &egrave; &igrave; &ograve; | � � � �               |

|  &ugrave;                            | �                     |

|  &Agrave; &Egrave; &Igrave; &Ograve; | � � � �               |

|  &Ugrave;                            | � (Accent grave)      |

|  &aacute; &eacute; &iacute; &oacute; | � � � �               |

|  &uacute;                            | �                     |

|  &Aacute; &Eacute; &Iacute; &Oacute; | � � � �               |

|  &Uacute;                            | � (Accent agiu)       |

|  &acirc; &ecirc; &icirc; &ocirc;     | � � � �               |

|  &ucirc;                             | �                     |

|  &Acirc; &Ecirc; &Icirc; &Ocirc;     | � � � �               |

|  &Ucirc;                             | � (Accent circonflex) |

|  &ccedil; &Ccedil;                   | � �(Cedilla)          |

|  &ntilde; &Ntilde;                   | � �                   |

|  &aring; &Aring;                     | � �                   |

|  &aelig; &Aelig;                     | � �                   |

|  &iquest; &iexcl;                    | inverted ! or ?       |

+--------------------------------------+-----------------------+

Table J.1: Syntax for special character in rescomp

J.6. Creation of Documentation

------------------------------

A  source distribution of AS contains  this documentation in LaTeX for-

mat  only. Other  formats are  created from  this one automatically via

tools  provided  with  AS.  One  reason  is  to  reduce the size of the

source  distribution, another reason is  that changes in the documenta-

tion only have to be made once, avoiding inconsistencies.

LaTex  was  chosen  as  the  master  format because...because...because

it's  been there all the time before. Additionally, TeX is almost arbi-

trarily  portable and fits quite well to  the demands of AS. A standard

distribution  therefore allows  a nice  printout on  about any printer;

for  a conversion to an ASCII file that used to be part of earlier dis-

tributions,  the converter  tex2doc is  included, additionally the con-

verter tex2html allowing to put the manual into the Web.

Generation of the documentation is started via a simple

make docs

The  two converters mentioned  are be built  first, then applied to the

TeX  documentation and  finally, LaTeX  itself is  called. All  this of

course for all languages...

J.7. Test Suite

---------------

Since  AS deals with  binary data of  a precisely defined structure, it

is  naturally sensitive for system  and compiler dependencies. To reach

at  least a  minimum amount  of secureness  that everything  went right

during  compilation, a set of test sources is provided in the subdirec-

tory  tests that allows to test the freshly built assembler. These pro-

grams  are primarily trimmed  to find faults  in the translation of the

machine  instruction set,  which are  commonplace when  integer lenghts

vary.  Target-independent features like the  macro processors or condi-

tional  assembly are  only casually  tested, since  I assume  that they

work everywhere when they work for me...

The  test run is started  via a simple make  test. Each test program is

assembled,  converted to a binary file, and compared to a reference im-

age.  A test is  considered to be  passed if and  only if the reference

image  and the  newly generated  one are  identical on a bit-by-bit ba-

sis.  At  the  end  of  the  test,  the assembly time for every test is

printed  (those who  want may  extend the  file BENCHES  with these re-

sults),  accompanied  with  a  success  or  failure message. Track down

every  error that occurs, even  if it occurs in  a processor target you

are  never going to use!  It is always possible  that this points to an

error  that may also come up for  other targets, but by coincidence not

in the test cases.

J.8. Adding a New Target Processor

----------------------------------

The  probably most common reason to modify  the source code of AS is to

add  a new target  processor. Apart from  adding the new  module to the

Makefile,  there are few places in  other modules that need a modifica-

tion.  The new  module will  do the  rest by  registering itself in the

list  of code generators. I  will describe the needed  steps in a cook-

book style in the following sections:

Choosing the Processor's Name

.............................

The name chosen for the new processor has to fulfill two criterias:

1 The  name must not be  already in use by  another processor. If one

starts  AS without any  parameters, a list  of the names already in

use will be printed.

2 If  the name shall  appear completely in  the symbol MOMCPU, it may

not  contain other  letters than  A..F (except  right at the begin-

ning).  The  variable  MOMCPUNAME  however  will  always report the

full  name during assembly. Special characters are generally disal-

lowed,  lowercase letters will  be converted by  the CPU command to

uppercase  letters  and  are  therefore  senseless in the processor

name.

The  first step for registration is making an entry for the new proces-

sor  (family) in  the file  headids.c. As  already mentioned, this file

is  also used  by the  tools and  specifies the  code ID  assigned to a

processor  family, along with  the default hex  file format to be used.

I would like to have some coordination before choosing the ID...

Definition of the Code Generator Module

.......................................

The  unit's  name  that  shall  be  responsible  for  the new processor

should  bear at  least some  similarity to  the processor's  name (just

for  the  sake  of  uniformity)  and  should  be  named in the style of

code.....  The head with include statements  is best taken from another

existing code generator.

Except  for an  initialization function  that has  to be  called at the

begginning  of the main() function in  module as.c, the new module nei-

ther  has to export variables nor  functions as the complete communica-

tion  is done at runtime via indirect  calls. They are simply done by a

call  to  the  function  AddCPU  for  each processor type that shall be

treated by this unit:

CPUxxxx:=AddCPU('XXXX',SwitchTo_xxxx);

'XXXX'  is the name chosen  for the processor which  later must be used

in  assembler programs to  switch AS to  this target processor. Switch-

To_xxxx  (abbreviated as the ''switcher'' in the following) is a proce-

dure  without parameters that  is called by  AS when the  switch to the

new  processor actually takes  place. AddCPU delivers  an integer value

as  result that  serves as  an internal  ''handle'' for the new proces-

sor.  The global variable MomCPU always contains the handle of the tar-

get  processor  that  is  currently  set.  The value returned by AddCPU

should  be stored in a private  variable of type CPUVar (called CPUxxxx

in  the  example  above).  In  case  a code generator module implements

more  than one  processor (e.g.  several processors  of a  family), the

module  can find out  which instruction subset  is currently allowed by

comparing MomCPU against the stored handles.

The  switcher's task is to ''reorganize'' AS for the new target proces-

sor. This is done by changing the values of several global variables:

- ValidSegs:  Not all processors  have all address  spaces defined by

AS.  This set defines which subset the SEGMENT instruction will en-

able  for the currently active target  processor. At least the CODE

segment  has to  be enabled.  The complete  set of allowed segments

can be looked up the file fileformat.h (Seg.... constants).

- SegInits:  This  array  stores  the  initial program counter values

for  the individual segments (i.e.  the values the program counters

will  initially take when there is no ORG statement). There are on-

ly  a few exceptions (like  logically separated address spaces that

physically overlap) which justify other initial values than 0.

- Grans:  This array specifies  the size of  the smallest addressable

element  in bytes  for each  segment, i.e.  the size  of an element

that  increases an  address by  1. Most  processors need a value of

1,  even if  they are  16- or  32-bit processors,  but the PICs and

signal processors are cases where higher values are required.

- ListGrans:  This  array  specifies  the  size  of  byte groups that

shall  be shown in  the assembly listing.  For example, instruction

words  of the 68000  are always 2  bytes long though  the code seg-

ment's  granularity is  1. The  ListGran entry  therefore has to be

set to 2.

- SegLimits:  This  array  stores  the  highest  possible address for

each  segment, e.g.  65535 for  a 16-bit  address space. This array

need  not  be  filled  in  case  the  code generator takes over the

ChkPC method.

- ConstMode:  This variable may take  the values ConstModeIntel, Con-

stModeMoto,  or ConstModeC  and rules  which syntax  has to be used

to specify the base of integer constants.

- PCSymbol:  This variable  contains the  string an assembler program

may  use to to get the current  value of the program counter. Intel

processors for example usually use a dollar sign.

- TurnWords:  If the target processor  uses big-endian addressing and

one  of the fields  in ListGran is  larger than one,  set this flag

to true to get the correct byte order in the code output file.

- SetIsOccupied:  Some processors have a  SET machine instruction. If

this  callback is set  to a non-NULL  value, the code generator may

report  back whether  SET shall  not be  interpreted as  pseudo in-

struction.  The return  value may  be constant  True or or e.g. de-

pend on the number of argument if a differentiation is possible.

- HeaderID:  This variable contains  the ID that  is used to mark the

current  processor family in the the  code output file (see the de-

scription  of the code  format described by  AS). I urge to contact

me  before selecting  the value  to avoid  ambiguities. Values out-

side  the range of $01..$7f should  be avoided as they are reserved

for  special purposes  (like a  future extension  to allow linkable

code).  Even though  this value  is still  hard-coded in  most code

generators,  the preferred method  is now to  fetch this value from

headids.h via FindFamilyByName.

- NOPCode:  There are  some situations  where AS  has to  fill unused

code  areas  with  NOP  statements.  This variable contains the ma-

chine code of the NOP statement.

- DivideChars:  This string  contains the  characters that  are valid

separation  characters for instruction parameters. Only extreme ex-

otics  like the  DSP56 require  something else  than a single comma

in this string.

- HasAttrs:  Some processors  like the  68k series additionally split

an  instruction into mnemonic  and attribute. If  the new processor

also  does something like that,  set this flag to  true and AS will

deliver  the instructions'  components readily  split in the string

variables  OpPart  and  AttrPart.  If  this  flag is however set to

false,  no splitting  will take  place and  the instruction will be

delivered  as a  single piece  in OpPart.  AttrPart will stay empty

in  this case. One really should set this flag to false if the tar-

get  processor  does  not  have  attributes as one otherwise looses

the  opportunity to  use macros  with a  name containing dots (e.g.

to emulate other assemblers).

- AttrChars:  In case  HasAttrs is  true, this  string has to contain

all  characters that can  separate mnemonic and  attribute. In most

cases, this string only contains a single dot.

Do  not assume that any of these  variables has a predefined value; set

them all!!

Apart  from  these  variables,  some  function  pointers have to be set

that  form the link form AS to the ''active'' parts of the code genera-

tor:

- MakeCode:  This  routine  is  called  after  a source line has been

split  into mnemonic  and parameters.  The mnemonic  is stored into

the  variable OpPart,  and the  parameters can  be looked up in the

array  ArgStr. The  number of  arguments may  be read  from ArgCnt.

The  binary  code  has  to  be  stored into the array BAsmCode, its

length  into CodeLen. In  case the processor  is word oriented like

the  68000 (i.e. the ListGran element corresponding to the current-

ly  active segment is  2), the field  may be addressed wordwise via

WAsmCode.  There  is  also  DAsmCode  for extreme cases... The code

length  has to be given in  units corresponding to the current seg-

ment's granularity.

- SwitchFrom:  This parameter-less procedure enables the code genera-

tor  module to do  ''cleanups'' when AS  switches to another target

processor.  This hook allows e.g. to  free memory that has been al-

located  in the  generator and  that is  not needed  as long as the

generator  is not  active. It  may point  to an  empty procedure in

the  simplest case. One example  for the usage of  this hook is the

module  CODE370 that builds its  instruction tables dynamically and

frees them again after usage.

- IsDef:  Some processors know additional  instructions that impose a

special  meaning on a label in the first row like EQU does. One ex-

ample  is the  BIT instruction  found in  an 8051 environment. This

function  has  to  return  TRUE  if  such  a special instruction is

present.  In the simplest case  (no such instructions), the routine

may return a constant FALSE.

Optionally,  the  code  generator  may  additionally  set the following

function pointers:

- ChkPC  : Though  AS internally  treats all  program counters as ei-

ther  32 or 64 bits,  most processors use an  address space that is

much  smaller. This  function informs  AS whether  the current pro-

gram  counter has exceeded  its allowed range.  This routine may of

course  be much more  complicated in case  the target processor has

more   than   one   address   space.   One  example  is  in  module

code16c8x.c.  In case everything  is fine, the  function has to re-

turn  TRUE, otherwise FALSE. The code  generator only has to imple-

ment  this function if it did not  set up the array SegLimits. This

may  e.g. become necessary  when the allowed  range of addresses in

a segment is non-continuous.

- InternSymbol  : Some  processorcs, e.g.  such with  a register bank

in  their internal RAM, predefine  such 'registers' as symbols, and

it  wouldn't make much  sense to define  them in a separate include

file  with 256 or maybe  more EQUs. This hook  allows access to the

code  generator of AS: It obtains  an expression as an ASCII string

and  sets up  the passed  structure of  type TempResult accordingly

when  one of these 'built-in' symbols  is detected. The element Typ

has  to be set  to TempNone in  case the check  failed. Errors mes-

sages  from this  routine should  be avoided  as unidentified names

could  signify ordinary symbols (the  parser will check this after-

wards).  Be extreme careful  with this routine  as it allows you to

intervene into the parser's heart!

- DissectBit  :  In  case  the  target platform supports bit objects,

i.e.  objects that  pack both  a register  or memory  address and a

bit  position into one integer number, this is the callback to dis-

sect  such a  packed representation  and transform  it back  into a

source-code  like, human-readable form.  This provides better read-

ability of the listing.

- DissectReg  : In  case the  target platform  supports register sym-

bols,  this  is  the  callback  that translates register number and

size  back to a source-code  like, human-readable form. Again, this

function is used for the listing.

- QualifyQuote  : This optional  callback allows to  define on a per-

platform  base situations  when a  single quotation  character does

not  lead in a character  string. An example for  this is the Z80's

alternate  register bank, which is written as AF', or the hexadeci-

mal constant syntax H'... used on some Hitachi processors.

By  the way:  People who  want to  become immortal  may add a copyright

string.  This is  done by  adding a  call to the procedure AddCopyright

in the module's initialization part (right next to the AddCPU calls):

AddCopyright(

"Intel 80986 code generator (C) 2010 Jim Bonehead");

The  string passed to  AddCopyright will be  printed upon program start

in addition to the standard message.

If  needed, the unit may also use  its initialization part to hook into

a  list of procedures that  are called prior to  each pass of assembly.

Such  a need for  example arises when  the module's code generation de-

pends  on certain flags  that can be  modified via pseudo instructions.

An  example is a processor that can  operate in either user or supervi-

sor  mode. In user mode, some  instructions are disabled. The flag that

tells  AS whether  the following  code executes  in user  or supervisor

mode  might be set via a special pseudo instruction. But there must al-

so  be an  initialization that  assures that  all passes start with the

same  state. The  hook offered  via AddInitPassProc  offers a chance to

do  such initializations. The callback function  passed to it is called

before a new pass is started.

The  function chain built up via calls to AddCleanUpProc operates simi-

lar  to AddInitPassProc: It enables code generators to do clean-ups af-

ter  assembly (e.g. freeing  of literal tables).  This makes sense when

multiple  files are assembled with a  single call of AS. Otherwise, one

would  risk to have 'junk'  in tables from the  previous run. No module

currently uses this feature.

Writing the Code Generator itself

.................................

Now  we finally reached the point  where your creativity is challenged:

It  is up to  you how you  manage to translate  mnemonic and parameters

into  a sequence of machine  code. The symbol tables  are of course ac-

cessible  (via the formula  parser) just like  everything exported from

ASMSUB. Some general rules (take them as advises and not as laws...):

- Try  to split the instruction set  into groups of instructions that

have  the same operand  syntax and that  differ only in  a few bits

of  their machine  code. For  example, one  can do all instructions

without parameters in a single table this way.

- Most  processors have a  fixed spectrum of  addressing modes. Place

the  parsing of an address expression  in a separate routine so you

an reuse the code.

- The  subroutine WrError defines  a lot of  possible error codes and

can  be easily extended. Use this! It  is no good to simply issue a

''syntax error'' on all error conditions!

Studying  other existing code generators should  also prove to be help-

ful.

Modifications of Tools

......................

A  microscopic change to the tolls'  sources is still necessary, namely

to  the  routine  Granularity()  in  toolutils.c:  in  case  one of the

processor's  address  spaces  has  a  granularity  different  to 1, the

swich  statement in  this place  has to  be adapted accordingly, other-

wise PLIST, P2BIN, and P2HEX start counting wrong...

J.9. Localization to a New Language

-----------------------------------

You  are interested in this topic? Wonderful!  This is an issue that is

often  neglected by other  programmers, especially when  they come from

the country on the other side of the big lake...

The  localization to a  new language can  be split into  two parts: the

adaption  of program  messages and  the translation  of the manual. The

latter  one is definitely  a work of  gigantic size, however, the adap-

tion  of program messages should be a work doable on two or three week-

ends,  given  that  one  knows  both  the  new  and  one of the already

present  messages. Unfortunately, this translation  cannot be done on a

step-by-step  basis  because  the  resource  compiler  currently cannot

deal  with a  variable amount  of languages  for different messages, so

the slogan is 'all or nothing'.

The  first oeration is to add the  new language to header.res. The two-

letter-abbreviation  used for  this language  is best  fetched from the

nearest  Unix system (in case you don't work on one anyway...), the in-

ternational telephone prefix from a DOS manual.

When  this is complete, one can rebuild all necessary parts with a sim-

ple  make and obtains an assembler  that supports one more language. Do

not  forget to forward the results to me. This way, all users will ben-

efit from this with the next release :-)

Bibliography

============

[ 1] Steve Williams:

68030 Assembly Language Reference.

Addison-Wesley, Reading, Massachusetts, 1989

[ 2] Advanced Micro Devices:

AM29240, AM29245, and AM29243 RISC Microcontrollers.

1993

[ 3] Apple Corporation::

6502 Floating Point Routines.

Apple II Reference Manual (Red Book), January 1978, pages 94-95

http://www.6502.org/source/floats/wozfp3.txt

[ 4] Atmel Corp.:

AVR Enhanced RISC Microcontroller Data Book.

May 1996

[ 5] Atmel Corp.:

8-Bit   AVR   Assembler   and   Simulator   Object   File  Formats

(Preliminary).

(part of the AVR tools documentation)

[ 6] Commodore Semiconductor Group:

65CE02 Microprocessor Preliminary Data Sheet.

[ 7] CMD Microcircuits:

G65SC802/G65SC816 CMOS 8/16-Bit Microprocessor.

Family Data Sheet.

[ 8] Freescale Semiconductor:

Digital  Signal Processing  Libraries Using  the ColdFire eMAC and

MAC User's Manual. DSPLIBUM, Rev. 1.2, 03/2006

[ 9] National Semiconductor:

COP410L/COP411L/COP310L/COP311L  Single-Chip  N-Channel  Microcon-

trollers. RRD-B30M105, March 1992

[10] National Semiconductor:

COPS Family User's Guide.

[11] General Instrument Microelectronics:

Series 1600 Microprocessor System Documentation..

S16DOC-CP-1600-04, May 1975

http://www.bitsavers.org/components/gi/CP1600/

CP-1600_Microprocessor_Users_Manual_May75.pdf

[12] Digital Research:

CP/M 68K Operating System User's Guide.

1983

[13] Cyrix Corp.:

FasMath 83D87 User's Manual.

1990

[14] Dallas Semiconductor:

DS80C320 High-Speed Micro User's Guide.

Version 1.30, 1/94

[15] Fairchild Semiconductor:

ACE1101 Data Sheet.

Preliminary, May 1999

[16] Fairchild Semiconductor:

ACE1202 Data Sheet.

Preliminary, May 1999

[17] Fairchild Semiconductor:

ACEx  Guide  to  Developer  Tools.  AN-8004, Version 1.3 September

1998

[18] Fairchild Micro Systems:

F8 User's Guide. 67095665, 02-13-1976

[19] Fairchild Micro Systems:

F8 Guide to Programming 67095664, 1976

[20] Freescale Semiconductor:

S12XCPUV1 Reference Manual. S12XCPUV1, v01.01, 03/2005

[21] Freescale Semiconductor:

RS08 Core Reference Manual. RS08RM, Rev. 1.0, 04/2006

[22] Freescale Semiconductor:

MC9S12XDP512 Data Sheet. MC9S12XDP512, Rev. 2.11, 5/2005

[23] Fujitsu Limited:

June 1998 Semiconductor Data Book.

CD00-00981-1E

[24] Fujitsu Semiconductor:

F�MC16LX  16-Bit Microcontroller MB90500  Series Programming Manu-

al.

CM44-00201-1E, 1998

[25]  CPU Comparison with Z80.

https://gbdev.io/pandocs/CPU_Comparison_with_Z80.html

[26] Hitachi Ltd.:

8-/16-Bit Microprocessor Data Book.

1986

[27] Trevor J.Terrel & Robert J. Simpson:

Understanding HD6301X/03X CMOS Microprocessor Systems.

published by Hitachi

[28] Hitachi Microcomputer:

H8/300H Series Programming Manual.

(21-032, no year of release given)

[29] Hitachi America, Ltd.:

HD641016 User's Manual.

ADE-602-003A, September 1989

[30]  HuC6280 CMOS 8-bit Microprocessor Software Manual.

[31] Rockwell:

R65C19 Microcomputer Data Sheet.

Document Number 29400N10, January 1992

[32] Hitachi Semiconductor Design & Development Center:

SH Microcomputer Hardware Manual (Preliminary).

[33] Hitachi Semiconductor and IC Div.:

SH7700 Series Programming Manual.

1st Edition, September 1995

[34] Hitachi  America Ltd.:  HMCS400 Series  Handbook: Users Manual AD-

E00078, March 1988

[35] Hitachi Semiconductor and IC Div.:

H8/500 Series Programming Manual.

(21-20, 1st Edition Feb. 1989)

[36] Hitachi Ltd.:

H8/532 Hardware Manual.

(21-30, no year of release given)

[37] Hitachi Ltd.:

H8/534,H8/536 Hardware Manual.

(21-19A, no year of release given)

[38] Hewlett Packard:

Nano Processor's Users Guide.

Drawing Number A-5955-0331-1

https://www.hp9845.net/9845/downloads/manuals/Nanoprocessor.pdf

[39] IBM Corp.:

PPC403GA Embedded Controller User's Manual.

First Edition, September 1994

[40] IBM Corp.:

IBM5100 ISA Opcodes Micro Instruction Set.

https://github.com/voidstar78/IBM_5100_DOCS/blob/main/

PDFs_5100/IBM5100_ISA_opcodes_micro-instruction_set.pdf

[41] IBM Corp.:

IBM5110 BASIC Reference Manual.

SA21-9306-0, First Edition, January 1978

http://www.bitsavers.org/pdf/ibm/5110/

SA21-9308-0_IBM_5110_BASIC_Reference_Manual_Jan1978.pdf

[42] IBM Corp.:

IBM5100 Maintenance Information Manual.

SY31-0405-3, October 1979

[43] Dokumentation zum IBM5110:

http://computermuseum.informatik.uni-stuttgart.de/dev/

ibm_5110/technik/5110.html

[44] Intel Corp.:

Embedded Controller Handbook.

1987

[45] Intel Corp.:

Microprocessor and Peripheral Handbook, Volume I Microprocessor.

1988

[46] Intel Corp. :

MCS-48 Family of Single Chip Microcomputers User's Manual.

September 1980

[47] Intel Corp. :

80960SA/SB Reference Manual.

1991

[48] Intel Corp.:

8XC196NT Microcontroller User's Manual.

June 1995

[49] Intel Corp.:

8XC251SB High Performance CHMOS Single-Chip Microcontroller.

Sept. 1995, Order Number 272616-003

[50] Intel Corp.:

80296SA Microcontroller User's Manual.

Sept. 1996

[51] Intel Corp.:

4040: Single-Chip 4-Bit P-Channel Microprocessor.

(no year of release given)

[52] Intersil:

CDP1802A, CDP1802AC, CDP1802BC CMOS 8-Bit Microprocessors.

March 1997

[53] : RCA Inc.: CDP1804, CDP1804C Types Objective Data.

(no year of release given)

[54] Intersil:

CDP1805AC,  CDP1806AC CMOS  8-Bit Microprocessor  with On-Chip RAM

and Counter/Timer.

March 1997

[55] Intersil:

IM6100 CMOS 12 Bit Microprocessor.

(no year of release given)

[56] Hirotsugu Kakugawa:

A memo on the secret features of 6309.

(available via World Wide Web:

http://www.cs.umd.edu/users/fms/comp/CPUs/6309.txt)

[57] KENBAK:

Programming Reference Manual KENBAK-1 Computer.

4/1/1971

[58] Lattice Semiconductor Corporation:

LatticeMico8 Microcontroller Users Guide.

Reference Design RD1026, February 2008

[59] Microchip Technology Inc.:

Microchip Data Book.

1993 Edition

[60] US Department Of Defense:

Military  Standard Sixteen-Bit Computer  Instruction Set Architec-

ture.

MIL-STD-1750A (USAF), 2 July 1980

[61] Mitsubishi Electric:

Single-Chip 8-Bit Microcomputers.

Vol.2, 1987

[62] Mitsubishi Electric:

Single-Chip 16-Bit Microcomputers.

Enlarged edition, 1991

[63] Mitsubishi Electric:

Single-Chip 8 Bit Microcomputers.

Vol.2, 1992

[64] Mitsubishi Electric:

M34550Mx-XXXFP Users's Manual.

Jan. 1994

[65] Mitsubishi Electric:

7751 Series Software Manual.

Rev. 1.01, 980731

[66] Mitsubishi Electric:

M16 Family Software Manual.

First Edition, Sept. 1994

[67] Mitsubishi Electric:

M16C Software Manual.

First Edition, Rev. C, 1996

[68] Mitsubishi Electric:

M30600-XXXFP Data Sheet.

First Edition, April 1996

[69] documentation  about  the  M16/M32-developer's  package from Green

Hills Software

[70] Mostek Corporation:

Circuits and Systems Product Guide.

1980, STD No 01009

[71] Mostek Corporation:

3870/F8 Microcomputer Data Book.

1981, Publication Number MK79602

[72] Motorola Inc.:

Microprocessor, Microcontroller and Peripheral Data.

Vol. I+II, 1988

[73] Motorola Inc.:

MC68881/882 Floating Point Coprocessor User's Manual.

Second Edition, Prentice-Hall, Englewood Cliffs 1989

[74] Motorola Inc.:

MC68851 Paged Memory Management Unit User's Manual.

Second Edition, Prentice-Hall, Englewood Cliffs 1989,1988

[75] Motorola Inc.:

CPU32 Reference Manual.

Rev. 1, 1990

[76] Motorola Inc.:

DSP56000/DSP56001 Digital Signal Processor User's Manual.

Rev. 2, 1990

[77] Motorola Inc.:

MC68340 Technical Summary.

Rev. 2, 1991

[78] Motorola Inc.:

CPU16 Reference Manual.

Rev. 1, 1991

[79] Motorola Inc.:

Motorola M68000 Family Programmer's Reference Manual.

1992

[80] Motorola Inc.:

MC68332 Technical Summary.

Rev. 2, 1993

[81] Motorola Inc.:

PowerPC 601 RISC Microprocessor User's Manual.

1993

[82] Motorola Inc.:

PowerPC(tm) MPC505 RISC Microcontroller Technical Summary.

1994

[83] Motorola Inc.:

PowerPC(tm) MPC821 Portable Microprocessor User's Manual.

1996

[84] Motorola Inc.:

CPU12 Reference Manual.

1st edition, 1996

[85] Motorola Inc.:

CPU08 Reference Manual.

Rev. 1 (no year of release given im PDF-File)

[86] Motorola Inc.:

MC68360 User's Manual.

[87] Motorola Inc.:

MCF 5200 ColdFire Family Programmer's Reference Manual.

1995

[88] Motorola Inc.:

M*Core Programmer's Reference Manual.

1997

[89] Motorola Inc.:

DSP56300 24-Bit Digital Signal Processor Family Manual.

Rev. 0 (no year of release given im PDF-File)

[90] Motorola Inc.:

MC68HC11K4 Technical Data. 1992

[91] National Semiconductor:

SC/MP Programmier- und Assembler-Handbuch.

Publication Number 4200094A, Aug. 1976

[92] National Semiconductor:

COP800 Assembler/Linker/Librarian User's Manual.

Customer Order Number COP8-ASMLNK-MAN

NSC Publication Number 424421632-001B

August 1993

[93] National Semiconductor:

COP87L84BC   microCMOS   One-Time-Programmable   (OTP)   Microcon-

troller.

Preliminary, March 1996

[94] National Semiconductor:

IMP-16 Programming and Assembler Manual.

Order No. IMP-16S/102YB, Pub. No 4200002B, November 1973

[95] National Semiconductor:

IMP-16L Users Manual.

Order No. IMP-16L/928X, Pub. No 4200028X, April 1974

[96] National Semiconductor:

IPC-16A/520D MOS/LSI Single Chip Microprocessor (PACE).

[97] National Semiconductor:

PACE Microprocessor Assembly Language Programming Manual.

Order No. IPC-16S/969Y, Pub. No 4200130A, January 1977

[98] National Semiconductor:

INS8900 Single-Chip 16-Bit N-Channel Microprocessor.

1977

[99] National Semiconductor:

SC14xxx DIP commands Reference guide.

Application Note AN-D-031, Version 0.4, 12-28-1998

[100] National Semiconductor:

INS8070-Series Microprocessor Family. October 1980

[101] National Semiconductor:

Series 32000 Databook.

1986

[102] National Semiconductor:

NS32CG16-10/NS32CG16-15  High Performance  Printer/Display Proces-

sor.

Preliminary, November 1995

[103] National Semiconductor and Udo M�ller:

Series 32000 Instruction Set Manual.

6. September 2017, available via www.cpu.ns32k.net

[104] NEC Corp.:

�pD70108/�pD70116/�pD70208/�pD70216/�pD72091 Data Book.

(no year of release given)

[105] NEC Electronics Europe GmbH:

User's Manual �COM-87 AD Family.

(no year of release given)

[106] NEC Corp.:

�COM-75x Family 4-bit CMOS Microcomputer User's Manual.

Vol. I+II (no year of release given)

[107] NEC Corp.:

78K/II  Series 8-Bit  Single-Chip Microcontroller  User's Manual -

Instructions.

Document No. U10228EJ6V0UM00 (6th edition), December 1995

[108] NEC Corp.:

uPD78310/312CW/G 8 Bit CMOS Microcomputer Product Description.

[109] NEC Corp.:

Digital Signal Processor Product Description.

PDDSP.....067V20 (no year of release given)

[110] NEC Corp.:

�PD78070A,  78070AY 8-Bit Single-Chip Microcontroller User's Manu-

al.

Document No. U10200EJ1V0UM00 (1st edition), August 1995

[111] NEC Corp.:

Data Sheet �PD78014.

[112] NEC Corp.:

�PD70616 Programmer's Reference Manual.

Preliminary Information, November 1986

[113] NEC Corp.:

�COM-43 Single Chip Microcomputer Users' Manual.

January 1978

[114] NXP/Freescale:

CPU S12Z Reference Manual.

CPUS12ZRM, Rev. 1.01, 01/2013

[115] NXP:

MC9S12ZVC-Family Reference Manual and Datasheet.

MC9S12ZVCRMV1, Rev. 1.9, 29-January-2018

[116] Parallax Inc.

SX20AC/SX28AC Data Sheet. Revision 1.7, 4/23/2008

[117] Digital Equipment Corporation:

PDP-11 Architecture Handbook. 1983 Edition, EB-23657-18

[118] Digital Equipment Corporation:

PDP-11  MACRO-11 Language Reference  Manual. March 1983, AA-V027A-

TC

[119] OKI Semiconductor:

Microcontroller Data Book. Second Edition, December 1986

[120] Philips Semiconductor:

MAB84X1, MAF84X1, MAF84AX1 Family Datasheet.

August 1990

[121] Philips Semiconductor:

16-bit 80C51XA Microcontrollers (eXtended Architecture).

Data Handbook IC25, 1996

[122] Rockwell International Corporation:

Parallel  Processing System (PPS-4) Microcomputer Data Sheet. Doc-

ument No. 29003 N11, 1973

[123] RX Family Product Page,

https://www.renesas.com/us/en/products/

microcontrollers-microprocessors/

rx-32-bit-performance-efficiency-mcus

[124] RX Family Compiler and Assembler Manual:

https://www.renesas.com/us/en/document/mat/

high-performance-embedded-workshop-rx-family-

cc-compiler-package-v101-users-manual-includes-v102

[125] Sharp:

Pocket Computer PC-1350 Machine Language Reference Manual.

https://pockemul.com/wp-content/uploads/2022/12/

PC1350_ML_EN.pdf

[126] Sharp Corporation:

ESR-L Instruction Manual.

https://web.archive.org/web/20060521205349if_/http://

www.sarnau.info:80/files/ESR-LInstructionManual.pdf

[127] Signetics Corporation:

Signetics 2650 Microprocessor.

1975

[128] Signetics Corporation:

2650 User's Manual.

[129] SGS-Ates:

M3870 Microcomputer Family Databook.

1st edition, issued January 1983

[130] SGS-Thomson Microelectronics:

8 Bit MCU Families EF6801/04/05 Databook.

1st edition, 1989

[131] SGS-Thomson Microelectronics:

ST6210/ST6215/ST6220/ST6225 Databook.

1st edition, 1991

[132] SGS-Ates: M380 uP System and Instruction Set.

6.4.1976

[133] SGS-Thomson Microelectronics:

ST7 Family Programming Manual.

June 1995

[134] SGS-Thomson Microelectronics:

ST9 Programming Manual.

3rd edition, 1993

[135] Siemens AG:

SAB80C166/83C166 User's Manual.

Edition 6.90

[136] Siemens AG:

SAB C167 Preliminary User's Manual.

Revision 1.0, July 1992

[137] Siemens AG:

Telephone Controller (Single-Chip 8-Bit

CMOS Microcontroller) SAB80C382/SAB80C482.

May 1987

[138] Siemens AG:

SAB-C502 8-Bit Single-Chip Microcontroller User's Manual.

Edition 8.94

[139] Siemens AG:

SAB-C501 8-Bit Single-Chip Microcontroller User's Manual.

Edition 2.96

[140] Siemens AG:

C504 8-Bit CMOS Microcontroller User's Manual.

Edition 5.96

[141] ST Microelectronics:

STM8 CPU Programming Manual.

PM0044, Doc ID 13590 Rev 3, September 2011

[142] ST Microelectronics:

STM8S  Series and  STM8AF Series  8-bit Microcontrollers Reference

Manual.

RM0016, DocID14587 Rev 14, October 2017

[143] ST Microelectronics:

STM8S003F3 STM8S003K3 Data Sheet.

DS7147 Rev 10, August 2018

[144] C.Vieillefond:

Programmierung des 68000.

Sybex-Verlag D�sseldorf, 1985

[145] Symbios Logic Inc:

Symbios Logic PCI-SCSI-I/O Processors Programming Guide.

Version 2.0, 1995/96

[146] Texas Instruments:

Model  990 Computer/TMS9900 Microprocessor  Assembly Language Pro-

grammer's Guide.

1977, Manual No. 943441-9701

[147] Texas Instruments:

TMS9995 16-Bit Microcomputer.

Preliminary Data Manual 1981

[148] Texas Instruments:

TMS99105 and TMS99110A 16-Bit Microprocessors.

Preliminary Data Manual 1982

[149] Texas Instruments:

First-Generation TMS320 User's Guide.

1988, ISBN 2-86886-024-9

[150] Texas Instruments:

TMS7000 Family Data Manual.

1991, DB103

[151] Texas Instruments:

TMS320C3x User's Guide.

Revision E, 1991

[152] Texas Instruments:

TMS320C2x User's Guide.

Revision C, Jan. 1993

[153] Texas Instruments:

TMS320C4x User's Guide.

SPRU063C, May 1999

[154] Texas Instruments:

TMS370 Family Data Manual.

1994, SPNS014B

[155] Texas Instruments:

MSP430 Family Software User's Guide.

1994, SLAUE11

[156] Texas Instruments:

MSP430 Metering Application.

1996, SLAAE10A

[157] Texas Instruments:

MSP430 Family Architecture User's Guide.

1995, SLAUE10A

[158] Texas Instruments:

MSP430 MSP430x5xx and MSP430x6xx Family User's Guide.

October 2016, SLAU208

[159] Texas Instruments:

TMS320C62xx CPU and Instruction Set Reference Manual.

Jan. 1997, SPRU189A

[160] Texas Instruments:

TMS320C20x User's Guide.

April 1999, SPRU127C

[161] Texas Instruments:

TMS320C54x DSP Reference Set; Volume 1: CPU and Peripherals.

March 2001, SPRU172C

[162] Texas Instruments:

TMS320C54x DSP; Volume 2: Mnemonic Instruction Set.

March 2001, SPRU172C

[163] Texas Instruments:

TMS  1000 Series MOS/LSI One-Chip Microcomputers Programmer's Ref-

erence Manual.

CM122-1 1275, 1975

[164] Toshiba Corp.:

8-Bit Microcontroller TLCS-90 Development System Manual.

1990

[165] Toshiba Corp.:

8-Bit Microcontroller TLCS-870 Series Data Book.

1992

[166] Toshiba Corp.:

16-Bit Microcontroller TLCS-900 Series Users Manual.

1992

[167] Toshiba Corp.:

16-Bit Microcontroller TLCS-900 Series Data Book:

TMP93CM40F/ TMP93CM41F.

1993

[168] Toshiba Corp.:

4-Bit   Microcontroller  TLCS-47E/47/470/470A  Development  System

Manual.

1993

[169] Toshiba Corp.:

TLCS-9000/16 Instruction Set Manual Version 2.2.

10. Feb 1994

[170] Toshiba Corp.:

TC9331 Digital Audio Signal Processor Application Information.

[171] Toshiba Corp.:

Microcomputer LSI Data Book. July 1984

[172] Valvo GmbH:

Bipolare Mikroprozessoren und bipolare LSI-Schaltungen.

Datenbuch, 1985, ISBN 3-87095-186-9

[173] Digital Equipment Corporation:

VAX Architecture Reference Manual.

Order Number EY-3459E-DP, 1987

[174] Western Digital Corporation:

WD16 Microcomputer Programmer's Reference Manual.

4 October 1976

[175] Ken Chapman (Xilinx Inc.):

PicoBlaze  8-Bit Microcontroller  for Virtex-E  and Spartan-II/IIE

Devices.

Application Note XAPP213, Version 2.1, February 2003

[176] Xilinx Inc.:

PicoBlaze  8-bit Embedded Microcontroller  User Guide for Spartan-

3, Virtex-II, and Virtex-II Pro FPGAs.

UG129 (v1.1) June 10, 2004

[177] David May::

The XMOS XS1 Architecture.

Publication Date: 2009/10/19, Copyright 2009 XMOS Ltd.

[178] data sheets from Zilog about the Z80 family

[179] Zilog Inc.:

Z8 Microcontrollers Databook.

1992

[180] Zilog Inc.:

Discrete Z8 Microcontrollers Databook.

(no year of release given)

[181] Zilog Inc.:

Z380 CPU Central Processing Unit User's Manual.

(no year of release given)

[182] Zilog Inc.:

eZ8 CPU User Manual.

UM01285-0503

[183] Zilog Inc.:

Z88C00 CMOS Super8 ROMless MCU Product Specification

2003, PS014602-0103

[184] Zilog Inc.:

Z8 Encore! F0830 Series Product Specification

2012, PS025113-1212

[185] Zilog Inc.:

Z8000 Technical Manual

January 1983

[186] Zilog Inc.:

eZ80 CPU User Manual

2015, UM007715-0415

[187] Zilog Inc.:

eZ80190 Product Specification

2008, PS006614-1208

[188] Zilog Inc.:

eZ80F91 MCU Product Specification

2022, PS019217-1222

[189] Zilog Inc.:

eZ80F92/eZ80F93 Product Specification

2020, PS015317-0120

[190] Zilog Inc.:

eZ80L92 MCU Product Specification

2016, PS013015-0316